3GPP TSG-RAN WG1 Meeting #107-e	R1-2111927

# e-Meeting, November 11th – 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | XXX | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Changes of channel access procedure in TS 37.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed e?planations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In Section 4.1.3, “gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16 symbols from the end to the slot where CG-UCI is detected,….” However, in TS38.331, “cg-COT-SharingOffset indicates the offset from the end of the slot where the COT sharing indication in UCI is enabled where the offset in symbols is equal to 14*n, where n is the signaled value for cg-COT-SharingOffset”. The unit of cg-COT-SharingOffset-r16 in TS 38.331 and TS 37.213 are not consistent. - Editorial changes like missing of “s” for complex words or missing/redundancy words are also found in other sections of this specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to align description between 37.213 and 38.331 - In Section 4.1.3, “DL transmission X= cg-COT-SharingOffset-r16 “->“DL transmission X= cg-COT-SharingOffset-r16*14” Editorial changes - In Section 4.2.1.0.3, "DL transmission" -> "DL transmissions" - In Section 4.2.1.2.1, " one slot sensing slot" -> "one sensing slot" - In Section 4.2.2.2, “not been for”-> “not been used for “ - In Section 4.2.3.1 “is the set to the value”-> “is set to the value” |  |  |  |  |  |  |  |  |
| T |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistency between specification of TS37.213 and TS38.331 on meansing of cg-COT-SharingOffset. - Specicification publised with editorial typos |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3, 4.2.1.0.3, 4.2.1.2.1, 4.2.2.2, 4.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.1.3	DL channel access procedures in a shared channel occupancy

<Unchanged parts omitted>

For the case where a gNB shares a channel occupancy initiated by a UE with configured grant PUSCH transmission, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE as follows:

-	If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is provided, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer. Each row of the table provides a channel occupancy sharing information given by higher layer parameter CG-COT-Sharing-r16. One row of the table is configured for indicating that the channel occupancy sharing is not available.

-	If the 'COT sharing information' in CG-UCI detected in slot n indicates a row index that corresponds to a CG-COT-Sharing-r16 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy assuming a channel access priority class p= channelAccessPriority-r16, starting from slot n+O, where O=offset-r16 slots, for a duration of D=duration-r16 slots where duration-r16, offset-r16, and channelAccessPriority-r16 are higher layer parameters provided by CG-COT-Sharing-r16.

-	If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is not provided, and if 'COT sharing information' in CG-UCI indicates '1', the gNB can share the UE channel occupancy and start the DL transmission X= cg-COT-SharingOffset-r16*14 symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer. The transmission shall not include any unicast transmissions with user plane data and the transmission duration is not more than the duration of 2, 4 and 8 symbols for subcarrier spacing of 15, 30 and 60 kHz of the corresponding channel, respectively.

<Unchanged parts omitted>

### 4.2.1.0.3	Conditions for indicating Type 2 channel access procedures

An eNB/gNB may indicate Type 2 channel access procedures in the DCI of a UL grant or DL assignment scheduling transmission(s) including PUSCH or PUCCH on a channel, respectively, as follows:

If the UL transmissions occur within the time interval starting at t0 and ending at t0+TCO, where

-	TCO=Tmcot,p+Tg,

-	t0 is the time instant when the eNB/gNB has started transmission on the carrier according to the channel access procedure described in clause 4.1.1,

-	Tmcot,p value is determined by the eNB/gNB as described in clause 4.1.1,

-	Tg is the total duration of all gaps of duration greater than 25us that occur between the DL transmissions of the eNB/gNB and UL transmissions scheduled by the eNB/gNB, and between any two UL transmissions scheduled by the eNB/gNB starting from t0,

<Unchanged parts omitted>

### 4.2.1.2.1	Type 2A UL channel access procedure

If a UE is indicated to perform Type 2A UL channel access procedures, the UE uses Type 2A UL channel access procedures for a UL transmission. The UE may transmit the transmission immediately after sensing the channel to be idle for at least a sensing interval Tshort_ul=25us. Tshort_ul consists of a duration Tf=16usimmediately followed by one sensing slot and Tfincludes a sensing slot at start of Tf. The channel is considered to be idle for Tshort_ul if both sensing slots of Tshort_ul.are sensed to be idle.

<Unchanged parts omitted>

### 4.2.2.2	Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

<Unchanged parts omitted>

If a UE transmits transmissions using Type 1 channel access procedures associated with the channel access priority class p on a channel and the transmissions are not associated with explicit or implicit HARQ-ACK feedbacks as described above in this clause, the UE adjusts CWp before step 1 in the procedures described in clause 4.2.1.1, using the latest CWp used for any UL transmissions on the channel using Type 1 channel access procedures associated with the channel access priority class p. If the corresponding channel access priority class p has not been used for any UL transmission on the channel, CWp=CWmin,p is used.

#### <Unchanged parts omitted>

#### 4.2.3.1	Default maximum energy detection threshold computation procedure

If the higher layer parameter absenceOfAnyOtherTechnology-r14 or absenceOfAnyOtherTechnology-r16 is provided

-	X'Thresh_max=min&Tmax+10dB&Xr where

-	Xr is Maximum energy detection threshold defined by regulatory requirements in dBm when such requirements are defined, otherwise Xr=Tmax+10dB

otherwise

-	X'Thresh_max=max&-72+10⋅log10(BWMHz /20MHz) dBm,&min&Tmax,&Tmax-TA+PH+10⋅log10(BWMHz /20MHz)-PTX

where

-	TA=10dB;

-	PH=23dBm;

-	PTX is set to the value of PCMAX_H,c as defined in [3];

-	Tmax(dBm)=10⋅log103.16228⋅10-8(mW/MHz) ⋅ BWMHz (MHz);

-	BWMHz is the single channel bandwidth in MHz.