3GPP TSG RAN WG1 Meeting #107-e	R1-2111933

e-Meeting, November 11th – 19th, 2021

Agenda Item:	7.2.12

Source:	Huawei, HiSilicon

Title:	Discussion on the remaining problems of supporting Tx switching between two uplink carriers

Document for:	Discussion and Decision

Introduction

In RAN1#105-e, regarding the discussions on Rel-16 Tx switching, some progress has been made as summarized in [1]. While in RAN1#106-e, discussion as denoted in [2] has reached limited progress. In this contribution, our view on the remaining issues for the UL Tx switching are provided.

Discussion

Issue #1: Clarification on CA based SRS carrier switching

The compatibility between UL Tx switching and SRS carrier switching was intensively discussed in RAN1 #104e and RAN1 #104b-e but no consensus on how to clarify it in the specification has been achieved. Note that all components share the understanding that both the “suspending” function and the function of prioritization rules defined in the TS 38.214 clause 6.2.1.3 need clarification. Regarding a concern that the aforementioned two functions should be clarified together to solve the issue, we will try to find a complete solution for it.

Regarding the “suspending” function, as we have already analyzed in our discussion paper [3], the suspension is conditional on the SRS transmission as a winner of the prioritization rules.

A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2.

Obviously, the “suspending” and the determination and application procedure of prioritization rules are two non-overlapping functions of SRS carrier switching. Such understanding is also verified by the agreed CR [4] where “suspending” function is kept for updating prioritization rules for SRS carrier switching. As a result, the specification related to the “suspending” function can be modified independently without impacting prioritization rules. In conclusion, we suggest to adopt the TP1 in Appendix.

Proposal 1: Adopt the TP1 for TS 38.214 clause 6.2.1.3 in Appendix for uplink suspension of SRS carrier switching.

Note that there is a parallel discussion on SRS carrier switching, which is focused on how to determine the applicability of the prioritization rules. The achieved agreements are summarized as below [5]:

Agreement The prioritization rules of SRS carrier switching apply to at least the source CC. - FFS : Whether the specification needs to be updated or not Agreement Down select one from the following two options in RAN1#106-e meeting to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules: - Option 1: The UL CCs in the same band as the source CC - Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE ’s indicated uplink carrier aggregation capability. - Companies should indicate how “UE’s indicated uplink CA capability” is derived based on current ASN.1 signaling.

Essentially, the key point to determine the applicability of the prioritization rules is whether any UE RF hardware is shared or not between component carriers. When SRS carrier switching is triggered, the “switch to” carrier borrows RF hardware from the “switch to” carrier, thus the prioritization rules should be applied to the “switch from” carrier (source carrier). Similarly, in case of UL Tx switching, not only the source carrier but also the other carrier configured as a paired carrier of the source carrier should be also taken into account for the prioritization rules.

For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell and PUSCH transmission carrying aperiodic CSI happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on another serving cell configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. For a carrier of a serving cell with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

Besides, as we can see from the current version of spec, a UE will determine whether to apply the prioritization rules or not based on the “uplink carrier aggregation capability”. It was realized that how UE derives which carrier should be taken into account for the prioritization rules is not clear enough. The LTE-like definition is suggested here by introducing the carrier set, then UE will judge whether an overlapping uplink transmission is beyond “uplink carrier aggregation capability” or not on each carrier in the carrier set:

define the UL CC set S(d)= {s0(d)… sN-1(d)} as the set of carriers of serving cells that meet all the following conditions, where d is the target CC, and s0(d) is the source CC. - {s0(d)… sN-1(d)} are in the same band as s0(d). - {s0(d)… sN-1(d)} are in the same TAG as s0(d).

Based on this structure, we feel add “configured with UL Tx switching” into the condition of UL carrier set is an appropriate fix of the function of prioritization rules. In conclusion, we suggest to adopt the TP2 in Appendix.

Proposal 2: Adopt the TP2 for TS 38.214 clause 6.2.1.3 in Appendix for prioritization rules of SRS carrier switching.

Issue #2: Back-to-back switching with SRS carrier switching

In RAN1#105-e, the issue “Back-to-back switching with SRS carrier switching” has been discussed in Rel-17 UL Tx switching [6]. Since the issue associated with SRS carrier switching is not specific to Rel-17 UL Tx switching but also applicable to Rel-16, so we suggest the discussion to be handled in Rel-16.

In UL Tx switching, considering UE complexity of frequent UE RF retuning, the max number of switching for a UE is restricted to be one switching per one slot. If a UE is configured with SRS carrier switching and UL Tx switching, there may be too many switching occurrences in a slot. For example, as shown in Figure 1 and 2, a UE is configured with UL Tx switching between CC1 and CC2 and is configured with SRS carrier switching between CC2 and CC3. CC2 is the switching-from carrier and CC3 is the switching-to carrier. If the UE is to transmit on CC1 after the SRS transmission on CC3, the UE may firstly switch to CC2 and then switch to CC1 (Figure 1), resulting in two Tx switches in a short period. Alternatively, in this scenario, the UE can directly switch to CC1 after the SRS transmission on CC3 (Figure 2). According to current spec, a UE has support of a switching between CC1 and CC3 as long as the UE reports supports of both UL Tx switching and SRS carrier switching. No new capabilities are required to indicate the direct switching between CC1 and CC3.

Thus, UE can avoid unnecessary frequent Tx switching. For this, the DCI that schedules the uplink transmission succeeding a SRS carrier switching should be received by the UE early enough so that a sufficient UE processing time to prepare and handle such Tx switching is guaranteed.

Figure 1 two Tx switching after the SRS transmission on CC3

Figure 2 one Tx switching after the SRS transmission on CC3

In other words, a cut-off time to schedule the succeeding uplink transmission is defined as that the time interval between the last symbol of PDCCH scheduling the succeeding uplink transmission and the first symbol of SRS transmission on CC3 should be at least at least N2 symbols and an additional time duration (for example, the RF retuning time).

This method not only releases UE burden from too frequent RF retuning but also allows a gNB to schedule the succeeding slot so that no UL throughput loss for network operation, the cost is the DCI should be received by the UE earlier. With the help of earlier arrival of scheduling DCI, if a UE prefer to implement two switches/RF retunings in this case, then it is still up to UE to do it. But it provides the availability to avoid frequent RF retunings. For example, if the switching time from CC3 to CC1 is the sum of two reported switching gaps, UE can choose the best implementation, either direct switching or two-step switching.

Certainly, if the succeeding uplink transmission is too much later from the SRS transmission on CC3, there is no frequent Tx switching in a short period. Similar to the agreed restriction of one switching per one slot for UL Tx switching, the short period to define a too frequent Tx switching is also one slot. In other words, as shown in Figure 3, the scheduling cut-off time is applicable to a uplink transmission succeeding a SRS carrier switching occurrence only if the time interval between the last symbol of the RF retuning time after SRS transmission and the first symbol of the succeeding uplink transmission is less than or equal to 13 symbols.

Figure 3 PDCCH schedules the next transmission after the SRS transmission on CC3

Proposal 3: For a UE configured with UL Tx switching on two uplinks and configured with SRS carrier switching for a third uplink, if a uplink transmission is scheduled after a SRS carrier switching occurrence and the time interval between the first symbol of the uplink transmission and the last symbol of SRS transmission is less than or equal to an interval of 13 symbols plus the RF retuning time required by SRS carrier switching, then the last symbol of PDCCH scheduling the uplink transmission should be no later than at symbol L, where the time interval between symbol L and the first symbol of SRS transmission is larger than  N2 symbols plus the RF retuning time.

- In case of different SCS between the uplink transmission and the SRS transmission, the 13 symbols are with respect to the smaller SCS.

Conclusions

In this contribution, remaining issues of uplink Tx switching are discussed. The following proposals are provided:

Proposal 1: Adopt the TP1 for TS 38.214 clause 6.2.1.3 in Appendix for uplink suspension of SRS carrier switching.

Proposal 2: Adopt the TP2 for TS 38.214 clause 6.2.1.3 in Appendix for prioritization rules of SRS carrier switching.

Proposal 3: For a UE configured with UL Tx switching on two uplinks and configured with SRS carrier switching for a third uplink, if a uplink transmission is scheduled after a SRS carrier switching occurrence and the time interval between the first symbol of the uplink transmission and the last symbol of SRS transmission is less than or equal to an interval of 13 symbols plus the RF retuning time required by SRS carrier switching, then the last symbol of PDCCH scheduling the uplink transmission should be no later than at symbol L, where the time interval between symbol L and the first symbol of SRS transmission is larger than  N2 symbols plus the RF retuning time.

- In case of different SCS between the uplink transmission and the SRS transmission, the 13 symbols are with respect to the smaller SCS.

- References

R1-2106151, “[105-e-NR-R16-TxSwitching-01] Summary of email discussion on maintenance of Rel-16 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #105-e, May 10th – 27th, 2021.

R1-2108417, “[106-e-NR-Maintenance-Others-02] Summary of email discussion on remaining issues on Rel-16 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #106-e, August 16th – 27th, 2021.

R1-2105925, “Discussion on the remaining problems of supporting Tx switching between two uplink carriers”, Huawei, HiSilicon, RAN1#105-e, May 10th – 27th, 2021.

R1-2104043, “Timelines for SRS carrier switching”, Moderator (Qualcomm), 3GPP RAN1 #104bis-e, April 12th – 20th, 2021.

R1-2106100, “Summary of [105-e-NR-7.1CRs-12] Issue#26 SRS carrier switching”, Moderator (ZTE), 3GPP RAN1 #105-e, May 10th – 27th, 2021.

R1-2106150, “[105-e-NR-R17-TxSwitching-01] Summary of email discussion on Rel-17 uplink Tx switching”, Moderator (China Telecom), 3GPP RAN1 #105-e, May 10th – 27th, 2021.

- Appendix

- TP1 for the clarification of SRS carrier switching in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> - 6.2.1.3 UE sounding procedure between component carriers A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier c2, and also the uplink transmission on carrier c3 if the UE is configured with uplinkTxSwitching-r16 for uplink switching between uplink carrier c2 and c3. <Unchanged parts are omitted – 38.214>

- TP2 for the clarification of SRS carrier switching in 38.214 clause 6.2.1.3

<Unchanged parts are omitted – 38.214> - 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell d with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sd={s0d…sN-1(d)} as the set of carriers of serving cells that each carrier meets one of the following conditions: - si(d) is in the same band as s0(d), or s0(d) and si(d) are both configured with uplinkTxSwitching-r16. - si(d) is in the same TAG as s0(d). where 1≤i≤N-1. <Unchanged parts are omitted – 38.214> For an SRS transmission starting in symbol Nc1d of carrier dc1 and a conflicting transmission in carrier sc2i(d) starting in symbol Nsc2i, where 1≤i≤N-1, the UE shall apply the prioritization / dropping rules in the remainder of this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and NdNc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between the last symbol of PDCCH and NsiNc2 is at least N2 symbols; and - semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before Nc1d, and considered active at least N2 symbols before Nsic2. where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across c1d, si(d)c2 and their corresponding scheduling cells. The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier and transmission of a physical signal/channel over a carrier of a serving cell in set Sd: - the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set Sd happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set Sd happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall drop PUCCH/PUSCH transmission carrying periodic/semi-persistent CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set Sd configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. - the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in the set Sd whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306]. <Unchanged parts are omitted – 38.214>