3GPP TSG RAN WG1 #107-e	R1-2111937

e-Meeting, November 11th – 19th, 2021

Title:	Draft Reply LS on initial state of elements controlled by MAC CEs

Response to:	R2-2109098/R1-2110756

Release:	Rel-15, Rel-16

Work Item:	NR_newRAT-Core

Source:	[RAN1], Huawei

To:	RAN2

Cc:	RAN4

Contact Person:

Name:	Xi Zhang

E-mail Address:	panda.zhang@huawei.com

# 1. Overall Description

RAN1 would like to thank RAN2 for the questions related to initial state of elements controlled by MAC CEs. RAN1 provides the following answers.

Question 1: Firstly, since a handover only refers to PCell change, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using handover should be applied for both PCell change and PSCell change/addition in case of DC. Answer 1: Yes. Question 2: Secondly, RAN2 respectfully asks RAN1 to clarify whether the initial deactivation when using configuration should be applied for both “initial configuration by RRC” and “reconfiguration by RRC”. Answer 2: The initial state of deactivation is applied for “initial configuration by RRC”. For “reconfiguration by RRC” excluding the handover case, as long as the corresponding elements are NOT impacted by the reconfiguration message, they should maintain their previous state; and for elements newly added or modified by the reconfiguration message, initial state of deactivation is applied. Question 3: Thirdly, RAN2 respectfully asks RAN1 to clarify whether the UE behavior relevant to (Enhanced) PUCCH spatial relation Activation/Deactivation MAC CE should be aligned with the other MAC CEs. Answer 3: This MAC-CE is used for activating one spatial relation for one or multiple PUCCH resource(s), and initial state of deactivation is applied for configured candidate spatial relations.

# 2. Actions:

To: RAN2

ACTION: RAN1 respectfully asks RAN2 to take the above information into account for future work.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1   Meeting #107b-e               Jan. 17 – 25, 2022           Online