3GPP TSG RAN WG1 #107-e			R1-2111974

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.5.2

Source:	LG Electronics

Title: 	Discussion on accuracy improvement for UL-AoA positioning

Document for:  	Discussion and decision

Introduction

This contribution discusses Rel-17 UL-AoA enhancements specifically in the UL-AoA measurement report enhancement.

Accuracy improvement for UL-AoA positioning

UL-AOA measurement report enhancements

Agreement: - For the first arrival path measurements on SRS for positioning resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference} - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - For the first arrival path measurements on SRS for MIMO resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF Note: The operation of SRS for MIMO is transparent to the UE

When the spacing of antenna array with elements exceeds λ/2 distance, there can be antenna pattern that has multiple side lobes. This may result in the ambiguity issue while selecting the direction for the UL-AOA reporting. To resolve the ambiguity, it was agreed to allow reporting multiple UL-AoAs.

In addition, in the previous meeting [1], it was agreed that gNB can report not only multiple UL-AoAs and but also related timing/power measurement results to LMF. The remaining issue is whether gNB reports either multiple SRS-RSRPs or single SRS-RSRP in the measurement report.

If gNB can measure SRS-RSRP for each UL AoA depending on gNB implementation, it would be allowed that gNB reports to LMF multiple SRS-RSRPs. But, because single timing information (UL-RTOA or gNB Rx-Tx time difference) is reported, it is doubt whether reporting multiple SRS-RSRPs is beneficial for providing better accuracy of positioning estimation than reporting single SRS-RSRP. Hence, we think if a benefit of reporting multiple SRS-RSRPs is not clearly justified, it does not need to introduce gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference }.

Proposal #1:

- If a benefit of reporting multiple SRS-RSRPs is not clearly justified, it does not need to introduce gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference }.

Conclusion

In this contribution, we have discussed potential enhancements for UL-AOA positioning in Rel-17, and our proposal is summarized below.

UL-AOA Report Enhancements

Proposal #1:

- If a benefit of reporting multiple SRS-RSRPs is not clearly justified, it does not need to introduce gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference }.

Reference

- 3GPP RAN1, Chairman’s Notes RAN1#106bis-e meeting.