3GPP TSG RAN WG1 #107			         	          R1-2111994

e-Meeting, November 11th – 19th, 2021

Source: 	ETRI

Title: 	Type A PUSCH repetitions for Msg3

Agenda Item:	8.8.3	 Type A PUSCH repetitions for Msg3

Document for:	Discussion

- Introduction

In this contribution, we address views mainly about time domain enhancements for Msg3 PUSCH repetition

- Discussion

  - Indication of the number of repetitions for Msg3 initial/re-transmission

Indication of the number of repetitions for Msg3 initial transmission:

Working Assumption - 106b Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field.

The remaining issues about the working assumption is to determine one field to indicate the repetition factor. We prefer the new TDRA table since it is, at least for our perspective, aligned to the NR design philosophy. In the Rel-16, the repetition factor is indirectly indicated within the TDRA index, and the TDRA table is extended for this purpose. We think this approach can be reused in the Rel-17 work item.

Introducing a new TDRA table may have less specification efforts if additional default TDRA table is not specified. In other words, additional TDRA table can reuse the legacy default TDRA table with repetition factor. We believe this can save the discussion efforts.

Observation 1: Introducing a new TDRA table with repetition factor requires minimal efforts if the default TDRA table is reused as much as possible.

For the MCS field, the UE in edge would have a low code rate and the high code rate with high order QAM may not be used. In our understanding, it is allowed that the gNB schedules a TB with the high MCS and also with a repetition. This scheduling can be useful when UL resources are scarce, e.g., in some of TDD slot patterns. Thus, we think to keep the flexibility in the MCS field.

Following the above arguments, we prefer to enhance the TDRA table to indicate repetition factors. In addition, we would like to introduce a unified framework to both initial transmission and retransmissions. The common configuration in SIB1 can affect the retransmission for the Msg3.

Proposal 1: Introduce a TDRA table with repetition factors (Alt 1 in the working assumption).

This approach is summarized as the Alt 1 in the working assumption. Even we can consider a few repetition factors are contained in the SIB1 for flexibility perspective. In this way of extension, the association between a repetition factor and a TDRA index should also be provided in the SIB1.

Proposal 2: The TDRA index listed in the pusch-configCommon can additionally include the repetition factor.

Interpreting the TDRA table:

Agreement - 106 Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. - Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. - Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

In order to indicate whether a legacy behaviour (i.e., UE’s only assumption of Msg3 transmission once) or a Rel-17 behaviour (i.e., UE may read different TDRA table or may interpret the legacy TDRA table differently, etc), there were discussions in the previous meeting. In our understanding, two options have little performance difference, but have different flexibility. For option 1, the UE decides whether legacy behaviour or the new behaviour. For option 2, the gNB decides whether legacy behaviour or the new behaviour.

In our view, if a UE determines to repeat Msg3 by choosing a specific preamble, then the gNB should allow it. There is no strong reason to fallback to the legacy operation, because the SIB1 may include the RSRP threshold to balance the legacy Msg3 load and the repeated Msg3 load. In our view, the option 2 have one more step for the same goal and may not be necessary.

Proposal 3: A UE expects the repetition factor once the UE request Msg3 repetition (Option 1).

Regarding the DCI format 1_0,

Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted. Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

The retransmitting Msg3 is scheduled by the DCI format 0_0 with TC-RNTI. The TDRA table can be either the default table or the list from pusch-configCommon. Comparing the initial transmission, the retransmission can be UE-specific and the number of repetitions can be from the TDRA list or additional field. Both alternatives are feasible if the UE shares the same TDRA list, or if the UE can recognize the additional field in the DCI format 0_0 by TC-RNTI when the UE is determined to be in the coverage edge.

Proposal 4: Retransmitting Msg3 keeps the unified framework with the initial Msg3 transmission (option 1).

The same TDRA list from the initial transmission can be applied. If we allow implicit indication, then the initial transmission can determine the number of repetitions for the retransmission. We think that we need more flexible solution because the retransmission may need less repetitions than the initial transmission when any contention has not occurred.

- Conclusion

We address our view on coverage enhancements and propose the followings:

Proposal 1: Introduce a TDRA table with repetition factors (Alt 1 in the working assumption).

Proposal 2: The TDRA index listed in the pusch-configCommon can additionally include the repetition factor.

Proposal 3: A UE expects the repetition factor once the UE request Msg3 repetition (Option 1).

Proposal 4: Retransmitting Msg3 keeps the unified framework with the initial Msg3 transmission (option 1).