3GPP TSG RAN WG1 Meeting #107-e  			    			             R1-2111997

e-Meeting, November 11st – 19th, 2021

# Source:	ETRI

# Title:		Discussion on resource allocation for power saving

# Agenda item:	8.11.1.1

# Document for:	Discussion

Introduction

The following agreements for resource allocation for power saving have been made in the RAN1#105-e, RAN1#106-e, and RAN1#106bis-e meetings [1][2][3]:

Agreement [1]: - For the set of Preserve values in periodic-based partial sensing, - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList. - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList - This is per mode 2 Tx resource pool (pre-)configuration - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx - FFS whether the monitoring can be mandatory Agreement [2]: Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met: - L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool - FFS: When the trigger will be received by L1 - The resource pool is (pre-)configured to enable partial sensing - Partial sensing is configured by higher layer in the UE Agreement [3]: When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value. Agreement [3]: When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only - Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS) - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min. - Tproc,0, Tproc,1 are in units of physical time/slots - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed). - For the CPS monitoring window [n+TA, n+TB]: - TA = X - FFS value X for TA including X=1 and negative value - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1). - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-) configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - Approach 3: (independent approach for different case) - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies. - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies. Agreement [2]: For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing, - Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc - Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable. - FFS: If additional enhancements are needed for enabling/disabling - The triggering of re-evaluation and pre-emption checking is as in R16. Agreement [2]: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e - Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed - Note, lower value means higher priority - FFS whether resource pool partitioning can be additionally applied - Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement, - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement. - Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking - Option 12: No special consideration

Based on the above agreements, we present our views on resource allocation for power saving in this contribution.

Sidelink power saving

Periodic-based partial sensing

It has been agreed that a single set of Preserve can be (pre-)configured and a UE may also monitor other Preserve values not part of the configured subset by implementation. It is FFS whether the UE mandatorily monitors the occasions corresponding to Prsvp_TX or not. In order to avoid consecutive resource collision, it would be much safer for the UE to additionally monitor the occasions corresponding to Prsvp_TX when Prsvp_TX is not in the restricted subset. However, if this UE behavior is mandated, UE power consumption will be increased more than expected. As long as whether to monitor the occasions corresponding Prsvp_TX can be fully handled by the proper subset (pre-)configuration, it should be left up to UE implementation rather than mandate it.

Proposal 1:  It is up to UE implementation to additionally monitor occasions corresponding to Prsvp_TX.

Contiguous partial sensing

Many conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc) for triggering contiguous partial sensing have been discussed over the past several meetings. We do not think it is proper that L1 decides whether or not to perform contiguous partial sensing by itself based on some of the conditions or its combinations. Since the exact triggering condition can be very complicated and different depending on the scenarios, it should be left as UE implementation rather than specifying a concrete triggering condition. It is sufficient that the higher layer determines whether or not to trigger the contiguous partial sensing and just informs the result to L1. Accordingly, the L1 will be provided an enabling indication for contiguous partial sensing by the higher layer. When the resource (re-)selection is triggered in slot n, the contiguous partial sensing can be triggered before or in slot n depending on the TA and TB values.

Proposal 2: L1 is triggered to perform contiguous partial sensing by the higher layer before or in slot n.

Based on the agreements in the previous meetings, TA and TB values for determining the sensing window of contiguous partial sensing can be zero, positive or negative and it is still under discussion for the details. Similar to the triggering condition, since the optimized TA and TB values may be very different depending on the various scenarios, it is more desirable that TA and TB values are determined and explicitly informed to L1 by the higher layer rather than predefined as some specific values or implicitly determined by other parameter values. Such flexible configurability would be beneficial to minimize the power consumption due to unnecessary sensing operation.

Proposal 3: L1 is provided TA and TB values from the higher layer before or in slot n.

Depending on the TA and TB values, the sensing window of the contiguous partial sensing can be overlapped with the sensing occasions of periodic-based partial sensing. In this case, both the sensing results from contiguous partial sensing and periodic-based partial sensing can be used for resource (re-)selection.

Observation 1: When the sensing occasions for contiguous partial sensing and periodic-based partial sensing are overlapped, the UE will take into account both sensing results for the purpose of resource (re-)selection.

For full sensing, a sensing window length is (pre-)configured and the sensing window is also used for re-evaluation/pre-emption checking. This principle can be reused for the partial sensing case as well. For contiguous partial sensing, the sensing window length is given by W=TB–TA.  For re-evaluation/pre-emption checking when partial sensing (including both periodic-based partial sensing and contiguous sensing) is performed for resource (re-)selection, the same sensing window length W can be assumed for the power saving purpose. If UE is not provided TA and TB values because of not using contiguous partial sensing, then the UE can assume a default value (e.g., 32 slots) for the sensing window for re-evaluation/pre-emption checking.

Proposal 4: For the UE with resource pool configured by partial sensing, contiguous partial sensing is assumed to be used for re-evaluation/pre-emption checking.

- The sensing window length for re-evaluation/pre-emption is provided by the higher layer.

- If the sensing window length for re-evaluation/pre-emption is not provided, a fixed 32-slot is assumed as default.

Random resource selection

The resource collision issue of random resource selection has been identified and discussed over several meetings. Type-A/B UE can do random resource selection but cannot perform re-evaluation/pre-emption checking. As a result, when the priority of the random selection UE is lower than the priority of a sensing UE, the resource collision between these two UEs cannot be avoided. In order to resolve this issue, increasing the priority of random selection UE by a certain level has been proposed by several companies. Basically, this solution may lead to a similar result of using a 1-bit field in the SCI to indicate the random selection UE is performing random resource selection. In other words, we don’t need to increase the priority of the random selection based on pre-defined rules,

but it is enough to indicate whether the UE is a random resource UE or not. Therefore, it is proposed to include an extra 1-bit field indicating that the UE is performing random resource selection in the 1-st SCI.

Proposal 5: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a 1-bit field in the SCI is used to indicate that the UE is performing random resource selection or not.

Another issue of random resource selection is contiguous resource collision among the random selection UEs with the same periodic traffic. If the periodicities are the same or one periodicity is multiple of the other periodicity, once the same resources are selected by two different UEs, then the persistent resource collision can happen. To resolve this issue, resource randomization based on sub-channel hopping can be considered. For N reserved resources, Rx1,y, Rx2,y+P, ..., RxN,y+(N-1)*P, the subchannel indexes x1, x2, ..., xN, can be pseudo-randomly changed based on source ID.

Proposal 6: For random resource selection, the starting subchannel indices for the reserved resources are pseudo-randomly changed based on Source ID.

Additional enhancements

In addition to random resource selection and partial sensing, additional enhancements for power saving can be considered. First of all, the concept of partial sensing can be extended in a different domain (e.g., frequency) rather than the time domain. In Uu link, BWP adaptation has been used for power saving via BWP switching. Supporting narrow BWP (and/or reduced resource pool) operation with BWP switching could be one solution. For example, in power saving mode, a UE can operate in narrow BWP in order to reduce power consumption for monitoring. When a large amount of data transmission/reception is needed, the UE can switch its BWP into a larger one. In order to support BWP switching, it may need to support configuration of multiple SL BWPs, cross-BWP switching, cross-slot switching, SCI-only transmission in a single subchannel, and so on. SL resource pool switching can be also considered instead of SL BWP switching. Since it may cause a heavy specification workload, it should be studied carefully. Even though the main objective of inter-UE coordination is for enhanced reliability and reduced latency in resource allocation mode 2, the inter-UE coordination can be also used for power saving. For example, if a UE selects resource(s) for its own transmission within a white list which is provided as a coordination message, the power consumption for resource selection can be reduced. Other enhancements such as reduced number of blind decoding/estimations, simplified sensing with 1st stage SCI only, etc. could be also studied.

Proposal 7: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

Summary

In this contribution, the following proposals and observations are made for resource allocation for power saving.

Proposal 1:  It is up to UE implementation to additionally monitor occasions corresponding to Prsvp_TX.

Proposal 2: L1 is triggered to perform contiguous partial sensing by the higher layer before or in slot n.

Proposal 3: L1 is provided TA and TB values from the higher layer before or in slot n.

Proposal 4: For the UE with resource pool configured by partial sensing, contiguous partial sensing is assumed to be used for re-evaluation/pre-emption checking.

- The sensing window length for re-evaluation/pre-emption is provided by the higher layer.

- If the sensing window length for re-evaluation/pre-emption is not provided, a fixed 32-slot is assumed as default.

Proposal 5: For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, a 1-bit field in the SCI is used to indicate that the UE is performing random resource selection or not.

Proposal 6: For random resource selection, the starting subchannel indices for the reserved resources are pseudo-randomly changed based on Source ID.

Proposal 7: Additional enhancements besides random resource selection and partial sensing for power saving should be also discussed.

Observation 1: When the sensing occasions for contiguous partial sensing and periodic-based partial sensing are overlapped, the UE will take into account both sensing results for the purpose of resource (re-)selection.

References

- Chairman’s notes, RAN1#105-e, May 2021.

- Chairman’s notes, RAN1#106-e, August 2021.

- Chairman’s notes, RAN1#106bis-e, October 2021.