3GPP TSG RAN WG1 Meeting #107e	R1-2112024

e-Meeting, November 11th – 19th, 2021

Source:	Sharp

Title:	Discussion on resource allocation for power saving

Agenda Item:	8.11.1.1

Document for:	Discussion and Decision

Introduction

A revised work item on NR sidelink enhancement was approved in RAN#90-e meeting [1], with one of the objectives to specify resource allocation to reduce power consumption of the UEs, as follows:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2] - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2. - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly. - This work should consider the impact of sidelink DRX, if any.

In this document, we share our views on a few aspects relating to resource allocation for power saving.

Discussion

Details of TA, TB for aperiodic transmission

The latest proposal on TA and TB for aperiodic transmission are as follows,

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only - Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS) - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min. - Tproc,0, Tproc,1 are in units of physical time/slots - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed). - For the CPS monitoring window [n+TA, n+TB]: - TA = X - FFS value X for TA including X=1 and negative value - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1). - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-) configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - Approach 3: (independent approach for different case) - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies. - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.

The details of TA and TB for periodic transmission were discussed and agreed in RAN1#106bis. In this case, UE would randomly select Y candidate slots within RSW and each slot corresponds to a set of periodic sensing occasions for PBPS. In our view, even the resource pool may disable the periodic reservation for another TB, uniform design for both aperiodic transmission and periodic transmission is preferred, i.e. UE selects some candidate slots within the RSW, which is Approach 1.

Regarding Approach 1, since UE is unable to predict the triggering slot n, the TA is set to non-negative value, e.g. 1. In our understanding, there is no benefit to configure a minimum of M consecutive slots for CPS, since if the remaining PDB is closely to the configured M, the RSW would be almost empty w/o candidate slots. Thus, we propose to remove the (pre-)configuration of M for aperiodic transmission. Therefore, for aperiodic transmission, TA =1 and n+ TB is (Tproc,0SL+Tproc,1SL) slots earlier than ty0SL, where ty0SL is the first slot of the selected Y’ candidate slots.

Proposal 1: For a resource (re-)selection procedure triggered by aperiodic transmission, Approach 1 with updates is adopted.

Proposal 2: For Approach 1, remove the (pre-)configuration of M consecutive logical slots.

Proposal 3: For a resource (re-)selection procedure triggered by aperiodic transmission, TA =1 and n+ TB is (Tproc,0SL+Tproc,1SL) slots earlier than ty0SL, where ty0SL is the first slot of the selected Y’ candidate slots.

Pre-emption check and re-evaluation

In Rel-16 NR V2X, if any resource(s) is reported by PHY as re-evaluation or pre-emption, MAC layers would remove the resource(s) from the sidelink grant and randomly select resource(s) from the reported set of candidate resources to replace the removed resource(s). Note that the reported set of candidate resources is a remaining set after the exclusion procedure, i.e. sensing, which means when MAC layer triggers the re-evaluation or pre-emption check, PHY would perform all the steps specified for resource selection procedure, i.e. step 1-7 in clause 8.1.4 of TS38.214. Basically, except for determination of whether to report any resource(s) as “re-evaluation/pre-emption”, resource selection and re-evaluation/pre-emption check share common procedures. It was noticed that for all the agreements in Rel-17, re-evaluation/pre-emption check and resource selection for partial sensing were separated discussed. With the above understanding in Rel-16 NR V2X, the sensing occasions of PBPS and CPS defined for resource selection in Rel-17 partial sensing can be applied for re-evaluation and pre-emption check as well.

For PBPS, if UE selects Y candidate slots up to implementation, the Y candidate slots shall include at least the slots corresponding to the resource(s) subject to pre-emption check or re-evaluation provided by MAC layer, otherwise UE would be unable to determine whether to report re-evaluation or pre-emption check.

Proposal 4: The sensing occasions of PBPS and CPS defined for resource selection apply for re-evaluation and pre-emption in partial sensing.

Proposal 5: For periodic-based partial sensing, the selected Y candidate slots include at least the slots corresponding to the resources subject to pre-emption check and/or re-evaluation.

In Rel-16 NR V2X, resource(s) of a sidelink grant subject to pre-emption check has been indicated by a prior SCI. It is also noted that each candidate slot among the Y candidate slots in NR periodic-based partial sensing corresponds to a set of periodical sensing occasions if the resource pool enables periodic reservation. For the default case (i.e. no configuration), the set of periodical sensing occasions include sensing occasions corresponding to every periodicity in sl-ResourceReservePeriodList, which may include the slot of the prior SCI which indicates the resource subject to pre-emption check. Due to half-duplex constraint, the slot of the prior SCI cannot be monitored. Therefore, it is straightforward to preclude the slot which indicates the resource subject to pre-emption check from the periodical sensing occasions, if any.

Proposal 6:  If tySLis included in the set of Y candidate slots and at least a resource subject to pre-emption check is in slot tySL, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL except for the slot of a prior SCI which indicates the resource.

Handling of mixed resource allocation schemes in a resource pool

It was agreed in RAN1#103-e that a resource pool can be configured to enable any combination of full sensing, partial sensing, and random resource selection, indicating that a mixed use of resource allocation schemes in a same resource pool is allowed. A few questions related to this agreement arise.

Question #1: How does a UE capable of multiple resource allocation schemes configured to enable in a TX pool determines which one to use?

In LTE V2X, V2X sidelink transmissions are divided into the following:

- “Non-P2X related V2X sidelink communication” where full sensing is used, unless in exceptional cases where the UE falls back to random resource selection.

- “P2X related V2X sidelink communication” where either partial sensing or random resource selection is used as per configuration or it is left to UE implementation which one to use if both are configured. Similarly to the non-P2X case, the UE falls back to random resource selection in case of exception and when partial sensing is chosen.

In NR sidelink, with the support for a much wider range of scenarios / use cases, it is highly undesirable to hard-code “P2X related V2X sidelink communication” into the specs. On the other hand, it seems natural to follow the principle in LTE V2X that if multiple resource allocation schemes are enabled, instead of (pre-) configuring a fixed resource allocation scheme for all UEs, each UE should be able to choose one of the configured schemes, depending on at least UE capability and the usage scenario. And we think this should not be completely left to UE implementation, otherwise the system performance may fluctuate dramatically as the resource allocation schemes chosen by the UEs communicating with each other vary. For example, the system performance might be severely degraded if a large number of UEs use random resource selection. It is proposed that some kind of prioritization is applied for selection of resource allocation schemes in case multiple resource allocation schemes are configured to enable in a resource pool, e.g. partial sensing should be prioritized if the UE is in power-saving mode, and full sensing should be prioritized if the UE is in non-power-saving mode.

Proposal 7: Prioritization is applied for selection of resource allocation schemes in case the UE is capable of multiple resource allocation schemes configured to enable in a resource pool.

Question #2: Should a Rel-16 UE use full sensing even if a chosen resource pool is not configured to enable full sensing?

In our view, in order to be backward compatible with Rel-16 UEs, full sensing should be always allowed in a resource pool, even if not explicitly configured to enable in the resource pool configuration.

Proposal 8: Full sensing is always allowed in a resource pool.

Conclusion

In this contribution, we discuss a few aspects relating to relating to resource allocation for power saving, and make the following proposals:

Proposal 1: For a resource (re-)selection procedure triggered by aperiodic transmission, Approach 1 with updates is adopted.

Proposal 2: For Approach 1, remove the (pre-)configuration of M consecutive logical slots.

Proposal 3: For a resource (re-)selection procedure triggered by aperiodic transmission, TA =1 and n+ TB is (Tproc,0SL+Tproc,1SL) slots earlier than ty0SL, where ty0SL is the first slot of the selected Y’ candidate slots.

Proposal 4: The sensing occasions of PBPS and CPS defined for resource selection apply for re-evaluation and pre-emption in partial sensing.

Proposal 5: For periodic-based partial sensing, the selected Y candidate slots include at least the slots corresponding to the resources subject to pre-emption check and/or re-evaluation.

Proposal 6: If tySLis included in the set of Y candidate slots and at least a resource subject to pre-emption check is in slot tySL, the UE monitors slots of periodic sensing occasions ty-k×PreserveSL except for the slot of a prior SCI which indicates the resource.

Proposal 7: Prioritization is applied for selection of resource allocation schemes in case the UE is capable of multiple resource allocation schemes configured to enable in a resource pool.

Proposal 8: Full sensing is always allowed in a resource pool.

References

RP-202846, “WID revision: NR Sidelink enhancement”, LG Electronics, RAN#90-e.

RAN1 agreements on SL resource allocation for power saving

RAN1#103-e

Conclusion

- SL reception Type A and Type D should be used as the reference for evaluation and designing of SL power saving features in R17.

  - Type A: UE is not capable of performing reception of any SL signals and channels, FFS with exception of performing PSFCH and S-SSB reception (aim to conclude in RAN1#104-e)

  - Type D: UE is capable of performing reception of all SL signals and channels defined in R16. It does not preclude UE to perform reception of a subset of SL signals/channels

  - If there are evaluations with assumptions other than the above reference, the detailed assumptions need to be reported

  - Note: the types and the associated capability defined here are not intended to be defined as Rel-17 UE features as is.

Agreements:

- Partial sensing based RA is supported as a power saving RA scheme

  - FFS details

- Random resource selection is supported as a power saving RA scheme

  - FFS any changes or enhancement

  - FFS on conditions to apply random resource selection

Agreements:

- In R17, a SL Mode 2 Tx resource pool can be (pre-)configured to enable full sensing only, partial sensing only, random resource selection only, or any combination(s) thereof

  - FFS details, including usage, potential restrictions, whether/how any enhancement or condition is needed for the coexistence of full sensing and power saving RA scheme(s) in a same resource pool, etc.

Agreements:

- Re-evaluation and pre-emption checking are not supported by UEs that do not perform any sensing (i.e. PSCCH reception)

- Re-evaluation and pre-emption checking are supported by UEs that perform sensing

  - FFS details and any conditions(s) in which re-evaluation and pre-emption can be performed

- FFS whether/how re-evaluation and pre-emption can be supported by UEs performing random resource selection that do perform sensing

- Note: details about sensing in this context, including when it is performed, are not decided yet.

Agreements:

- Further study congestion control based on CBR and CR for power saving RA schemes

  - Identify necessary changes from R16 CBR/CR (if any), including transmission resource selection and transmission parameters that can be adjusted and applicable to power savings RA schemes

  - Note: this is not intended to require all UEs to perform sensing for the purpose of CBR measurement

RAN1#104-e

Agreements:

- Random resource selection is applicable to both periodic and aperiodic transmissions

  - FFS conditions for random resource selection

Conclusion:

- PSFCH reception is not included for Type A UE

- S-SSB reception is not included for Type A UE

- SL reception Type B is additionally added

  - Type B: Same as Type A with an exception of performing PSFCH and S-SSB reception

- Note: the same conditions as in RAN1#103-e regarding the context of the discussion of Type A and Type D still apply (also applicable to type B)

Agreements:

In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, it is up to UE implementation to determine a set of Y candidate slots within a resource selection window, where

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- The resource selection window is [n+T1, n+T2]

  - As a baseline, T1 and T2 are defined in the same way as in R16 NR-V2X according to step 1 [TS 38.214 Sec. 8.1.4]

  - Further discuss whether or not to introduce a threshold to re-define T1 and T2 such that

    - T1 ≥ 0 (subject to processing time constraint Tproc, 1), and T2 ≤ remaining PDB

    - T2-T1 ≤ (pre-)configured threshold

- A minimum value for Y is (pre-)configured from a range of values, FFS details

- FFS any restriction to determine Y candidate slots (including its relationship with SL-DRX)

- FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note: The terminology “periodic-based partial sensing” is based on the “partial sensing” used in LTE-V and it is intended to be used for the design and discussion of partial sensing in Rel-17.

Agreements:

In a resource pool (pre-)configured with at least partial sensing, if UE performs periodic-based partial sensing, at least when the reservation for another TB (when carried in SCI) is enabled for the resource pool and resource selection/reselection is triggered at slot n, the UE monitors slots of at least one a set of periodic sensing occasions, where a periodic sensing occasion is a set of slots according to

if tvSL is included in the set of Y candidate slots.

- Preserve is a periodicity value from the configured set of possible resource reservation periods allowed in the resource pool (sl-ResourceReservePeriodList). Down select to one:

  - Option 1:  Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

  - Option 2:  PreservePreserve Preserve corresponds to a subset of values from the configured set sl-ResourceReservePeriodList

    - FFS how to determine the subset (e.g., by (pre-)configuration, UE determination)

  - Option 3: PreservePreserve Preserve is a common divisor among values in the configured set sl-ResourceReservePeriodList

  - Option 4: FFS others

- k equals tois selected according to (down select to one)

  - Option 1: Only the most recent sensing occasion within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 2: The two most recent sensing occasions within sensing window for a given reservation periodicity before the resource (re)selection trigger or the set of Y candidate slots subject to processing time restriction

  - Option 3: All possible sensing occasions after n –T0

  - Option 4: Only one periodic sensing occasion for one reservation period. The k value is up to UE implementation. Max value for k is (pre-)configured.

  - Option 5: k is (pre-)configured, including multiple values

  - Option 6: (pre-)configuration of a bitmap, same as in LTE-V

  - Option 7: FFS others

- FFS relationship between periodic sensing occasions and SL-DRX

- FFS condition(s) and timing(s) for which periodic-based partial sensing is performed by UE

- Note: companies are encouraged to show performance data for the down selections

Agreements:

- In a resource pool (pre-)configured with at least partial sensing, if UE performs contiguous partial sensing and resource (re-)selection is triggered in slot n, support the following option:

  - Option 1: For the purpose of resource (re-)selection, the UE monitors slots between [n+TA, n+TB] and performs identification of candidate resources, in or after slot n+TB, based on all available sensing results, including periodic-based partial sensing results (if applicable).

    - FFS TA, TB (including the possibility of equal to zero, positive or negative) and remaining details (in particular, whether there should be exclusion of slots, changes in TA/TB values for different purposes, etc.)

    - FFS whether n can be replaced by e.g., index of some of Y candidate slots

  - FFS condition(s) in which contiguous partial sensing is performed by UE

  - FFS interaction with SL-DRX, if any

  - FFS interaction with periodic-based partial sensing, if any

  - Other options are not precluded

  - Note: This option is not to replace random resource selection only without sensing or re-evaluation and pre-emption checking

RAN1#104b-e

Conclusion:

- In periodic-based partial sensing,

  - It is not necessary to further discuss whether or not to introduce a threshold to re-define T1 and T2.

Agreements:

- In periodic-based partial sensing,

- For the set of Preserve values, down-select to one of the following in RAN1#105-e

    - Alt.1: Preserve corresponds to all values from the configured set sl-ResourceReservePeriodList

    - Alt.2: A set of Preserve values is (pre-)configured and includes up to the full set of values from the configured set sl-ResourceReservePeriodList

      - FFS if support multiple sets of Preserve values based on one or more metrics

      - FFS whether/how to restrict the set of values

- For the k value, down-selection to one of the following in RAN1#105-e (further refinement of each of the alternatives is possible)

        - Alt 1: Option 1 as in RAN1#104-e

        - Alt 2: A modified Option 5 as in RAN1#104-e, where the modification is such that it also includes option 1

          - FFS how to (pre-)configure (e.g. including bitmap), whether a maximum number of k values is needed, and whether it can be up to UE implementation to select a k value based on the (pre-)configuration

        - FFS details, e.g., sensing before the resource (re)selection trigger or the first slot of the set of Y candidate slots subject to processing time restriction, etc.

      - Note: companies are encouraged to provide more evaluations

Agreement:

- When periodic-based partial sensing is potentially performed by UE in a mode 2 Tx resource pool provided by higher layer, at least all of the followings are met:

  - Periodic reservation for another TB (sl-MultiReserveResource) is enabled for the resource pool

  - The resource pool is (pre-)configured to enable partial sensing

  - Partial sensing configured by higher layer in the UE

RAN1#105-e

Agreem Agreement:

- For the set of Preserve values in periodic-based partial sensing,

  - If no (pre-)configuration (i.e., by default), Preserve corresponds to all values from the (pre-)configured set sl-ResourceReservePeriodList.

  - Otherwise, a single set of Preserve values can be (pre-)configured, where the set of Preserve values are restricted to a subset of the (pre-)configured set sl-ResourceReservePeriodList

    - This is per mode 2 Tx resource pool (pre-)configuration

    - A UE by implementation may also monitor other sl-ResourceReservePeriodList values not part of the restricted subset

      - In particular, the UE may additionally monitor occasions corresponding to P_RSVP_Tx

        - FFS whether the monitoring can be mandatory

Agreement:

- In periodic-based partial sensing for resource (re)selection, the UE at least monitors in periodic sensing occasion(s) for a given reservation periodicity before the first slot of the selected Y candidate slots subject to processing time restriction for the identification of candidate resources.

  - The processing time restriction includes Tproc,0SL  and Tproc,1SL.

  - Aspects relating to sensing during SL DRX are to be discussed separately

- Relationship to re-evaluation and pre-emption operation for periodic-based partial sensing to be discussed separately

  - FFS details including whether monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection or re-evaluation and pre-emption checking

Agreement:

- For the k value in periodic-based partial sensing for resource (re)selection,

  - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction.

  - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value

    - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included.

    - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions)

    - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring.

  - FFS relationship between periodic-based partial sensing occasions and SL-DRX

  - Note:

    - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement:

- For random resource selection,

  - Reuse the maximum distance separation of 32 logical slots for a HARQ retransmission resource reserved by a prior SCI for the same TB, which was defined in R16 for full sensing operation.

  - SL HARQ feedback enabled transmission is supported (FFS applicable conditions if any)

    - The minimum HARQ feedback time gap (Z) shall be respected between any two selected resources of a TB where a HARQ feedback for the first of these resources is expected.

- FFS the impact of resource collision when random resource selection is performed by a UE which does not perform sensing / re-evaluation and pre-emption checking in a resource pool with mixed RA schemes (e.g. for low priority or any priority transmissions).

  - Including study potential solution(s) if the impact is not negligible (e.g. threshold based, raising priority, minimum time gap, pattern based, a priori SCI reserving initial transmissions, resource pool partitioning, and etc.).

Agreement:

In contiguous partial sensing for resource (re)selection, TA and TB values can be zero, positive or negative

- TA and TB values or range depend on different operating scenarios or conditions (e.g., periodic/aperiodic traffic, predictability of triggering slot n, remaining PDB, re-evaluation/pre-emption checking, HARQ feedback, CBR/CR parameter, power saving, etc)

  - FFS details

- FFS: details of how periodic-based partial sensing and contiguous partial sensing are used for re-evaluation and pre-emption checking. Including how to reduce UE’s power consumption (caused by additional sensing operation of re-evaluation/pre-emption) after its resource selection, with the considerations of different operating scenarios or conditions (e.g., pre-emption enabled/disabled, HARQ-ACK enabled/disabled, etc).

RAN1#106-e

Agreement

In periodic-based partial sensing, UE monitoring of periodic sensing occasions between triggering slot n and the first slot of the selected Y candidate slots subject to processing time restriction is performed as part of resource (re)selection.

Agreement

Conditions in which contiguous partial sensing is performed by UE, when at least all of the followings are met:

- L1 [is expected to be or] is triggered by higher layer to report resources for resource (re-)selection in a mode 2 Tx pool

  - FFS: When the trigger will be received by L1

- The resource pool is (pre-)configured to enable partial sensing

- Partial sensing is configured by higher layer in the UE

Agreement

For a resource pool (pre-)configured with at least partial sensing and UE is configured by its higher layer for partial sensing,

- Periodic-based partial sensing and contiguous partial sensing schemes are supported for resource re-evaluation and pre-emption checking

  - FFS details of partial sensing for re-evaluation and pre-emption checking, including any restrictions / conditions on performing PBPS and CPS, subset of resources, timing, candidate resource set (SA) and etc

- Same as in Rel-16, the higher layer indicates a set of resources (r0,r1,r2,…) and/or a set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively

  - Pre-emption checking is enabled according to the Release-16 interpretation of sl-PreemptionEnable.

    - FFS: If additional enhancements are needed for enabling/disabling

- The triggering of re-evaluation and pre-emption checking is as in R16.

Agreement

When UE performs only contiguous partial sensing (CPS) in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) disabled, and a resource (re)selection is triggered in slot n,

- The resource selection window (RSW) is [n+T1, n+T2] where T2 is defined based on step 1) of Rel-16 TS 38.214 Sec. 8.1.4

  - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- On the sensing window [n+TA, n+TB] for CPS,

  - Details of TA and TB values based on the agreements from previous RAN1 meetings

  - FFS whether and how to define a minimum CPS window size, including (pre-)configurability and the case when TB - TA is smaller than the minimum CPS window size

  - FFS whether and how to define a maximum value / upper bound for TB with respect at least to the minimum RSW size and the remaining PDB, including (pre-)configurability

- FFS how a set of candidate resource (SA) is initialized considering candidate single-slot resources, including

  - Whether and how to define a minimum size for the RSW (e.g., Rel-16 T2min), including (pre-)configurability

  - Whether the set SA is confined within a set of Y candidate slots within the RSW

- UE performs resource exclusion from the set SA based on at least all available sensing results and based on step 6) and 7) of Rel-16 TS 38.214 Sec. 8.1.4

- Note, re-evaluation and pre-emption checking in a resource pool with periodic reservation for another TB (sl-MultiReserveResource) disabled is considered separately.

- FFS: Details on T1

Agreement

For random resource selection in a resource pool (pre-)configured with full/partial sensing and random resource selection, down-select to one of the followings in RAN1#106bis-e

- Option 1: A priority threshold value or a range of priority levels is (pre-)configured for the resource pool, below or within which random resource selection is allowed

  - Note, lower value means higher priority

  - FFS whether resource pool partitioning can be additionally applied

- Option 2: Increase the priority for the transmission based on random selection and indicate the new priority value in the priority field in the 1st-stage SCI

  - FFS: An extra field is added in SCI for indicating the original priority value associated with QoS requirement,

  - FFS: A 1-bit field in the SCI indicates that the UE is performing random resource selection, or

  - FFS: An extra field is added in SCI for indicating the mapping to the original priority value associated with QoS requirement.

- Option 7: Exclude resources reserved by UE performing random selection without re-evaluation / pre-emption checking, regardless of their priorities. E.g. a 1-bit field in the SCI indicates that the UE is performing random resource selection and not performing re-evaluation and pre-emption checking

- Option 12: No special consideration

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n,

  - The resource selection window (RSW) is [n+T1, n+T2], and T1 and T2 are defined in the same way according to step 1) of Rel-16 TS 38.214 Sec. 8.1.4

    - FFS whether UE determines a new set of Y candidate slots within the RSW and monitors corresponding periodic sensing occasions between slot n and the first slot of the new Y candidate slots subject to processing constraints

    - FFS how to initialize a set of candidate resource (SA) for the triggered resource (re)selection procedure and which partial sensing scheme(s) and results can be used for resource exclusion in the resource (re)selection procedure

    - FFS whether the resource selection window [n+T1, n+T2] should be confined within a set of periodic set of resources and its relationship with SL-DRX

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n

  - A set of candidate resource (SA) is initialized to the set of selected Y candidate slots of PBPS

    - UE performs contiguous partial sensing in [n+TA, n+TB] for resource exclusion from the initialized candidate resource set (SA)

      - FFS details of TA and TB based on the agreement(s) from previous RAN1 meetings

- Note, re-evaluation and pre-emption checking based on periodic-based and contiguous partial sensing schemes is considered separately

FFS: The condition under which UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled

RAN1#106b-e

Agreement

In the agreement from RAN1#105-e, the working assumption is confirmed and the FFS bullet (in RED) is closed without any agreement.

Agreement from RAN1#105-e: - For the k value in periodic-based partial sensing for resource (re)selection, - By default, the UE monitors the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots subject to processing time restriction. - If (pre-)configured, UE additionally monitors periodic sensing occasions that correspond to a set of values which can be (pre-)configured with at least one value - (Working assumption) Possible values correspond to the most recent sensing occasion for a given reservation periodicity before the resource (re)selection trigger slot n or the first slot of the set of Y candidate slots, and the last periodic sensing occasion prior to the most recent one for the given reservation periodicity are included. - FFS: whether/which other values and details of the (pre-)configuration (e.g. max number of values or sensing occasions) - FFS: whether a value denotes a specific occasion to monitor or the earliest occasion to start the monitoring. - FFS relationship between periodic-based partial sensing occasions and SL-DRX - Note: - This is for the case when the resource (re)selection triggering slot n is expected by UE

Agreement

When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled,

- For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings:

  - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots.

    - By default, M is 31 unless (pre-)configured with another value.

Agreement

For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitored in PBPS, it shall be (pre-)configured jointly for all Preserve values.

- The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured

Working Assumption

In a resource pool (pre-)configured to enable partial sensing, when UE is configured with partial sensing by its higher layer, the resources for which the UE performs re-evaluation and/or pre-emption checking are for the initial transmission and retransmissions of every TB according to Rel-16 specification based on partial sensing results.

- Same as in Rel-16, for periodic transmission, re-evaluation check is not applied to the resources that have been signalled in current period or previous periods, except that it is up to UE implementation whether to apply re-evaluation check to the resources in non-initial reservation period that have been signalled neither in the immediate last nor in the current period.

- The resource in the main bullet is the set of resources (r0,r1,r2,…) and/or the set of resources (r0',r1',r2',…)  for re-evaluation and/or pre-emption checking, respectively, which has been agreed in RAN1 #106-e.

Agreement

When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated.

- Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS)

  - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW.

    - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission

    - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s)

    - FFS whether/how to prioritize/select resources based on partial sensing results.

    - FFS: How to select Y’ in case of CPS only

  - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots.

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots.

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-)configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

  - FFS: RSW in case of CPS only

- Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS)

  - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min.

    - Tproc,0, Tproc,1 are in units of physical time/slots

    - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed).

  - For the CPS monitoring window [n+TA, n+TB]:

    - TA = X

      - FFS value X for TA including X=1 and negative value

    - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1).

      - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority

      - FFS the range of (pre-) configured M from a TBD lowest value up to 30

      - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed

- Approach 3: (independent approach for different case)

  - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies.

  - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies.