3GPP TSG RAN WG1 #107-e						R1- 2112055

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.3.4

Source: 	LG Electronics

Title: 	Discussion on propagation delay compensation enhancements

Document for:	Discussion and decision

Introduction

In the last e-meeting, following agreement and guidance were made [1]:

Agreement For evaluation of the overall time synchronization error for RTT-based propagation delay compensation, - Alt.1 for RTT-based PDC Agreement For evaluation of the overall time synchronization error for TA-based propagation delay compensation, - Alt.1 for TA-based PDC Agreement If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC. - FFS whether/how to configure UL signal for enhanced TA-based PDC Agreement If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5. - FFS the value of k - FFS the reporting range of Rx-Tx time difference measurement for PDC

In this contribution, we discuss and provide our views on propagation delay compensation enhancements and necessity of RAN1 involvement.

Discussion

  - Propagation delay compensation schemes

In the last RAN meeting, it has been discussed whether to support both TA-based PDC and RTT-based PDC. From discussion in RAN1 so far, it would be common understanding that TA-based PDC with existing assumption could not cover control-to-control scenario. Therefore, in order to support control-to-control scenario, it would be necessary to support either RTT-based PDC or TA-based PDC with reduced timing error.

As RAN guidance, reply LS from RAN 4 would be essential to discuss whether TA-based PDC with reduced timing error can be used for time synchronization. If both RTT-based PDC and TA-based PDC can achieve error budget requirement of control-to-control scenario, it would be reasonable to take one of PDC schemes for minimal specification impact on RAN1 and RAN2.

Observation #1: Based on reply LS from RAN4, discuss to support either RTT-based PDC or TA-based PDC for minimal specification effort in RAN1#107-e.

Conclusions

In this contribution, propagation delay compensation enhancements and necessity of RAN1 involvement were discussed, and the followings are proposed.

Observation #1: Based on reply LS from RAN4, discuss to support either RTT-based PDC or TA-based PDC for minimal specification effort in RAN1#107-e.

References

RAN1 Chairman notes, RAN1#106bis-e