3GPP TSG RAN WG1 #107-e		R1- 2112079

E-Meeting, November 11th – November 19th, 2021

Source:	China Unicom

Title:	Performance Evaluation Results

Agenda Item:	8.14.1

Document for:	Discussion and Decision

Introduction

In previous RAN1 e-meetings, basic traffic model and evaluation methodology for XR evaluation has been agreed. Based on the agreements, we simulate DL video stream with SU-MIMO. In this contribution, we provide our evaluation results focus on XR capacity.

Discussion

2.1 Traffic model

- Single-stream Traffic Model

In this contribution, the network deployment was Urban Macro and Dense Urban. Statistical traffic model is used for the capacity simulation, and packet arrival interval for each stream is set to 60fps. Both XR and CG services were evaluated for downlink, detailed parameters of the single-stream model are shown in the following table.

Table 1: The Single-stream Traffic Model Simulation Assumptions

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Services | VR/AR |  | CG |  |
| Average data rate | 30Mbps | 45Mbps | 30Mbps | 8Mbps |
| Mean Packet size | 62.5K Bytes | 93.75K Bytes | 62.5K Bytes | 16.67K Bytes |
| Packet distribution | Truncated Gaussian Distribution: - STD: 0.105*mean - Max: 1.5*mean - Min: 0. 5*mean |  |  |  |
| Periodicity | 60fps |  |  |  |
| PDB | 10 ms |  | 15 ms |  |
| Jitter | Truncated Gaussian Distribution: - Mean: 0ms - STD: 2ms - Range: 2ms |  |  |  |

- Multi-stream Traffic Model

According to previous agreements, there are two options for I/P-frames multi-stream modeling:

- Option 1A: slice-based traffic model

- Option 1B: Group-Of-Picture (GOP) based traffic model

In this contribution, Option 1B is simulated for VR/AR service. The PER and PDB requirements are defined as [1%, 1%, 10ms, 10ms], and alpha values are considered in this section. The details of the GOP-based traffic model for downlink are listed in the following table.

Table 2: The Multi-stream Traffic Model Simulation Assumptions

| Parameter | Value |
| --- | --- |
| Services | VR/AR |
| Packet modelling | I/P-stream Option 1B: GOP-based |
| Average data rate | 30Mbps |
| alpha | 1.5, 2.0 |
| [PER_I, PER_P, PDB_I, PDB_P] | [1%, 1%, 10ms, 10ms] |

2.2 Baseline capacity results for Single-stream

In this section, we present the XR capacity results for Single stream. The system level simulation assumptions are listed in Appendix A. For the definition of a satisfied UE, we use the agreement that more than 99% of packets are successfully delivered within the given air interface PDB. The (PSR, PDB) combinations in this section is (99, 10) for VR/AR, and (99, 15) for CG.

Figure 1. Capacity results of the single-stream in FR1 Urban Macro

Figure 2. Capacity results of the single-stream in FR1 Dense Urban

Figure 1 show the percentage of satisfied UE under the Urban Macro deployment, and Figure 2 show the percentage of satisfied UE under the Urban Macro deployment. We use a parameter called Traffic_model - Maen_packet_size - PDB to distinguish the different configurations. As can be seen, UE satisfaction rate gradually declines with the increase of the UE numbers per cell. The system capacity is obtained under the definition that 90% of users being satisfied. It is observed that the capacity of XR system is listed in Table 3.

Observation 1： For single DL video stream with SU-MIMO, the average numbers of supported users per cell in Urban Macro and Dense Urban deployments are:

Table 3: Capacity results of the single-stream model

| Traffic Model | AR/VR-45 | AR/VR-30 | CG-30 | CG-8 |
| --- | --- | --- | --- | --- |
| Urban Macro | 4.7 | 5.5 | 7.9 | >30 |
| Dense Urban | 1.7 | 6.3 | 6.7 | >30 |

2.2 Baseline capacity results for multi-stream

Downlink system capacity evaluations for the multi-stream model are presented in this section. The traffic model parameters indicated in Table 2 and Table A-1 are used for 30 Mbps AR/VR service. Alpha=1.5 and 2.0 are considered. The simulation results for Urban Macro and Dense Urban deployments are given below:

Figure 3. Capacity results of the multi-stream in FR1 Urban Macro

Figure 2. Capacity results of the multi-stream in FR1 Dense Urban

Observation 2： For Multi DL video stream with SU-MIMO, the average numbers of supported users per cell in Urban Macro and Dense Urban deployments are:

Table 4: Capacity results of the Multi-stream model

| Traffic Model | AR/VR-30 |  |
| --- | --- | --- |
| Alpha | 1.5 | 2 |
| Urban Macro | 4.2 | 2.4 |
| Dense Urban | 1.5 | 6.1 |

Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1： For single DL video stream with SU-MIMO, the average numbers of supported users per cell in Urban Macro and Dense Urban deployments are:

Table 3: Capacity results of the single-stream model

| Traffic Model | AR/VR-45 | AR/VR-30 | CG-30 | CG-8 |
| --- | --- | --- | --- | --- |
| Urban Macro | 4.7 | 5.5 | 7.9 | >30 |
| Dense Urban | 1.7 | 6.3 | 6.7 | >30 |

Observation 2： For Multi DL video stream with SU-MIMO, the average numbers of supported users per cell in Urban Macro and Dense Urban deployments are:

Table 4: Capacity results of the Multi-stream model

| Traffic Model | AR/VR-30 |  |
| --- | --- | --- |
| Alpha | 1.5 | 2 |
| Urban Macro | 4.2 | 2.4 |
| Dense Urban | 1.5 | 6.1 |

# References

- 3GPP RAN1#104-e Chairman’s Notes, February 2021.

- 3GPP RAN1#104bis-e Chairman’s Notes, April 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 2021.

- 3GPP RAN1#106-e Chairman notes, August, 2021.

- 3GPP RAN1#106bis-e Chairman notes, October, 2021.

# Appendix A – System level Simulation assumptions

Table A-1. Simulation assumption for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenarios | Urban Macro | Dense Urban |
| Layout | 21cells with wraparound |  |
| Channel Model | UMa |  |
| Carrier frequency | 4.0 GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | [DDDSU DDDSU] Detailed S slot format is 10D:2F:2U. |  |
| Inter Site Distance | 500m | 200m |
| BS Antenna Height | 25m |  |
| BS Antennas | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |
| CSI-RS Port | 32 |  |
| UE Antenna Height | General equation: hUT=3(nfl – 1) + 1.5 - nfl for outdoor UEs: 1 - nfl for indoor UEs: nfl ~ uniform(1, Nfl ) where Nfl ~ uniform(4,8) |  |
| UE Antennas | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |  |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| BS TX Power | 49 dBm per 20MHz | 44 dBm per 20MHz |
| UE MAX Power | 23dBm |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | SU-MIMO Proportional Fair |  |
| MCS | Up to 64QAM |  |
| UE distribution | 80% indoor, 20% outdoor |  |
| UE speed | 3 km/h |  |
| Channel Estimation | Ideal |  |
| Downtilt | 12 degree |  |
| CSI acquisition | CSI-RS: Period=5ms, Density=1, feedback delay=1ms, 4bit quantization, and modelling the error SRS: Period=5ms |  |
| Transmission scheme | Close loop rank adaptation |  |
| PHY processing delay | UE PDSCH processing Capability #1 |  |
| PDCCH overhead | 2 symbols |  |
| Target BLER | 10% |  |
| Max HARQ transmission | 4 |  |