3GPP TSG-RAN WG1 #107-e		R1-2112156

eMeeting, Nov 11- 19, 2021

Source:	Ericsson

Title:	CRS Rate-matching and PDCCH enhancement for DSS

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction

In this document, we discuss CRS rate-matching and PDCCH reception enhancements to improve DSS performance.

# Discussion

## CRS Rate-matching enhancement

Figure 1. Impact of CRS interference level and potential gains from rate-matching around it

Figure 1 shows UE throughput for different cases of CRS rate-matching around a neighbor cell interferer.

- ‘baseline’ case in the figure is UE throughput without inter-cell CRS interference (though with rate-matching around the own CRS), i.e. the performance that would be expected with colliding CRS between cells.

- ‘ratematched CRS RE’ corresponds to RE level rate-matching around the interferer CRS REs only

- ‘ratematched symbol’ corresponds to rate-matching around the full symbols with CRS interference.

- SIR levels in the legend refer to cases with CRS interference with different signal to CRS interference levels.

As illustrated in the figure, CRS interference limits performance especially when noise and interference from other sources is low (medium/high SNR in the plot). For such cases, rate-matching around neighbor cell CRS REs is useful to improve performance. The gain depends on the strength of the interferer, with larger gains for stronger interferer (lower SIR in the figure). For the simulated scenario, RE-level rate-matching provides around ~20% throughput gain compared to symbol-level rate-matching. The gain can vary based on total available PDSCH REs.

Currently, the CRS rate-matching pattern can only be configured via RRC which limits flexibility compared to symbol/PRB level rate-matching that can be adapted by DCI based L1 signaling. The level of CRS interference and the strongest interferer itself (i.e., v-shift) can change dynamically. Considering this, supporting dynamic adaptation between different CRS rate-matching patterns via DCI would be beneficial as it provides fast adaptation without need for frequent RRC signaling to turn on/off rate-matching or to match the pattern of strongest interferer.

## PDCCH reception in symbols overlapping with LTE CRS

Currently, UEs are not expected to decode NR PDCCH that overlaps with LTE CRS. This limits PDCCH capacity in DSS deployments, as symbol 0 and 1 cannot be used for PDCCH if the CRS is transmitted on four ports. Removing the restriction that UEs are not expected to decode PDCCH in symbols that overlap with LTE CRS can increase PDCCH capacity.

NR PDCCH could then be transmitted in symbols overlapping with LTE CRS, with the CRS puncturing the PDCCH and DMRS. This will naturally impact the decoding performance of the PDCCH, as shown in Figure 2. 1 symbol (solid blue) corresponds to PDCCH transmitted only in symbol 2, while 2 symbol (dashed red) corresponds to PDCCH transmitted in symbol 1 and 2, where the PDCCH is punctured by CRS in symbol 1. The simulations are for a DCI payload of 55 bits excluding CRC.

Assuming the SINR distribution in Figure 3 (a low-band UMA scenario), the decrease in PDCCH decoding performance corresponds to an increase in average CCEs per DCI of 28% (5.8 CCEs for PDCCH in symbol 2 and 7.4 CCEs for PDCCH in symbol 1 and 2). However, allowing PDCCH in two symbols results in 67% more resources for PDCCH (one third of the resources in symbol 1 are occupied by CRS). The net expected gain in PDCCH capacity would be approximately 1.67/7.4 1/5.8=1.31, i.e. 31%.

Since no change is needed for PDCCH transmission or reception (ratematching around CRS not required), the implementation impact by removing the restriction is expected to be limited.

Figure 2. NR PDCCH decoding performance

Figure 3. Example SINR distribution

# Annex A: Simulation assumptions

## Link level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 100 ns |
| UE speed | 3 km/h |
| Correlation | Medium |
| Symbols reserved for PDCCH | For evaluation in section 2.1. -- Symbol 0 and 1 reserved for LTE PDCCH, symbol 2 reserved for NR PDCCH |
| Number of BS antennas | 4 Tx (Two cross-polarized antenna pairs) |
| Number of UE antennas | 2 Rx (One cross-polarized antenna pair) |
| Channel estimation | Practical |
| Receiver type | MRC |
| DMRS | Type A, 1+1 symbol, no FDM with data |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 55 bits |
| Interleaving | Interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 PRBS |

## System level

| Parameters | Values |
| --- | --- |
| Carrier frequency | 2 GHz |
| SCS | 15 kHz |
| Simulation bandwidth | 20 MHz |
| BS antenna height | 25 m |
| UE height | 1.5m |
| TRP transmit power | 46 dBm for 10MHz |
| Scenario | Urban Macro |
| ISD | 500m |
| TRP antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) |
| UE antenna configuration | (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) |
| Device deployment | 80% indoor, 20% outdoor |
| UE speeds | Indoor users: 3km/h |
|  | Outdoor users (in-car): 30 km/h |
| BS noise figure | 5 dB |
| BS antenna element gain | 8 dBi |
| UE noise figure | 9 dB |
| Thermal noise level | -174 dBm/Hz |
| Traffic | Full Buffer |
| Macro sites | 19 |
| Downtilt | 102° |
| Minimum BS to UE distance | 35m |