3GPP TSG RAN WG1 #107-e	    	     			R1-212157

e-Meeting, Nov 11th -19th, 2021

Agenda Item:	8.16.7

Source:	Ericsson

Title:	UE features for UEPS

Document for:	Discussion/Decision

# 1	Introduction

This document discusses UE features related to Rel-17 UE power savings work item.

Following agreements were in RAN1#106bis-e regarding Rel-17 UE power savings features.

Agreement

FG 29-2 is kept as “TRS resources for idle/inactive UEs” as follows.

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS eceiving for idle/inactive Ues 1. Support reading TRS configuration from SIB 2. Support eceiving L1 indication for TRS availability FFS whether to separate the capability for receiving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

Agreement

FG 29-1 is kept as placeholder as “Paging enhancement” as follows

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  | High idle/inactive mode UE power consumption if NR SA networks | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.

# 2	Discussion and proposals

Below are some of the main changes proposed for the FGs (using track changes in below table).

- For FG 29-1 (PEI),

  - FG name: “Paging enhancement” could have a descriptive FG name ‘Paging Early Indication’

  - ‘Consequence column’: Current sentence (High idle/inactive mode UE power consumption  in NR SA networks) should be removed. We are OK to add “UE does not support PEI and UE subgroup indication” or be left empty.

  - Allowing optional UE capability signalling can be useful for NW to know when to turn on these features. However, NW can also infer it from other capabilities related to PEI (e.g. RAN2 related). Therefore, from RAN1 perspective, FG29-1 can be supported as optional without capability. If ‘optional with capability signalling’ is identified as essential, it should be per-UE granularity. So, at this point it can be concluded that the granularity should be per-UE and leave the last column in yellow for further discussion.

- For FG 29-2 (TRS occasions),

  - Components: should be updated – there is no need to separate out UE capability for receiving L1 indication for availability.

  - ‘Consequence column’: The current sentence (Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode) should be removed. OK to add ’Support “UE does not support PEI and UE subgroup indication” or be left empty.

  - Allowing optional UE capability signalling can be useful for NW to know when to turn on these features, but it is not essential to have capability signalling for this or any additional separate capabilities (for reception of L1 signalling). TRS occasion configuration and L1 availability configuration is not UE-specific. Idle/Inactive UEs can ignore any TRS occasion-related information they are not interested in/capable of receiving. If ‘optional with capability’ signalling is identified as essential, it should be per-UE granularity.

- For FG29-3 (PDCCH monitoring adaptation)

  - FG name: should be descriptive. Suggest to use “29-3a: PDCCH skipping”, “29-3b: SSSG switching”, etc

  - Component column: Prefer to use more descriptive wording and alignment with current draft spec

  - ‘Consequence column’ can be left empty – there is no need to say that the feature is not supported as a consequence.

  - We are OK to use the four FGs from FL summary as starting point and update the components, etc as shown in below table. FGs 29-3c and FG 29-3d could be left in square brackets for now since they can be incorporated/subsumed into the FGs 29-3a/3b – For example, FG 29-3c could be merged into FG 29-3b and UE can report the number of supported SSSGs (2 or 3) as a component of FG29-3b. FG29-3d could be implicitly supported by a UE supporting 29-3a and 29-3b.

  - The FGs should be ‘Per UE’ with at most FR1/FR2 and TDD/FDD differentiation, if needed. Handling of licensed/unlicensed, etc can be discussed separately for all the FGs.

As a general comment, ‘Consequence column: The ‘consequence if feature is not supported by the UE’ should be left empty’ and making generalized statements about deployments, etc should be avoided. All features being developed in the WI are for UE power savings.

Based on above discussion, below are proposed updates to the UEPS FG table.

Proposal 1: Update the UE feature list for UEPS as shown in below Table.

- NR_UE_pow_sav_enh

| 29. NR_UE_pow_sav_enh | 29-1 | Paging Early Indication | 1. Support paging early indication 2. Support UE subgroup indication |  | N |  |  | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS reception for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  |  | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of PDCCH monitoring adaptation based on PDCCH skipping for up to M= 3 skipping durations |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | SSSG switching with 2 SSSGs | Support of PDCCH monitoring adaptation based on SSSG switching for up to 2 SSSGs Timer-based SSSG switching |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3c] | [SSSG switching with 3 SSSGs] | Support of PDCCH monitoring adaptation based on SSSG switching for up to 3 SSSGs Timer-based SSSG switching | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | [29-3d] | [PDCCH skipping and SSSG switching with 2 SSSGs via single DCI field] | Support of PDCCH monitoring adaptation based on PDCCH skipping and SSSG switching for 2 SSSGs via single DCI field Timer-based SSSG switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

# References

[1] R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN WG1 #106-e, Aug 2021.

[2] R1-2109713	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.), RAN WG1#106bis-e, Oct 2021.