3GPP TSG RAN WG1 #107-e		                    R1-2112159

e-Meeting, Nov 11th -19th, 2021

Agenda Item:	8.16.16

Source:	Ericsson

Title:	UE features for 1024-QAM

Document for:	Discussion

# 1	Introduction

RAN1#90-e updated the WI on introduction of 1024-QAM for NR PDSCH with RAN4 as the leading WG, with objectives for RAN1 and RAN2. The objectives are as follows:

This document discusses the UE features for 1024-QAM. Preliminary input for Rel-17 UE feature groups from rapporteurs was in [2] and agreements from last meeting for 1024QAM were captured in [4].

# 2	Discussion

## 2.0	Discussion on UE capabilities for 1024-QAM

According to the WID objective, the support of 1024-QAM should be defined as per-band UE capability. We are not in favor of introducing a per-FSPC indication of 1024QAM support as it is not aligned with the WI objective, and it also causes significantly large increase in capability signaling due to per-CC per band combination indication signaling.

Next, there are capabilities for the scaling factor and supportedModulationOrderDL which are used in data rate calculations. Some background on the two data rate related capabilities is in [3].

If Rel-17 UE reports these parameters assuming the maximum modulation order of 1024-QAM, then if UE connects to a legacy network, there might be unnecessary reduction in peak rate, e.g. if UE indicates scaling factor of 0.75 with max 1024-QAM, but legacy NW interprets it as scaling factor 0.75 with max 256-QAM.

The motivations for scalingFactor from Rel-15/16 are still applicable and hence scaling factors for 1024-QAM with same value range can be supported.

The supportedModulationOrderDL signaling from the UE leads to extremely low data rates that do not seem useful with 1024-QAM, for example, (e.g. when supportedModulationOrderDL = pi/2-BPSK, scalingFactor = 0.4). Therefore, there is no strong need to introduce a new capability supportedModulationOrderDL for 1024-QAM and the scaling factor is sufficient. However, supportedModulationOrderDL needs to be defined for data rate calculation when UE reports 1024-QAM support. For this out of below two options, we prefer Option 1.

Option 1 : Define supportedModulationDL = 1024QAM when UE indicates 1024QAM support for a band.

Option 2 : Update the field description for existing capability supportedModulationOrderDL as shown below.

| supportedModulationOrderDL Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled per band i.e. [pdsch-1024QAM-FR1] if [pdsch-1024QAM-FR1] is signalled for the band , otherwise the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12]. | FSPC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Regarding introducing a new capability for supportedModulationOrderDL-r17 for 1024QAM (especially as per FSPC), we are not supportive of adding such a capability since its main purpose (as explained above) is just for data rate indication without provide significant benefit to the UE since UE still has to support 1024-QAM based PDSCH demodulation for each carrier perhaps with a lower data rate, and the scaling factor is enough for such purpose already. Moreover per-FSPC signaling of this leads large signaling overhead.

## 2.1	UE feature table for 1024-QAM

An updated UE FG table based on the above discussion in shown in below table, where prominent changes (relative to the table agreed in last meeting) are highlighted in red font. We also note that a TP may also be needed e.g. to update the data rate formula (in 38.306) including definition of supportedModulationDL when UE indicates support of 1024-QAM.

Adopt the updates for UE feature group list in table (from section 2.1) for support of 1024-QAM (red text highlights the relative changes on top of the agreed FG table).

For data rate calculation in 38.306, define supportedModulationDL = 1024QAM for carrier(s) in a band when UE indicates support of 1024QAM support for that band.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_DL1024QAM_FR1 | 36-1 | 1024QAM for PDSCH for FR1 | Support 1024QAM for PDSCH for FR1 including 1024QAM modulation scheme as defined in TS 38.211, MCS and CQI feedback tables based on 1024QAM modulation order as defined in TS 38.214. | pdsch-256QAM-FR1 | Yes | N/A | No support of 1024 QAM for PDSCH | Per Band | N/A | No Applicable only to FR1 | N/A | Note from WI objective: DL PDSCH 1024QAM for FR1 should be defined as a per-band UE capability | Optional with capability signalling |
| 36. NR_DL1024QAM_FR1 | 36-2 | scalingFactor for 1024QAM | Indicates the scaling factor to be applied to the band in the max data rate calculation as defined in 4.1.2 when support of 1024-QAM is signalled for the band. |  |  |  |  |  |  |  |  | Supported values [0.4, 0.75, 0.8, 1.0] | Optional with capability signaling |

# References

- [1] RP-202044, New WID on Introduction of DL 1024QAM for NR FR1, Ericsson, RAN#89e, October 2020.

- [2] R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN WG1 #106-e, Aug 2021.

- [3] R1-1912779, Rel-15 NR capability signaling related to UE data rate, Ericsson, RAN1#99, November 2019.

[4] R1-2109919, Summary of UE features for DL 1024QAM for NR FR1, Moderator (AT&T), RAN