3GPP TSG-RAN WG1 Meeting #107-e	R1-2112198

e-Meeting, November 11th – November 19th, 2021

Agenda item:	8.1.2.2

Source: 	Qualcomm Incorporated

Title: 	Enhancements on Multi-TRP inter-cell operation

Document for:	Discussion/Decision

# Introduction

The Rel-17 WID for further enhancements on MIMO (FeMIMO) is approved [1] and further revised in [2], which includes the following objective:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2:

  - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

Rel. 16 mTRP enhancements only supports intra-cell operation. This is because for the case of inter-cell multi-TRP operation, SSBs of the second TRP cannot be used as a QCL source as they are transmitted with a different PCI and UE is only aware of one PCI in the serving cell. In fact, the issue was discussed even during Rel. 16, but there was not enough time to specify the required enhancements to support the inter-cell operation case. In this contribution, we discuss the aspects related to inter-cell multi-TRP operation including QCL / spatial relation info and PL RS enhancements for DL / UL, respectively.

# Discussions

When SSB of the non-serving cell is used in QCL-Info to define the TCI state, DL beam is based on the non-serving cell SSB. If UL spatial relation / PL-RS is not enhanced similarly, it becomes unclear how PUCCH/PUSCH/SRS can be transmitted toward the corresponding TRP for multi-DCI based multi-TRP. Of course, one possibility is to always rely on CSI-RS for spatial relation info / PL-RS. However, this requires additional overhead and reduces the flexibility since the option of using SSB to indicate UL beam and PL-RS is effectively removed for the second TRP. In addition, in RAN #92e, it is agreed that inter-cell multi-TRP operation should be based on Rel-15/16 TCI framework, which means that unified TCI state (for both DL and UL) cannot be used for this purpose in Rel. 17. Hence, UL spatial relation / PL-RS should be enhanced accordingly.

The required enhancements for this purpose are basically the same as the enhancements for QCL-Info. That is, when SSB is used as reference signal in each of the SRS-SpatialRelationInfo, PUCCH-SpatialRelationInfo, PUCCH-PathlossReferenceRS, PUSCH-PathlossReferenceRS, and pathlossReferenceRS under SRS-ResourceSet the corresponding IE can indicate whether the SSB-Index is associated with the serving cell or is associated with non-serving cell. For example, this is shown below for SRS-SpatialRelationInfo. Similar to the case of QCL-Info, this is an example of configuration and the detailed design for RRC configuration is up to RAN2.

SRS-SpatialRelationInfo ::=     SEQUENCE {

servingCellId                       ServCellIndex                          OPTIONAL,   -- Need S

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

ssb-nonserving						SSB-Index

csi-RS-Index                        NZP-CSI-RS-ResourceId,

srs                                 SEQUENCE {

resourceId                          SRS-ResourceId,

uplinkBWP                           BWP-Id

}

}

}

Proposal 1: When SSB is used as reference signal in SRS-SpatialRelationInfo, PUCCH-SpatialRelationInfo, PUCCH-PathlossReferenceRS, PUSCH-PathlossReferenceRS, and pathlossReferenceRS under SRS-ResourceSet, the configuration indicates whether the SSB-Index is associated with the serving cell PCI or the other PCI.

- RRC signalling details are up to RAN2 to decide.

There are a number of UL-related procedures that are function of which symbols are SSBs (as indicated by RRC param ssb-PositionsInBurst) in TDD systems:

- Procedure 1: When SSB overlaps with UL channel/RS, UE does not transmit the UL channels/RS [38.213, Section 11.1].

- Procedure 2: UE does not expect the set of SSB symbols (indicated by ssb-PositionsInBurst) to indicated as uplink symbols either semi-statically or dynamically (by SFI) [38.213, Section 11.1 and Section 11.1.1].

- Procedure 3: SSB symbols are assumed to be invalid symbols in a nominal repetition for PUSCH repetition Type B [38.214, Section 6.1.2.1].

- Procedure 4: For determination of the NPUCCHRepeat slots in the case of PUCCH repetition, i.e., a slot is not counted toward the NPUCCHRepeat slots if the PUCCH resource in that slot overlaps with a SSB [38.213, Section 9.2.6].

In the case of inter-cell multi-TRP operation, a second ssb-PositionsInBurst is configured to the UE associated with non-serving cell SSBs, which is anyway needed for the PDSCH rate matching agreement discussed above. Then, the question is that how the non-serving cell SSBs should be treated with respect to the UL-related Procedures 1-4 above. At high-level there are two options:

- Option 1: Same behaviour as serving cell SSBs are followed also for the non-serving cell SSBs, i.e., UE does not transmit any UL signal/channel in the symbols indicated by the non-serving cell ssb-PositionsInBurst.

- Option 2: UE can transmit UL signal/channel in the symbols indicated by the non-serving cell ssb-PositionsInBurst. This may be irrespective of association of the UL signal/channel associated with the serving cell PCI or non-serving cell PCI, or this behaviour is only followed for the UL signal/channel associated with the serving cell PCI.

It should be noted that in the case of Option 2 above, the fact that non-serving cell TRP cannot transmit and receive at the same time in TDD systems (TRPs are not full-duplex) should be considered. As a result, in the symbols of the non-serving cell SSBs, at least UL signal/channel associated with the non-serving PCI should not be transmitted, and Option 2 should be only applicable to the UL signal/channel associated with the serving cell PCI.

Proposal 2: In the set of symbols indicated to a UE by non-serving cell ssb-PositionsInBurst,

- Option 1: The UE does not transmit any UL signal/channel.

- Option 2: The UE can only transmit UL signal/channel associated with the serving cell PCI.

- Further study the impact on the following Rel. 15/16 procedures based on a selected option from Option 1 or 2 above:

  - Procedure 1: When SSB overlaps with UL channel/RS, UE does not transmit the UL channels/RS [38.213, Section 11.1].

  - Procedure 2: UE does not expect the set of SSB symbols to indicated as uplink symbols either semi-statically or dynamically (by SFI) [38.213, Section 11.1 and Section 11.1.1].

  - Procedure 3: SSB symbols are assumed to be invalid symbols in a nominal repetition for PUSCH repetition Type B [38.214, Section 6.1.2.1].

  - Procedure 4: For determination of the NPUCCHRepeat slots in the case of PUCCH repetition, i.e., a slot is not counted toward the NPUCCHRepeat slots if the PUCCH resource in that slot overlaps with a SSB [38.213, Section 9.2.6].

# Conclusion

In this contribution, we proposed:

Proposal 1: When SSB is used as reference signal in SRS-SpatialRelationInfo, PUCCH-SpatialRelationInfo, PUCCH-PathlossReferenceRS, PUSCH-PathlossReferenceRS, and pathlossReferenceRS under SRS-ResourceSet, the configuration indicates whether the SSB-Index is associated with the serving cell PCI or the other PCI.

Proposal 2: In the set of symbols indicated to a UE by non-serving cell ssb-PositionsInBurst,

- Option 1: The UE does not transmit any UL signal/channel.

- Option 2: The UE can only transmit UL signal/channel associated with the serving cell PCI.

- Further study the impact on the following Rel. 15/16 procedures based on a selected option from Option 1 or 2 above:

  - Procedure 1: When SSB overlaps with UL channel/RS, UE does not transmit the UL channels/RS [38.213, Section 11.1].

  - Procedure 2: UE does not expect the set of SSB symbols to indicated as uplink symbols either semi-statically or dynamically (by SFI) [38.213, Section 11.1 and Section 11.1.1].

  - Procedure 3: SSB symbols are assumed to be invalid symbols in a nominal repetition for PUSCH repetition Type B [38.214, Section 6.1.2.1].

  - Procedure 4: For determination of the NPUCCHRepeat slots in the case of PUCCH repetition, i.e., a slot is not counted toward the NPUCCHRepeat slots if the PUCCH resource in that slot overlaps with a SSB [38.213, Section 9.2.6].

- References

- RP-193133, New WID: Further enhancements on MIMO for NR, Samsung.

- RP-211586, Revised WID: Further enhancements on MIMO for NR, Samsung.