| 3GPP TSG-RAN WG1 Meeting #107-e e-Meeting, November 11th – 19th, 2021 | R1-2112230 |
| --- | --- |

Agenda item:	8.8.1.1

Source: 	Qualcomm Incorporated

Title: 	 Enhancements on PUSCH repetition type A

Document for:	Discussion/Decision

# Introduction

Following the conclusion of RAN1-103e and RAN-90e meetings, the following enhancement to PUSCH was agreed to be specified as part of the coverage enhancements WID in RP-202928:

- Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

  - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

  - The number of repetitions counted on the basis of available UL slots.

Agreements made in RAN1-105-e, RAN1-106e and RAN1-106bis-e meeting are included in the appendix. In this document we present our views on this topic, focused on the open issues that were identified in the previous meetings.

# Open Issues

On DCI formats supporting the repetition factors indicated/configured via TDRA lists

As per the current spec., when DG-PUSCH is scheduled by DCI Format 0_0, no repetitions are permitted. The clause on determining the number of repetitions is as given below:

We prefer to preserve this behavior in R17 and therefore make the following proposal:

Proposal 1: For DG-PUSCH triggered by DCI Format 0_0 follow legacy R15/R16 behavior, i.e., no repetitions are supported.

Now, since Type 1 CG-PUSCH uses the same TDRA table as DG-PUSCH scheduled by DCI Format 0_0, and since this table does not have a field specifying the number of repetitions, we suggest that we change the range of values that RepK parameter in ConfiguredGrantConfig takes to ensure that Type 1 CG-PUSCH is capable of going up to 32 repetitions. We make the following proposal:

Proposal 2: For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_0, the range of values for RepK parameter is increased to support up to 32 repetitions.

Configuration/indication of CovEnh functions

With half-duplex FDD Redcap UEs in mind, we prefer to let the new method of counting available slots be applicable to paired spectrum as well. This motivates the following proposal:

Proposal 3: For a UE operating in half duplex mode in an FDD band, support counting of repetitions based on available slots.

On PUSCH collision handling

We don’t see a strong need to revisit the existing rules on collision handling between PUSCH and other channel/signals. For any special handling related to Release 17 changes in other work items, we suggest it be handled within the scope of that work item. We therefore suggest the following:

Proposal 4: Reuse existing collision handling rules for collision between enhanced Type A PUSCH repetitions and other UL channels/signals.

# Conclusions

Based on the discussions presented in the earlier sections, we have the following proposals:

Proposal 1: For DG-PUSCH triggered by DCI Format 0_0 follow legacy R15/R16 behavior, i.e., no repetitions are supported.

Proposal 2: For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_0, the range of values for RepK parameter is increased to support up to 32 repetitions.

Proposal 3: For a UE operating in half duplex mode in an FDD band, support counting of repetitions based on available slots.

Proposal 4: Reuse existing collision handling rules for collision between enhanced Type A PUSCH repetitions and other UL channels/signals.

# References

- “New WID on NR coverage enhancements”, RP-202928, 3GPP TSG RAN meeting #90e, December 7 - 11, 2020

- Chairman notes, 3GPP TSG RAN1 meeting #105-e, e-Meeting, May 10th – 27th, 2021.

- Chairman notes, 3GPP TSG RAN1 meeting #106-e, e-Meeting, August 16th – 27th, 2021.

- Chairman notes, 3GPP TSG RAN1 meeting #106bis-e, e-Meeting, October 11th – 19th, 2021.

# Appendix A: Appendix B: Agreements from RAN1-105-e meeting

Agreement: - RV cycling is based on available slot for the Type A PUSCH repetition enhancement with repetitions counted based on available slot in Rel-17 Agreement: - Down-selection in RAN1#106-e: - Alt 1: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is 32, irrespective of counting method, - Alt 2: The maximum number of repetitions supported by Rel-17 PUSCH repetition Type A is: 32 for the counting based on physical slots; and 16 (i.e. no change from Rel-16) for the counting based on available slots. Conclusion: - The following agreement in RAN1#104-e is applied to all slots including special slots. Agreements: For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions. - FFS details. Agreement: In addition to {1, 2, 3, 4, 7, 8, 12, 16} and {32}, the following additional value set for repetition factor is supported in Rel-17. - {20, 24, 28} Agreement: - Each available slot identified by the UE is considered as a transmission occasion for PUSCH repetition. - RV is cycled across transmission occasions, irrespective of whether PUSCH transmission in the transmission occasion is further omitted or not. Agreement: - If PUSCH symbol in a slot overlaps with flexible symbol(s) with SSB transmission, the slot is determined as not available during the counting of repetitions. As there is no PUSCH in the slot, no PUSCH omission applies to the slot. Agreement: Select one from the following (further refinement of the alternatives can be further discussed), for the procedure of Rel-17 PUSCH repetition Type A (other alternatives are not precluded) - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - Alt 1-B’ consisting of two steps - Step 1: Determine K repetitions based on available slots, where the available slot is the UL slot and flexible slot indicated by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated. - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: handling of dynamic signaling (e.g. UL CI, DCI for high priority channel), e.g., UE without CI capability - Alt 2-A consisting of a single step - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic signaling (e.g. SFI, UL CI, DCI for high priority channel) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Alt 2-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) and dynamic SFI in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - FFS timeline for the dynamic signalling - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions.

# Appendix B: Agreements from RAN1-106-e meeting

Agreement: - For Rel-17 PUSCH repetition Type A without joint channel estimation, no new inter-slot frequency hopping mechanism is introduced. Agreement Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots. - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s) Agreement For PUSCH repetition Type A for Rel-17 CG-PUSCH, semi-static flexible symbol is considered as available. Agreement For PUSCH repetition Type A for Rel-17 DG-PUSCH, semi-static flexible symbol is considered as available. Note: The applicability for Msg 3 is to be discussed in 8.8.3 Agreement - DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists. Agreement - For DG-PUSCH with counting based on the available slots, count of available slots continues until satisfying the conditions defined for DG-PUSCH repetition Type A in Rel-16. Working Assumption The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32

# Appendix C: Agreements from RAN1-106bis-e meeting

Agreement Working Assumption is confirmed Working Assumption The maximum number of repetitions accounted for available slots supported by Rel-17 PUSCH repetition Type A is 32 Conclusion: For CG-PUSCH repetitions counted on the basis of available slots, all the K transmission occasions including the 1st transmission occasion are determined on the basis of available slots. Agreement For CG-PUSCH repetition Type A with the counting based on available slots, the R16 existing restrictions as defined in Clause 6.1.2.3.1 of TS38.214 at least on the initial transmission of a transport block are applied, assuming the K repetitions of R17 determined based the rule of counting available slots. Observation - Whether or not the counting based on available slots is applicable only to unpaired spectrum is not discussed under AI 8.8.1.1 in RAN1#106bis-e. Discussions on how HD-FDD RedCap UEs support the available slot counting may take place in AI 8.8.1.1 in RAN1#107-e, depending on the progress of RedCap WI discussions. Agreement - For the K repetitions of DG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the slot which is determined by the slot offset K2. - No RAN1 spec impact is expected in terms of the relation with the slot which is determined by the slot offset K2. - Note: The available slot determination is to be specified. - For the K repetitions of CG-PUSCH, Step 1 of the previously agreed two-step procedure (i.e., Alt 1-B) determines the K earliest available slots no earlier than the first slot which is determined by at least ConfiguredGrantConfig. - No RAN1 spec impact is expected in terms of the relation with the first slot which is determined by at least ConfiguredGrantConfig. - Note: The available slot determination is to be specified. Agreement - Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots. - Any other RRC configuration is not considered for the determination of available slots. Agreement - The existing restriction “The UE is not expected to be configured with the time duration for the transmission of K repetitions larger than the time duration derived by the periodicity P” applies to both the counting based on physical slots and the counting based on available slots. - The above “the time duration for the transmission of K repetitions” means the time duration between the start of the 1st slot of the K repetitions and the end of the last slot of the K repetitions for any instance of a CG period.