3GPP TSG-RAN WG1 Meeting #107-e	R1-2112246

e-Meeting, November 11th – November 19th, 2021

Agenda item:	8.16.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on FeMIMO UE features

Document for:	Discussion/Decision

- Introduction

Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e is captured in R1-2110587, where some of the basic FGs are agreed at high-level while many of the details are still under discussion. In this contribution, we discuss our views on various FeMIMO UE features.

- Multi-Beam Operation

| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] |  |  |  |  | [per band] |  |  |  | [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 3. candidate values FFS 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 23-1-1, intra-BM and inter-cell BM should have separate capabilitites, e.g. UE supporting inter-cell BM should additionally support QCL source RS from non-serving SSB as well as L1-RSRP measurement on non-serving SSB.

Proposal 2-1: For FG 23-1-1, the unified TCI for intra-cell BM and inter-cell BM should have separate capabilities, e.g. separate in FG 23-1-1 and FG 23-1-2

For intra-cell BM, we also have the following proposals

Proposal 2-2: For FG 23-1-1, UE supporting intra-cell BM can optionally support the following sub-features, which are NOT basic functions and hence should have their own rows

- Support of separate DL/UL TCI state

  - E.g. corresponding bracket in Component 1 should be moved to a new row, e.g. FG 23-1-1x

- Support of MAC-CE + DCI based beam indication without DL assignment

  - E.g. this can be moved to a new row, and Component 3 should only mention DCI formats 1_1/1_2 with DL assignment as baseline

- Support of a common TCI state ID activated/selected for a set of BWPs/CCs

  - E.g. corresponding part in bracket in Component 1 should be moved to a new row

- Support a TCI state pool configured on a reference BWP/CC shared by a set of BWPs/CCs

  - E.g. corresponding part in bracket in Component 1 should be moved to a new row

Proposal 2-3: For FG 23-1-1, Component 2 should be split into association between TCI state and UL PC setting for PL RS and for other parameters, respectively

- Suggest for Component 2 to only include support of association between TCI state and PL RS with beam alignment, similar to Component 9 in bracket

  - Support of beam misalignment for PL RS and associated TCI state should be UE optional in a new row, e.g. FG 23-1-1x

- Suggest to add a new component for support of TCI-independent UL PC settings except for PL RS

  - Support of TCI-dependent UL PC settings except for PL RS should be UE optional in a new row

Proposal 2-4: Introduce UE capability of supporting configuration of either R17 unified TCI states or R15/16 TCI states and spatial relations at a given time

- This can be in a new row, e.g. FG 23-1-1x

For inter-cell BM, we prefer to have a separate FG, e.g. FG 23-1-Y, which should also include a component for maximum number of MAC-CE activated PCI(s) different from serving cell PCI. This component is for inter-cell BM operation, not for measurement. So the corresponding part should be deleted from Component 3 and 4 in FG 23-1-2.

Proposal 2-5: Supporting inter-cell BM implies supporting SSB from non-serving cell as direct/indirect beam indication RS in unified TCI state

- This should be a separate FG from intra-cell BM, e.g. FG 23-1-Y

- Same FG should include a component for maximum number of MAC-CE activated PCI(s) different from serving cell PCI

| 23. NR_FeMIMO | 23-1-Y | Unified TCI for intra-cell beam management | 1. Support of SSB from non-serving cell as direct/indirect QCL source RS and/or spatial relation RS in unified TCI state 2. Maximum number of MAC-CE activated PCI(s) different from serving cell PCI | FG 23-1-1 |  |  |  | Per band |  |  |  | Note: | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 23-1-2, we have the following proposals, where the 1st proposal is based on the following highlighted FFS

Proposal 2-6: Introduce UE capability on maximum number of overlapped SSB burst sets from different PCIs configured for L1-RSRP measurement

- This can be a new component in FG 23-1-2

- Note: For UE indicating maximum number of zero, the SSB burst sets from different PCIs configured for L1-RSRP measurement should not overlap in time

Agreement

On Rel-17 enhancements for inter-cell beam management and inter-cell mTRP, NMAX (the maximum number of RRC-configured PCIs different from the serving cell for measurement/reporting) is up to UE capability with candidate values of at least 1 and X.

- Note: The upper bound for X as agreed in AI 8.1.2.2

- When NMAX is configured to be X, the UE is RRC-configured for L1-RSRP measurement with up to X PCIs different from the serving cell PCI

- Additional restriction may be added by RAN4

- FFS: UE measurement behaviour when SSBs associated with different PCIs overlap, including whether this is up to UE capability

Proposal 2-7: In FG 23-1-2, “/ MAC-CE activated” should be deleted from Component 3 and 4, since the maximum number of MAC-CE activated PCI(s) different from serving cell PCI is for inter-cell BM operation, not for measurement.

- mTRP PDCCH

The following has been agreed (excluding the yellow text) for PDCCH repetition:

| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component1: details FFS Component2: details FFS Component3: details FFS Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

In Rel-15, PDCCH monitoring capability is defined for different cases including Case 1-1 for basic capability (FG 3-1), Case 1-2 for single occasion monitoring (FG 3-2), and Case 2 for PDCCH monitoring with span gap (FG 3-5b). Furthermore, Rel-16 introduced additional capabilities (FG 11-2 family). The support of Rel-17 mTRP PDCCH repetition should be separately indicated for each of these PDCCH monitoring capabilities. In other words, the basic FG for Rel-17 mTRP PDCCH repetition should only include the basic PDCCH monitoring capability. This is because PDCCH repetition can significantly increase the UE complexity and some of the advanced PDCCH monitoring features that the UE is able to support in the absence of PDCCH repetition need a separate capability when combined with PDCCH repetition.

Furthermore, for the proposed FGs 23-2-1b and 23-2-1c, UE should be able to indicate the supported mode of PDCCH repetition wrt intra-span versus inter-span. In fact, such capability is already agreed:

Agreement

When 3 BDs are counted for two linked candidates

- The third BD is counted in the later span for inter-span PDCCH repetition when r16monitoringcapablityis configured.

- Note: Inter-span repetition is UE optional

Proposal 3-1: The basic FG for Rel-17 mTRP PDCCH repetition (FG 23-2-1) should only include the basic PDCCH monitoring capability, i.e., the highlighted note is required.

- PDCCH repetition for more advanced PDCCH monitoring capabilities need a separate capability for each of the following

  - Add FG 23-2-1a for “PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot”

  - Add FG 23-2-1b for “PDCCH repetition for Case 2 PDCCH monitoring with a span gap”

    - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both}

  - Add FG 23-2-1c for “PDCCH repetition for Rel-16 PDCCH monitoring”

    - Add a component for “Supported mode of PDCCH repetition” with candidate values {intra-span, inter-span, both}

Regarding components 3 and 4 of FG 23-2-1, we think there should not be any dependency on component 2. This is because for whether the UE performs soft-combining or not and the relationship to number of BDs was extensively discussed in RAN1 and no consensus was achived. Hence, component 3 does not need to depend on what UE indicates for component 2. Furthermore, component 4 is already agreed as shown below, and it needs to be included that this max number is per scheduled CC per slot.

Agreement

When one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET

- Interpretation of the detected DCI is based on Rel. 17 PDCCH repetition rules (wrt reference PDCCH candidate).

  - Whether the individual candidate is monitored or not is determined by a UE capability

    - FFS (In UE feature session): The details including reusing the reported number of BDs for this purpose, or relation to reported number of BDs

  - In both cases, the individual candidate is not counted toward the BD limit.

- UE capability for max number of such overlaps is introduced

  - FFS: Value of 0 is included as a candidate value for the UE capability

  - The details to be discussed as part of UE capability discussions

- FFS: When the individual candidate is monitored, the scenario where the other linked candidate is also “overlapping” (same CORESET, DCI size, CCEs, scrambling) with a second individual candidate

Proposal 3-2:

- Coponent 3 of FG 23-2-1 should be “Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET” with candidate values of {monitored, not monitored}.

- Component 4 of FG 23-2-1 should be “Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot”

Based on the above discussions, we propose the following FGs for mTRP PDCCH:

Proposal 3-3: Adopt the following for Rel-17 mTRP PDCCH UE features.

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. Support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET 4. Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  |  |  |  | Per FS |  |  |  | Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. Component2: {2, 3,} Component3: {monitored, not monitored} Component4: {2,4,6,10,20,40} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1a | PDCCH repetition with PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | Support of PDCCH repetition for PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | FG 3-2 FG 23-2-1 |  |  |  | Per Band |  |  | FR1 only |  | Optional |
| 23. NR_FeMIMO | 23-2-1b | PDCCH repetition for Case 2 PDCCH monitoring with a span gap | 1. Support of PDCCH repetition for PDCCH monitoring of any occasions with span gap as defined in FG 3-5b. 2. Supported mode of PDCCH repetition | FG 3-5b FG 23-2-1 |  |  |  | Per FS |  |  |  | This capability is necessary for each SCS. Component2: {intra-span, inter-span, both} | Optional |
| 23. NR_FeMIMO | 23-2-1c | PDCCH repetition for Rel-16 PDCCH monitoring | 1. Support of PDCCH repetition with Rel-16 PDCCH monitoring capability as defined in FG 11-2 family. 2. Supported mode of PDCCH repetition 3. Supported max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per span | FG 11-2 FG 23-2-1 |  |  |  | Per FS |  |  |  | This capability is signalled for SCS 15 kHz and 30 kHz. Component2: {intra-span, inter-span, both} Component3: {1,2,3,5,10,20} | Optional |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition | FG 23-2-1 |  |  |  | Per Band |  |  | FR2 only |  | Optional |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS | FG 23-2-1 |  |  |  | Per Band |  |  |  |  | Optional |

- mTRP PUSCH

The following is the outcome of discussions for mTRP PUSCH capability in the previous meeting. While nothing was agreed, the following is added for further discussions.

| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

The following aspects need to be considered when designing UE capability for mTRP PUSCH repetitions:

- Separate capabilities are required for codebook-based and non-codebook based mTRP PUSCH.

  - For codebook-based mTRP PUSCH, the structure of Rel-15 capability (FG 2-14) including indication on max number of SRS resources need to be taken in to account, and should be extended for the case that two SRS resource sets are configured.

  - For non-codebook based mTRP PUSCH, the structure of Rel-15 capability (FGs 2-15, 2-15a, 2-15b) including indication on max number of SRS resources and various capabilities for associated CSI-RS (for precoding calculation) need to be taken in to account, and should be extended for the case that two SRS resource sets / two associated CSI-RS resources are configured.

- Separate capabilities for mTRP PUSCH repetition are required for repetition Type A and repetition Type B.

- Capabilities are needed for transmitting AP/SP-CSI on two PUSCH repetitions.

- Per TRP default power control pramaters or per TRP OLPC set indication features require UE capability.

From our perspective, the first and most important aspect is to have separate capabilities consistent with Rel-15 for codebook-based and non-codebook based mTRP PUSCH.

Based on the above discussions, we propose the following FGs for mTRP PUSCH:

Proposal 4-1: Adopt the following for Rel-17 mTRP PUSCH UE features.

| 23. NR_FeMIMO | 23-3-1a | Multi-TRP PUSCH repetition for codebook based | 1. Support of codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to codebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets | FG 2-14 |  |  |  | Per FSPC |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,4} Component 4: {2,3,4,5,6,8} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1b | Multi-TRP PUSCH repetition for non-codebook based | 1. Support of non-codebook-based multi-TRP PUSCH repetition for repetition Type A 2. Support of two SRS resource sets with usage set to noncodebook 3. Supported number of SRS resources in one SRS resource set 4. Supported number of SRS resources across two SRS resource sets | FG 2-15 |  |  |  | Per FSPC |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: {1,2,3,4} Component 4: {2,3,4,5,6,7,8} | Optional |
| 23. NR_FeMIMO | 23-3-1b-1 | Two associated CSI-RS resources | Support of up to two CSI-RS resources associated with the two SRS resource sets for non-codebook-based mTRP PUSCH | FG 2-15a FG 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1b-2 | CSI-RS processing framework for SRS with two associated CSI-RS resources | 1. Maximum number of periodic SRS resources associated with first or second CSI-RS per BWP 2. Maximum number of aperiodic SRS resources associated with first or second CSI-RS per BWP 3. Maximum number of semi-persistent SRS resources associated with first or second CSI-RS per BWP 4. UE can process Y SRS resources associated with first or second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. | FG 2-15b FG 23-3-1b |  |  |  | Per Band |  |  |  | Component 1: {1 to 8} Component 2: {1 to 8} Component 3: {1 to 8} Component 4: {1 to 16} | Optional |
| 23. NR_FeMIMO | 23-3-1c | Multi-TRP PUSCH repetition for repetition Type B | Support of multi-TRP PUSCH repetition for repetition Type B | FG 11-5 FG 23-3-1a or 23-3-1b |  |  |  | Per FS |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1d | Two TPC fields in UL DCI | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 | FG 8-8 FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1e | Cyclic beam mapping | Support of cyclic mapping when the number of repetitions is larger than 2 | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1f | Two PHRs per CC | Support of reporting two PHRs per CC (for a CC configured with mTRP PUSCH repetition) | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1g | Multi-TRP PUSCH repetition for configured grant | 1. Support of multi-TRP PUSCH repetition for configured grant 2. Supported type of configured grant for multi-TRP PUSCH repetition | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  | Component 2: {Type1, Type2, both} | Optional |
| 23. NR_FeMIMO | 23-3-1h | AP-CSI on two PUSCH repetitions | Support of transmitting AP-CSI on two PUSCH repetitions | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1i | SP-CSI on two PUSCH repetitions | Support of transmitting SP-CSI on two PUSCH repetitions | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1j | Per TRP OLPC set configurations | Support configurations of two (per TRP) p0-PUSCH-SetList | FG 11-8 FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-1k | Per TRP default power control parameters | Support of two (per TRP) sets of default power control parameters when SRI fields are absent from the DCI | FG 23-3-1a or 23-3-1b |  |  |  | Per Band |  |  |  |  | Optional |

- mTRP PUCCH

The following is the outcome of discussions for mTRP PUSCH capability in the previous meeting. While nothing was agreed, the following is added for further discussions.

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The following aspects need to be considered for UE capability of mTRP PUCCH repetitions:

- Separate capabilities are needed for mTRP PUCCH Scheme 1 versus Scheme 3

  - Supported PUCCH format(s) need to be indicated for each of the Scheme 1 and Scheme 3

  - For the details of the capability for Scheme 3, we need to consider how the capability of sub-slot based PUCCH repetition with sTRP in Rel-17 IIoT AI is indicated.

- Updating two beams / power control parameters for a group of PUCCH resources requires a UE capability

Proposal 5-1: Adopt the following for Rel-17 mTRP PUCCH UE features.

| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of multi-TRP PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of updating two Spatial Relation Info’s / two sets of power control parameters for a PUCCH resource by MAC-CE 3. Supported PUCCH formats for this scheme |  |  |  |  | Per FS |  |  |  | Note: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-2a | Multi-TRP PUCCH repetition with sub-slots | 1. Support of multi-TRP PUCCH repetition scheme 3 (sub-slot based repetition) 2. Supported PUCCH formats for this scheme | FG 23-3-2 |  |  |  | Per FS |  |  |  | Note1: For more than 2 repetitions, sequential beam mapping is supported as part of this FG. Note2: The details of this FG should be decided after FG(s) related to sTRP sub-slot based PUCCH repetition in Rel-17 IIoT AI. Component 3: A bit map of size 5 for PUCCH format 0-4. At least one value of the bitmap should be set to 1. | Optional |
| 23. NR_FeMIMO | 23-3-2b | Two TPC fields in DL DCI | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FG 8-9 FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-2c | Cyclic beam mapping | Support of cyclic mapping when the number of repetitions is larger than 2 | FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-3-2d | Updating two Spatial relation or two sets of power control parameters for PUCCH group | Support of updating two Spatial Relation Info’s / two sets of power control parameters for a group of PUCCH resources in a CC by MAC-CE | FG 16-1b-3 FG 23-3-2 |  |  |  | Per Band |  |  |  |  | Optional |

- Inter-cell mTRP

The following was agreed (excluding the yellow part) in the previous meeting:

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The following agreement needs to be implemented:

Agreement

Support two independent X values (X1, X2) are reported as a UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB.

- X1 (Case 1)= The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2)= The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1

- Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously

- Supported values for X1 and X2 include at least 0,1,2,3 and 7. FFS on other values

- This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per band)

Proposal 6-1: Adopt the following for Rel-17 inter-cell mTRP UE features.

| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI (X1 for Case 1) 3. The maximum number of configured additional PCIs when the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1 (X2 for Case 2) | FG 16-2a |  |  |  | Per Band |  |  |  | Component 2: {0,1,2,3,7} Component 3: {0,1,2,3,7} Note: UE indicating support of this FG indicates a non-zero value for at least one of component 2 or component 3 | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- mTRP BM

| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 23-5-2, we also have the following proposals, where the 2nd proposal is based on the agreement below

Proposal 7-1: Introduce UE capability on maximum number of configured NBI-RS resources per NBI-RS set

- This can be a new component in FG 23-5-2

Proposal 7-2: Introduce UE capability on support of a configured association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per-TRP BFR

- This can be a new component in FG 23-5-2

Agreement

Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR.

FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR

A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

- mTRP HST-SFN

The following has been agreed (excluding the yellow text) for HST-SFN.

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. [Support of scheme B for PDCCH] 2. Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] | [23-6-1] |  |  |  |  |  | [FR1 only] |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs [(HST/URLLC)] | Support of simultaneous activation of two TCI states [for CORESETs with the same CORESET ID] in all BWPs across a set of configured component carriers [by single MAC-CE] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Regarding FG 23-6-1 for the support SFN scheme A (scheme 1), we agree with the split the FG into two FGs. Where FG 23-6-1 describes UE capability for supporting SFN scheme 1 for both PDDCH and PDSCH or only PDSCH and FG 23-6-1a describes the UE capability of dynamic switching between single TRP and SFN PDSCH. Based on RAN1 agreements, Table 1 summerized the supported combination of transmission schemes for both HST and URLLC use-case in green color. SFN scheme A (scheme-1) can be supported for both PDSCH + PDCCH, or SFN PDSCH + single TPR PDCCH. That is why FG 23-6-1 should have two components. The first component for the support of SFN PDSCH plus SFN PDCCH and the second component for the support of SFN PDSCH + single TRP (Rel-15) PDCCH. We would like to emphasize that UEs supporting the same SFN scheme for PDCCH and PDSCH does NOT mean that this UE can SFN PDSCH + single TRP PDCCH as this requires the UE to dynamic switch between single TRP and SFN schemes between PDCCH and PDSCH.

Agreement Support the following combination of the transmission schemes - Single-TRP PDCCH + Rel-17 Scheme 1 PDSCH - Single-TRP PDCCH + Rel-17 TRP-based pre-compensation PDSCH - FFS: Other combinations of the transmission scheme Note: The PDSCH corresponds to the PDSCH scheduled by DCI formats 1_1 and 1_2. Agreement - UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.

Table 1: PDSCH and PDCCH transmission mode

|  |  | PDSCH |  |  |  |
| --- | --- | --- | --- | --- | --- |
| PDCCH |  | Rel-15 | SFN scheme A | SFN scheme B |
|  | Rel-15 | N/A | Supported | Supported |
|  | SFN scheme A | Supported (URLLC) | Supported | No supported |  |
|  | SFN scheme B | Not support | Not supported | Supported |

Proposal 8-1: Support the split of FG 23-6-1 into main FG for the support of scheme A for PDDCH/PDSCH and FG 23-6-1A for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme A.

Proposal 8-2: FG 23-6-1 should have two componeonets:

- Support of Scheme A for both PDSCH and PDCCH

- Support of Scheme A for PDSCH only, where PDCCH is single TRP.

In the last RAN1 meeting #106bis-e, it was agreed to support the combination for Rel-17 SNF PDCCH scheme and single-TPR PDSCH as optional UE feature for the URCCL use-case. This is captured in blue color in Table -1. A new UE FG should be added to reflect the RAN1 agreement.

Proposal 8-3: Add new FG 23-6-1b for the support of SFN PDCCH + nonSFN PDSCH with one component of support of scheme-A for PDCCN and single TRP PDSCH.

Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only.

A similar discussion is applicable to SFN scheme B, where based on RAN1 agreements SFN scheme B is applicable on to HST deployment. FG 23-6-2 can be split into main FGs. FG 23-6-2 for the support of SFN scheme B for both PDCCH and PDSCH or PDSCH only where PDCCH is single TRP and FG 23-6-2a for the dynamic switching between SFN scheme-B and single TPR.

Proposal 8-4: Support the split of FG 23-6-2 into main FG for the support of scheme B for PDDCH/PDSCH and FG 23-6-2a for the support of dynamic switching between single TRP PDSCH and SFN PDSCH scheme B.

Proposal 8-5: FG 23-6-2 should have two componeonets:

- Support of Scheme B for both PDSCH and PDCCH

- Support of Scheme B for PDSCH only, where PDCCH is single TRP.

Regarding FG 23-6-3, we agree with the updated description as it makes clearer. Regarding FG 23-6-4, it could be split into multiple FGs, the main one on the default beam for PDSCH and CSI-RS, FG 23-6-4A for UE capality of two QCL TypeD properories for multiple overlapping CORESETs when a CORESET is activated with two TCI as per RAN1-agreement below and FG 23-6-4b for default spatial relation and pathloss RS.

Agreement When a CORESET is activated with two TCI states which overlaps with another CORESET, support extension of Rel-15 prioritization rule for PDCCH monitoring of PDCCH candidates in overlapping monitoring occasions with different QCL-TypeD - FFS: Prioritization rule considers CORESETs indicated with 1 and/or 2 TCI states - Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs - UE capability is introduced - FFS other details - FFS: Strive to have same / similar solution as discussed under AI 8.1.2.1

Proposal 8-6: Split FG 23-6-4 into main FG for the support of default beam for SFN PDSCH and CSI-RS, and

- FG 23-6-4A: Two QCL TypeD properties for overlapping CORESETs

- FG 23-6-4B: Default spatial relation and pathloss RS

Regarding componenet 2 of supporting PDSCH reception when tci filed not present, we believe that this component should be either removed as it has been agreed only for DCI format 1_0 which already doesn’t have tci-field or explcilty clarified in the description that DCI refers to DCI format 1_0.

Proposal 8-7:  Update the description of component 2 of FG 23-6-4 such that that DCI refer for DCI format 1_0.

Proposal 8-8: Adopt the following for Rel-17 HST-SFN UE features.

| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. Support of scheme A for PDSCH and single TRP PDCCH 2. Support of scheme A for PDSCH and Scheme A PDCCH |  |  |  |  | Per band |  |  | For each component: {Support, Not support} | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 | 23-6-1 |  |  |  | Per band |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A for PDCCH for URLLC | Support of scheme A for PDCCH and single TRP PDSCH |  |  |  |  | Per band |  |  | For each component: {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | 1. Support of scheme B for PDSCH and single TRP PDCCH 2. Support of scheme B for PDSCH and scheme B for PDCCH. |  |  |  |  | Per band |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 | 23-6-2 |  |  |  | Per band |  |  | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs | Support of simultaneous activation of two TCI states for CORESETs with the same CORESET ID in all BWPs across a set of configured component carriers by single MAC-CE |  |  |  |  | Per UE |  |  | For each component {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4 | Default DL beam setup | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI for DCI format 1_0. 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold |  |  |  |  | Per band |  | FR2 only | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4a | Two QCL TypeD properties for SFN PDCCH | Supports identifying two QCL-TypeD properties for multiple overlapping CORESETs when a CORESET is activated with two TCI |  |  |  |  | Per Band |  | FR2 only | {Support, Not support} | Optional |
| 23. NR_FeMIMO | 23-6-4b | Default spatial relation and pathloss RS | 1. Support of single-TRP PUCCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 2. Support of single-TRP PUSCH transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured 3. Support of single-TRP SRS resource transmission using default beam and PL-RS when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  | Per Band |  | FR2 only | For each component: {Support, Not support} | Optional |

- SRS enhancement

The following has been agreed (excluding the yellow text) for SRS enhancement.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP [and 1 periodic] SRS sets for antenna switching | Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching |  |  |  |  |  |  |  |  | [Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial [(RB level)] frequency sounding for SRS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | [23-8-6] |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

In the last RAN1 meeting #106bis-e, it was agreed to extend the number of aperiodic SRS sets for legacy xTyR configuration as separate UE feature. A separate UE feature 23-8-9 should be added to reflect the RAN1 agreement. In addition, we agree with the yellow highlighted text that further clarify the description of the UE feature.

Agreement For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R. - The above extension is UE optional.

Proposal 9-1: Adopt the following for Rel-17 HST-SFN UE features

- Add FG 2-52 (basic SRS) as prerequisite for FG 23-8-1 and FG 23-8-2

- Add FG 2-55 (SRS Tx switch) as prerequisite for FG 23-8-3 and further discuss extension of Rel-16 of combo SRS Tx-switch.

- Add FG 2-53 (SRS resources) as prerequisite for FG 23-8-4.

- Add FG10-11 (SRS transmission at any symbol) as prerequisite for FG 23-8-5.

- Add FG 23-8-6 (Partial frequency sounding of SRS) as prerequisite for FG 23-8-7.

| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2 | FG 2-52 |  |  |  | Per FS |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | FG 2-52 |  |  |  | Per FS |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report whether the uplink TX switching impact to downlink receiving in a band] 3. Report whether the UL Tx is switched together with UL Tx in another band | FG 2-55 |  |  |  | Per BC |  |  |  |  | {1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | FG 2-53 |  |  |  | Per FS |  |  |  |  | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | FG 10-11 |  |  |  | Per FS |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial RB level frequency sounding for SRS |  |  |  |  | Per Band |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial RB level frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent SRS | FG 23-8-6 |  |  |  | Per Band |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  |  |  |  | Per FS |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | extension of SRS sets for antenna switching | Support the extension of number of sets (N) for aperiodic SRS antenna switching to N = 4 for 1T4R, N = 2 for 1T2R/2T4R. | FG 2-55 |  |  |  | Per BC |  |  |  |  | {Support, Not support} | Optional with capability signalling |

- mTRP CSI

The following was agreed (excluding the yellow part) in the previous meeting:

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Maximum number of NZP CSI-RS resources per CMR group: Ks,max=[2,4] - FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | [Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis] [FR2 only] |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

The following aspects need to be considered when designing UE capability for mTRP CSI:

- The basic FG need to consider the overall complexity for CSI processing across all CCs and all CSI report settings even when some of them are not configured with NCJT CSI

  - The FG needs to be consistent with the Rel-15 and Rel-16 CSI capability framework (e.g., FGs 2-35 and 2-36)

  - Due to complexity for UE to calculate two PMIs jointly for NCJT, a list of vectors for NCJT capabilities and sTRP capabilities are needed.

  - It is important that this list is included in the main (basic) FG for mTRP CSI since otherwise a default assumption would be needed, which may not be possible to have since these capabilities are across all CSI report settings and across all CCs (some of which may not be even configured with NCJT CSI).

  - Note that proposed components 3 and 4 are extensions of Rel-15 components 1 and 2 of FG 2-36, and proposed components 5 and 6 are extensions of Rel-15 componets 8 and 9 of FG 2-35.

  - Proposed component 3 should be reported per band and separately per band combination. This is same as legacy behavior for reporting triplets for each codebook per band and separately per band combination.

  - Proposed component 6 should be reported per band combination similar to Rel-15 capability “simultaneousCSI-ReportsAllCC”.

- For NCJT CSI reporting, UE should be able to indicate whether option 1, option 2, or both are supported.

  - For Option 1, UE should be able to indicate which values of X (X=0,1,2) are supported.

  - Default capability for Option 1 is X=0.

Proposal 10-1: Adopt the following for Rel-17 mTRP CSI UE features.

| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | 1. Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 2. Supported option(s) for CSI reporting 3. A list of supported combinations, each combination is {max # of ports per resource for NCJT, max # of resources for NCJT, max # of total ports for NCJT, max # of ports per resource for sTRP, max # of resources for sTRP, max # of total ports for sTRP} simultaneously across all CCs 4. Supported codebook modes for NCJT CSI 5. A list of (Y1,Y2): UE can process Y1 NCJT CSI report(s) and Y2 sTRP CSI report(s) simultaneously in a CC 6. A list of (X1,X2): UE can process X1 NCJT CSI report(s) and X2 sTRP CSI report(s) simultaneously across all CCs |  |  |  |  | Per Band Per Band Combination |  |  |  | Component 2: {Option 1, Option 2, both} If UE supports ‘Option 1’ or ‘both’, X=0 CSIs associated with single-TRP measurement hypotheses is assumed Compenent 3: The list can have maximum of 16 combinations. For each combination, candidate values are - max # of ports per resource for NCJT: {2, 4, 8, 12, 16, 24, 32} - max # of resources for NCJT: {2 to 64} - max # of total ports for NCJT: {4 to 256} - Max # of Tx ports in one resource for sTRP: {2, 4, 8, 12, 16, 24, 32} - Max # resources for sTRP: {1 to 64} - Max # total ports for sTRP: {2 to 256} Note: Component 3 is also reported per band combination (in addition to per band). Compenent 4:{ mode 1, both mode 1 and mode 2} Compoentn 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8} Compoentn 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32} Note: When no NCJT CSI is configured/active, sTRP CSI capabilities are based on Rel-15/16 capabilities. | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-2 | Supported value of X for Option 1 | Supported values of X (reported CSIs associated with single-TRP measurement hypotheses) for Option 1 | FG 23-7-1 |  |  |  | Per Band |  |  |  | Note: UE supporting this feature indicates ‘Option 1’ or ‘both’ in component 4 of FG 23-7-1. Candidate values: {1, 2, 1and2} | Optional |
| 23. NR_FeMIMO | 23-7-3 | More than two resources in a resource set for Multi-TRP CSI | Supported maximum # of CSI-RS resources in a resource set for Multi-TRP CSI (value Ks,max ) | FG 23-7-1 |  |  |  | Per Band |  |  |  | Candidate values: {3…8} | Optional |
| 23. NR_FeMIMO | 23-7-4 | Support two pairs of CMRs | Support of two pairs of CMRs from the two CMR groups (Nmax=2) for a given CSI report setting | FG 23-7-1 |  |  |  | Per Band |  |  |  |  | Optional |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis | FG 23-7-1 |  |  |  | Per Band |  |  |  |  | Optional |

- FDD CSI

Following FGs are agreed as baseline for further discussion in RAN1 #107-e.

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - [{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to] support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support parameter combinations with Mv=1 - FFS others | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-1a | Codebook concurrency of Type I codebook and Rel-17 FeType II codebook | - Supported mixed codebook types | [23-9-1] |  |  |  |  |  |  |  | [Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. Support rank 1,2] 2. Support parameter combinations with Mv=2 [3. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2] | [23-9-1] |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | [23-9-1, 23-9-2] |  |  |  |  |  |  |  |  | Optional with capability signalling |

In our view, following aspects need to be considered for FG 23-9 class.

- In last meeting, it was agreed in MIMO session that R=2 PMIs per subbands is supported with additional capability. Based this, we think following change should be considered

  - Adding R=1 as component 4 in FG 23-9-1 as a basic feature

  - Adding a new row for support of R=2, e.g., FG 23-9-4. In this row, UE report additional CSI-RS triplet {max # of Tx ports in one resource, max # resources, total # Tx ports} (same to the capability signaling for Rel-16 Type II port-selection codebook, e.g., FG 16-3b-1).

    - This signaling provide more flexibity for UE to support R=2. Otherwise, UE may have to report same CSI-RS triplet for both R=1 and R=2, and this may lead to capability underreporting for R=1 because the complexity of R=2 is much higher than R=1.

- Regarding support of codebook concurrency of Type I and FeType II codebook, i.e., FG23-9-1a, it is preferred to reuse the framework in FG 16-8. Specifically,

  - It is better to be a separate FG as it is combination of FG23-9 with other FGs.

  - Component 1: supported mixed codebook types, i.e., {codebook1, codebook2}, wherein codebook1={Type I SP, Type I MP}, codebook2  = FeType II

    - The useful scenario of concurrency of three codebooks is unclear.

  - Component 2: supported CSI-RS triplets in terms of {max # of Tx ports in one resource, max # resources, total # Tx ports}

  - Adding all notes in FG 16-8 to clarify that

    - The codebook concurrencies other than the reported combinations is not supported

    - gNB needs to honer this FG together with per-codebook capability

- Regarding support of M=2 for FeType-II, i.e., FG 23-9-2

  - Component 1 can be removed, as supporting rank is indicated in FG 23-9-1 and FG 23-9-3.

  - Prefer component 3, i.e., reporting supportive of M=2 with CSI-RS triplets in terms of {max # of Tx ports in one resource, max # resources, total # Tx ports}.

Proposal 11-1: adopt following for Rel-17 FDD CSI UE feature

| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - [{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to] support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support parameter combinations with Mv=1 - Support number of PMI subbands with R=1 - FFS others | FFS 2-35 |  |  |  | Per band Per BC |  |  |  | Component-1 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-9-1a 23-10 | Codebook concurrency of Type I codebook and Rel-17 FeType II codebook | 1. Report a list of codebook combinations as {codebook 1, codebook 2} 2. For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports} | 2-36, 23-9-1 |  |  |  | Per band Per BC |  |  |  | Component 1: Codebook 1 = {Type I SP, Type I MP} Codebook 2 = FeType II PS Note 1：if a UE reports one or more codebook combinations in 23-10, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-10 and per-codebook capability 2-36/40/41/43, 16-3a/b and 16-3a-1/16-3b-1, and 23-9-1 Component-2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. Support rank 1,2] 2. Support parameter combinations with Mv=2 3. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2 | 23-9-1 |  |  |  | Per band |  |  |  | Candidate values for component 3: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | 23-9-1 |  |  |  | Per band |  |  |  | {support, not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of number of PMI subbands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 | 23-9-1 |  |  |  | Per band Per BC |  |  |  | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |

- References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.).