3GPP TSG RAN WG1 #107-e						                R1-2112250

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.15

Source: 	Qualcomm Incorporated

Title: 		        Discussion on Positioning Enhancements UE Features

Document for:		Discussion and Decision

# Introduction

This contribution includes our comments and suggestions on the Positioning Enhancements features appeared in R1-2110587, which can also be found in the Appendix).

# General Observations

We make the following general comments with regards to the first draft of the UE feature list on NR Positioning Enhancements:

Observation 1: The feature group for the support of a feature should be separated from the feature group for values associated to the feature (as it was done in NR Rel-16 UE feature discussion).

Observation 2: UE capabilities related to RRC Inactive Positioning are missing.

Observation 3: Any additional information in the assistance data should be associated with a UE capability so that the LMF would know that the UE can accept the new assistance data.

Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.

# Detailed Comments

For each suggested Feature group found in the R1-2109915, we provide a first collection of comments and suggested changes as shown below:

## FG 27-1-1

The support of the feature should be separated from the number of TEGs the UE have. The TEG is a feature that is considered across all the bands (per UE), whereas whether the UE supports the reporting of this feature is a per-band capability.

Proposal 2: Support two separate rows for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT”

- Per band reporting

Proposal 3: If a UE supports the RxTEG capability with a value > 1, if the UE does not include an RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement.

Proposal 4: If a UE supports the RxTEG capability with a value=1, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.

## FG 27-1-2

The support of the feature should be separated from the number of TEGs the UE have. The TEG is a feature that is considered across all the bands (per UE), whereas whether the UE supports the reporting of this feature should be a per-band capability.

Also, the reporting of this feature for UL-TDOA and the M-RTT features need to be separated because there are different reporting mechanisms. For UL-TDOA, the UE reports this information through RRC to the serving gNB, and for Multi-RTT, it reports this information through LPP to the LMF.

Proposal 5: Split the FG 27-1-2 into 2 rows, one for Multi-RTT and the other for UL-TDOA.

Proposal 6: With regards to the FG 27-1-2, support two separate FGs for “Support of UE-TxTEG reporting for UL-TDOA”, and “Support of UE-TxTEG reporting for M-RTT”

- Per band reporting

## FG 27-1-3

The support to UE RxTx TEG reporting should be a per-band-combo capability reporting since the PRS and the SRS may be on a different band.

Proposal 7: With regards to the FG 27-1-3, support a separate FG for “Support of UE-RxTxTEG reporting for M-RTT”

- Per band combination reporting

## FG 27-1-4

Not all UEs will be able to measure the same PRS resource simultaneously with multiple TEGs. For example, in FR2, measuring simultaneously with both panels should be a separate feature than measuring in a TDM fashion a PRS resource with the 2 panels.

Proposal 8: Introduce a separate row for Multi-RTT and DL-TDOA for the FG 27-1-4

- Per band reporting

Proposal 9: Introduce separate rows with the maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously for Multi-RTT and DL-TDOA

- Per band reporting

## FG 27-2-1

This row should be related only to DL-AoD, where the earliest path-RSRP for this method is a main new measurement that is added in NR Rel-17. Note that, even though path-RSRP can be related to the DL-TDOA, Multi-RTT, for these methods, such features are considered within the multipath positioning features. Different requirements may exist for path-RSRP associated with DL-AoD compared to the path-RSRP for the multipath positioning (which, as agreed in RAN4, there will not be any requirements).

Proposal 10: With regards to the FG 27-2-1, should only be applicable to UE-assisted DL-AoD. Path-RSRP reporting for DL-TDOA/Multi-RTT should be a separate FG.

Proposal 11: Introduce an additional row (FG 27-2-1b) for the maximum number of first path RSRP measurements the UE can report.

## FG 27-3-2 and 27-3-3

With regards to MG-less PRS processing, if the UE reports a Type-1A MG-less PRS capability it could be able to report the highest PRS processing capability for a given latency, or vice versa, the shortest latency for a given number of PRS resources per slot, since the UE would have all the processing/memory resources of the modem to finish the processing. On the other hand, for Type-1B or Type-2, the UE would have to share the modem processing/memory resources with other functionalities, which will increase the latency or decrease the PRS processing capabilities. Depending on the use-case, it may be more appropriate for the UE to be allowed to report that it is capable of multiple of such PRS processing, and let the LMF recommend to the serving gNB which one is needed on different scenarios.

Proposal 12: A UE should be able to report multiple of the Type-1A, Type-1B, Type-2 MG-less PRS processing capabilities, each one associated with a different PRS processing capability, to the LMF.

There haven’t been any discussion yet on the maximum number of PFLs that a UE would be processing within a single PRS processing window. For example,

- a UE may receive assistance data that have 2 PFLs with the same SCS on the same BWP and could be encompassed by the same PRS processing window.

- a UE may receive assistance data that have 2 PFLs on 2 active BWPs on different CCs. A Type-1A UE may get a single processing window that could encompass both those PFLs, or a Type-1B/2 UE may get separate PRS processing window on different CCs, each one enabling the measurement of a different PFL.

Without complicating the feature, and following the NR Rel-16 single-PFL processing window, we make the following proposal:

Proposal 13: Inside each single instance of a PRS processing window, a single PFL can be measured. This is applicable to all Types of MG-less PRS processing.

Proposal 14: Simultaneously processing of multiple PRS processing windows on different CCs/Bands is not supported for the MG-less PRS processing feature: The UE is not expected to be activated with multiple PRS processing windows that overlap.

# UE capability for NR positioning in RRC_INACTIVE state

Transmission of SRS in RRC inactive is a new feature and should be clearly be associated with a UE capability. It has very specific UE impacts, for example, for RTT positioning, a UE would need to keep the group delay calibration between the PRS and the SRS. Such feature is only specified in NR Rel-16 for PRS and SRS in RRC Connected state. Keeping the same (or even different) group delay calibration if the PRS and/or SRS are in RRC Inactive would be a new UE feature that needs additional implementation efforts. Also, in RAN4, new requirements may need to be defined for measurement accuracy, measurement period and reporting. For these requirements to be really testable, there should be the corresponding capability bits.

Proposal 15: Introduce a per-band UE capability for UEs transmitting SRS in RRC inactive state.

With regards to DL PRS processing, similar to the SRS transmission, a UE may be capable of different number of PRS resources per slot, different maximum PRS resource configurations, different group delay calibration for RSTD measurement (related to RAN4 margins for RSTD measurements).

Proposal 16: A per-band DL positioning capabilities should be defined in inactive state, e.g. including at least

- DL PRS processing capability in inactive state

- UE Rx-Tx measurement reporting

- DL RSTD measurement reporting

- RSRP measurement reporting

- Path-RSRP measurement reporting

If there is no UE capability for PRS processing or SRS transmission in RRC inactive, there will be NO way to test whether a feature is supported or not. A gNB doesn’t know whether the UE will be able to do PRS and/or SRS transmission; it will be a non-existent, non-deployable, and non-testable feature.

Observation 3: No UE capability for PRS reception or SRS transmission in RRC inactive, will render the feature practically non-existent, non-deployable and non-testable.

# Multipath Positioning UE capabilitites

There have not been introduced any UE capabilities for the multipath positioning features for the timing domain methods. We make the following proposals:

Proposal 17: Introduce the following UE capabilities:

- Support of Maximum number N>2 additional path reporting

- Support of path-RSRP reporting for additional Paths for Multi-RTT

- Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA

- Support of path-RSRP reporting for first Path for Multi-RTT

- Support of path-RSRP reporting for first Path for UE-assisted DL-TDOA

- Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD

# Multiple measurement instances in a single measurement report

In the previous RAN1 meeting, the following agreement was reached with regards to Batch Measurement Reporting:

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

Even though this feature hasn’t been completed yet, we think it is important to put a placeholder in the UE features on the maximum number of measurement instances which can be included in a single measurement.

Proposal 18: Introduce a per-UE capability on the maximum number of measurement instances which can be included in a single measurement report with the values {2,4,5,8,10,16,20,32}.

# 5 Conclusions

We make the following observations and proposals:

Observation 1: The feature group for the support of a feature should be separated from the feature group for values associated to the feature (as it was done in NR Rel-16 UE feature discussion).

Observation 2: UE capabilities related to RRC Inactive Positioning are missing.

Observation 3: Any additional information in the assistance data should be associated with a UE capability so that the LMF would know that the UE can accept the new assistance data.

Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band.

Proposal 2: Support two separate rows for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT”

- Per band reporting

Proposal 3: If a UE supports the RxTEG capability with a value > 1, if the UE does not include an RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement.

Proposal 4: If a UE supports the RxTEG capability with a value=1, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.

Proposal 5: Split the FG 27-1-2 into 2 rows, one for Multi-RTT and the other for UL-TDOA.

Proposal 6: With regards to the FG 27-1-2, support two separate FGs for “Support of UE-TxTEG reporting for UL-TDOA”, and “Support of UE-TxTEG reporting for M-RTT”

- Per band reporting

Proposal 7: With regards to the FG 27-1-3, support a separate FG for “Support of UE-RxTxTEG reporting for M-RTT”

- Per band combination reporting

Proposal 8: Introduce a separate row for Multi-RTT and DL-TDOA for the FG 27-1-4

- Per band reporting

Proposal 9: Introduce separate rows with the maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously for Multi-RTT and DL-TDOA

- Per band reporting

Proposal 10: With regards to the FG 27-2-1, should only be applicable to UE-assisted DL-AoD. Path-RSRP reporting for DL-TDOA/Multi-RTT should be a separate FG.

Proposal 11: Introduce an additional row (FG 27-2-1b) for the maximum number of first path RSRP measurements the UE can report.

Proposal 12: A UE should be able to report multiple of the Type-1A, Type-1B, Type-2 MG-less PRS processing capabilities, each one associated with a different PRS processing capability, to the LMF.

Proposal 13: Inside each single instance of a PRS processing window, a single PFL can be measured. This is applicable to all Types of MG-less PRS processing.

Proposal 14: Simultaneously processing of multiple PRS processing windows on different CCs/Bands is not supported for the MG-less PRS processing feature: The UE is not expected to be activated with multiple PRS processing windows that overlap.

Proposal 15: Introduce a per-band UE capability for UEs transmitting SRS in RRC inactive state.

Proposal 16: A per-band DL positioning capability should be defined in inactive state, e.g. including at least

- DL PRS processing capability in inactive state

- UE Rx-Tx measurement reporting

- DL RSTD measurement reporting

- RSRP measurement reporting

- Path-RSRP measurement reporting

Observation 3: No UE capability for PRS reception or SRS transmission in RRC inactive, will render the feature practically non-existent, non-deployable and non-testable.

Proposal 17: Introduce the following UE capabilities:

- Support of Maximum number N>2 additional path reporting

- Support of path-RSRP reporting for additional Paths for Multi-RTT

- Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA

- Support of path-RSRP reporting for first Path for Multi-RTT

- Support of path-RSRP reporting for first Path for UE-assisted DL-TDOA

- Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD

Proposal 18: Introduce a per-UE capability on the maximum number of measurement instances which can be included in a single measurement report with the values {2,4,5,8,10,16,20,32}.

# NR_pos_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Maximum number of UE-RxTEGs for UE-assisted DL TDOA positioning | The maximum number of UE-RxTEG, which is supported and can be reported by UE for UE-assisted DL TDOA positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. | 13-3, 27-1-1b | No |  | UE-RxTEG reporting is not supported | Per UE | n/a | n/a | n/a | The candidate values are {1,2,4,6,8,12,16,24,32} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-1b | Support of UE-RxTEG reporting for UE-assisted DL-TDOA | Support of UE-RxTEG reporting for UE-assisted DL-TDOA | 13-3a | No |  | Mitigation of UE Rx timing delays is not supported in this band | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-1c | Maximum number of UE-RxTEGs for Multi-RTT positioning | The maximum number of UE-RxTEG, which is supported and reported by UE Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, if if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. | 13-4 | No |  | UE-RxTEG reporting is not supported | Per UE | n/a | n/a | n/a | The candidate values are {1,2,4,6,8,12,16,24,32} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-1d | Support of UE-RxTEG reporting for UE-assisted DL-TDOA | Support of UE-RxTEG reporting for Multi-RTT positioning | 13-4a, 27-1-1c | No |  | Mitigation of UE Rx timing delays is not supported in this band | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Maximum number of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA | 13-8, 27-1-2b | Yes |  | UE-TxTEG reporting is not supported | Per UE | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8, 16, 32} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2b | Support of UE-TxTEG reporting for UL-TDOA | Support of UE-TxTEG reporting for UL-TDOA positioning | 13-8 | Yes |  | Mitigation of UE Tx timing delays is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2c | Maximum number of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource Note 2: If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. | 13-4, 13-8 | No |  | Mitigation of UE Tx timing delays is not supported | Per UE | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8, 16, 32} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2d | Support of UE-TxTEG reporting for Multi-RTT | Support of UE-TxTEG reporting for Multi-RTT positioning | 13-8 | No |  | UE-TxTEG reporting is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. | 13-4, 13-8 | No |  | Mitigation of UE RxTx timing delays is not supported | Per UE | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, 128} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3b | Support of UE-RxTxTEG reporting for Multi-RTT | Support of UE-RxTxTEG reporting for Multi-RTT positioning. | 13-8, 27-1-3 | No |  | UE-RxTxTEG reporting is not supported | Per band combination | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource in UE-assisted DL-TDOA | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP in UE-assisted DL-TDOA. | 27-1-1b | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | Per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4b | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource in Muti-RTT | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP in Multi-RTT. | 27-1-1d | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | Per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4c | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously in UE-assisted DL-TDOA | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously in UE-assisted DL-TDOA | 27-1-4 | No |  | Measuring the same instance of a DL PRS resource simultaneously with multiple RxTEGs is not supported | Per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4d | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously in Multi-RTT | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously in Multi-RTT | 27-1-4 | No |  | Measuring the same instance of a DL PRS resource simultaneously with multiple RxTEGs is not supported | Per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | DL PRS RSRP of the first path for UE-assisted DL-AoD | Support of measuring and reporting the PRS RSRP of the first path for UE-assisted DL-AoD positioning method | 13-5 | No |  | PRS RSRP of the first path is not supported | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1b | Maximum number of the first path PRS RSRP measurements on different PRS resources for UE-assisted DL-AoD positioning | Maximum number K of the first path PRS RSRP measurements on different PRS resources from the same TRP for UE-assisted DL-AoD positioning | 27-2-1 | No |  |  | Per Band | n/a | n/a | n/a | The candidate values are {2,4,8,12, 16,24} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | per band | n/a | FFS | n/a | The candidate values are {12, 16, 24} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements of DL PRS | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1 If the UE does not provide the capability, the UE support M=4 only. | 13-1 | No |  |  | per band | n/a | n/a | n/a | The candidate values are 1 Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing priority window- processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP Note: Inside each single instance of a PRS processing window, a single PFL can be measured. | 13-1 | FFS |  |  | per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Candidate values: A UE may declare any one or multiple from the set {Type 1A, Type 1B, Type 2}. Total 6 options - 1 bit for Type 1A - 1 bit for Type 1B - 1 bit for Type 2 | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability if Type-1A is declared | 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {4, 6, 8, 16, 20, 30, 40} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 5, 6, 8, 10, 12, 16, 20} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3b | DL PRS Processing Capability outside MG - buffering capability if Type-1B is declared | 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3c | DL PRS Processing Capability outside MG - buffering capability if Type-2 is declared | 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF |  | No |  |  | per UE | n/a | n/a | n/a | The candidate value are [0,1] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | UE-initiated on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS request signalling | 13-1 | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2a | Additional path reporting N>2 for UE-assited DL-TDOA and/or Multi-RTT | Maximum number N>2 additional path reporting |  | No |  |  | Per Band | n/a | n/a | n/a | N = {4,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2e | Support of path-RSRP reporting for first Paths for Multi-RTT | Support of path-RSRP reporting for first Path for Multi-RTT |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2e | Support of path-RSRP reporting for additional Paths for Multi-RTT | Support of path-RSRP reporting for additional Paths for Multi-RTT |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2e | Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA | Support of path-RSRP reporting for first Path for UE-assisted DL-TDOA |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-v2e | Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA | Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c1 | Support of Transmission of periodic SRS in RRC Inactive State | Support of Transmission of periodic SRS in RRC Inactive State |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c2 | Support of Transmission of Semi-persistent SRS in RRC Inactive State | Support of Transmission of Semi-persistent SRS in RRC Inactive State |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c3 | Support of PRS processing in RRC inactive state | Support of DL PRS processing in RRC inactive state |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c4b | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c4c | Support of DL RSTD measurement in RRC Inactive state | Support of DL RSTD measurement in RRC Inactive state |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c4c | Support of UE Rx-Tx measurement in RRC Inactive state | Support of UE Rx-Tx measurement in RRC Inactive state |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c4c | Support of UE Rx-Tx measurement in RRC Inactive state | Support of RSRP measurement in RRC Inactive state |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-c4c | Support of UE Rx-Tx measurement in RRC Inactive state | Support of path-RSRP measurement in RRC Inactive state |  | No |  |  | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-d4 | Support beam Information in the Assistance Data for UE-based DL-AoD | Support beam Information in the Assistance Data for UE-based DL-AoD |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-d5 | Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD | Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD |  | No |  |  | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-d5 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  | No |  |  | Per UE | n/a | n/a | n/a | Candidate values {2,4,5,8,10,16,20,32}. Need for location server to know if the feature is supported. | Optional with capability signaling |

# Appendix: NR_pos_enh (R1-2110587)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-1 | Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | [13-3] | No |  | Mitigation of UE Rx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2 | Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8] | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8, 16, 32}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4, 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS | n/a | [The candidate values are {12, 16, 32, 64}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3}] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |