3GPP TSG RAN WG1 #107-e			R1-2112254

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.9

Source: 	Qualcomm Incorporated

Title: 	UE features for NB-IoT and LTE-MTC enhancements

Document for:	Discussion and Decision

# Background

In RAN1#106b-e, the following agreements were reached regarding UE features for NB-IoT and eMTC:

Agreement

FG 1-1 is kept as “16-QAM for unicast NPDSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-1 | 16-QAM for unicast NPDSCH | 1. Reception of unicast NPDSCH modulated with 16-QAM 2. CQI report to support 16-QAM modulation 3. Downlink power allocation for 16-QAM | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPDSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

FG 1-2 is kept as “16-QAM for unicast NPUSCH” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-2 | 16-QAM for unicast NPUSCH | 1. Transmission of unicast NPUSCH modulated with 16-QAM 2. New term in the UE’s transmit power control equation. | Category NB-2 | Yes | N/A | The network cannot schedule a unicast NPUSCH modulated with 16-QAM for the UE | Per UE | [Yes] | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

FG 1-4 is kept as “A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only” as follows.

| 1. NB_IOTenh4_LTE_eMTC6 | 1-4 | A maximum DL TBS of 1736 bits for HD-FDD Cat. M1 Ues in CE mode A only | 1. Support of 1736 bits max DL TBS for unicast in HD-FDD in CE mode A in RRC_CONNECTED | 1. Category M1 2. HD-FDD 3. CE Mode A | Yes | N/A | The network cannot schedule a PDSCH with TBS larger than 1000 bits for Cat. M1 Ues | Per UE | FDD only | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Decision: As per email decision posted on Oct 20th,

Agreement

For component 3 in FG 1-3, select one of the following options:

- Option 1: UE reports one of {alt1, alt2-e, both}

- Option 2: UE supports both Alt-1 and Alt-2e

Agreement

Following features are removed from the prerequisite feature groups for FG 1-3

- CE Mode A

- Support of 10 DL HARQ processes in HD-FDD in CE mode A

- HARQ-ACK bundling support in HD-FDD in CE mode A

Agreement

CE Mode A is removed from the prerequisite feature groups for FG 1-4.

In this contribution we present our views on the remaining issues for UE features for eMTC/NB-IOT.

# General principles

One of the most contentious issues in the last meeting was whether the features for eMTC and NB-IoT were “per UE” or “per band”.

In our view, the newly introduced features should be “per band” for the following reason: with the introduction of NTN IOT, there should be a way to differentiate the capability of using 16-QAM or 14 HARQ processes for terrestrial and non-terrestrial cases. “Per band” capability indication allows for this indication.

Observation 1: All the IOT features should be “per band” to allow differentiation of support of a feature in terrestrial and non-terrestrial networks.

# NB-IoT features

For support of 16-QAM, in line with the observation above, we propose the type of this feature to be “per band”.

For the particular case of 16-QAM, there is an additional reason to make this feature “per band”: different frequency bands may have different difficulties to meet the necessary RF requirements to support 16-QAM (e.g. in terms of transmit / receive EVM).

Proposal 1 : The capability for 16-QAM for NPUSCH and NPDSCH (FG1-1 and 1-2) is “per band”.

# eMTC features

One of the open items for eMTC is whether the UE shall support both alternatives 1 and 2-e, or it is allowed to support signaling to only one of them.

In our view, the UE should be able to signal separate support for both alternatives for the following reasons:

- The feature works even if the UE supports only one of the alternatives.

- If all networks implement only one of the two alternatives, the UE cannot do IODT testing for the other alternative. This can result in faulty UEs misbehaving in the field if the feature is enabled in the future.

Due to this reason, we propose to allow the UE to signal the support of alt-1 and 2-e separately:

Proposal 2: For component 3 in FG 1-3, UE reports one of {alt1, alt2-e, both} (Option 1)

Additionally, and in line with the observation in Section 2, we propose the eMTC features to be “per band”.

Proposal 3: FG 1-3 and 1-4 are “per band”.

# Summary of proposals

In this contribution we presented our views on UE features for NB-IOT and eMTC. We made the following observations and proposals.

Observation 1: All the IOT features should be “per band” to allow differentiation of support of a feature in terrestrial and non-terrestrial networks.

Proposal 1 : The capability for 16-QAM for NPUSCH and NPDSCH (FG1-1 and 1-2) is “per band”.

Proposal 2: For component 3 in FG 1-3, UE reports one of {alt1, alt2-e, both} (Option 1)

Proposal 3: FG 1-3 and 1-4 are “per band”.