3GPP TSG RAN WG1 #107-e			R1-2112273

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.14

Source:	Xiaomi

Title:	Discussion on UE features for IoT NTN

Document for:	Discussion

Introduction

In [1], the updated RAN1 UE features for Rel-17 LTE IoT NTN are provided as the basis for RAN1-107b-e meeting discussion. In this contribution, we provide our comments on the update RAN1 UE features.

Comments on UE features

On 26-1

For bullet 1-1, we have discussed about the UE reporting of GNSS position fix validity duration, this includes not only the validity duration, but also the the time of last GNSS fix or the time at which GNSS position fix will become invalid. It is suggested to capture it.

It seems bullet 4 and bullet 5 are overlapped to some extent, it is suggested to remove the bullet 4.

On bullet 6, we are confused why it is captured as the UE feature, it is suggested to remove this bullet

On bullet 12 – 15, it is more related to UE FG 26-2, it is suggested to move these bullet to 26-2.

The detailed updates can be found in the appended table.

Conclusions

In this contribution, we present the discussion on the UE features for NR NTN. Based on our analysis, we make some updates on the UE features as in the appendix.

References

- R1-2110588, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #106bis-e, Moderaators (AT&T, NTT DOCOMO)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE report the GNSS position fix validity duration] 2. [Indicate/receive] serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Derive epoch time of serving satellite ephemeris from implicit epoch time] 3. Receive ephemeris orbital parameter for the serving satellite 5. UE specific TA calculation [in [RRC_IDLE and RRC_CONNECTED state] based on its GNSS-acquired position and the serving satellite ephemeris. [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 10. Support a validity timer of UL synchronization is configured by the network 11. Support segment duration configuration is by the network [and updating time and frequency pre-compensation between segments but not within segments during UL repetition] |  | [No] | N/A | Release 17 UE cannot access [NTN/satellite] |  | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | - UE receives UE specific K_offset - 2. UE applies K_offset in timing relationship enhancements 3. UE receives cell specific K_offset 4. UE applies K_offset in timing relationship enhancements 5.UE estimates of UE-gNB RTT - [6. delaying the starts of ra-ResponseWindow] | 2-1 [, 2-3] | [No] | N/A | UE does not know the offset to apply for UL transmission |  | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | [2-3] | [UE reporting of information about the UE specific TA pre-compensation] | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - [frequency of the reports] - [granularity of the reported content] | [2-1] | No | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD |  | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |