3GPP TSG RAN WG1 Meeting #107-e                                     R1-2112287

e-Meeting, November 11th – 19th, 2021

Agenda Item: 8.3.3

Source: MediaTek Inc.

Title: Methods for Intra-UE Multiplexing and Prioritization

Document for: Discussion and Decision

# Introduction

During RAN#88-e plenary [1], it was agreed to specify the required enhancement for intra-UE Multiplexing and Prioritization, and in particular:

The multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

# Multiplexing between different priorities

## Multiplexing into a PUCCH

In RAN1#102e [2], the following agreement has been made:

Agreements: Support multiplexing for following scenarios in R17: - Multiplexing a high-priority HARQ-ACK and a low-priority HARQ-ACK into a PUCCH in R17. - Multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH for some HARQ-ACK/SR PF combinations (FFS applicable combinations). - Multiplexing a low-priority HARQ-ACK, a high-priority HARQ-ACK and a high-priority SR into a PUCCH. For the above multiplexing scenarios, - FFS conditions, if needed, for the multiplexing, e.g - Whether to support multiplexing between different resources not confined within a sub-slot. - Whether to support multiplexing in case a PUCCH overlaps with more than one PUCCH. - Timeline requirements. - FFS: details, if needed, of the multiplexing scheme, e.g. - How to minimize impact on the latency for high-priority HARQ-ACK. - How to determine the PUCCH resource used for multiplexing (e.g. HP or LP PUCCH resource, or a dedicated PUCCH resource for the multiplexing). - How to multiplex the HARQ-ACK bits (e.g. multiplexing, bundling). - How to encode the UCIs with different priorities (e.g. separate coding vs. joint coding) - How to guarantee the target code rate (e.g. payload control, multiplexing priority, LP HARQ-ACK compression/compaction). - Explicit indication for enabling multiplexing. - Multiplexing rule and order (e.g. HP/LP multiplexing is after resolving collision within the same priority).

### Indication of Multiplexing

Dynamically indicating to the UE if multiplexing is taking place is very complex to handle at the UE side and requires a lot of implementation effort as the UE needs to accommodate two scenarios for each case which will complicate the implementation.

Dynamic indication of the multiplexing activation/de-activation is not supported.

### Multiplexing timeline

In last RAN1 meeting, the following working assumption was reached:

Working assumption: Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met

In R15, there is a “guard gap” where the PUCCH can’t overridden anymore. This to give the UE time to build the codebook and prepare the PUCCH. When multiplexing HARQs from two priorities, there will be two guard gaps: one associated with the original LP-PUCCH and one associated with the HP-PUCCH (which will contain the LP HARQ codebook as well). When multiplexing HP-PUCCH and LP-PUCCH, the guard gaps of the two PUCCHs should be taken into consideration.

To avoid the cancelation process for the LP-PUCCH, the UE needs to know about the multiplexing between HP and LP channels before “guard gap” of the earliest PUCCH.

Figure 1: Multiplexing timelines

Guard gap timeline of the new multiplexed PUCCH is of the earliest PUCCH.

### Multiplexing conditions

In last RAN1 meeting, the following agreement was reached:

Agreements: For multiplexing UCIs of different priorities in a PUCCH in R17, - Support of multiplexing between different resources not confined within a sub-slot if conditions are met - FFS: Details - Support multiplexing in case a PUCCH overlaps with more than one PUCCH if conditions are met - FFS details

Multiplexing between a first HP-PUCCH transmitted on a sub-slot and LP-PUCCH transmitted on a slot-based could impact a second HP-PUCCH if the new multiplexed PUCCH overlaps with the second HP-PUCCH and the guard gap is not enough to multiplex the second HP-PUCCH.

Multiplexing between PUCCHs spanning different sub-slot/slot durations, raises a lot of concern and some HP-PUCCHs could be dropped or delayed in multiple scenarios. Hence, we think that multiplexing of UCI of different priorities should be allowed only if PUCCHs are within the same sub-slot/slot otherwise the Rel-16 dropping rules are to be used.

However, to ensure lower latency of the URLLC traffic, sub-slots will be usually configured. And if eMBB traffic is also simultaneously supported larger sub-slots or slot based will be usually configured. Hence, if multiplexing between PUCCHs on different sub-slot/slot lengths is not supported, then this functionality will be rarely used in practice and if the gNB wants to use it then it needs to configure sub-slot with the same duration for eMBB which will limit the PUCCH durations used for eMBB. One possible solution is to allow multiplexing only if the resulted PUCCH is confined within the sub-slot of the HP-PUCCH sub-slot.

Multiplexing allowed only if the resulted PUCCH is confined within the sub-slot of the HP-PUCCH sub-slot.

Some of conditions to also possibly consider for multiplexing is the size of the LP HARQ-ACK bits. However this condition could be restrictive as it will limit the possibility of multiplexing.

Hence, we propose to support group-bundling. Group bundling is bundling every small group of N1 bits to N2 (E.g. N1 = 2, N2 =1). The group bundling is to be used only if the new UCI payload (number of bits) is above a certain threshold maxUCIPayload where the parameter maxUCIPayload is specified or signalled by the gNB and which guarantee good reliability and latency of the multiplexed PUCCH.

Group-bundling could be applied per TB of the LP-HARQ feedback. For example, if CBG-level HARQ-ACK feedback is enabled for a TB, only the TB HARQ-ACK feedback is sent when the LP-UCI is multiplexed on HP channel.

Group-bundling is supported when multiplexing and when the resulted UCI payload is large.

Regarding the case of HP-SR with LP-HARQ: the PUCCH format on which the HARQ is carried is very important to take into consideration in specifying this scenario. For collision with HARQ transmitted on PF 0/1, the Rel-15 rules are not optimal but could be maintained. For example the Rel-15 rule for multiplexing PF1 SR with PF1 HARQ preserves both reliability and latency of SR and also similar with PF0 SR and PF0 HARQ. In the case of more than two HARQ bits, scheduling PF2 HARQ around SR can mitigate situations when multiplexing would fail to meet the SR reliability and latency.

## Simultaneous PUSCH and PUCCH for intra-band CA.

The following agreement has been made in RAN1:

Agreements: (RAN1#102e) Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA. Agreements: (RAN1#104e) Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

The following cases could be distinguished for the case of intra-band CA:

- Same numerology

  - Aligned channel case:

    - simultaneous PUCCH/PUSCH transmissions on different cells could be supported for this case with no issue.

  - Non-aligned channel case:

    - The performance in the non-aligned UL channels is to be impacted by phase discontinuity. However, this is under the network control and the UE doesn’t need to worry about it. The gNB can allow this to take place if it finds that the performance degradation is still within the targeted reliability.

- Different numerology

  - The UE implementation is very challenging to be able to support setting different gain of a symbol in the same TX path (co-PA) in mixed numerology. The set gain is usually done per symbol as a granularity and it is very challenging on the HW to set different gain level during an ongoing symbol.

  - If the transmission is aligned on symbol-level (with the lowest SCS) in mixed numerology, it could resolve most of cases, but the SW implementation will be difficult.

There is no difference between if the channels are PUSCHs or PUSCH vs PUCCH, i.e., the above issues exist already for PUSCH transmissions in intra-band CA.

Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA for the same numerology both with aligned and non-aligned channel case.

Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA for different numerology if the transmissions are aligned on symbol-level (with the symbol of the lowest SCS as a reference).

- i.e. Allocation on the carrier with higher numerology doesn’t start during an ongoing symbol on the other carrier with the smaller numerology.

The UE could be configured by high layer parameters to enable this functionality for inter-band and intra-band CA. This could be done separately for intra-band and inter-band CA.

The UE is to be configured separately for inter-band and intra-band simultaneous PUCCH/PUSCH transmissions.

For UE that is capable of simultaneous PUCCH/PUSCH transmission over different cells, there is no difference from UE implementation if the PUCCH and PUSCH were of the same or different PHY priorities.

The gNB should be able to enable/disable this feature for the case of same PHY priority separately from the case of different PHY priorities. So, simultaneous PUCCH/PUSCH transmission of the same PHY priority over different cells could be RRC configured within the same PUCCH group. Hence, if the PUSCH and PUCCH have different PHY-level priorities, the simultaneous PUCCH/PUSCH transmissions is enabled, otherwise, if the PUSCH and PUCCH have the same PHY-level priority, the PUSCH and PUCCH are multiplexed. Another possibility is to enable simultaneous PUCCH/PUSCH transmissions of different priorities only if the low priority channel would have been dropped otherwise. For example, if the PUSCH and PUCCH have different PHY-level priorities and can’t be multiplexed (e.g. the low priority channel would have been dropped based on R16 or R17 prioritization rules), the simultaneous PUCCH/PUSCH transmissions is enabled for this case. Otherwise, if the PUSCH and PUCCH can be multiplexed (e.g. based on R17 multiplexing rules between channels of different PHY-level priorities), then the PUSCH and PUCCH are multiplexed.

Also, the simultaneous PUCCH/PUSCH transmissions could be restricted to the case of LP-PUCCH carrying HARQ feedback. For example, if the LP-PUCCH does not carry HARQ feedback, simultaneous PUCCH/PUSCH transmissions is disabled and the prioritization rules of R16/R17 could be performed (e.g. dropping the LP-PUCCH).

Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of the same PHY priority over different cells can be RRC configured within the same PUCCH group.

Simultaneous PUCCH/PUSCH transmissions is enabled based on specific conditions. E.g. LP-PUCCH carrying HARQ feedback.

# Conclusion

In this contribution, we have made the following proposals:

- Dynamic indication of the multiplexing activation/de-activation is not supported.

- Guard gap timeline of the new multiplexed PUCCH is of the earliest PUCCH.

- Multiplexing allowed only if the resulted PUCCH is confined within the sub-slot of the HP-PUCCH sub-slot.

- Group-bundling is supported when multiplexing and when the resulted UCI payload is large.

- Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA for the same numerology both with aligned and non-aligned channel case.

- Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA for different numerology if the transmissions are aligned on symbol-level (with the symbol of the lowest SCS as a reference).

- i.e. Allocation on the carrier with higher numerology doesn’t start during an ongoing symbol on the other carrier with the smaller numerology.

- The UE is to be configured separately for inter-band and intra-band simultaneous PUCCH/PUSCH transmissions.

- Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of the same PHY priority over different cells can be RRC configured within the same PUCCH group.

- Simultaneous PUCCH/PUSCH transmissions is enabled based on specific conditions. E.g. LP-PUCCH carrying HARQ feedback.

# References

- RP-201310 “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR” Nokia, Nokia Shanghai Bell, 3GPP TSG RAN Meeting #88e.

- Chairman’s Notes RAN1#102e