3GPP TSG RAN WG1 #107-e			R1-2112291

e-Meeting, November 11th – November 19th, 2021

Agenda item:	8.2.6

Title:	On the channel access mechanisms for 52.6-71 GHz NR operation

Source:	MediaTek Inc.

Document for:	Discussion

- Introduction

The channel access mechanisms for 52.6-71 GHz NR operation were agree to be part of work items during RAN #90e and the WID was further revised in RAN #93-e meeting [1]. The agreements are listed as follows.

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement

In this contribution, we discuss different aspects for channel access mechanisms in 60 GHz, including sensing structure aspects, multi-channel channel access and receiver-assisted LBT.

- LBT mechanisms enhancements

2.1 Sensing structure aspects

In RAN 1 #106-e meeting, the sensing structure aspects for 60 GHz was agreed to down select following alternatives.

Agreement: For energy measurement in 8us deferral period, at least a single measurement within 8us is performed, and the measurement duration is selected from one of the following alternatives: - Alt 1: At least 3+X us (FFS X, such as X=1). - Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot and is within the 5 us observation slot. - Alt 3: At least a contiguous duration of X+Y us where the Y us part of the measurement is done at the end of the first 3 us and X is the same as the minimum measurement duration in a 5 us observation slot and is at the beginning of the 5 us duration.

As shown in the Fig. 1, the sensing structure of LBT scheme in 60 GHz consists of a defer duration (Td=8us) and several sensing slots (Tsl=3us) in the random backoff before acquiring a COT. Based on EN 302 567 [2], the number of sensing slot in the random backoff is randomly generated from the interval [0, CW], where CW cannot be less than 3. The sensing structure shown in the Fig. 2 is consistent with that in sub-6 NR-U, i.e., a defer duration and several sensing slots in the random backoff.

Fig. 1 Overall sensing structure of CAT 4 LBT in 60 GHz

The 8 us deferral period in 60 GHz discussed by 3GPP meetings actually corresponds to a defer duration in sub-6 NR-U, which is illustrated in Fig. 2. The defer duration contains an idle period (Tf) and

Fig. 2 Structure of a defer duration in sub-6 NR-U

mp sensing slots (Tsl), where mp depends on the channel access priority class (CAPC). Generally, the transmitted signals/channels possesses higher priority when the value of CAPC is smaller. In this case,

mp is smaller, which implies shorter LBT time. The purpose of idle period (Tf) in a defer duration is to ensure the previous transmission is finished. Hence, the number of energy measurement in a defer duration in sub-6 NR-U is only one. Besides, the CCA procedure in EN 302 567 is summarized as Fig. 3. It can be seen in step (b) of the Fig. 3 that EN 302 567 does not regulate additional energy measurement for 8 us deferral period. Moreover, in 802.11 ad [3], 8 us deferral period includes aSlotTime (5us) and aSIFSTime (3 us). In this 8 us, the energy measurement only needs to be implemented for one time within aSlotTime since aSIFSTime accommodates MAC processing delay and Rx-Tx turnaround time. The limitation for the duration of energy measurement in 802.11 ad is to be less than 3 us and the precise time duration of energy measurement is left for implementation.

Fig. 3 CCA procedure in EN 302 567

In our view, Alt 3 contradicts with the agreement in RAN 1 #106b-e meeting, which is listed as follows. Therefore, Alt 3 should be excluded. As for comparison between Alt 1 and Alt 2, the

Agreement: Confirm the WA with the following updates: For energy measurement in 5us observation slot, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

main difference between these alternatives is whether the energy measurement is completely within the last 5 us. Since the situation that a UE may need to have the transition from a receiver to a transmitter at the beginning of the energy measurement duration should be considered, compared with Alt 1, Alt 2 can leave more room for UE. Besides, Alt 2 is consistent with the scheme in the 802.11 ad. Therefore, Alt 2 should be supported

Proposal 1: For sensing structure within a 8 us deferral period, support Alt 2.

2.2 Multi-channel channel access

In RAN 1 #104-e  meeting, the agreement relevant to multi-channel channel access is achieved and listed as follows.

Agreement: Define Type A and Type B multi-channel channel access as: - Type A: Perform independent eCCA for each channel - Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot Down-selection between - Alt1: Support Type A multi-channel channel access only - Alt2: Support both Type A and Type B multi-channel channel access. Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

The main difference between type A and type B is whether all channels needs to perform cat 4 LBT. Compared with type A, type B needs to choose one channel to perform cat 4 LBT and the rest of channels perform cat 2 LBT. Currently, cat 2 LBT has been introduced for COT sharing scenario in 60 GHz, which is acceptable since in the COT sharing case, the introduction of cat 2 LBT gives rise to a more conservative COT sharing mechanism. However, for multi-channel channel access, type A is more conservative than type B. Besides, only performing cat 2 LBT could be regarded as a violation to ETSI regulation for 60 GHz. Therefore, we support only type A multi-channel channel access.

Proposal 2: Support only type A multi-channel channel access scheme.

2.3 Receiver-assisted LBT

In RAN 1 #106b-e meeting, several schemes for receiver-assisted LBT was discussed. In particular, an agreement that extends legacy L3-RSSI measurement to support receiver-assisted LBT is achieved and listed as follows

Agreement: Support extending Rel.16 L3-RSSI to unlicensed operation in FR2-2 - Introduce RRC configuration for reference SCS, measurement duration, and measurement bandwidth - Extend the reference SCS/CP field (ref-SCS-CP-r16) and measurement duration field (measDurationSymbols-r16) in RMTC-Config - FFS value range and valid combinations for ref-SCS-CP-r16 and measDurationSymbols-r16 - Introduce parameter in RMTC-Config to indicate the measurement bandwidth - FFS: Value range for measurement bandwidth - For the QCL Type-D of L3-RSSI measurement, down-select one or both of the following alternatives - Alt 1: gNB configures the beam when configures the L3-RSSI measurement - Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET

The first bullet of the above agreement comes from the fact that current RMTC parameters does not include the supported SCS in 60 GHz, i.e., 120 kHz, 480 kHz and 960 kHz. As shown in the Fig. 4, legacy RSSI measurement only supports up to 60 kHz SCS. Since the symbol duration of 480 and 960 kHz SCS are much shorter than that of 60 kHz SCS, the value for rmtc-Periodicity-r16 should also be adjusted.

Proposal 3: down-scale rmtc-Periodicity-r16 according to the supported numerology in RMTC-Config-r16.

Fig. 4 Detailed RRC Parameters for RMTC

As for the 2-nd bullet of the above agreement, we support Alt 2. Current unified TCI framework uses a multi-level based scheme. Specifically, a set of candidate TCI states are configured at RRC level. Each TCI state may correspond to a TRP beam. A subset of TCI states is activated in MAC level using MAC-CE activation, where UE needs to monitor source reference signal of the activated TCI states. One of the activated TCI states is indicated in DCI level. After receiving such DCI, an UE will update its serving TCI state at the moment of the application time. Based on the above procedure, it’s possible that the TCI state configured in RMTC-Config-r16 is different with that indicated in DCI, which will cause incorrect RSSI measurement results. With the support of Alt 2, it’s guaranteed that the TCI states for PDSCH reception or from DCI indication and for RSSI measurement are not different

Proposal 4: For QCL type-D of L3-RSSI measurement, support Alt 2.

- Conclusion

In this contribution we have discussed various aspects for channel access above 52.6 GHz and provided following proposals:

Proposal 1: For sensing structure within a 8 us deferral period, support Alt 2.

Proposal 2: Support only type A multi-channel channel access scheme.

Proposal 3: down-scale rmtc-Periodicity-r16 according to the supported numerology in RMTC-Config-r16.

Proposal 4: For QCL type-D of L3-RSSI measurement, support Alt 2.

- References

- RP- 212637, “Revised WID: Extending current NR operation to 71GHz”, Qualcomm, Intel, RAN#93-e

- EN 302 567 v2.2.0, Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum, Accessed on 2021-1-18

- IEEE Std 802.11ad™-2012: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems; Local and Metropolitan Area Networks—Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications; Amendment 3: Enhancements for Very High Throughput in the 60 GHz Band