3GPP TSG RAN WG1 #107bis-e			R1-2112319

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.16.8

Source: 	MediaTek Inc.

Title: 	Discussion on UE features for NR coverage enhancement

Document for:	Discussion and Decision

# Introduction

This document is to discuss the questions in FL summary for discussion in RAN1-106b_e [1].

# Overview on FGs for Coverage Enhancements

Proposal 1: For enhancement of PUSCH Type A repetitions, separated FGs for DG, CG Type 2 and CG Type 1 (if supported).

Proposal 2: For TBoMS, separated FGs for DG, CG Type 2 and CG Type 1 (if supported).

Proposal 3: For DMRS bundling, separated FGs for PUSCH and PUCCH enhancements.

Proposal 4: For DMRS bundling, separated FGs for B2B transmissions across the consecutive slots, non-B2B transmissions across the consecutive slots w/o other uplink transmissions, and B2B transmission within one slot.

Proposal 5: For DMRS bundling, separated FGs for Type A and Type B repetitions.

Proposal 6: For dynamic PUCCH repetition indication, UE capability for repetition of PUCCH format 3 and 4 are defined in FGs for Rel’17 CovEnh. UE capability for support repetition of PUCCH format 0 and 2 should be defined in FGs for Rel’17 URLLC.

Proposal 7: All UE features are per band.

Proposal 8: For DMRS bundling, the maximum duration is differentiated at least for FR1/FR2.

Proposal 9: All UE features are optional with capability signalling

More detailed comments can be found in the following sections to address the questions listed in the FL summary [1].

# 30-1 to 30-2a: Enhancements for PUSCH Type A repetitions

In [1], FGs 30-1 to 30-2a are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configured grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

## Discussion

[FL1] High priority question 2-1:

- Companies are encouraged to provide views on whether/how to separate/merge FGs 30-1 to 30-2a, e.g.,

  - Option 1:

    - Merge FGs 30-1 and 30-1a into an FG

    - Merge FGs 30-2 and 30-2a into an FG

  - Option 2:

    - Merge FGs 30-1, 30-1a, 30-2, and 30-2a into an FG

  - Option 3:

    - Split 30-1 and 30-1a into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

    - Split 30-2 and 30-2a into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

  - Option 4:

    - Any other FG structures

| Company | Comment |
| --- | --- |
| FL4 | According to the comments provided so far, companies have different views: - Option 1: DOCOMO, Nokia, NSB, ZTE, CMCC Lenovo, Motorola Mobility, Huawei, Hisilicon, Ericsson - Option 2: Nokia, NSB, ZTE, Samsung, CMCC, Apple - Option 3: Qualcomm, Intel, Apple, MediaTek - Option 4 (Current structure is kept): vivo, Sharp |
| MediaTek | Option 3, it is reasonable and easier for linkage with the prerequisites of the separated FGs defined in Rel’15. |

Medium priority question 2-2:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FGs 30-1 to 30-2a can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| MediaTek | Optional with capability signalling. |

Medium priority question 2-3:

- Companies are encouraged to provide views on whether the type of FGs 30-1 to 30-2a should be per UE or per band

| Company | Comment |
| --- | --- |
| Most companies | Per UE |
| Qualcomm | Please see our first proposal in our tdoc. With UE testing and certification in mind, we would like this feature type to be per band. Proposal 1: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. |
| MediaTek | per band. Because UE may not need to support such feature for some bands, e.g., unlicensed band and ITS band |

Low priority question 2-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 30-1 to 30-2a

| Company | Comment |
| --- | --- |
| MediaTek | Firstly, separated FGs for DG, Type 2 CG and Type 1 CG. Accordingly, the prerequisite for each of them can be 5-17, 5-16 and 5-14 respectively. |

Reference:

R-15:

| 5-14 | Type 1 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5-16 | Type 2 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 5-17 | PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |

R-16:

| 11-6 | PUSCH repetition Type A | PUSCH transmission with Rel-15 behavior with or without slot aggregation. - With slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16). - When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | One of {5-16, 5-17] | pusch-RepetitionTypeA-r16 { sharedSpectrumChAccess-r16, non-SharedSpectrumChAccess-r16 } | Phy-ParametersCommon | No | No | Note: RAN1 agreed it should be possible to separately indicate support of this FG based on whether the UE is operated with or without shared spectrum access. It is left to RAN2 how to implement this while leaving the type as"per UE" | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 30-3: TB processing over multi-slot PUSCH

In [1], FG 30-3 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

## Discussion

[FL1] High priority question 3-1:

- Companies are encouraged to provide views on whether/how to separate FG 30-3, e.g.,

  - Option 1:

    - Split 30-3 into 3 separate FGs: 1st one for DG, 2nd one for type 1 CG, 3rd one for type 2 CG

  - Option 2:

    - Split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG

  - Option 3:

    - Any other FG structures

| Company | Comment |
| --- | --- |
| FL2 | According to the comments provided so far, companies have different views: - Option 1: Qualcomm, Intel, vivo, Apple, MediaTek - Option 2: Qualcomm, DOCOMO, vivo, Apple, MediaTek - Option 3: - Current structure: Nokia, NSB, ZTE, Samsung, CMCC, Lenovo, Motorola Mobility, Huawei, Hisilicon - Follow the structure for FGs 30-2/2a/2b: Sharp |
| Qualcomm | The prerequisites go back to R15 features 5-16 and 5-17. We prefer to preserve separate lineage of prerequisites and not merge them into one. |
| MediaTek | Separated FGs are needed for linkage with the prerequisites, following the same rule as R15. Thus, Option 1 is more preferred. |

[FL1] High priority question 3-2:

- Companies are encouraged to provide views on whether/how to include the capability for the repetition of TB processing over multi-slot PUSCH

| Company | Comment |
| --- | --- |
| FL4 | According to the comments provided so far, companies still have different views: - Need to introduce an FG: DOCOMO, ZTE, Sharp, Lenovo, Motorola Mobility, Apple, Ericsson, QC, MediaTek - No need to introduce an FG: Intel, vivo, Samsung, CMCC, Huawei, Hisilicon Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| MeidaTek | Separated capability for TBoMS repetition. |

[FL1] High priority question 3-3:

- Companies are encouraged to provide views on whether/how to include the capability for the maximum concurrent TBoMS transmissions supported by a UE across all carriers when operating in UL-CA

| Company | Comment |
| --- | --- |
| FL2 | According to the comments provided so far, companies have different views: - Need to introduce an FG: Qualcomm, ZTE - Wait for some progress in AI 8.8.1.2: DOCOMO, Intel, Samsung, CMCC, Lenovo, Motorola Mobility, Huawei, Hisilicon, MediaTek Therefore, no additional proposal is made for now, but any company can propose to add the FG when some progress is made AI 8.8.1.2. |
| MediaTek | No discussion to support CA. Additionally, there seems no need of CA support for the cell edge operation. |

Medium priority question 3-4:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FG 30-3 can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| MediaTek | OK to confirm. |

Medium priority question 3-5:

- Companies are encouraged to provide views on whether the type of FG 30-3 should be per UE or per band

| Company | Comment |
| --- | --- |
| MediaTek | per band. Because UE may not need to support such feature for some bands, e.g., unlicensed band and ITS band |

Medium priority question 3-6:

- Companies are encouraged to provide views on xDD/FRx differentiation for FG 30-3

| Company | Comment |
| --- | --- |
| MediaTek | Maybe No need of differentiation. |

# 30-4 to 30-4g: [DM-RS bundling]

In [1], FGs 30-4 to 30-4g are captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

## Discussion

[FL1] High priority question 4-1:

- Companies are encouraged to provide views on whether UE can report different values of maximum duration for DMRS bundling for (a) different modulation orders, (b) back-to-back and non-back-to-back transmissions

| Company | Comment |
| --- | --- |
| FL2 | According to the comments provided so far, companies have different views: (a) different modulation orders - UE can report different values: Qualcomm - UE reports a single value: - Wait for RAN4 reply: DOCOMO, Intel, ZTE, vivo, Sharp, Samsung, Lenovo, Motorola Mobility, Apple, MediaTek (b) back-to-back and non-back-to-back transmissions - UE can report different values: Qualcomm - UE reports a single value: Intel, ZTE, Samsung, Lenovo, Motorola Mobility - Wait for RAN4 reply: vivo, Sharp, Apple, MediaTek Therefore, no additional proposal is made for now, but any company can propose them when RAN1 gets RAN4 reply. |
| MediaTek | Wait for RAN4 feedback. |

[FL1] High priority question 4-2:

- Companies are encouraged to provide views on whether/how to revise the structure for FGs 30-4 and 30-4x, e.g.,

  - Q1: Whether to wait for RAN4 input before discussing the structure

  - Q2: Whether to split to back-to-back transmission and non-back-to-back transmission

    - Q2a: For non-back-to-back transmission, whether to add an FG for the case when other UL signals/channels are inserted in the gap between repetitions with same setting

  - Q3: Whether to merge FGs 30-4a, 30-4b, 30-4c and 30-4d into FG 30-4

  - Q4: Whether to merge FGs 30-4b, 30-4c, 30-4d, and 30-4g into FG 30-4a

  - Q5: Whether to merge FG 30-4f into FG 30-4e or remove FG 30-4f from the UE feature list

| Company | Comment |
| --- | --- |
| FL2 | According to the comments provided so far, companies have different views: Q1: Whether to wait for RAN4 input before discussing the structure - Yes: DOCOMO, ZTE, Lenovo, Motorola Mobility, Huawei, Hisilicon, Apple - No: Qualcomm, vivo Q2: Whether to split to back-to-back transmission and non-back-to-back transmission - Yes: Qualcomm, Intel, Sharp, MediaTek - Q2a: For non-back-to-back transmission, whether to add an FG for the case when other UL signals/channels are inserted in the gap between repetitions with same setting - Yes: Qualcomm, Intel, Huawei, Hisilicon, MediaTek - No: - No: Nokia, NSB, ZTE, Samsung, Lenovo, Motorola Mobility Q3: Whether to merge FGs 30-4a, 30-4b, 30-4c and 30-4d into FG 30-4 - Yes: Lenovo, Motorola Mobility - No: Intel, Sharp, Apple, MediaTek Q4: Whether to merge FGs 30-4b, 30-4c, 30-4d, and 30-4g into FG 30-4a - Yes: Lenovo, Motorola Mobility - No: Intel, Sharp, Samsung (remove FG 30-4g), Apple, MediaTek Q5: Whether to merge FG 30-4f into FG 30-4e or remove FG 30-4f from the UE feature list - Yes: Nokia, NSB, Lenovo, Motorola Mobility - No: Intel, Sharp, MediaTek Therefore, no additional proposal is made for now, but any company can propose them when RAN1 gets RAN4 reply or some progress is made in AI 8.8.1.3. |
| MediaTek | In general, the features should be set separately for different channels (PUCCH/PUSCH) and the different use case (back-to-back，non-back-to-back) and slot level (for Type B PUSCH repetition since there is within-slot repetition and cross-slot B2B repetitions respectively). Moreover, PUCCH may not be the bottleneck for the most cases/bands, it is reasonable to separate it with PUSCH. |

Medium priority question 4-3:

- Companies are encouraged to provide views on whether the column for “Mandatory/Optional” in FGs 30-4 and 30-4x can be confirmed as “Optional with capability signaling”

| Company | Comment |
| --- | --- |
| MediaTek | OK to confirm |

Medium priority question 4-4:

- Companies are encouraged to provide views on whether the type of FGs 30-4 and 30-4x should be per UE, per band, or per FS

| Company | Comment |
| --- | --- |
| MediaTek | per band. Because UE may not need to support such feature for some bands, e.g., unlicensed band and ITS band |

Medium priority question 4-5:

- Companies are encouraged to provide views on xDD/FRx differentiation for FGs 30-4 and 30-4x

| Company | Comment |
| --- | --- |
| MediaTek | No need of differentiation on xDD. For FRx differentiation, the maximum duration is differentiated at least for FR1/FR2. |

Low priority question 4-6:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 30-4 and 30-4x

| Company | Comment |
| --- | --- |
| MediaTek | For 30-4d, it may also depend on the PUCCH format to be supported for repetition in Rel’17 CovEnh. R’17 CovEnh may not need to support format 0/1/2 at least. |

# 30-5: Dynamic PUCCH repetition indication

In [1], FG 30-5 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-5 | Dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | [Optional with capability signalling] |

## Discussion

[FL1] High priority question 5-1:

- Companies are encouraged to provide views on whether FG 30-5 can be kept as “Slot based Dynamic PUCCH repetition indication”

  - Note: Subslot based dynamic PUCCH repetition indication is currently captured as FG 25-3a “Repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots using dynamic repetition indication” in [1]

| Company | Comment |
| --- | --- |
| FL4 | Following was agreed at the 1st check point (October 14) Agreement - FG 30-5 is kept as “Slot based dynamic PUCCH repetition indication” as follows 30. NR_cov_enh; 30-5; Slot based dynamic PUCCH repetition indication; Support dynamic PUCCH repetition indication; [4-23]; Yes; N/A; UE does not support Dynamic PUCCH repetition indication; [Per UE]; FFS; No; N/A; [Optional with capability signalling]. Let’s further discuss the contents highlighted in yellow in the next step. |
| MediaTek | Support slot based dynamic PUCCH repetition indication only for format 3 and 4. Accordingly, the prerequisite 4-23 is limited for format 3 and 4 rather than 1. And whether to support format 0/2 is up to URRLC capability. |

[FL4] Medium priority question 5-3:

- Companies are encouraged to provide views on whether the type of FG 30-5 should be per UE or per band

| Company | Comment |
| --- | --- |
| MeidaTek | per band. Because UE may not need to support such feature for some bands, e.g., unlicensed band and ITS band |

[FL4] Low priority question 5-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FG 30-5

| Company | Comment |
| --- | --- |
| MediaTek | Essentially, there may be two components. One is support of the new (unified) DCI for repetition. The other is support of repetition of some format (e.g., format 3). |

Reference：

| 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# 30-6: Msg3 repetition

In [1], FG 30-6 is captured as below.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |

## Discussion

[FL1] High priority question 6-1:

- Companies are encouraged to provide views on whether FG 30-6 is necessary or not

| Company | Comment |
| --- | --- |
| FL4 | According to the comments provided so far, companies have different views: - Wait for RAN2 progress: DOCOMO - FG 30-6 is necessary: Nokia, NSB, ZTE, vivo, Sharp, CMCC, Huawei, Hisilicon, Ericsson - FG 30-6 is not necessary: Intel, Samsung, Apple Therefore, no additional proposal is made for now, but companies are encouraged to check the comments provided so far and indicate if their position is changed. |
| MediaTek | Such feature may be needed. But how to indicate it can be up to FFS. |

Medium priority question 6-2:

- Companies are encouraged to provide views on whether capability signaling is necessary for FG 30-6, i.e., whether to support as optional with capability signaling or optional without capability signaling

| Company | Comment |
| --- | --- |
| Most companies | Optional with capability signaling. |
| Sharp | For allocating random access resource dedicated for RRC_CONNECTED UE with CBRA (e.g., for BFR, handover), capability signaling is required. |
| MediaTek | It may depend on RAN2’s configuration. If the separated resources are anyway configured even for the case w/o need of repetition, then it seems no need of such signaling. |

Low priority question 6-5:

- Companies are encouraged to provide views on whether/how to revise any other contents in FG 30-6 which do not have capability signaling impacts

| Company | Comment |
| --- | --- |
| MediaTek | OK to remove “in RRC connected mode” to cover both idle (transaction phase) and connected mode. |

# References

[1]	R1-2109714	Summary on UE features for NR coverage enhancement