3GPP TSG RAN WG1 #107-e	R1-2112329

e-Meeting, Nov 11th – 19th, 2021

Agenda item:		8.15.1

Source:	Nordic Semiconductor ASA

Title:					Enhancements to time and frequency synchronization

Document for:		Discussion and Decision

# Introduction

RAN#92 approved WID on NTN IoT in [1]. One of the RAN1 objectives are

Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763: - UE pre-compensation including ephemeris format (orbital / Position -Velocity) - UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris - Timing advance formula (granularity of the timing advance may be different) - Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any. Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763: - Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments. - Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects - DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB. - GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission

# Discussion on synchronization

UL pre-compensation segments

In the previous meetings, many companies have pointed out that new UL gaps may be needed during long UL transmission so that consecutive UL pre-compensation segments could be separated in time. Since the range of segment lengths is quite large (e.g. from 2 ms up to 256 ms for NPUSCH), the network can always configure UE with the segment length guaranteeing small TA change from segment to segment and, therefore, new UL gaps between segments are not needed. If the consecutive segments overlap due to TA change, a part of the cyclic prefix of the latter segment can be discarded.

Proposal-1: No new gaps are introduced for long UL transmissions.

Although UE position within a cell has an impact on needed segment length, the segment length can be common to all UEs within a cell and determined by eNB, and thus it can be signaled in SIB also after initial access.

Proposal-2: The configuration of UE pre-compensation segment should be signaled in SIB during initial access and after initial access.

Ephemeris and almanac

As pointed out in [2] in last meeting, prediction of at least 60s ahead is possible with very good accuracy. Moreover, for example GPS ephemeris is valid up to 2-4h to determine satellite position with sufficient precision to estimation position with several meter accuracy. GPS ephemeris comprises of 600 bits, while almanac 15kbit. Therefore, when accessing a particular satellite reading one additional SIB is sufficient. Nevertheless, we believe it should be up to eNB to determine the validity of ephemeris it broadcasts in a SIB. Every time UE acquires new ephemeris, it will reset the timer to initial value X.

Proposal-3: If serving satellite ephemeris and common TA are signaled in separate SIB messages, a separate validity timer for serving satellite ephemeris and timer for common TA is configured by eNB with initial timer values X and Y. Validity timer for SIB ephemeris is reset at least upon UE reading SIB with ephemeris and validity timer for common TA is reset at least upon UE receiving SIB with common TA.

# Conclusions

Regarding the synchronisation of NTN we had the following Proposals:

Proposal-1: No new gaps are introduced for long UL transmissions.

Proposal-2: The configuration of UE pre-compensation segment should be signaled in SIB during initial access and after initial access.

Proposal-3: If serving satellite ephemeris and common TA are signaled in separate SIB messages, a separate validity timer for serving satellite ephemeris and timer for common TA is configured by eNB with initial timer values X and Y. Validity timer for SIB ephemeris is reset at least upon UE reading SIB with ephemeris and validity timer for common TA is reset at least upon UE receiving SIB with common TA.

References

- RP-211601, “WID on NB-IoT/eMTC support for NTN”, MediaTek Inc., Eutelsat S.A.

- R1-2104565,” UE Time and frequency Synchronisation for NR-NTN”, MediaTek