3GPP TSG-RAN WG1 Meeting #107-e	R1-2112413

# e-Meeting, November 11th – November 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | xxx | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on prioritization rules of SRS carrier switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current specification has defined the prioritization/collision handling rules for SRS carrier switching in S6.2.1.3 of TS 38.214. However, it is unclear on which carrier the transmissions should be taken into account with the prioritization rules and how to determine a case “beyond” a UE’s indicated UL-CA capability. For example, it is not clear enough whether an uplink carrier in the same band with the uplink carrier configured as a “switch-from” carrier for SRS carrier switching is also applied with the prioritization rules. Such issue has been discussed in Rel-14 LTE by R1-1712771, R1-1714652 and R1-1809001, and solved in Rel-14 LTE by CR R1-1721095 and R1-1809554/R1-1809555. E.g. RAN1#90: Agreement in Principle: Capture the following in 36.213: - The “same PA” is implicitly identified by the following. Two “CC with the same PA” are those that: - Are in the same band - Are in the same TAG - Have the same CP - Adopt the following solution for the cases of collision between SRS switching and victim CC: - Extend collision rules to victim CC (i.e., the transmission of SRS depends on the information transmitted in the source CC and the victim CC). CR to be prepared for RAN1#90bis. According to the RAN1 agreement below, collision handling of SRS carrier switching that is similar to Rel-14 LTE is specified. RAN1#90bis: Agreement: Specify NR SRS switching among CCs similar to Rel-14 LTE SRS carrier-based switching design including Periodic/aperiodic/semi-persistent SRS on a CC without PUCCH/PUSCH configured TA (through PRACH) on TAG without PUSCH/PUCCH configured Power control separated from that of PUSCH Group common DCI for aperiodic SRS triggering and TPC DL/UL interruptions and collision handling due to SRS switching Therefore, similar clarifications on collision handling are also needed for NR. On this issue, some preliminary agreements are reached in RAN1#105-e. Agreement The prioritization rules of SRS carrier switching apply to at least the source CC. - FFS : Whether the specification needs to be updated or not Agreement Down select one from the following two options in RAN1#106-e meeting to determine which UL CCs other than the source CC should be used for SRS carrier switching priority rules: - Option 1: The UL CCs in the same band as the source CC - Option 2: The UL CCs can be any carriers which result in uplink transmissions beyond the UE ’s indicated uplink carrier aggregation capability. - Companies should indicate how “UE’s indicated uplink CA capability” is derived based on current ASN.1 signaling. An example of Option 2 in the above agreement is the case where - a UE is configured with higher layer parameter uplinkTxSwitching-r16 for two uplinks - one of the two uplinks is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier for SRS transmission on a third uplink with SRS carrier switching. Because the UE Tx RF chains are shared between the two uplinks configured with uplinkTxSwitching-r16, both uplinks are impacted and suspended by a SRS transmission on the third uplink triggered by SRS carrier switching, whose corresponding UE capability for SRS carrier switching is reported via BandCombinationList-UplinkTxSwitch. As a result, the prioritization rules for SRS carrier switching should take into account this case. Note that the UE capability container BandCombinationList-UplinkTxSwitch is dedicated to UE feature UL Tx switching. If a UE reports a support of SRS carrier switching via BandCombinationList-UplinkTxSwitch, the UE has indicated that its UE Tx RF chains are shared between uplinks configured with UL Tx switching and uplinks configured with SRS carrier switching. Therefore, “the UE's indicated uplink carrier aggregation capability” in sub-clause 6.2.1.3 of TS 38.214 can refer to this existing UE capability for this case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Clarify the conditons on which an uplink carrier should comply with the prioritization rules for SRS carrier switching, for both the intra-band case and the inter-band case of uplink Tx switching. 2) Clarify that the UE cannot transmit simultaneously in source and target, for both the intra-band case and the inter-band case of uplink Tx switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification for SRS carrier switching in term of transmission prioritization rules. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on SRS carrier switching with respect to the prioritization rules between uplink transmissions on different uplink carriers. Since the same clarification for the intra-band case has been adopted into LTE Rel-14 for a long time, it is not expected to impact on gNB/UE implementation. For the inter-band case with UL Tx switching, because the UE is always not able to retain any transmission on the uplinks configured with UL Tx switching when a related SRS carrier switching is performed, it is not expected to impact on gNB/UE implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BWP	Bandwidth part

CBG	Code block group

CLI	Cross Link Interference

CP	Cyclic prefix

CQI	Channel quality indicator

CPU	CSI processing unit

CRB	Common resource block

CRC	Cyclic redundancy check

CRI	CSI-RS Resource Indicator

CSI	Channel state information

CSI-RS	Channel state information reference signal

CSI-RSRP	CSI reference signal received power

CSI-RSRQ	CSI reference signal received quality

CSI-SINR	CSI signal-to-noise and interference ratio

CW	Codeword

DCI	Downlink control information

DL	Downlink

DM-RS	Dedicated demodulation reference signals

DRX	Discontinuous Reception

EPRE	Energy per resource element

IAB-MT	Integrated Access and Backhaul – Mobile Terminal

L1-RSRP	Layer 1 reference signal received power

LI	Layer Indicator

MCS	Modulation and coding scheme

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PSS	Primary Synchronisation signal

PUCCH	Physical uplink control channel

QCL	Quasi co-location

PMI	Precoding Matrix Indicator

PRB	Physical resource block

PRG	Precoding resource block group

PRS	Positioning reference signal

PT-RS	Phase-tracking reference signal

RB	Resource block

RBG	Resource block group

RI	Rank Indicator

RIV	Resource indicator value

RS	Reference signal

SCI	Sidelink control information

SLIV	Start and length indicator value

SR	Scheduling Request

SRS	Sounding reference signal

SS	Synchronisation signal

SSS	Secondary Synchronisation signal

SS-RSRP	SS reference signal received power

SS-RSRQ	SS reference signal received quality

SS-SINR	SS signal-to-noise and interference ratio

TAG	Timing advance group

TB	Transport Block

TCI	Transmission Configuration Indicator

TDM	Time division multiplexing

UE	User equipment

UL	Uplink

<Unchanged parts are omitted>

#### 6.2.1.3	UE sounding procedure between component carriers

For a carrier of a serving cell d with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote as s0(d) the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set S(d)= {s0(d)… sN-1(d)} as the set of carriers of serving cells that meet all the following conditions:

-	si(d) is in the same band as s0(d), or s0(d) and si(d) are both configured with uplinkTxSwitching-r16.

-	si(d) is in the same TAG as s0(d).

where 1≤i≤N-1.

A UE can be configured with SRS resource(s) on a carrier d with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier d, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier s0(d)  which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier d (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carrier S(d).

For an SRS transmission starting in symbol NdNc1 of carrier dc1 and a conflicting transmission in carrier si(d)c2 starting in symbol NsiNc2, where 1≤i≤N-1, the UE shall apply the prioritization / dropping rules in the remainder of this subclause taking into account:

- DCI(s) for which the time interval between the last symbol of PDCCH and NdNc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and NsiNc2 is at least   N2 symbols; and

- semi-persistent CSI reports or SRS considered active at least N2 symbols and an additional time duration TSRSCS before NdNc1, and considered active at least N2 symbols before NsiNc2.

where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the smaller subcarrier spacing across d, si(d)c1, c2 and their corresponding scheduling cells.

The following prioritization rules shall be applied in case of collision between a transmission of SRS over carrier d and transmission of a physical signal/channel over a carrier of a serving cell in set S(d):

-	the UE shall not transmit SRS whenever SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH/PUCCH transmission carrying HARQ-ACK/positive SR/RI/CRI/SSBRI and/or PRACH on a carrier of a serving cell in set S(d) happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

-	the UE shall not transmit a periodic/semi-persistent SRS whenever periodic/semi-persistent SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d and PUSCH transmission carrying aperiodic CSI on a carrier of a serving cell in set S(d) happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

-	the UE shall drop PUCCH/PUSCH transmission carrying periodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR, and/or SRS transmission on a carrier of a serving cell in set S(d)  configured for PUSCH/PUCCH transmission whenever the transmission and SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

-	the UE shall drop PUSCH transmission carrying aperiodic CSI comprising only CQI/PMI/L1-RSRP/L1-SINR on a carrier of a serving cell in set S(d) whenever the transmission and aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133]) as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on the carrier of the serving cell d happen to overlap in the same symbol and that can result in uplink transmissions beyond the UE's indicated uplink carrier aggregation capability included in [13, TS 38.306].

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeA', and given by SRS-CarrierSwitching, without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells in the indicated set of serving cells configured by higher layers, where the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

For an aperiodic SRS triggered in DCI format 2_3 and if the UE is configured with higher layer parameter srs-TPC-PDCCH-Group set to 'typeB' without PUSCH/PUCCH transmission, the order of the triggered SRS transmission on the serving cells follow the order of the serving cells with aperiodic SRS triggered in the DCI, and the UE in each serving cell transmits the configured one or two SRS resource set(s) with higher layer parameter usage set to 'antennaSwitching' and higher layer parameter resourceType in SRS-ResourceSet set to 'aperiodic'.

If the UE is not configured for PUSCH/PUCCH transmission on carrier c1 with slot formats comprised of DL and UL symbols, and if the UE is not capable of simultaneous reception and transmission on carrier c1 and serving cell c2, the UE is not expected to be configured or indicated with SRS resource(s) such that SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) would collide with the REs corresponding to the SS/PBCH blocks configured for the UE or the slots belonging to a control resource set indicated by MIB or SIB1 on serving cell c2.

For n-th (n ≥ 1) aperiodic SRS transmission on a cell c, upon detection of a positive SRS request on a grant, the UE shall commence this SRS transmission on the configured symbol and slot provided

-	it is no earlier than the summation of

-	the maximum time duration between the two durations spanned by N OFDM symbols of the numerology of cell c and the cell carrying the grant respectively, and

-	the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

-	it does not collide with any previous SRS transmissions, or interruption due to UL or DL RF retuning time.

otherwise, n-th SRS transmission is dropped, where N is the reported capability as the minimum time interval in unit of symbols, between the DCI triggering and aperiodic SRS transmission.

In case of inter-band carrier aggregation, a UE can simultaneously transmit SRS and PUCCH/PUSCH across component carriers in different bands subject to the UE's capability.

In case of inter-band carrier aggregation, a UE can simultaneously transmit PRACH and SRS across component carriers in different bands subject to UE's capability.

<Unchanged parts are omitted>