3GPP TSG-WG1 Meeting # 106bis-e	R1-2112424

# e-Meeting, October 11th - 19th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks in 36.213 s14-xx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN-Core |  |  |  |  |  | Date: |  |  | 2021-11-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support for NB-IoT/eMTC support for Non-Terrestrial Networks features according to RAN1 agreements: - timing relationship enhancements - TA advance adjustment timing, NPRACH transmission timing, Msg3 transmission timing, NPDCCH monitoring timing, and HARQ-ACK reporting transmission timing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of NB-IoT/eMTC support for Non-Terrestrial Networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.1.2, 16.3.2, 16.4.2, 16.5.1, 16.6, 16.6.1, 16.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

# 16	UE Procedures related to narrowband IoT

## 16.1	Synchronization procedures

### 16.1.1	Cell search

Cell search is the procedure by which a UE acquires time and frequency synchronization with a cell and detects the narrowband physical layer Cell ID.

If the higher layer parameter operationModeInfo indicates 'inband-SamePCI' or samePCI-Indicator indicates 'samePCI'' for a cell, the UE may assume that the physical layer cell ID is same as the narrowband physical layer cell ID for the cell.

The following signals are transmitted in the downlink to facilitate cell search for Narrowband IoT: the narrowband primary and narrowband secondary synchronization signals.

A UE may assume the antenna ports 2000 – 2001 and the antenna port for the narrowband primary/secondary synchronization signals of a serving cell are quasi co-located (as defined in [3]) with respect to Doppler shift and average delay.

### 16.1.2	Timing synchronization

Upon reception of a timing advance command, the UE shall adjust uplink transmission timing for NPUSCH, and SR if configured with higher layer parameter sr-WithoutHARQ-ACK-Config, based on the received timing advance command.

The timing advance command indicates the change of the uplink timing relative to the current uplink timing as multiples of 16. The start timing of the random access preamble is specified in [3].

In case of random access response, an 11-bit timing advance command [8], TA, indicates NTA values by index values of TA = 0, 1, 2, ..., 1536, where an amount of the time alignment is given by NTA = TA 16. NTA is defined in [3].

In other cases, a 6-bit timing advance command [8] or the Timing advance adjustment field in DCI format N0 if present [4], TA, indicates adjustment of the current NTA value, NTA,old, to the new NTA value, NTA,new, by index values of TA = 0, 1, 2,..., 63, where NTA,new = NTA,old + (TA 31)16. Here, adjustment of NTA value by a positive or a negative amount indicates advancing or delaying the uplink transmission timing by a given amount respectively.

For a timing advance command reception ending in DL subframe n, the corresponding adjustment of the uplink transmission timing shall apply from the first available NB-IoT uplink slot following the end of n+12+Koffset DL subframe and the first available NB-IoT uplink slot is the first slot of a NPUSCH transmission, where Koffset is provided by higher layers, otherwise Koffset=0. When the UE's uplink NPUSCH transmissions in NB-IoT uplink slot n and NB-IoT uplink slot n+1 are overlapped due to the timing adjustment, the UE shall complete transmission of NB-IoT uplink slot n and not transmit the overlapped part of NB-IoT uplink slot n+1.

If the received downlink timing changes and is not compensated or is only partly compensated by the uplink timing adjustment without timing advance command as specified in [10], the UE changes NTA accordingly.

## 16.2	Power control

### 16.2.1	Uplink power control

Uplink power control controls the transmit power of the different uplink physical channels.

#### 16.2.1.1	Narrowband physical uplink shared channel

##### 16.2.1.1.1	UE behaviour

The setting of the UE Transmit power for a Narrowband Physical Uplink Shared Channel (NPUSCH) transmission is defined as follows. For FDD, if the UE is capable of enhanced random access power control [12], and it is configured by higher layers, and for TDD,  enhanced random access power control shall be applied for a UE which started the random access procedure in the first or second configured NPRACH repetition level.

The UE transmit power  for NPUSCH transmission in NB-IoT UL slot i for the serving cell is given by:

For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH (re)transmission corresponding to preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2:

[dBm]

otherwise

[dBm]

where,

-	is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell .

-	MNPUSCH,c(i) is the NPUSCH transmission resource bandwidth normalized by 15 kHz, where {1/4} is used for 3.75 kHz subcarrier spacing and {1, 3, 6, 12} are used for 15kHz subcarrier spacing

-	PO_NPUSCH,cj is a parameter composed of the sum of a component  provided from higher layers and a component  provided by higher layers for j=1, 3 and for serving cell where . For NPUSCH (re)transmissions corresponding to a dynamic scheduled grant or a semi-persistent grant then j=1, for NPUSCH (re)transmissions corresponding to the random access response grant then j=2 and for NPUSCH transmission using preconfigured uplink resource then j=3. PO_UE_NPUSCH,c2=0. If enhanced random access power control is not applied, PO_NOMINAL_NPUSCH,c(2)=PO_PRE+∆PREAMBLE_Msg3, where the parameter preambleInitialReceivedTargetPower [8] () and ∆PREAMBLE_Msg3 are signalled from higher layers for serving cell . If enhanced random access power control is applied,

PO_NOMINAL_NPUSCH,c2=MSG3_RECEIVED_TARGET_POWER +ΔPREAMBLE_Msg3

-	For j=1, for NPUSCH format 2, =1; for NPUSCH format 1, is provided by higher layers for serving cell . For j=2,  For j=3,  is the parameter alpha in PUR-Config-NB provided by higher layers for serving cell .

-	 is the downlink path loss estimate calculated in the UE for serving cell  in dB and  = nrs-Power + nrs-PowerOffsetNonAnchor – NRSRP, where nrs-Power is provided by higher layers and Clause 16.2.2, and nrs-PowerOffsetNonAnchor is set to zero if it is not provided by higher layers and NRSRP is defined in [5] for serving cell .

##### 16.2.1.1.2	Power headroom

If the UE transmits NPUSCH in NB-IoT UL slot  for serving cell , power headroom is computed using

[dB]

where, , , , and, are defined in Clause 16.2.1.1.1.

The power headroom shall be rounded down to the closest value in the set [PH1, PH2, PH3, PH4] dB if enhanced PHR is not configured and [PH1, PH2, …, PH15, PH16] dB if enhanced PHR is configured as defined in [10]. The power headroom is delivered by the physical layer to higher layers.

#### 16.2.1.2	SR

##### 16.2.1.2.1	UE behaviour

If the UE is configured with higher layer parameter sr-WithoutHARQ-ACK-Config, the setting of the UE transmit power for SR transmission without HARQ-ACK is defined as follows.

The UE transmit power  for SR transmission in NB-IoT UL slot i for the serving cell is given by:

[dBm]

where,

-	is the configured UE transmit power defined in [6] in NB-IoT UL slot i for serving cell .

-	is {1/3} for NPRACH format 2 and {1}for NPRACH format 0/1.

-	is signaled from higher layers for serving cell .

-	 is signaled from higher layers for serving cell .

-	 is defined in Clause 16.2.1.1.1.

### 16.2.2	Downlink power allocation

The eNodeB determines the downlink transmit energy per resource element.

For an NB-IoT cell, the UE may assume NRS EPRE is constant across the downlink NB-IoT system bandwidth and constant across all subframes that contain NRS, until different NRS power information is received.

The downlink NRS EPRE can be derived from the downlink narrowband reference-signal transmit power given by nrs-Power + nrs-PowerOffsetNonAnchor, where the parameter nrs-Power is provided by higher layers and nrs-PowerOffsetNonAnchor is zero if it is not provided by higher layers. The downlink narrowband reference-signal transmit power is defined as the linear average over the power contributions (in [W]) of all resource elements that carry narrowband reference signals within the operating NB-IoT system bandwidth.

A UE may assume that the ratio of NWUS EPRE to NRS EPRE is 0 dB.

A UE may assume the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports.

A UE may assume the ratio of NPBCH EPRE to NRS EPRE among NPBCH REs (not applicable to NPBCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports.

A UE may assume the ratio of NPDCCH EPRE to NRS EPRE among NPDCCH REs (not applicable to NPDCCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports.

If higher layer parameter operationModeInfo indicates '00' or samePCI-Indicator indicates 'samePCI' for a cell, the ratio of NRS EPRE to CRS EPRE is given by the parameter nrs-CRS-PowerOffset if the parameter nrs-CRS-PowerOffset is provided by higher layers, and the ratio of NRS EPRE to CRS EPRE may be assumed to be 0 dB if the parameter nrs-CRS-PowerOffset is not provided by higher layers. If nrs-CRS-PowerOffset is provided by higher layers and is a non-integer value, the value of nrs-Power is 0.23 dBm higher than indicated.

## 16.3	Random access procedure

Prior to initiation of the non-synchronized physical random access procedure, Layer 1 shall receive the following information from the higher layers:

-	Narrowband Random access channel parameters (NPRACH configuration)

### 16.3.1	Physical non-synchronized random access procedure

From the physical layer perspective, the L1 random access procedure encompasses the transmission of narrowband random access preamble and narrowband random access response. The remaining messages are scheduled for transmission by the higher layer on the shared data channel and are not considered part of the L1 random access procedure. A random access channel occupies one subcarrier per set of consecutive symbols reserved for narrowband random access preamble transmissions.

The following steps are required for the L1 random access procedure:

-	Layer 1 procedure is triggered upon request of a narrowband preamble transmission by higher layers.

-	A target narrowband preamble received power (NARROWBAND_PREAMBLE_RECEIVED_TARGET_POWER), a corresponding RA-RNTI and a NPRACH resource are indicated by higher layers as part of the request.

-	If enhanced random access power control is not applied, for the lowest configured repetition level; and if enhanced random access power control is applied then for all configured repetition levels, a narrowband preamble transmission power PNPRACH is determined as 
PNPRACH = min{, NARROWBAND_PREAMBLE_RECEIVED_TARGET_POWER +  }_[dBm], where  is the configured UE transmit power for narrowband IoT transmission defined in [6] for subframe i of serving cell  and  is the downlink path loss estimate calculated in the UE for serving cell . If enhanced random access power control is not applied, for a repetition level other than the lowest configured repetition level, PNPRACH is set to .

-	The narrowband preamble is transmitted with transmission power PNPRACH commencing on the indicated NPRACH resource. The narrowband preamble is transmitted for the number of NPRACH repetitions for the associated NPRACH repetition level as indicated by higher layers.

- Detection of a NPDCCH with DCI scrambled by RA-RNTI is attempted during a window controlled by higher layers (see [8], Clause 5.1.4). If detected, the corresponding DL-SCH transport block is passed to higher layers. The higher layers parse the transport block and indicate the Nr-bit uplink grant to the physical layer, which is processed according to Clause 16.3.3

### 16.3.2	Timing

For the L1 random access procedure, UE's uplink transmission timing after a random access preamble transmission is as follows.

a)	If a NPDCCH with associated RA-RNTI is detected and the corresponding DL-SCH transport block ending in subframe n contains a response to the transmitted preamble sequence, the UE shall, according to the information in the response, transmit an UL-SCH transport block according to Clause 16.3.3.

b)	If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

c)	If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

d)	If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Koffset,  and  Koffset is provided by higher layers, otherwise Koffset=0, where a NPRACH resource is available.

The "PDCCH order" in DCI format N1 indicates to the UE,

-	allocated subcarrier for NPRACH, where  is the subcarrier indication field in the corresponding DCI, is reserved for preamble format 0/1, is reserved for preamble format 2 if nprach-ParametersListFmt2 is configured and the UE indicates the nprach-Format2 capability and Preamble format indicator is set to 1.

-	a repetition number () for NPRACH determined by the repetition number field () in the corresponding DCI according to Table 16.3.2-1 where R1, R2 (if any) and R3 (if any) are given by the higher layer parameter numRepetitionsPerPreambleAttempt for each NPRACH resource, respectively. R1 < R2 <R3.

Table 16.3.2-1: Number of repetitions () for NPRACH following a "PDCCH order"

| 0 | R1 |
| --- | --- |
| 1 | R2 |
| 2 | R3 |
| 3 | Reserved |

The UE shall transmit random access preamble on the NB-IoT carrier indicated by "Carrier indication of NPRACH" field, if the field is present in the "PDCCH order". If the value of "Carrier indication of NPRACH" is non-zero, it indicates a NPRACH carrier derived from SystemInformationBlockType22-NB [11] for which the index in the list is equal to the carrier indication. If the value of "Carrier indication of NPRACH" is zero, the uplink carrier used for NPRACH is derived from SystemInformationBlockType2-NB [11].

If nprach-ParametersListFmt2 is configured and the UE indicates the nprach-Format2 capability, the UE shall transmit the preamble format indicated by "Preamble format indicator" field, otherwise the UE shall transmit preamble format 0/1.

### 16.3.3	Narrowband random access response grant

The higher layers indicate the Nr-bit UL Grant to the physical layer, as defined in 3GPP TS 36.321 [8]. 
This is referred to as the Narrowband Random Access Response Grant in the physical layer.

Nr-bit =15, and the content of these 15 bits starting with the MSB and ending with the LSB are as follows:

-	Uplink subcarrier spacing  is '0'=3.75 kHz or '1'=15 kHz – 1 bit

-	Subcarrier indication field  as determined in Clause 16.5.1.1 – 6 bits

-	Scheduling delay field () as determined in Clause 16.5.1 with k0 = 12 for IDelay = 0 , where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the Narrowband Random Access Response Grant is transmitted – 2 bits

-	Msg3 repetition number  as determined in Clause 16.5.1.1 – 3 bits

-	MCS index indicating TBS, modulation, and number of RUs for Msg3 – 3 bits

The redundancy version for the first transmission of Msg3 is 0.

If the UE is not using higher layer parameter edt-Parameters, or the UE is using higher layer parameter edt-parameters and ,

-	the TBS, modulation, and number of RUs for Msg3 are determined according to Table 16.3.3-1

otherwise,

-	if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to 'false',

-	the TBS is given by higher layer parameter edt-TBS

-	otherwise,

- the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2

- the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where  is the selected TBS for Msg3, and  is given by higher layer parameter edt-TBS

-	if  and  and , then is used in clause 16.5.1.2, otherwise is used

-	the number of RUs for Msg3 are determined according to Table 16.3.3-3

- π/4 QPSK modulation is used for  and for  with ; QPSK modulation is used for with

Table 16.3.3-1: MCS index for Msg3 NPUSCH

| MCS Index | Modulation or and | Modulation and | Number of RUs | TBS |
| --- | --- | --- | --- | --- |
| '000' | pi/2 BPSK | QPSK | 4 | 88 bits |
| '001' | pi/4 QPSK | QPSK | 3 | 88 bits |
| '010' | pi/4 QPSK | QPSK | 1 | 88 bits |
| '011' | reserved | reserved | reserved | reserved |
| '100' | reserved | reserved | reserved | reserved |
| '101' | reserved | reserved | reserved | reserved |
| '110' | reserved | reserved | reserved | reserved |
| '111' | reserved | reserved | reserved | reserved |

Table 16.3.3-2: EDT TBS for Msg3 NPUSCH with edt-SmallTBS-Enabled set to 'true'

| edt-TBS | edt-SmallTBS-Subset | Allowable TBS values |
| --- | --- | --- |
| 408 | not configured | 328, 408 |
| 504 | not configured | 328, 408, 504 |
| 504 | enabled | 408, 504 |
| 584 | not configured | 328, 408, 504, 584 |
| 584 | enabled | 408, 584 |
| 680 | not configured | 328, 456, 584, 680 |
| 680 | enabled | 456, 680 |
| 808 | not configured | 328, 504, 680, 808 |
| 808 | enabled | 504, 808 |
| 936 | not configured | 328, 504, 712, 936 |
| 936 | enabled | 504, 936 |
| 1000 | not configured | 328, 536, 776, 1000 |
| 1000 | enabled | 536, 1000 |

Table 16.3.3-3: MCS index for Msg3 NPUSCH and EDT

| MCS Index | Number of RUs |  |  |
| --- | --- | --- | --- |
|  | edt-TBS = 328, 408, 504, or 584 | edt-TBS = 680 | edt-TBS = 808, 936, or 1000 |
| '011' | 3 | 3 | 4 |
| '100' | 4 | 4 | 5 |
| '101' | 5 | 5 | 6 |
| '110' | 6 | 8 | 8 |
| '111' | 8 | 10 | 10 |

## 16.4	Narrowband physical downlink shared channel related procedures

A NB-IoT UE shall determine whether a downlink subframe or a TDD special subframe configured for NB-IoT DL transmission is a NB-IoT DL subframe as follows

-	If the UE determines that the subframe contains NPSS/NSSS/NPBCH/ SystemInformationBlockType1-NB transmission, then the subframe is not assumed as a NB-IoT subframe.

-	Else if higher layer parameter resourceReservationConfigDL is configured

-	for NPDSCH transmission associated with C-RNTI using UE-specific NPDCCH search space

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT DL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols are reserved in the subframe).

-	for NPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space

-	the subframe is assumed as a NB-IoT DL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols are reserved in the subframe).

-	In all other cases, a NB-IoT UE shall assume a subframe as a NB-IoT DL subframe if

-	for a NB-IoT carrier that a UE receives higher layer parameter operationModeInfo, the subframe is configured as NB-IoT DL subframe or the subframe is a TDD special subframe configured for NB-IoT DL transmission after the UE has obtained SystemInformationBlockType1-NB.

-	the subframe is configured as NB-IoT DL subframe by the higher layer parameter downlinkBitmapNonAnchor.

-	except when the UE is configured with higher layer parameter additionalTxSIB1-Config set to TRUE, subframe #3 not containing additional SystemInformationBlockType1-NB transmission is assumed as a NB-IoT DL subframe if the UE monitors a NPDCCH UE-specific search space or decodes NPDSCH transmission scheduled by NPDCCH in the UE-specific search space.

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npdsch-MultiTB-Config, there shall be a maximum of 2 downlink HARQ processes.

### 16.4.1	UE procedure for receiving the narrowband physical downlink shared channel

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N1, N2 ending in subframe n intended for the UE, decode, starting in

-	n+5 DL subframe for FDD,

-	n+5 subframe for TDD,

the corresponding NPDSCH transmission in N consecutive NB-IoT DL subframe(s) ni with i = 0, 1, …, N-1 according to the NPDCCH information, where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI;

-	subframe(s) ni with i=0,1,…,N-1 are N consecutive NB-IoT DL subframe(s) excluding subframes used for SI messages or scheduling gap (if any) or processing gap (if any) where, n0<n1<…,nN-1 ,

-	, where the value of  is determined by the repetition number field in the corresponding DCI (see Clause 16.4.1.3), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.4.1.3), and the value of is determined by the Number of scheduled TB for Unicast field or Number of scheduled TB for SC-MTCH field, if present, in the corresponding DCI,  otherwise,

-	k0 is the number of NB-IoT DL subframe(s) starting in DL subframe n+5 for FDD or subframe n+5 for TDD, until DL subframe n0, where k0 is determined by the scheduling delay field () for DCI format N1, and k0 = 0 for DCI format N2. For DCI CRC scrambled by G-RNTI, k0 is determined by the scheduling delay field () according to Table 16.4.1-1a, otherwise k0 is determined by the scheduling delay field () according to Table 16.4.1-1. The value of is according to Clause 16.6 for the corresponding DCI format N1,

-	for ,

-	if the UE is configured with higher layer parameter multiTB-Config in npdsch-MultiTB-Config set to 'interleaved', and NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT DL subframes  with  are associated with TBr+1 ,

-	for  and NPDSCH corresponding to an NPDCCH with DCI CRC scrambled by G-RNTI,

-	if multiTB-Gap is not configured and , a processing gap of 20ms is inserted after every 2 TBs

-	otherwise, a scheduling gap with a length equal to the indicated value of multiTB-Gap is inserted between TBr and TBr+1, .

-	If the scheduling gap or the processing gap overlaps with the NPDSCH transmission gap defined in [3], the overlapped part of the scheduling gap or processing gap is also counted as the part of NPDSCH transmission gap.

Table 16.4.1-1: for DCI format N1.

| 0 | 0 | 0 |
| --- | --- | --- |
| 1 | 4 | 16 |
| 2 | 8 | 32 |
| 3 | 12 | 64 |
| 4 | 16 | 128 |
| 5 | 32 | 256 |
| 6 | 64 | 512 |
| 7 | 128 | 1024 |

Table 16.4.1-1a: for DCI format N1 with DCI CRC scrambled by G-RNTI.

| 0 | 0 |
| --- | --- |
| 1 | 4 |
| 2 | 8 |
| 3 | 12 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

If a UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	for FDD, the UE is not expected to receive transmissions in the Type B half duplex guard periods as specified in [3]

otherwise

-	for FDD, the UE is not expected to receive transmissions in 3 DL subframes following the end of a NPUSCH transmission by the UE.

-	for TDD, the UE is not expected to receive transmissions in 3 subframes following the end of a NPUSCH transmission by the UE.

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the P-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-2. 
The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by P-RNTI.

Table 16.4.1-2: NPDCCH and NPDSCH configured by P-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N2 | Type-1 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the RA-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-3. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by RA-RNTI.

Table 16.4.1-3: NPDCCH and NPDSCH configured by RA-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | Type-2 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the C-RNTI except during random access procedure, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-4. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by C-RNTI.

Table 16.4.1-4: NPDCCH and NPDSCH configured by C-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | UE specific by C-RNTI | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the Temporary C-RNTI and is not configured to decode NPDCCH with CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH and the corresponding NPDSCH according to the combination defined in Table 16.4.1-5. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by Temporary C-RNTI.

If a UE is also configured by higher layers to decode NPDCCH with CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH and the corresponding NPDSCH according to the combination defined in Table 16.4.1-5. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by C-RNTI.

Table 16.4.1-5: NPDCCH and NPDSCH configured by Temporary C-RNTI and/or C-RNTI during random access procedure

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | Type-2 Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

For NPDSCH carrying SystemInformationBlockType1-NB and SI-messages, the UE shall decode NPDSCH according to the transmission scheme defined in Table 16.4.1-6. The scrambling initialization of NPDSCH is by SI-RNTI.

Table 16.4.1-6: NPDSCH configured by SI-RNTI

Transmission scheme of NPDSCH

If the number of NPBCH antenna ports is one, Single-antenna port, port 0 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2).

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the SC-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-7. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by SC-RNTI.

Table 16.4.1-7: NPDCCH and NPDSCH configured by SC-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N2 | Type-1A Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the G-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combinations defined in Table 16.4.1-8. The scrambling initialization of NPDSCH corresponding to these NPDCCHs is by G-RNTI.

Table 16.4.1-8: NPDCCH and NPDSCH configured by G-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | Type-2A Common | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

If a UE is configured by higher layers to decode NPDCCH with CRC scrambled by the PUR-RNTI, the UE shall decode the NPDCCH and the corresponding NPDSCH according to any of the combination defined in Table 16.4.1-9. The scrambling initialization of the NPDSCH corresponding to these NPDCCHs is by PUR-RNTI.

Table 16.4.1-9: NPDCCH and NPDSCH configured by PUR-RNTI

| DCI format | Search Space | Transmission scheme of NPDSCH corresponding to NPDCCH |
| --- | --- | --- |
| DCI format N1 | UE specific by PUR-RNTI | If the number of NPBCH antenna ports is one, Single-antenna port, port 2000 is used (see Clause 16.4.1.1), otherwise Transmit diversity (see Clause 16.4.1.2). |

A UE is not required to receive NPDSCH assigned by NPDCCH with DCI CRC scrambled by G-RNTI in subframes in which the UE monitors a Type1A-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI

A UE is not required to receive NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI or G-RNTI in subframes in which the UE monitors a Type1-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by P-RNTI

A UE is not required to receive NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI or G-RNTI in subframes in which the UE monitors a Type2-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by C-RNTI or Temporary C-RNTI.

The transmission schemes for NPDSCH are defined in the following Clauses.

#### 16.4.1.1	Single-antenna port scheme

For the single-antenna port transmission schemes (port 2000) of the NPDSCH, the UE may assume that an eNB transmission on the NPDSCH would be performed according to Clause 6.3.4.1 of [3].

#### 16.4.1.2	Transmit diversity scheme

For the transmit diversity transmission scheme of the NPDSCH, the UE may assume that an eNB transmission on the NPDSCH would be performed according to Clause 6.3.4.3 of [3]

#### 16.4.1.3	Resource allocation

The resource allocation information in DCI format N1, N2 (paging) for NPDSCH indicates to a scheduled UE

-	a number of subframes () determined by the resource assignment field () in the corresponding DCI according to Table 16.4.1.3-1.

-	a repetition number () determined by the repetition number field () in the corresponding DCI according to Table 16.4.1.3-2.

Table 16.4.1.3-1: Number of subframes () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.4.1.3-2: Number of repetitions () for NPDSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |
| 8 | 192 |
| 9 | 256 |
| 10 | 384 |
| 11 | 512 |
| 12 | 768 |
| 13 | 1024 |
| 14 | 1536 |
| 15 | 2048 |

For FDD, the number of repetitions for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-3.

Table 16.4.1.3-3: Number of repetitions for NPDSCH carrying SystemInformationBlockType1-NB, FDD.

| Value of schedulingInfoSIB1 | Number of NPDSCH repetitions |
| --- | --- |
| 0 | 4 |
| 1 | 8 |
| 2 | 16 |
| 3 | 4 |
| 4 | 8 |
| 5 | 16 |
| 6 | 4 |
| 7 | 8 |
| 8 | 16 |
| 9 | 4 |
| 10 | 8 |
| 11 | 16 |
| 12-15 | Reserved |

For FDD, the starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB is determined according to Table 16.4.1.3-4.

Table 16.4.1.3-4: Starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB, FDD.

| Number of NPDSCH repetitions |  | Starting radio frame number for SystemInformationBlockType1-NB repetitions (nf mod 256) |
| --- | --- | --- |
| 4 | mod 4 = 0 | 0 |
|  | mod 4 = 1 | 16 |
|  | mod 4 = 2 | 32 |
|  | mod 4 = 3 | 48 |
| 8 | mod 2 = 0 | 0 |
|  | mod 2 = 1 | 16 |
| 16 | mod 2 = 0 | 0 |
|  | mod 2 = 1 | 1 |

For the TDD NB-IoT carrier on which NPSS/NSSS/NPBCH are detected, the number of repetitions and subframe index for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-5.

Table 16.4.1.3-5: Number of repetitions and subframe index for NPDSCH carrying SystemInformationBlockType1-NB, TDD.

| Value of schedulingInfoSIB1 | Number of NPDSCH repetitions | Subframe index |
| --- | --- | --- |
| 0 | 4 | 0 |
| 1 | 8 | 0 |
| 2 | 16 | 0 |
| 3 | 4 | 0 |
| 4 | 8 | 0 |
| 5 | 16 | 0 |
| 6 | 4 | 0 |
| 7 | 8 | 0 |
| 8 | 16 | 0 |
| 9 | 4 | 0 |
| 10 | 8 | 0 |
| 11 | 16 | 0 |
| 12-15 | 16 | 4 |

For the TDD NB-IoT carrier on which NPSS/NSSS/NPBCH are detected, the starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB is determined according to Table 16.4.1.3-6.

Table 16.4.1.3-6: Starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB, TDD.

| Subframe index | Number of NPDSCH repetitions |  | Starting radio frame number for SystemInformationBlockType1-NB repetitions (nf mod 256) |
| --- | --- | --- | --- |
| 0 | 4 | mod 4 = 0 | 1 |
|  |  | mod 4 = 1 | 17 |
|  |  | mod 4 = 2 | 33 |
|  |  | mod 4 = 3 | 49 |
| 0 | 8 | mod 2 = 0 | 1 |
|  |  | mod 2 = 1 | 17 |
| 0 | 16 | Any | nf mod 256 = 1 |
| 4 | 16 | mod 2 = 0 | nf mod 256 = 0 |
|  |  | mod 2 = 1 | nf mod 256 = 1 |

For a higher layer configured TDD NB-IoT carrier, the number of repetitions and subframe index for the NPDSCH carrying SystemInformationBlockType1-NB is determined based on the parameter schedulingInfoSIB1 configured by higher-layers and according to Table 16.4.1.3-7.

Table 16.4.1.3-7: Number of repetitions and subframe index for NPDSCH carrying SystemInformationBlockType1-NB, TDD.

| Value of schedulingInfoSIB1 | Number of NPDSCH repetitions | Subframe index |
| --- | --- | --- |
| 0 | 8 | 0, 5 |
| 1 | 16 | 0, 5 |
| 2 | 8 | 0, 5 |
| 3 | 16 | 0, 5 |
| 4 | 8 | 0, 5 |
| 5 | 16 | 0, 5 |
| 6 | 8 | 0, 5 |
| 7 | 16 | 0, 5 |

For a higher layer configured TDD NB-IoT carrier, the starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB is determined according to Table 16.4.1.3-8.

Table 16.4.1.3-8: Starting radio frame for the first transmission of the NPDSCH carrying SystemInformationBlockType1-NB, TDD.

| Number of NPDSCH repetitions |  | Starting radio frame number for SystemInformationBlockType1-NB repetitions (nf mod 256) |
| --- | --- | --- |
| 8 | mod 2 = 0 | 0 |
|  | mod 2 = 1 | 16 |
| 16 | mod 2 = 0 | 0 |
|  | mod 2 = 1 | 1 |

#### 16.4.1.4	NPDSCH starting position

The starting OFDM symbol for NPDSCH is given by index  in the first slot in a subframe  and is determined as follows

-	if subframe  is a subframe used for receiving SIB1-NB

-	if the value of the higher layer parameter operationModeInfo is set to '00' or '01'

-	 if the value of the higher layer parameter operationModeInfo is set to '10' and the value of the higher layer parameter sib-GuardbandInfo is set to '10' or '11' for TDD

-	otherwise

-	elseif subframe  is a special subframe for NPDSCH without repetition

-	 where is given by the higher layer parameter eutraControlRegionSize if the value of the higher layer parameter eutraControlRegionSize is present

-	otherwise

-	else

-	is given by the higher layer parameter eutraControlRegionSize if the value of the higher layer parameter eutraControlRegionSize is present

-	otherwise

#### 16.4.1.5	Modulation order and transport block size determination

The UE shall use modulation order, = 2.

To determine the transport block size in the NPDSCH, the UE shall first,

-	if NPDSCH carriers SystemInformationBlockType1-NB

-	set  to the value of the parameter schedulingInfoSIB1 configured by higher-layers

-	otherwise

-	read the 4-bit "modulation and coding scheme" field () in the DCI and set .

and second,

-	if NPDSCH carriers SystemInformationBlockType1-NB

-	use Clause 16.4.1.5.2 for determining its transport block size.

-	otherwise,

-	read the 3-bit "resource assignment" field () in the DCI and determine its TBS by the procedure in Clause 16.4.1.5.1.

For a NPDCCH UE-specific search space, if the UE is configured with higher layer parameter twoHARQ-ProcessesConfig, or the UE is configured with higher layer parameter npdsch-MultiTB-Config and single TB is scheduled in the corresponding DCI

-	the NDI and HARQ process ID as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers,

otherwise

-	the NDI as signalled on NPDCCH, and the TBS, as determined above, shall be delivered to higher layers. If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.

##### 16.4.1.5.1	Transport blocks not mapped for SystemInformationBlockType1-NB

The TBS is given by the (,) entry of Table 16.4.1.5.1-1. If for the carrier on which NPSS/NSSS/NPBCH are detected the value of the higher layer parameter operationModeInfo is set to '00' or '01', or if the value of the higher layer parameter inbandCarrierInfo-r13 is configured for a higher layer configured carrier if any, .

Table 16.4.1.5.1-1: Transport block size (TBS) table.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

##### 16.4.1.5.2	Transport blocks mapped for SystemInformationBlockType1-NB

The TBS is given by theentry of Table 16.4.1.5.2-1 for FDD, and Table 16.4.1.5.2-2 for TDD NB-IoT carrier on which NPSS/NSSS/NPBCH are detected and Table 16.4.1.5.2-3 for a higher layer configured TDD NB-IoT carrier.

Table 16.4.1.5.2-1: Transport block size (TBS) table for NPDSCH carrying SystemInformationBlockType1-NB, FDD

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 208 | 208 | 208 | 328 | 328 | 328 | 440 | 440 | 440 | 680 | 680 | 680 | Reserved |  |  |  |

Table 16.4.1.5.2-2: Transport block size (TBS) table for NPDSCH carrying SystemInformationBlockType1-NB, TDD

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 208 | 208 | 208 | 328 | 328 | 328 | 440 | 440 | 440 | 680 | 680 | 680 | 208 | 328 | 440 | 680 |

Table 16.4.1.5.2-3: Transport block size (TBS) table for NPDSCH carrying SystemInformationBlockType1-NB, TDD

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 208 | 208 | 328 | 328 | 440 | 440 | 680 | 680 |

### 16.4.2	UE procedure for reporting ACK/NACK

The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of

-	DL subframe for FDD,

-	 NB-IoT UL subframes following the end of n+12+Koffset subframe for TDD,

transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where

-	, where

-	the value of is given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise,

-	the value of  is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and

-	if the UE is configured with higher layer parameter harq-ACK-Bundling in npdsch-MultiTB-Config, then , otherwise , where the value of is determined by the Number of scheduled TB for Unicast field if present in the NPDCCH corresponding to the NPDSCH, otherwise ,

-	allocated subcarrier for ACK/NACK and value of k0 is determined by the ACK/NACK resource field in the DCI format of the corresponding NPDCCH according to Table 16.4.2-1, and Table 16.4.2-2,

-	for FDD, .

-	for TDD, .

-	For

-	if the UE is configured with higher layer parameter harq-AckBundling in npdsch-MultiTB-Config, and the NPDSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI,

-	the ACK/NACK response is generated by performing a logical AND operation of HARQ-ACKs corresponding to the TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  of the NPUSCH carry ACK/NACK response for TBr+1 ,

- Koffset is provided by higher layers, otherwise Koffset=0

Table 16.4.2-1: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 38 | 13 |
| 1 | 39 | 13 |
| 2 | 40 | 13 |
| 3 | 41 | 13 |
| 4 | 42 | 13 |
| 5 | 43 | 13 |
| 6 | 44 | 13 |
| 7 | 45 | 13 |
| 8 | 38 | 21 |
| 9 | 39 | 21 |
| 10 | 40 | 21 |
| 11 | 41 | 21 |
| 12 | 42 | 21 |
| 13 | 43 | 21 |
| 14 | 44 | 21 |
| 15 | 45 | 21 |

Table 16.4.2-2: ACK/NACK subcarrier and for NPUSCH with subcarrier spacing .

| ACK/NACK resource field | ACK/NACK subcarrier |  |
| --- | --- | --- |
| 0 | 0 | 13 |
| 1 | 1 | 13 |
| 2 | 2 | 13 |
| 3 | 3 | 13 |
| 4 | 0 | 15 |
| 5 | 1 | 15 |
| 6 | 2 | 15 |
| 7 | 3 | 15 |
| 8 | 0 | 17 |
| 9 | 1 | 17 |
| 10 | 2 | 17 |
| 11 | 3 | 17 |
| 12 | 0 | 18 |
| 13 | 1 | 18 |
| 14 | 2 | 18 |
| 15 | 3 | 18 |

## 16.5	Narrowband physical uplink shared channel related procedures

For a NB-IoT UE that supports twoHARQ-Processes-r14 or the UE is configured with higher layer parameter npusch-MultiTB-Config, there shall be a maximum of 2 uplink HARQ processes.

For a NB-IoT UE and NPUSCH transmission using preconfigured uplink resource, there shall be 1 uplink HARQ process.

A NB-IoT UE shall determine whether a subframe is a NB-IoT UL subframe as follows

-	If higher layer parameter resourceReservationConfigUL is configured

-	for NPUSCH format 1 transmission associated with C-RNTI or SPS C-RNTI using UE-specific NPDCCH search space including NPUSCH format 1 transmission without a corresponding NPDCCH

-	if the Resource reservation field in the DCI is set to 0, then the subframe is assumed as a NB-IoT UL subframe

-	else if the Resource reservation field in the DCI is set to 1, then the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe).

-	for NPUSCH format 2 transmission

-	the subframe is assumed as a NB-IoT UL subframe if it is not fully reserved according to the higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols are reserved in the subframe).

-	In all other cases,

-	for TDD, a NB-IoT UE shall assume a subframe as a NB-IoT UL subframe if, for a NB-IoT carrier, it is configured as NB-IoT UL subframe by higher layers

-	for FDD, a NB-IoT UE shall always assume a subframe as a NB-IoT UL subframe.

### 16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel

NPUSCH format 1 transmission can be scheduled by a NPDCCH with DCI format N0, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14] , while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a NPDCCH with DCI format N0.

A UE shall upon detection on a given serving cell of a NPDCCH with DCI format N0 ending in NB-IoT DL subframe n scheduling NPUSCH intended for the UE, perform, at the end of

-	n+k0+Koffset DL subframe for FDD,

-	k0 NB-IoT UL subframes following the end of n+8+Koffset subframe for TDD,

a corresponding NPUSCH transmission using NPUSCH format 1 in N consecutive NB-IoT UL slots ni with i = 0, 1, …, N-1 according to the NPDCCH information where

-	subframe n is the last subframe in which the NPDCCH is transmitted and is determined from the starting subframe of NPDCCH transmission and the DCI subframe repetition number field in the corresponding DCI; and

-	, where the value of is determined by the repetition number field in the corresponding DCI (see Clause 16.5.1.1), the value of is determined by the resource assignment field in the corresponding DCI (see Clause 16.5.1.1), the value of  is the number of NB-IoT UL slots of the resource unit (defined in clause 10.1.2.3 of [3]) corresponding to the  allocated number of subcarriers (as determined in Clause 16.5.1.1) in the corresponding DCI, and the value of is determined by the Number of scheduled TB for Unicast field, if present, in the corresponding DCI,  otherwise

-	n0 is the first NB-IoT UL slot starting after the end of subframe n+k0 for FDD

-	n0 is the first NB-IoT UL slot starting after k0 NB-IoT UL subframes following the end of n+8 subframe for TDD

-	value of k0 is determined by the scheduling delay field () in the corresponding DCI according to Table 16.5.1-1 for FDD and Table 16.5.1-1A for TDD

-	Koffset is provided by higher layers, otherwise Koffset=0

-	For ,

-	if the UE is configured with higher layer parameter npusch-MultiTB-Config set to 'interleaved', and NPUSCH corresponding to a NPDCCH with DCI CRC scrambled by C-RNTI, and  where  for ,  otherwise.

-	NB-IoT UL slots  with  are associated with TBr+1 ,

-	otherwise,

-	NB-IoT UL slots  with  are associated with TBr+1 ,

Table 16.5.1-1: for DCI format N0 for FDD.

| 0 | 8 |
| --- | --- |
| 1 | 16 |
| 2 | 32 |
| 3 | 64 |

Table 16.5.1-1A: for DCI format N0 for TDD.

| 0 | 0 |
| --- | --- |
| 1 | 8 |
| 2 | 16 |
| 3 | 32 |

If a NPUSCH transmission without a corresponding NPDCCH collides partially or fully with a NPDSCH transmission, the NPUSCH transmission is dropped.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-2 and transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by C-RNTI.

Table 16.5.1-2: NPDCCH and NPUSCH configured by C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by C-RNTI |

If a UE is configured to receive random access procedures initiated by "PDCCH orders", the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-3.

Table 16.5.1-3: NPDCCH configured as "PDCCH order" to initiate random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format N1 | UE specific by C-RNTI |

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the Temporary C-RNTI regardless of whether UE is configured or not configured to decode NPDCCH with the CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-4 and transmit the corresponding NPUSCH. The scrambling initialization of NPUSCH corresponding to these NPDCCHs is by Temporary C-RNTI.

If a Temporary C-RNTI is set by higher layers, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by Temporary C-RNTI. Otherwise, the scrambling initialization of NPUSCH corresponding to the Narrowband Random Access Response Grant in Clause 16.3.3 and any NPUSCH retransmission(s) for the same transport block is by C-RNTI.

If a UE is also configured by higher layers to decode NPDCCH with CRC scrambled by the C-RNTI during random access procedure, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-4 and transmit the corresponding NPUSCH. The scrambling initialization of NPUSCH corresponding to these NPDCCH is by C-RNTI.

Table 16.5.1-4: NPDCCH and NPUSCH configured by Temporary C-RNTI and/or C-RNTI during random access procedure

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | Type-2 Common |

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-5 and transmit a corresponding NPUSCH if a transport block corresponding to the HARQ process of the NPUSCH transmission is generated as described in [8]. 
The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and NPUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this NPUSCH without a corresponding NPDCCH and the NPUSCH retransmission for the same transport block is by SPS C-RNTI.

Table 16.5.1-5: NPDCCH and NPUSCH configured by SPS C-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by C-RNTI |

A UE may transmit NPUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of NPUSCH transmission using preconfigured uplink resource is by PUR-RNTI.

If a UE is configured by higher layers to decode NPDCCHs with the CRC scrambled by the PUR-RNTI, the UE shall decode the NPDCCH according to the combination defined in Table 16.5.1-6 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding NPUSCH. The scrambling initialization of this NPUSCH corresponding to these NPDCCHs and the NPUSCH retransmission for the same transport block is by PUR-RNTI.

Table 16.5.1-6: NPDCCH and NPUSCH configured by PUR-RNTI

| DCI format | Search Space |
| --- | --- |
| DCI format N0 | UE specific by PUR-RNTI |

#### 16.5.1.1	Resource allocation

The resource allocation information in uplink DCI format N0 for NPUSCH transmission or configured by higher layers for NPUSCH transmission using preconfigured uplink resource indicates to a scheduled UE

- a set of contiguously allocated subcarriers () of a resource unit determined by the Subcarrier indication field, or by the higher layer parameter npusch-SubCarrierSetIndex in PUR-Config-NB

- a number of resource units () determined by the resource assignment field according to Table 16.5.1.1-2, or by the higher layer parameter npusch-NumRUsIndex in PUR-Config-NB

- a repetition number () determined by the repetition number field according to Table 16.5.1.1-3. For a NPUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number configured by higher layers.

The subcarrier spacing  of NPUSCH transmission is determined by

-	the higher layer parameter npusch-SubCarrierSetIndex, in the case of NPUSCH transmission using preconfigured uplink resources and subsequent NPUSCH transmissions until a Narrowband Random Access Response Grant is received,

-	the uplink subcarrier spacing field in the Narrowband Random Access Response Grant according to Clause 16.3.3 otherwise.

For NPUSCH transmission with subcarrier spacing, where  is the subcarrier indication field and is reserved, or nsc is configured by higher layers parameter npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources.

For NPUSCH transmission with subcarrier spacing, the subcarrier indication field () in the DCI or npusch-SubCarrierSetIndex in PUR-Config-NB for NPUSCH transmissions using preconfigured uplink resources determines the set of contiguously allocated subcarriers () according to Table 16.5.1.1-1.

Table 16.5.1.1-1: Allocated subcarriers for NPUSCH with .

| Subcarrier indication field () | Set of Allocated subcarriers () |
| --- | --- |
| 0 – 11 |  |
| 12-15 |  |
| 16-17 |  |
| 18 |  |
| 19-63 | Reserved |

Table 16.5.1.1-2: Number of resource units () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 3 |
| 3 | 4 |
| 4 | 5 |
| 5 | 6 |
| 6 | 8 |
| 7 | 10 |

Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

#### 16.5.1.2	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the NPUSCH, the UE shall first

-	read the "modulation and coding scheme" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and

-	read the "redundancy version" field () in the DCI or initiate with rvDCI=0 for NPUSCH transmission using preconfigured uplink resource, and

-	read the "resource assignment" field () in the DCI or configured by higher layers for NPUSCH transmission using preconfigured uplink resource, and

-	compute the total number of allocated subcarriers (), number of resource units (), and repetition number () according to Clause 16.5.1.1.

If the UE is configured with higher layer parameter edt-Parameters and the most recent NPUSCH transmission including a transport block with EDT, the UE is not expected to receive a DCI indicating a NPUSCH retransmission as part of the contention based random access procedure with 3 ≤ IMCS ≤ 14.

If the UE is configured with higher layer parameter edt-Parameters, and for a NPUSCH retransmission of the same transport block including EDT as part of the contention based random access procedure with  in the DCI,

-	the modulation order is set to .

-	if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to 'true', the repetition number for the NPUSCH retransmission is the smallest integer multiple of  value that is equal to or larger than where  is the TBS corresponding to the NPUSCH transmission scheduled by the Narrowband Random Access Response Grant, and  is given by the higher layer parameter edt-TBS.

elseif the UE is configured with higher layer parameter edt-Parameters, and if the DCI indicates a retransmission as part of the contention based random access procedure with  and the most recent NPUSCH transmission including a transport block with EDT,

-	the TBS and modulation are determined according to Table 16.3.3-1 in Clause 16.3.3, for  and the transport block does not include EDT

otherwise, the UE shall use modulation order, = 2 if . The UE shall useand Table 16.5.1.2-1 to determine the modulation order to use for NPUSCH if .

Table 16.5.1.2-1: Modulation and TBS index table for NPUSCH with .

| MCS Index | Modulation Order | TBS Index |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 2 |
| 2 | 2 | 1 |
| 3 | 2 | 3 |
| 4 | 2 | 4 |
| 5 | 2 | 5 |
| 6 | 2 | 6 |
| 7 | 2 | 7 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |

If the UE is configured with higher layer parameter npusch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI,  is used for each TB.

The NPUSCH associated with a TB is transmitted in N NB-IoT UL slots associated with the TB, ni , i=0,1,…,N-1. For the NPUSCH transmission in jth block of B consecutive NB-IoT UL slots associated with the TB ni ,, the redundancy version  associated with the TB is determined by, , where  if ,  otherwise. Portion of NPUSCH codeword with  associated with a TB as defined in clause 6.3.2 in [4] mapped to slot  of allocated  resource unit(s) is transmitted in NB-IoT UL slots associated with the TB ni for and  for

The UE shall use (,) and Table 16.5.1.2-2 to determine the TBS to use for the NPUSCH. is given in Table 16.5.1.2-1 if ,  otherwise.

Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

For a NPDCCH UE-specific search space, if the UE is configured with higher layer parameter twoHARQ-ProcessesConfig, or the UE is configured with higher layer parameter npusch-MultiTB-Config and single TB is scheduled in the corresponding DCI

-	the NDI and HARQ process ID as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers,

otherwise

-	the NDI as signalled on NPDCCH, and the RV and TBS, as determined above, shall be delivered to higher layers. If the UE is configured with higher layer parameter npusch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, HARQ process ID of 0 shall be assumed for the first TB and HARQ process ID of 1 shall be assumed for the second TB.

### 16.5.2	UE procedure for NPUSCH retransmission

For a NPUSCH retransmission, the UE shall follow the HARQ information in DCI as specified in [8].

### 16.5.3	UE procedure for transmitting SR

If the UE is configured with higher layer parameter sr-WithoutHARQ-ACK-Config, the UE is configured with Narrowband Random access channel parameters (NPRACH configuration) for SR transmission by higher layers.

The UE shall, if requested by higher layers for transmitting SR, start transmission of a narrowband random access preamble on the NB-IoT carrier configured in sr-NPRACH-Resource at the next available NPRACH resource, unless the transmission would overlap with any subframe(s) of NPDSCH reception. The narrowband preamble is transmitted on the allocated subcarrier and a number of NPRACH repetitions for the associated NPRACH repetition level as indicated by higher layers. The narrowband random access preamble is transmitted with transmission power as determined in clause 16.2.1.2, commencing on the indicated NPRACH resource.

## 16.6	Narrowband physical downlink control channel related procedures

A UE shall monitor a set of NPDCCH candidates (described in Clause 10.2.5.1 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the NPDCCHs in the set according to all the monitored DCI formats.

The set of NPDCCH candidates to monitor are defined in terms of NPDCCH search spaces.

The UE shall monitor one or more of the following search spaces

-	a Type1-NPDCCH common search space,

-	a Type1A-NPDCCH common search space,

-	a Type2-NPDCCH common search space,

-	a Type2A-NPDCCH common search space, and

-	a NPDCCH UE-specific search space.

A UE is not required to simultaneously monitor a NPDCCH UE-specific search space and a Type-1-NPDCCH common search space.

A UE is not required to simultaneously monitor a NPDCCH UE-specific search space and a Type2-NPDCCH common search space.

A UE is not required to simultaneously monitor a Type-1-NPDCCH common search space and a Type2-NPDCCH common search space.

A UE is not required to monitor Type1A-NPDCCH common search space or Type2A-NPDCCH common search space in subframes in which the UE monitors a Type1-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by P-RNTI

A UE is not required to monitor Type1A-NPDCCH common search space or Type2A-NPDCCH common search space in subframes in which the UE monitors a Type2-NPDCCH common search space or in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by C-RNTI or Temporary C-RNTI.

A UE is not required to monitor Type2A-NPDCCH common search space in the same subframe in which it monitors Type1A-NPDCCH common search space.

UE is not required to monitor Type1A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by SC-RNTI.

UE is not required to monitor Type2A-NPDCCH common search space in subframes in which the UE receives NPDSCH assigned by NPDCCH with DCI CRC scrambled by G-RNTI or SC-RNTI.

Until UE receives higher layer configuration of NPDCCH UE-specific search space, the UE monitors NPDCCH according to the same configuration of NPDCCH search space as that for NPDCCH scheduling Msg4.

A UE is not required to monitor Type1-NPDCCH common search space or NWUS if the set of subframes comprising the NPDCCH candidates or the set of subframes where NWUS may be received include any subframes in which the UE has initiated an NPUSCH transmission using preconfigured uplink resource on a given serving cell.

A UE is not required to monitor Type-1 NPDCCH common search space or NWUS in subframes in which the UE monitors a UE-specific NPDCCH search space given by PUR-RNTI.

An NPDCCH search space  at aggregation level  ( for TDD special subframe,  otherwise), and repetition level  is defined by a set of NPDCCH candidates where each candidate is repeated in a set of consecutive NB-IoT downlink subframes excluding subframes used for transmission of SI messages starting with subframe .

For NPDCCH UE-specific search space, the aggregation and repetition levels defining the search spaces and the corresponding NPDCCH candidates are listed in Table 16.6-1 by substituting the value of with the higher layer configured parameter npdcch-NumRepetitions, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-NumRepetitions in PUR-Config-NB.

For Type1-NPDCCH common search space and Type1A-NPDCCH common search space, the aggregation and repetition levels defining the search spaces are listed in Table 16.6-2 by substituting the value of

-	with the higher layer configured parameter npdcch-NumRepetitionPaging for Type1-NPDCCH common search space;

-	with the higher layer configured parameter npdcch-NumRepetitions-SC-MCCH for Type1A-NPDCCH common search space.

For Type2-NPDCCH common search space and Type2A-NPDCCH common search space, the aggregation and repetition levels defining the search spaces and the corresponding monitored NPDCCH candidates are listed in Table 16.6-3 by substituting the value of

-	with the higher layer configured parameter npdcch-NumRepetitions-RA for Type2-NPDCCH common search space;

-	with the higher layer configured parameter npdcch-NumRepetitions-SC-MTCH for Type2A-NPDCCH common search space.

The locations of starting subframe  are given by where is the th consecutive NB-IoT DL subframe from subframe , excluding subframes used for transmission of SI messages, and , and , and where

-	subframe  is a subframe satisfying the condition , where , T≥4.

-	for NPDCCH UE-specific search space,

-	is given by the higher layer parameter npdcch-StartSF-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-StartSF-USS in PUR-Config-NB,

-	is given by the higher layer parameter npdcch-Offset-USS, except for NPDCCH candidates associated with PUR-RNTI in which case it is given by higher layer parameter npdcch-Offset-USS in PUR-Config-NB,

-	for NPDCCH Type2-NPDCCH common search space,

-	is given by the higher layer parameter npdcch-StartSF-CSS-RA,

-	is given by the higher layer parameter npdcch-Offset-RA,

-	for NPDCCH Type2A-NPDCCH common search space,

-	is given by the higher layer parameter npdcch-startSF-SC-MTCH,

-	is given by the higher layer parameter npdcch-Offset-SC-MTCH,

For Type1-NPDCCH common search space,and is determined from locations of NB-IoT paging opportunity subframes.

For Type1A-NPDCCH common search space, and subframe  is a subframe satisfying the condition , where , T≥4 and

-	is given by the higher layer parameter npdcch-StartSF-SC-MCCH,

-	is given by the higher layer parameter npdcch-Offset-SC-MCCH.

For UE-specific search space by C-RNTI,

if the UE is configured by higher layers with a NB-IoT carrier for monitoring of NPDCCH UE-specific search space,

-	the UE shall monitor the NPDCCH UE-specific search space on the higher layer configured NB-IoT carrier,

-	the UE is not expected to receive NPSS, NSSS, NPBCH on the higher layer configured NB-IoT carrier.

otherwise,

-	the UE shall monitor the NPDCCH UE-specific search space on the same NB-IoT carrier on which NPSS/NSSS/NPBCH are detected.

For UE-specific search space by PUR-RNTI, the UE is configured by the higher layer parameter carrierConfig in PUR-Config-NB with a NB-IoT carrier for monitoring of NPDCCH UE-specific search space,

-	the UE shall monitor the NPDCCH UE-specific search space on the higher layer configured NB-IoT carrier,

-	the UE is not expected to receive NPSS, NSSS, NPBCH on the higher layer configured NB-IoT carrier if the NB-IoT carrier is not the same as the NB-IoT carrier on which NPSS/NSSS/NPBCH are detected.

If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration, where Kmac is provided by higher layers, otherwise Kmac=0. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration.

Table 16.6-1: NPDCCH UE- specific search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- |
|  |  |  | L'=1 | L'=2 |
| 1 | 1 | 00 | {0},{1} | {0,1} |
| 2 | 1 | 00 | {0},{1} | {0,1} |
|  | 2 | 01 | - | {0,1} |
| 4 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
|  | 4 | 10 | - | {0,1} |
| >=8 |  | 00 | - | {0,1} |
|  |  | 01 | - | {0,1} |
|  |  | 10 | - | {0,1} |
|  |  | 11 | - | {0,1} |
| Note 1: {x}, {y} denotes NPDCCH Format 0 candidate with NCCE index 'x', and NPDCCH Format 0 candidate with NCCE index 'y' are monitored Note 2: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |

Table 16.6-2: Type 1/Type 1A - NPDCCH common search space candidates

|  |  |  |  |  |  |  |  |  | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | L'=1 | L'=2 |
| 1 | 1 | - | - | - | - | - | - | - | - | {0,1} |
| 2 | 1 | 2 | - | - | - | - | - | - | - | {0,1} |
| 4 | 1 | 2 | 4 | - | - | - | - | - | - | {0,1} |
| 8 | 1 | 2 | 4 | 8 | - | - | - | - | - | {0,1} |
| 16 | 1 | 2 | 4 | 8 | 16 | - | - | - | -- | {0,1} |
| 32 | 1 | 2 | 4 | 8 | 16 | 32 | - | - | - | {0,1} |
| 64 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | - | - | {0,1} |
| 128 | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 | - | {0,1} |
| 256 | 1 | 4 | 8 | 16 | 32 | 64 | 128 | 256 | - | {0,1} |
| 512 | 1 | 4 | 16 | 32 | 64 | 128 | 256 | 512 | - | {0,1} |
| 1024 | 1 | 8 | 32 | 64 | 128 | 256 | 512 | 1024 | - | {0,1} |
| 2048 | 1 | 8 | 64 | 128 | 256 | 512 | 1024 | 2048 | - | {0,1} |
| DCI subframe repetition number | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |  |  |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |  |  |  |  |  |  |

Table 16.6-3: Type 2/Type 2A - NPDCCH common search space candidates

|  |  | DCI subframe repetition number | NCCE indices of monitored NPDCCH candidates |  |
| --- | --- | --- | --- | --- |
|  |  |  | L'=1 | L'=2 |
| 1 | 1 | 00 | - | {0,1} |
| 2 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
| 4 | 1 | 00 | - | {0,1} |
|  | 2 | 01 | - | {0,1} |
|  | 4 | 10 | - | {0,1} |
| >=8 |  | 00 | - | {0,1} |
|  |  | 01 | - | {0,1} |
|  |  | 10 | - | {0,1} |
|  |  | 11 | - | {0,1} |
| Note 1: {x,y} denotes NPDCCH Format1 candidate corresponding to NCCEs 'x' and 'y' is monitored. |  |  |  |  |

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npusch-MultiTB-Config and if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k,

-	if the corresponding NPDCCH with DCI format N0 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1, otherwise the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1; and

- the UE does not expect to receive a DCI Format N0 before subframe n+k-2 for which the corresponding NPUSCH format 1 transmission ends later than subframe n+k+255 if the corresponding NPDCCH with DCI format N0 schedules one transport block.

-	for TDD, and if the corresponding NPUSCH format1 transmission ends in subframe n+m, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the NB-IoT UE detects NPDCCH with DCI Format N0 ending in subframe n or receives a NPDSCH carrying a random access response grant ending in subframe n, and if the corresponding NPUSCH format 1 transmission ends in n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k.

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig or npdsch-MultiTB-Config

-	and if the NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if a NPDSCH transmission starts from n+k,

-	if the corresponding NPDCCH with DCI format N1 with CRC scrambled by C-RNTI schedules two transport blocks as determined by the Number of scheduled TB for Unicast field if present, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+1 to subframe n+k-1;

-	otherwise, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1;

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, and

-	for FDD, if the corresponding NPUSCH format 2 transmission starts from subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

-	for TDD, if the corresponding NPUSCH format 2 transmission ends in subframe n+m the UE is not required to monitor NPDCCH in any subframe starting from subframe n+ k to subframe n+m-1.

If a NB-IoT UE detects NPDCCH with DCI Format N1 for "PDCCH order" ending in subframe n, and

-	for FDD, if the corresponding NPRACH transmission starts from subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

-	for TDD, if the corresponding NPRACH transmission ends in subframe n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the UE has a NPUSCH transmission ending in subframe n,

-	the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and

-	the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3;

else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and

-	if the NB-IoT UE has a NPUSCH transmission ending in subframe n , the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3.

otherwise,

-	If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3.

If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12.

If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5

otherwise

-	the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

An NB-IoT UE is not required to monitor NPDCCH candidates of an NPDCCH search space during an NPUSCH UL gap.

An NB-IoT UE is not required to monitor NPDCCH candidates of a Type2A-NPDCCH common search space during the scheduling gap or the processing gap.

For an NB-IoT UE configured with higher layer parameter sr-WithoutHARQ-ACK-Config, if the transmission of a narrowband random access preamble for SR ends on subframe n,

-	in case of frame structure type 1 with NPRACH format 0 and 1 when the number of NPRACH repetitions is greater than or equal to 64, or NPRACH format 2 when the number of NPRACH repetitions is greater than or equal to 16, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+40,

-	otherwise, the UE is not required to monitor NPDCCH UE-specific search space from subframe n to subframe n+3.

### 16.6.1	NPDCCH starting position

The starting OFDM symbol for NPDCCH given by index  in the first slot in a subframe  and is determined as follows

-	if higher layer parameter eutraControlRegionSize is present

-	if subframe  is a special subframe for NPDCCH without repetition

-	 where is given by the higher layer parameter eutraControlRegionSize

-	else  is given by the higher layer parameter eutraControlRegionSize

- otherwise

-

### 16.6.2	NPDCCH control information procedure

A UE shall discard the NPDCCH if consistent control information is not detected.

### 16.6.3	NPDCCH validation for semi-persistent scheduling

A UE shall validate a Semi-Persistent Scheduling assignment NPDCCH only if all the following conditions are met:

-	the CRC parity bits obtained for the NPDCCH payload are scrambled with the Semi-Persistent Scheduling C-RNTI

-	the new data indicator field is set to '0'.

Validation is achieved if all the fields for the used DCI format N0 are set according to Table 16.6.3-1 or Table 16.6.3-2.

If validation is achieved, the UE shall consider the received DCI information accordingly as a valid semi-persistent activation or release.

If validation is not achieved, the received DCI format shall be considered by the UE as having been received with a non-matching CRC.

Table 16.6.3-1: Special fields for Semi-Persistent Scheduling Activation NPDCCH Validation

|  | DCI format N0 |
| --- | --- |
| HARQ process number (present if UE is configured with 2 uplink HARQ processes) | set to '0' |
| Redundancy version | set to '0' |
| Modulation and coding scheme | set to '0000' |
| Resource assignment | set to '000' |

Table 16.6.3-2: Special fields for Semi-Persistent Scheduling Release NPDCCH Validation

|  | DCI format N0 |
| --- | --- |
| HARQ process number (present if UE is configured with 2 uplink HARQ processes) | set to '0' |
| Redundancy version | set to '0' |
| Repetition number | set to '000' |
| Modulation and coding scheme | set to '1111' |
| Subcarrier indication | Set to all '1's |

### 16.6.4	Preconfigured uplink resource ACK/fallback procedure

If a UE has initiated a NPUSCH transmission using preconfigured uplink resource on a given serving cell, and upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the PUR search space window as defined in Clause 16.6, and the value of "modulation and coding scheme" field () in the corresponding DCI set to '14', the UE shall deliver the PUR ACK/fallback indication and the NPUSCH repetition adjustment, as signaled on the NPDCCH, to the higher layers.

## 16.7	Assumptions independent of physical channel related to narrowband IoT

A UE may assume the antenna ports 2000 – 2001 of a serving cell are quasi co-located (as defined in [3]) with respect to delay spread, Doppler spread, Doppler shift, average gain, and average delay.

## 16.8	UE procedure for acquiring cell-specific reference signal sequence and raster offset

If the higher layer parameter operationModeInfo indicates inband-SamePCI for a cell, the UE may derive cell-specific reference signal sequence and raster offset from the higher layer parameter eutra-CRS-SequenceInfo according to Table 16.8-1, where E-UTRA PRB index  is defined as .

Table 16.8-1: Definition of eutra-CRS-SequenceInfo

| eutra-CRS- SequenceInfo | E-UTRA PRB index for odd number of | Raster offset | eutra-CRS- SequenceInfo | E-UTRA PRB index for even number of | Raster offset |
| --- | --- | --- | --- | --- | --- |
| 0 | -35 | -7.5 kHz | 14 | -46 | +2.5 kHz |
| 1 | -30 |  | 15 | -41 |  |
| 2 | -25 |  | 16 | -36 |  |
| 3 | -20 |  | 17 | -31 |  |
| 4 | -15 |  | 18 | -26 |  |
| 5 | -10 |  | 19 | -21 |  |
| 6 | -5 |  | 20 | -16 |  |
| 7 | 5 | +7.5 kHz | 21 | -11 |  |
| 8 | 10 |  | 22 | -6 |  |
| 9 | 15 |  | 23 | 5 | -2.5 kHz |
| 10 | 20 |  | 24 | 10 |  |
| 11 | 25 |  | 25 | 15 |  |
| 12 | 30 |  | 26 | 20 |  |
| 13 | 35 |  | 27 | 25 |  |
|  |  |  | 28 | 30 |  |
|  |  |  | 29 | 35 |  |
|  |  |  | 30 | 40 |  |
|  |  |  | 31 | 45 |  |

## 16.9	UE procedure for receiving narrowband wake up signal

A NB-IoT UE can be configured with up to two NWUS [14]. A UE may assume that no more than one NWUS sequence is transmitted per NWUS resource at a given time.

A NB-IoT UE using NWUS can assume the actual duration of NWUS is one of the values in the set listed in Table 16.9-1 corresponding to the maximum duration of NWUS, LNWUS_max, configured by higher layers. There is a total of LNWUS_max NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the maximum duration of NWUS. The NWUS starts in subframe w0, where w0 is the latest subframe such that there is a total of  NB-IoT DL subframes and subframes #4 carrying SystemInformationBlockType1-NB in the duration that ends in subframe (g0-1), where g0 is defined by [14] and NIDresource is the NWUS resource that the UE is associated to as defined in [3]. The UE may assume that NWUS and its associated NB-IoT paging occasion subframes are on the same NB-IoT carrier.

Table 16.9-1: Actual NWUS durations in NB-IoT DL subframes or subframes containing SystemInformationBlockType1-NB.

| LNWUS_max | Actual NWUS durations set |
| --- | --- |
| 1 | {1} |
| 2 | {1, 2} |
| 4 | {1, 2, 4} |
| 8 | {1, 2, 4, 8} |
| 16 | {1, 2, 4, 8, 16} |
| 32 | {1, 2, 4, 8, 16, 32} |
| 64 | {1, 2, 4, 8, 16, 32, 64} |
| 128 | {1, 2, 4, 8, 16, 32, 64, 128} |
| 256 | {1, 2, 4, 8, 16, 32, 64, 128, 256} |
| 512 | {1, 2, 4, 8, 16, 32, 64, 128, 256, 512} |
| 1024 | {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024} |

A NB-IoT UE using NWUS can assume there are at least 10 NB-IoT DL subframes between the end of the maximum duration of NWUS and the first associated NB-IoT paging occasion subframe.

<Unchanged parts are omitted>