3GPP TSG-RAN WG1 Meeting #107	R1-2112544

e-Meeting, 11th – 19th November 2021

Agenda Item:	8.6.1.2

Title:	FL summary #1 on other aspects for RedCap

Source:	Moderator (Qualcomm Inc.)

Document for:	Discussion, Decision

# Introduction

This feature lead (FL) summary (FLS) concerns the Rel-17 work item (WI) for support of reduced capability (RedCap) NR devices [1]. Earlier RAN1 agreements for this WI are summarized in [2].

This document summarizes contributions [3] – [25] submitted to agenda item 8.6.1.2 and captures this email discussion on other aspects operation for RedCap:

/This one is to use NWM – please use RAN1-107-e-NWM-NR-R17-RedCap-02 as the document name [107-e-NR-R17-RedCap-02] Email discussion regarding other aspects of UE complexity reduction – Chao (Qualcomm) - 1st check point: November 15 - Final check point: November 19

The following proposals are candidates for treatment in the Friday GWT session on Nov 12th.

FL1 High Priority Proposal 2.1-1: - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission FL1 High Priority Proposal 2.3-1: - For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied

Attached to this cover-page document is the report of the NWM discussion (v0.0.1) with the detailed discussion included.