3GPP TSG RAN WG1 #107-e                                           R1-2112556

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.8.3

Title: 	Feature lead summary #1 on support of Type A PUSCH repetitions for Msg3

Source: 	Moderator (ZTE Corporation)

Document for:	Discussion and Decision

- Introduction

One objective of the coverage enhancement WID is to specify mechanism(s) to support Type A PUSCH repetitions for Msg3. This contribution provides a summary of proposed Msg3 enhancements in contributions submitted under AI 8.8.3 and AI 8.8.4.

- Summary of Tdocs

  - Indication of the number of repetitions for Msg3

[Open] Issue#1: Indication of the number of repetitions for Msg3 initial transmission

For Msg3 initial transmission, the following WA was reached for repetition indication in RAN1#106-e.

Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field.

Companies’ views are summarized in the following table.

Table 2.1-1 Summary of companies’ views for repetition indication for Msg3

| Alternative | Support | Concern | Can live with | Detailed views |
| --- | --- | --- | --- | --- |
| Alt 1. TDRA | Sharp, CATT, Ericsson, vivo, Xiaomi, Panasonic, ZTE, CMCC, OPPO, InterDigital, ETRI, LG, Spreadtrum Communication, [Lenovo/Motorola Mobility], [NEC] （15） |  | DCM | Pros: - Use similar mechanism defined for regular PUSCH repetition in Rel-16, which may require less spec efforts - gNB can choose to schedule Msg3 repetition by less number of PRBs with high MCS index to save frequency domain resources. Cons: - Impact the flexibility of time domain resource indication. - It may or may not impact the flexibility of time domain resource indication for legacy UEs, depending on NW configuration. (More detailed analysis please find in the Note 1 below). |
| Alt 2. MCS | Apple, Intel, Samsung, Huawei/HiSilicon Nokia/NSB, DCM, China Telecom, WILUS （8） |  |  | Pros: - May have less impact to legacy UEs - For Msg3 repetition, gNB can choose to use default TDRA table to save SIB1 signaling. - There is no clear performance loss in case the payload size is larger than 72 bits and the number of repetitions is smaller than 4. Cons: - Impact the flexibility of MCS indication; - The MCS filed length are 4 bits in RAR UL grant and 5 bits in DCI format 1-0 with TC-RNTI. It would require additional effort to discuss how to repurpose MCS bit field for Msg3 re-transmission. - When the number of repetitions is equal to or larger than 4, limiting the MCS index no larger than 3 could degrade the performance (~0.7 dB in average) for payload size smaller than 72 bits. This is because using more repetitions could decrease the effective coding gain from using low MCS index while PSD reduction due to using more PRBs would become more evident. |

Note 1: Impact on legacy UEs for Alt1: Alt 1 with sharing the same {K2, mapping type, SLIV} between legacy UEs and CE UEs could impact the flexibility of time domain indication for all UEs.

- To avoid causing any impacts on legacy UEs, NW could configure the {K2, mapping type, SLIV} fully depend on only legacy UEs. Then, it would mean the flexibility of time domain indication including the number of repetitions for Rel-17 CE UEs could be then potentially limited. For instance, for those rows, if any, configured with a short Msg3 PUSCH, e.g., 4-OS, it may not be suitable for Rel-17 CE UEs.

  - In practical, most of the rows may be configured with a length L>=10 symbols for Msg3 PUSCH, similar as the default TDRA table A where 13 out of 16 rows are configured with L>=10. In such case, most of rules configured for legacy UEs could also applied to Rel-17 CE UEs.

5 companies ([1, Huawei, HiSilicon], [4, ZTE], [16, Nokia/NSB], [18, Sharp], [22, LG]) provided evaluation results, which are summarized in Appendix-C. The evaluation is to compare the performance for the following two cases.

- Case 1: scheduling with a minimum number of PRBs and the corresponding MCS index could be larger than 3.

- Case 2: scheduling with a larger number of PRBs to ensure the MCS index no larger than 3.

Apparently, gNB can schedule either Case 1 or Case 2 by Alt 1 (TDRA based solution), while only Case 2 can be applied to Alt 2 (MCS based solution).

In general, companies’ results are aligned, and the following observations can be obtained.

- When the number of repetitions is no larger than 2, Case 2 can provide better performance for a majority of evaluation cases, according to [1, Huawei, HiSilicon] and 16, Nokia/NSB].

  - The larger the payload size, the more Case 2 outperforms Case 1.

- When the number of repetitions is 4 or 8, and the payload size is 72 bits, Case 1 can provide better performance than Case 2 based on the results from all 5 companies. This is because the effective coding gain of Case 1 by using lower MCS index is not competitive anymore in case of using a relatively larger number of repetitions. Meanwhile, the effect of the PSD increase with a smaller number of PRBs for Case 1 becomes more evident.

  - The performance gain is about

    - less than 0.5 dB, according to [1, Huawei, HiSilicon].

    - 0 dB~0.65 dB, according to [4, ZTE].

    - 0.4 dB~1.76 dB, according to [16, Nokia/NSB]

    - 0.13 dB~1.43 dB, according to [18, Sharp].

    - 0.4 dB~1 dB, according to [22, LG].

  - The averaged performance gain is about 0.7 dB. It expects the performance gain would become larger for a lower payload size (e.g., 56 bits) or for a larger number of repetitions.

- When the number of repetitions is 4 or 8 and the payload size is larger than 72 bits, the evaluation results are a bit divergent.

  - 8 out of 8 cases show Case 2 can provide better or approximate performance over Case 1, according to [1, Huawei, HiSilicon].

  - 8 out of 8 cases show Case 1 can provide about 0.16 dB~0.65 dB performance gain over Case 2, according to [4, ZTE].

  - 13 out of 15 cases show Case 1 can provide about 0.38 dB~2.53 dB performance gain over Case 2, according to [16, Nokia/NSB].

  - 3 out of 4 cases show Case 2 can provide about 0.13 dB~0.21 dB performance gain over Case 1, according to [18, Sharp].

  - 5 out of 6 cases show Case 1 can provide about 0.3 dB~0.8 dB performance gain over Case 2 performance gain, according to [22, LG].

  - In summary, 2 companies ([1, Huawei, HiSilicon], [18, Sharp]) show Case 1 has better performance while 3 companies ([4, ZTE], [16, Nokia/NSB], [22, LG]) show Case 2 is better.

First round

Based on companies input, FL further updates the pros and cons for each alternative in Table 2.1-1. From FL perspective, both alternatives can work well, and the benefits of one over another is not overwhelming. In such circumstance, a decision (have to make for sure) based on the majority seems the only way we can go. Thus, FL would recommend to go with Alt 1 (TDRA based solution), and strongly encourage companies could be constructive here.

Proposal for Issue#1: For indication of the number of repetitions of Msg3 initial transmission, Alt 1 (i.e., using TDRA information field) is adopted.

Companies are encouraged to update your company position (if necessary) in Table 2.1-1 provide your comments for the proposal if any below.

| Company | Comments |
| --- | --- |
| Sharp | Support the proposal. I would like to emphasize that, for Alt.2, 72 bits TBS for RRC resume request is not supported for 1 PRB allocation. |
| CATT | We support the proposal. TDRA based indication has least flexibility sacrifice, which is already applied in Rel-16 PUSCH repetition number indication. It would not introduce a new crossing-function in another DCI field, which also makes the specification less readable. |
| Panasonic | We support the proposal. |
| Nokia/NSB | We do not agree that repurposing the MCS IE of DCI 1_0 with CRC scrambled by TC-RNTI complicates the discussion. There is only one logic that can be used, as used showed in our paper. It should be noted that this allows the MCS index for Msg3 re-transmission to be up to MCS7. |
| Intel | We do not support the proposal. It is clear that TDRA based solution has certain limitation in terms of signalling overhead, and forces gNB to configure this in order to support the Msg3 repetition. Our view is that MCS based solution is suitable for coverage enhancement. |
| NTT DOCOMO | Although our first preference is MCS, we are fine with the proposal for the progress. |
| Apple | We do NOT support the proposal. It is more than clear that higher payload for a UE in limited coverage is not justified (so there is pretty much no reasonable scenario that justifies the need for larger MCS over a smaller PRB). So, the performance gain for larger MCS over smaller PRB AND larger repetitions, if any, is for a made scenario which is only due to larger PSD… In addition, it is very clear that TDRA based procedure impacts scheduling flexibility even to legacy UEs. |
| LG | Alt 1 could be adopted for the indication of the number of repetition of Msg3 initial transmission. However, it seems many companies are supporting Alt 2. We could consider to find some middle ground between Alt 1 and Alt 2, instead of selecting only one of them and eliminating the other. Alt 1 could be adopted for the indication of the number of repetition of Msg3 initial transmission. However, it seems many companies are supporting Alt 2. We could consider to find some middle ground between Alt 1 and Alt 2, instead of selecting only one of them and eliminating the other. So, we suggest to let Alt 2 to use higher level of MCS indices rather than restricting MCS indices to the lowest 4 level. This could be a middle ground of Alt 1 and Alt 2. We can escape from Cons of both sides. The only prices to pay is small overhead increase in SIB, but it seems to be much smaller than the one of the Alt 1 (TDRA based method). With the small overhead increase in SIB, any 4 level of MCS indices can be configured and then one of them will be indicated via remaining 2 bits of MCS field in RAR UL grant. Also, if we compare our suggested way (Alt2 with higher level of MCS indices) which will lead to high PSD and high coding rate to the Alt 2 using only the lowest 4 level of MCS indices which will lead to low PSD and low coding rate, as we mentioned in our contribution, it is hard to say that there are significant performance difference between them. |
| OPPO | We support the proposal. |
| Samsung | We do NOT support the proposal. We agree Apple and intel’s comments. In addition, here we are discussing the time domain resource configuration, but the proponent rather taking the flexibility of MCS over the flexibility of the TDRA, which is not the suitable argument. With 2bits explicit indication, all the Rows in the TDRA table can be used, but if a new table is introduced, not only more than signalling overhead will be needed, but the flexibility on the SLIV * repetition number is quite reduced. Regarding the performance: less than 2 repetition, case 2 is better, more than 2 repetition, case 1 has 0.7db better, in larger TBS, 3 over 2 shows gain of case 2 is better. In simple word, new table method did not provide better performance. With such, we really don't understand FL’s proposal is just based on the counting number of supporting companies. |
| WILUS | We still prefer the Alt 2. Even at the expense of the additional number of bits, the need to configure a new TDRA table that may only be used during initial access is questionable. |
| China Telecom | We do not support the proposal. We share the similar view as Apple, Intel and Samsung. Regarding the performance, it can be observed that Alt.1 (TDRA based indication mechanism) doesn’t outperform Alt.2 (MCS based). Thus, we think we should focus on other aspects for down selection. Regarding the flexibility aspect, it is clear that Alt.1 impact the flexibility of time domain resource indication. Although Alt.2 may impact the flexibility of MCS indication which may impact the performance, but the above evaluation results have shown that this is not one critical issue. Thus, we think Alt.2 should be supported. |
| Ericsson | We support the proposal. We would like to clarify the signaling overhead concern. The following working assumption in RAN1#106bis-e says Option 2 is supported for Alt 1. The alleged drawback of Alt1 that large SIB1 payload increase is mainly for Option 1, which was not agreed. Option 2 for Alt 1 has minor increase of SIB1 payload. Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. |
| ZTE | - Support the proposal. - We think it is more important to reserve the flexibility of MCS since using higher MCS index could provide better performance in case the number of repetitions is no smaller than 4. As RAN2 has agreed to support Group B payload size for Msg3 PUSCH repetition, NW should be able to use higher MCS index for scheduling Msg3 with relatively large payload size. |
| CMCC | We support the proposal. |
| vivo | Support FL proposal. TDRA based method in alt1 is more forward compatible, avoids any changes in L1 signaling, is aligned with the dynamic PUSCH repetition mechanism introduced in NR Rel-16, and can be applied by repetition of both Msg3 initial transmission and retransmission. |
| InterDigital | We support the proposal. |
| Qualcomm | We do not support the proposal. Similar to Samsung, Apple and Intel, we think that using TDRA has limitations in terms of signaling overhead, and on the other hand, large MCS is not really needed. So, it is better to prioritize flexibility of TDRA, rather than MCS. Therefore, it is more reasonable to use MCS bitfield for indication of repetition number. |
| Huawei, HiSilicon | It should be noted that, as shown in the following observation, the TDRA based method will increased the payload of SIB1. So, the coverage of SIB1 will be degraded and the degradation cannot be compensated. However, the coverage of Msg3 could be compensated by re-transmission. Observation 1: Compared with the case that the TDRA table can be absent in SIB1 in Rel-15/16, Alt1 forces the gNB to configure a TDRA table in SIB1 and the mandatory TDRA table will increase the signaling overhead of SIB1. Observation 2: Compared with the SIB1-configured TDRA table in Rel-15/16, Alt 1 requires the Rel-17 TDRA table to contain multiple rows that indicate same (K2, mapping type and SLIV) and different repetition factors, so that multiple duplicate (K2, mapping type and SLIV) configurations will be provided by SIB1 which results in additional signaling overhead. |
| FL | - @ NTT DOCOMO, Thanks for being constructive. - @ LG, Thanks for the comments. I can understand your proposal, and similar approach could apply to Alt 1. Thought I am afraid it may not be acceptable for the majority, I can open the discussion on whether to pursue further optimization of current Alt 1 or Alt 2 once we agree one of them. - For other companies, it seems their position and arguments are not changed. - I will bring both alternatives for next GTW session. As you can see, this proposal is very important and only after we finalizing this issue, we can then make decision for issue #2~4. I hope companies could keep this in mind, and be constructive in the next GTW session. |

[Closed] Issue#2: Candidate values for Msg3 initial/re-transmission repetitions

In RAN1#106-e, the repetition factor K = {1, 2, 4}was agreed for Msg3 repetition, and other values are FFS [4].

Agreement - Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition. - FFS whether to support other values, e.g., 8. - Note: K=1 is supported and how to support K=1 is FFS.

Companies’ views on the candidate values including the maximum number of repetitions are summarized below

- [1, Huawei, HiSilicon]: Support at least repetition factor K = {2, 4, 8} for Msg3 PUSCH repetition. The maximal repetition number up to 16 can be considered for Msg3 PUSCH repetition.

- [8, Xiaomi]: The maximum number of repetitions for type A PUSCH repetition in release 17 can be adopt for Msg.3 repetition.

- [9, InterDigital]: Support repetition factors {1, 2, 3, 4, 7, 8, 12, 16}

- [10, Intel]: ~2dB performance gain can be achieved for Msg3 PUSCH when the repetition level is doubled.

- [12, Qualcomm]: Support repetition factor K = 8 and 16 for Msg3 PUSCH repetition.

- [16, Nokia/NSB]: Supported Msg3 repetition numbers other than K=1 should be limited to already agreed K∈{2, 3, 4, 7, 8}.

- [19, CMCC]: The repetition factor K should not be larger than 8.

- [21, ZTE]: NR Rel-16 repetition factor set {1, 2, 3, 4, 7, 8, 12, 16} is supported for Msg3 repetition.

- [22, LG]: Adopt setting default values for each element of TDRA table if TDRA based method is selected.

FL suggests to discuss this issue under the email thread [107-e-R17-RRC-CovEnh] for discussion of RRC parameters.

[Closed] Issue#3: Indication of the number of repetitions for Msg3 re-transmission

For Msg3 re-transmission, the following agreements were reached for repetition indication in RAN1#105-e.

Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

Companies’ preference for the two options about repetition indication for Msg3 re-transmission are summarized below.

- Option 1: Use the same mechanism as supported for Msg3 initial transmission

  - [1, Huawei, HiSilicon], [ 2, OPPO], [4, ZTE], [5, vivo], [6, CATT], [8, Xiaomi], [9, InterDigital], [13, Panasonic], [14, Samsung] (if initial transmission adopts TDRA based method), [15, ETRI], [17, Ericsson], [20, NTT DOCOMO], [22, LG]

- Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

    - [10, Intel], [11, Apple], [12, Qualcomm], [14, Samsung] (if initial transmission doesn’t adopt TDRA based method), [23, WILUS]

[16, Nokia/NSB]: Discussion on the indication of the repetition number for Msg3 re-transmission should be postponed until a solution for the indication of repetition number for Msg3 initial transmission is agreed.

The situation is rather clear, FL suggests to postpone the discussion on this issue until there is a clear solution for initial transmission.

[Closed] Issue#4: How to interpret the information field (legacy or new interpretation)

As for how a UE is managed to know whether to use legacy interpretation or new interpretation on the bit field indicating the number of repetitions, there are the following two options agreed in RAN1#106-e.

Agreement Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. - Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. - Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn't request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field.

Companies’ support of each option is summarized below.

- Option 1:

  - Support: [1, Huawei, HiSilicon], [2, OPPO], [3, Spreadtrum Communications], [4, ZTE], [5, vivo], [7, China Telecom], [8, Xiaomi], [9, InterDigital], [10, Intel], [12, Qualcomm], [13, Panasonic], [15, ETRI], [17, Ericsson], [18, Sharp], [19, CMCC], [20, NTT DOCOMO], [22, LG], [23, WILUS], [26, NEC]

- Option 2:

  - Support: [6, CATT], [11, Apple], [14, Samsung] (for TDRA based solution), [16, Nokia/NSB]

  - [6, CATT]: CSI request bit field can be used as an explicit indication of which TDRA table is used for time domain resource allocation.

  - [11, Apple]: A reserved bit in DCI 1-0 to indicate repurposing some of the bit fields in RAR UL grant.

  - [16, Nokia/NSB]: The indication of which interpretation of the UL grant is to be used by UE is provided implicitly to the latter (e.g., using TC-RNTI).

The fundamental difference between Option 1 and Option 2 is whether to introduce additional indication to implicitly or explicitly indicate how the UE should interpret the TDRA table/ repurposed information field. Based on companies’ input, it is clear that the majority prefer Option 1, i.e., they don’t see the necessity to introduce such additional indication. Even among companies preferring Option 2, the proposed indication methods are rather different.

From FL perspective, Option 2 is kind of further optimization to provide additional flexibility for repetition indication while the necessity might not be strong. In addition, this also depends on which information field is chosen and how much flexibility it can offer to indicate the candidate repetition factors. FL suggests further discussing this issue until finalizing Issue #1.

  - Counting on the basis of available slots for Msg3 repetition

In RAN1#105-e, the following agreements were reached for counting the number of repetitions on the basis of available slots for Msg3 repetition.

Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon. Agreement - The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered. - If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot. Agreement The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot. - FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17. - FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)

First of all, there are two steps for Msg3 PUSCH repetition counting based on available slot.

- Step 1: How to determine available slot

- Step 2: Determine the actual transmission of a Msg3 repetition in an available slot.

We will focus on the remaining issues for Step 1 under Issue#5, and discuss Step 2 under Issue#6.

[Closed] Issue #5 Determination of available slot for Msg3 PUSCH repetition

There are two remaining issues for determination of available slot for Msg3 PUSCH repetition.

- 1) Whether to consider ‘N Gap symbols after SSB’ as unavailable symbols.

- 2) Whether to consider flexible symbol indicated by tdd-UL-DL-ConfigurationCommon as available symbol.

Regarding 1), it has been extensively discussed in last two meetings. A vast majority of companies support not to consider ‘N Gap symbols after SSB’ for determination of available slot. The reason is summarized as follows.

- If DL and UL has the same SCS, SSB transmission in a slot ends at 6th, 10th or 12th symbol. For the typical SLIV of Msg3, e.g., L=10, 12, 14, no matter whether to additionally consider the 2 Ngap symbols, the UE will not determine the slot with SSB transmission as available slot since the Msg3 PUSCH repetition would anyway collide with SSB symbols in the slot.

- Otherwise, SSB transmission in a slot may end at 3th symbol. But, it still makes no difference if L = 12 or 14. Even if gNB wants to use the slot with SSB transmission, gNB can still schedule Msg3 PUSCH repetition with L= 8, though L=10 cannot be used.

Based on previous discussion, FL believes the optimization with additionally considering ‘N Gap symbols after SSB’ is NOT essential. Therefore, FL would like to close any further discussion about this issue, meaning it will not be considered unless any critical issue would be identified during maintenance phase.

Regarding 2), the following proposal was discussed and almost reached consensus in RAN1#106bis-e. In this meeting, many companies also propose to confirm the proposal.

Proposal: Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition.

Note: the other potential mechanisms to use the flexible symbols are separately discussed.

Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon.

First round

Regarding the first note, FL makes very minor editorial changes to make it more accurate. Regarding the second note, it is a bit redundant as we have already agreed to reuse legacy collision handling rule for Msg3 repetition (except for FFS case about collision between Msg3 and downlink symbol indicated by tdd-UL-DL-ConfigurationDedicated ). However, FL believes this note could make further clarify particularly for the concerned case and could address the concerns from some companies, without introducing any harm.

Proposal 5 for Issue#5: Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition.

Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed.

Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon.

This has been discussed extensively in the last meeting, and above is already the consensus among companies. Please do NOT make further refinement for the proposal if no REALLY STRONG CONCERNS.

| Company | Comments |
| --- | --- |
| Apple | Do not support. In our view, we all agree that we are not trying to introduce new procedure on which F symbols are usable for Msg3 repetition. On the other hand, we cannot agree on exactly which symbols are available or not. We can agree with something like this: Re-use Rel-15/16 rules to determine whether or not flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition. |
| vivo | Support FL proposal. Available slot determination is Rel-17 specific and is just a first step to derive a set of slots equal to the signalled number of repetitions. Actual usage of flexible symbols for msg3 repetition transmissions can be the same as that used for msg3 transmission without repetition, which is in the 2nd step as we’ve discussed many times in earlier meetings. |
| FL | After further offline clarification with Apple, the proposal is now also acceptable for them. So, I would like to ask for email approval for this proposal. |

[Open] Issue #6 Transmission of Msg3 repetition in available slots

In RAN1#106bis-e, the following agreements were reached for transmission of Msg3 repetition in available slots.

Agreement The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot. - FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17. - FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s)

As the example shown in Figure 3.3-1 of FL summary in [26], if one Msg3 repetition transmitting on flexible symbols indicated by tdd-UL-DL-ConfigurationCommon can be canceled, it may cause ambiguity between gNB and UE about whether a Msg3 repetition is transmitted or not because gNB cannot identify which UE is transmitting Msg3 during CBRA procedure and whether the UE transmitting Msg3 is configured with tdd-UL-DL-ConfigurationDedicated or not. In Rel-16, there are two cancellation cases:

- Case a): Downlink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated.

- Case b): Symbols configured for SSB transmission.

- Note: based on above agreements, legacy rules are reused (except for the FFS case), meaning that

  - it is not expected that there is collision between Msg3 transmission and SFI indication, i.e., Msg3 PUSCH repetition cannot be canceled by SFI.

  - Msg3 PUSCH repetition cannot be canceled by UL CI.

However, it has already been agreed that tdd-UL-DL-ConfigurationCommon and SSB transmission are used for available slot determination. In other words, tdd-UL-DL-ConfigurationCommon and SSB transmission would not be used for canceling Msg3 repetition as there is no Msg3 repetition in the slot with DL symbols or SSB transmission that would overlap with Msg3 repetition. Then, the only canceling case is due to ‘downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated’, i.e., the FFS case in above agreements.

One way to solve the ambiguity issue is to change legacy cancellation rule, i.e., prohibiting the cancellation case for Msg3 PUSCH repetition due to downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. In such case, there would be no ambiguity issue due to cancellation. Also, it implies there is no need to discuss Issue#7, as the main motivation of introducing additional explicit indication is to solve the ambiguity issue due to cancellation.

- Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated, i.e., Msg3 PUSCH repetition can be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17

  - [2, OPPO], [4, ZTE], [5, vivo], [6, CATT], [10, Intel], [16, Nokia/NSB], [19, CMCC], [22, LG]

- Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17

  - [1, Huawei, HiSilicon], [13, Panasonic], [20, NTT DOCOMO], [18, Sharp]

First round

Q1: Whether or not Msg3 PUSCH repetition can be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17?

| Yes | No |
| --- | --- |
| DCM (if UE is in RRC_CONNECTED), LG, WILUS (for repetitions other than the 1st repetition) | Sharp, CATT, Panasonic, Intel, DCM (if UE is in IDLE/INACTIVE), WILUS(for the 1st repetition), ZTE |

If the answer is yes, then we need to further discuss Issue #7 regarding how to solve the ambiguity issue (by implementation or introducing explicit signaling). Otherwise, it means no need to discuss Issue#7 as the ambiguity issue has been avoided at the first place.

Companies are encouraged to indicate your position in above table and also provide comments if any below.

| Company | Comments |
| --- | --- |
| Sharp | Reusing Rel-15 handling of cancelling msg3 PUSCH by downlink indicated by tdd-UL-DL-ConfigurationDedicated would cause bad behaviour for Rel-17 msg3 PUSCH repetition. gNB will combine LLR for each slot for repetition. Then, if mis-understanding occurs between gNB and UE, bad LLR is input to decoder, which leads to serious decoding error. The bad LLR input will remain when additional LLR is input by msg3 repetition. |
| CATT | For a UE in RRC_IDLE state, UE has not been configured with tdd-UL-DL- ConfigurationDedicated. For a UE in RRC_CONNECTED state, usually the gNB CANNOT distinguish whether the UE performing CBRA is a UE in RRC_IDLE or RRC_CONNECTED state. Hence, conservative scheduling is preferred, i.e. the gNB can only assume that Msg3 PUSCH repetition will be transmitted on available symbols determined by tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst only. |
| Panasonic | The actual transmission of Msg.3 PUSCH repetition in an available slot should not be impacted by tdd-UL-DL-ConfiguraitonDedicated since gNB cannot identify whether the Msg.3 transmission is from UE of RRC CONNECTED or not (since tdd-UL-DL-ConfiguraitonDedicated is valid only for RRC_CONNECTED UE). If actual transmission of Msg.3 PUSCH repetition in an available slot is impacted by tdd-UL-DL-ConfiguraitonDedicated, the gNB needs the blind detection of the actual transmission of Msg.3 repetition in an available slot. |
| Intel | For Msg3 PUSCH transmission, gNB may not know whether UEs are in RRC_CONNECTED or RRC_IDLE/INACTIVE mode during RACH procedure. In some cases, e.g., during initial access, UE may not be configured with dedicated RRC parameters including tdd-UL-DL-ConfigurationDedicated. No additional rule is needed for transmission of Msg3 PUSCH repetition in an available slot. |
| NTT DOCOMO | Since there are several issues to handle ambiguous issue with implementation, the ambiguity issue should be solved by modifying collision handling rules. ・If gNB tries to decode symbols corresponding to downlink symbols of tdd-UL-DL-ConfigurationDedicated It requires gNB to perform blind decode, which is the burden for gNB operation. If UE does not transmit Msg3 on those symbols, it results in the waste of resource. ・If gNB does not decode symbols corresponding to downlink symbols of tdd-UL-DL-ConfigurationDedicated It is the waste of resource, as gNB cannot schedule other channel due to potential inference. |
| LG | Legacy rule and gNB implementation would be enough to avoid such collisions. There is no need to adopt any additional rule. |
| OPPO | We prefer to reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. The ambiguity issue due to cancellation can be solved by introducing explicit signaling. |
| Samsung | In any case, does gNB knows who is transmitting msg3? Seems not, then there is no ground to discuss this optimization for handling such collision between DL and msg3. |
| WILUS | Legacy collision handling rule should be applied for backward compatibility. For the 1st repetition where the same behaviour is expected as single-slot Msg3 PUSCH transmission in Rel-15/16, the Msg3 PUSCH cannot be cancelled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated since it is dynamically indicated via TDRA. For remaining repetitions other than the 1st repetition, Msg3 PUSCH can be cancelled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. It may increase gNB complexity to take into account all the flexible symbols for remaining repetitions of Msg3 PUSCH, especially when the number of repetition is large. |
| Ericsson | We support reusing legacy collision handling rule. In Rel-15/16 there is only one Msg3 transmission. If the gNB does not decode symbols corresponding to downlink symbols of tdd-UL-DL-ConfigurationDedicated, the whole Msg3 may be lost. With Rel-17 Msg3 repetition, it is likely that only one of K Msg3 repetitions is lost. Reusing legacy rule in Rel-17 has much smaller impact on Msg3 performance than earlier releases. Anyway, it is up to gNB’s scheduling and configuration to avoid such collision if needed, regardless of UE requesting Msg3 repetitions or not. |
| ZTE | Slightly prefer to reuse legacy collision handling rule. Instead of directly restricting the collision, legacy rule could leave more flexibility for gNB’s scheduling. Basically, gNB can either avoid the collision or allow the collision but resolve the ambiguity issue by implementation. |
| CMCC | We support reusing the legacy collision handling rule and not adopt any other rule. |
| vivo | Isn’t it clear enough with following text already specified in section 11 of 38.213? For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.. Regarding the CBRA case, as indicated by CATT, gNB can just assume the UE will transmit the msg3 even if the UE does not actually transmit the msg3 in which case msg3 repetitions not cancelled or further msg3 retransmissions can be relied on for reception of msg3. According to above and in our understanding, no additional specification change is needed for this. |
| InterDigital | Same view as Panasonic. There should not be any dependency on a configuration that is unknown to the gNB. |
| Huawei, HiSilicon | Once the ambiguity exists between the gNB and UE, the gNB needs blind detection. It not only increases the complexity of gNB but also the blind detection may be incorrect because the transmit power of the CE UE is relatively low and then the uncertain blind detection results will degrade the performance of the Msg3. In addition, the gNB cannot know which UEs (only including Idle UE, or including RRC-connected and Idle UE, or RRC-connected UEs with different tdd-uldl-configurationdedicated) initiate random access. Even though the additional indication in issue 7 is adopted, it cannot ensure the gNB does not give an incorrect indication. So, it means the slot/symblos indicated by the additional indication will not reuse the legacy collision rule in Rel-15/16. Otherwise, the introduction of the additional indication makes no sense. In such case, at least the Msg3 repetition will not be cancelled according to tdd-UL-DL-ConfigurationDedicated should also be declared. |

Second round

@DCM, Considering gNB may not know whether UEs are in RRC_CONNECTED or RRC_IDLE/INACTIVE mode during RACH procedure, it would not appropriate to define different UE behaviors for different connection modes.

@WILUS It seems there is some misunderstanding. In legacy, Msg3 PUSCH can be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. In addition, using different handling rules for different repetitions seems not preferred by other companies, and it could complicate the collision handling procedure and increase the complexity for implementation. Therefore, FL suggests not going to this direction.

Based on the input, a majority of companies prefer to reuse the legacy behavior. Indeed, if legacy behavior is reused, it means gNB could choose 1) avoid such collision, which could be the most typical case in practice, 2) allow such collision with bearing the ambiguity issue, but it’s up to gNB implementation about how to handle the issue (e.g, blind detection or re-transmission etc). So, reusing legacy behavior could provide more alternatives for gNB for handling the collision, and I think it could be also ok for opponents considering ‘avoid such collision’ is still one way for implementation.

With above, FL would like to check whether the following proposal is acceptable for all companies.

Proposal for Issue #6:

Reuse legacy collision handling rule between Msg3 PUSCH transmission and downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated.

- Note: there is no specification impact.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |

[Closed] Issue #7 whether to introduce additional indication for use of flexible slots/symbols indicated via TDD-UL-DL-Configcommon

Regarding whether to additionally introduce explicit indication for indicating whether flexible slots/symbols indicated via TDD-UL-DL-Configcommon can be used for Msg3 repetition, companies’ views are summarized below.

  - Option 1: Additional explicit indication is introduced to indicate whether flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition.

    - [8, Xiaomi], [9, InterDigital], [12, Qualcomm],[14, Samsung]

  - Option 1-A: Introduce InvalidSymbolPattern in SIB1.

    - Support: [8, Xiaomi], [9, InterDigital], [14, Samsung]

    - The signaling design of InvalidSymbolPattern is the same as Rel-16.

  - Option 1-B: Introduce a bitmap indication in DCI format 1-0 scrambled with RA-RNTI or DCI format 0-0 scrambled with TC-RNTI

    - Support: [12, Qualcomm]

  - Option 2: No need additional indication.

    - [6, CATT], [10, Intel], [13, Panasonic], [22, LG], [16, Nokia/NSB]

Note: For a UE requesting Msg3 PUSCH repetition while scheduled without repetition, Rel-16 rules are reused i.e., flexible symbol indicated by tdd-UL-DL-ConfigurationCommon is regarded as available symbols

First round

If Msg3 PUSCH repetition can be cancelled due to ‘downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated’, there would be ambiguity issue between gNB and UE about where the UE transmits Msg3 PUSCH repetition for CBRA procedure. Then, the fundamental question would be: 1) ‘whether it is possible/efficient for gNB by implementation to avoid canceling Msg3 repetition due to collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated. 2) Or, if ambiguity happens due to cancellation, whether it is affordable for gNB to solve this issue by implementation, e.g., do some blind decoding?

- Specifically for 1), whether the following ways could be possible/efficient?

  - 1) gNB does not configure too much flexible symbols by tdd-UL-DL-ConfigurationCommon, e.g., only few flexible symbols for DL-UL switching.

  - 2) The flexible symbols configured by tdd-UL-DL-ConfigurationCommon cannot be changed to DL symbols semi-statically by tdd-UL-DL-ConfigurationDedicated, while gNB can still schedule DL transmissions dynamically on flexible symbols.

  - 3) The flexible symbols configured by tdd-UL-DL-ConfigurationCommon can be carefully changed to DL symbols semi-statically by tdd-UL-DL-ConfigurationDedicated, while gNB cannot schedule Msg3 repetition on these DL symbols.

Based on the input, fewer companies show support for introducing additional signaling compared to the last meeting. In such case, FL would like to check companies’ views on the following conclusion.

Proposed conclusion: There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |
| CATT | Support to have the conclusion. |
| Nokia/NSB | Support the conclusion. |
| Intel | We support the conclusion |
| NTT DOCOMO | Support the conclusion. |
| Apple | Support FL’s proposal |
| LG | Support the conclusion. |
| WILUS | Support the conclusion. |
| Ericsson | Support the conclusion. |
| ZTE | Fine with the conclusion. |
| CMCC | We support the conclusion. |
| vivo | Fine with the conclusion. |
| InterDigital | It seems premature to have this conclusion at this point, as many companies have not provided views. Our concern is that this conclusion would restrict gNB possibility to configure flexible symbols in tdd-UL-DL-ConfigurationCommon. |
| Qualcomm | Fine with the conclusion |
| FL | @InterDigital, We have discussed this for several meetings. A clear majority companies don’t see the necessity, and even the proponents have different proposals in mind. In addition, based on the following alternative implementation as I summarized at the beginning of the first round discussion, it seems gNB can still configure flexible symbols in tdd-UL-DL-ConfigurationCommon. - 2) The flexible symbols configured by tdd-UL-DL-ConfigurationCommon cannot be changed to DL symbols semi-statically by tdd-UL-DL-ConfigurationDedicated, while gNB can still schedule DL transmissions dynamically on flexible symbols. Based on such situation, it seems this is the only way we can go. Hope this could be also acceptable thought not perfect for you. FL would like to ask for email approval. |

  - [Closed] Differentiation and triggering mechanisms for Msg3 repetition

In RAN1#105-e, it was agreed to use separate preamble with share ROs for requesting Msg3 repetition while other solutions are FFS.

Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission.

Agreements reached in RAN2#116-e in CE agenda. FL highlights some of the agreements to draw companies’ attention below.

Agreements: - Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL. - Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition. - If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA (ASN.1 details can be discussed in session on RACH partitioning)

Agreements via email - from offline 112: - ra-ContentionResolutionTimer is started or restarted in the first symbol after all Msg3 repetitions - In shared RO case, it is not supported to configure a separate set of RACH parameters (preambleReceivedTargetPower, powerRampingStep, preambleTransMax) for requesting Msg3 repetition. - In shared RO case, it is not supported to separately configure following parameters for requesting Msg3 repetition: prach-ConfigurationIndex msg1-FDM msg1-FrequencyStart zeroCorrelationZoneConfig totalNumberOfRA-Preambles ssb-perRACH-OccasionAndCB-PreamblesPerSSB rsrp-ThresholdSSB-SUL prach-RootSequenceIndex msg1-SubcarrierSpacing restrictedSetConfig msg3-transformPrecoder - In shared RO case, it is up to the common RACH session to decide how to configure the number of preamble per SSB per RO, and how to indicate the start of preamble index for requesting Msg3 repetition. - A separate rsrp-ThresholdSSB threshold is introduced for requesting Msg3 repetition.

Working Assumptions (to be confirmed in the common RACH session): - From CE perspective, carrier selection and BWP selection are performed ahead of CE selection during RACH procedure. - From CE perspective, UE compares the RSRP of DL path-loss reference with the Msg3 repetition threshold [rsrp-Threshold-Msg3Rep] during the RACH initialization procedure and decides whether to use CE or non-CE RA. - From CE perspective, if CE RA is selected, then the decision doesn’t change during the entire RACH procedure (i.e. until RACH failure).

Agreements reached in RAN2#116-e in RACH common session. FL highlights some of the agreements to draw companies’ attention below.

Agreements: - No new feature and/ feature combination specific preambles are defined within the “not available” preambles defined at the end of a RO through the legacy totalNumberOfRA-Preambles - Specification allows for use of Separate time-frequency resources, not defined through legacy RRC signalling, within Contention free preamble defined through legacy RRC signaling and the combination of these (i.e. using the reserved preamble at the end of SSBs like 2-step RACH) - RAN2 baseline is that preambles for a particular feature combination shall be present in all SSBs (e.g., a feature combination cannot only have preambles in SSB0 but not SSB1) - As a baseline, a feature combination shall have the same number of preambles in all SSBs - Signalling should allow that a particular feature/feature combination can be mapped only to a subset of the RACH occasions of a RACH configuration. - The legacy masking index approach is reused in Rel-17 RA partitioning - RAN2 adopts Approach A as baseline (an IE contains one field for each of the features) for indicating which feature/feature combination a partition applies to. Details are FFS, e.g. details around slicing. FFS how to encode and design the signaling in a future compatible way (i.e. naming) - As a baseline, multiple "RA partitions" for one RA type which map to the same feature/feature combination is not supported on a given BWP. FFS if there is any special use case that requires multiple RA partition configuration.

Working Assumptions (to be confirmed in the common RACH session): - From CE perspective, carrier selection and BWP selection are performed ahead of CE selection during RACH procedure. - From CE perspective, UE compares the RSRP of DL path-loss reference with the Msg3 repetition threshold [rsrp-Threshold-Msg3Rep] during the RACH initialization procedure and decides whether to use CE or non-CE RA. - From CE perspective, if CE RA is selected, then the decision doesn’t change during the entire RACH procedure (i.e. until RACH failure). => confirm working assumption

Agreements - RAN2 assumes that the network may not provide all possible permutation. FFS whether the selection in case of missing combination is specified or left to UE implementation - For slicing, unified partitioning framework should take priority

Based on above agreements/WA in RAN2, it can be found that:

- RAN2 will continue the discussion in common RACH session about how to configure the number of preamble and the start of preamble index for requesting Msg3 repetition. ==> No need discussion on RRC configuration of separate preambles in RAN1.

- RAN2 has already agreed that signalling should allow that a particular feature/feature combination can be mapped only to a subset of the RACH occasions of a RACH configuration. ==> No need to discuss whether/how to introduce a PRACH mask in RAN1.

- RAN2 has already agreed to consider to use separate time-frequency resources for a particular feature/feature combination. It expects RAN2 will continue discussing related signaling in RACH common session. ==> No need to discuss whether/how to introduce separate RO for request of Msg3 in RAN1.

First round

Based on above analysis, FL’s understanding is all remaining details for differentiation mechanisms for Msg3 PUSCH repetition can be left to RAN2, and no further RAN1 work is needed. However, FL would like to collect companies’ views and check whether anything is missing.

| Company | Comments |
| --- | --- |
| Sharp | We have similar understanding as FL. |
| CATT | If possible, RAN1 should give a clear view on supporting separate ROs for early identification for request Msg3 repetition to RAN2, which can provide more flexibility to the gNB. |
| Nokia/NSB | We have a similar understanding as FL. |
| Intel | We share similar view as CATT that separate ROs for requesting Msg3 repetition should be considered/discussed in RAN1 first. |
| Apple | In our view, the interaction between newly introduced threshold(s) on each UL carrier needs to be discussed in RAN1. |
| Ericsson | We share the same view as CATT. |
| ZTE | Given RAN2 is discussing the details, there is no need further RAN1 discussion. |
| CMCC | We share similar view with FL. |
| vivo | According to RAN2 agreements, it seems separate RO configuration is already agreed to be supported which should be a baseline for RAN1 to discuss how to capture the separate parameters in 38.213 as we discussed in draft CR discussions and also in the RRC parameter discussions. So we also think it would be good that RAN1 can complete the work required to complete RAN1 spec. changes in time with respect to separate RO configuration. |
| InterDigital | We have a similar understanding as FL. |
| Qualcomm | We have similar view as CATT. |
| FL | @CATT, Intel, Ericsson vivo, As RAN2 would discuss the signaling aspects together with other Rel-17 features, it seems no need to open the discussion in RAN1. In addition, I am afraid we may end up with no consensus in RAN1 if we only consider the necessity of using separate RO for CE feature only. @ vivo, FL understanding is we may only be able to change RAN1 spec after RAN2 completes how to use separate RO together with other Rel-17 features, based on the discussion for draft CR. So, for now, it seems no specific RAN1 issues we can do, as also confirmed by other companies. @Apple. The procedures for determining the threshold(s) are specified in RAN2 specs, and RAN2 has already made some agreements and will continue the discussion. It seems not appropriate to discuss in RAN1. Based on above input, FL would like to close the discussion. |

  - RV pattern

[Closed] Issue#9: RV pattern for Msg3 repetition

In NR Rel-15/16, a UE shall use RV0 for Msg3 initial transmission, and use the 2-bit RV bit field in DCI format 0_0 scrambled by TC-RNTI for RV indication. If Msg3 repetition is enabled, the following rules were agreed in RAN1#105-e.

Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot.

Regarding the FFS point, several companies raising this issue propose that similar approach that defined for enhanced PUSCH repetition type A can be reused for Msg3 repetition.

First round

- Proposal for Issue#9: RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |
| Sharp | We support FL proposal. |
| CATT | We support the proposal. |
| Panasonic | We support the proposal. |
| Nokia/NSb | Support |
| Intel | We are fine with FL’s proposal. |
| NTT DOCOMO | Support the proposal. |
| Apple | Support FL’s proposal |
| LG | Supportive. |
| OPPO | We are fine with the proposal. |
| Samsung | Fine. |
| WILUS | We support the proposal. |
| China Telecom | Fine with the proposal. |
| Ericsson | Support FL’s proposal |
| ZTE | Support. |
| CMCC | We support the proposal. |
| vivo | Fine. |
| Qualcomm | Support |
| Huawei, HiSilicon | We are fine with the proposal. |
| FL | The proposal seems agreeable. FL would like to ask for email approval for this proposal. |

  - Frequency hopping related issues.

[Closed] Issue#10: Inter-slot FH

In RAN1#106bis, the following conclusion and agreement for frequency hopping are reached.

Conclusion There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17. Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition. Agreement If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping.

In this meeting, several companies propose some further details for inter-slot FH for Msg3 PUSCH repetition. Though it is straightforward, FL thinks it could be better to make explicit agreements for the proposals.

[8, Xiaomi]: For Msg.3 inter-slot FH, reuse the RB offset determination mechanism for Msg.3 intra-slot FH in rel-16.

For a PUSCH transmission with frequency hopping scheduled by RAR UL grant or for a Msg3 PUSCH retransmission, the frequency offset for the second hop [6, TS 38.214] is given in Table 8.3-1. Table 8.3-1: Frequency offset for second hop of PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission Number of PRBs in initial UL BWP; Value of NUL,hop Hopping Bits; Frequency offset for 2nd hop. NBWPsize<50; 0; NBWPsize2. 1; NBWPsize4. NBWPsize≥50; 00; NBWPsize2. 01; NBWPsize4. 10; -NBWPsize4. 11; Reserved.

[18, Sharp]: ‘pos2’ for additional DMRS configuration is assumed for msg3 PUSCH repetition with inter-slot frequency hopping.

- In Rel-16 specification, for additional DMRS configuration, it is specified that ‘pos2’ should be assumed for msg3 PUSCH without intra-slot hopping and ‘pos1’ should be assumed for msg3 PUSCH with intra-slot frequency hopping. Therefore, additional DMRS configuration should be specified for inter-slot frequency hopping. In our understanding, ‘pos2’ should be used for inter-slot frequency hopping.

When transmitted PUSCH is neither scheduled by DCI format 0_1/0_2 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, nor corresponding to a configured grant, nor being a PUSCH for Type-2 random access procedure, the UE shall use single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 0 and the remaining REs not used for DM-RS in the symbols are not used for any PUSCH transmission except for PUSCH with allocation duration of 2 or less OFDM symbols with transform precoding disabled, additional DM-RS can be transmitted according to the scheduling type and the PUSCH duration as specified in Table 6.4.1.1.3-3 of [4, TS38.211] for frequency hopping disabled and as specified in Table 6.4.1.1.3-6 of [4, TS38.211] for frequency hopping enabled, and If frequency hopping is disabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos2' and up to two additional DM-RS can be transmitted according to PUSCH duration, or If frequency hopping is enabled: - The UE shall assume dmrs-AdditionalPosition equals to 'pos1' and up to one additional DM-RS can be transmitted according to PUSCH duration.

In addition, it’s better to also clarify the time domain pattern of inter-slot FH for Msg3 PUSCH repetition. It seems straightforward to reuse the legacy inter-slot FH for PUSCH repetition type A.

In case of inter-slot frequency hopping, the starting RB during slot is given by: , where is the current slot number within a radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

First round

With above summary, FL would like whether the following proposal is acceptable for all.

- Proposal for Issue#10:

- For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules.

- The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused.

- The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case FH is disabled is reused.

- The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |
| Sharp | We agree in principle. For the first/third bullet, no specification change is expected. |
| CATT | Generally agree. For the 2nd bullet, is it the correct understanding that ‘for Msg3 PUSCH in case FH is disabled is reused’ means ‘intra-slot FH is disabled’? |
| Panasonic | We are fine with FL proposal. |
| Nokia/NSB | Agree in principle. Our understanding of the second bullet is that for Msg3 with repetitions, when FH is enabled, this can only be inter-slot. This implies that intra-slot FH is disabled. This case can then be handled as Rel-16 when (intra-slot) FH for Msg3 is disabled. |
| Intel | We are fine with the proposal in principle. |
| LG | Supportive. |
| OPPO | We are fine with the proposal. |
| WILUS | We support the proposal. |
| China Telecom | Fine with the proposal. |
| Ericsson | Support FL’s proposal |
| ZTE | Fine with the proposal. |
| CMCC | Support. |
| vivo | Fine with the proposal. |
| Qualcomm | Fine with the proposal |
| FL | @CATT and Nokia/NSB: Yes, the intention is to refer to legacy intra-slot FH. The proposal is updated accordingly. @All, It seems all companies could agree or fine with the proposal. I will ask for email approval for Proposal-v1 for Issue#10. - Proposal-v1 for Issue#10: - For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules. - The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused. - The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused. - The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused. |

  - Support of PUSCH repetition for CFRA PUSCH scheduled by RAR UL grant.

[Open] Issue#11:Support of PUSCH repetition for CFRA PUSCH

The following agreements were reached for CFRA PUSCH in RAN1#102e.

Agreements: Enhancement to PUSCH scheduled by RAR UL grant will not consider the optimization specific for CFRA case in NR coverage SI.

Above agreements implies that PUSCH repetition can be supported for CFRA while we will not do any optimization specific for CFRA. However, further clarification is required.

FL would like to highlight that the following points:

- In most places of the NR specifications, it uses ‘a PUSCH scheduled by RAR UL grant’ which includes both Msg3 initial transmission and CFRA PUSCH. That is, there is no differentiation in PHY layer regarding handling of Msg3 PUSCH and CFRA PUSCH. The question is whether we need to differentiate in Rel-17 specs. If differentiation is needed, more spec impacts are expected.

- Based on RAN2 discussion, RAN2 is also wondering whether or not CE UEs can also support repetition for CFRA PUSCH. RAN2 is waiting for RAN1 input regarding this aspect. Similar as RAN1, not supporting repetition of CFRA may also cause more spec impacts in RAN2.

- Technically speaking, a UE may trigger CFRA in RRC connected mode when the UE is in cell edge. Support of CFRA PUSCH is technically beneficial.

- Strictly speaking according to the WI scope, CFRA is out of scope as the WI only targets for Msg3 transmission.

Given this could impact the further discussion in RAN2 and also may have RRC impacts, FL would like to check companies’ views on this aspect.

First round

Based on above input, FL would like to check companies views about whether or not to support PUSCH repetition for CFRA PUSCH, i.e., which alternative do you prefer?

Alt 1: Do not support repetition of CFRA PUSCH.

Alt 2: Support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - No optimization specific for CFRA PUSCH is considered.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |
| Sharp | It’s up to RAN2. |
| CATT | From RAN1’s view, we think it is unnecessary to introduce different handling of Msg3 PUSCH and CFRA PUSCH. However, if RAN2 does not specify related RRC parameters for case of CFRA PUSCH repetition, eventually this will not be supported (due to absent of higher layer parameters). Sow whether to support repetition of CFRA PUSCH is still up to RAN2. |
| Panasonic | We share the CATT’s view. |
| Nokia/NSB | As per agreement, no optimization or spec change should be needed to support CFRA PUSCH specifically. This is a RAN2’s problem. |
| Intel | Share similar view as other companies that this should be handled in RAN2. |
| NTT DOCOMO | Support Alt2. There is no reason to restrict PUSCH repetitions scheduled by RAR UL grant for CFRA. |
| LG | In our view, we think that CFRA PUSCH can be repeated by means of normal PUSCH repetition, so it seems that there is no need to discuss CFRA PUSCH under this Msg3 repetition subject. Since a UE who is indicated CFRA is in RRC CONNECTED state, the UE will have C-RNTI and will be indicated a PUSCH transmission through the DCI scrambled with C-RNTI. A normal PUSCH repetition could be indicated by the DCI with C-RNTI. FL: As Samsung replied below, CFRA PUSCH is scheduled by UL RAR grant not DCI scrambled with C-RNTI. |
| OPPO | Share the view as other companies. It is up to RAN2 discussion. |
| Samsung | To LG, even UE in connected mode and doing CFRA, it will still use RA-RNTI to monitor the feedback, that’s why in some place of RAN1 spec, using RAR UL grant scheduled PUSCH to cover both msg3 and CFRA case; To this proposal, we tends to share the view of CATT, the differentiation of msg3 and CFRA pusch happens in RAN2 mostly, let’s defer this discussion. |
| Ericsson | Support Alt2. We think no differentiation between Msg3 PUSCH and CFRA PUSCH has totally smaller impact on RAN1 and RAN2 standards. |
| ZTE | We are fine with either alternative, and suggest to make a conclusion at RAN1 as it has RRC impacts. |
| CMCC | We support Alt2. No need additional optimization for CFRA PUSCH. |
| vivo | Since repetition of PUSCH scheduled by RAR in CFRA will mainly have impact on RAN2 specs. assuming no additional CFRA preamble partitioning is necessary, it can be up to RAN2 to discuss on whether and how to support this if needed. |
| InterDigital | In the second bullet, “RAN2 is waiting for RAN1 input “ is misleading. This was discussed in RAN2, and most companies preferred not to send an LS to RAN1, as shown in R2-2111346, Q4.1. We prefer Alt2, given the UE is in connected mode and link adaptation can be already in place; the MCS and size of the UL grant in RAR in such case therefore can be selected according to the UE's coverage. Indeed, this optimization is not part of WI scope as there isn’t a msg3. |
| Qualcomm | We support Alt 2 |
| Huawei, HiSilicon | Msg3 is a term within 4-step RACH procedure so there is no Msg3 in CFRA. Alt 2 seems out of scope of the WI. If Alt 2 is supported, then UE capability reporting after initial access is needed and should be agreed together. |

Second round

It seems many companies think this should be up to RAN2 discussion. Meanwhile, a large number of companies point out it is not necessary to differentiate between Msg3 and CFRA PUSCH from RAN1 perspective, and many of them prefer to discuss in RAN1 with supporting Alt 2.

Regarding whether to make decision in RAN1 or RAN2, it seems we are in circles. Based on RAN2 discussion in R2-2111346, it was discussed whether to send an LS to RAN1 about support of repetition for CFRA. However, it ends up with no consensus for sending the LS. Finally, RAN2 FL suggests not to ask RAN1, and they will wait for RAN1 inputs. So, we have to make decision instead of kicking balls between RAN1 and RAN2. Considering RAN1 is the leading WG, FL feels we RAN1 has responsibility to resolve the dilemma in such situation.

Based on the following, FL suggests to go with Alt 2 with some necessary updates to address the concerns from opponents.

  - Majority companies think it is unnecessary to introduce different handling of Msg3 PUSCH and CFRA PUSCH

  - A UE may trigger CFRA in RRC connected mode when the UE is in cell edge. Support of CFRA PUSCH is technically beneficial.

  - It could save spec impacts.

  - Based on companies’ input, it seems no need to define separate preambles for repetition of CFRA PUSCH if UE capability is reported in idle/connected mode. Because, the preambles for CFRA is dedicated, and gNB can choose to schedule repetition or not if gNB knows the UE has corresponding capability.

Proposal for Issue#11:

Support repetition for a PUSCH scheduled by RAR UL grant, including both Msg3 PUSCH and CFRA PUSCH.

  - Use the same mechanism of Msg3 PUSCH repetition, when applicable, for CFRA PUSCH.

    - No separate CFRA preamble/RO for repetition of CFRA PUSCH is introduced.

    - No additional optimization specific for CFRA PUSCH is considered.

Companies are encouraged to provide your comments if any below.

| Company | Comments |
| --- | --- |

  - Other issues

Support of joint channel estimation for Msg3 repetition

[19, CMCC] and [20, NTT DOCOMO] propose to support joint channel estimation for Msg3 repetition

[12, Qualcomm]: If JCE is supported for Msg3 PUSCH repetitions with subject to power consistency and phase continuity requirements, only back-to-back PUSCH transmission is supported and the UE indicates its capability of supporting JCE during RACH procedure. UE is not expected to monitor Msg2 PDCCH between the repetitions of Msg3. gNB should be able to expect no UL beam switching among repetitions of Msg3. Support transmission of PTRS inside Msg3 repetitions.

==> FL: This has been discussed before and no consensus has been reached. No further discussion would be pursued, meaning it will not be supported in Rel-17, unless any critical motivation is identified with consensus in further meetings.

Conditions of requesting Msg3 repetition

[11, Apple]: UE is implicitly indicated, based on relative values of TS and TR to select the UL carrier and whether on that carrier repetition is demanded or not.

- If TS < TR, UE is indicated once SUL is selected (Tp < TS), UE shall demand Msg3 repetition on SUL

- If TS > TR, UE is indicated once SUL is selected (TR < Tp < TS,), UE shall not demand Msg3 repetition on SUL

[12, Qualcomm]: The RSRP threshold depends on SS-RSRP and/or UE power class.

[14, Samsung]: The RSRP threshold should be explicitly configured and indicated by gNB.

[22, LG]:The RSRP threshold is determined by the UE side considering on UE capability (e.g., UE maximum Tx power, UE Tx beam gain, number of UE Tx antenna.)

==> FL: RAN2 has already agreed to introduce a separate RRC parameter for rsrp-ThresholdSSB threshold for requesting Msg3 repetition. Further details are under discussion in RAN2, and no further action in RAN1 is needed unless any clarification/request from RAN2.

Early termination of Msg3 repetition

In Rel-15/16 RACH procedure, a UE starts the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission. The UE shall monitor PDCCH for Contention Resolution while the ra-ContentionResolutionTimer is running. If Msg3 repetition is enabled, then it needs to discuss whether to support early termination, i.e., whether the ra-ContentionResolutionTimer can start or re-start after each repetition.

[14, Samsung]: Support starting	DCI Monitoring after the end of the first msg3 transmission.

==> FL: RAN2 has already agreed that ra-ContentionResolutionTimer is started or restarted in the first symbol after all Msg3 repetitions.

Support of qam64-LowSE MCS

In [8, Xiaomi]: QAM64-LowSE MCS table provides lower coding rate, which is benefit for Msg.3 coverage enhancement with lower required SNR. So, QAM64-LowSE MCS table can be used for Msg.3 transmission in bad coverage. Therefore, it proposes to support the use of QAM64-LowSE MCS table for Msg.3 transmission with repetitions.

Support of a separate maximum number of preamble transmissions for CE UEs without msg.3 repetition requesting.

[8, Xiaomi] proposes to convert 4-step RACH without msg.3 repetition requesting into 4-step RACH with msg.3 repetition requesting for CE UEs when the failure of 4-step RACH without msg.3 repetition requesting happens, and configure a separate maximum number of preamble transmissions for CE UEs without msg.3 repetition requesting.

Waveform indication for Msg3

[11, Apple]: Specify a UE-specific procedure to enable/disable transformprecoder for Msg3 transmission, via:

- Alt1: explicit indication, e.g., repurpose some bits in RAR UL grant (for initial Msg3 transmission) or DCI format 0_0 with CRC scrambled by TC-RNTI (for Msg3 retransmission) to indicate whether transformprecoder enabled or not

- Alt2: implicit indication, for example, transformprecoder is enabled if UE indicates to require coverage enhancement/recovery.

Spatial domain transmission relation

[12, Qualcomm]: Consider one of the following options on spatial domain transmission relation for Msg3 PUSCH transmission:

- Option 1: The UE transmits the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the same spatial domain transmission relation.

- Option 2: The UE may transmit the Msg3 PUSCH repetitions within a transmission (initial transmission or re-transmission) using the different spatial domain transmission relations.

[14, Samsung]: The repetitions for the msg3 PUSCH transmission that is scheduled by RAR use the same beam (spatial setting) as the one for the corresponding PRACH transmission. On the other hand, the UE can select the beam for msg3 re-transmissions.

Support of additional C-RNTI and HARQ-ACK resource for MSG.4 PDSCH

[24, vivo] observes that NW may decode MSG.3 PUSCH from multiple UEs, which have transmitted the same preamble on the same RO, if MSG.3 PUSCH repetition is introduced. It is beneficial to support contention resolution for multiple UEs simultaneously to reduce the access delay of CBRA procedure, if MSG.3 PUSCH repetition is supported. Therefore, it proposes additional (T)C-RNTI can be provided UE to support contention resolution for multiple UEs without initiating a new RACH attempt.

- Proposals for GTW session

Proposal for Issue#1:

- For indication of the number of repetitions of Msg3 initial transmission, Alt 1 (i.e., using TDRA information field) is adopted.

Alternative proposal for Issue#1:

- For indication of the number of repetitions of Msg3 initial transmission, Alt 2 (i.e., using MCS information field) is adopted.

Table 2.1-1 Summary of companies’ views for repetition indication for Msg3

| Alternative | Support | Concern | Can live with | Detailed views |
| --- | --- | --- | --- | --- |
| Alt 1. TDRA | Sharp, CATT, Ericsson, vivo, Xiaomi, Panasonic, ZTE, CMCC, OPPO, InterDigital, ETRI, LG, Spreadtrum Communication, [Lenovo/Motorola Mobility], [NEC] （15） |  | DCM | Pros: - Use similar mechanism defined for regular PUSCH repetition in Rel-16, which may require less spec efforts - gNB can choose to schedule Msg3 repetition by less number of PRBs with high MCS index to save frequency domain resources. Cons: - Impact the flexibility of time domain resource indication. - It may or may not impact the flexibility of time domain resource indication for legacy UEs, depending on NW configuration. |
| Alt 2. MCS | Apple, Intel, Samsung, Huawei/HiSilicon Nokia/NSB, DCM, China Telecom, WILUS （8） |  |  | Pros: - May have less impact to legacy UEs - For Msg3 repetition, gNB can choose to use default TDRA table to save SIB1 signaling. - There is no clear performance loss in case the payload size is larger than 72 bits and the number of repetitions is smaller than 4. Cons: - Impact the flexibility of MCS indication; - The MCS filed length are 4 bits in RAR UL grant and 5 bits in DCI format 1-0 with TC-RNTI. It would require additional effort to discuss how to repurpose MCS bit field for Msg3 re-transmission. - When the number of repetitions is equal to or larger than 4, limiting the MCS index no larger than 3 could degrade the performance (~0.7 dB in average) for payload size smaller than 72 bits. This is because using more repetitions could decrease the effective coding gain from using low MCS index while PSD reduction due to using more PRBs would become more evident. |

Proposal for Issue#3: For repetition indication for Msg3 re-transmission, Option 1 (i.e., use the same mechanism as supported for Msg3 initial transmission) is adopted.

- Support: Huawei, HiSilicon, OPPO, ZTE, vivo, CATT, Xiaomi, InterDigital, Panasonic, Samsung (if initial transmission adopts TDRA based method), ETRI, Ericsson, NTT DOCOMO, LG

- Option 2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI.

- Support: Intel, Apple, Qualcomm, Samsung (if initial transmission doesn’t adopt TDRA based method), WILUS

- Proposals for email approval

Proposal 5 for Issue#5: Flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and not overlapped with SSB symbols indicated by ssb-PositionsInBurst can be regarded as available symbols for Msg3 PUSCH repetition.

Note: whether and how to introduce other potential mechanisms to use the flexible symbols are separately discussed.

Note: The Rel-15/16 rules are reused for collision handling between Msg3 PUSCH transmission and a CORESET for Type0-PDCCH CSS set indicated to a UE by pdcch-ConfigSIB1 in MIB in a set of flexible symbols indicated by tdd-UL-DL-ConfigurationCommon.

Proposed conclusion: There is no consensus to additionally introduce explicit indication to indicate whether or not flexible slots/symbols configured via TDD-UL-DL-Configcommon are available for Msg3 repetition.

- Proposal for Issue#9: RV cycling for Msg3 PUSCH repetition is based on transmission occasions on available slots.

- Proposal-v1 for Issue#10:

- For inter-slot FH for Msg3 PUSCH repetition, adopt the following legacy rules.

- The Rel-16 RB offset determination mechanism defined in Table 8.3-1 of TS 38.213 for intra-slot FH for Msg3 PUSCH is reused.

- The Rel-16 additional DMRS configuration defined in Clause 6.2.2 of TS 38.214 for Msg3 PUSCH in case intra-slot FH is disabled is reused.

- The Rel-16 inter-slot FH pattern defined in Clause 6.3.1 of TS 38.214 for PUSCH repetition type A is reused.

Appendix-A: Previous agreements

RAN1#104-e

Agreements: - For indication of the number of repetitions for Msg3 initial transmission, down-select one option from the options below. - Option1: UL grant scheduling Msg3. - FFS details. - FFS fallbackRAR UL grant. - Note: Optimization specific for fallbackRAR UL grant in 2-step RACH is not considered in Rel-17 CovEnh WI, if supported. - Option2: DCI format 1_0 with CRC scrambled by RA-RNTI - FFS details. - Option3: SIB1 only - Any modifications of RAR UL grant or DCI format 1_0 with CRC scrambled by RA-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the RAR or DCI format 1_0 with CRC scrambled by RA-RNTI respectively Agreements: - For indication of the number of repetitions for Msg3 re-transmission, down-select one option from the options below. Option1: DCI format 0_0 with CRC scrambled by TC-RNTI. FFS details. Any modifications of DCI format 0_0 with CRC scrambled by TC-RNTI for indicating Msg3 repetitions shall not impact the legacy UE interpretation of the DCI format 0_0 with CRC scrambled by TC-RNTI. Option2: Can be determined based on the repetition number for Msg3 initial transmission Agreements: Support inter-slot frequency hopping for repetition of Msg3 initial and re-transmission. FFS details, e.g., signaling etc. Agreements: For Msg3 PUSCH repetition, the following options are considered, aiming for down-selection in RAN1#104b-e: - Option 1-1: For gNB scheduled Msg3 PUSCH repetition without UE request, - A UE indicates to support of Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - For a UE supporting Msg3 PUSCH repetition, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - FFS details if any. - Option 1-2: For gNB scheduled Msg3 PUSCH repetition without UE request, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - For UE does not support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH without repetition - For UE does support Msg3 PUSCH repetition, UE transmits Msg3 PUSCH with repetition as indicated by gNB and UE uses, e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Note: e.g., this can be for differentiation between UEs not supporting Msg3 PUSCH repetition and Rel-17 CE UEs supporting Msg3 PUSCH repetition or between RACH procedure with Msg3 PUSCH repetition and Msg3 PUSCH without repetition, etc. - gNB blindly decodes Msg3 PUSCH with two different assumptions, w/ and w/o repetition. - FFS details if any. - Option 2-1: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions. - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is triggered by UE, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS details if any. - Option 2-2: For UE triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions. - If Msg3 PUSCH repetition is scheduled, UE transmits Msg3 PUSCH with or without repetition. If UE transmits Msg3 PUSCH repetition, the number of repetition follows the indication of gNB and UE uses e.g., separate DMRS configuration or UCI multiplexing with Msg3 PUSCH (or other ways) - Whether a UE would trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - FFS details if any. - Other options are not precluded.

RAN1#104b-e

Agreement: For Msg3 PUSCH repetition, support the following modified Option 2-1. - Option 2-1: For UE requested triggered Msg3 PUSCH repetition with gNB indicating the number of repetitions, - A UE can request trigger RACH procedure with Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.). - Whether a UE would request trigger is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact. - If Msg3 PUSCH repetition is requested triggered by UE, gNB decides whether to schedule Msg3 PUSCH repetition or not. If scheduled, gNB decides the number of repetitions for Msg3 PUSCH 3 (re)-transmission. - FFS the UE capability of supporting Msg3 PUSCH repetition can be reported after initial access procedure as usual - FFS details if any. Agreements: For the determination of RV for Msg3 PUSCH repetition, - RV of the first repetition is determined in the same way as legacy. - Use RV 0 for the first repetition of Msg3 PUSCH initial transmission. - Use a dynamically indicated RV id via DCI 0_0 with CRC scrambled by TC-RNTI for the first repetition of Msg3 PUSCH re-transmission. - FFS determination of the RV sequence. Agreements: For indication of the number of repetitions for Msg3 initial transmission, Option 1 (i.e., using UL grant scheduling Msg3) is adopted. - FFS additionally using MAC RAR for indication. Agreements: For indication of the number of repetitions for Msg3 re-transmission, Option 1 (i.e., using DCI format 0_0 with CRC scrambled by TC-RNTI) is adopted. Working assumption: The number of repetitions is counted on the basis of available slots for Type A PUSCH repetitions for Msg3. - FFS: the determination of available slots.

RAN1#105-e

Agreement: A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. - FFS the determination of the RSRP threshold. Agreement: - For requesting Msg3 PUSCH repetition, support the following: - Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs. - FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE. - FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH). - FFS whether or not to additionally support one (& only one) more option: - E.g., option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs - E.g., Option 3: Use separate RO, which include - the separate RO configured by a separate RACH configuration index from legacy UE, and - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission. Working assumption: - Using an information field from the existing information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission - Down-select only one from the following information fields in RAR UL grant for indication of the number of repetition of Msg3 initial transmission. - TDRA information field with introducing a new TDRA table including the repetition factors. - MCS information field - TPC information field - CSI request information field - FDRA information field - The total size of RAR UL grant does not change. - Position of all fields in the bit sequence of the RAR UL grant does not change, regardless of whether they are repurposed or not. - FFS details, e.g., TDRA table selection, or whether/how to indicate which interpretation UE should use for the repurposed information field (legacy vs repurposed interpretation) etc. Agreement: For repetition indication of Msg3 re-transmission, select one options from the following two options. - Option 1: Use the same mechanism as supported for Msg3 initial transmission. - Option2: Use HARQ process number bit field in DCI format 0_0 with CRC scrambled by TC-RNTI. Agreement: Use a fixed RV sequence [0 2 3 1] for repetition of Msg3 initial and re-transmission. - The RV cycling for Msg3 initial transmission follows the rule specified in the first row in Table 6.1.2.1-2 in TS38.214. - The RV cycling for Msg3 re-transmission follows the rules specified in Table 6.1.2.1-2 in TS38.214. - FFS: The RV cycling for Msg3 is based on transmission occasions on available slot. Conclusion: - Companies are encouraged to perform additional evaluations regarding intra-slot frequency hopping for Msg 3 with repetition. Aim to conclude whether or not to support this feature in RAN1#106-e (note: if supported, the intention is to not configure intra- and inter-slot frequency hopping simultaneously) Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on dynamic SFI in DCI format 2-0. Agreement: Available slots for Msg3 PUSCH repetition do not depend on tdd-UL-DL-ConfigurationDedicated. Agreement: Available slot for Msg3 PUSCH repetition doesn’t depend on UL CI. Agreement: Available slot for Msg3 PUSCH repetition depends on TDD-UL-DL-Configcommon. - A slot is determined as available for Msg3 repetition only if the consecutive symbols allocated for Msg3 repetition in the slot are all available symbols. - UL symbols indicated by TDD-UL-DL-Configcommon are determined as available for Msg3 repetition. - FFS whether and how to use flexible symbols indicated by TDD-UL-DL-Configcommon.

RAN1#106-e

Agreement Do NOT support fallback RAR UL grant in 2-step RACH for indicating Msg3 repetition. Agreement The separate preambles for requesting Msg3 repetition could be configured only in an RO configured with 4-step RACH preambles not for requesting Msg3 repetition. Working Assumption Down-select only one from the following methods for indication of the number of repetition of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, introducing a new configurable TDRA table including the repetition factors. - The new TDRA table is configured by SIB1, with selecting one of the two options below. - Option 1: The new TDRA table includes separate new indication for K2, mapping type, SLIV and repetition factor. - Option 2: The new TDRA table includes legacy indication for K2, mapping type and SLIV from legacy TDRA table, and new indication for repetition factor. - If a new TDRA table is not configured, the legacy default TDRA table is used, and repetition factor K=1 is applied. - K=1. - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - X MSB bits of the MCS information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Alt 3: If TPC information field is chosen, repurpose the TPC information field by selecting one of the two options below. - Option 1: X LSB bits of the TPC information field are used for repetition indication. - FFS the value of X. - FFS whether the X bits are directly used for indicating the repetition factor (i.e., the decimal value of X is equal to the repetition factor) or used for selecting one repetition factor from a predefined/SIB1 configured set. - Option 2: A predefined TPC command table with including repetition factor K is introduced. - FFS details. Agreements Down-select one of the two options on how a UE should interpret the selected information field for indication of the number of repetitions. Option 1: - When a UE requests Msg3 repetition, the new TDRA table or repurposed information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy TDRA table or legacy information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Option 2: - When a UE requests Msg3 repetition, gNB schedules Msg3 with or without repetition by respectively using the new TDRA table or legacy TDRA table; or gNB schedules Msg3 with or without repetition by respectively using repurposed information field or legacy interpretation of information field. Whether the UE should apply the new or the legacy TDRA table, or apply repurposed or legacy interpretation of the information field, is indicated by gNB. - FFS details, e.g. implicit or explicit indication or predefined. - Repetition factor K=1 is NOT included in the TDRA table or one entry/codepoint of the repurposed information field. - When the UE doesn’t request Msg3 repetition (including legacy UE), gNB schedules Msg3 without repetition. The UE applies the legacy TDRA table, or the legacy interpretation of the information field. Agreement - Support at least repetition factor K = {2, 4} for Msg3 PUSCH repetition. - FFS whether to support other values, e.g., 8. - Note: K=1 is supported and how to support K=1 is FFS. Agreement - The available slot of Msg3 PUSCH repetition is only determined by the tdd-UL-DL-ConfigurationCommon and ssb-PositionsInBurst, no other additional Rel-16 signals/signalings will be considered. - If a symbol for Msg3 repetition in a slot overlaps with SSB transmission [FFS:N Gap symbols after SSB], the slot is determined as not available during the counting of repetitions. As there is no Msg3 repetition in the slot, no Msg3 repetition omission applies to the slot. Agreements: Do not support TBoMS for Msg3 in Rel-17 coverage enhancement WI.

RAN1#106bis-e

Working Assumption Down-select only one from the following methods for indication of the number of repetitions of Msg3 initial transmission. - Alt 1: If TDRA information field is chosen, Option 2 is supported. - The candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} - Alt 2: If MCS information field is chosen, repurpose the MCS information field as follows. - 2 MSB bits of the MCS information field are used for selecting one repetition factor from a SIB1 configured set with 4 candidate values. - The set of candidate values for repetition factor could be chosen from {[1], 2, 3, 4, 7, 8, [12], [16]} Note: Whether ‘1’ is included depends on the outcome of interpretation of the selected information field. Conclusion There is no consensus to additionally support intra-slot frequency hopping for Msg3 PUSCH with repetition in Rel-17. Note: intra-slot FH is supported when a UE is scheduled Msg3 PUSCH without repetition. Agreement If UE is indicated with Msg3 PUSCH with repetition, the frequency hopping flag information field in UL RAR grant or DCI format 0_0 with CRC scrambled by TC-RNTI is reused to enable/disable inter-slot frequency hopping. Agreement The Rel-15/16 Msg3 PUSCH collision handling rules are reused for transmission of Msg3 PUSCH repetition in an available slot. - FFS whether collision with downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated is an exceptional case, i.e., Msg3 PUSCH repetition cannot be canceled by downlink symbols indicated by tdd-UL-DL-ConfigurationDedicated in Rel-17. - FFS: Rel-17 Msg3 PUSCH collision rules are also applied if introduced in other WI(s) Agreement Include the following into the reply LS to R1-2108712(R2-2109195). RAN1 thinks at least the number of preambles per SSB per RO for request of Msg3 repetition, i.e., CB-PreamblesPerSSB, is needed. It’s up to RAN2 whether to indicate the start of preamble index for request of Msg3 repetition with shared RO. Agreement Include the following into the reply LS to R1-2108712(R2-2109195). - From RAN1 perspective, there is no need to separately configure the following legacy RACH parameters configured in RACH-ConfigCommon for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier. - prach-ConfigurationIndex - msg1-FDM - msg1-FrequencyStart - zeroCorrelationZoneConfig - totalNumberOfRA-Preambles - ssb-perRACH-OccasionAndCB-PreamblesPerSSB - FFS: rsrp-ThresholdSSB - rsrp-ThresholdSSB-SUL - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig - msg3-transformPrecoder Agreement Include the following into the reply LS to R1-2108712(R2-2109195) - From RAN1 perspective, it can be beneficial to separately configure rsrp-ThresholdSSB for requesting Msg3 PUSCH repetition with shared RO on a given UL carrier.

Appendix-B: Legacy rules for use of flexible symbol for Msg3 transmission

- FL’s understanding about Rel-15/16 rules for use of flexible symbol for Msg3 transmission is summarized below.

If SFI is not configured, the following symbols are available symbols for Msg3 transmission according to current specification. - 1) Uplink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated - 2) Flexible symbols indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided. - If a UE is only provided by tdd-UL-DL-ConfigurationCommon, and a symbol is indicated as flexible symbol by tdd-UL-DL-ConfigurationCommon, it is a common understanding that the flexible symbol can be used for Msg3 transmission. - If a UE is provided by both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, and a symbol is indicated as flexible symbol by both tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, the flexible symbol is available for Msg3 transmission. If a UE is not configured to monitor PDCCH for DCI format 2_0, for a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 1_0, DCI format 1_1, or DCI format 0_1 - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format 0_0, DCI format 0_1, DCI format 1_0, DCI format 1_1, DCI format 2_3, or a RAR UL grant. Meanwhile, the following symbols are not available for Msg3 transmission if SFI is not configured. - 1) Downlink symbols indicated by tdd-UL-DL-ConfigurationCommon, or by tdd-UL-DL-ConfigurationDedicated For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.. - 2) Symbols configured for SSB transmission For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.. - Note that, a symbol for a CORESET for Type0-PDCCH CSS set indicated by pdcch-ConfigSIB1 in MIB can be indicated as flexible symbol by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, which can be used for Msg3 transmission. In other words, as long as it is a flexible symbol indicated by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, it can be used for Msg3 transmission. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated..

If dynamic SFI is configured, the Rel-15/16 legacy UE behavior for collision handling of Msg3 transmission is summarized below. - If dynamic SFI is configured, a UE does not expect collision between Msg3 transmission and SFI indication. - If dynamic SFI is configured and the DCI format 2_0 is detected by UE, the flexible symbols indicated by the DCI format 2_0 are available symbols for Msg3 transmission. - If dynamic SFI is configured and while DCI format 2_0 is not detected by UE, the flexible symbols indicated by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided are available symbols for Msg3 transmission. The related specification texts are also copied below. For a set of symbols of a slot, a UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols in the slot as downlink and to detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot. For a set of symbols of a slot indicated to a UE as flexible by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE detects a DCI format 2_0 providing a format for the slot using a slot format value other than 255 - ... - if an SFI-index field value in DCI format 2_0 indicates the set of symbols of the slot as flexible and the UE detects a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot the UE transmits the PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot - a UE does not expect to detect an SFI-index field value in DCI format 2_0 indicating the set of symbols of the slot as downlink and also detect a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR indicating to the UE to transmit SRS, PUSCH, PUCCH, or PRACH, in one or more symbols from the set of symbols of the slot - ... For a set of symbols of a slot that are indicated as flexible by tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated if provided, or when tdd-UL-DL-ConfigurationCommon, and tdd-UL-DL-ConfigurationDedicated are not provided to the UE, and if the UE does not detect a DCI format 2_0 providing a slot format for the slot - the UE receives PDSCH or CSI-RS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format - the UE transmits PUSCH, PUCCH, PRACH, or SRS in the set of symbols of the slot if the UE receives a corresponding indication by a DCI format, a RAR UL grant, fallbackRAR UL grant, or successRAR.

Regarding collision with UL CI, it was agreed that RACH related UL transmissions cannot be cancelled by UL CI in RAN1#98bis.

Agreements: - SRS can be cancelled by UL CI - PUCCH cannot be cancelled by UL CI - RACH related UL transmissions cannot be cancelled by UL CI, including MSG 1/3 in case of 4-step RACH, MSG A in case of 2-step RACH.

The related spec is:

An indication by a DCI format 2_4 for a serving cell is applicable to a PUSCH transmission or an SRS transmission on the serving cell. If the PUSCH transmission or the SRS transmission is scheduled by a DCI format, the indication by the DCI format 2_4 is applicable to the PUSCH transmission or SRS transmission only if the last symbol of the PDCCH reception providing the DCI format is earlier than the first symbol of the PDCCH reception providing the DCI format 2_4. For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after T'proc,2 from the end of a PDCCH reception where the UE detects the DCI format 2_4, where T'proc,2 is obtained from Tproc,2 for PUSCH processing capability 2 [6, TS 38.214] assuming d2,1=doffset⋅2-μUL2-μ where doffset is provided by delta_Offset, μ being the smallest SCS configuration between the SCS configuration of the PDCCH and the smallest SCS configuration μUL provided in scs-SpecificCarrierList of FrequencyInfoUL or FrequencyInfoUL-SIB. The UE does not expect to cancel the PUSCH transmission or the SRS transmission before a corresponding symbol that is Tproc,2 assuming that d2,1=0 after a last symbol of a CORESET where the UE detects the DCI format 2_4.

Appendix-C: Evaluation comparison between Alt 1 and Alt 2

[1, Huawei, HiSilicon]: Form Figure 1, it can be observed that, when the TB size carried by Msg3 PUSCH is about 72 bits and the number of repetitions is smaller than 4, the performance of (MCS1, 2PRB) and (MCS0,3PRB) is better than or approximate to (MCS4, 1PBR) and (MCS5, 1PRB). When the TB size carried by Msg3 PUSCH are is about 72bits and the number of repetitions is 4 or 8, the performance of (MCS1, 2PRB) and (MCS0, 3PRB) decreases slightly compared with that of (MCS4, 1PBR) and (MCS5, 1PRB). When TB size carried by Msg3 PUSCH are 144bits and 208bits, and the number of repetitions is one of {1,2,4,8}, the performance of (MCS2, 3PRBs), (MCS0, 6PRBs), (MCS1, 5PRB) and (MCS2, 5PRBs) are better than or approximate to the performance of (MCS4, 2PRB), (MCS5, 2PRBs), (MCS6, 2PRB) and (MCS7, 2PRBs). The above observations show that, thanks to the coding gain of the lower coding rate, when Msg3 carrying same payload is transmitted, larger PRB number, lower PSD and lower MCS do not seem to cause performance degradation compared to smaller PRB number, larger PSD and higher MCS. Therefore, for coverage limited UE, the gNB will prefer to schedule lower MCS and MCS 0~3 are sufficient for Msg3 PUSCH transmission. Observation 4: For power limited UEs with about 72-bit TB size of Msg3 PUSCH, compared to the performance of (MCS4, 1PBR) and (MCS5, 1PRB), - the performance of (MCS1, 2PRB) and (MCS0, 3PRB) is better or has no obvious degradation when the number of repetitions is smaller than 4; - the performance of (MCS1, 2PRB) and (MCS0, 3PRB) has no obvious degradation when the number of repetitions is 4 or 8. Observation 5: For power limited UEs with both about 144-bit and 208-bit TB size of Msg3 PUSCH, compared to the performance of (MCS4, 2PRB), (MCS5, 2PRBs), (MCS6, 2PRB) and (MCS7, 2PRBs), the performance of (MCS2, 3PRBs), (MCS0, 6PRBs), (MCS1, 5PRB) and (MCS2, 5PRBs) are better than or has no obvious degradation when the number of repetitions is any of {1,2,4,8}. Observation 6: Thanks to the coding gain of the lower coding rate, when the same payload is transmitted in Msg3 without repetition for power limited UEs, a scheduling setting of larger PRB number, lower PSD and lower MCS does not have to cause performance degradation compared to a scheduling setting of smaller PRB number, larger PSD and higher MCS. Observation 7: For power limited UE, when the number of Msg3 repetitions is smaller than 4, the MCS set of MCS 0~3 is sufficient to provide the same performance as the full MCS set, irrespective of TB size. Observation 8: For power limited UE, when the TB size of Msg3 is not smaller than 144 bits, the MCS set of MCS 0~3 is sufficient to provide the same performance as the full MCS set, irrespective of repetition numbers. Observation 9: For power limited UE, when the TB size of Msg3 is 72 bits and the number of Msg3 repetitions is 4 or 8, another MCS set of MCS 4~7 is sufficient to provide the same performance as the full MCS set, but its performance is very close to the MCS set of MCS 0~3. Proposal 1: MCS information field in RAR UL grant is adopted to indicate the repetition number for Msg3 initial transmission. - TB ~ 72bits, L=14 (b) TB ~ 144bits, L=14 (c) TB ~ 208bits,L=14 - TB ~ 72bits, L=12 (e) TB ~ 144bits, L=12 (f) TB ~ 208bits,L=12 Figure 1. Evaluation results for Msg3 PUSCH carrying large TB Table 2: Simulation parameter setting for Msg3 The number of allocated symbols L=14. ~72 bits; ~144 bits; ~208 bits. PRBs; MCS#; PRBs; MCS#; PRBs; MCS#. Case 1; 1; 4; 2; 4; 2; 6. Case 2; 2; 1; 3; 2; 5; 1. Case3; -; -; 5; 0; -; -. The number of allocated symbols L=12. Case 1; 1; 5; 2; 5; 2; 7. Case 2; 3; 0; 4; 2; 4; 3. Case 3; -; -; 6; 0; 5; 2. Table 3: TB size for Msg3 L=14. MCS0; MCS1; MCS2; MCS3; MCS4; MCS5; MCS6; MCS7. 1RB; 24; 40; 48; 64; 72; 96; 112; 128. 2RB; 56; 80; 96; 128; 152; 192; 224. 3RB; 88; 120; 144; 192; 240. 4RB; 120; 160; 192; 256. 5RB; 152; 208. 6RB; 184. L=12. 1RB; 24; 32; 40; 48; 64; 72; 88; 128. 2RB; 48; 64; 80; 104; 128; 152; 184; 224. 3RB; 72; 96; 120; 152; 192; 240. 4RB; 96; 128; 160; 208. 5RB; 120; 160; 208. 6RB; 144; 192; 240.

[4, ZTE]: Observation 1: Alt 1(TDRA based solution) has better scheduling flexibility with either using Option 1 (a minimum number of PRBs and the corresponding MCS index could be larger than 3), or Option 2 (a larger number of PRBs to ensure the MCS index no larger than 3), while Alt 2 can only use Option 2. Observation 2: For Msg3 PUSCH repetition, Alt 1 (TDRA based solution) could provide better performance by enabling scheduling with Option 1 (a minimum number of PRBs and the corresponding MCS index could be larger than 3). ==>FL note: The assumed repetition factor K is 4 and 8 for Urban and Rural scenario respectively. Table 1. The results for TBsize 72 bits Option1; Option2; Link budget Gain of Option 1 over Option 2. (#of RBs, MCS); Required SNR; Power normalization to 1 PRB; (#of RBs, MCS); Required SNR; Power normalization to 1 PRB. Rma, L=14; (1 RB, MCS#4); -8.63 dB; -8.63 dB; (2 RBs, MCS#1); -11.3 dB; -8.29 dB; 0.34 dB. Rma, L=12; (1 RB, MCS#5); -7.98 dB; -7.98 dB; (3 RBs, MCS#0); -12.11 dB; -7.33 dB; 0.65 dB. Uma, L=14; (1 RB, MCS#4); -8.8 dB; -8.8 dB; (2 RBs, MCS#1); -11.9 dB; -8.89 dB; -0.09 dB. Uma, L=12; (1 RB, MCS#5); -8.15 dB; -8.15 dB; (3 RBs, MCS#0); -12.63 dB; -7.86 dB; 0.29 dB. Table 2. The results for TBsize 144 bits Option1; Option2; Link budget Gain of Option 1 over Option 2. (#of RBs, MCS); Required SNR; Power normalization to 1 PRB; (#of RBs, MCS); Required SNR; Power normalization to 1 PRB. Rma, L=14; (2 RBs, MCS#4); -8.79 dB; -5.78 dB; (4 RBs, MCS#1); -11.41 dB; -5.39 dB; 0.39 dB. Rma, L=12; (1 RB, MCS#5); -7.99 dB; -4.97 dB; (3 RBs, MCS#0); -12.38 dB; -4.6 dB; 0.37 dB. Uma, L=14; (2 RBs, MCS#4); -9.18 dB; -6.16 dB; (4 RBs, MCS#1); -11.9 dB; -5.87 dB; 0.29 dB. Uma, L=12; (1 RB, MCS#5); -8.51 dB; -5.5 dB; (3 RBs, MCS#0); -12.63 dB; -4.85 dB; 0.65 dB. Table 3. The results for TBsize 208 bits Option1; Option2; Link budget Gain of Option 1 over Option 2. (#of RBs, MCS); Required SNR; Power normalization to 1 PRB; (#of RBs, MCS); Required SNR; Power normalization to 1 PRB. Rma, L=14; (2 RBs, MCS#6); -6.92 dB; -3.91 dB; (5 RBs, MCS#1); -10.73 dB; -3.74 dB; 0.17 dB. Rma, L=12; (2 RBs, MCS#7); -6.19 dB; -3.18 dB; (7 RBs, MCS#1); -11.36 dB; -2.9 dB; 0.28 dB. Uma, L=14; (2 RBs, MCS#6); -7.53 dB; -4.52 dB; (5 RBs, MCS#1); -11.27 dB; -4.28 dB; 0.24 dB. Uma, L=12; (2 RBs, MCS#7); -6.82 dB; -3.81 dB; (7 RBs, MCS#1); -12.11 dB; -3.65 dB; 0.16 dB.

- [16, Nokia/NSB] - Keeping MCS index below 4 does not hinder the performance of the system if the number of repetitions is not larger than 2. - It can be inferred from the results that configuring Alt. 2 with MCS index 2 and MCS index 3 can provide better performance than Alt. 1 also for number of repetitions larger than 2. - The larger the payload size, the more Option 2 outperforms Option 1, if the number of repetitions is not larger than 2. - This is because the beneficial impact of the lower MCS index set for Option 2 is visible as long as the effective coding gain of Alt. 1 is not competitive. As the number of repetitions increases. The effective coding gain of Alt. 1 becomes more competitive, and the effect of the PSD reduction when a larger number of PRBs are configured becomes more evident. - Further studies would be necessary for larger repetition numbers, however the results during the SI showed that configuring a repetition number larger than 4 may not be needed in a large majority of the cases. - Performance difference is never very large. The two alternatives slightly outperform one another, depending on the considered configuration. In this context, it is observed that the performance difference a bit larger when Alt. 2 outperforms Alt. 1 than when Alt. 1 outperforms Alt. 2. Number of Msg3 repetitions; Alt. 1; Alt. 2; Gap [dB]. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 5.63; 5.63; 1.69; 4.7; 0.93. L=12; 1; 6.88; 6.88; -0.5; 4.27; 2.61. L=10; 1; 9; 9; 0.5; 5,27; 3.73. L=14; 2; 0.75; 0.75; -1.88; 1.13; -0.38. L=12; 2; 1.63; 1.63; -3; 1.77; -0.14. L=10; 2; 3.25; 3.25; -2; 2.77; 0.48. L=14; 4; -2.5; -2.5; -3.8; -0.74; -1,76. L=12; 4; -1.88; -1.88; -6.3; -1,48; -0.4. L=10; 4; -0.88; -0.88; -4; 0,77; -1.65. Table 2- Evaluation results of Alt. 1 and Alt. 2 when TBS=72. A summary of evaluation results when TBS=144, TBS=208, TBS=282, TBS=480 and TBS=640 is provided in Table 4, Table 5, Table 6, Table 7 and Table 8, respectively. Number of Msg3 repetitions; Alt 1; Alt 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 4.25; 7.26; 0.63; 6.65; 0.61. L=12; 1; 5.44; 8.45; -0.88; 6.91; 1,54. L=10; 1; 6.88; 9.89; 0.38; 8.16; 1,73. L=14; 2; 0.5; 3.51; -2; 4.02; -0.51. L=12; 2; 1.38; 4.39; -2.5; 5.28; -0.89. L=10; 2; 2.25; 5.26; -2.13; 5.66; -0.4. L=14; 4; -2.13; 0.89; -3.5; 2.52; -1.63. L=12; 4; -1.75; 1.26; -4.75; 3.03; -1,77. L=10; 4; -1.38; 1.63; -3.63; 4.16; -2,53. Table 3- Evaluation results of Alt. 1 and Alt. 2 when TBS=144. Number of Msg3 repetitions; Alt 1; Alt 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 6.81; 9.82; 1.5; 8.49; 1.33. L=12; 1; 9.5; 12.51; 0.63; 9.08; 3.43. L=10; 1; 5.88; 10.64; 1.63; 10.08; 0.56. L=14; 2; 3.5; 6.51; -1.5; 5.49; 1,02. L=12; 2; 3.88; 6.89; -1.63; 6.83; 0.06. L=10; 2; 3; 7.77; -1.5; 6.95; 0.82. L=14; 4; -0.25; 2.76; -3.38; 3,61; -0.85. L=12; 4; 0.5; 3.51; -3.5; 4.95; -1.44. L=10; 4; -1; 3.77; -3; 5.45; -1.68. Table 4- Evaluation results of Alt. 1 and Alt. 2 when TBS=208. Number of Msg3 repetitions; Alt 1; Alt 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 9.25; 12.26; 0.88; 9.33; 2.93. L=12; 1; 5.9; 10.65; 0.38; 9.92; 0.72. L=10; 1; 7.69; 12.46; 0.94; 10.48; 1.98. L=14; 2; 4.5; 7.51; -3; 5.45; 2.06. L=12; 2; 3; 7.77; -3.13; 6.42; 1.35. L=10; 2; 4.13; 8.9; -3; 6.54; 2.36. L=14; 4; 1; 4.01; -5.13; 3.33; 0.68. L=12; 4; -1; 3.77; -5; 4.54; -0.77. L=10; 4; 0.19; 4.96; -4.88; 4.67; 0.29. Table 5- Evaluation results of Alt. 1 and Alt. 2 when TBS=282. Number of Msg3 repetitions; Alt 1; Alt 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 7.69; 12.46; 0.32; 10.31; 2.15. L=12; 1; 7.94; 13.96; 2.13; 11.67; 2.29. L=10; 1; 7.44; 14.43; 1.19; 11.98; 2.45. L=14; 2; 4.13; 8.9; -3.25; 6.75; 2.15. L=12; 2; 3.5; 9.52; -1.81; 7.73; 1.79. L=10; 2; 3.38; 10.36; -2.75; 8.04; 2.32. L=14; 4; 0.19; 4.96; -4.5; 5.5; -0.54. L=12; 4; -0.94; 5.08; -4; 5.54; -0.46. L=10; 4; -0.69; 6.3; -4; 6.79; -0.49. Table 6- Evaluation results of Alt. 1 and Alt. 2 when TBS=480. Number of Msg3 repetitions; Alt 1; Alt 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. L=14; 1; 7.94; 13.96; 1.62; 11.63; 2.33. L=12; 1; 7.44; 14.43; 1.88; 11.98; 2.45. L=10; 1; 6.9; 15.35; 1.13; 12.89; 2.46. L=14; 2; 3.5; 9.52; -2.37; 7.63; 1.89. L=12; 2; 3.38; 10.36; -2.75; 8.04; 2.32. L=10; 2; 3.3; 11.75; -2.94; 8.82; 2.93. L=14; 4; -0.94; 5.08; -3.88; 6.12; -1.04. L=12; 4; -0.69; 6.3; -4; 6.79; -0.49. L=10; 4; -0.94; 7.51; -3.88; 7.89; -0.38. Table 7- Evaluation results of Alt. 1 and Alt. 2 when TBS=640.

[18, Sharp] Observation 1: With 2 PRBs, 2 to 8 symbol allocation cannot support 72 bits TBS for RRC resume request with 2 PRBs with MCS index smaller than 4. Observation 2: Alt 2 with 1 PRB allocation has an issue on scheduling msg3 with 56/72 bits TBS. ==>FL note: The assumed repetition factor K is 4 and 8 for Urban and Rural scenario respectively. Table 1: TBS for 2 PRBs TBS; 2 OS; 4 OS; 6 OS; 8 OS; 10 OS; 12 OS; 14 OS. MCS 0; 24; 24; 24; 32; 32; 48; 56. MCS 1; 24; 24; 32; 40; 48; 64; 80. MCS 2; 24; 24; 40; 48; 56; 80; 96. MCS 3; 24; 32; 56; 64; 80; 104; 128. MCS 4; 24; 40; 72; 80; 96; 128; 152. MCS 5; 24; 48; 88; 104; 120; 152; 192. MCS 6; 24; 56; 104; 120; 144; 184; 224. Table 2: TBS for 1 PRB TBS; 2 OS; 4 OS; 6 OS; 8 OS; 10 OS; 12 OS; 14 OS. MCS 0; 24; 24; 24; 24; 24; 24; 24. MCS 1; 24; 24; 24; 24; 24; 32; 40. MCS 2; 24; 24; 24; 24; 24; 40; 48. MCS 3; 24; 24; 24; 32; 40; 48; 64. MCS 4; 24; 24; 32; 40; 48; 64; 72. MCS 5; 24; 24; 40; 48; 56; 72; 96. MCS 6; 24; 24; 48; 56; 72; 88; 112. Table 3: MPL values MPL [dB]; 72 bits; 144 bits; 208 bits. Option 1, L=14; 122.06; 120.02; 118.89. Option 2, L=14; 121.93; 120.23; 119.08. Gap; 0.13; -0.21; -0.19. Option 1, L=12; 121.35; 119.30; 117.97. Option 2, L=12; 119.92; 119.43; 117.92. Gap; -1.43; -0.13; 0.05. FL note：It seems there is a typo in Table 3 above, where’ -1.43’ should be ‘1.43’

[22, LG]: Observation 1: Option2 requires more PRBs than option1 without additional performance gain. Tx4Rx, 300ns, 4rep Option 1; Option 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. TBS 72 bits; -7.8; -7.8; -11.6; -6.8; -1.0. TBS 120 bits; -6.3; -6.3; -12.0; -5.0; -1.3. TBS 224 bits; -6.5; -3.5; -12.1; -2.6; -0.9. 1Tx2Rx, 30ns, 4rep Option 1; Option 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. TBS 72 bits; -3.3; -3.3; -7.7; -2.9; -0.4. TBS 120 bits; -1.8; -1.8; -8.0; -1.0; -0.8. TBS 224 bits; -1.8; 1.2; -8.0; 1.5; -0.3. 1Tx2Rx, 300ns, 4rep Option 1; Option 2; Gap. 10% BLER SNR [dB]; w/ power normalization [dB]; 10% BLER SNR [dB]; w/ power normalization [dB]. TBS 72 bits; -3.8; -3.8; -8.0; -3.2; -0.6. TBS 120 bits; -2.3; -2.3; -8.7; -1.7; -0.6. TBS 224 bits; -2.3; 0.7; -9.1; 0.4; 0.3.

Reference

R1-2112543 Discussion on Msg3 repetition for coverage enhancement	 Huawei, HiSilicon

- Revision of R1-2110793

R1-2111332 	Type A PUSCH repetitions for Msg3 coverage	 OPPO

R1-2111110	 	Discussion on Type A PUSCH repetitions for Msg3 	Spreadtrum Communications

R1-2110922 Discussion on support of Type A PUSCH repetitions for Msg3 	ZTE

R1-2111031 Discussion on Type A PUSCH repetitions for Msg3	 vivo

R1-2111275		 Discussion on Type A PUSCH repetitions for Msg3 	CATT

R1-2111431 	Remaining issues on type A PUSCH repetitions for Msg3	 China Telecom

R1-2111588	 	Discussion on Type A PUSCH repetition for Msg3	 Xiaomi

R1-2111796	 Type A PUSCH repetitions for Msg3	 InterDigital, Inc.

R1-2111511 	On Msg3 PUSCH repetition 	Intel Corporation

R1-2111891		 Discussion on Msg3 Coverage Enhancement	 Apple

R1-2112234	 Type A PUSCH repetition for Msg3	 Qualcomm Incorporated

R1-2111440	 	Discussion on Type A PUSCH repetitions for Msg.3	 Panasonic Corporation

R1-2111755	 Type A PUSCH repetitions for Msg3	 Samsung

R1-2111994	 Type A PUSCH repetitions for Msg3	 ETRI

R1-2110867 Approaches and solutions for Type A PUSCH repetitions for Msg3 Nokia, Nokia Shanghai Bell

R1-2112039	 Remaining Issues for Type A PUSCH Repetition for Msg3	 Ericsson

R1-2112023	 Type-A PUSCH repetition for msg3 	Sharp

R1-2111624 	Discussion on type A PUSCH repetitions for Msg3 	CMCC

R1-2112123		 Type A PUSCH repetitions for Msg3	 NTT DOCOMO, INC.

R1-2110923		 Discussion on RRC parameters for coverage enhancement	 	ZTE

R1-2111982 	Discussion on coverage enhancement for Msg3 PUSCH 	LG Electronics

R1-2112393	 Discussion on Type A PUSCH repetitions for Msg3 	WILUS Inc.

R1-2111032	 	Enhanced contention resolution mechanism for CBRA procedure with MSG3 PUSCH repetition 	vivo

R1-2111589 	Other considerations for coverage enhancement	 Xiaomi

3GPP RAN1#106-e, R1-2108585 Feature lead summary #4 on support of Type A PUSCH repetitions for Msg3, Moderator (ZTE Corporation).