3GPP TSG RAN WG1 Meeting #107-e	R1-2112628

e-Meeting,  November 11th – 19th, 2021

Agenda Item:	7.1

Source:	Moderator (Huawei)

Title:	Summary of [107-e-NR-7.1CRs-03]: Correction on data and control multiplexing

Document for:	Discussion and Decision

Introduction

This document is created to collect company views on the proposed changes in [1].

Background

According to [1], in section 6.2.7 of TS 38.212 on Data and control multiplexing, if the number of HARQ-ACK bits is less than or equal to 2, and only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, the reserved resource for potential HARQ-ACK transmission is not subtracted when calculating the number of coded bit for CSI part 1.

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

Discussion

During the preparation phase, two companies raised similar comments as follows

Comment 1: CR is not needed since it was already specified in clause 6.3.2.1.1 in TS 38.212 that UE always assume 2 bits AN transmission when there is no more than 2 bits AN if UCI is transmitted on PUSCH without UL-SCH and the UCI includes CSI part 1 without CSI part 2, which means that G-ACK equals to G-rvd-ACK for such case.

Comment 2: According to section 6.3.2.1.1 in 212, the number of HARQ-ACK bits is set to 2 if the number of HARQ-ACK bits < 2 bits. With this condition, G^ACK(1) = G^ACK_rvd(1) in case of  the number of HARQ-ACK bits = 2 bits (according to section 6.2.7). Therefore, the end result is the same even without the proposal.

From the moderator’s point of view, it is true that one can infer that G-ACK equals to G-rvd-ACK for such case if one looks at both section 6.3.2.1.1 and section 6.2.7. Therefore, it may be good to collect company views on whether the other companies share the same view and whether there is a need to adopt the change to make the specification clearer.

Company views

Q1: Do you agree that according to section 6.3.2.1.1 in TS 38.212, a UE always assume 2 HARQ-ACK bits when there is no more than 2 HARQ-ACK bits if UCI is transmitted on PUSCH without UL-SCH and the UCI includes CSI part 1 without CSI part 2, which means that in section 6.2.7, G-ACK equals to G-rvd-ACK for such case? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| Samsung | Agree | G-ACK and G-rvd-ACK is the same by specification. In Rel-15, the basic design principle is that all reserved REs for potential HARQ-ACK transmission (assuming 2bits) is used for actual HARQ-ACK transmission when only HARQ-ACK and CSI part 1 are present on the PUSCH without UL-SCH. Also, if G-ACK is not equal to G-rvd-ACK, it means that there are some ‘EMPTY’ REs not mapped to HARQ-ACK and CSI part 1 but the specification does not describe how to fill the “EMPTY” REs, e.g., 0s or dummy bits. Detail descriptions in the specification are shown below: In section 6.3.2.1.1, if the number of HARQ-ACK is less than 2 bits (i.e., 0bit or 1bit), add 0s to generate 2 bits ##### 6.3.2.1.1 HARQ-ACK If HARQ-ACK bits are transmitted on a PUSCH, the UCI bit sequence is determined as follows: - If UCI is transmitted on PUSCH without UL-SCH and the UCI includes CSI part 1 without CSI part 2, - - if there is no HARQ-ACK bit given by Clause 9.1 of [5, TS 38.213], set , , and ; - - if there is only one HARQ-ACK bit given by Clause 9.1 of [5, TS 38.213], set , , and ; - otherwise, set for and , where the HARQ-ACK bit sequence is given by Clause 9.1 of [5, TS 38.213]. In section 6.3.2.4.1.1, the number of symbols, Q’_ACK is determined by the number of HARQ-ACK bits, where the number of HARQ-ACK bits is O_ACK = A and A = 2 in this case. Note that O_ACK = A is not described in the specification clearly, so we suggest to clarity it (see the potential conclusion below). ###### 6.3.2.4.1.1 HARQ-ACK […] For HARQ-ACK transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK transmission, denoted as , is determined as follows: where - is the number of HARQ-ACK bits; - if , ; otherwise is the number of CRC bits for HARQ-ACK defined according to Clause 6.3.1.2.1;; In section 6.2.7, G_rvd^ACK is derived by setting O_ACK = 2 so that we can arrive at G_rvd^ACK = G^ACK. ### 6.2.7 Data and control multiplexing […] - if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let - ; - ; - ; and - ; […] if the number of HARQ-ACK information bits to be transmitted on PUSCH is 0, 1 or 2 bits the number of reserved resource elements for potential HARQ-ACK transmission is calculated according to Clause 6.3.2.4.2.1, by setting ; denote as the number of coded bits for potential HARQ-ACK transmission using the reserved resource elements; if frequency hopping is configured for the PUSCH, let and ; if frequency hopping is not configured for the PUSCH, let ; One thing to be clarified here, we assume OACK = A when determining Q’_ACK in 6.3.2.4.1.1. So, we suggest to make a conclusion to avoid any potential confusion. Potential conclusion: In section 6.3.2.4.1.1 in TS38.212, when determining Q’_ACK, O_ACK is the number of HARQ-ACK bits derived in section 6.3.2.1.1 in TS38.212. |
| NTT DOCOMO | Yes | We do not think Samsun’s proposing conclusion is necessary. It seems that current spec is saying that O_ACK = A = 2 at 6.3.2.1.1 and Q’_ACK is calculated based on the O_ACK. |
| QC | Yes | Although we think the CR proposed by Huawei is technically correct. But reading the comments from other companies, we agree with them that the spec is already clear enough. No conclusion or spec change is needed. |
| vivo | Yes | Similar view with NTT DOCOMO and QC. The current spec is already clear. |
| ZTE | Yes | There would be no misunderstanding according to current spec. |
| Intel | Yes | We share similar view as other companies that current spec is clear for G^ACK calculation. When A = 2, G^ACK = G_rvd^ACK. |
| Apple | Yes | As explained by Samsung, there should be no ambiguity on the current spec already (even though there may be room for improved clarity). |
| Spreadtrum | Yes | Agree with the spec is clear. G^ACK = G_rvd^ACK in this case |
| LG | Yes | It should be correct based on the following agreement made in RAN1#92. Agreements: - When only 0 or 1 bit HARQ-ACK and CSI part 1 (without CSI part 2) are multiplexed on PUSCH without UL-SCH, fill the empty reserved REs with bits that are generated by appending NACK to HARQ-ACK payload to make sure 2 bits HARQ-ACK payload before encoding. |
| Huawei, HiSilicon | Yes | Agree that the specification is technically correct even though there is room to improve the clarity. |

Q2: Do you think the changes in section 6.2.7 as proposed in [1] is necessary? If not, why?

| Company | Agree or not | Comment |
| --- | --- | --- |
| Samsung | Not | As we commented in Q1, it is obvious that G^ACK = G_rvd^ACK. So, the current specification is not broken. |
| NTT DOCOMO | Either is fine | More accurate text would be G_rvd^ACK, but G^ACK is also OK since the value is the same as discussed above. |
| QC | NO | Although we think the CR proposed by Huawei is technically correct. But reading the comments from other companies, we agree with them that the spec is already clear enough. No conclusion or spec change is needed. |
| Vivo | NO | According to the current spec, it already has G^ACK = G_rvd^ACK. No change is needed. |
| ZTE | Either is fine | There would be no misunderstanding according to current spec. But we are also ok to update for better clarity. |
| Intel | No | Our view is that current spec is clear and the update or conclusion is not necessary. |
| Apple | Either is fine | We are fine either way. The spec can be confusing in the sense that in section 6.2.7, the case with <=2 HARQ-ACK bits are separated out for other scenarios, but not this particular scenario. The inconsistent way of writing can cause confusion. |
| Spreadtrum | Either is fine | We can go with either way. |
| LG | No | We also think no conclusion or spec change is needed since the current spec seems to be already clear. |
| Huawei, HiSilicon | Either is fine |  |

Conclusions

Based on discussion, 10 companies provided feedback and there is common understanding that according to section 6.3.2.1.1 in TS 38.212, a UE always assume 2 HARQ-ACK bits when there is no more than 2 HARQ-ACK bits if UCI is transmitted on PUSCH without UL-SCH and the UCI includes CSI part 1 without CSI part 2, which means that in section 6.2.7, G-ACK equals to G-rvd-ACK for such case.

Regarding whether a CR is needed, 5 companies think there is no need to make any changes to the spec while 5 companies are fine with either way. In addition, 1 companies proposed to capture a conclusion to avoid any potential confusion while 2 companies think there is no such need.

Therefore, the CR is rejected and the discussion is closed.

- References

R1-2110870, Correction on data and control multiplexing, Huawei, HiSilicon

- Appendix: Proposed CR in R1-2110870

### 6.2.7	Data and control multiplexing

< Unchanged part is omitted >

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,  GCSI-part11=M1∙NL∙Qm-GACK1; otherwise, GCSI-part11=M1∙NL∙Qm-GrvdACK1

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,	; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if only CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

< Unchanged part is omitted >