3GPP TSG RAN WG1 #107-e		           R1-2112644

e-Meeting, November 11th – 19th, 2021

Source:	Moderator (CATT)

Title:	Summary of enhancements on beam management for multi-TRP (Round 2)

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

Background

This document summarizes the remaining issues on enhancements of beam management for multi-TRP.

Beam measurement/reporting

Issue 1.1: UE reporting of information related to Rx panel/antenna group

### Round 1

Views from company contributions on issue 1.1 are summarized as follows:

UE indicates if reported beams are associated to different RX spatial filters, or maximum number of supported layers corresponding to DL RS in a group, or whether two beams in a beam pair can be used for spatial multiplexing or diversity:

- Associated to same or different RX spatial filters:

  - UE reports: Samsung (also include RX panel activation/deactivation status), QC, Apple, CMCC, Xiaomi, CATT

  - gNB configures: Nokia, ETRI, DCM (configures UE to report), CATT, Sony

  - Not support: OPPO, LGE

- Maximum number of supported layers:

  - MTK, E///, Apple (per Rx beam), ZTE

  - Not support: OPPO

- Whether two beams can be used for spatial multiplexing or diversity:

  - UE reports: ZTE

  - gNB configures: Intel (without UE feedback)

- Not support any gNB indication/configuration of UE panel related hypothesis

  - MTK, Spreadtrum, OPPO, LGE

Based on views of majority companies, the following proposal is suggested.

FL Proposal 1.1: For beam reporting option 2

- Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters.

- Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | We do not support either Alt1 or Alt 2. The issue of Alt1: whther and how to use Rx spatial filters to receive and measure CSI-RS or SSB is part of UE implementation. It shall be something that can be configured or commanded by the gNB. The issue of Alt2: reporting “same or different Rx spatial filters” does not give the system any useful information. The intention of Alt2 seems to be that “using same or different Rx spatial filter” might imply the information of number of layers that can be supported. The fact is that is not true. Because the number of layers depends on the number of antenna ports and the condition of CSI, which can not be measured during beam measurement. |
| Apple | Support Alt-2. For Alt-1, in our view, UE panel is UE’s hardware, it is not reasonable for gNB to control UE’s hardware. |
| ZTE | If reporting the same spatial Rx filter, what the gNB should assume for the subsequent transmission? Up to two layers or four layers DL transmission? Some clarification is needed. For progress, we can further support the report of ‘maximum number of supported layers’ from UE side. |
| NTT DOCOMO | Support Alt-1. gNB knows the traffic type and overall scheduling information, so that gNB knows what type of two beams are needed from UE. Otherwise, UE reported beam groups may be not useful for gNB scheduling. |
| Lenovo/MotM | We don’t support either Alt1 or Alt2. For Alt1, it’s not clear for the UE when it can not find beam pairs satisfying gNB’s indiction. For Alt2, we fail to see the benefit of the additional reported information as in Rel-15. |
| LGE | We have similar view with OPPO, Lenovo/MM. Regarding Alt-2, there is ongoing discussion on the association between DL RS and UE panel in AI 8.1.1, so prefer to wait the result of the discussion. |
| vivo | We share similar view with LGE that this issue shall be discussed after associated discussion in AI 8.1.1. |
| Xiaomi | Prefer Alt2. It is UE that controls the activation of one or two spatial Rx filter. And gNB can’t make sure that UE can receive any two beams by the same spatial Rx filter. |
| Mod | From our understanding, Rx panel information might be beneficial for gNB to select a proper beam group in scheduling. If NW knows whether the reported beams in a beam group are associated with the same of different Rx beams, NW can choose the transmission scheme that is more suitable for this pair of beam. For example, a beam pair that is associated with different Rx beams can be used to reduce the probability of blockage. |
| InterDigital | Support FL’s proposal. |
| Huawei, HiSilicon | Support Alt-1. |
| Samsung | Support FL’s proposal. We prefer Alt. 2. |
| Futurewei | Support FL’s proposal and we prefer Alt-1. |
| Qualcomm | Support Alt2 |
| ZTE2 | For either Alt-1 or Alt-2, we need to have a clear definition of how many layers/ports to be associated with a spatial domain filter. To merge the candidates, we have the following suggestion: FL Proposal 1.1: For beam reporting option 2 - Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters. - Alt-2: UE informs to NW whetehr whether the reported beams in a beam group are associated with same or different RX spatial filters. - Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability siganling |
| Sony | We support Alt.2. As for Alt-1, we don’t think NW should force UE to measure with the same or different panels. It should be up to UE. After measurement, UE reports to NW whether same of different panels are used and to be used. We are fine with direction of FL proposal 1.1. If the intention is to down-select between Alt.1 and Alt.2, we suggest to capture the intention in the main bullet, such as FL Proposal 1.1: For beam reporting option 2, support either Alt-1 or Alt-2 - Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters. Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters. |
| MediaTek | We support ZTE’s revision, and prefer Alt2 |
| Mod | Based on comments from companies FL Proposal 1.1 is revised as follows. Updated FL Proposal 1.1: For beam reporting option 2, down select one out of the following alternatives - Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters. - Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters. - Alt-3: Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling. |
| Nokia/NSB | We support Alt 1. The information shall be configured according to NW’s scheduling interest or intention. General usecase of same RX filter is when two CMRs are associated with two panels in a TRP, while different RX filter is when two CMRs are associated with panels in two separate TRPs. So, when gNB configures two CMR reseource sets, only one use case is usually applicable, and it can configure proper types of beam group constraint. From this logic, we can avoid UE’s reporting a beam group which is not available for gNB scheduling. In addition, we dont need additional UCI overhead per reported beam pair required in alt 2. To Lenovo, in Alt 1. UE has enough knowledge of beam group association to RX beams. |
| CMCC | We prefer Alt.2. For Alt-1, gNB has no idea about the beam quality under Alt-1 or Alt-2. It is more reasonable to let UE select Alt-1 or Alt-2 based on the measurement result, and inform it to NW. |
| Intel | How is “different Rx filters” associated with reported L1-RSRP ? – lets say UE has 2Rx for each panel, then L1-RSRP reported is based on 2 Rx (single panel is used for measurement) or 4Rx (both panel used for measurement)? (we think this information is needed for gNB to determine mTRP or sTRP operating mode) |
| Ericsson | This issue has been discussed for more than 3 meetings now. Given this is the last meeting for Rel-17 feMIMO enhancemtns, we suggest to directly downselect now. Although our preference is to support maximum number supported layers per DL RS in a group, we can accept Alt-2 for the sake of progress. |
| ZTE2 | Thank you so much for the FL’s considering our comments. In our initial thought, ‘Alt 3 Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling’ should be considered as a subbulet of Alt1/2, rather than a new candidate, considering that it is to clarify the meaning of the transmission impacts of same or different Rx spatial filters. Updated FL Proposal 1.1: For beam reporting option 2, down select one out of the following alternatives - Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters. - Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling. - Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters. - Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling. |
| NTT DOCOMO | We agree with ZTE2’s assessment on Alt 3. |
| Lenovo/MotM | To Nokia, it’s correct that the UE knows whether the reported beams are associated with a same or different RX beams. Our concern is when the gNB indicates the UE to report two beams associated with different RX beams, while the UE can’t find such two beams. How the UE should report the required beams? |
| Spreadtrum | Share the same view with OPPO/Lenovo/LG. We can wait the progress of AI8.1.1. |
| TCL | We support Alt.2, where UE can handle the measurement by same or different RX spatial filters. |

### Round 2

FL Proposal 1.1-2: For beam reporting option 2, down select one out of the following alternatives

- Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters.

- Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters.

- Alt-3: Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling.

- Aly-4: no spec impact.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support Alt-1 and Alt-2. |
| Lenovo/MotM | As we commented in the 1st round, we don’t support all those alternatives. |
| H3C | We are fine with Alt.2 |
| ZTE | Support Alt-2, and then Alt-3 can be considered as a subbullet. |
| InterDigital | Support Alt-2. |
| Nokia/NSB | Support Alt-1. |
| Futurewei | Fine with either Alt-1 or Alt-2. |
| OPPO | We are nor supportive of any one of Alt1~Alt 3. The problem for Alt-1 is: How the UE receive the CSI-RS is up to UE implementation, which can not be configured by the gNB. The problem for Alt-2: reporting “same or different Rx spatial filters” does not give the system any useful information. The intention of Alt2 seems to be that “using same or different Rx spatial filter” might imply the information of number of layers that can be supported. The fact is that is not true. Because the number of layers depends on the number of antenna ports and the condition of CSI, which can not be measured during beam measurement. Problem of Alt3: the number of layers can only be measured in CSI measurement, not in beam measurement. Thus we suggest to add one more Alt: Alt4: no spec impact. |
| NTT DOCOMO | We prefer Alt-1. But we can also accept to support both Alt-1 and Alt-2. In this case, gNB configures the reporting type, and UE reports beams and corresponding reporting type if such beams can be found, otherwise, for example, UE could report beams with different reporting type indication. We think supporting both alts may address the concern from each side. |
| Samsung | We prefer Alt-2. |
| Xiaomi | We prefer Alt-2 |
| Mod | @OPPO: I tend to agree with you that for Alt-1 and 2, the information of number of layers might not be accurate, and such information can be obtained from CSI reporting, rather than from beam reporting. However, as mentioned before, if NW knows whether the reported beams in a beam group are associated with the same of different Rx beams, NW can choose the transmission scheme that is more suitable for this pair of beam. For example, a beam pair associated with different Rx beams can be used to reduce the probability of blockage. In the following update, Alt-3 is revised according to ZTE’s suggestion and Alt-4 is added as OPPO suggested. Updated FL Proposal 1.1-2: For beam reporting option 2, down select one out of the following alternatives - Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters. - Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters. - Alt-3: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters. - Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling. - Alt-4: not supportive of any one of Alt1~Alt 3 |
| Sony | Support Alt-2. Since there seems to be one last GTW session in 107e, can we suggest that let’s give the most popular scheme a try (no need to list others to trigger more technical discussion)? |
| CMCC | Support Alt-2. In our understanding, how to receive the RS is up to UE, and reporting the information would be helpful for gNB scheduling. |
| LGE | Support Alt-4. We prefer to wait the progress of AI 8.1.1. |
| ETRI | Support Alt-1 and Alt-2. |
| Lenovo/MotM | Support Alt-4. |

Updated FL Proposal 1.1-2: For beam reporting option 2, down select one out of the following alternatives

- Alt-1: gNB configures UE whether to report beams associated with same or different RX spatial filters.

- Alt-2: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters.

- Alt-3: UE informs to NW whetehr the reported beams in a beam group are associated with same or different RX spatial filters.

  - Maximum number of supported layers per RX spatial filter is signaled to gNB by UE capability signaling.

- Alt-4: not supportive of  any one of Alt1~Alt 3

Issue 1.2: Support of L1-SINR report

### Round 1

Views from company contributions on issue 1.2 are summarized as follows:

- Support L1-SINR report

  - Yes: QC, ZTE, LGE, Huawei, HiSilicon, CMCC, Spreadtrum, DCM, Xiaomi, Lenovo, Motorola Mobility, Intel, ETRI, TCL, Sony, CATT, NEC

  - No: OPPO, Apple, vivo

- Support dedicated/explicit IMR

  - Yes: CMCC, IMR, LGE, ETRI, TCL, DCM, Sony

Based on views of majority companies, the following proposal is suggested.

FL Proposal 1.2: Support L1-SINR for beam reporting option 2

- IMR resource assumptions:  explicit IMR configuration, including ZP or NZP IMR

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Do not support proposal 1.2 The benefit of using L1-SINR in beam reporting option 2 is not clear. And thee is no technical justification for why introducing L1-SINR can better support pairing two CRI/SSBRI that can be received simultaneously. |
| Apple | According to our simulation results, there is no performance gain to support enhancement for group based L1-SINR based beam selection. |
| ZTE | Support. We observe significant performance gain as in our contribution R1-2110951. |
| NTT DOCOMO | Support Proposal 1.2. |
| Lenovo/MotM | Support FL’s proposal. |
| LGE | Support the FL proposal. |
| NEC | Update our preference to support the proposal. |
| Xiaomi | Support the FL proposal 1.2 |
| Mod | In Rel.16, L1-SINR based beam reporting was supported since L1-RSRP based beam reporting cannot always reflect the actual signal quality. A beam with largest L1-RSRP value may suffer server interference from other cells. Similarly, in M-TRP transmission with multi-panel reception scenario, a beam pair selected based on L1-RSRP metric may not be actually suitable for M-TRP transmission. At least according to the evaluation results QC (R1-2112199), L1-SINR based beam group selection can achieve substantial UE throughput gain over L1-RSRP based beam group selection. |
| InterDigital | Support FL’s proposal, agree to support L1-SINR. |
| Huawei, HiSilicon | Support. |
| Samsung | We support FL’s proposal. |
| Futurewei | Support FL’s proposal. |
| Qualcomm | Support 1.2 |
| Sony | Support the FL proposal. |
| Nokia/NSB | We don’t have strong view, but specification impact should be carefully checked. |
| CMCC | Support the FL proposal. |
| Intel | Support |
| Ericsson | Support FL’s proposal. |
| Spreadtrum | Support |
| TCL | Support FL’s proposal. |

### Round 2

FL Proposal 1.2-2: Support L1-SINR for beam reporting option 2

- IMR resource assumptions:  explicit IMR configuration, including ZP or NZP IMR

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support. |
| Lenovo/MotM | Support. |
| H3C | Support |
| ZTE | Support. This issue has RRC impact and thus should be handled with high priority. |
| InterDigital | Support |
| Futurewei | Support. |
| OPPO | Two problems of the proposal 1.The benefit of L1-SINR measurement for beam reporting 2 is not clear. 2. The IMR configuration is not clear. Is it configured per CMR of each TRP or shared by two TRPs? |
| NTT DOCOMO | Support. |
| Samsung | Support L1-SINR |
| Xiaomi | Support |
| Mod | @OPPO: for the first question from OPPO, evaluation results from QC (R1-2112199) clearly show the benefit of L1-SINR. And from my understanding, such benefit is reasonable, as the quality of beam can be reflected more accurately by L1-SINR. For the second question, to measure inter-beam interference, I think it’s configured per CMR of each TRP. Maybe proponents of L1-SINR can provide their thought on IMR configuration further. |
| Sony | Support the FL proposal. |
| CMCC | Support. We think IMR can be configured per CMR of each TRP. |
| ZTE2 | We share the same views with moderator. Based on OPPO’s second comment, we may clarify the above proposal as follows: FL Proposal 1.2-2: Support L1-SINR for beam reporting option 2 - IMR resource assumptions: explicit IMR configuration, including ZP or NZP IMR - Note: ZP or NZP IMR is configured per CMR for each of CMR sets |
| LGE | Support. Regarding OPPO’s comment, similar view as moderator and ZTE. we think IMR can be configured per CMR of each CMR resource set. Furthermore, if CMR#1 of CMR resource set#1 has CMR#2 of CMR resource set#2 as IMR and also CMR #2 has CMR#1 as IMR, it can be considered that CMR pairing is done to measure inter-TRP/CMR interference. Since L1-RSRP does not consider inter-TRP interference, reported best beam pair based on L1-RSRP can be suboptimal for NCJT. |
| ETRI | Support the FL proposal. |
| Lenovo/MotM | Support the FL proposal. |

Summary of email discussion

Updated FL Proposal 1.2-2: Support L1-SINR for beam reporting option 2

         IMR resource assumptions:  explicit IMR configuration, including ZP or NZP IMR

- Note: ZP or NZP IMR is configured per CMR for each of CMR sets

| Company | Comments |
| --- | --- |
| Sony | Support the FL proposal. |
| Apple | We think this proposal is not quite clear. Especially for the IMR configuration part. Is the L1-SINR used to measure inter-beam interference or not? Without any inter-beam interference measurement, we cannot see any gain. We also provided simulation results about it. But as discussed in previous meetings, it is hard for UE to measure inter-beam interference due to the problem of UE beam selection for each resource. |
| DOCOMO | Support |
| ZTE | Support the FL proposal. Regarding Apple’s comment, it seems that two move-forward direction at the table are blocked J. In our views, for moving forward, proponent companies can compromise not to support the inter-beam interference in this release. So non inter-beam interference is assumed herein. Regarding performance evaluation, please review our contribution where we do not assume any inter-beam interference, and the performance gain is obtained from deriving low-interference beam impliciately. |
| LGE | Support. We think IMR can be configured per CMR of each CMR resource set. Furthermore, if CMR#1 of CMR resource set#1 has CMR#2 of CMR resource set#2 as IMR and also CMR #2 has CMR#1 as IMR, it can be considered that CMR pairing is done to measure inter-TRP/CMR interference. Then, best CMR pair based on L1-SINR can be reported by UE. Since L1-RSRP does not consider inter-TRP interference, reported best beam pair based on L1-RSRP can be suboptimal for NCJT. |
| OPPO | We share the same concerns on measurement related with inter-beam interference measurement as Apple. If inter-beam interference is not counted, it seems there is no benefit to introduce SINR here. But if inter-beam interference is counted, the UE is not able to measure it because there is no CMR-to-CMR mapping in the resource configuration and the UE does not know which CMR shall be the ‘interference source’ to one CMR. Beam reporting Option 2 does not allow CMR mapping since the CMR grouping shall be reported by the UE. The UE meets a dilemma. |
| Spreadtrum | Support |
| Mod | As mentioned by ZTE, even if inter-beam interference is not available, at least beams with lower interference level can be selected. This is also verified by evaluation results from ZTE. In addition, in some cases, such as SFN, the level of inter-beam interference may not be the criteria of selecting beam pair. Instead, the pair of beams with lower interference from beams outside of this pair would be a better choice. |
| ETRI | Support the FL proposal. |
| Ericsson | Support |
| Samsung | Support L1-SINR. |
| Qualcomm | Support FL’s proposal. To our understanding, the NZP IMR in the proposal can be used to measure inter-beam interference. The spec change for this should also be small. Suppose we build over the two CMR set framework in R17, and NW configures set1 including (CMR1+IMR1, CMR2+IMR2), and set 2 including (CMR3+IMR3,CMR4+IMR4). We only need a rule to say like CMR1 and CMR3 correspond to 1st candidate beam group, and CMR2 and CMR4 correspond to 2nd candidate beam group. Then UE will report the best candidate beam group with the two SINRs reflecting the cross-beam interference for that group. For example, if the best beam group corresponds to CMR1 and CMR3, then UE will report SINR1 and SINR3 computed based on CMR1+IMR1 and CMR3+IMR3, which captures cross-beam interference based on NW config. |
| Huawei, HiSilicon | Support. |
| Lenovo/MotM | Support |
| Intel | Support |
| Xiaomi | Support |
| CMCC | Support |
| Futurewei | Support |
| Nokia/NSB | Support |

Updated FL Proposal 1.2-2: Support L1-SINR for beam reporting option 2

         IMR resource assumptions:  explicit IMR configuration, including ZP or NZP IMR

- Note: ZP or NZP IMR is configured per CMR for each of CMR sets

- Supported by: (16) Sony, DOCOMO, ZTE, LGE, Spreadtrum, ETRI, Ericsson, Samsung, Qualcomm, Huawei, HiSilicon, Lenovo/MotM, Intel, Xiaomi, CMCC, Futurewei, Nokia/NSB

- Concerned by: (2)Apple, OPPO,

Other issues of beam measurement/reporting

### Round 1

In addition to the above issues, the issues listed in the following table are also raised by some companies.

| Issues | Companies | Views |
| --- | --- | --- |
| Beam measurement/reporting option 1 & 3 | OPPO (N=2 groups and M=1/2), Sony | Support beam measurement/reporting option 1. |
|  | E/// | Do not support Option 1 and Option 3. |
| CMR configuration | vivo | For semi-persistent CMR resource configuration, support to activate two SP CSI-RS resource sets simultaneously. |
|  | DCM, vivo | For aperiodic CMR resource configuration, default beam of AP CSI-RS resource should be clarified. |
|  | LGE | Support gNB to dynamically indicate CMR resource set(s) for a CSI trigger state configured with two CMR resource sets. (switching between STRP and MTRP, reduce DCI overhead) |
|  | ZTE | - Repetition from different sets should be configured with same value. - AperiodicTriggeringOffset from different sets can be configured with different value(s). |
|  | QC | Two reported beams per group can be from serving and non-serving cells, respectively. |
| Others | HW: support ZTE: not support | # R17 group-based beam reporting shares the same RRC enabler (i.e., groupBasedBeamReporting) as R15/16 group-based beam reporting. |
|  | Intel | # A UE is not expected to receive downlink signals/channels or transmit uplink signals/channels in a time overlapping manner with a beam pair combination that it has not reported within a certain interval of time |
|  | Apple | # The beams in a beam pair reported in a group-based beam report instance can be simultaneously received by UE within an effective time window. The duration of the effective window can be reported by UE capability. |

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | We think it is important to have a clear definition of “simultaneous reception”. After UE reports two beams can be received simultaneously, it does not mean UE can always receive them simultaneously in future. The proposal from QC seems to be a good point to discuss. Currently we are not sure whether the group based beam reporting enhancement can also be extended to support inter-cell mTRP or not. In addition, we think the proposal from Intel is also an important aspect. We need to consider some connection between beam report and beam indication for group based beam report. This is different from non-group based beam report, as for non-group based approach, the beam status can be measured from uplink, and even some beams without report, gNB can know the beam quality. But for group based beam report, gNB has no information on the simulanteous reception tatus. |
| ZTE | Generally speaking, issues with RRC impact should be handled firstly. So, we suggest to check the companies’ views on the following RRC parameter as the FL raised, and go have a quick agreement. In our views, the parameter of ‘groupBasedBeamReportingR17’ is necessary for enabling the Rel-17 group based reporting. Then, we identify that one company suggests to use the condition of ‘#. Of RS resource set > 1’ for flagging the Rel-17 group base reporting. It is not beneficial for forward compatibility, besides that it may introduce a complicate rule. [groupBasedBeamReportingR17]; [Group-based reporting for Rel.17 M-TRP as described in Clause Z.Y in TS38.214[. Regarding Apple’s comment, we agree that it is a good topic, and can be open to further discussion. |
| NTT DOCOMO | Do not support Option3. Okay to support Option1 if time allows. For CMR configuration, okay to study the problem for SP/AP CMR configuration. Okay to clarify whether group-based beam reporting option2 can be applied in MTRP inter-cell scenario. |
| Vivo | For MTRP beam report, the default beam mechanism for aperiodic CSI-RS resource should be enhanced to achieve TRP-specific default beam. In the previous agreement, two aperiodic CSI-RS resource set have been agreed to be triggered by a DCI. In this case, directly reusing the default beam mechanism defined in Rel-16 may be problematic for mDCI MTRP scenario, since both sets of the triggered CSI-RS shall be with the same QCL assumption from the TRP which transmits the PDCCH triggering the aperiodic CSI-RS. Therefore, it is better to define the default beam per TRP. For example, introduce an association between CSI-RS set and CORESETPoolindex and then based on the association define TRP-specific default beam. |
| Huawei, HiSilicon | For RRC configuration, we do not support to introduce a new parameter groupBasedBeamReportingR17, which is redundant. Since the basic difference between R17 and R15/R16 group-based beam reporting is the number of resource sets, it is more natural to differentiate them by the number of resource sets. This is similar to what we did for R16: the basic difference between R16 and R15 group-based beam reporting is report quantity, so they are differentiated by report quantity. As for forward compatibility metioned by ZTE, we don’t see any issue. Instead, we see more issues if groupBasedBeamReportingR17 is introduced, e.g., whether groupBasedBeamReportingR17 and groupBasedBeamReporting can be configured simultaneously. |
| Qualcomm | For the proposal below, it is just agreed to discuss in MB session as FFS. So we may not need to repeat in this session. Two reported beams per group can be from serving and non-serving cells, respectively. |
| ZTE2 | Regarding Huawei’s comment, the motivation of new RRC parameter is to distinguish Rel-17 group based reporting from legacy one. Implicate manner as Huawei mentioned will make this procedure complicate. It is also not good for the potential enhancement in the future release. Since this RRC parameter is to distinguish two features and select one out of them, why we need to consider two cases simultaneously? Sorry to say that we fail to follow the logic. Although we do not agree with Huawei’s views, it seems that we both are on the same page that this discussion is essential for completing RRC signaling and also spec draft. We suggest to check the companies views and go with majority views: Regarding how to differentiate Rel-15 and Rel-17 RSRP based group based report procedure, - Alt-1 (explicit): to introduce a RRC parameter groupBasedBeamReportingR17 - Alt-2 (implicit): to be based on the number of configured resource sets |
| Mod | Agree with ZTE that RRC related issues should be discussed with high priority. Based on comments from ZTE and HW, the following FL proposal is added for discussion. Companies are invited to show your preference. FL Proposal 1.3: Regarding how to differentiate Rel-15/16 and Rel-17 group-based beam reporting procedure, - Alt-1 (explicit): to introduce a RRC parameter groupBasedBeamReportingR17 - Alt-2 (implicit): to be based on the number of configured resource sets |
| Nokia/NSB | For proposal 1.3, support alt-1. We don’t think there is any room to agree any other proposals. Some remaining issues can be further checked through the maintenance. |
| Vivo | It may not be necessary for us to waste time on signaling design on this issue. RAN2 can handle this well. |
| Mod | @vivo: Yes, RAN2 can handle this indeed. However, RAN2 also need us to feedback the list of required RRC parameters. |
| Ericsson | On FL proposal 1.3, our preference is Alt-1. We suggest to do downselect based on company inputs in this meeting. |
| MediaTek | On FL proposal 1.3, we support Alt-1. |
| NTT DOCOMO | On FL proposal 1.3, our preference is Alt-1. |
| Lenovo/MotM | Support Al-2. It is not necessary to introduce such RRC parameter. |
| TCL | For FL proposal 1.3, we support Alt-1. |

### Round 2

FL Proposal 1.3-2: Regarding how to differentiate Rel-15/16 and Rel-17 group-based beam reporting  procedure,

- Alt-1 (explicit): to introduce a RRC parameter groupBasedBeamReportingR17

- Alt-2 (implicit): to be based on the number of configured resource sets

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support Alt-2. Similar to R16, tbere is no need to introduce a new RRC parameter. |
| Lenovo/MotM | We support Alt-2. It is unnecessary to have dedicated RRC parameter. There is no ambiguities without this RRC parameter when two CMR sets are configured. |
| H3C | Support Alt-1 |
| ZTE | Support Alt-1. We disagree with Huawei, but in fact, for Rel-16, we have new reporting quantities for Rel-16 SINR. Similarly, we need to have a RRC parameter for enabling this feature rather than being based on an implicit rule. |
| InterDigital | Support Alt-1 |
| Mod | For this issue, please provide inputs to https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_107-e/Inbox/drafts/8.1.2.3/RRC |
| Nokia/NSB | Support Alt-1 |
| Futurewei | Support Alt-2. |
| OPPO | Alt1. This proposal is essential to beam reporting option 2 and has impact on RRC. So we shall discuss it with high priority. |
| NTT DOCOMO | Support Alt-1. |
| Samsung | Support Alt-1. |
| Mod | @all: Above inputs are copied to https://www.3gpp.org/ftp/TSG_RAN/WG1_RL1/TSGR1_107-e/Inbox/drafts/8.1.2.3/RRC. No more input in this table please. |
| Sony | Support Alt-2. No need for additional RRC parameter, if UE can realize the Rel.17group-based beam reporting from two configured resource sets in one CSI-ResourceConfig. |

M-TRP beam failure recovery

Issue 2.1: Simultaneous configuration of cell-level BFR and TRP-specific BFR

### Round 1

Views from company contributions on issue 2.1 are summarized as follows:

- Support simultaneous configuration of cell-level and TRP-specific BFR: Sony, TCL (for both SpCell and Scell)

- Not support simultaneous configuration of cell-level and TRP-specific BFR: QC, Intel, DCM, Spreadtrum (obaservation), CATT

  - A UE is configured with either “Rel-15/16 BFD” or “TRP-specific BFD” on one BWP: QC, CMCC, CATT

- E///, APT, DCM, CATT: At most 2 BFD-RS sets can be configured in each DL CC/BWP (including Scell and SpCell)

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.1: Simultaneous configuration of cell-level and TRP-specific BFR is not supported

- A UE is configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Support. |
| Apple | We suggest we defer the decision. Technically, we would like to understand what would be the problem if both are enabled in a CC? |
| FGI/APT | Support FL’s proposal |
| ZTE | Not our first preference. Based on the many round discussion, we can support it. |
| NTT DOCOMO | Support Proposal 2.1. |
| LGE | Support the FL proposal. @Apple, to our understanding, if both are configured in a same BWP, UE is required to track 3 BFD RS sets(one from Rel-15 or Rel-16 and two from Rel-17) |
| NEC | Considering the long discussion and FL’s effort. We are fine with the subbullet. While the main bullet seems no need. The agreed RACH based BFR as a fall back scheme is actually cell level BFR. |
| Vivo | Support the FL’s proposal. |
| Xiaomi | Support the proposal. Since RACH-based transmission can be triggered on a SpCell When beam failure is detected on all BFD-RS sets on the SpCell, the issue can be considered on Scell only. For Scell, when beam failure is detected on all BFD-RS sets on the Scell, the difference on cell-level BFR and TRP-specific BFR is the information in MAC CE. If it is cell-level BFR, only cell ID and at most one qnew will be indicated in MAC CE. While for TRP-specific BFR, cell ID, TRP index (or BFD-RS set index) and at most two qnew will be indicated in MAC CE. Thus if TRP-specific BFR is configured, the information in MAC CE is a superset of that for cell-specific BFR. It is not necessary to configure both cell-level and TRP-specific BFR on one Scell. |
| Mod | @Apple: for implicit BFD-RS set determination, if “cell-specific BFD” is configured, UE will derive one BFD-RS set according to the rule in Rel.15/16. If “TRP-specific BFD” is configured, UE will derive two BFD-RS sets based on X TCI of CORESETs with CORESETPoolIndex = k. It is not expected that both “cell-specific BFR” and “TRP-specific BFR” are configured simultaneously; otherwise it is unclear about how many BFD-RS sets need to be derived by UE. |
| InterDigital | Support FL’s proposal. |
| Huawei, HiSilicon | We support simultaneous configuration of cell-level and TRP-specific BFR at least for SpCell. For SpCell, cell-level BFR is essential. In addition, TRP-level BFR can be configured for extra robustness. On this line, we support configuring at most 2 BFD-RS sets which are shared by cell-level BFR and TRP-level BFR. |
| Samsung | The “simultaneous configuration” in the main bullet is open for many interpretations. We support the subbullet as below, which is sufficient (or clarify the simultaneous configuration in the main bullet) FL Proposal 2.1: Simultaneous configuration of cell-level and TRP-specific BFR is not supported - A UE is configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP. |
| Futurewei | It seems the issue here is the maximum number of BFD-RS sets the UE should be configured/derived. One way to move forward is to specify that a UE is configured at most 2 BFD-RS sets on one BWP. |
| Qualcomm | Support 2.1. Otherwise, interactions between these two BFR mechanisum need to clarify, e.g. whether they can have independent BFD RS config. If this is allowed, should UE terminate one ongoing BFR if the other type is newly triggered? Perfer a simple rule in last meeting. |
| ZTE | In our views, the motivation of this proposal is to preclude the case of simultaneous configuration for cell-level and TRP-specific BFR, right? If so, besides for BFD RS, we also do not need individual configuration for PUCCH-SR and new candidate RS, etc. So we prefer the original version, and if really needed, some futher clarification may be also acceptable for us. |
| Fujitsu | Support the proposal revised by Samsung. |
| Sony | On Proposal 2.1, we would like to share some of our observations. First, in RAN1#106bis-e, RAN1 agreed to trigger RACH-based transmission on a SpCell when beam failure is detected on all BFD RS sets. In our reading, RACH implies the fall back BFRQ scheme (i.e. cell-specific) which recovers to one new beam of single-TRP (previously associated with preamble). Afterwards, NW may keep the UE in the fallback mode for a while. During this period of time, we think the 1 BFD RS set should be used. Of course, the periodic RSs within “Rel-15/16 BFD” and “TRP-speicifc BFD” can be the same to save UE’s effort in montoring. Agreement FL proposal 2.12-1: RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell - FFS: other scenarios - Scenario 2: at least one TRP fails on SpCell - Scenario 3: at least one pre-defined TRP fails on SpCell - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - Scenario 6: When no PUCCH-SR is configured Secondly, like many others, we also feel the main bullet containing “simultaneous” can be interpreted in many different ways, such as cell-level BFR on one CC and TRP-specific BFD on another CC. We do believe that’s not the original intention, so we hope it can be removed as suggested by Samsung. With respect to the majority view, we are fine to apply the restriction at least to Scell as FL Proposal 2.1: For Scell, a UE is configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP. |
| MediaTek | Supprot the FL’s proposal |
| Mod | According to suggestion from Samsung, this proposal is revised as follows: FL Proposal 2.1: A UE is configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP. @Sony: thanks for your detailed explanation. Regarding whether this restriction applies to Scell only, I would like to know the opinion from more companies. |
| CMCC | Support the revised proposal. |
| Convida | Support FL’s proposal. |
| Intel | Support |
| Ericsson | We do not support FL proposal 2.1. As we commented in previous meetings, ‘Rel-15/16 BFD’ and ‘TRP-specific BFD’ are independent features, and RAN1 should not put limitations in its specification that forbit the combinations of these features. It seems some companies are concerned that the simultaneous configuration will lead to 3 BFD RS sets. As suggested by Huawei and Futurewei, this can be simply addressed by agreeing that ‘At most 2 BFD-RS sets can be configured in each DL CC/BWP’. We think this is a reasonable way forward rather than limiting combinations of the two features. |
| ZTE2 | Support the FL’s proposal. Regarding Ericsson’s comment, in our views, we do not agree that the mode of a single BFD-RS can be applied to mTRP-BFR. If must have 2 BFD-RS sets for mTRP-BFR, and if having the rule of up to 2 BFD-RS sets, we can directly derive the conclusion that ‘A UE is configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.’ |
| NTT DOCOMO | Agree with ZTE2’s comment on always 2 BFD-RS sets for per-TRP BFR. And as discussed in email reflector, following revision is suggested. FL Proposal 2.1: A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP. |
| Lenovo/MotM | We share the same view with Ercisson. |
| Spreadtrum | Support |
| TCL | We do not support FL proposal 2.1. We share the similar view as Ericsson that RAN1 should not put limitations on combinations of the ‘Rel-15/16 BFD’ and ‘TRP-specific BFD’. |

Summary of discussion on issue 2.1 over email:

Updated FL Proposal 2.1: A UE iscan be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

         Support: OPPO, DOCOMO, vivo, Sony, Xiaomi, CMCC, MTK, NEC, FGI/APT, ZTE, LGE, InterDigital, Qualcomm, Fujitsu, Convida, Intel

          Not support: Ericsson, Huawei, HiSilicon, Nokia/NSB, IDC

| Company | Comments |
| --- | --- |
| OPPO | we support the proposal and suggest to change the “is” to “can” in main bullet. |
| DOCOMO | Agree with OPPO to change the ‘is’ to ‘can be’. |
| ZTE | Same views as DOCOMO and OPPO. |
| Sony | Thank to FL for hearing more views on our suggestion (applying Proposal 2.1 only to SCell(s)). If that’s the majority view to apply this configuration constraint to both PCell and SCells, we can go with it. Moreover, if we change “is” to “can be”, then it reads that Proposal 2.1 is not exclusive to the case of both “Rel-15/16 BFD” and “TRP-specific BFD” configured. We understand the concern of changing “is” to “can be” is to avoid mandatory configuration on any BFD, so we would suggest to add one more word to address the concern as Proposal 2.1: A UE is optionally configured with either “Rel-15/16 BFD ” (i.e., 1 BFD -RS set per BWP ) or “TRP -specific BFD ” (i.e. 2 BFD -RS sets per BWP ) on one BWP . |
| Ericsson | We do not support FL proposal 2.1. As we commented in previous meetings, ‘Rel-15/16 BFD’ and ‘TRP-specific BFD’ are independent features, and RAN1 should not put limitations in its specification that forbit the combinations of these features. It seems some companies are concerned that the simultaneous configuration will lead to 3 BFD RS sets. As suggested By Huawei and Futurewei, this can be simply addressed by agreeing that ‘At most 2 BFD-RS sets can be configured in each DL CC/BWP’. We think this is a reasonable way forward rather than limiting combinations of the two features. |
| vivo | Support |
| Apple | Suggest we defer the decision after we see more details on TRP-specific BFD. |
| Huawei, HiSilicon | Similar view as Ericsson. |
| Nokia/NSB | Similar view with Ericsson. |
| InterDigital | We have a similar view as Ericsson. |
| Xiaomi | Support the proposal. Since RACH-based transmission can be triggered on a SpCell When beam failure is detected on all BFD-RS sets on the SpCell, the issue can be considered on Scell only. For Scell, when beam failure is detected on all BFD-RS sets on the Scell, the difference on cell-level BFR and TRP-specific BFR is the information in MAC CE. If it is cell-level BFR, only cell ID and at most one qnew will be indicated in MAC CE. While for TRP-specific BFR, cell ID, TRP index (or BFD-RS set index) and at most two qnew will be indicated in MAC CE. Thus if TRP-specific BFR is configured, the information in MAC CE is a superset of that for cell-specific BFR. It is not necessary to configure both cell-level and TRP-specific BFR on one Scell. |
| Leonvo/MotM | We have a similar view as Ericsson. |
| Samsung | Support the proposal. Not sure what is the combination of the two features. |

| LGE | Support the proposal. We think that “configuration of either cell-specific BFD-RS or TRP-specific BFD-RS per CC/BWP” is simple way forward. Also, we are confused about Ericsson’s comment regarding ‘At most 2 BFD-RS sets can be configured in each DL CC/BWP’, is it mean that cell-specific BFD-RS set and one of TRP-specific BFD-RS sets can exist in a CC/BWP? |
| --- | --- |
| Spreadtrum | Support |
| CMCC | Support the proposal. This is our understanding of the proposal. If a UE is configured with 2 BFD-RS sets, each BFD-RS set represents a TRP, R17 TRP-specific BFR will be trigger when beam failure is detected on one of the BFD-RS set, and R15/R16 cell- specific BFR can be triggered when beam failure is detected on both BFD-RS sets. If a UE is configured with 1 BFD-RS set, R15/R16 cell- specific BFR will be triggered when beam failure is detected on the BFD-RS set. |
| Futurewei | Similar view with Ericsson. |
| NEC | Fine with the proposal. |
| Mod | @Sony: thanks for your comment. Regarding the wording suggested by you, it seems that more companies prefer OPPO’s revision, i.e., “can be”. Anyway, we can check more companies’ preference later. @Ericsson and companies not supporting 2.1: regarding LGE’s question, could you please respond? |
| MediaTek | Support the updated proposal |
| Lenovo/MotM | We prefer that “up to 2 BFD-RS sets can be configured per BWP”. |

      - Updated FL Proposal 2.1

Updated FL Proposal 2.1: A UEcan be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

        Support(16): OPPO, DOCOMO, vivo, Sony, Xiaomi, CMCC, MTK, NEC, FGI/APT, ZTE, LGE, InterDigital, Qualcomm, Fujitsu, Convida, Intel

        Not support(5): Ericsson, Huawei, HiSilicon, Nokia/NSB, IDC, TCL

### Round 2

FL Proposal 2.1-2: A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support. |
| H3C | Support |
| Convida | Support |
| ZTE | Support |
| Nokia/NSB | It can be up to RAN2. |
| Vivo | Support. |
| Futurewei | Support. |
| OPPO | Support. |
| NTT DOCOMO | Support. Should be decided in RAN1. |
| MediaTek | Supprot. Should be decided in RAN1. |
| Samsung | Support |
| Xiaomi | Support |
| Sony | Support. We can go with “can be”, since it reads like no restriction at all. |
| CMCC | Support |
| LGE | Support. |
| Lenovo/MotM | We prefer that “up to 2 BFD-RS sets can be configured per BWP”. |

Summary of email discussion

FL Proposal 2.1-2: A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

| Company | Comments |
| --- | --- |
| Sony | Support. We can go with “can be”, since it reads like no restriction at all. |
| Apple | We do not think this is urgent. But we are open for majority’s view. In our view, the key point is how many BFD RSs instead of how many BFD RS sets. Although there are 3 sets, if they are overlapped, that might not be critical. |
| DOCOMO | Support |
| ZTE | It is not our preference, but we can support it. |
| LGE | Support the proposal. |
| MediaTek | Support |
| vivo | Support |
| OPPO | Support the proposal |
| Spreadtrum | Support |
| Ericsson | Don’t support. We agree with Apple: provided that the UE complexity can be handled, the specification should not prevent configurations. We also think the current proposal is unclear: what does the text in quotation marks mean? Can we simply remove that for the future discussion: FL Proposal 2.1-2: A UE can be configured with either 1 BFD-RS set per BWP or 2 BFD-RS sets per BWP. |
| Convida | Support, but can't the UE also be configured with 0 BFD-RS sets in a BWP? Also, some clarification regarding implicitly determined BFD-RS sets may be needed (M-DCI case). If the UE isn't explicitly configured with any BFD-RS set, should the UE assume 1 implicit BFD-RS set or 2 implicit BFD-RS sets? One way to solve it to specify that the UE implicitly determines as many BFD-RS sets as there are configured NBI-RS sets in the BWP, which follows the Rel-15/16 principle. |

| Qualcomm | Support. This is the simplest way. Otherwise, more clarifications may be needed for the interaction between these two similar mechanisms. For example, do we allow the two types of BFD RSs to be configured independently? If so, one type of BFD can be triggered first with corresponding BFR ongoing, but the other type of BFD can be later triggered in the middle. We may need to clarify corresponding UE behavior, e.g. whether to stop the 1st triggered BFD or ignore the 2nd triggered BFD. The current spec is unclear for the UE to whether initiate another BFR for 2nd triggered cell-level BFD while waiting for the BFR response for 1st triggered TRP-level BFD. To avoid all interaction related clarifications, a simple way is to only configure one BFD type at each time, also considering only 1 or 2 days left for R17. |
| --- | --- |
| Huawei, HiSilicon | Support the revision from Ericsson. |
| Mod | @All: this proposal is updated according to Ericsson’s suggestion. @Convida: thanks for the support. In the following update, it’s further clarified that this applies to explicit BFD-RS configuration, so at least one BFD RS set needs to be configured. Hopefully, this can address your concern. Updated FL Proposal 2.1-2: For explicit BFD-RS configuration, a A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP. |
| Lenovo/MotM | We prefer that “up to 2 BFD-RS sets can be configured per BWP”. |
| Xiaomi | We are fine with the updated FL proposal |
| LGE2 | We are not sure the updated FL proposal is clear enough. Our question is that what is the 1 BFD-RS set and 2 BFD-RS set then? For 2 BFD-RS set, is it mean that cell-specific BFD-RS set and one of TRP-specific BFD-RS sets can exist in a CC/BWP? I think it is more complicated and the original proposal from moderator is crystal clear and simple way to go. |
| Intel | Same view as LGE2 and QC, we think the original version is clearer |
| CMCC | We support the original proposal. We think if beam failure is detected on cell-specific BFD-RS, it means beam failure is happened on both TRPs. No need to configure both cell-specific BFD-RS and TRP-specific BFD-RS |
| Futurewei | Support FL’s updated proposal. |
| Mod | Seems people have even stronger concerns on the update from moderator. As mentioned by LG and some other companies, restricting the maximum number of BFD-RS sets may still lead to some unexpected combinations, e.g. the combination of one cell-specific BFD-RS set and one TRP-specific BFD-RS set. So, to avoid such cases, let’s try the following revision. Updated FL Proposal 2.1-2: For explicit BFD-RS configuration, a A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 cell-specific BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 TRP-specific BFD-RS sets per BWP) on one BWP. |
| Nokia/NSB | We share view with E///. Lenovo’s proposal is fine for us. |

Updated FL Proposal 2.1-2:  For explicit BFD-RS configuration, a UE can be configured with either 1 cell-specific BFD-RS set per BWP or  2 TRP-specific BFD-RS sets per BWP.

Issue 2.2: Update of explicit BFD-RS set

### Round 1

Views from company contributions on issue 2.2 are summarized as follows:

- Support to introduce MAC-CE for updating explicit BFD-RS set: ZTE, OPPO, Apple, Samsung (under Rel.17 unified TCI framework), DCM, CATT

  - Associates each TCI state for PDCCH with a BFD RS: OPPO, Apple

  - Use MAC-CE to update QCL assumption of BFD-RS: vivo

- Not support to Introduce MAC-CE for updating explicit BFD-RS set: Nokia (observation), Lenovo, Motorola Mobility, LGE

- Not support explicit BFD-RS configuration for Rel-17 MTRP BFR: MTK

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.2: Support to introduce MAC-CE for updating explicit BFD-RS set

- Alt-1: update BFD-RS resource(s) in the explicit BFD-RS set

- Alt-2: update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set

- Alt-3: associate each TCI state for PDCCH with a BFD RS

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | The fomulaion of proposal 2.2 seem to have some issue. Per our understanding, Alt-3 does not use MAC CE. So suggest to revise the wording of Proposal 2.2 as follows: FL Proposal 2.2: Support to introduce MAC-CE for updating providing explicit BFD-RS set - Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set - Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set - Alt-3: associate each TCI state for PDCCH with a BFD RS Among those three Alts. Alt-3 is our 1st preference. Its pros is the extra MAC CE control ehavior is not needed. When the TCI state of PDCCH is switched, the BFD RS is automatically updated. We do not support Alt2: The UE measures “the” RS configured as BFD-RS to detect beam failure. Updating the QCL assumption does not change the RS and there would be still misalignment between the BFD RS and Tx beam of PDCCH. For Alt-1: we can live with it. |
| Apple | Support the revision from OPPO. If we cannot reach any consensus, we can also accept no support explicit BFD RS as proposed by MTK. |
| ZTE | Support. Either goint with the FL’s or OPPO’s proposal, we need to make down-selection this meeting. From ZTE perspective, we can support Alt-1. Echo Apple and MTK’s views that if no consensus, we need to consider to revert the previous agreement. |
| NTT DOCOMO | Okay with the revision from OPPO for the 3 alternatives. We support Alt-1. And we’d like to clarify that Alt-1 and Alt-2 mean the support of RRC configured explicit BFD-RS set. |
| LGE | Not support. RRC-based explicit BFD-RS configuration is sufficient. |
| NEC | Just for clarification, what’s the difference between Alt-3 and implicit BFD RS set configuration? If we go with Alt-3, does it mean implicit BFD RS configuration is finally/actually used regardless implicit or explicit configuration. And if we cannot reach any consensus, we are fine to revert previous agreement, and support implicit BFD RS configuration for single-DCI based mutli-TRP. |
| Vivo | Support Alt-2. And if no consensus, we can accept RRC reconfiguration to update, rather than to revert the previous agreement. |
| Xiaomi | For Alt 3, it is not a MAC CE mechanism. In addition, we also share same confusion as NEC on the difference between Alt 3 and implicit BFD-RS set. While for Alt 2, does it mean the QCL of the BFD-RS will be updated to align with the TCI state of the PDCCH? In this case, what is the difference between Alt 2 and Alt 3? |
| Mod | @NEC @Xiaomi: from our understanding, explicit BFD-RS set only means that BFD-RS set for each TRP is configured explicitly. However, update of QCL assumption(s) of BFD-RS resource(s) in the explicitly configured BFD-RS set doesn’t conflict with such understanding. @Xiaomi: Regarding Alt 2, it doesn’t necessarily mean that BFD-RS will be updated to align with the TCI state of the PDCCH. @NEC: reversion of previous agreement might be possible, but from FL’s perspective, this is not recommended. As mentioned by vivo, RRC reconfiguration can still be used for such updating. FL proposal 2.2 is revised according to OPPO’s suggestion: Updated FL Proposal 2.2: Support providing explicit BFD-RS set - Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set - Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set - Alt-3: associate each TCI state for PDCCH with a BFD RS |
| InterDigital | Support FL’s proposal or OPPO’s revision. In either case, we support Alt-1. |
| Huawei, HiSilicon | We support Alt-4 added below. Instead of asking gNB to send in a MAC-CE to update explicit BFD-RS, it is more natural for the UE to automatically include the RS corresponding to reported new beam into BFD-RS set after receiving BFRR. Updated FL Proposal 2.2: Support providing explicit BFD-RS set - Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set - Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set - Alt-3: associate each TCI state for PDCCH with a BFD RS - Alt-4: UE to include the RS corresonding to reported new beam into BFD-RS set after receiving BFRR |
| Samsung | Fine with FL’s proposal, we support Alt-1. |
| Qualcomm | 1st preference for Alt1, 2nd preference for Alt2 |
| ZTE | Fine with FL’s updated proposal, and we support Alt-1. Reverting already agreement is always not our, ZTE, first preference. But, unfortunately, if we fail to identify appropriate solution to make NW work well, it seems that we have to do that. |
| Fujitsu | Support FL’s proposal and support Alt-1. |
| Sony | Support the updated FL proposal 2.2. We think Alt-1 is more reasonable one than other alternatives. |
| Mod | Proposal 2.2 is revised to include one more alternative according to HW’s suggestion. Updated FL Proposal 2.2: Support providing explicit BFD-RS set - Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set - Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set - Alt-3: associate each TCI state for PDCCH with a BFD RS - Alt-4: UE to include the RS corresonding to reported new beam into BFD-RS set after receiving BFR response |
| Nokia/NSB | We don’t support the proposal 2.2. This can be handled by general beam update procedure for periodic RS if supported. No separate procedure is required for BFR. |
| CMCC | Support FL’s proposal. We prefer Alt-1. |
| Convida | Support the proposal, but some clarification of Alt-3 may be needed. Is the intention to explicitly configure a set of periodic CSI-RS as BFD-RS, and use the activated TCI state(s) for CORESETs (in the same BWP) as TCI state(s) for these periodic BFD-RS? A difference with implicit BFD-RS would be that aperiodic/semi-persistent CSI-RS could be QCL source RS for the CORESETs? |
| Ericsson | We do not support Updated FL proposal 2.2. RRC based BFD-RS set configuration is sufficient for explicit update. No need for MAC CE based mechanism. |
| ZTE2 | Support the updated FL proposal. Alt-1 is our preference. |
| NTT DOCOMO | Support Alt-1. |
| Lenovo/MotM | We share the similar view with Ericsson. |
| Spreadtrum | Not support. In Rel-15/16, for cell level explicit BFS-RS update, only RRC based is supported. We have not seen the new requeiremnt for TRP level BFR. |
| TCL | Support the updated FL proposal 2.2. We prefer Alt-1. |

### Round 2

FL Proposal 2.2-2: Support providing explicit BFD-RS set

- Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set

- Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set

- Alt-3: associate each TCI state for PDCCH with a BFD RS

- Alt-4: UE to include the RS corresonding to reported new beam into BFD-RS set after receiving BFR response

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support Alt-4, as it helps reduce signaling overhead and latency. |
| H3C | Support Alt-1 |
| Convida | Support the proposal, but Alt-3 could be further clarified. Prefer Alt-2. |
| ZTE | Support Alt-1. |
| InterDigital | Support Alt-1. |
| Nokia/NSB | We don’t support proposal. |
| vivo | Support the FL proposal and prefer Alt-2. |
| OPPO | Alt-3 is our 1st preference but we can live with Alt-1 if it is the majority. |
| NTT DOCOMO | Support Alt-1. |
| Samsung | We support alt-1. |
| Xiaomi | We can accept Alt 1 |
| Sony | Support Alt-1. |
| CMCC | Support Alt-1. |
| LGE | Not support. RRC-based configuration is sufficient. |
| Lenovo/MotM | We shared the same view with LGE. |

FL Proposal 2.2-2: Support providing explicit BFD-RS set

- Alt-1: using MAC CE to update BFD-RS resource(s) in the explicit BFD-RS set

  - H3C, ZTE, InterDigital, OPPO(2nd), NTT DOCOMO, Samsung, Xiaomi, Sony, CMCC

- Alt-2: using MAC CE to update QCL assumption(s) of BFD-RS resource(s) in the explicit BFD-RS set

  - Convida, vivo

- Alt-3: associate each TCI state for PDCCH with a BFD RS

  - OPPO(1st)

- Alt-4: UE to include the RS corresonding to reported new beam into BFD-RS set after receiving BFR response

- Alt-5: RRC-based configuration is sufficient.

  - Nokia/NSB, LGE, Lenovo/MotM

Issue 2.3: Implicit BFD-RS set configuration for CORESET with one TCI state

### Round 1

In #106e, the following agreement was reached:

Agreement

Support the following BFD-RS configurations in Rel.17 for Ues with one activated TCI state per CORESET:

- Implicit configuration:

  - M-DCI:

    - BFD-RS set k (k = 0, 1) is derived based on X TCI of CORESETs with CORESETPoolIndex = k

    - FFS: value of X (determined in spec or UE capability), and TCI selection rule when the number of CORESETs with CORESETPoolIndex = k exceeds X (e.g. reuse RLM RS selection rule)

- FFS: CORESETs with more than 1 activated TCI states

For implicit BFD-RS set configuration for Ues with one activated TCI state per CORESET, views from company contributions of this meeting are summarized as follows:

- The number of TCI states (X)

  - Equals to the number of TCI states of CORESETs with CORESETPoolIndex = k: CMCC, ITRI, CATT

  - Equal to 2, fixed in TS38.213: E///

- When the total number of QCL type-D RSs (TCI states, e.g. X) across CORESETs exceeds the reported UE capability

  - Similar mechanism as specified for rel-15 RLM is used (CORESETs with smallest periodicities): Huawei, HiSilicon, QC, Nokia, Nokia Shanghai Bell, E///, LGE, ITRI

  - Reuse the Rel-15/16 rule (UE implementation): Convida, CATT

  - X should be configured no more than UE capability. If the value of X exceeds UE capability, TCI state selection is up to UE implementation: vivo

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.3: For implicit configuration of BFD-RS set for M-DCI

- The number of TCI states (X)  in implicit BFD-RS determination

  - Alt-1: X=min(2, the number of TCI states of CORESETs with CORESETPoolIndex = k)

  - Alt-2: X=the number of TCI states of CORESETs with CORESETPoolIndex = k

- TCI state selection when X exceeds the UE capability on the maximum number of BFD-RS resources per set

  - Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs.

  - Alt-2: X should be configured no more than UE capability. If the value of X exceeds UE capability, TCI state selection is up to UE implementation.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | First bullet: we support Alt-1. Second bullet: we support Alt-2 |
| Apple | As discussed in last meeting, we think the proposal can be reformulated as follows: Proposal: For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives: - Alt1: Re-use the same or modified RLM-RS selection rule for BFD RS selection - Alt2: BFD RS selection is up to UE implementation |
| ZTE | First issue: Alt-2. Second issue: Alt-1 but with simplified rule rather than directly reusing. If no concensus, we think that it should be fine as in Rel-15 (up to UE). |
| NTT DOCOMO | For first issue, we support Alt-2. For second issue, we support Alt-1: re-use or similar to the RLM-RS selection rule. We’re also fine with the revised main bullet from Apple. Then we just need to discuss the second issue. |
| Lenovo/MotM | For the first bullet, we support Alt-2. For the second bullet, we support Alt-1. Besides, we can accept the version reformulated by Apple. |
| LGE | We think that Apple’s proposal is better for reducing alts, and support Alt1. |
| Vivo | We revise the proposal as follows: FL Proposal 2.3: For implicit configuration of BFD-RS set for M-DCI - The number of TCI states (X) in implicit BFD-RS determination - Alt-1: X=min(2, the number of TCI states of CORESETs with CORESETPoolIndex = k) - Alt-2: X=the number of TCI states of CORESETs with CORESETPoolIndex = k - Alt-3: X should be configured no more than UE capability - TCI state selection when X exceeds the UE capability on the maximum number of BFD-RS resources per set - Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs. - Alt-2: X should be configured no more than UE capability. If the value of X exceeds UE capability, TCI state selection is up to UE implementation. For the updated proposal above, we support Alt-3 for the first bullet and Alt-2 for the second bullet. |
| Xiaomi | For the first bullet, we are confused on the motivation of X, anyway the number of BFD-RS resources per set depends on the UE capability. We just need to specify the candidate BFD-RS resource, i.e., TCI states of CORESETs with CORESETPoolIndex = k. and if the number of TCI states of CORESETs with CORESETPoolIndex = k exceeds the UE capability, spec on how to select it is needed. Thus we prefer Apple’s version and support Alt 1. |
| Mod | Agree with Apple to restart this discussion based on the formulation of the last meeting. Updated FL Proposal 2.3: For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives: - Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs. - Alt2: BFD RS selection is up to UE implementation |
| InterDigital | First bullet: support Alt-2. Second bullet: support Alt-1. |
| Futurewei | For the updated FL proposal, we prefer Alt-1. |
| Qualcomm | 1st bullet: support Alt2 2nd bullet: support Alt1 |
| ZTE2 | If our understanding is correct, we have already agreed that the total #. Of RS in all sets is subject to UE capability, rather than #. Of RSs per set. So, if going with FL’s proposal, it seems to be ‘… if total number of RSs in two BFR-RS sets per DL BWP number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set,…’ Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 |
| Lenovo/MotM | Support the updated FL proposal 2.3, and we slightly prefer Alt-1. |
| Fujitsu | Support the updated FL proposal and prefer Alt-1. |
| Mod | @ZTE: as mentioned by you, in #104b-e we have agreed that Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 It’s noted that only the number of BFD-RS is mentioned in the agreement shown above. In #105-e, no further agreement on this has been reached. And then, in #106-e, regarding how to derive BFD-RS set for implicit configuration, it’s agreed that: Agreement Support the following BFD-RS configurations in Rel.17 for Ues with one activated TCI state per CORESET: - Implicit configuration: - M-DCI: - BFD-RS set k (k = 0, 1) is derived based on X TCI of CORESETs with CORESETPoolIndex = k - FFS: value of X (determined in spec or UE capability), and TCI selection rule when the number of CORESETs with CORESETPoolIndex = k exceeds X (e.g. reuse RLM RS selection rule) - FFS: CORESETs with more than 1 activated TCI states So, what we are trying to resolve here in this proposal is the first FFS part above. |
| Nokia/NSB | Support the updated FL proposal, and prefer For 1st bullet: Alt-2 For 2nd bullet Alt-1 |
| CMCC | For the first bullet, support Alt-2. For the second bullet, support Alt-1. |
| Convida | Support Alt2 in the updated FL proposal 2.3. |
| vivo | Support Alt-2 in the update FL proposal. |
| Intel | First issue: Alt-2. Second issue: Alt-1 |
| Ericsson | In the updated FL proposal 2.3, for the second issue, our preference is Alt-1. |
| TCL | Support the updated FL proposal. For the second issue, support Alt-1. |

Summary of discussion on issue 2.3 over email:

Updated FL Proposal 2.3(possible offline agreement): For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives:

        Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs.

         Ericsson, ZTE, DOCOMO, Lenovo/MotM, Xiaomi, InterDigital, Futurewei, Qualcomm, Fujitsu, Nokia/NSB, CMCC, Intel, NEC, MTK

         Alt2:  No RAN1 specification impact: BFD RS selection is up to UE implementation

           OPPO, Convida

         Alt3:  No RAN1 specification impact: UE does not expect such case happen/This is error case

           vivo

| Company | Comments |
| --- | --- |
| vivo | It seems there are some changed to our previously commented versions. We would like to add another alts. |
| ZTE | Regarding 'Updated FL Proposal 2.3(possible offline agreement', does it mean that we need to introduce a new UE capability of 'maximum number of BFD -RS resources per set' which should be explicitly mentioned in our views, regardless of subsequent selection. |
| Ericsson | Regarding Updated FL proposal 2.3, we wonder if it makes much sense to keep multiple alternatives still on the table. Given it is the last meeting for Rel-17, we think it is better to have a proposal for one down-selected Alternatives. |
| Sony | Regarding updated proposal 2.3, we tend to agree with Peng’s suggestion by adding the 3rd sub-bullets. But in our view, Alt2 seems to the one without RAN1 specification impact, since it allows UE to handle this case. For Alt3, the impact would be such restriction (UE does not expect…) in Spec. If I get it wrong, please correct me. Thank you. Finally, we are fine with either Alt2 or Alt3. |
| Lenovo/MotM | We are OK with Alt 1 and Alt 2 where we slightly prefer Alt 1. |
| LGE | Support. And prefer Alt 1. @ZTE, we already have the following agreement. Agreement The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. |
| Spreadtrum | Fine with either Alt2 or Alt3 revised by OPPO |
| CMCC | Support Alt1, can live with Alt2. |
| NEC | Prefer Alt 1. |
| Mod | 2.3 is further updated according to comments from companies. @LGE: thanks for your clarification. @ZTE: based on LGE’s explanation, which alternative do you prefer? Updated FL Proposal 2.3(possible offline agreement): For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives: - Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs. • Ericsson, ZTE, DOCOMO, Lenovo/MotM, Xiaomi, InterDigital, Futurewei, Qualcomm, Fujitsu, Nokia/NSB, CMCC, Intel - Alt2: No RAN1 specification impact: BFD RS selection is up to UE implementation • OPPO, Convida, Spreadtrum - Alt3: No RAN1 specification impact: UE does not expect such case happen/This is error case, which is captured in RAN1 spec or concluded in Chairman’s notes. • Vivo, Spreadtrum |
| MediaTek | We prefer Alt1. For Alt3, it is unclear which one (above the table or in the table) is the updated proposal. Anyway, if UE doesn't expect something, we think this does cause spec impact. |
| Mod | Revision of 2.3 according to vivo’s suggestion is as follows. @MTK: please further check the update in this table. Sorry for the inconvenience. Updated FL Proposal 2.3(possible offline agreement): For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives:   Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs.   Ericsson, ZTE, DOCOMO, Lenovo/MotM, Xiaomi, InterDigital, Futurewei, Qualcomm, Fujitsu, Nokia/NSB, CMCC, Intel, NEC, MTK   Alt2: No RAN1 specification impact: BFD RS selection is up to UE implementation   OPPO, Convida  Alt3: No RAN1 specification impact: UE does not expect such case happen/This is error case, which is captured in RAN1 spec or concluded in Chairman’s notes.  Vivo, Spreadtrum |

      - Updated FL Proposal 2.3

Updated FL Proposal 2.3(possible offline agreement): For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, down-select one of the following alternatives:

        Alt-1: re-use or similar to the RLM-RS selection rule. For example, the RSs in TCI states of CORESETs with lower IDs are selected as BFD RSs.

         (15) Ericsson, ZTE, DOCOMO, Lenovo/MotM, Xiaomi, InterDigital, Futurewei, Qualcomm, Fujitsu, Nokia/NSB, CMCC, Intel, NEC, MTK, TCL

         Alt2:  No RAN1 specification impact: BFD RS selection is up to UE implementation

           (2) OPPO, Convida

         Alt3:  UE does not expect such case happen/This is error case, which is captured in RAN1 spec or concluded in Chairman’s notes.

          (2) Vivo, Spreadtrum

      - Agreement of round 1 discussion

Updated FL Proposal 2.3(possible offline agreement):

For implicit BFD RS configuration, if number of TCI states for CORESETs associated with a CORESETPoolIndex exceeds the UE capability on maximum number of BFD-RS resources per set, re-use the RLM-RS selection rule.

Issue 2.4: Association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration

### Round 1

Regarding whether to support association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration, views from company contributions are summarized as follows:

- Yes: Nokia, Nokia Shanghai Bell, CMCC, OPPO, Xiaomi, ETRI, Fujitsu, CATT

- No: ZTE, Lenovo, Motorola Mobility, DCM, NEC, Convida

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.4: Down select one out of the following alternatives

- Alt-1: Configure an association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is supported

- Alt-2: Configure an association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is not supported

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Support Alt1. |
| Apple | OK with Alt1 to be aligned with Pcell. But we would like to understand what the “association” means. Somhow we also feel confused with previous agreement. In our view, “association” means the event to trigger the SR is based on the failure of the associated BFD RS set. |
| ZTE | Alt-2. If going with Alt-1, we think that we need to have a clear requirement that the SR corresponding to a CORESET pool ID in Scell should be associated with the SR corresponding to another CORESET pool ID in Pcell. It is due to the fact that we need to prioritize the SR transmission to the TRP with same CORESET pool ID for Scell, but on contrary, considering the TRP fails, the SR should be transmitted to the another non-failed TRP in Pcell. |
| NTT DOCOMO | Our first preference is Alt-2, but we can also accept Alt-1. For a Scell, Alt-2 is slightly preferred considering different interference conditions and different BFD results on Scell and SpCell even if the TRP information is the same. In addition, the association between TRP on Scell and a PUCCH-SR resource on SpCell is meaningless if the TRP on Scell is different from the TRPs on SpCell or if an Scell is configured with one BFD-RS set only, because in this case, any PUCCH-SR can be selected. |
| Lenovo/MotM | Support Alt-2. Considering that PUCCH-SR resources are configured on SpCell, UE only need to aware the beam link quality of each TRP on SpCell for PUCCH-SR resource selection. |
| NEC | Support Alt-1. |
| Vivo | Support Alt-1. |
| Xiaomi | Support Alt-1. Since common TCI state update can be supported for intra-band CA, TCI state will be updated for SpCell and Scell at the same time. In order to improve the gNB flexibility, we prefer the wording below: FL Proposal 2.4: Down select one out of the following alternatives - Alt-1: an association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR can be configured. - Alt-2: an association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR can not be configured. |
| Mod | @Apple: regarding the term “association”, we have similar view as you. @ZTE: the selection of PUCCH-SR resource(i.e., issue 2.5) can be discussed in section 3.5. @Xiaomi: wording of this proposal is updated as follows Updated FL Proposal 2.4: Down select one out of the following alternatives - Alt-1: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is supported - Alt-2: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is not supported |
| InterDigital | Support Alt-1. |
| Huawei, HiSilicon | Support Alt-2. |
| Futurewei | Support FL’s updated proposal and we prefer Alt-2. |
| Qualcomm | Support Alt1 |
| ZTE | Support FL’s updated proposal, and we are fine with Alt-2. |
| Lenovo/MotM | Support the updated proposal, and we are fine with Alt-2. |
| Fujitsu | For the updated proposal, slightly prefer Alt-1. |
| Sony | Support Alt-2. Given the fact that PUCCH-SR resource can be configured with up to 2 spatial relations (pointing to different TRPs), such so-call association may not able to provide additional benefits. Thus we incline to leave it to UE’s implementation and no need to design such an association at the late stage. |
| MediaTek | Support Alt-2 in the updated FL Proposal 2.4 |
| Nokia/NSB | Support Alt-1 |
| CMCC | Support Alt-2. |
| Convida | Support Alt-2. |
| Intel | We think Alt-2 is sufficient |
| Ericsson | Support Alt-2 in updated FL Proposal 2.4 |
| Lenovo/MotM | Support Alt-2. |
| Spreadtrum | Support Alt-1, but also can live with Alt-2 |
| TCL | Support FL’s updated proposal and we prefer Alt-2. |

Summary of discussion on issue 2.4 over email:

Updated FL Proposal 2.4: Down select one out of the following alternatives

         Alt-1: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is supported

         OPPO, DOCOMO(2nd), ZTE(2nd), Sony(1st), Nokia/NSB, IDC, Xiaomi, vivo, LGE, Spreadrum, NEC, Fujitsu, QC

         Alt-2: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is not supported

          DOCOMO(1st), Sony(2nd), Ericsson, vivo, Huawei, HiSilicon, Lenovo/MotM, Samsung, Futurewei, MTK, ZTE(1st), Intel

| Company | Comments |
| --- | --- |
| OPPO | We prefer Alt1. The association can be used by MAC layer to trigger SR with proper SR configuration when beam failure of one TRP happens. |
| DOCOMO | Our first preference is Alt-2. But we can also accept Alt-1. For Alt-1, we want to clarify following points. · If one TRP on SCell is different from the TRPs on SpCell, is it allowed to associate the corresponding BFD-RS set on SCell with zero or one or two PUCCH/SR on SpCell? · For a SCell with per-cell BFR, is it allowed to associate the BFD-RS set on SCell with zero or one or two PUCCH/SR on SpCell? |
| ZTE | Alt-2 but we can live with Alt-1 |
| Sony | We prefer Alt.1 and can live with Alt-2. |
| Ericsson | Alt-2. |
| vivo | We prefer Alt1, Alt-2 seems overkill. At least for intra-band cases, we don’t see any strong motivation to prohibit the association between PUCCH-SR and TRPs. Do not fully understand DOCOMO’s question, what does it mean by “one TRP on SCell is different from the TRPs on SpCell”. |
| Apple | In our view, this proposal is not needed, since in last meeting is was agreed to FFS whether this is supported or not supported. There should be no 3rd alternative. We can directly discuss the down-selection. |
| Huawei, HiSilicon | Alt-2 |
| Nokia/NSB | We prefer Alt-1. |
| InterDigital | Prefer Alt-1 |
| Xiaomi | We prefer Alt-1 |
| Lenovo/MotM | Support Alt-2 |

| LGE | Agree with vivo. Support Alt-1 at least for intra-band CA (with SpCell) case. |
| --- | --- |
| Samsung | We slightly prefer Alt-2, i.e., it is not needed. |
| Spreadtrum | Support Alt-1, but can live with Alt-2 |
| CMCC | Either one is OK to us. |
| Futurewei | We support Alt-2. |
| NEC | Support Alt 1. |
| MediaTek | Support Alt-2 |

      - Updated FL Proposal 2.4

Updated FL Proposal 2.4: Down select one out of the following alternatives

         Alt-1: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is supported

         (11+2 2nd) OPPO, DOCOMO(2nd), ZTE(2nd), Sony(1st), Nokia/NSB, IDC, Xiaomi, vivo, LGE, Spreadrum, NEC, Fujitsu, QC

         Alt-2: association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR is not supported

          (10+1 2nd) DOCOMO(1st), Sony(2nd), Ericsson, vivo, Huawei, HiSilicon, Lenovo/MotM, Samsung, Futurewei, MTK, ZTE(1st), Intel

No further discussion.

Issue 2.5: PUCCH-SR resource selection

### Round 1

Views from company contributions are summarized as follows

- Alt-1: select the PUCCH-SR associated to failed (associated) BFD-RS set:

- InterDigital, QC, OPPO, Apple, Spreadtrum, Xiaomi, vivo, CATT

- Alt-2: select the PUCCH-SR associated to non-failed BFD-RS set:

- CMCC, Samsung, Xiaomi, Lenovo, Motorola Mobility, DCM, NEC, ETRI, vivo, Sony

- Other alternatives

- Apple: if multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused.

- LGE: support defining selection rule for all cases (e.g., including S-TRP Pcell + M-TRP Scell).

- TCL: UE implementation

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured:

- If all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource

  - Alt-1: select the PUCCH SR associated to failed BFD-RS set

  - Alt-2: select the PUCCH SR associated to non-failed BFD-RS set

- Else PUCCH-SR resource selection is up to UE implementation

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Do not support the proposal. The formulation of the proposal seeme not right. The condition “all failed BFD RS set coss CCs” is not valid. At MAC CE, the beam failure detection is done per TRP per CC. When beam failure is claimed for one TRP in a particular CC, the UE would start the procedure find UL resource for sending the PUSCH carrying the MAC CE. The UE would not wait to see if other CC will claim beam failure in the future. The correct procedure as defined in RAN2 is: when UE detects beam failure of one TRP in one CC, the UE first check if there is avaible uplink grant for the UE to transmit the MAC CE. If so, the UE sends the PUSCH with the MAC CE. If not, the UE can trigger SR to request for uplink grant. Revised Proposal 2.5: When beam failure is detected for one BFD-RS set and the UE needs to trigger SR - Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with this BFD-RS set. - Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with the other BFD-RS set - Alt3: it is up to UE implementation. |
| Apple | We do not think the whole proposal is necessary. In our view, when the BFD RS set fails, the associated SR should be triggered. Whether the associated SR is with the same TRP as the BFD RS set or not is up to gNB configuration. |
| ZTE | If up to gNB implementation, we think that Alt-1 is sufficient, and some refinement from OPPO looks good for sake of presentation. |
| NTT DOCOMO | The condition to trigger PUCCH-SR selection should be discussed after Issue 2.4/Proposal 2.4 is resolved. |
| Lenovo/MotM | Do not support the proposal. Since 2 PUCCH-SR resources are configured on SpCell, it only needs to consider the beam failure of BFD-RS sets on SpCell. Therefore, we propose the following proposal. Revised Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: - If only one BFD-RS set is failed on SpCell, - Alt-1: select the PUCCH SR associated to failed BFD-RS set on SpCell - Alt-2: select the PUCCH SR associated to non-failed BFD-RS set on SpCell - Else PUCCH-SR resource selection is up to UE implementation Based on this revised proposal, we can support Alt-2. |
| LGE | Prefer to defer the discussion after concluding FL proposal 2.4. |
| NEC | We are fine with the proposal, and support Alt-2. |
| Vivo | Support the FL’s proposal. |
| Xiaomi | It depends on the FL proposal 2.4. if Alt 1 in Proposal 2.4 is agreed, we prefer to go with OPPO’s revision. |
| Mod | Based on the comments from companies, FL Proposal 2.5 is revised as follows. Updated FL Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: - Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with this BFD-RS set. - Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with the other BFD-RS set - Alt3: it is up to UE implementation. |
| InterDigital | Support FL’s proposal and Alt-1. |

| Huawei, HiSilicon | Support Alt-2. |
| --- | --- |
| Samsung | Support Alt-2. |
| Futurewei | We prefer Alt-2. |
| Qualcomm | Suport Alt1, can also live with Alt2 |
| ZTE2 | Support Alt-1. |
| Lenovo/MotM | We are not OK with the updated FL proposal. “this BFD-RS set” and “the other BFD-RS set”in the proposal is not clear since BFD-RS set is configured per CC. If TRP1 is failed on SpCell, and TRP1 and TRP2 are both failed on Scell, how to trigger the PUCCH-SR/SR configuration since different TRP failure situation in different CC. The motivation of configurating 2 PUCCH-SR resources is to increase the probability of TRP-specific BFR successful report. And the 2 PUCCH-SR resources are configured on SpCell, therefore, we only need to consider the beam failure of BFD-RS sets on SpCell. Therefore, we still proposal the revised proposal as follows. Revised Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: - If only one BFD-RS set is failed on SpCell, - Alt-1: select the PUCCH SR associated to failed BFD-RS set on SpCell - Alt-2: select the PUCCH SR associated to non-failed BFD-RS set on SpCell - Else PUCCH-SR resource selection is up to UE implementation Based on this revised proposal, we can support Alt-2. |
| Fujitsu | Support the updated proposal. In our opinion, Alt-1 and Alt-2 is the same since gNB can associate PUCCH-SR / SR configuration in one way or the other. We slightly prefer Alt1 since it is more aligned with Rel-16 UE behavior. |
| Sony | Support the updated proposal in principle. But we wonder is it possible that the selected PUCCH-SR resource can be configured with 2 spatial relations (supported in AI 8.1.2.1) which are associated to 2 BFD RS sets (2 TRPs)? If that’s not the case, we are fine with Alt-2. |
| Nokia/NSB | We have similar view with Apple, in that sense, we prefere Alt-1. But, we are also fine with Alt-2. NW can handle it with either Alt-1 or Alt-2. We don’t support Alt 3. |
| Mod | @Lenovo/MotM: from my reading, in the revised proposal from you, it’s assumed there is no association between SR and BFD-RS set in Scell. As the discussion on whether the association is needed or not for Scell is still ongoing, such assumption might not be appropriate by far. @Sony: it’s observed from the discussion on issue 2.11, most of companies support not to restrict the number of spatial filters for PUCC-SR resource. In my opinion, whether further optimization is needed for such case can be discussed later. |
| CMCC | Support the revised proposal. Support Alt-2. |
| Convida | Do not support the proposal. The association between BFD-RS set PUCCH-RS resource on SpCell is sufficient. The revised proposal by Lenovo is OK. |
| Vivo | We think “this BFD-RS set” and “the other BFD-RS set”in the proposal is not clear. Dose they mean “ failed BFD-RS set” and “non-failed BFD-RS set” respectively ? if so, we suggest revising the wording. |
| Intel | ok with both Alt-1, Alt-2 |
| Mod | As suggested by Lenovo/MotM and vivo, to avoid any unnecessary ambiguity, “this” and “the other” are replaced with “failed BFD-RS set” and “non-failed BFD-RS set” respectively. Updated FL Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: - Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with failed BFD-RS set. - Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with non-failed BFD-RS set - Alt3: it is up to UE implementation. |
| ZTE | Support the FL’s proposal. |
| Lenovo/MotM | The updated FL proposal is still unclear since there may be multiple BFD-RS sets failed on different CCs and there is no agreement yet that each BFD-RS sets on different CCs can be associated with one PUCCH-SR resource/SR configuration. We proposal that ‘failed BFD-RS set’ and ‘non-failed BFD-RS set’ are revised as ‘failed BFD-RS set which is configured to be associated with one PUCCH-SR/SR configuration’ and ‘non-failed BFD-RS set which is configured to be associated with one PUCCH-SR/SR configuration’ respectively. Besides, Alt1 and Alt 2 may trigger two PUCCH-SR/SR configurations when two BFD-RS sets configured to be associated with 2 PUCCH-SR resources are failed. But according to the previous agreement, when 2 BFD-RS sets are both failed on SpCell, it will fallback to RACH. Therefore, Alt1 and Alt2 are not accurate. Only one BFD-RS set of two BFD-RS set configured to be associated with 2 PUCCH-SR resource is failed can apply the selection. Therefore, we propose to revise the proposal as follows: Revised FL Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: - Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with the failed BFD-RS set of two BFD-RS set if only one BFD-RS set of the two BFD-RS sets configured to be associated with 2 PUCCH-SR resources is failed; otherwise, it is up to UE implementation. - Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with the non-failed BFD-RS set of two BFD-RS set if only one BFD-RS set of the two BFD-RS sets configured to be associated with 2 PUCCH-SR resources is failed; otherwise, it is up to UE implementation. Alt3: it is up to UE implementation. And we support Alt-2 based on the revised proposal. |
| TCL | Support FL’s updated proposal. We prefer Alt-3 as it does not limit the PUCCH-SR resource selection. |

Summary of discussion on issue 2.5 over email:

Updated FL Proposal 2.5: On the PUCCH-SR resource/SR configurations selection rule when SR is triggered and 2 PUCCH-SR resources are configured,

    Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with failed BFD-RS set.

          OPPO, Nokia/NSB, InterDigital, Spreadtrum, ZTE

     Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with non-failed BFD-RS set

          DOCOMO(1st), ZTE, Sony(1st), Huawei, HiSilicon, CMCC, Samsung, Futurewei, NEC

         Alt3: it is up to UE implementation.

          Sony(2nd)

| Company | Comments |
| --- | --- |
| OPPO | Regarding the listed Alts, we prefer Alt1. And we also suggest to update the main bullet as follows: “On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources/SR configurations are configured” |
| DOCOMO | Our first preference is Alt2. |
| ZTE | Our first preference is Alt-2 |
| Sony | It seems Proposal 2.5 depends on the result of Proposal 2.4. If Alt.2 in Proposal 2.4 is supported, then we don’t need to discuss Proposal 2.5. Is this understanding correct? Assume Alt.1 in Proposal 2.4 is supported, our first preference is Alt2, and we can live with Alt3. |
| vivo | Either Alt1 or Alt2 is working. |
| Apple | We think firstly we need to clarify what “association” means. |
| Huawei, HiSilicon | Alt-2 |
| Nokia/NSB | Prefer Alt-1, also fine with Alt-2. gNB can configure proper PUCCH-SR for either option once agreed. |
| InterDigital | In principle, once we agree on configuration of an association, a UE can be configured either with a failed or non-failed BFD-RS set. But, if we want to pick one, we prefer Alt-1. |
| Xiaomi | Either Alt 1 or Alt 2 is OK to us. |
| Lenovo/MotM | The updated FL proposal is still unclear since there may be multiple BFD-RS sets failed on different CCs and there is no agreement yet that each BFD-RS sets on different CCs can be associated with one PUCCH-SR resource/SR configuration. We proposal that ‘failed BFD-RS set’ and ‘non-failed BFD-RS set’ are revised as ‘failed BFD-RS set which is configured to be associated with one PUCCH-SR/SR configuration’ and ‘non-failed BFD-RS set which is configured to be associated with one PUCCH-SR/SR configuration’ respectively. Besides, Alt1 and Alt 2 may trigger two PUCCH-SR/SR configurations when two BFD-RS sets configured to be associated with 2 PUCCH-SR resources are failed. But according to the previous agreement, when 2 BFD-RS sets are both failed on SpCell, it will fallback to RACH. Therefore, Alt1 and Alt2 are not accurate. Only one BFD-RS set of two BFD-RS set configured to be associated with 2 PUCCH-SR resource is failed can apply the selection. Therefore, we propose to revise the proposal as follows: Revised FL Proposal 2.5: On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured: o Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with the failed BFD-RS set of two BFD-RS set if only one BFD-RS set of the two BFD-RS sets configured to be associated with 2 PUCCH-SR resources is failed; otherwise, it is up to UE implementation. o Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with the non-failed BFD-RS set of two BFD-RS set if only one BFD-RS set of the two BFD-RS sets configured to be associated with 2 PUCCH-SR resources is failed; otherwise, it is up to UE implementation.Alt3: it is up to UE implementation. And we support Alt-2 based on the revised proposal |

| LGE | Either Alt 1 or Alt 2 is fine. |
| --- | --- |
| Spreadtrum | Slightly prefer Alt-1. |
| CMCC | Prefer Alt-2 |
| Samsung | We support Alt-2. |
| Futurewei | We support Alt-2. |
| NEC | Prefer Alt-2. |
| Mod | @ Lenovo/MotM: from my understanding, “UE triggers the PUCCH-SR/SR configuration that is associated with failed/non-failed BFD-RS set” implies that each BFD-RS set at least on that CC is associated with one PUCCH-SR resource/SR configuration. This doesn’t necessarily mean that association between a BFD-RS set on Scell and a PUCCH-SR resource / SR configuration for per TRP BFR has to be supported (i.e., Alt-1 in 2.4). As mentioned in your comment above, when 2 BFD-RS sets are both failed on SpCell, it will fallback to RACH. So, it seems not necessary to emphasis SR selection is needed “if only one BFD-RS set of the two BFD-RS sets configured to be associated with 2 PUCCH-SR resources is failed”. If we share similar understanding, I prefer to keep current wording of 2.5. |

      - Updated FL Proposal 2.5

Updated FL Proposal 2.5: On the PUCCH-SR resource/SR configurations selection rule when SR is triggered and 2 PUCCH-SR resources are configured,

    Alt1: The UE triggers the PUCCH-SR/SR configuration that is associated with failed BFD-RS set.

          (5) OPPO, Nokia/NSB, InterDigital, Spreadtrum, ZTE

     Alt2: The UE triggers the PUCCH-SR/SR configuration that is associated with non-failed BFD-RS set

          (8) DOCOMO(1st), ZTE, Sony(1st), Huawei, HiSilicon, CMCC, Samsung, Futurewei, NEC

     Alt3: it is up to UE implementation.

          (1) Sony(2nd)

      - Agreement of round 1 discussion

Updated FL Proposal 2.5:

On the PUCCH-SR resource/SR configurations selection rule when SR is triggered and 2 PUCCH-SR resource/SR configurations are configured, the UE triggers the PUCCH-SR resource/SR configuration that is associated with failed BFD-RS set.

Issue 2.6: Beam/power update for PUCCH after receiving gNB response

### Round 1

Views from company contributions on issue 2.6 are summarized as follows:

- Support to update spatial relation info. And power control parameter of PUCCH: QC, ZTE, DCM, ETRI, FutureWei (configured with joint TCI), Sony, Lenovo, Motorola Mobility, TCL, Fujitsu, APT, CATT

- Not support: MTK, OPPO (no explicit association between PUCCH and TRP), vivo

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.6: Support beam/power update for PUCCH after receiving gNB response.

- Introduce association between PUCCH and TRP, e.g. through BFD-RS set ID, CORESETPoolIndex, etc.

Note: the term TRP is used only for the purposes of discussions

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | We do not support to introduce association between PUCCH and TRP. First of all, the intention of introducing this extra association is to “fall back” the Tx beam and PC parameters of some PUCCH resource after BFR. That is not a essential issue. We should not introduce something significant just to support some uncritical/non-essential feature. Beam failure is the beam failure of PDCCH beam. It does not imply the link on PUCCH fail. Furthermore, it is multiTRP system, even the DL link of one TRP fails, the link between UE and another TRP is still good. Secondly, introducing this association has significant impact on uplink transmission design. Associating PUCCH resource with TRP has been dicussed heavily in rel16 mTRP. It would have critical impact on the whole uplink transmission procedure. It has imact on HARQ transmission, the multiplexing of different UCI and/or PUSCH. It has impact on how to handle the priority of PUCCH and PUSCH. |
| Apple | Support the proposal. We think the same approach should also be applied to PUSCH/SRS. |
| FGI/APT | Support. We share similar understanding with Apple that it can be applied for PUSCH |
| ZTE | Support. It is essential for maintaining UL transmission well after TRP-specific BFR. In Rel-16, association between PUCCH and CORSET pool ID was agreed, but just some details failed to be captured in the spec due to the lack of TU. In such case, we just need to complete this association by explicit RRC configuration. |
| NTT DOCOMO | Support the proposal. The association configuration between PUCCH and TRP is important for per-TRP UL operation in MTRP scenario. |
| Lenovo/MotM | Support it since PUCCH resources carrying HARQ-ACK are very important for network throughput. Besides, PUCCH beam/power updating in cell-specific BFR is supported already in R15/R16, it is very natural to extend this feature in TRP-specific BFR. |
| NEC | Fine with the proposal. |
| Vivo | We share similar view with OPPO that beam/power update for PUCCH resource is unnecessary. |
| Xiaomi | Support the FL proposal 2.6. since in Rel 16, the QCL of PUCCH can be updated after receiving gNB response, it is straight forward to update it in Rel 17 based on the association between PUCCH and BFD-RS set ID or CORESETPoolIndex. |
| Mod | @OPPO: in our opinion, this proposal is more like an extension of current mechanism to the case of TRP-specifc BFR. As mentioned by many companies, beam/power update for PUCCH after receiving gNB response is supported in legacy spec. So, we don’t understand why such updating can not be extended in Rel-17. Furthermore, we don’t think there’re issues with HARQ or multiplexing of UL channels. Actually, the association between PUCCH and TRP is supported in Rel-16 as well. One of the motivations of introducing such association was just for HARQ related operations in M-DCI. Regarding the multiplexing of UL channels, according to Rel-16 spec, the overlapping rule is applied to the UL channels corresponding to the same TRP, and NW is obligated to keep non-overlapped transmission of UL channels to different TRPs in M-DCI. |
| Huawei, HiSilicon | Support |
| Samsung | Support updating uplink channels after BFRR. |
| Futurewei | Support FL’s proposal. |
| Qualcomm | Support |
| ZTE2 | Support |
| Fujitsu | Support |
| Sony | Support |
| MediaTek | We can compromise on this if this is the majority view. Then, we prfer to resolve the association now, i.e., FL Proposal 2.6: Support beam/power update for PUCCH after receiving gNB response. - Introduce association between PUCCH and TRP, e.g. through a BFD-RS set ID, CORESETPoolIndex, etc. Note: the term TRP is used only for the purposes of discussions In fact, during the email discussion in the previous meeting, there was a more clear proposal as follows, we can start from it. Updated FL Proposal 2.8: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) - The power for the corresponding PUCCH transmission is set to [], , and [], where is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set - FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set |
| Mod | Thanks MTK for reminding me. Previous FL proposal seems more clearer to us. Updated FL Proposal 2.6: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) - The power for the corresponding PUCCH transmission is set to [], , and [], where is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set - FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set |
| CMCC | Support the proposal. |
| Vivo | Do not support the latest FL proposal. The PUCCH spatial relation can be updated by legacy signaling. |
| ZTE | Support the FL’s updated proposal |
| NTT DOCOMO | Support the proposal. |
| Lenovo/MotM | Support the updated proposal in princle. However, we have concern about the qu updating since 2 PUCCH resources transmitting to 2 TRPs should have different power, therefore, the qu should be different for the PUCCH resources transmitting to different TRPs. |
| Spreadtrum | Support |
| TCL | Support the proposal. |

### Round 2

FL Proposal 2.6-2:  After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found)

- The power for the corresponding PUCCH transmission is set to  [], , and [], where  is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set

- FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support |
| Lenovo/MotM | Support the updated proposal in princle. However, we have concern about the qu updating since 2 PUCCH resources transmitting to 2 TRPs should have different power, therefore, the qu should be different for the PUCCH resources transmitting to different TRPs. For example, qu=0 for PUCCH resources associated with BFD-RS set 0 and qu=1 for PUCCH resources associated with BFD-RS set 1. And gNB can configure qu=0 for TRP1 associated with BFD-RS set 0 and configured qu=1 for TRP2 associated with BFD-RS set 1. |
| H3C | Support |
| ZTE | Support |
| InterDigital | Support |
| vivo | Not support to reset the spatial relation info of PUCCH resource by q_new. |
| Futurewei | Support. |
| OPPO | Four comments: - Our 1st concern is this proposal would introduce the assocaition between PUCCH resource and TRP. That was dicussed heavily in rel16 mTRP but not supported. Introducing such association would have critical impact on all the UL UCI transmission in mTRP system. Therefore, we prefer to reuse the current RRC parameter to support this function. The PUCCH group used for PUCCH group-based spatial relation info update can be used here. So we suggest to use that. - We shall only reset those PUCCH resources that are configured with spatial relation info because those PUCCH resource that are not provided with spatial relation info would follow the default Tx beam, which was specified in rel16. - We share the same understanding as vivo that we can not use q_new to reset the PUCCH. It should be: the latest PRACH transmission for Pcell and q_new on SCell. - If the UE already use the PUCCH to transmit SR, the PUCCH shall not be reset because the SR is sent and that means the UL beam is fine. Therefore, suggest the following changes for the proposal: Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set: - The power for the corresponding PUCCH transmission is set to [], , and [], where is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission. - FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set The PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set. |
| NTT DOCOMO | Support. UL spatial filter and power reset after BFR has been supported in legacy spec. Extend current mechanism to the case of TRP-specifc BFR is straightforward. And it is essencial for UL operation. |
| Samsung | Support FL’s proposal. Not sure why q_new cannot be applied to PCell in Rel. 17. |
| Xiaomi | Support |
| Mod | @ Lenovo/MotM: thanks for the support. Regarding the value of qu, in my opinion, whether to set independedt indexes of object receiving power or just to use the default value(i.e., 0) seems not that enssential. Anyway, as qu=0 is still within [ ], we can discuss this further this week. @OPPO: the proposal is reviesed based on your suggestion. Updated Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set: - The power for the corresponding PUCCH transmission is set to [], , and [], where is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission. - FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set - E.g. the PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set. |
| Sony | Support to recover PUCCH’s beam and Tx power in principle. One thing we feel the same with OPPO is that for PUCCH’s new beam, qnew could be determined either by the reported beam from candidate beam RSs or by the selected PRACH in PCell. We think similar recover behavior in AI 8.1.1. was also discussed on Tuesday’s GTW. |
| CMCC | Support the revised proposal. |
| ZTE2 | We are fine with the updated proposal from FL. BTW, clearly, this issue may have some RRC impacts, and so we think that we may need to handle the last FFS with high priority. |
| ETRI | Support the latest proposal. |
| Lenovo/MotM | We have concern about the PRACH Tx beam part on PCell. In cell-specific BFR on SpCell, it is correct that the new beam is the beam used to transmit the latest PRACH on SpCell since it is RACH-based cell-specific BFRQ. PRACH will always be transmitted during the BFRQ procedure of cell-specific BFR on SpCell. However, in TRP-specific BFR, if the new beam is indicated by a TRP-specific BFR MAC-CE, there is no such limitation that a PRACH will be transmitted using the new beam during the TRP-specific BFRQ procedure. Therefore, the beam used to transmit the latest PRACH on PCell is not always the new beam. And we propose to revise the proposal as follows: Updated Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set: - The power for the corresponding PUCCH transmission is set to [], , and [], where is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission. - FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set - E.g. the PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set. |

Summary of email discussion

Updated Proposal 2.6-2:  After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set:

         The power for the corresponding PUCCH transmission is set to  [], , and [], where  is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission.

         FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set

o    E.g. the PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set.

| Company | Comments |
| --- | --- |
| Sony | Support to recover PUCCH’s beam and Tx power in principle. One thing we feel the same as OPPO in their email inputs is that for PUCCH’s new beam, qnew could be determined either by the reported beam from candidate beam RSs or by the selected PRACH in PCell. We think similar recover behavior in AI 8.1.1. was also discussed on Tuesday’s GTW. |
| Apple | Support in principle. Suggest changing “PUCCH cell” into “spCell/PUCCH-SCell" |
| DOCOMO | Support the FL proposal and also support Apple’s revision, which is more accurate. |
| ZTE | Support. BTW it seems that the formula can not be presented well. |
| LGE | Generally fine in principle. |
| vivo | The gif is missing, not sure about the details. |
| OPPO | Generally ok in principle. |
| Spreadtrum | Support |
| Mod | @ZTE, Apple and vivo: 2.6-2 is revised based on your suggestions with the missing formulas fixed. Updated Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell spCell/PUCCH-SCell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell spCell/PUCCH-SCell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set:  The power for the corresponding PUCCH transmission is set to [qu=0], qd= qnew, and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission.  FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set o E.g. the PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set. |
| ETRI | Support the latest updated proposal. |
| Ericsson | Support the latest updated proposal |
| Samsung | Support |
| Convida | Generally ok in principle. |
| Qualcomm | Suggest to remove the PRACH Tx beam part. Resetting to PRACH beam may not work. Suppose TRP1 fails, and UE then initiates CBRA with best PRACH beam, which could be on TRP2. So resetting to TRP2 beam cannot recover the communications on the failed TRP1. So we suggest to always use the reported q_new, which is selected among candidate beams for the failed TRP. If no such q_new, better not to reset and wait for beam measurement. Updated Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell spCell/PUCCH-SCell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell spCell/PUCCH-SCell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set:  The power for the corresponding PUCCH transmission is set to [qu=0], qd= qnew, and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission. |
| Huawei, HiSilicon | Prefer the revised version from Qualcomm. |
| Mod | @OPPO: please confirm if the revision from Qualcomm is acceptable to you. |
| OPPO | @Mod and QC: I think the revision by QC does not make sense. In RACH-based PCell BFR, the UE choose the Tx beam for RACH based on the associated DL RS, i.e., the qnew. So if the qnew is for this TRP, the corresponding Tx beam on the PRACH is also for the same TRP. So, we do not agree with the revision by QC. |
| Lenovo/MotM | We support the revised version from Qualcomm. |
| Xiaomi | We are fine with the version from Qualcomm. |
| DOCOMO2 | After carefully reading the FL’s Proposal 2.6-2, we feel that it mixed different cases together. If we’d like to consider different cases, we suggest discussing difference cases separately and we can have different proposals for different cases. For the QC’s version, I think we just need to focus on the case when one TRP on SpCell/PUCCH-SCell fails and its associated PUCCH beam/power can be reset if new beam is reported in the BFR MAC CE. For the other cases mentioned by OPPO, we can have separate proposal for it. In addition, we do not understand the meaning of ‘if SR is not transmitted …’ in main bullet. We suggest removing it. Updated Proposal 2.6-2: After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a PUCCH cell spCell/PUCCH-SCell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the PUCCH cell spCell/PUCCH-SCell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) on SCell or the beam used to transmit the latest PRACH on PCell, if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set:  The power for the corresponding PUCCH transmission is set to [qu=0], qd= qnew, and [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set or the Tx beam used on latest PRACH transmission. |
| LGE2 | OK with Qualcomm’s revision. |
| Intel | OK with DOCOMO2 revision |
| OPPO | We are not ok with the revision by DOCOMO: - The revision of DOCOMO seems to assume to only support SCell. In SpCell, the BRF is PRACH-based, where the qnew is reported through RACH preamble transmission. Thus, the only valid Tx beam for PUCCH is the Tx beam used in latest PRACH transmission. That is also what specified in PCell BFR. Please check the rel15 design. - We do not agree to delete “if SR is not transmitted on ..”. If SR can be transmitted successfully on the PUCCH resource associated with the TRP, that means the uplink connection is till fine, then we should not reset the Tx beam of PUCCH. Please check the current design of SCell BFR, the same condition is used there too. |
| DOCOMO3 | To OPPO’s comments. 1. For PRACH based BFR for SpCell, we just suggested using separate proposal instead of mixing all the cases together, which makes the Proposal 2.6-2 unreadable. In Rel15 spec., a separate paragraph in 213 is used for CFRA BFR for SpCell. In Rel16 spec., another separate paragraph is added in 213 for CBRA BFR if BFR MAC CE is transmitted in Msg3/MsgA. In Rel17 spec., for CBRA BFR for per-TRP BFR, we also suggest using a separate proposal and separate paragraph. Could OPPO kindly provide a new proposal for CBRA BFR on SpCell and then we can discuss the two clear separate proposals ? 2. We deleted it as it is quite confusing with ‘PUCCH resource associated this failed BFD-RS set’. We just had an agreement to trigger PUCCH-SR associated with failed BFD-RS set. Based on this agreement, if a BFD-RS set fails, the associated PUCCH-SR will be triggered and transmitted. But I think when you say ‘if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set’, you mean the PUCCH resources and the association that are already mentioned in the main bullet before this sentence (not the association in the agreement). With above understanding in mind, we can accept to keep ‘and if SR is not …’. We hope above clarifies our intension. |
| Lenovo/MotM | We support the revised proposal by DOCOMO. In cell-specific BFR on SpCell, it is correct that the new beam is the beam used to transmit the latest PRACH on SpCell since it is RACH-based cell-specific BFRQ. PRACH will always be transmitted during the BFRQ procedure of cell-specific BFR on SpCell. However, in TRP-specific BFR, if the new beam is indicated by a TRP-specific BFR MAC-CE, there is no such limitation that a PRACH will be transmitted using the new beam during the TRP-specific BFRQ procedure. Therefore, the beam used to transmit the latest PRACH on PCell is not always the new beam. Besides, in UE sides, if SR resources associated with the failed TRP are transmitted, it does not mean that UE can know whether they can be transmitted successfully. |
| CMCC | We prefer Qualcomm’s revision. For RACH-based BFR, the Tx beam for RACH is not specified in the spec, we think it is up to UE implementation, hence Qualcomm’s comments make sense. |
| Futurewei | OK with Qualcomm’s revision. |
| Mod | @OPPO: I have one question for clarification. In the response to QC, it’s mentioned that “In RACH-based PCell BFR, the UE choose the Tx beam for RACH based on the associated DL RS, i.e., the qnew. So if the qnew is for this TRP, the corresponding Tx beam on the PRACH is also for the same TRP”. Does this mean that the latest reported new candidate beam for the failed BFD –RS set is just the Tx beam used on latest PRACH transmission? If so, why do we need to differentiate such two kinds of beams? |

Updated Proposal 2.6-2:  After 28 symbols from receiving the BFR response, if a failed BFD –RS set on a spCell/PUCCH-SCell is indicated in the TRP –specific BFR MAC-CE, the UL spatial filter for PUCCH resource(s) associated with the failed BFD –RS set on the spCell/PUCCH-SCell is updated by the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set (if found) , if the PUCCH resource is provided with spatial relation info and if SR is not transmitted on any PUCCH resource associated this failed BFD-RS set:

  The power for the corresponding PUCCH transmission is set to  [qu=0], qd= qnew, and  [l=0], where qnew is the RS resource associated with the latest reported new candidate beam for the failed BFD –RS set.

   FFS : Detail on the association between the PUCCH resource(s) and the failed BFD –RS set

o    E.g. the PUCCH group for group-based PUCCH-spatial relation info update is associated with each BFD-RS set.

Issue 2.7: Beam update for PDSCH after receiving gNB response

### Round 1

Views from company contributions on issue 2.7 are summarized as follows:

- Update QCL of PDSCHs after receiving BFR response: MTK, OPPO, Samsung

Based on views of majority companies, the following proposal is suggested.

Updated FL Proposal 2.7: After receiving gNB response, the UE assumes that

- The QCL assumption of PDSCH associated with the failed TRP link to be the latest reported qnew.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Support the proposal |
| Apple | Support the proposal |
| FGI/APT | Support FL’s proposal |
| ZTE | Support the proposal in principle. The PDSCH should be clarified as ‘PDSCH scheduled by the PDCCH in the CORESET associated with failed TRP link’. |
| NTT DOCOMO | We think it is not needed. But we can live with it. |
| Lenovo/MotM | We don’t think it’s needed considering the beam of PDSCH is determined according to PDCCH beam, and the beam updating of PDCCH is already supported in TRP-specific BFR. |
| LGE | Not support. It is purely up to gNB. Copy and paste our comment in the last meeting as below. We don’t think that there is no legacy mechanism specified for automatic TCI update for PDSCH. The reason was that PDSCH TCI can be well updated by existing PDSCH TCI update mechanisms, e.g. PDSCH TCI follows that of scheduling PDCCH when no TCI present in DCI. For another example, when PDSCH is scheduled within timedurationforQCL, PDSCH TCI follows the TCI of lowest ID CORESET, as vivo mentioned. In these cases, the CORESET TCI update mechanism (already agreed) is sufficient for PDSCH TCI update without any further additional spec impact. If TCI presents in DCI and if PDSCH is scheduled above the threashold, it is up to gNB whether to indicate the same TCI as PDSCH or different TCI. The proposal will only kill such gNB’s dynamic TCI indication functionality with no gain. |
| NEC | We are open to discuss this. While one thing to clarify, does this mean after gNB response, the TCI state field (if present) in the detected DCI is ignored? And the duration/timing (both starting and until timing) for this ehavior should be considered, for example, after X symbols and until a new MAC CE activation. |
| Vivo | Do not support FL’s proposal. |
| Xiaomi | We think the TCI state of PDSCH can be determined by the existed mechanism of default TCI state or indication in TCI field. |
| Mod | @NEC: regarding your question for clarification, my understanding is yes. @Proponents to this proposal: as raised by NEC, clarification is needed on the duration of such behavior. For further discussion, FL Proposal 2.7 is updated according to ZTE’s suggestion. Updated FL Proposal 2.7: After receiving gNB response, the UE assumes that the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew. |
| Samsung | Support the FL’s proposal. Our understanding is that the behavior continues until beam activation/update. |
| Futurewei | As commented by some companies, the duration/until timing is also needed. We suggest the following modifications: Updated FL Proposal 2.7: After receiving gNB response, the UE assumes that the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew until the UE receives TCI update from the gNB. |
| Qualcomm | Support |
| ZTE2 | We are fine with FL’s update, and open to consider the until timing. |
| Fujitsu | Support the updated FL’s proposal. |
| Sony | Not support. It seems not necessary in our view. A) the recovered PDCCH can provide TCI state for PDSCH to follow or b) legacy default beam rules can be applied. |
| MediaTek | We prefer to add the PDSCH update and timing in the previois agreement for PDCCH beam update as follows, which will be more clear and consistent. For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) and for corresponding PDSCH reception is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD –RS set k (k=0,1) in the MAC-CE for TRP –specific BFR until the UE receives any TCI update from the gNB. - The above applies to Scell and SpCell - The above applies for the multi-DCI case |
| Mod | Further update of this proposal as recommended by NEC, Samsung, Futurerwei and MTK: Updated FL Proposal 2.7: After receiving gNB response, the UE assumes that the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew until the UE receives any TCI update from the gNB. - The above applies to Scell and SpCell - The above applies for the multi-DCI case @MTK: update of QCL assumption for CORESET has already been captured in previous agreement. So, this part is not repeated here. |
| Nokia/NSB | Support the proposal. |
| CMCC | Support. |
| Vivo | Do not support the updated FL proposal. We do not see the necessity to reset beams of PDSCH after beam failure. The legacy QCL indication mechanism for PDSCH is sufficient. |
| Ericsson | Support |
| MediaTek | Supprot the asts proposal in principle. However, one protensial issue is the timelines for PDSCH beam update and PDCCH beam update ate different. For PDCCH beam update agreed in previous meeting, there is no “until the UE receives any TCI update from the gNB”. We sugget to align these two timelines, either w/ or w/o “until the UE receives any TCI update from the gNB”. |
| ZTE2 | Regarding ‘until….’, we sympathize the MediaTek’s views that the corresponding part may not be very necessary. |
| TCL | Support FL’s proposal |

### Round 2

FL Proposal 2.7-2: After receiving gNB response, the UE assumes that

the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew until the UE receives any TCI update from the gNB.

- The above applies to Scell and SpCell

- The above applies for the multi-DCI case

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support the proposal. One question is whether this behavior (appling q_new as the QCL source of PDSCH) is also applicable for unified TCI framework being discussed in 8.1.1? |
| Lenovo/MotM | We still don’t see the necessary to support this proposal. However, if majority supports it, we can live with it. |
| H3C | Support |
| ZTE | Support. Alternatively, we may treat this issue for next meeting considering that there is no RRC impact. |
| InterDigital | Support FL proposal |
| vivo | Not support. |
| Futurewei | Support. |
| OPPO | Support |
| NTT DOCOMO | Not needed. But can live with it. |
| MediaTek | Fine in principle. However, one protensial issue is the timelines for PDSCH beam update and PDCCH beam update ate different. For PDCCH beam update agreed in previous meeting, there is no “until the UE receives any TCI update from the gNB”. We sugget to align these two timelines, either w/ or w/o “until the UE receives any TCI update from the gNB”. |
| Samsung | Support the proposal |
| Xiaomi | We think it is not needed but we can accept it if majorities support it. |
| Mod | @HW: thanks for supporting this proposal. Regarding the question, my understanding is that per-TRP BFR is the enhancement based on Rel-15/16 spec. So, I don’t think it’s applicable for unified TCI framework. @MTK: thanks for supporting. For the timeline issue, adding “until the UE receives any TCI update from the gNB” seems not necessary for CORESET, since QCL assumption of CORESET can be updated by gNB, and previous agreement on update of QCL assumption for CORESET after receiving response doesn’t preclude this. @HW, MTK and all: please correct me, if I’m wrong. |
| Sony | Not Support. The QCL assumption of PDSCH in nature can be dynamic, and this is different from that of CORESET which is activated with 1 or 2 TCI state(s). Moreover, it works via implementations that a) the recovered PDCCH can provide TCI state for PDSCH to follow or b) legacy default beam rules (PDSCH either follows the QCL assumption of scheduling CORESET or CORESET with lowest ID in the most recent slot) can be applied. Hence, similar as legacy BFR, it seems not necessary to specify such UE behavior. |
| CMCC | Support. |
| LGE | Not support. As many companies commented, it kills gNB flexibility of dynamic TCI indication. Furthermore, PDSCH TCI can be well updated by existing PDSCH TCI update mechanisms, e.g., PDSCH TCI follows that of scheduling PDCCH when no TCI present in DCI, and PDSCH TCI follows the TCI of lowest ID CORESET when PDSCH is scheduled within timedurationforQCL. |
| Lenovo/MotM | We shared similar view with LGE. |

Summary of email discussion

FL Proposal 2.7-2: After receiving gNB response, the UE assumes that

the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew until the UE receives any TCI update from the gNB.

        The above applies to Scell and SpCell

        The above applies for the multi-DCI case

| Company | Comments |
| --- | --- |
| Sony | Not Support. The QCL assumption of PDSCH in nature can be dynamic, and this is different from that of CORESET which is activated with 1 or 2 TCI state(s). Moreover, it works via implementations that a) the recovered PDCCH can provide TCI state for PDSCH to follow or b) legacy default beam rules (PDSCH either follows the QCL assumption of scheduling CORESET or CORESET with lowest ID in the most recent slot) can be applied. Hence, similar as legacy BFR, it seems not necessary to specify such UE behavior. |
| Apple | Support |
| DOCOMO | Not needed. But can accept it. |
| ZTE | Support |
| LGE | Not support. As many companies commented, it kills gNB flexibility of dynamic TCI indication. Furthermore, PDSCH TCI can be well updated by existing PDSCH TCI update mechanisms, e.g., PDSCH TCI follows that of scheduling PDCCH when no TCI present in DCI, and PDSCH TCI follows the TCI of lowest ID CORESET when PDSCH is scheduled within timedurationforQCL. |
| MediaTek | “UE receives any TCI update from the gNB” is not needed. This proposal w/o the sentence doesn't prevent NW to indicate a TCI state for the PDSCH reception later. |
| vivo | Not support. Similar view as DCM/LGE. |
| OPPO | Support |
| Mod | 2.7-2 is revised according to MTK’s comment. Updated FL Proposal 2.7-2: After receiving gNB response, the UE assumes that the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew until the UE receives any TCI update from the gNB.  The above applies to Scell and SpCell  The above applies for the multi-DCI case |
| Ericsson | Support the updated proposal. |
| Samsung | Support |
| Convida | Doesn't seem to be needed. |

| Qualcomm | Fine with the latest FL proposal |
| --- | --- |
| Huawei, HiSilicon | Fine with the latest FL proposal. |
| Lenovo/MotM | Not support. Similar view with DCM/LGE. |
| Xiaomi | Not needed, but we can accept it. |
| LGE2 | As we commented, it is not needed. |
| DOCOMO3 | After some further checking, we do not support this Proposal. When tci-PresentInDCI is not enabled and scheduling offset is no less than UE capability, QCL of PDSCH should follow the scheduling PDCCH, based on existing spec.. However, if this proposal is applied, after BFR, PDSCH is reset to q_new. After that, if TCI state of PDCCH is updated to TCI#1, based on existing spec., beam of PDSCH will be updated to TCI#1 as well. However, with this proposal, we’re not sure whether it means beam of PDSCH will be kept as q_new. To avoid misunderstanding brought by this proposal, we think existing spec. on PDSCH QCL is sufficient. |
| CMCC | Support |
| Futurewei | OK with FL’s latest proposal. |
| Nokia/NSB | We don’t think PDSCH QCL assumption is need to be discussed. |

Updated FL Proposal 2.7-2: After receiving gNB response, the UE assumes that

the QCL assumption of PDSCH scheduled by the PDCCH in the CORESET associated with the failed TRP link to be the latest reported qnew .

        The above applies to Scell and SpCell

        The above applies for the multi-DCI case

Issue 2.8: Association between CORESET(s) and failed BFD-RS set

### Round 1

Views from company contributions on issue 2.8 are summarized as follows:

- Update of channels/RSs for explicit BFD-RS set configuraiton of S-DCI

- Associate CORESETs with BFD-RS sets by RRC signaling or MAC-CE: QC, Lenovo, Motorola Mobility, Intel, DCM, CATT

- Associate TCI state with BFD-RS sets: DCM

- Not support: MTK, vivo

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.8: To associate CORESET(s) with failed BFD-RS set

- For explicit BFD-RS configuration of S-DCI

- Alt1: Support association configuration between TCI state and BFD-RS set

- Alt2: Support association configuration between CORESET and BFD-RS set

Companies’ views on issue 2.8 are listed as follows:

| Company | Comments |
| --- | --- |
| OPPO | Do not support the proposal. There is no motivation to associate CORESETs with BFD-RS set in sDCI. |
| Apple | This depends on the outcome of issue 3.2. |
| ZTE | We are open to have some further discussion. We slightly prefer to Alt-2, but Alt-1 may be good for the subsequent unified TCI enhancement for the M>1 and N>1 case. |
| NTT DOCOMO | Support the proposal. Alt1 is our first preference. |
| Lenovo/MotM | Support the proposal. And we slightly prefer Alt-1. |
| LGE | We have similar view as OPPO. The original purpose of the association between BFD-RS set (index) and CORESET (pool) was for implicit BFD-RS determination for M-DCI based M-TRP case. We don’t think it is necessary. |
| NEC | Support the proposal. And either alternative is fine. |
| Vivo | Do not support the FL’s proposal. We do not see the necessity to reset beams of CORESETs after beam failure for S-DCI. The beam can always be updated with the existing MAC CE. |
| Xiaomi | It seems to solve the problem of explicit configuarion of BFD-RS by RRC. For CORESET with only one active TCI state, we prefer Alt 2. For CORESET with two active TCI states, Alt 1 is more better. |
| Mod | @OPPO @vivo: in the last meeting, we have the following agreement on the association of BFD-RS set and CORESET. Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD –RS set k (k=0,1) in the MAC-CE for TRP –specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case For the case of S-DCI, update of QCL assumption of the CORESETs from the failed TRP is also needed. |
| Huawei, HiSilicon | Support Alt-2. |
| Samsung | Support FL’s proposal. Both alternatives are fine for us. |
| Qualcomm | Support Alt2 |
| Fujitsu | Although the FL’s proposal is not our first preference, we can live with it. |
| Sony | Support Alt2. |
| MedaiTek | Alt1 is unclear for us. For the TCI state here, does it mean the configured TCI states? So, there will be two sets of configured TCI states associated with different BFD-RS sets, respectively? |
| Mod | @MTK: from my understanding, in Alt-1, the configured/activated TCI states can be divided into two sets, and each of them is associated with a BFD-RS set. |
| Nokia/NSB | Support Alt-2. |
| CMCC | Slightly prefer Alt2. |
| Vivo | Do not support updated FL proposal. Overall, we have concerns on agreeing the behavior of resetting beams in the last meeting of Rel-17 FeMIMO discussion, which involve grouping of CORESETs, grouping of PDSCH, grouping of PUSCH and grouping of PUCCH. All these would require quite a lot of specification enhancement. On the other hand, reusing the legacy beam indication mechanism can already provide the mechanisms to update the beam for CORESETs/PDSCH/PUSCH/PUCCH. We don’t think there is any need to further enhance beam resetting behavior for sDCI scenarios |
| Ericsson | We share the concern expressed by OPPO, vivo, and LGE. Note that for Multi-DCI, we discussed association between BFD-RS set and CORESETs associated with a CORESETPoolIndex. But that is for implicit BFD-RS and not for explicit BFD-RS. |
| MediaTek | We prefer Alt2, or to postpone this low priority scenario |
| Spreadtrum | Not support. The motivation is not clear to us. |
| TCL | We support Alt-2. |

### Round 2

FL Proposal 2.8-2: To associate CORESET(s) with failed BFD-RS set

- For explicit BFD-RS configuration of S-DCI

- Alt1: Support association configuration between TCI state and BFD-RS set

- Alt2: Support association configuration between CORESET and BFD-RS set

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support Alt-2. |
| Lenovo/MotM | We are OK with both alternatives. |
| H3C | Support Alt.2 |
| Convida | Given all the concerns in the 1st round, we suggest to instead conclude that there is no consensus to support association of CORESETs with failed BFD-RS set for S-DCI. |
| ZTE | Support Alt-2. Alternatively, we may conclude this issue as Convida suggested. |
| InterDigital | Support Alt-2. |
| vivo | Do not support the FL proposal. On the one hand, resetng beams for sDCI scenario may results in quite a lot of specification effort, since it requires grouping of CORESETs, grouping of PDSCH, grouping of PUCCH, and grouping of PUCCH, which are not supported in the current specification. On the other hand, update of QCL assumption of channels mentioned above can be achieved by legacy beam indication mechanism. In all, there is no need to further enhance beam resetting behavior for sDCI scenarios. |
| Futurewei | Support Alt-2. |
| OPPO | Do not support the Alt1 or Alt2 in the proposal. In S-DCI system, there is actually no multi TRP concept on PDCCH. Please note, in sDCI system, the mTRP transmission scheme is defined for PDSCH and simultaneous transmission of PDSCH is supported. It does not make sense to paritition the PDCCH into TRP. |
| NTT DOCOMO | Support the proposal. Alt1 is our first preference. Can also accept Alt2. |
| Xiaomi | Support Alt 2 |
| Sony | Support Alt2. |
| CMCC | Support Alt2. |
| LGE | Same view as vivo, OPPO. We don’t see the benefit. |

Summary of email discussion

FL Proposal 2.8-2: To associate CORESET(s) with failed BFD-RS set

         For explicit BFD-RS configuration of S-DCI

        Alt2: Support association configuration between CORESET and BFD-RS set

| Company | Comments |
| --- | --- |
| Sony | Support the FL proposal. |
| Apple | We think we need to decide the details for explicit BFD RS configuration first. |
| DOCOMO | Support. |
| ZTE | We share the same views with Apple. |
| LGE | Not support. We don’t see the benefit. For implicit BFD-RS case, BFD-RS is derived from CORESET, so it is natural to update CORESET beam (which is already beam failed) to new beam after gNB response. But for explicit BFD-RS case, we cannot find the needs to do so. |
| MediaTek | Share the same view with Apple and ZTE. If no conclusion is made for explicit BFD-RS determination, we don't need to spend time discussing this issue. |
| vivo | Not support. |
| OPPO | do not support this proposal. In S-DCI system, there is no partition on PDCCH according to the TRP. Instead, all the schemes specified in S-DCI system so far are about simultaneous transmission from both TRPs. |
| Spreadtrum | Not support. Not clear why to associate configuration between CORESET and BFD-RS set for explicit configuration for S-DCI based, |
| Ericsson | Do not support. Similar view as LGE. |
| Samsung | We are fine to discuss explicit BFD-RS configuration first. |
| Convida | Doesn't seem to be needed. |

| Qualcomm | Support. Our understanding is that DCI can also be sent from both TRPs in sDCI mTRP. In R16, each CORESET has only one TCI, and hence must be associated with one TRP. So it is a valid scenario to have two CORESET groups for two TRPs respectively to improve DCI robustness against beam blocking. |
| --- | --- |
| Lenovo/MotM | Support. |
| Xiaomi | We need to clarify that this proposal is for association CORESET(s) with failed BFD-RS set or for explicit BFD-RS configuration of S-DCI? From the current version, it seems there are two purposes. We suggest to remove “To associate CORESET(s) with failed BFD-RS set” first to make it clear. |
| Intel | Support, same view as QC |
| CMCC | Support. We are also supportive to discuss the details of explicit BFD RS configuration. |
| Futurewei | OK with the proposal. |
| Mod | @Xiaomi: to my understanding, the intention of this proposal is to associate CORESET(s) with failed BFD-RS set. In the last meeting, we have the following agreement on updating of QCL assumption of all the CORESETs associated with the failed BFD-RS set. Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case For S-DCI, if similar mechanism of beam update for CORESET after reception of response is to be introduced, we still need to associate CORESET(s) with BFD-RS set. |
| Nokia/NSB | We were originally fine with this, but understand LG’s concern. gNB scheduling flexibility will be limited. We need more time for this issue including other option association of TCI states. |

FL Proposal 2.8-2: To associate CORESET(s) with failed BFD-RS set

         For explicit BFD-RS configuration of S-DCI

        Alt2: Support association configuration between CORESET and BFD-RS set

- Support: (6) Sony, Qualcomm, Lenovo/MotM, Intel, CMCC, Futurewei

- Not support: (8) DOCOMO, LGE, vivo, OPPO, Spreadtrum, Ericsson, Convida, Nokia/NSB

Issue 2.9: RACH based transmission

### Round 1

In the last meeting, the following agreement on scenarios of triggering RACH-based transmission was reached.

Agreement

RACH-based transmission can be triggered on a SpCell at least in the following scenarios

- Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

- FFS: other scenarios

  - Scenario 2: at least one TRP fails on SpCell

  - Scenario 3: at least one pre-defined TRP fails on SpCell

  - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

  - Scenario 5: If MAC-CE based reporting does not work (details FFS)

  - Scenario 6: When no PUCCH-SR is configured

For the scenarios listed as FFS, views from company contributions are summarized as follows:

- Not support additional scenarios triggering RACH-based transmission on SpCell: MTK, CATT

- Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available: InterDigital, NEC, ITRI, TCL, APT (for bth SpCell and Scell)

- Scenario 5: If MAC-CE based reporting does not work (details FFS): Convida

- Scenario 6: When no PUCCH-SR is configured: InterDigital, TCL

- OPPO: CBRA-based transmission can be triggered when MAC CE based BFRQ transmission per TRP is not transmitted successfully in SpCell.

Based on views of majority companies, the following proposal is suggested.

FL Proposal 2.9: On RACH-based transmission on a SpCell

- Alt-1: Not support additional scenarios triggering RACH-based transmission on SpCell

- Alt-2: Down select among the following scenarios or triggering RACH-based transmission

- Scenario 4: At least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

- Scenario 5: If MAC-CE based reporting does not work (details FFS)

- Scenario 6: When no PUCCH-SR is configured

- Scenario 7: CBRA-based transmission is triggered when MAC CE based BFRQ transmission per TRP is not transmitted successfully in SpCell

Companies’ views on issue 2.9 are listed as follows:

| Company | Comments |
| --- | --- |
| OPPO | How and when to trigger RACH transmission is per RAN2 procedure. We can live with Alt-1 |
| Apple | Scenario 6 should be supported at least, which is the default behavior for legacy SR triggering. We should be careful to make a decision like Alt1, since RAN2 is also discussing something related. |
| FGI/APT | We support Scenario 4. Under this scenario, it seems no way for UE to report BFRQ MAC-CE without performing RA. In addition, it seems Scenario 5 and 7 point to the same thing ? |
| ZTE | Scenario 6 should be supported by default as in RAN2 spec. Besides, we support Scenario 3. |
| NTT DOCOMO | There is no need to further discuss this issue in RAN1. Leave it to RAN2. |
| LGE | We are not sure whether other scenarios such as scenario 4/6 has additional RAN1 impact. Our suggestion is to move forward with agreed scenario(i.e. when both TRPs are in BF) and see whether other RAN1 aspects need to be accommodated for other scenario(s), which might be requested by RAN2 later. |
| NEC | We are fine with scenario 4. |
| Vivo | We do not support additional scenarios. The already agreed scenario has many remaining issues to resolve. |
| Xiaomi | We are confused on the difference of scenario 4/6, we prefer to support scenario 4. But for scenario 6, it is not reasonable since it doesn’t mention that one TRP fails. |
| Mod | According to the comment from FGI/APT, we have the following revision. FL Proposal 2.9: On RACH-based transmission on a SpCell - Alt-1: Not support additional scenarios triggering RACH-based transmission on SpCell - Alt-2: Down select among the following scenarios or triggering RACH-based transmission - Scenario 4: At least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - E.g. CBRA-based transmission is triggered when MAC CE based BFRQ transmission per TRP is not transmitted successfully in SpCell - Scenario 6: When no PUCCH-SR is configured |
| InterDigital | Scenario 4 and 6 should be supported, but we are ok to wait for RAN2 to discuss it. |
| Huawei, HiSilicon | Support Alt-1. |
| Samsung | We are open to discuss Scenarios 4 and 6. |
| Futurewei | Support Alt-1. |
| Qualcomm | Support Scenario 4 in Alt2 |
| ZTE | We think that Scenario 6 should be supported by default. If fail to be agreed in RAN1, we may not make the final conclusion right now as Alt-1. |
| Fujitsu | Support Alt-1. |
| Sony | Support Alt-2. At least, the BFR under Scenario 4 should be supported. |
| NTT DOCOMO | Based on existing spec., if there is no SR configured for a UE, and UE has UL data for transmission, RACH will be triggered by UE to acquire UL grant. It has been supported in existing spec. And there is no need to further discuss this. In addition, it should be RAN2 issue. Support Alt-1. |
| MediaTek | Support Alt-1. |
| Nokia/NSB | We don’t think any further agreement is required. Whole discussion should be up to RAN2. |
| Convida | Support the proposal. Scenario 4 and 5 can be supported. |
| Ericsson | Support Alt-1 (not support additional scenarios). |
| Lenovo/MotM | Support Alt-1. |
| TCL | We think support Scenario 4 and Scenario 6 should be supported. |

### Round 2

FL Proposal 2.9-2: On RACH-based transmission on a SpCell

- Alt-1: Not support additional scenarios triggering RACH-based transmission on SpCell

- Alt-2: Down select among the following scenarios or triggering RACH-based transmission

- Scenario 4: At least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

- Scenario 5: If MAC-CE based reporting does not work (details FFS)

  - E.g. CBRA-based transmission is triggered when MAC CE based BFRQ transmission per TRP is not transmitted successfully in SpCell

- Scenario 6: When no PUCCH-SR is configured

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Huawei, HiSilicon | Support Alt-1. |
| Lenovo/MotM | Support Alt-1. |
| H3C | Support Alt-1. |
| Convida | Support the proposal. |
| ZTE | No more discussion is needed. It seems that some on-going discussion has been kicked off in RAN2. |
| InterDigital | We support Alt-2, with scenario 4 and 6. We are also ok to leave this discussion to RAN2. |
| vivo | Support Alt-1. |
| Futurewei | Support Alt-1. |
| OPPO | Alt-1 |
| NTT DOCOMO | Not needed. RAN2 issue. |
| MediaTek | Alt-1 |
| Xiaomi | We are open to discuss Scenrio 4 in Alt-2. |
| Mod | Given the situation by far, I don’t think we can agree on supporting any additional scenarios. So, the proposal is reviesed as follows. Updated FL Proposal 2.9-2: On RACH-based transmission on a SpCell, not support additional scenarios triggering RACH-based transmission on SpCell |
| Sony | Support Alt-2. At least, the BFR under Scenario 4 should be supported. |
| CMCC | Support the proposal. |
| LGE | Support Alt-1. And support updated FL proposal. |
| ETRI | Support Alt-1, so support updated FL proposal. |
| Lenovo/MotM | Support the updated FL proposal. |

Updated FL Proposal 2.9-2: On RACH-based transmission on a SpCell, not support additional scenarios triggering RACH-based transmission on SpCell.

- Support: (11) Huawei, HiSilicon, Lenovo/MotM, H3C, vivo, Futurewei, OPPO, MediaTek, CMCC, LGE, ETRI, Lenovo/MotM

- Not support: (3) InterDigital, Xiaomi, Sony

Issue 2.10: CORESET activated with two TCI states

### Round 1

As mentioned by some companies in their contribution, HST-SFN is an important scenario for practical deployment. In addition to legacy SFN scheme based on Rel-15 transmission scheme, enhanced SFN transmission schemes for both PDCCH and PDSCH are introduced in Rel-17. Regarding the BFR for the case of CORESET with 2 activated TCI states, views from company contributions are summarized as follows:

- Support: Lenovo, Motorola Mobility, ZTE, E///, NEC, ITRI, DCM, CATT

- Not support: Nokia, Nokia Shanghai Bell

Based on views of companies that are interested in the enhancement on HST-SFN, the following proposal is listed for discussion.

FL proposal 2.10: TRP-specific BFR for the case of CORESET with 2 TCI states is supported in Rel-17.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | Do not support. The perTRP BFR is based on rel-16 beam management framework, in which no CORESET is configured with 2 TCI states. |
| Apple | According to our understanding, this issue is finalized in 8.1.2.4, where 2 RSs would be selected but how to calculate BLER is up to RAN4. |
| FGI/APT | Support |
| ZTE | Support. Before kicking off this issue to RAN4, we need to make the conclusion of BLER hypothesis and BFD-RS determination firstly. In such case, we prefer to have a single BLER hypothesis while using the two TCI states. |
| NTT DOCOMO | Support the proposal. |
| Lenovo/MotM | Support. And the beam updating of CORESETs activated with 2 TCI states should be further studied after gNB’s confirm of TRP-specific BFR. |
| LGE | We are open for this issue but needs to be handled after settling down TRP-specific BFR and SFN-PDCCH designs. |
| NEC | Support |
| vivo | This issue should be discussed in AI 8.1.2.4. |
| Xiaomi | Support. It has some relationship with Proposal 2.8, for CORESET with two active TCI states, the association between TCI state and BFD-RS set can be configured. |
| InterDigital | Support FL’s proposal. |
| Huawei, HiSilicon | Similar view as OPPO. |
| Samsung | Support FL’s proposal. |
| Futurewei | We shared same view as vivo that this issue should be discussed in AI 8.1.2.4. |
| Qualcomm | Support |
| Fujitsu | Support |
| Sony | Support FL proposal 2.10. |
| NTT DOCOMO | @vivo@Futurewei For per-cell BFR for the case of CORESET with 2 TCI states, we agree it should be discussed in AI 8.1.2.4. But for per-TRP BFR for the case of CORESET with 2 TCI states, we think it should be discussed here. |
| Nokia/NSB | Do not support. When CORESET is indicated with two TCI states, this is SFN PDCCH, which means a PDCCH is associated with two TRP. What does per-TRP BFR mean in this case? We have already agreed that SFN PDCCH cannot be used for M-DCI M-TRP. Agreement (RAN1#105e) Enhanced MAC CE signaling is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP. Please clarify the usecase. |
| Convida | This issue has already been discussed in AI 8.1.2.4. No need to repeat here. |
| Ericsson | Support |
| Lenovo/MotM | Support. |
| Spreadtrum | Not support |
| TCL | We shared same view as DOCOMO/Convida that this issue should be discussed in AI 8.1.2.4. |

Summary of discussion on issue 2.10 over email:

FL proposal 2.10: TRP-specific BFR for the case of CORESET with 2 TCI states is supported in Rel-17.

         Support: DOCOMO, ZTE, Ericsson, Nokia/NSB, Xiaomi, CMCC, NEC, FGI/APT, Lenovo/MotM, InterDigital, Samsung, Qualcomm, Fujitsu, Ericsson

         Not support: OPPO, Apple, vivo, Huawei, HiSilicon, Lenovo/MotM, Spreadtrum, MTK, Convida

| Company | Comments |
| --- | --- |
| OPPO | Do not support the proposal. The reasons are (1) It was clarified that the BM of mTRP in 8.1.2.3 is based on rel15/rel16 BM framework where there is no CORESET with 2 TCI states (2) as pointed out by Nokia in FL summary, a CORESET configured with two TCI states is the PDCCH transmitted by two TRPs. It is problematic to conduct TRP-specific BFR on a PDCCH associated with two TRPs. |
| DOCOMO | Support |
| ZTE | Support. By default, we should reuse Cell-specific BFR rather than TRP -specific BFR , and it is just relevant to how to determinate the BFD -RS in our views. The rest should be the same as Cell-specific BFR |
| Sony | For the TRP-specific BFR with CORESET activated with 2 TCI states, we used to think this referred to SFN BFR that should be handled in AI 8.1.2.4. But checking inputs from companies, we realize that Proposal 2.10 can be interpreted as the BFR for CORESET activated with 2 TCI states from any single TRP. If that’s the case, we feel it might be too late to discuss this use case. If not, can we understand as the BFR under discussion in AI 8.1.2.4? |
| Ericsson | Support |
| vivo | We don’t support this proposal. |
| Apple | We do not think this proposal is needed. Some aspects on BFD over 2 TCI States per CORESET were agreed in HST-SFN agenda in last meeting. |
| Huawei, HiSilicon | Do not support. |
| Nokia/NSB | Support. Initially we didn’t support due to confusion related to Rel-16 M-TRP operation. If this is only for SFN operation, we are fine to support for enhanced SFN operation. @Apple, we didn’t agree per-TRP BFR with SFN. Also, we have agreed following (see note) Agreement (RAN1#105e) If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD · Down-select one alternative for implicit configuration o Alt 1-2: RS of CORESETs with both single and two TCI states are used o Alt 1-3: RS of CORESETs with only two TCI states are used · Down-select one alternative for explicit configuration o Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS § FFS other details o Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration · Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3 |
| Xiaomi | Support. For CORESET with two active TCI states, the association between two TCI states and two BFD-RS sets need to be configured. |
| Lenovo/MotM | Support. |
| Samsung | We support the proposal. |

| LGE | Support. |
| --- | --- |
| Spreadtrum | Not support. |
| CMCC | Support |
| Futurewei | Not support. |
| NEC | Support |
| MediaTek | Not support |

      - Updated FL Proposal 2.10

FL proposal 2.10: TRP-specific BFR for the case of CORESET with 2 TCI states is supported in Rel-17.

         Support (14): DOCOMO, ZTE, Ericsson, Nokia/NSB, Xiaomi, CMCC, NEC, FGI/APT, Lenovo/MotM, InterDigital, Samsung, Qualcomm, Fujitsu, Ericsson

        Not support (8): OPPO, Apple, vivo, Huawei, HiSilicon, Lenovo/MotM, Spreadtrum, MTK, Convida

### Round 2

FL proposal 2.10-2: TRP-specific BFR for the case of CORESET with 2 TCI states is supported in Rel-17.

- Support: Lenovo/MotM, H3C, InterDigital, Nokia/NSB, Xiaomi, CMCC, LGE

- Not support: Convida, ZTE, vivo, Futurewei, OPPO, MediaTek, Sony

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Lenovo/MotM | Support it. And for the beam updating of CORESETs activated with two TCI states, an association between one TCI state of two activated TCI states and one BFD-RS set of two BFD-RS sets should be build further. |
| H3C | Support FL proposal. |
| Convida | Given the many companies not supporting, we suggest to conclude “not supported in Rel-17”. If one of the two TRPs fails, the SFN-PDCCH still works, so a premature BFR doesn’t seem necessary. |
| ZTE | Not support. How to support SDCI-mTRP is still pending, and so we suggest not to discuss this proposal in such situation. |
| InterDigital | Support FL proposal |
| Nokia/NSB | Support FL proposal. |
| Vivo | Not support. We do not see any necessity to do TRP-specific BFR for SFN scenario. |
| Futurewei | Not support. |
| OPPO | Do not support. CORESET with 2 TCI state is PDCCH transmitted from both TRP. How can we do TRP-specific BFR on such PDCCH? |
| NTT DOCOMO | Okay to not discuss it further. Support to discuss sDCI MTRP before this. |
| MediaTek | No support |
| Xiaomi | Suppor the FL proposal. But we also have concern on the remaining time for discussion. Since details about association between two TCI states and two BFD-RS sets need to be configured. |
| Sony | Not support. For CORESET with two active TCI states (transmitted from two TRPs), we don’t even know how a UE would calculate the BLER, i.e. BFD, only taking into the account from any single TRP. I tend to believe such discussion will trigger heavy discussion in the last meeting. In addition, the TRP-specific BFR can cover the case that when any of TRP fails, we don’t have to duplicate the BFR design. |
| CMCC | Support the proposal, but we worry that there is no time to handle all the details of this feature. |
| LGE | Support. |

Conclusion: TRP-specific BFR for the case of CORESET with 2 TCI states is not supported in Rel-17.

Issue 2.11: Number of activated spatial filters for PUCCH-SR resource

### Round 1

Views from company contributions on issue 3.11 are summarized as follows:

Whether PUCCH-SR resource can have 1 or 2 activated spatial filters

- Single: Spreadtrum

- One or two: vivo, APT, ETRI, CMCC, Xiaomi, Sony

Based on views of majority companies, the following proposal is suggested.

Proposed Conclusion: There is no further restriction on the TCI/spatial relation configuration of a PUCCH-SR resource.

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | It is about the design of PUCCH resource. We do not iscuss it here. |
| Apple | OK with the conclusion. |
| FGI/APT | Support the conclusion |
| ZTE | Support |
| NTT DOCOMO | OK with the conclusion. |
| LGE | Fine. |
| NEC | Fine |
| vivo | Support the conclusion. |
| Xiaomi | Fine with the conclusion |
| InterDigital | Support FL’s conclusion. |
| Huawei, HiSilcon | Support |
| Futurewei | Support FL’s conclusion. |
| Qualcomm | Fine |
| Fujitsu | Support FL’s conclusion. |
| Sony | Support. |
| MediaTek | OK to the conclusion |
| Nokia/NSB | Support the conclusion. |
| CMCC | Support. |
| Convida | Didn’t we already bring this offline conclusion to the online session in the previous meeting, and the chairman thought it wasn’t needed? Maybe my memory fails me? |
| Mod | @Convida: Yes, you’re right. However, as shown in contributins, some companies still think it’s necessary to conclude this. |
| Ericsson | Ok |
| Lenovo/MotM | Ok |
| Spreadtrum | We have one question for clarification: Does the conclusion mean that gNB can configure any number of spatial information for one PUCCH resource? In our understanding, since BM for m-TRP in Rel-17 is based on R15/R16 BM framework, only up to 1 spatial information can be configured for one PUCCH resource for BFR for m-TRP. Maybe I missed something. If so, please point it out and correct me. Thanks! |
| TCL | Support FL’s proposal |

Summary of discussion on issue 2.11 over email:

Proposed Conclusion: There is no further restriction on the TCI/spatial relation configuration of a PUCCH-SR resource.

           Concerned by:

| Company | Comments |
| --- | --- |
| OPPO | we support |
| DOCOMO | Support |
| ZTE | Support |
| Sony | Support |
| Ericsson | Support |
| vivo | Fine with Apple’s rewording. |
| Apple | We suggest the following revision. Currently unified TCI is not supported for R17 mTRP. Proposed Conclusion: There is no further restriction on the TCI /spatial relation configuration of a PUCCH-SR resource. |
| Huawei, HiSilicon | Support |
| Nokia/NSB | Support |
| Xiaomi | Support |
| Lenovo/MotM | Support |

| Samsung | Fine with the conclusion. |
| --- | --- |
| LGE | Support. |
| Spreadtrum | We have one question for clarification: Does the conclusion mean that gNB can configure any number of spatial information for one PUCCH resource? In our understanding, since BM for m-TRP in Rel-17 is based on R15/R16 BM framework, only up to 1 spatial information can be configured for one PUCCH resource for BFR for m-TRP. |
| Mod | @Spreadtrum: thanks for your question. Based on discussion in the last meeting, most of companies thought that a PUCCH-SR resource can have 2 spatial relations, just like other PUCCH resources in Rel-17 |
| CMCC | Support |
| Futurewei | Support. |
| NEC | Support |
| MediaTek | Okay |

      - Updated FL Proposal 2.11

Proposed Conclusion: There is no further restriction on the spatial relation configuration of a PUCCH-SR resource.

      - Conclusion of round 1 discussion

Conclusion

For per-TRP BFR, no further restriction will be introduced on the spatial relation configuration of a PUCCH-SR resource.

Other issues of BFR

### Round 1

In addition to the above issues, the issues listed in the following table are also raised by some companies.

| Issues | Companies | Views |
| --- | --- | --- |
| # NBI-RS set | Convida | The number of NBI-RS in the first and second NBI-RS set is N1 and N2, respectively, and the maximum value of N1+N2 is 64 (same value as Rel-16). The candidate beams in the first set are indexed from 0 to N1-1 and the candidate beams in the second set are indexed from N1 to N1+N2-1. |
|  | Vivo | - Support to optionally configure TRP-specific NBI-RS for multi-DCI-based MTRP (UE should be able to freely select new candidate beams of other TRPs). - It is not necessary to configure cell-specific NBI and TRP-specific NBI simultaneously. |
| # PUCCH-SR | DCM | Two dedicated PUCCH-SR resources can be configured, only when SpCell is configured with per-TRP BFR. |
|  | QC (mapped to the same schedulingRequestId), Nokia, DCM | The number of SR configurations is one. |
|  | Vivo | Clarify whether the two PUCCH-SR share the same set of higher layer parameters (sr-ProhibitTimer, sr-TransMax and SR_COUNTER) if two PUCCH-SR resources are under the same SR configurations. |
| # BFRQ | DCM | Include beam failure information of CCs configured with per-TRP BFR, as well as CCs configured with per-cell BFR. |
|  | Intel, ZTE | Support CFRA |
|  | vivo | The failure events of different BFD-RS sets are declared within a predefined time window, the failure event of two TRPs failing simultaneously would be declared. |
|  | Xiaomi | RACH-based BFRQ will be triggered if the beam failure of the second TRP is detected before receiving the response from gNB during the BFRQ procedure of the first TRP. |
| # If there is no new beam found | Nokia | - UE is not required to monitor PDCCH on the CORESET(s) associated with failed BFD-RS set and falls back to single TRP operation. - In case both TRPs fail but no new candidate beam can be indicated for the TRPs, the UE transmits cell-specific BFR (i.e. fallback to rel15/16 BFR/legacy BFR MAC CE) |
|  | vivo, ZTE | Support to deactivate TRP through MAC CE if there is no new beam found. |
| # Inter-cell BFR | DCM | - For per-cell BFR, SSB associated with additional PCI can be configured as BFD-RS explicitly/implicitly. - For per-TRP BFR, SSB associated with additional PCI can be configured as BFD-RS in the BFD-RS set associated with corresponding CORESETPoolIndex. |
|  | Apple | Support to configure non-serving cell SSB for candidate beam detection. |
|  | E/// | Support per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation. |
| # Threshold | QC, ZTE | The threshold Qout,LR for TRP specific BFD can reuse the existing default value of rlmInSyncOutOfSyncThreshold for cell level BFD. |
|  | Apple | Support to configure cell-specific candidate beam detection RSRP threshold. |
| # S-DCI | Intel, DCM | For single-DCI case also support association between a TRP (e.g., BFD-RS set) on a SpCell and a PUCCH-SR resource on the same SpCell. |
| # Beam update | ZTE | The QCL assumption of all CORESETs associated with a same CORESETPoolIndex in a CC list should be updated: ZTE |
| Others | QC | # In presence of PDCCH repetition, the 28 symbols start from the last repetition. |
|  | Vivo | # Support configure TRP-specific RRC parameters, e.g., beamFailureInstanceMaxCount-1, beamFailureInstanceMaxCount-2, and beamFailureDetectionTimer-1, beamFailureDetectionTimer-2. |
|  | Lenovo, Motorola Mobility | # The priority of positive LRR of TRP-specific BFR is higher than positive SR. |
|  | Lenovo, Motorola Mobility | # Further study the priority of positive LRR of TRP-specific BFR and positive LRR of Scell BFR considering whether Pcell/PSCell is configured with TRP-specific BFR. |

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Apple | We would like to clarify whether TRP-specific BFR is applicable to inter-cell mTRP. If the answer is yes, we need to discuss whether neighbor cell SSB can be configured for BFD/CBD. This has RRC impact, and we think it should be prioritized. |
| NTT DOCOMO | We suggest prioritizing following issues: - PUCCH-SR (one schedulingRequestId with two PUCCH resources) - Inter-cell BFR |
| LGE | Regarding BFRQ, as vivo and Xiaomi suggested, the condition of both TRP failure can be discussed for RACH-based BFRQ in SpCell. We think vivo’s proposal is simple way forward. |
| NEC | We also support to discuss inter-cell BFR. |
| Vivo | We would like to clarify the condition of simultaneous beam failure on both BFD-RS sets. According to the previous agreement, that RACH-based transmission will be triggered when beam failure is detected on all BFD-RS sets on the SpCell, it has been supported at least for SpCell that beam failure detection for both TRPs is based on the two TRP-specific BFD-RS set, rather than configure one extra cell-specific BFD-RS set. For Scell, we think it can reuse the same mechanism. However, there is ambiguity regarding how to determine both BFD-RS sets are in beam failure. Rarely in practical scenarios would the two TRPs fail exactly at the same moment. It is highly possible that even when the radio link condition of two TRPs is not suitable for transmission anymore, the failure event detection would happen sequentially within a short period. Thus it is more reasonable to define a time window to determine whether two TRPs fail simultaneously. |
| Xiaomi | We suggest to define the condition of both TRP failure for triggering RACH-based BFRQ in SpCell. |
| NTT DOCOMO | @vivo @Xiaomi This is RAN2 issue, and RAN2#116-e has made following agreement. There is no need to discuss this issue in RAN1. - The meaning of “beam failure is detected on both TRPs” is to be clarified, It is FFS which of the following options shall be applied: Option 1 (12/17): “beam failure is detected on both TRPs” means that BFR is triggered for a TRP of the serving cell while the BFR for another TRP of same serving cell is still pending (i.e. not cancelled). Option 2 (4/17): “beam failure is detected on both TRPs” means that BFR is triggered for a TRP of the serving cell while the BFR for another TRP of same serving cell is still pending (i.e. not successfully completed) But for PUCCH-SR (one schedulingRequestId with two PUCCH resources) issue, RAN2 is still waiting for RAN1’s decision, based on lastest RAN2#116-e discussion progress. |
| Mod | According to suggestion from DOCOMO, the following draft proposal can be discussed if companies are interested in this issue. FL Proposal 2.12: Two dedicated PUCCH-SR resources can be configured, only when SpCell is configured with per-TRP BFR.. |
| vivo | Thanks for the clarification from NTT DOCOMO. As for the Proposal 2.12, condition that “only when SpCell is configured with per-TRP BFR” is much restricted. In contrast, we think if at least one CC is configured with per-TRP BFR, two dedicated PUCCH-SR resources should be configured. Therefore, we revise the proposal as follows: FL Proposal 2.12: Two dedicated PUCCH-SR resources can be configured, only when at least one CC SpCell is configured with per-TRP BFR. |
| Ericsson | Wc support to extend per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation. The other issues are low priority for us. |
| NTT DOCOMO | Sorry for misunderstanding. I mean, RAN2 is still waiting for RAN1’s decision on whether the two dedicated PUCCH-SR resources are corresponding to one schedulingRequestId or two schedulingRequestId. RAN1 needs to make a down-selection from following Alt1 and Alt2. Even if RAN1 decides to leave it to RAN2, we need a clear agreement. Because currently RAN2 thinks they should wait for RAN1’s decision. On the association between two dedicated PUCCH-SR resources for per-TRP BFR and schedulingRequestId, - Alt1: Two dedicated PUCCH-SR resources are associated with one schedulingRequestId - Alt2: Two dedicated PUCCH-SR resources are associated with two separate schedulingRequestId - [Alt3: leave it to RAN2] |
| Lenovo/MotM | Support the FL proposal 2.12. |

### Round 2

FL Proposal 2.12-2: Regarding whether the two dedicated PUCCH-SR resources are corresponding to one schedulingRequestId or two schedulingRequestId

- Alt1: Two dedicated PUCCH-SR resources are associated with one schedulingRequestId

- Alt2: Two dedicated PUCCH-SR resources are associated with two separate schedulingRequestId

- Alt3: Leave it to RAN2

Companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| Lenovo/MotM | Support Alt2. |
| ZTE | No more discussion is needed, and it seems that RAN2 is starting to handle corresponding RRC/MAC-CE procedure. |
| Nokia/NSB | This is only RAN2 issue. No discussion is required. |
| Vivo | Support Alt3. |
| Futurewei | Support Alt3. |
| OPPO | Support Alt2. But we are ok to leave it to RAN2. |
| NTT DOCOMO | RAN1 should give a clear message to RAN2. If you check the lastest RAN2#116-e meeting discussion, RAN2 discussed this issue as Q16. Q16. Which options do you prefer for SR configuration for TRP specific BFR? - Option 1: Up to two SR configurations are configured for TRP-BFR within the same cell group, each SR configuration is associated with a PUCCH resource - Option 2: One SR configuration associated with up to two PUCCH resources. But, the final observation is to wait for RAN1 progress, as shown below. Summary: Majority view (12/17) is to wait for RAN1 progress. So no proposal is made for this issue. Hence, RAN1 should either make a decision between Alt1 and Alt2, or make a clear conclusion to leave it to RAN2. Otherwise, this issue will be continue pending like this. From the 3 alts, we support Alt1, and we can also accept Alt3. But we donot support Alt2. The SR configuration is limited for a UE (up to 8). It is not desired to occupy 2 SRs for the same BFR purpose. In addition, even though the SR for BFR can be shared with other logic channel, in gNB implementation, with 1 SR for Rel17 per-TRP BFR, it is still possible and it has higher probaibility for gNB to config a dedicated SR for BFR when the SR config is sufficient for this UE, which is beneficial for NW to understand the real purpose of this SR. But if two SRs are configured for BFR, it becomes more difficult for gNB to config dedicated two SR for BFR purpose only. |
| Mod | We tend to agree with DOCOMO. |
| Sony | We sympathize the rationale for discussing the PUCCH-SR resource and SR configuration association. We support Alt1 when considering SR configuration is more scarce when compared with PUCCH SR resources which in essence is PUCCH resource. And if RAN1 cannot decide, we are also fine to leave it to RAN2, though RAN2 may expect RAN1 to have a clear association approach. |
| LGE | Support Alt-3. |

Summary of email discussion

FL Proposal 2.12-2: Regarding whether the two dedicated PUCCH-SR resources are corresponding to one schedulingRequestId or two schedulingRequestId

         Alt1: Two dedicated PUCCH-SR resources are associated with one schedulingRequestId

         Alt2: Two dedicated PUCCH-SR resources are associated with two separate schedulingRequestId

         Alt3: Leave it to RAN2

| Company | Comments |
| --- | --- |
| Sony | We sympathize the rationale for discussing the PUCCH-SR resource and SR configuration association. We support Alt1 when considering SR configuration is more scarce when compared with PUCCH SR resources which in essence is PUCCH resource. And if RAN1 cannot reach a decision, we are also fine to leave it to RAN2, though RAN2 may expect RAN1 to have a clear association approach. |
| Apple | Maybe we do not need to discuss this issue. It was agreed it is two SR configuration. If we want to make it clear, we would like to support Alt2. |
| DOCOMO | We support Alt1. And we can also accept Alt3. We do not agree with Apple that two SR configuration was agreed. We do not have such agreement on this issue. And latest RAN2 meeting had an observation to wait for RAN1 progress. |
| ZTE | We do not identify the necessity of this proposal. It can be left to RAN2 by default. |
| LGE | We support Alt3, i.e., it is up to RAN2 decision. |
| MediaTek | Support Alt3 |
| OPPO | We do not see the necessarity of this proposal too. This shall be discussed and decided by RAN2. |
| DOCOMO2 | Our concern is ‘it is not left to RAN2 by default’. RAN1 should make a clear agreement/conclusion, even if RAN1 finally decide to leave it to RAN2. We invite companies to check the last RAN2 discussion in Nov. And we copy the RAN2 discussion and observation below. RAN2 discussed this issue as Q16. Q16. Which options do you prefer for SR configuration for TRP specific BFR?  Option 1: Up to two SR configurations are configured for TRP-BFR within the same cell group, each SR configuration is associated with a PUCCH resource  Option 2: One SR configuration associated with up to two PUCCH resources. But, the final observation is to wait for RAN1 progress, as shown below. Summary: Majority view (12/17) is to wait for RAN1 progress. So no proposal is made for this issue. Hence, RAN1 should either make a decision between Alt1 and Alt2, or make a clear conclusion to leave it to RAN2. Otherwise, this issue will be continue pending like this. |
| Spreadtrum | Alt3 |
| Ericsson | Alt3. This should be left to RAN2 |
| Convida | Alt1. In our understanding, all agreements until RAN1#106bis are about two PUCCH-SR resources, and we (RAN1) discussed at length PUCCH-SR resource selection rules (not selection of SR configuration!). We think this should be understood as Alt1, i.e. that the single BFR SR is triggered and that [the physical layer] selects a PUCCH resource. In RAN1#106bis and in this meeting, we have unfortunately confused the issue by adding “/ SR configuration” as a way to make progress. However, we think we should respect the previous RAN1 agreements and discussions about two PUCCH-SR resources and PUCCH-SR resource selection (not selection of SR configuration!). |
| Qualcomm | Prefer Alt1 to save SR ID |
| Lenovo/MotM | Prefer Alt 2 and can also accept Alt3. |
| Xiaomi | Prefer Alt 1 |
| Intel | Alt1 |
| CMCC | Support Alt1. Fine with Alt3 |
| Futurewei | Prefer Alt3 |
| Nokia//NSB | Alt 3. No discussion in RAN1. |

FL Proposal 2.12-2: Regarding whether the two dedicated PUCCH-SR resources are corresponding to one schedulingRequestId or two schedulingRequestId

         Alt1: Two dedicated PUCCH-SR resources are associated with one schedulingRequestId

         Alt2: Two dedicated PUCCH-SR resources are associated with two separate schedulingRequestId

         Alt3: Leave it to RAN2

- Alt 1: Sony(1st), DOCOMO(1st) , Convida, Qualcomm, Xiaomi, Intel, CMCC(1st)

- Alt 2: Apple(2nd), Lenovo/MotM(1st)

- Alt 3: Sony(2nd), Apple(1st) , DOCOMO(2nd), ZTE, LGE, MediaTek, OPPO, Spreadtrum, Ericsson, Lenovo/MotM(2nd), CMCC(2nd), Futurewei, Nokia//NSB

Simultaneous reception of signals with different QCL-typeD assumptions

Issue 3.1: Simultaneous reception of signals with different QCL-typeD assumptions

As shown below, in #102e meeting, it’s agreed the study of simultaneous reception of signals with different QCL-type D assuptions is with low priority.

Agreement

Study Rel.17 enhancements on beam management for multi-TRPs with following priority

- High priority:

  - Beam measurement/reporting enhancement

  - Beam failure recovery for multi-TRP

- Low priority

  - Simultaneous reception of same type of channel/RS with different QCL-TypeD

  - Simultaneous reception of different type of channel/RS with different QCL-TypeD

Furthermore, the following statement was captured in the Chairman note of #102e meeting.

In RAN1#102-e, the following combinations of physical channels were discussed but there was no consensus among the companies whether or not to study further in future meetings

Study simultaneous reception of the same type of channels/RS with different QCL-TypeD assumption, including at least the following combinations:

- PDCCH+PDCCH, CSI-RS + CSI-RS

Study simultaneous reception of different types of channels with different QCL-TypeD assumptions, including at least the following combinations:

- SSB+PDCCH/PDSCH,  PDCCH+PDSCH, PDCCH+CSI-RS, PDSCH+CSI-RS

Other combinations of channels/RS are not precluded.

In the contribution of this meeting, the following companies show their interests in this issue: QC, DCM, Spreadtrum, Lenovo, Motorola Mobility, Apple, Sony.

Regarding this issue, companies are invited to provide their preferences and comments in the table below.

| Company | Comments |
| --- | --- |
| OPPO | We support to dicuss the simultaneous reception of channels with different QCL-TypeD. Please note per TRP BFR for sDCI is also low priority. It does not make sense to not discuss this simultaneous reception but discuss sDCI considering both of them are low priority. Agreement - Support S-DCI and M-DCI in TRP-specific BFR in Rel.17 - S-DCI is low priority, M-DCI is high priority - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization |
| Apple | In our view, at least we can discuss some new combinations from inter-cell mTRP and BM, e.g. SSB from cell 1 + DL signals from cell 2. PDCCH+PDCCH is finished in 8.1.2.1. |
| ZTE | Postpone the discussion to Rel-18. |
| NTT DOCOMO | Support to discuss above combinations. We can prioritize some combinations for discussion, e.g., PDCCH+PDCCH, CSI-RS+CSI-RS, PDCCH+PDSCH, SSB+PDCCH/PDSCH. |
| Lenovo/MotM | Support at least to discuss simultaneous reception of the same type of channels/RS with different QCL-TypeD if time is available. If there is not enough time, it can be postponed to Rel-18. |
| vivo | Support to postpone the discussion. |
| InterDigital | Not enough time to discuss this in Rel-17, wait for Rel-18. |
| MediaTek | Prefer to postpone |
| Mod | Companies that are interested in simultaneous reception can provide their views/proposals. This issue can be discussed if there is time left; otherwise we can only postpone it. |

Previous agreements

RAN1#102-e

Agreement

For L1-RSRP, consider measurement / reporting enhancement to facilitate inter-TRP beam pairing

- Option-1: Group-based reporting,

  - e.g., beam restriction to facilitate inter-TRP pairing.

- Option-2: Non-group-based reporting

Agreement

Evaluate and study at least but not limited to the following issues for multi-beam enhancement

- Issue 1: Consideration of inter-beam interference

- Issue 2: For group-based reporting, increased number of groups and/or beams per group

- Issue 3: UE Rx panel related beam measurement/report

  - NOTE: “UE panel” is used for discussion purpose only

Agreement

- Evaluate enhancement to enable per-TRP based beam failure recovery starting with Rel-15/16 BFR as the baseline.

- Consider following potential enhancement aspects to enable per-TRP based beam failure recovery

  - Issue 1: TRP-specific BFD

  - Issue 2: TRP-specific new candidate beam identification

  - Issue 3: TRP-specific BFRQ

  - Issue 4: gNB response enhancement

  - Issue 5: UE behavior on QCL/spatial relation assumption/UL power control for DL and UL channels/RSs after receiving gNB response

Agreement

Study Rel.17 enhancements on beam management for multi-TRPs with following priority

- High priority:

  - Beam measurement/reporting enhancement

  - Beam failure recovery for multi-TRP

- Low priority

  - Simultaneous reception of same type of channel/RS with different QCL-TypeD

  - Simultaneous reception of different type of channel/RS with different QCL-TypeD

RAN1#103-e

Agreement

Down-select at least one of the following options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing in RAN1 #104-e

- Option 1: In a CSI-report, UE can report N>1 pair/groups and M>=1 beams per pair/group

  - Different beams in different pairs/groups can be received simultaneously

  - FFS: whether M is equal or can be different across different pair/group

- Option 2: In a CSI-report, UE can report N(N>=1) pairs/groups and M (M>1) beams per pair/group

  - Different beams within a pair/group can be received simultaneously

- Option 3: UE report M(M>=1) beams in N (N>1) CSI-reports corresponding to N report setting

  - Different beams in different CSI-reports can be received simultaneously

  - FFS: whether/how to introduce an association between different CSI-reports

  - FFS: whether/how to differentiate reported measurements for beams that are received simultaneously vs. beams that are not received simultaneously

    - whether/how to introduce an indication along with the CSI-reports to indicate whether the beams in different CSI-reports can be received simultaneously

- FFS: value of N and M in each option

- FFS: Association between different beams in above options and different TRP/UE panels

- FFS: Identify new use cases per option compared with R16 (including backhaul)

- FFS: whether different beams in different pairs/groups/reports can be received by same spatial filter per option

Agreement

- For M-TRP beam failure detection, support independent BFD-RS configuration per-TRP, where each TRP is associated with a BFD-RS set.

  - FFS: The number of BFD RSs per BFD-RS set, the number of BFD-RS sets, and number of BFD RSs across all BFD-RS sets per DL BWP

  - Support at least one of explicit and implicit BFD-RS configuration

    - With explicit BFD-RS configuration, each BFD-RS set is explicitly configured

      - FFS: Further study QCL relationship between BFD-RS and CORESET

    - FFS: How to determine implicit BFD-RS configuration, if supported

- For M-TRP new beam identification

  - Support independent configuration of new beam identification RS (NBI-RS) set per TRP if NBI-RS set per TRP is configured

    - FFS: detail on association of BFD-RS and NBI-RS

    - Support the same new beam identification and configuration criteria as Rel.16, including  L1-RSRP, threshold

Agreement

- Support TRP-specific BFD counter and timer in the MAC procedure

  - The term TRP is used only for the purposes of discussions in RAN1 and whether/how to capture this is FFS

Agreement

- Support a BFRQ framework based on Rel.16 SCell BFR BFRQ

  - In RAN1#104-e, select one from the following options

    - Option 1: Up to one dedicated PUCCH-SR resource in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: number of spatial filters associated with the PUCCH-SR resources

      - FFS: How the SR configuration is done

    - Option 2:  Up to two (or more) dedicated PUCCH-SR resources in a cell group

      - A cell group refers to either MCG, SCG, or PUCCH cell group

      - FFS: whether each PUCCH-SR resource is restricted to be associated to one spatial filter

      - FFS: How the SR configuration is done

  - FFS: Whether no dedicated PUCCH-SR resource can be supported in addition to Option 1 or Option 2

- Study whether and how to provide the following information in BFRQ MAC-CE

  - Index information of failed TRP(s)

  - CC index (if applicable)

  - New candidate beam index (if found)

  - Indication whether new beam(s) is found

  - FFS: whether/how to incorporate multi-TRP failure

RAN1#104-e

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

Agreement

- For M-TRP BFR Support 1-to-1 association between each BFD-RS set and an NBI-RS set

  - FFS: Association details

Agreement

For M-TRP BFR

- Support 2 BFD-RS sets per BWP, and up to N resources per BFD-RS set

  - FFS: value of N (e.g. fixed in specification, or UE capability)

- FFS: number of BFD RSs across all BFD-RS sets per DL BWP (e.g. fixed maximum value or UE capability)

Agreement

For BFRQ of M-TRP BFR

- Option 3: Up to two dedicated PUCCH-SR resources in a cell group

- FFS: Whether PUCCH-SR for SCell can be reused for M-TRP

- Support BFRQ MAC-CE that can convey information of failed CC indices, one new candidate beam for the failed TRP/CC (if found), and whether new candidate beam is found

  - Support at least indication of a single TRP failure

    - FFS: whether/what information of failed TRP(s) is conveyed in the MAC-CE

    - FFS: whether/how to support  indication of more than one TRP failure, corresponding BFR procedure, and applicable cell type (SCell vs. SpCell)

- FFS: UE behavior when TRP failure status is different across cells

- FFS: Whether PUCCH SR resource can be configured with 2 spatial relations

RAN1#104b-e

Agreement

For beam reporting option 2

- On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e:

  - Alt1: Support maximum value N = {1, 2}

  - Alt2: Support maximum value N = {1, 2, 3, 4}

- FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17

- On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e

  - Alt1: The value of N is fixed by RRC configuration

  - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

Agreement

On CMR resource configuration for beam reporting option 2, adopt the following alternative:

- Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

  - FFS: extension to aperiodic CMR resource setting

- Each reported beam pair in a single CSI-report consists of M = 2 SSBRI / CRI values, where each SSB-RI / CRI points to a CMR resource in a different CMR resource set or subset.

- Decide in RAN1#104b-e whether to adopt “set” or “subset” in the above.

Agreement

- Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs.

- FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Agreement

On BFD-RS of TRP-specific BFR

- BFD-RS resource number:

  - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability

  - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e

    - Alt1: max value is 2

    - Alt2: max value is a UE capability, including possible candidate value of 1

Agreement

Adopt the following beam failure detection criteria for each BFD-RS set

- The physical layer in the UE assesses the radio link quality per BFD-RS set and indicates the BFD-RS set index to higher layers every X ms, if the hypothetical PDCCH BLER of all BFD-RS in the corresponding set of BFD-RS is higher than a threshold

  - X is max{minimal periodicity of BFD RS in the set, 2ms}

Agreement

A UE configured with TRP-specific BFR can be configured with 1 PUCCH-SR resource in a cell group

- NOTE: it has been agreed in RAN1#104-e that a UE can be configured with up to 2 PUCCH-SR resources in a cell group

Agreement

For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs,

- Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e

  - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

  - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

  - Alt-3: Leave it up to UE implementation

- Note: PUCCH-SR resource is PUCCH resource carrying SR

- FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

Agreement

On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”:

- NOTE: the following has been agreed

  - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting

    - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported

  - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset.

- FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets

RAN1#105-e

Agreement

For CMR configuration for option 2, adopt

- Alt-1: “set”

Agreement

The bitwidth of each SSBRI/CRI is determined based on the number of SSB/CSI-RS resources in the associated CMR resource set

- FFS: specify the association between SSBRIs/CRIs in a reported group and CMR resource sets

Agreement

- For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17.

  - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how

- The number of beam groups (N) reported in a single CSI-report

  - Alt1: The value of N is configured by RRC signalling

Agreement

Select one of the following alternatives with possible modification in RAN1#106-e

- Alt 2.5.2 A:

  - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 B:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 C:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 D:

  - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

RAN1#106-e

Agreement

For aperiodic report of beam reporting option 2,

- When associated with aperiodic resource setting, extend the existing RRC parameter CSI-AssociatedReportConfigInfo to be configured with two CMR resource sets where each may be configured with their corresponding QCL information.

  - FFS: Detailed association scheme

- When associated with periodic/semi-persist resource setting, the resource setting comprises two CMR resource sets.

Conclusion

There is no consensus to support M>2 beams per group for beam reporting option 2 in Rel.17.

Agreement

Support differential L1 RSRP reporting as a UCI reduction scheme for beam measurement/reporting option 2.

Agreement

Differential reporting across all beam groups in a CSI-report

- Including 1-bit indicator of the CMR set associated with the largest RSRP value in all groups

  - NOTE: best beam is assumed in the 1st group

  - 1-bit indicating CMR set with higher RSRP value (e.g. 0 indicating 1st SSBRI/CRI from 1st CMR set, 1 indicating 1st SSBRI/CRI from 2nd CMR set); UCI payload partitioning = 7/4 bits for 1st/2nd SSBRI/CRI in first beam group; 4 bits for all beams in other groups;

Agreement

For multi-TRP BFR, a single MAC-CE is used at least for BFRQ for all TRPs in all CCs in a cell group, which includes

- Indices of failed BFD-RS set (as an indication of failed TRP link)

- Indices of CC containing the failed TRP link

- An indicator whether a new candidate beam is identified in the NBI-RS set associated with the failed BFD-RS set, and an resource indicator representing the new candidate beam (if identified) based on the number of NBI-RS resources in the corresponding NBI-RS set.

- FFS: Content of MAC-CE related to SpCell when transmitted on msg3, msgA

- Note: MAC-CE signaling design details are up to RAN2

- The term “failed TRP link” is used here for discussion purposes only

Agreement

The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17.

Agreement

Support the following BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET:

- Implicit configuration:

  - M-DCI:

    - BFD-RS set k (k = 0, 1) is derived based on X TCI of CORESETs with CORESETPoolIndex = k

    - FFS: value of X (determined in spec or UE capability), and TCI selection rule when the number of CORESETs with CORESETPoolIndex = k exceeds X (e.g. reuse RLM RS selection rule)

- FFS: CORESETs with more than 1 activated TCI states

Possible Agreement

Support the following BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET:

- Explicit configuration: RRC configuration BFD-RS resources in BFD-RS set k, k = 0, 1

  - With reference to how UE selects the BFD-RS, it is the same as in Rel-15

  - FFS: CORESETs with more than 1 activated TCI states.

Conclusion

BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET via implicit configuration for S-DCI mTRP is not supported in Rel-17.

RAN1#106b-e

Agreement

Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR.

- FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR

A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

Agreement

RACH-based transmission can be triggered on a SpCell at least in the following scenarios

- Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell

- FFS: other scenarios

  - Scenario 2: at least one TRP fails on SpCell

  - Scenario 3: at least one pre-defined TRP fails on SpCell

  - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available

  - Scenario 5: If MAC-CE based reporting does not work (details FFS)

  - Scenario 6: When no PUCCH-SR is configured

Agreement

To associate BFD-RS set k and NBI-RS set j

- Alt-1: 1-to-1, fixed in spec

- Whether NBI-RS configuration is mandatory is separate discussion

Conclusion

Design of MAC-CE related to SpCell when transmitted on msg3, msgA is up to RAN2.

Agreement

For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs  associated with CORESETPoolIndex  k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR

- The above applies to Scell and SpCell

- The above applies for the multi-DCI case

Agreement

SCS of the 28 symbols is the smallest SCS of the active DL BWP for the response reception CC and of the active DL BWP (s) of the CC(s) with the failed TRP link(s) reported in BFR MAC CE.

R1-2110576	Summary of enhancements on beam management for multi-TRP (Round 3)	Moderator (CATT)

Agreement

For RACH-based transmission, at least when all BFD-RS sets fail in SPCell, CBRA is supported

Reference

- R1-2110764	Further Discussion on Beam Management Enhancements for Multi-TRP  InterDigital, Inc.

- R1-2110784	Enhancements on beam management for multi-TRP in Rel-17	Huawei, HiSilicon

- R1-2110881	Beam management for simultaneous multi-TRP transmission with multi-panel reception	FUTUREWEI

- R1-2110935	Enhancements on beam management for multi-TRP	Lenovo, Motorola Mobility

- R1-2110951	Enhancements on beam management for Multi-TRP	ZTE

- R1-2110993	Remaining issues on MTRP multibeam enhancement	vivo

- R1-2111087	Discussion on enhancements on beam management for multi-TRP	Spreadtrum Communications

- R1-2111145	Enhancements on beam management for multi-TRP	Fujitsu

- R1-2111224	Remaining issues on beam management for multi-TRP	CATT

- R1-2111282	Enhancements on beam management for multi-TRP	OPPO

- R1-2111382	Enhancements on beam management for multi-TRP	Sony

- R1-2111456	Enhancements on beam management for multi-TRP	LG Electronics

- R1-2111479	Multi-TRP enhancements for beam management	Intel Corporation

- R1-2111543	Enhancements on beam management for Multi-TRP	Xiaomi

- R1-2111600	Enhancements on beam management for multi-TRP	CMCC

- R1-2111686	Discussion on beam management for multi-TRP	NEC

- R1-2111720	Enhancements on beam management for multi-TRP	Samsung

- R1-2111856	Views on Rel-17 multi-TRP BM enhancement	Apple

- R1-2111986	Enhancements on beam management for multi-TRP	ETRI

- R1-2112027	On Multi-TRP BFR	Convida Wireless

- R1-2112077	Discussion of enhancements on beam management for multi-TRP	FGI, Asia Pacific Telecom

- R1-2112092	Discussion on beam management for MTRP	NTT DOCOMO, INC.

- R1-2112171	Discussion on beam management for multi-TRP  ITRI

- R1-2112179	Enhancements on Beam Management for Multi-TRP/Panel Transmission  Nokia, Nokia Shanghai Bell

- R1-2112199	Enhancements on beam management for multi-TRP	Qualcomm Incorporated

- R1-2112274	Enhancements on beam management for multi-TRP	TCL Communication Ltd.

- R1-2112278	Enhancement on beam management for multi-TRP	MediaTek Inc.

- R1-2112321	Remaining issues on beam management for multi-TRP	Ericsson