3GPP TSG RAN WG1 Meeting #107-e	  R1-2112728

E-meeting, November 11–19, 2021

Agenda Item:	8.3.4

Source:	Moderator (Huawei)

Title:	Feature lead summary#3 on propagation delay compensation enhancements

Document for:	Discussion and Decision

Introduction

The revised IIoT / URLLC work item description for Rel-17 [1] has enhancements for time synchronization as one of its main objectives:

- Enhancements for support of time synchronization: - RAN impacts of SA2 work on uplink time synchronization for TSN, if any. [RAN2] - Propagation delay compensation enhancements (including mobility issues, if any). [RAN2, RAN1, RAN3, RAN4]

This document summarizes the key issues discussed under agenda item 8.3.4 based on the views in [3][4][5][6][7][8][9][10][11][12][13], and aims to discuss a set of issues in RAN1#107-e. The agreements in past meetings are captured in the Appendix.

Potential enhancements for propagation delay compensation

In RAN1#102-e meeting, the following option 1 and option 2 are agreed for further study in RAN1.

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

TA-based propagation delay compensation

The following agreement was achieved in RAN1#106-e:

Agreement

Send LS to RAN4 to ask for feedback on the following questions:

- Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most.

- Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition.

- Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4

- Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also.

- Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information.

- Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

In the RAN#93-e meeting, the following was concluded.

conclusion: For the objective on enhancements for support of time synchronization, RAN should provide the following guidance: - RAN4 to provide reply LS to RAN1 (e.g. in response to R1-2108635 on TA-based PDC and a potential RAN1 LS on RTT-based PDC) before the start of RAN1#107-e (Nov 11th)

In RAN4#101-e meeting, the reply LS R4-2120336 was agreed in RAN4 and sent to RAN1 as shown in R1-2112594. The key part of the LS R4-2120336 is copied below for your information.

- Overall Description: RAN4 thanks RAN1 for the LS R1-2108635 about TA-based propagation delay compensation. RAN4 has discussed the questions in R1-2108635 and RAN4 provides the following answers: Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most. Answer: - There is no consensus in RAN4 whether it is feasible to support a smaller value than the current Te assuming the existing conditions in TS 38.133 for Te requirement. - RAN4 considers that it is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under the assumption of using TRS (or other RS used for Te estimation) instead of SSB. - A smaller Te can be achieved when TRS (or other RS) bandwidth is larger than SSB bandwidth. - A Smaller Te can be achieved for UE is operating in RRC_CONNECTED mode. - There is no consensus in RAN4 about how much Te value can be reduced at most or the related conditions, which RAN4 will discuss further. - Support of Smaller Te can be defined as an optional Rel-17 capability and should not apply to all UEs. - From RAN4 view it is still being studied whether a smaller Te can be achieved for the first transmission in the DRX cycle and would like to ask RAN1 the following: - If the presence of TRS (or other RS) can be guaranteed during the DRX OFF? Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2), similar to the granularity for Rel-16 IAB based on the Timing Delta MAC CE) and related condition. Answer: - It is feasible to introduce enhanced TA command indication granularity. - It is RAN4 understanding that: - Enhanced granularity will apply for UE supporting PDC - This agreement does not impact UL timing accuracy requirement - TA command indication granularity can be reduced to 64Tc for both 15 kHz SCS and 30 kHz SCS. - RAN4 will further discuss whether there are any specific conditions to use improved TA command indication granularity - Actions: To RAN 1 group: RAN4 respectfully asks RAN1 to take the above information into consideration.

Feature lead: based on the feedback from RAN4 above, it is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under the assumption of using TRS (or other RS used for Te estimation) instead of SSB. But there is no consensus in RAN4 about how much Te value can be reduced at most. TA command indication granularity can be reduced to 64Tc for both 15 kHz SCS and 30 kHz SCS.

Evaluation on the achievable time synchronization accuracy for enhanced TA-based PDC

Based on inputs from companies in RAN1#107-e meeting, the total error results are summarized below. Note that the evaluation for the existing TA-based PDC, i.e. no any enhancements on Te and/or TA command indication granularity, is not included in the table, since we are mainly looking at the potential performance for enhanced TA-based PDC.

Table 1 Overall synchronization error summary for RTT-based PDC

| Source | overall synchronization error |  | Note |
| --- | --- | --- | --- |
|  | 15 kHz | 30 kHz |  |
| Nokia (R1-2111141) | 297 ns | 255 ns | 50 MHz (i.e.270 RB) bandwidth for DL RS for 15 kHz; 100 MHz (i.e.273 RB) bandwidth for DL RS for 30 kHz; errorBS,UL,RX = 100 ns; errorUE,DL,RX = 10 ns for 15 kHz, 5 ns for 30 kHz; Te = 262 ns for 15 kHz, Te = 196 ns for 30 kHz; (1/16)*existing TA granularity; |
| OPPO (R1-2111344) | >275 | 245.8+δ+Y2 | 24 RB bandwidth for DL RS for 15 kHz; 24 RB bandwidth for DL RS for 30 kHz; |
|  | 247.6+δ+Y2 | 184.5+δ+Y2 | 106 RB bandwidth for DL RS for 15 kHz; 51 RB bandwidth for DL RS for 30 kHz; |
| ZTE (R1-2110917) | 242 ns | 211 ns | 106 PRB bandwidth for RS for 15 kHz; 51 PRB bandwidth for RS for 30 kHz; 32 Tc for TA command indication granularity; Te: 195 ns for 15 kHz, 130 ns for 30 kHz; |
| Intel (R1-2111492) | 246 ns | 246 ns |  |
| Samsung （R1-2111733） | ~265 ns | 117ns | 132 PRB/176 PRB for DL/UL reference signal (1/4 )* Te, (1/16)*TA granularity |
| Huawei (R1-2111926) | 253ns+12Te | 174ns+12Te | 24RB for DL RS, 44 RB for UL RS, 15 kHz; 24RB for DL RS, 48 RB for UL RS, 30 kHz; (1/16)*TA granularity |
|  | 141ns+12Te | 117ns+12Te | 104RB for DL RS, 176 RB for UL RS, 15 kHz; 132B for DL RS, 176 RB for UL RS, 30 kHz; (1/16)*TA granularity |
| Feature lead | 166ns+12Te | 161ns+12Te | 50 MHz (i.e.270 RB) bandwidth for DL RS for 15 kHz; 100 MHz (i.e.273 RB) bandwidth for DL RS for 30 kHz; errorBS,UL,RX = 100 ns (i.e. no special enh. for UL RS); errorUE,DL,RX = 10 ns for 15 kHz, 5 ns for 30 kHz; (1/16)*existing TA granularity = 64 Tc; |
| Feature lead | 132ns+12Te | 119ns+12Te | 50 MHz (i.e.270 RB) bandwidth for DL RS for 15 kHz; 88 RB bandwidth for UL RS for 15 kHz; 100 MHz (i.e.273 RB) bandwidth for DL RS for 30 kHz; 88 RB bandwidth for UL RS for 30 kHz; errorBS,UL,RX = 0.5/(N_PRB_DL_RS*12*SCS; (1/16)*existing TA granularity; |
| Feature lead | 124ns+12Te | 115ns+12Te | 50 MHz (i.e.270 RB) for DL RS for 15 kHz; 176 RB bandwidth for UL RS for 15 kHz; 100 MHz (i.e.273 RB) for DL RS for 30 kHz; 64 MHz (i.e.176 RB) for UL RS for 30 kHz; errorBS,UL,RX = 0.5/(N_PRB_DL_RS*12*SCS; (1/16)*existing TA granularity = 64 Tc; |
| Note: errortotal, TAbased≤errorBS,DL,TX+errorUE,DL,RX+12errorBS,DL,TX+Te+errorBS,UL,RX+errorTA_indication |  |  |  |

Feature lead:

Based on the inputs in Table 1, although RAN4 didn’t provide the enhanced Te, it can be expected that enhanced TA-based PDC with 30 kHz is able to meet the upper bound of the synchronicity budget for control-to-control scenario, since there is sufficient room to cover the error due to Te. However, the case of 15 kHz would depend on the condition, e.g. the bandwidth of the RS.

To evaluate the best performance that enhanced TA-based PDC can be achieved, feature lead provides the potential overall synchronization error as shown in the last three rows in Table 1 above, assuming maximum bandwidth for DL RS, i.e. 50 MHz (i.e.270 RB) bandwidth for DL RS for 15 kHz and 100 MHz (i.e.273 RB) bandwidth for DL RS for 30 kHz. Note that it is assumed that TA command indication granularity can be reduced to 64 Tc.

According to the RAN4 discussions, it seems 262 ns for 15 kHz and 196 ns for 30 kHz corresponding to the maximum RS bandwidth is what RAN4 was considering, however in the end no consensus achieved, which means there is no any value sent from RAN4 to RAN1 on reduced Te. Note that 262 ns for 15 kHz and 196 ns for 30 kHz corresponding to the maximum RS bandwidth are also discussed in Nokia R1-2111141, which looks reasonable and we can use it for have some rough idea on the potential performance of enhanced TA-based PDC.

From RAN1 perspective, seems not much we can do without the enhanced Te value from RAN4, but at least we can try to see check the potential best performance that TA-based PDC can achieve, in order to compare with RTT-based PDC. Initial observations as shown in section 2.1.1.1.

First round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed observation 2.1.1.1-1: enhanced TA-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is the maximum bandwidth (i.e. 273 PRB).

| Company | View |
| --- | --- |
| Nokia, NSB | Agree with the observation. Propose to add SCS to the observations (applies to 2.1.1.1-1, 2.1.1.1-1-2, 2.2.1.1-1 and 2.2.1.1-2) [Feature lead]: SCS is already included in the observations. |
| ZTE | We agree with this observation. |
| OPPO | The statement itself says nothing wrong, but it misses the point that enhanced TA-based PDC can do much better. [Feature lead]: The problem is without enhanced Te from RAN4, you can expect that it would be very difficult to conclude that enhanced TA-based can do better, even the assumption of Te for the maximum bandwidth is challenged by companies as you could see. Based on Nokia R1-2111141, Te can include DL detection error and a Te margin. In the best case, DL detection error can be improved to 5 ns for 30 kHz assuming 100 MHz DL RS, and correspondingly Te can be enhanced to 196 ns if Te margin can not be reduced. Since no values from RAN4, it would be difficult in RAN1 to conclude whether the margin can be reduced or not, thus 196 ns for Te is the one we can use for now to evaluate the enhanced TA-based performance. For 30 kHz, OPPO evaluated DL RS 51 RB, without reduction of the margin, then Te would be about 216 ns, which cannot meet the budget based on formula given by OPPO. ZTE evaluated 51 RB also, however ZTE assumes 32 Tc which is not aligned with RAN4 reply, and also assume 130 ns for Te, without reduction of the margin we cannot assume this value. For 30 kHz, Samsung and Huawei evaluated 132 RB for DL RS, it is expected that Te can be reduced to about 206 ns without the reduction of the margin. Therefore, at least based on the formula given by Huawei and Samsung, it seems the budget can met 275 ns. Therefore, I will update the observation accordingly. The claimed observation is based on the use of Te as error bound for errorUE,UL,TX+errorUE,DL,RX, where errorUE,DL,RX is the dominant component. However, the original Alt-1 error formula for TA-based PDC does not contain Te. Te is just one most generous upper-bound that is applicable to any BW and meanwhile does not assume synchronization assistance from certain DL-RS. Given the observation targets to TA-based PDC capability per bandwidth and RAN1-106bis-e agreed the TA-based PDC error analysis can assume DL-RS based error formula (0.5/(12RB*SCS)), Te can be put aside now. Meanwhile, we think the use of TA granularity of 64Tc in error formula is too aggressive. When the TA granularity can be so low as 64Tc in TA signaling, there will be another factor - TA accuracy - starting to exhibit its effect. RAN4 currently defines TA accuracy as +/-256Tc. There is no RAN4 message saying this accuracy is also reduced for SCS equal to 15kHz and 30kHz. So total error for TA-based PDC with enhanced TA granularity for a given RB is: 65+100+120.5∙10912RB∙30k*2+margin+256Tc≤275. Assume a margin of 60ns, we can have RB≥93. So the enhanced TA-based PDC under 30kHz SCS can meet Uu error budget of 275ns when DL/UL RS BW is larger than 93RB, not just 273RB. |
| Samsung | Agree with the observations. And we share similar views with OPPO that other than the time when waking up from DRX, Te can be much smaller. And for RTT based method, we think it doesn’t assuming the case that just waking up from DRX. [Feature lead]: Please see my reply to OPPO above. |
| Vivo | Agree with the observation in principle. We share similar views with OPPO. We think the timing detection error calculation should consider the RS bandwidth according to existing agreement, i.e., 0.5/(N_PRB*12*SCS), because the TRS with wider bandwidth can be used for TA-based PDC. In this case, the required bandwidth for DL RS can be smaller. [Feature lead]: Please see my reply to OPPO above. |
| Qualcomm | Not support observation. We do not know why our results in R1-2112212 were not collected in the table. The further investigation is needed. [Feature lead]: Above the table you can see my note on the reason. I can copy here again. Note that the evaluation for the existing TA-based PDC, i.e. no any enhancements on Te and/or TA command indication granularity, is not included in the table, since we are mainly looking at the potential performance for enhanced TA-based PDC. |
| Intel | Agree |
| Ericsson | Do not agree. First, the TA indication granularity cannot be reduced and plugged into the evaluation equation. RAN4 LS sentence: “This agreement does not impact UL timing accuracy requirement”. The corresponding RAN4 agreement in R4-2120335 is: - New TA command indication granularity is only applicable to UEs supporting PDC and it is understood that granularity and accuracy are not the same. Hence, this agreement does not impact UL timing requirement. This means the TA accuracy is not improved even if indication granularity is improved. In our understanding, errorTA_indication in the evaluation formula is used as is because TA indication granularity is consistent with accuracy. If TA indication granularity is improved but accuracy is not, then one cannot plug in a smaller value for errorTA_indication, rather the existing value should still be used to represent TA error. [Feature lead]: errorTA_indication indeed means the indication granularity, according to agreements before TA adjustment accuracy is not considered for enhanced TA-based PDC. Therefore, I think it is fine to use 64 Tc given by RAN4 to do the evaluation. Second, RAN4 reply contains no reduced Te value. Hence, we don’t think it is justified that RAN1 creates a new Te value for RAN4. [Feature lead]: RAN4 replied it is feasible to support a smaller Te value than the current Te using TRS, just no accurate value given by RAN4 yet. In which case I think it is reasonable to use the principle given in Nokia paper to roughly evaluate the potential reduced Te, i.e. Te can include DL detection error and a Te margin, and with 0.5/(N_PRB*12*SCS) for DL detection error, it can be expected that the Te can be reduced to 262 ns for DL RS 50 MHz for 15 kHz and 196 ns for DL RS 100 MHz for 30 kHz, without reduction of the margin which may be more relevant to RAN4. So I think in RAN1 we can look at the performance for enhanced TA-based PDC assuming no reduction of the margin of Te. Thirdly, the observation fails to include all necessary conditions and assumptions: used max BW for UL; used a reduced Te which is not approved by RAN4, used 1/16 TA indication granularity to represent TA error (which is incorrect as discussed above). [Feature lead]: The original observation is assuming the maximum bandwidth, thus no restriction for UL RS, that’s why it is not mentioned. I can add the granularity. |
| HW/HiSi | Agree with the observation. |

Proposed observation 2.1.1.1-2: enhanced TA-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is the maximum bandwidth (i.e. 270 PRB) and the bandwidth for UL RS is larger than 44 RBs.

| Company | View |
| --- | --- |
| Nokia, NSB | Agree with the observation. |
| ZTE | We agree with this observation. |
| OPPO | Similar comment as above. The observation may say nothing wrong, but it does not precisely mention what enhanced TA-based PDC can do for the best. [Feature lead]: Since we don’t have values for Te from RAN4, then the best we can do is to use the principle given in Nokia paper to roughly evaluate the potential reduced Te, i.e. Te can include DL detection error and a Te margin, and with 0.5/(N_PRB*12*SCS) for DL detection error, it can be expected that the Te can be reduced to 262 ns for DL RS 50 MHz for 15 kHz and 196 ns for DL RS 100 MHz for 30 kHz, without reduction of the margin which may be more relevant to RAN4. Then for 104 or 106 RBs for DL RS, it is expected that about 277 ns Te can be achieved. Using 277 ns as the enhanced Te, OPPO formula cannot meet the budget for 15 kHz, Huawei formula cannot meet the budget. Samsung evaluated 132 RBs but use (1/4)*Te, ZTE also used a very small Te. It is expected that a observation to say better performance can be achieved cannot be agreed by all companies. |
| Samsung | Agree and also think TA based method can do better. [Feature lead]: Please see my replies above to OPPO. |
| vivo | Agree with the observation in principle. We share similar views with OPPO and Samsung. The bandwidth for DL RS can be smaller. [Feature lead]: Please see my replies above to OPPO. |
| Qualcomm | Not support observation. We do not know why our results in R1-2112212 were not collected in the table. The further investigation is needed. [Feature lead]: Above the table you can see my note on the reason. I can copy here again. Note that the evaluation for the existing TA-based PDC, i.e. no any enhancements on Te and/or TA command indication granularity, is not included in the table, since we are mainly looking at the potential performance for enhanced TA-based PDC. |
| Intel | Agree |
| Ericsson | Do not agree. See our input to Proposed observation 2.1.1.1-1. |
| HW/HiSi | Agree |

Proposed observation 2.1.1.1-3: If the bandwidth for DL RS is smaller than the maximum bandwidth (i.e. 270 PRB), enhanced TA-based PDC may or may not meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario.

| Company | View |
| --- | --- |
| Nokia, NSB | Do not agree The observation is in principle correct, but the “may or may not” renders the observation to give no useful information. Assuming that a DL RS with a smaller bandwidth than theoretical maximum, this will require other error components (Te margin, and/or UL RS) to be even further reduced in order to meet the budget. We can also turn it around and say that if Te margin is not changed, and we assume maximum theoretical UL bandwidth (50MHz for 15kHz and 100MHz for 30kHz SCS), the minimum DL RS bandwidth in order to meet the budget of 275ns is 19MHz. [Feature lead]: Maybe we can try the first direction you mention, since values used here are some theoretical ones, I guess it doesn’t mean much to give the accurate bandwidth. |
| ZTE | We agree with this observation. |
| OPPO | Do not agree. Our analysis above shows that whether DL-RS BW can be 270RB is not the key criteria. |
| Samsung | Agree with Nokia’s suggestion. |
| vivo | Agree with Nokia’s suggestion. |
| Qualcomm | Do not agree. Further investigation is needed. |
| Intel | Prefer Nokia’s version |
| Ericsson | Do not agree. Our calculation (R1-2111191) shows that TA-based method is not able to satisfy the control-to-control requirement. |
| HW/HiSi | Agree with the observation. |

Second round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised proposed observation 2.1.1.1-1: enhanced TA-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than or equal to 132 RBs, UL RS is larger than or equal to 176 RBs and the enhanced TA command indication granularity is 64 Tc.

| Company | View |
| --- | --- |
| Feature lead | Please all check my replies to OPPO in the table in section 2.1.1.1.1 to understand the reason to do the revision. |
| Samsung | Agree. |
| Nokia, NSB | Agree But we may prefer to be a bit more elaborate on the conditions on BW. As we understand, the conditions for 30kHz SCS, is met when we assume a 270RB DL RS, and then an additional 20RB UL RS or a 270RB UL RS and then an additional 55RB DL RS and then a region in-between. [Feature lead]: Correct, the current observation may mean that UL 176 RBs is always required. Let me try to update it for further discussion. |
| ZTE | We are agree with this observation. |
| Ericsson | Do not agree Suggest the following revisions: - Add: “based on theoretical estimate of possible Te reduction”. This is to clarify that Te was not based on link simulation studies which should be performed by RAN4, but no such value is available [Feature lead]: I added the formula we used to calculate Te in the observation, please check. - Add: “without considering TA adjustment accuracy even though TA granularity and accuracy are not the same”. This is to clarify that TA accuracy error is not reduced together with TA granularity reduction (see RAN4 agreement), but this is not accounted for. [Feature lead]: As we discussed several times, TA adjustment accuracy is not considered, therefore I think we don’t need to add it. ============= Conclusion When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error. Agreements: TA adjustment accuracy is not considered for the evaluation of time synchronization error. ============= |
| OPPO | Again, the statement says nothing wrong, but the TA-based PDC can do better than what the statement says, even without changing current TA granularity but with the DL-RS configuration allowed by existing spec, e.g., - certain DL-RS is in the same frame as a SIB9; and - DL/UL RS are used for UE/gNB to measure Rx timing. This should allow an error on UE side being smaller than what Te upper-bounds. This is the fundamental difference from FL’s calculation logic – FL seems to keep using Te. Please see our comments under “observation 2.2.1.1-3” for why we think the evaluation for comparison should not use Te. [Feature lead]: I didn’t see detailed comments from you under observation 2.2.1.1-3. In addition, enhanced TA-based PDC is with reduced Te and enhanced TA command granularity, i.e. the assumptions among companies for enhanced TA-based PDC, I don’t understand why we cannot use Te here. Of course, for the evaluation, as you can expected we use certain bandwidth for DL RS and UL RS to improve the detection error also, as agreed in RAN1#106b-e. To be more specific, assume SCS=30kHz: - If SIB9 does not have a DL-RS in the same frame, DL_BW=UL_BW=106RB and 0.5/(N_PRB*12*SCS)=13ns, 65+100+(Y+13+δ+13+130)/2=243+(Y+δ)/2; - If SIB9 does have a DL-RS in the same frame and DL_BW=UL_BW=24RB and 0.5/(N_PRB*12*SCS)=58ns, 65+58+(Y+58+δ+58+130)/2=246+(Y+δ)/2; People can see both cases requires less number of RBs than “DL_BW=132RB and UL_BW=176RB”. [Feature lead]: Yes, that is why in the observation the bandwidth of the RS are mentioned also. |
| HW/HiSi | Agree |
| vivo | In my opinion, errorUE,UL,TX and errorUE,DL,RX calculation can base on formula 0.5/(N_PRB*12*SCS). In this case, Te needn’t be considered. Thus, the required PRB can be smaller. If my understanding is wrong, please correct me. [Feature lead]: Yes this formula is used in the evaluations also. |
| Qualcomm | Do not agree. More accurate statement suggested by Ericson is better. [Feature lead]: Please see my replies to Ericsson. |

Revised proposed observation 2.1.1.1-2: enhanced TA-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is the maximum bandwidth (i.e. 270 PRB), the bandwidth for UL RS is larger than 44 RBs and the enhanced TA command indication granularity is 64 Tc.

| Company | View |
| --- | --- |
| Feature lead | Please all check my replies to OPPO in the table in section 2.1.1.1.1 to understand the reason to do the revision. |
| Samsung | Agree. |
| Nokia, NSB | Agree |
| ZTE | We are agree with this observation. |
| Ericsson | Do not agree - UL RS should be change to 176 PRB [Feature lead]: 44 RBs can meet the budget also based on the equation for TA-based PDC. - Add: “based on theoretical estimate of possible Te reduction”. [Feature lead]: I added the formula we used to calculate Te in the observation, please check. - Add: “without considering TA adjustment accuracy even though TA granularity and accuracy are not the same”. [Feature lead]: Please see my replies to your comments under the first question. |
| OPPO | Similar comment as for 30kHz case. Please see our comments under “observation 2.2.1.1-3” for why we think the evaluation for comparison should not use Te. To check the consumed RBs, assume SCS=15kHz and DL_BW=UL_BW=106RB. 0.5/(N_PRB*12*SCS)=26ns . In case DL-RS is in the same frame as a SIB9 and is used by UE to determine SFN timing, 65+26+(Y+26+δ+26+260)/2=247+(Y+δ)/2 This is the error even without reducing TA command granularity, and only needs 106RBs on DL and UL. [Feature lead]: Please see my replies to your comments under the first question. |
| HW/HiSi | Agree |
| Qualcomm | Support |

Revised proposed observation 2.1.1.1-3: If the bandwidth for DL RS is smaller than the theoretical maximum bandwidth (i.e. 270 PRB), enhanced TA-based PDC requires other error components, e.g. Te margin and/or bandwidth of UL RS, to be further reduced in order to meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario.

| Company | View |
| --- | --- |
| Samsung | Agree. |
| Nokia. NSB | Agree |
| ZTE | We are agree with this observation. |
| Ericsson | Agree |
| OPPO | Please see our comments under “observation 2.2.1.1-3” for why we think the evaluation for comparison should not use Te. [Feature lead]: Please see my replies to your comments under the first question. |
| Huawei, HiSilicon | Agree |
| Qualcomm | Agree |

2.1.1.3 Third round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised proposed observation 2.1.1.1-1: enhanced TA-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, assuming the enhanced TA command indication granularity is 64 Tc and (191 + 0.5/(N_PRB*12*SCS)) ns for Te, and at least under the following conditions:

- The bandwidth for DL RS is larger than or equal to 132 RBs and UL RS is larger than or equal to 176 RBs, or

- The bandwidth for DL RS is the maximum bandwidth (i.e. 273 PRB) and UL RS is larger than or equal to 20 RBs, or

- The bandwidth for UL RS is the maximum bandwidth UL RS (i.e. 273 PRB) and the bandwidth for DL RS is larger than or equal to 55 RBs.

- Note: 191 ns is assumed as the Te margin for 30 kHz.

| Company | View |
| --- | --- |

Revised proposed observation 2.1.1.1-2: enhanced TA-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, assuming the enhanced TA command indication granularity is 64 Tc and (252 + 0.5/(N_PRB*12*SCS)) ns for Te, and at least when the bandwidth for DL RS is the maximum bandwidth (i.e. 270 PRB) and the bandwidth for UL RS is larger than 44 RBs.

- Note: 252 ns is assumed as the Te margin for 15 kHz.

| Company | View |
| --- | --- |

Revised proposed observation 2.1.1.1-3: If the bandwidth for DL RS is smaller than the theoretical maximum bandwidth (i.e. 270 PRB), enhanced TA-based PDC requires other error components, e.g. Te margin and/or BS detection error depending on bandwidth of UL RS, to be further reduced in order to meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario.

| Company | View |
| --- | --- |
| Feature lead | @ OPPO Please check if you can accept it. It seems all other companies are fine with it. |

Whether/how to configure UL signal for enhanced TA-based PDC

In the RAN1#106bie-e meeting, the following was agreed with an FFS whether/how to configure UL signal for enhanced TA-based PDC.

Agreement If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC. - FFS whether/how to configure UL signal for enhanced TA-based PDC

In this meeting, companies share their views as below.

Nokia (R1-2111141) Also, in the previous meeting it was discussed whether an UL signal, such as SRS, would need to be configured for the UE so that the enhanced Te requirement would only apply for the configured TRS/SRS pair. In our view, imposing such UL signal configuration for enhanced TA based PDC by default should not be needed. After detecting the TRS, the UE should transmit at least one uplink signal complying with the enhanced Te requirement, while should be up to gNB implementation to schedule/configure the UL transmission. However, in case the Te margin reduction is concluded to be unfeasible in RAN4, the impact of UL reception error would need to be reduced for meeting the control-to-control budget with an enhanced TA. In that case, the configuration of a higher bandwidth SRS would be needed for reducing the gNB UL reception error. That means, instead of an UL reception error based on minimum UL bandwidth, such error component would need to be reduced by at least 44ns. So, UL reception gets to errorBS,UL,RX=100ns-44ns=56ns, which corresponds to an UL signal of 0.5/56ns=8,9MHz or 50RBs in 15kHz. Observation 2.1.2: For enhanced TA based PDC to meet the synchronicity budget of control-to-control in 15kHz, if a maximum bandwidth TRS is used for reducing Te while the Te margin for implementation is not changed, an UL signal of at least 8.9 MHz is needed in order to reduce the errorBS,UL,RX Proposal 2.1.1: If enhanced TA-based PDC is supported, the configuration of an UL signal (e.g. SRS) for enhanced TA should only be considered in case the Te margin component of Te cannot be reduced (depending on RAN4 input).

ZTE (R1-2110917) During the TA procedure in NR, SRS can be used for the network to perform measurement and adjust the UE TA in addition to PRACH. Therefore, it is straightforward to use SRS for TA-based PDC in Rel-17. Proposal 1: If enhanced TA-based PDC with reduced Te is supported in Rel-17, one SRS configuration can be configured for TA-based PDC.

vivo (R1- 2111008) For improving uplink receiving accuracy at gNB, it is unnecessary to restrict a specific UL signal/channel for enhanced TA-based PDC. Actually, any UL channel(s)/signalling(s) can be used for the enhanced TA-based PDC with reduced Te, e.g., SRS/PUSCH. After transmitting TRS, gNB can schedule aperiodic SRS or A-CSI or PUSCH for UE to transmit for the enhanced error detection. The detailed requirement needs RAN4 input e.g., the bandwidth and density of UL channel or signalling. Proposal 1: If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, UL signal for enhanced TA-based PDC is up to implementation.

OPPO (R1- 2111344) - gNB configures a list of SRS signals in SRS-Config (contained in BWP-UplinkDedicated), and uses any subsets of the received SRS to check alignment of UL-Rx timing. Whether there is certain SRS specifically for PDC purpose could be transparent to UE. - gNB configures one or more TRS signals, one in each NZP-CSI-RS-ResourceSet, in IE of CSI-MeasConfig contained in ServingCellConfig. It is up to UE implementation to use the TRS with the largest configured bandwidth, or the CSI-RS (but not TRS) with the largest configured bandwidth, or the combination of both plus even additionally the other configured CSI-RS and/or any other DL-RS (if transmitted in the same radio frame) to detect the DL-Rx timing. The gNB does not need to assign a specific TRS that UE should exclusively use for TA-based PDC purpose. Some of those DL-RS may even be transmitted in the same radio frame as SIB9 that carries ReferenceTimeInfo to improve the SFN timing detection accuracy, without introducing additional specification changes (at least in RAN1/RAN2). Observation 3: The existing specification does not prevent gNB and UE from using the CSI-RS/TRS and any other DL-RS to improve the DL-Rx timing detection accuracy. - There is no need to have PDC-specific DL/UL RS configuration for TA-based PDC. - The error performance of TA-based PDC can leave existing Te requirement unchanged, since Te is an upper-bound used by spec for “TA control” purpose and does not prevent UE from performing better by using DL-RS in “TA measurement”. Therefore, RAN4’s response on potential Te enhancement is not relevant to RAN1’s evaluation that uses DL-RS to improve TA interval measurement.

Samsung (R1-2111733) Proposal #2: If TA based method is adopted, TRS is used as DL reference signal and SRS can be used for UL reference signal.

Feature lead: Based on the analysis in section 2.1.1, it can be seen that for 15 kHz, enhanced TA-PDC can meet the budget only if the bandwidth for UL RS is larger than 44 RBs, which means we cannot rely on any uplink signal/channel for PDC, and thus specific SRS configuration should be introduced.

First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.1.2.1-1: If enhanced TA-based PDC with reduced Te is supported, one SRS configuration is configured at least for enhanced TA-based PDC under 15 kHz.

| Company | View |
| --- | --- |
| Nokia, NSB | Agree, However, we think this could also be left for gNB implementation to schedule an UL RS of the required BW to meet the accuracy desired and the UL RS does not have to be restricted to SRS. |
| ZTE | We support this proposal because SRS has already been able to be used for TA adjustment in Rel-15/Rel-16. It depends on gNB implementation. We understand this proposal does not mean gNB has to configure SRS dedicated for TA-based PDC. This is just for evaluation and discussion. The network can configure DL/UL RS if needed. In addition, for SRS, the configuration may always be needed for the network to measure the UL channel state. |
| OPPO | Agree to have a general SRS configuration, but do not agree to have PDC-oriented SRS configuration for TA-based PDC. Similar to Nokia’s comment, gNB configures an SRS with appropriate BW without having to tell UE what is the true purpose of SRS (i.e., it is supported by existing spec). UE transmits SRS per current spec and gNB receives/measures SRS given gNB knows what the SRS is for. |
| Samsung | Agree with Nokia. Maybe we can say, “SRS can be used for enhanced TA-based PDC”. |
| vivo | It is unnecessary to restrict a specific RS, so long as the bandwidth and density of RS can meet the error accuracy requirement. It can be left for gNB implementation. |
| Qualcomm | We share the same view as vivo. |
| Intel | It can be up to gNB. |
| Ericsson | Agree that a special SRS is needed if enhanced TA-based PDC. But what’s the reason to have ’15 kHz’ as a condition? |
| HW/HiSi | Agree. But leaving it to implementation is also fine for us. |

Feature lead: The original intention is to configure specific SRS to ensure the performance. However, if companies want to leave it to gNB implementation, that would be fine also. We can update the proposal accordingly.

Second round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion: If enhanced TA-based PDC with reduced Te is supported, UL RS with sufficient bandwidth is ensured by implementation in order to meet the synchronization budget.

| Company | View |
| --- | --- |
| Samsung | Agree |
| Nokia, NSB | Agree It seems that we have forgotten to discuss if an enhanced Te requirement is introduced and we do not configure an UL RS specifically for PDC where enhanced Te applies, should the enhanced Te requirement apply to all UL transmissions, or how does the UE know which UL RS it should comply with the enhanced Te and which it should comply with legacy Te? Lets remember that legacy Te is sufficient for all non PDC relevant UL transmissions. [Feature lead]: Yes we need to discuss this also, let me set up a proposal in the next round for this. My thinking is that the simplest way is to also configure a PDC specific SRS for enhanced TA-based PDC, then it would be very clear that the enhanced Te will be applied only to the SRS. Of course, in this case, gNB will only use this SRS for BS detection error detection, and UE will also apply this enhanced Te to this PDC specific SRS. As we agreed before, the enhanced TA command granularity will not have impact on the normal TA procedure, similarly I think the enhanced Te is better not to have impact on the normal uplink channel/signal also. Above is just my thinking, let’s further discuss. |
| ZTE | We are agree with this observation. |
| Ericsson | Do not agree. Why is reduced Te ensured by SRS bandwidth? Shouldn’t Te reduction be supported by large enough DL RS? It would make sense if the goal is to achieve improved TA accuracy and granularity. [Feature lead]: UL RS here is not only for reduced Te, but also for better BS detection error. According to the RAN4 discussion, the bandwidth of the uplink RS would have impact on the Te also actually, since Te includes both DL frame timing error and the uplink transmit timing error. If “with reduced Te” is confusing, let’s put in bracket to just explain what an enhanced PDC would look like. Or the intention is UL SRS is needed together with Te reduction to reach sync budget? If so, the above is still not agreeable since it’s unclear how gNB can always ensure large enough UL RS, especially given that largest BW SRS is required. What if there is not enough PRBs in the BWP? It’s more like: “If enhanced TA-based PDC with reduced Te is supported, UL RS with sufficient bandwidth may or may not be ensured by implementation in order to meet the synchronization budget.” [Feature lead]: I will say if gNB wants to use this enhanced TA-based method, then it needs to find an appropriate instance to transmit the UL RS. |
| OPPO | We are fine with the conclusion, although we do not think the use of sufficient UL-RS BW should be conditioned on “if Te is reduced”. Te is for a measurement on UE side while UL-RS is for measurement on gNB side. To provide UE with whatever UL-RS BW is an implementation allowed by existing spec, even without Te being reduced. |
| OPPO | We are fine with the conclusion, although we do not think the use of sufficient UL-RS BW should be conditioned on “if Te is reduced”. Te is for a measurement on UE side while UL-RS is for measurement on gNB side. To provide UE with whatever UL-RS BW is an implementation allowed by existing spec, even without Te being reduced. [Feature lead]: The bandwidth of the uplink RS would have impact on the Te also actually, since Te includes both DL frame timing error and the uplink transmit timing error. According to the discussion in RAN4, the bandwidth of the uplink signal matters also. In addition, agree with you that the uplink RS bandwidth would have impact on the BS detection error also, which means sufficient bandwidth is required also from the overall error budget perspective. |
| HW/HiSi | Agree |
| vivo | Agree |

2.1.2.3 Third round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion: If enhanced TA-based PDC (e.g. with reduced Te and enhanced TA command indication granularity) is supported, UL RS with sufficient bandwidth is ensured by implementation in order to meet the synchronization budget.

| Company | View |
| --- | --- |
| Feature lead | @Ericsson Please check my replies to your comments above. Hopefully this conclusion can be acceptable for you. |

Proposal 2.1.2.3-1: If enhanced TA-based PDC (e.g. with reduced Te and enhanced TA command indication granularity) is supported, one SRS configuration is configured for enhanced TA-based PDC.

| Company | View |
| --- | --- |
| Feature lead | @all Please check my replies to Nokia comments in the above section to understand why we have this proposal. Briefly speaking, the intention is to avoid impact on normal uplink channel/signal transmission due to enhanced Te, just similar as what we agreed before that the enhanced TA command granularity will not have impact on the normal TA procedure. |

How to signal the enhanced TA for enhanced TA-based PDC

ZTE (R1-2110917) Proposal 5: TA-based PDC should be supported for the propagation delay compensation enhancements with reducing the initial transmission error (Te) and reusing the enhanced timing advance MAC CE.

Feature lead: According to the current agreement in RAN4, it is feasible to reduce the TA command indication granularity to (1/16) of the existing granularity, similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE. Therefore, it should be straightforward to use MAC CE to indicate the TA command for enhanced TA-based PDC.

Huawei (R1-2111926) If TA-based PDC is supported, then we need to define the range of the enhanced TA command indication. This is related to the distance between the gNB and UE. The service area of control-to-control and smart grid is 1000m × 100m and 20km2, the radius would be 178m and 2.5km respectively assuming the area is a circle. Thus the enhanced TA command indication can be from 0 to 16384Tc. Proposal 3: If TA-based propagation delay compensation is supported for PDC, the enhanced TA command indication is from 0 to 16384 Tc.

Feature lead: The range for the TA command indication is needed. The proposal from Huawei is used for further discussion, companies can double check if it is correct.

First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.1.3.1-1: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the enhanced TA command is signaled by MAC CE.

| Company | View |
| --- | --- |
| Nokia, NSB | Agree, if we reuse IAB Timing Delta MAC CE to minimize specification impact. Disagree if we need to introduce a new MAC CE. Similarly to what is discussed for RTT based estimation, the PD estimation can be timely delivered with higher layer signalling, e.g. RRC. [Feature lead]: Since we don’t want to impact the legacy TA procedure, better to introduce a new MAC CE to indicate the absolute TA value. For RTT based PDC, the signaling is used to report the Rx-Tx time difference measurement. Can you explain more why RRC signal is preferred than MAC CE here? |
| ZTE | We support this proposal since MAC CE is needed just like the enhanced TA MAC CE introduced in IAB. |
| OPPO | Agree. |
| Samsung | Support. |
| vivo | We are fine with the proposal. |
| Qualcomm | Do not agree. We can decide this after comparing this with the other solutions. |
| Intel | OK |
| Ericsson | OK |
| Hw/HiSi | Support, the MAC CE signaling design is up to RAN2 |

Proposal 2.1.3.1-2: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the range of the enhanced TA command is from 0 to 16384Tc.

| Company | View |
| --- | --- |
| Nokia, NSB | Disagree, If we introduce an enhanced TA command, the range should not be bounded by the use cases considered in a single Release. Instead we would be more fond of basing the range out of either; - The legacy timing advance - A range covering the cyclic prefix (extended) Obvious this problem is also addressed if reusing the Timing Delta MAC CE which acts as a as an offset from the configured NTA. |
| ZTE | We agree with the analysis. We are also agree with Nokia that the range should not limited by the use case or a Release. So at least the value rang from 0 to 16384Tc can be supported. We are open to discuss the bigger range for better compatibility. |
| OPPO | Do not agree. RAN1 has not agreed to support absolute TA command for enhanced TA indication. The enhanced TA granularity, if eventually agreed, can work just for relative TA command. We think RAN1 should firstly discuss whether to have single TA loop with two different TA granularities or two TA loops with two different TA granularities, before considering this proposal. [Feature lead]: With the agreement below, the enhanced TA command indication granularity will not have impact on the normal TA procedure anyway. The original intention is to indicate absolute TA command directly, in which case we don’t need two loops. Let’s make some proposal for further discussion. Agreement If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17, - The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity. |
| Samsung | More study is needed on whether this is absolute TA or not. In our view, a simple, clean, solution is preferred, i.e. one absolute TA for PDC. The range can leave some room for future. |
| vivo | We share the similar view with Nokia and ZTE that the range should not limited by the use cases or a Release. |
| Qualcomm | Further study is necessary to make it mor clear. |
| Ericsson | Do not agree. It should be discussed first what this value indicates, absolute or relative TA. |
| HW/HiSi | Agree, but we are also fine with other ranges. Additionally, we think that the PDC procedure and the normal TA procedure should be de-coupled and this should not be based on the the normal TA command. |

Second round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.1.3.2-1: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, an absolute TA command is indicated for PDC with the enhanced TA command indication granularity.

| Company | View |
| --- | --- |
| Feature lead | If relative TA command is indicated, there will be two TA loops as raised by OPPO and discussed in RAN1#106bis-e meeting. OPPO has concern that it will increase the UE complexity. Compared to relative TA command, two TA loops can be avoided by indicating absolute TA command directly. With absolute TA command indication, it is better not to reuse the IAB Timing Delta MAC CE. |
| Samsung | We can live with the proposal. |
| Nokia, NSB | Would be OK, but we do not see any issue in signalling a relative TA for PDC of which the UE applies on top of the regular TA. Additionally, a relative indication could save redundant signalling as an absolute TA is anyway signalled to the UE for TA. |
| ZTE | We support this proposal. |
| OPPO | Disagree. The absolute TA has NTA=TA∙16∙64Tc≥0 given TA≥0 for absolute TAC; while relative TA has NTA,new=NTA,old+TA-31∙16∙64Tc and allows NTA,new being negative. If PDC only supports absolute PDC, it means the PDC only allows non-negative NTA; and because the intention is to have a single TA loop in UE, we see a potential impact to the non-PDC when the TA loop switches from PDC moment to non-PDC moment, which must leave non-PDC UL-Tx with a non-negative NTA and seems a conflict to RAN1 agreement “The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure”. [Feature lead]: Yes since you have concern on two TA loops, we tried to say here let’s just indicate the absolute TA directly. I don’t think there is impact to the non-PDC, since this absolute TA will not be applied to any uplink signal/channel transmission, it is just used to calculate the PDC. Note that even we have PDC specific SRS, there is no need to use this absolute TA for the SRS transmission according the principle. |
| HW/HiSi | Agree |
| vivo | We are fine with the proposal. |

Proposal 2.1.3.2-2: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the TA command is signaled by MAC CE.

| Company | View |
| --- | --- |
| Feature lead | It seems all companies are ok to use MAC CE to indicate the enhanced TA command, except that Nokia prefers RRC signaling. @ Nokia Can you explain more why RRC signal is preferred than MAC CE here? |
| Samsung | Support. |
| Nokia, NSB | As we see it, PDC is not a time critical process and hence we do not see any need to add an additional MAC CE when it is not strictly needed, if this is anyhow separate reporting (from normal TA reporting). But if all companies think we should use MAC CE, we would not be blocking progress here. |
| ZTE | We support this proposal. |
| HW/HiSi | Agree |
| vivo | We are fine with the proposal. |

Proposal 2.1.3.2-3: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the range of absolute TA command indicated with the enhanced TA command indication granularity at least cover 0 to 16384Tc.

- Whether larger range is supported or not is up to RAN2 based on the detailed signaling design

| Company | View |
| --- | --- |
| Samsung | Agree. |
| Nokia, NSB | Disagree on the range as mentioned earlier. But we are OK to leave this for RAN2 to decide overall (not just if larger, but also if a smaller range is supported). [Feature lead]: Leave it to RAN2 is fine also I think. |
| ZTE | We support this proposal. |
| OPPO | Disagree to using absolute TAC only (please see our comment for Proposal 2.1.3.2-1). In addition, the TAC range should be determined by RAN4 as a routine procedure in the past. [Feature lead]: Please see my replies to your comments above. |
| HW/HiSi | Agree |
| vivo | We are fine with the proposal. |
| Qualcomm | Support |

2.1.3.3 Third round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.1.3.2-1: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, an absolute TA command is indicated for PDC with the enhanced TA command indication granularity.

| Company | View |
| --- | --- |
| Feature lead | @OPPO Please check my replies to your comments above. Hopefully this can be acceptable for you. |

Proposal 2.1.3.2-2: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the TA command is signaled by MAC CE.

| Company | View |
| --- | --- |
| Feature lead | It seems this proposal is agreeable to companies. |

Revised Proposal 2.1.3.2-3: If enhanced TA-based PDC with reduced Te and enhanced TA command indication granularity is supported, the range of absolute TA command indicated with the enhanced TA command indication granularity is up to RAN2.

| Company | View |
| --- | --- |

Conclusion for enhanced TA-based PDC

Per the working assumption achieved in Tuesday GTW week 2, enhanced TA-based PDC is precluded for Rel-17 PDC. Therefore, the corresponding observations/conclusions/proposals will not be discussed again in Rel-17.

Working Assumption

For Rel-17

- Support RTT-based PDC method

- Support PDC method based on legacy TA-based mechanism

  - No RAN1/RAN4 specification impact expected

RTT based propagation delay compensation

This section will discuss some key issues for RTT-based propagation delay compensation.

Evaluation on the achievable time synchronization accuracy for RTT-based PDC

Based on inputs from companies in RAN1#107-e meeting, the total error results are summarized below:

Table Overall synchronization error summary for RTT-based PDC

| Source | overall synchronization error |  | Note |
| --- | --- | --- | --- |
|  | 15 kHz | 30 kHz |  |
| Ericsson (R1-2111191) | 251ns | 158ns | errorUE, DL, RX=116ns for 15kHz and 58ns for 30kHz errorgNB,RxTxDiff=63ns for 15kHz and 21ns for 30kHz errorUE,RxTxDiff=70ns for 15kHz and 44ns for 30kHz errorRxTxDiff_ indicate=8ns for 15kHz and 4ns for 30kHz 24 PRB for RS Note: Margin not considered |
| Huawei (R1-2111926) | 262ns+12Y+Tc | 160ns+12Y+Tc | 24RB for DL RS, 44 RB for UL RS, 15 kHz; 24RB for DL RS, 48 RB for UL RS, 30 kHz; SINR -13 dB; errorRxTxDiff_ indicate=8ns |
|  | 118ns+12Y+Tc | 93ns+12Y+Tc | 104RB for DL RS, 176 RB for UL RS, 15 kHz; 132B for DL RS, 176 RB for UL RS, 30 kHz; SINR -13 dB; errorRxTxDiff_ indicate=8ns |
| ZTE (R1-2110917) | 116.75 | 123.25 | errorUE,RxTxDiff=35 ns for 15kHz and 43 ns for 30kHz errorgNB,RxTxDiff= 16 ns for 15kHz and 19 ns for 30kHz errorRxTxDiff_ indicate=0.5ns 20 MHz for RS |
| Nokia (R1-2111141) | 250.723ns | 157.228ns | Minimum BW reference signals errorgNB,RxTxDiff = 122Tc (24 ≤ BW ≤ 40 RBs) (15 kHz) errorUE,RxTxDiff = 137Tc (24 RBs) (15 kHz) errorgNB,RxTxDiff = 32Tc (48 ≤ BW ≤ 84 RBs) (30 kHz) errorUE,RxTxDiff = 87Tc (24 RBs) (30 kHz) errorUE,DL,RX = 0.5/(N_PRB*12*SCS) (24 RBs) errorRxTxDiffindication= ½ *32Tc (k=5) Note: Margin not considered |
|  | 110.63ns | 93.414ns | errorgNB,RxTxDiff = 62Tc (44 ≤ BW ≤ 84 RBs) (15 kHz) errorUE,RxTxDiff = 62Tc (104 RBs) (15 kHz) rrorgNB,RxTxDiff = 62Tc (48 ≤ BW ≤ 84 RBs) (30 kHz) errorUE,RxTxDiff = 62Tc (132 RBs) (30 kHz) errorUE,DL,RX = 0.5/(N_PRB*12*SCS) (276 RBs) Note: Margin not considered |
| CATT (R1-2111251) | 216.25ns | 172.25ns | errorBS, DL, TX=32.5ns errorRxTxDiff_ indicate=16ns errorUE, DL,RX = 100ns 24 PRB for RS for both 15 kHz and 30 kHz |
| Samsung (R1-2111733) | 115 ns | 94 ns | 132 PRB/176 PRB for DL/UL reference signal |
| Intel (R1-2111492) | 239ns | 203ns | 24 ≤ BW ≤ 40 for RS for 15 kHz; 48 ≤ BW ≤ 84 for 30 kHz; |
|  | 193 ns | 186 ns | 176 ≤ BW for RS for 15 kHz; 176 ≤ BW for 30 kHz; |
| OPPO (R1-2111344) | >275 | 184.9+δ+Y2 | 24 RB for RS Note: The value of eUE,RxTxDiff and egNB,RxTxDiff are different from the agreed ones |
|  | 141.7+δ+Y2 | 144+δ+Y2 | 106 RB for 15 kHz, 51 RB for RS for 30 kHz Note: The value of eUE,RxTxDiff and egNB,RxTxDiff are different from the agreed ones |
| Qualcomm (R1-2112212) | 245 ns |  | errorgNB,RxTxDiff=60 ns errorUE, RxTxDiff=92 ns errorRxTxDiff_ indicate=8ns errorUE,DL,RX = 100ns errorBS,UL, RX = 100ns |
| Note: |  |  |  |

First round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed observation 2.2.1.1-1: RTT-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than 24 PRB and the bandwidth for UL RS is larger than 48 PRBs.

| Company | View |
| --- | --- |
| Nokia, NSB | Agree |
| ZTE | We agree with this observation. |
| OPPO | Agree if the error due to inconsistent RTT measurement is not counted; disagree otherwise. [Feature lead]: The inconsistent RTT measurement will be avoided or solved as discussed in section 2.2.6. A quick check: Without counting inconsistent RTT measurement error, the total error is 65+100+120.5∙10912*24∙30k+0.5∙10912*48∙30k+margin=208.4+margin/2 The total error would fail 275ns budget if the inconsistent RTT measurement error (5.5*64Tc/2) is added in. Please note: Per RAN1 #106e agreement, the note of “FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing” is still applicable to Alt-1 formula. |
| Samsung | We think the observations doesn’t considering the margin. We would like to make it clearer. [Feature lead]: For 30 kHz, you could see that the room leaving to margin is very large as shown in the table above, which means even with the margin the performance can still meet the budget. |
| vivo | If my understanding is correct, the margin is not considered in the proposal. [Feature lead]: please see my reply to Samsung above. |
| Qualcomm | Agree. |
| Intel | Agree |
| Ericsson | Agree in principle. Editorial: “larger than or equal to 24 PRB and the bandwidth for UL RS is larger than or equal to 48 PRBs” [Feature lead]: Will update accordingly. |
| HW/HiSi | Agree |

Proposed observation 2.2.1.1-2: enhanced TA-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than 104 PRB and the bandwidth for UL RS is larger than 44 PRBs.

| Company | View |
| --- | --- |
| Feature lead | Only the values that can leave sufficient room for margins are considered here, thus values like ~ 250 ns under 24 PRBs are not considered, since it is not clear yet whether it can meet the budget or not if margins Y and delta are considered. |
| Nokia, NSB | Agree |
| ZTE | We agree with this observation. |
| OPPO | Similar as above. Agree if the error due to inconsistent RTT measurement is not counted; disagree otherwise. [Feature lead]: The inconsistent RTT measurement will be avoided or solved as discussed in section 2.2.6. |
| Samsung | We think the observations doesn’t considering the margin. We would like to make it clearer. [Feature lead]: As pointed above, only values that can leave sufficient room for margins are considered here, thus we don’t need to describe margin here again. If we want to say without margin, then even 24 RB case can be claimed as meet the budget. |
| Qualcomm | Agree. |
| Intel | The beginning of the proposal has a copy-paste error from TA-based PDC. Agree in principle. [Feature lead]: Thanks, yes it is a typo. Will update later. |
| Ericsson | Agree with the intention, but do not agree that DL RS >=104 PRB is needed. Most companies used 24 PRB in calculation. Even in Hauwei calculation, with 24 PRB for DL RS, 262 ns (without RAN4 margin) satisfies the requirement. We understand that RAN4 margin is not included in the calculation, RAN4 margin was not considered in neither TA-based nor RTT-based. Suggest changing DL RS to >= 24 PRB. Need to add “or equal to” [Feature lead]: As pointed above, only values that can leave sufficient room for margins are considered here. Otherwise, you could expect that companies will say that 15 kHz cannot meet the budget either due to the margin. |
| HW/HiSi | Agree |

Proposed observation 2.2.1.1-3: To achieve the same overall synchronization error, RTT-based PDC requires smaller bandwidth for DL RS and/or UL RS, compared to enhanced TA-based PDC.

| Company | View |
| --- | --- |
| Nokia, NSB | Agree Based on our analysis that when we use the same UL and DL bandwidth that the enhanced TA method requires to meet the budget (max DL BW RS and ~9MHz UL RS), the RTT method is 164.4ns and 181.6ns more accurate than enhanced TA. This can be directly be translated to a bandwidth reduction and significant less RS overhead. |
| ZTE | We agree with this observation considering that Alt 1 is used for evaluation. |
| OPPO | Agree if the error due to inconsistent RTT measurement is not counted; disagree otherwise. [Feature lead]: The inconsistent RTT measurement will be avoided or solved as discussed in section 2.2.6. |
| Samsung | We think the observations doesn’t considering the margin. We would like to make it clearer. [Feature lead]: Please check my replies to your comment in the above two observations. . |
| vivo | In my understanding, the margin is not considered for RTT-based PDC. |
| Qualcomm | Agree. |
| Intel | Agree |
| Ericsson | Agree |
| HW/HiSi | Agree |

Second round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Revised Proposed observation 2.2.1.1-1: RTT-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than or equal to 24 PRB and the bandwidth for UL RS is larger than or equal to 48 PRBs.

| Company | View |
| --- | --- |
| Samsung | Can live with this. |
| Nokia/NSB | Support |
| ZTE | We are fine with this observation. |
| Ericsson | Support |
| OPPO | Although FL mentioned “The inconsistent RTT measurement will be avoided or solved as discussed in section 2.2.6”, we do not see the discussion in section 2.2.6 can ensure that target. [Feature lead]: Please check the discussion and replies in section 2.2.6, I think anyway this inconsistent RTT measurement issue will be avoided. |
| HW/HiSi | Agree |
| vivo | We can live with the observation. |
| Qualcomm | Agree |

Revised Proposed observation 2.2.1.1-2: RTT-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than or equal to 104 PRB and the bandwidth for UL RS is larger than or equal to 44 PRBs.

| Company | View |
| --- | --- |
| Samsung | Can live with this. |
| Nokia/NSB | Support |
| ZTE | We are fine with this observation. |
| Ericsson | Do not support The DL RS for 15 kHz in RAN4 table (38.133 V17.3.0, Table 10.1.25.2-2) is 24 PRB, which are used by most companies and showed that the error budget is met. The 104 PRB seems to be based on Nokia (R1-2111141), which is shown to achieve much reduced error of 110.63ns. We do not think this data point alone justifies increasing the DL RS by 4.3 times. We can be OK if 104 PRB is changed to 24 PRB. [Feature lead]: If you look at the table in section 2.2.1, you could see that the value for 15 kHz with 24 RBs for DL RS is very close to the budget 275 ns, while it is obvious the margin of Y and delta is not considered in those values, e.g. in your contribution. As I explained before, only the values that leave sufficient room for the margin are considered here. In addition, the observation didn’t preclude the case that it is possible the budget can be met under other conditions, here we only see “at least”. |
| OPPO | Similar comment as above. |
| vivo | We can live with the observation. |

Proposed observation 2.2.1.1-3: To achieve the same overall synchronization error, RTT-based PDC requires smaller bandwidth for DL RS and/or UL RS, compared to enhanced TA-based PDC.

| Company | View |
| --- | --- |
| Samsung | Can live with this. |
| Nokia/NSB | Support |
| ZTE | We are fine with this observation. |
| Ericsson | Support |
| OPPO | We do not think the comparison between TA-based PDC and RTT-based PDC is an apple-to-apple comparison. TA-based evaluation in section 2.1 uses an error upper bound of Te, which has a super large margin according to the following RAN4 discussion in a WF paper (R4-2120335): Based on the company views it may be possible to agree on some conservative estimate using 1/(2*DL_RS_BW) and using 15KHz and 30 KHz SCS. Using the above new conditions and the maximum RS BW for DL RS, the enhanced Te can be reduced to at most: 1) 262 ns (252ns Te margin +10ns DL detection error with 50MHz DL RS BW) for 15 kHz SCS, and 2) 196ns (191ns Te margin + 5ns DL detection error with 100MHz DL RS BW) for 30kHz SCS. As one can see, the Te margin as understood in RAN4 can consume almost more than 95% of total error, or about 200ns or more in Te. In comparison, RTT-based evaluation assumes that the margins (δ, Y) in gNB/UE Rx-Tx time difference accuracy are comparatively no larger than the DL/UL Rx timing error. Both margins (δ, Y) are even not counted in RAN1 evaluations of total errors. To our understanding, when RAN4 defined Te at the beginning of LTE/NR life cycle, RAN4 did not target Te as worst synchronization performance for certain reference signal; instead, RAN4 intended to make Te corresponding to the worst synchronization condition at which the SNR and traffic channel start go bad to influence demodulation performance. That is one of reasons why Te margin can be so large. In contrast, UE Rx-Tx time difference accuracy is RS-specific performance. Therefore the comparison between TA-based PDC performance based on Te and RTT-based PDC performance based on PRS may mean nothing. [Feature lead]: The problem is now that RAN4 was not able to provide us a reduced Te value, thus it is not clear yet whether RAN4 can further reduce the Te margin part. I think the comparison here still meaningful, RAN1 can only compare based on what we can get now and try to choose a solution for Rel-17 PDC. |

2.2.1.3 Third round email discussion

Feature lead: The original motivation to discuss these observations is to help do the down-selection between enhanced TA-based PDC and RTT-based PDC. Since now we already made the decision on the down-selection during Tuesday GTW, there is no need to discuss these observations again, because these observations anyway may not be helpful from real deployment perspective, since at least some of the error components, e.g. detection error, is achieved by theoretical estimate. Therefore, I will not set up the follow-up discussions for these observations.

Revised Proposed observation 2.2.1.1-1: RTT-based PDC under 30 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than or equal to 24 PRB and the bandwidth for UL RS is larger than or equal to 48 PRBs.

| Company | View |
| --- | --- |
| Feature lead | @OPPO Please check my replies to your comments above. Hopefully this can be acceptable for you since it is fine for all companies. |

Revised Proposed observation 2.2.1.1-2: RTT-based PDC under 15 kHz can meet the upper bound (i.e. ±275 ns) of Uu interface synchronicity error budget for control-to-control scenario, at least when the bandwidth for DL RS is larger than or equal to 104 PRB and the bandwidth for UL RS is larger than or equal to 44 PRBs.

| Company | View |
| --- | --- |
| Feature lead | @Ericsson @OPPO Please check my replies to your comments above section. Hopefully this can be acceptable for you since it is fine for all companies. |

Proposed observation 2.2.1.1-3: To achieve the same overall synchronization error, RTT-based PDC requires smaller bandwidth for DL RS and/or UL RS, compared to enhanced TA-based PDC.

| Company | View |
| --- | --- |
| Feature lead | @OPPO Please check my replies to your comments above. Hopefully this can be acceptable for you since it is fine for all companies. |

Reporting granularity of the Rx-Tx measurement

In RAN1#106 and RAN1#106bis-e meeting, the following was agreed for the granularity.

Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID Agreement If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least: - UE Rx-Tx time difference at a given granularity Agreement If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5. - FFS the value of k - FFS the reporting range of Rx-Tx time difference measurement for PDC

In this meeting, companies share views below.

Nokia (R1-2111141) In our view, we don’t need all the low granularity available for positioning since the accuracy level for PDC does not need to be as high. It is clear that the error contribution in PDC given by granularity with k=5, i.e. (±8.144 ns)/2, corresponds to less than 1.5% of the control-to-control budget of ±275ns. So, to simplify specification work in terms of granularity signaling, a single granularity can be supported for PDC, such as 32Tc. Observation 2.2.3: For a granularity of 2k*Tc with k=5, the contribution of the indication error corresponds to less than 1.5% of the control-to-control budget. Proposal 2.2.1: A single granularity for the Rx-Tx measurement report, such as 32Tc, is supported for RTT-based PDC. No specification of report granularity configuration should be needed.

ZTE (R1-2110917) In our understanding, if the RTT-based PDC is supported, it can also be used for the other scenario with lower requirement, e.g., smart grid, where a larger granularity can also work. Considering that multiple granularity can provide more flexibility and forward compatibility, we think all the granularity defined in positioning WI can be supported if RTT-based PDC is supported. In this case, the network can choose suitable granularity for measurement report delivery or configure the granularity for the UE to report the measurement result. Proposal 3: All the granularity for the Rx-Tx time difference measurement reporting defined in TS38.133 should be supported if RTT-based PDC is supported.

vivo (R1- 2111008) Considering PDC for URLLC support two use cases, i.e., smart grid and control to control use case, various SCS and/or RS bandwidths can be configured for these use cases. On the other hand, the operation frequency range may also be different, e.g., on FR 1 or FR 2. Supporting multiple reporting granularity can provide the better flexibility for the different scenarios and configurations. Therefore, supporting all those values is preferred. The reporting range of Rx-Tx time difference measurement for PDC reuse defined in positioning WI. Proposal 2: If RTT-based propagation delay compensation is supported, Rx-Tx time difference is reported with granularity 2k*Tc, all k values are supported, e.g., k is an integer satisfying 0<=k<=5.

CATT (R1-2111251) If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k *Tc, where k is an integer satisfying 0<=k<=5. The value of k needs to be determined. The selection of k is a trade-off between accuracy and reporting overhead. It is preferable that the value of k can be configurable. Proposal 1: The value of k is configurable.

Intel (R1-2111492) - Indication granularity for time difference reporting - Reuse positioning numbers. According to analysis, there is no need for very tight granularity numbers, since the accuracy is mostly limited by the measurements. From the candidate values 0 <= k <= 5, we think a single value can be picked, if the measurement is carried by PHY/MAC layer indication, or multiple values can be configurable for RRC based indication. Proposal 2 - For RTT-based scheme, - Rx-Tx time difference reporting granularity factor k is configurable from 0 to 5. - The Rx-Tx time difference reporting range may be reused from positioning

Huawei (R1-2111926) Observation 3: If RTT-based propagation delay compensation is supported, the granularity of 32 Tc for reporting Rx-Tx time difference is sufficient. Proposal 1: If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc and k=5.

Feature lead: Based on the contribution, some companies prefer configurable k while some other companies think that fixing it to one value is sufficient. From standard effort perspective, if fixing to one value is sufficient, it would be always good, e.g. we don't need to discuss how to signal the selected k value. Based on the inputs from companies who prefer configurable, seems the main motivation is for flexibility. If it works, at this late stage it would be recommended to fix to one value. In addition, according to the current RAN4 definition for positioning, it seems k = 5 is the one that for sure will be applied to both FR1 and FR2. Therefore, let’s make the tentative proposal to fix it to 5. If any problem identified, we can change to configurable values for sure.

First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.2.2.1-1: If RTT-based PDC is supported, a single granularity 32Tc (i.e. k=5) is supported for Rx-Tx measurement report.

| Company | View |
| --- | --- |
| Nokia, NSB | Support |
| ZTE | We don’t have strong views on this. Just slightly prefer multiple granularity. We also can accept this proposal if it is the majority view. |
| OPPO | Support |
| vivo | We are open for a single or multiple granularities. If majority of companies support this proposal, we can live with it. |
| Qualcomm | Support |
| Intel | Agree. Analysis shows that this error components is not the dominant one. |
| Ericsson | Support |
| HW/HiSi | Agree. |

Feature lead: Proposal 2.2.2.1-1 is stable based on inputs above.

Reporting range of the Rx-Tx measurement

Nokia (R1-2111141) From TS 38.133, there is the following gNB (and equivalently for the UE) Rx-Tx time difference measurement report mapping specified for the highest granularity (k=5): Table 13.2.1-5: gNB Rx-Tx time difference measurement report mapping for reporting resolution of 32Tc (k=5) Reported Value; Measured Quantity Value; Unit. RX-TX_0000; -985024 > RX-TX; Tc. RX-TX_0001; -985024  RX-TX < -984992; Tc. RX-TX_0002; -984992  RX-TX < -984960; Tc. ; ; …. RX-TX_30781; -64  RX-TX < -32; Tc. RX-TX_30782; -32  RX-TX  0; Tc. RX-TX_30783; 0 < RX-TX  32; Tc. RX-TX_30784; 32 < RX-TX  64; Tc. RX-TX_30785; 64 < RX-TX  96; Tc. …; …; …. RX-TX_61564; 984992 < RX-TX  985024; Tc. RX-TX_61565; 985024 < RX-TX; Tc. From the table it can be seen that the Rx-Tx measurement range is equivalent to approximately -0.5ms to 0.5ms, and the report resolution is uniform across the range. Such reporting range is relevant for positioning because of Rx-Tx measurement reports made towards neighbour cells, where there is no timing alignment between DL and UL subframe boundaries. However, for time synchronization we should only consider measurement for the serving cell. The Rx-Tx measurements from UE and serving cell are illustrated below. Figure 1 - Rx-Tx between UE and serving cell In case of UE Rx-Tx measurement report (if gNB-side PD estimation is assumed), it is our understanding that values can only be positive, considering that transmissions to the serving cell are time aligned (like UE Rx-Tx time difference measurement report defined for LTE in 36.133). However, the maximum range would depend on the assumption of maximum distance to serving cell. Observation 2.2.4: For UE Rx-Tx report, in case gNB-side PD estimation is supported, the measurement report value range should only be positive and designed for the maximum supported distance to the serving cell. On the other hand, for the case that UE side PD estimation is supported, the gNB Rx-Tx measurement report from serving cell to UE can consist of negative and positive values due to potential UE transmission timing error and gNB Rx measurement error. As UL transmission timing is controlled by TA, the range of values should not be large, i.e., measured DL and UL subframe boundaries are close in time. In practice, such timing difference should not be more that the cyclic prefix length. Observation 2.2.5: For gNB Rx-Tx report, in case UE side PD estimation is supported, the measurement report value range can be negative and positive. A smaller range, e.g. equivalent to cyclic prefix length, should be sufficient since UL subframe transmissions received in gNB are time aligned with DL subframe. Nevertheless, we can only conclude that a smaller report range should be supported for PDC which helps on reducing the overhead, but we think it is out of RAN1 scope to define the exact range. As in RAN1#99 discussions from positioning it was agreed the following: Agreement: - The reporting granularity for the UE/gNB timing measurements (DL RSTD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) is defined as T=Tc2k, where k is a configuration parameter with a minimum value of at most 0. - Note: RAN4 can determine if -1 can be a minimum value - RAN1 assumes that the details of the reporting granularity and ranges for the UE/gNB timing measurements (DL RTSD, the UE Rx-Tx time difference, UL RTOA, gNB Rx-Tx time difference) will be determined by RAN4, including the potential relation of the parameter k to DL PRS bandwidth.. Based on that, we suggest following the same approach here, i.e., that the exact range should be defined by RAN4. Proposal 2.2.2: RAN1 to conclude that a smaller reporting range for Rx-Tx measurement can be defined for RTT-based PDC, since only serving cell measurement is considered for PDC. The exact range should be determined by RAN4.

CATT (R1-2111251) The reporting range of gNB Rx-Tx time difference, as defined in Clause 5.2.3 of TS 38.215, is defined from -985024Tc to +985024Tc. We think there is no need to change it for PDC. Proposal 2: The reporting range of Rx-Tx time difference measurement defined in Rel-16 is reused for PDC.

Intel (R1-2111492) Proposal 2 - For RTT-based scheme, - Rx-Tx time difference reporting granularity factor k is configurable from 0 to 5. - The Rx-Tx time difference reporting range may be reused from positioning

Huawei (R1-2111926) Regarding the second FFS from the above mentioned agreement, in positioning the reporting range of Rx-Tx time difference measurement is from -985024Tc to 985024Tc, i.e. from -0.5ms to +0.5ms with the resolution step of 2kTc. It is expected that in PDC we can reuse this Rx-Tx time difference definition as much as possible to reduce the spec impact, so we think the reporting range of Rx-Tx time difference measurement for PDC can be the same as shown in Table 10.1.25.3.1-6 for k=5. Proposal 2: If RTT-based propagation delay compensation is supported for PDC, the reporting range of Rx-Tx time difference measurement is the same as table 10.1.25.3.1-6 in TS 38.133.

Feature lead: Most companies prefer to reuse the reporting range defined for positioning. Nokia provide some detailed analysis on both gNB Rx-Tx time difference report and UE Rx-Tx time difference report. Views from companies are needed on the proposal and observations from Nokia. If consensus cannot be achieved, I think we should leave it to RAN4, since RAN4 would be the group to determine the detailed values anyway.

First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 2.2.3.1-1: Do you agree that the measurement report value range should only be positive for UE Rx-Tx time difference if the propagation delay compensation is performed at the gNB side?

| Company | View |
| --- | --- |
| Nokia, NSB | Agree We do not see the need for resuing the existing range, as it is designed to cover neighbour cells with large DL frame offsets compared to the serving cell. Further, as this applies only for UE in RRC_CONNECTED, the UE will anyway be applying TA and hence the range can be restricted to be only positive. The exact range can be left for RAN4 to specify. |
| ZTE | In theory, it could be positive and 0 as well. For example, TA is 0 in the case of small cell. But I am not sure if could be negative in actual deployment considering the error components discussed in TA-based PDC when TA is 0. To be safety, it can be discussed by RAN4. |
| OPPO | We think such restriction is not necessary. There is no guarantee that the UE-side Rx-Tx time difference must be one-way positive. Depending on how gNB sets its local UL-Rx timing as well as the propagation delay, this time difference could be negative. |
| Samsung | We think this can be defined in RAN 4. |
| vivo | We think the range can be determined in RAN4. |
| Qualcomm | We share the same view as Samsung and vivo, just leave the range for RAN4. |
| Ericsson | Agree that it’s >=0, similar to NTA value. Suggest leaving the exact range to RAN4. |
| HW/HiSI | We think for simplicity that the defined range should be re-used. We are not sure about only having positive values but would like to hear more views: Assuming that DL PRS could be in subframe #i and the UL SRS may be in a subframe either ahead or after #i, if our understanding about the Rx-Tx measurement is correct, then it seems possible that both positive and negative values could occur. |

Question 2.2.3.1-2: Do you agree that the measurement report value range can be negative and positive for gNB Rx-Tx time difference if the propagation delay compensation is performed at the UE side? If your answer is yes, do you agree that a smaller value range, e.g. equivalent to cyclic prefix length, should be sufficient?

| Company | View |
| --- | --- |
| Nokia, NSB | Agree, It should be sufficient to set the range in the order of the cyclic prefix length, as the UE is already applying timing advance to offset its UL transmission timing. |
| ZTE | We think the measurement report value range can be negative and positive. For the value range, it should be discussed by RAN4. |
| OPPO | Agree the value range can be +/-. Do not agree the value range is restricted to CP length. gNB is allowed to use a larger Rx-Tx time difference, e.g. in FDD. |
| Samsung | This can be discussed in Ran 4 |
| vivo | We think this issue can be discussed in RAN4. |
| Qualcomm | Leave it for RAN4. |
| Ericsson | Agree that the measurement can be +/- on gNB side. Suggest leaving the exact range to RAN4. |
| HW/HiSi | Agree that the value range can be positive and negative. Regarding discussing a smaller value range, we are not so sure about it also based on our comment to Question 2.2.3.1-1. Wouldn’t it be simpler to just re-use the existing range without optimizing the RRC signaling overhead, this already covers the range equivalent to the cyclic prefix? |

Question 2.2.3.1-3: Do you agree that we can leave the reporting range to RAN4 if we cannot achieve any consensus for the above two questions?

| Company | View |
| --- | --- |
| Nokia, NSB | Agree |
| ZTE | Yes, it can be discussed by RAN4. |
| OPPO | Agree |
| Samsung | Yes |
| vivo | Yes |
| Qualcomm | Yes. |
| Ericsson | Agree |
| HW/HiSi | Agree |

Feature lead: It seems still different views on the first two questions, and several companies mentioned that it should be up to RAN4.

Second round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion: The reporting range of Rx-Tx time difference measurement for RTT-based PDC is up to RAN4.

| Company | View |
| --- | --- |
| Samsung | Agree |
| Nokia/NSB | Agree |
| ZTE | We are fine with this proposal. |
| Ericsson | Agree |
| OPPO | Agree |
| H3C | Agree |

Feature lead: The proposed conclusion is stable.

Detection of Reference Time

Ericsson (R1-2111191) - 3.1 Detection of Reference Time The reference time information (referenceTimeInfo) is provided by gNB via RRC message, where the reference time providing the time at the ending boundary of a system frame, see the quoted text in the Appendix. If referenceTimeInfo field is received in DLInformationTransfer message, referenceSFN field indicates the SFN of PCell. Hence the detection of the reference time depends on SSB detection in the PCell, since SSB provides SFN information. According to 38.211, PSS, SSS and PBCH within an SS/PBCH block are QCL, while different SS/PBCH blocks cannot be assumed QCL. Thus, different SSB in the SSB burst can be transmitted from different TRP. For data communication purpose, it is only required that DL signal from different TRPs arrive at the UE within a CP for coherent detection. However, CP duration is 4.69 µs and 2.34 µs for SCS = 15 kHz and 30 kHz, respectively, see Table 5. Compared with the Uu interface time synchronization accuracy requirement (hundreds of nanoseconds), CP duration is an order of magnitude too large. This means that the timing at different TRPs cannot be assumed to be synchronized to satisfy the tight clock synchronization needs shown in Table 1. The µs -level time difference detected by SSB at different TRP can overwhelm the propagation delay estimation accuracy. Reference time information reception and propagation delay compensation should be performed with reference to a single, identified, transmission point in the PCell. For any UE, propagation delay should be measured from the TRP that the reference time (referenceTimeInfo) is associated with, regardless of the spatial configuration for data communication. At physical layer specifications, TRP is not explicitly described. Instead, the TRP is indirectly identified via the SSB index(es) associated with the TRP. Consequently, it is necessary to clarify which SSB(s) the UE should use to detect SFN, so that gNB and UE know which TRP provides the reference time at the desired SFN boundary. Clarify the TRP that referenceTimeInfo is associated with via its SSB index(es). Additionally, to ensure that the measured propagation delay is for the radio path from the same TRP (e.g., TRP1) used for reference time detection, the DL RS (TRS or PRS) for time synchronization should be QCL-ed with the SSB(s) of the same TRP (e.g., TRP1). Similarly, the SRS should also have the spatial relationship to the same TRP (e.g., TRP1). This is illustrated in Figure 2. Given that the reference time (Tref) is the time at TRP1, both UEa and UEb should measure their propagation delay, Tp,a and Tp,b, from TRP1, so that the clock time at UEa and UEb can be correctly estimated as Tref + Tp,a and Tref + Tp,b, respectively. In contrast, if this is not clearly defined, then UEb may mistakenly measure propagation delay from TRP2, while Tref is the time at TRP1. Here TRP2 refers to the node that UEb uses for data communication. Define spatial property of DL RS (TRS or PRS) and SRS to measure the propagation delay from the TRP that referenceTimeInfo is associated with. Figure 2: A UE measures propagation delay from the TRP that the reference time Tref is associated with, regardless of TRP(s) related configuration for data communication. Table 5. CP duration for different SCS SCS; 15 kHz; 30 kHz. OFDM symbol, duration; 66.67 µs; 33.33 µs. Cyclic prefix Samples; 288; 288. Cyclic prefix duration; 4.69 µs; 2.34 µs.

Feature lead: The issues seems valid. However, whether any specific mechanism is needed to solve the issue is not clear yet. For example, once gNB configure the DL RS and/or UL RS for PDC measurement, then the associated QCL source (i.e. SSB) should be clear also. In this case, the assumption is that UE will detect the associated SSB to get the SFN information for the reference time information. From gNB side, gNB know the associated SSB and will use the corresponding TRP to transmit the reference time information. However, since this issue is brought up the first time, more views from other companies are needed.

First & Second round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 2.2.4.1-1: Do you agree that reference time information reception and propagation delay compensation should be performed with reference to a single, identified, transmission point in the PCell? If your answer is yes, please also provide your view on whether any additional mechanism need to be specified for it, i.e. whether the existing schemes is sufficient or not.

| Company | View |
| --- | --- |
| Nokia, NSB | The issue is not clear to us. As we understand, this can be assumed to be handled by UE and gNB implementation. A reasonably implemented UE would use the SFN frame timing from the same TRP as it received RTI from. The gNB likewise would ensure that UL and DL RS is from the same TRP as of which the UE is connected to (and the gNB knows which one that is) |
| ZTE | We agree that the two procedures should be performed under the same TRP. But we don’t think we should specify something for this because it can be handled by implementation. For example, if the reference time information is delivered by dedicated RRC signaling. Then the same TRP can be used for the signaling transmission in the two procedures. If the reference time information is delivered by SIB9. The UE may receive the SIB from the correct TRP. It should be noted SIB are transmitted in beam sweeping manner and each is associated with a SSB. The UE just receives the SIB and the corresponding reference frame associated with the SSB for its unicast transmission. Then the same TRP can be ensured. |
| OPPO | First, we are a bit confused by rational in R1-2111191. The paper says “if this is not clearly defined, then UEb may mistakenly measure propagation delay from TRP2, while Tref is the time at TRP1”, but there is no SSB/TRS sent from TRP2 in the figure. How could UE make the corresponding PD measurement referencing to TRP2? Secondly, the proposal seems to move ReferenceTimeInfo from “baseband” to “single TRP antenna connector”, we believe this should be justified by RAN2 first because this is not what 38.331 says. We think the issue raised in R1-2111191 could be handled by gNB implementation: the ReferenceTimeInfo and the DL-RS configured for PDC should associate with the same TRP(s), where such association is transparent to UE. |
| Samsung | We think this can be handled by implementation. other than DL-RS, we think gNB should indicate a SRS with a certain beam. There should not have mis-understanding of it. |
| vivo | Is the intention of the proposal to ensure the same beam assumption between reference time information reception and PDC? We think implementation can handle this issue. |
| Qualcomm | More study and discussion is needed. |
| Ericsson | Agree. It seems that companies agree with the principle, i.e., ReferenceTimeInfo, DL RS, SRS are all associated with the same TRP. On the other hand, we are not sure if this can be guaranteed by gNB implementation. Below text from 38.211 section 7.4.3.1 indicates that different SSB index in the SSB burst can be sent from different TRP; only SSB with same index can be assumed to be co-located. “The UE may assume that SS/PBCH blocks transmitted with the same block index on the same center frequency location are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE shall not assume quasi co-location for any other SS/PBCH block transmissions other than what is specified in [5, TS 38.213].” Regarding OPPO question on the figure: The figure intention is: SSB index1 is sent from TRP1, SSB index2 is sent from TRP2. For PDC, UE_b should detect SSB index 1 for timing estimation, even though normal data communication (together with SSB index2) can be via TRP2. [Feature lead]: What do you think about my thinking below for this issue? ============== Once gNB configure the DL RS and/or UL RS for PDC measurement, then the associated QCL source (i.e. SSB) should be clear also. In this case, the assumption is that UE will detect the associated SSB to get the SFN information for the reference time information. From gNB side, gNB know the associated SSB and will use the corresponding TRP to transmit the reference time information. ============== |
| HW/HiSi | Agree with Nokia that it is not clear to use whether it is an issue and it should be handled by implementation. A further question, if this issue would exist, is it also an issue in Rel-16 PDC? |
| Feature lead | The questions are not updated yet. Please companies check the inputs from companies and also please E// check my reply above to see if you agree with it. If you agree with my thinking, then same TRP can be ensured by implementation and no additional thing is needed. |
| Samsung | Agree with FL’s explanation. gNB implementation should solve the issue. |
| Nokia/NSB | Agree with FLÄs assessment & explanation. |
| Ericsson | Not sure what’s meant by gNB implementation. If it means gNB RRC configurations like the ones in existing spec can be borrowed or largely reused, then that’s fine. Otherwise, how is it possible that gNB know which SSB the UE used in detecting SFN? In general, the UE can use any SSB, right? QCL needs to be configured as can be seen in many places in current spec. Thus, we can agree with: “For PDC, RRC configuration ensures that DL RS, UL RS, and reference time are associated with a same TRP via SSB-Index.” [Feature lead]: My thinking on same TRP ensured by implementation is as below: 1. Based on the RRC configurations for DL RS and UL RS for PDC measurement, the associated QCL source (i.e. SSB) should be clear for UE. Then UE will detect the associated SSB of the DL RS/UL RS to get the SFN information for the reference time information. From gNB side, gNB knows the associated SSB of the DL RS/UL RS, and it will use the corresponding TRP to transmit the reference time information. 2. By gNB implementation, gNB will ensure DL RS and UL RS are associated with the same TRP via appropriate configuration. In a word, based on the above two points, by implementations means that even we don’t specify anything in the spec, same TRP can be ensured. If you are worried, probably we can try to see if we can achieve a conclusion to clarify this, as shown in section 2.2.4.2. Examples of existing RRC configuration for QCL (for DL RS) and spatial relation (for UL RS): NZP-CSI-RS-Resource information element NZP-CSI-RS-Resource ::= SEQUENCE { … qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } TCI-StateId information element QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } SRS-Config information element PathlossReferenceRS-Config ::= CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId } SRS-SpatialRelationInfo ::= SEQUENCE { servingCellId ServCellIndex OPTIONAL, -- Need S referenceSignal CHOICE { ssb-Index SSB-Index, csi-RS-Index NZP-CSI-RS-ResourceId, srs SEQUENCE { resourceId SRS-ResourceId, uplinkBWP BWP-Id } } } |

Feature lead: It seems common understanding that the reference time information reception, DL RS and SRS for propagation delay compensation are all associated with the same TRP. However, different views on whether it can be ensured by implementation. In my understanding, it can be ensured by implementation. Companies are encouraged to check the discussion above in the table.

Question 2.2.4.1-2: If your answer to question 2.2.4.1-1 is yes, do you agree to clarify the TRP that referenceTimeInfo is associated with via its SSB index(es)?

| Company | View |
| --- | --- |
| Nokia, NSB | No We do not see the need. |
| ZTE | No, please see our comments for question 2.2.4.1-1. |
| OPPO | Not sure we understand the purpose here. In the current spec, ReferenceTimeInfo is carried in PDSCH, whose DMRS could be associated with a SSB index. Is the purpose here to define a new association? In general, we do not see the need of such new clarification. |
| Samsung | Might need some further discussion. But so far, we don’t see a need. |
| vivo | NO. We do not see the need. |
| Ericsson | Yes. Basically, there should be a way to associate {referenceTimeInfo, DL RS} with a given TRP in PCell. As explained earlier, 38.211 text allows SSB of different index to be sent from different TRP. UE detects referenceTimeInfo by detecting SSB. [Feature lead]: What do you think about my thinking below for this issue? ============== Once gNB configure the DL RS and/or UL RS for PDC measurement, then the associated QCL source (i.e. SSB) should be clear also. In this case, the assumption is that UE will detect the associated SSB to get the SFN information for the reference time information. From gNB side, gNB know the associated SSB and will use the corresponding TRP to transmit the reference time information. ============== |
| HW/HiSi | We do not see the need so far. |
| Feature lead | The questions are not updated yet. Please companies check the inputs from companies and also please E// check my reply above to see if you agree with it. If you agree with my thinking, then same TRP can be ensured by implementation and no additional thing is needed. If you have further comment, please provide it below. |
| Ericsson | It’s not clear to us how QCL can be clear without gNB configuration. If the intention is the RRC configuration similar to existing ones can be largely reused to achieve the purpose, then we are fine. For example, 38.214 existing text defines the QCL types of TRS below. Which SSB the TRS is QCL with is configured via QCL-Info. Assuming that the existing configuration like this continues to apply, then something like this is adequate: “For PDC, RRC configuration ensures that reference info is associated with the same TRP that transmits the SS/PBCH block, where the SS/PBCH block is QCL-ed with TRS.” [Feature lead]: Please check my replies to the above question. 38.214: - 'typeC': {Doppler shift, average delay} - 'typeD': {Spatial Rx parameter} NZP-CSI-RS-Resource information element NZP-CSI-RS-Resource ::= SEQUENCE { … qcl-InfoPeriodicCSI-RS TCI-StateId OPTIONAL, -- Cond Periodic ... } TCI-StateId information element QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb SSB-Index }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... } |

Question 2.2.4.1-3: If your answer to question 2.2.4.1-1 is yes, do you agree to define spatial property of DL RS (TRS or PRS) and SRS to measure the propagation delay from the TRP that referenceTimeInfo is associated with for RTT-based PDC if supported?

| Company | View |
| --- | --- |
| Nokia, NSB | No We do not see the need. |
| ZTE | We are not sure the intention of this proposal. In current specification, for any RS, the spatial relationship can be configured. So the network may configure the spatial configuration for the RS if needed. |
| OPPO | No. In general, gNB should have the control to make any DL-RS configured to the UE for PDC purpose to have appropriate timing relation with the transmission of ReferenceTimeInfo. |
| Ericsson | Yes. The intention is that the ReferenceTimeInfo, DL RS, SRS are all associated with the same TRP, so that the propagation delay is measured from the point reference time is for. Otherwise, the propagation delay is measured for a wrong radio path, and all the error calculation is invalid. @ZTE, @OPPO: We are fine if mechanisms are shown that that gNB can configure like this. [Feature lead]: What do you think about my thinking below for this issue? ============== Once gNB configure the DL RS and/or UL RS for PDC measurement, then the associated QCL source (i.e. SSB) should be clear also. In this case, the assumption is that UE will detect the associated SSB to get the SFN information for the reference time information. From gNB side, gNB know the associated SSB and will use the corresponding TRP to transmit the reference time information. ============== |
| Feature lead | The questions are not updated yet. Please companies check the inputs from companies and also please E// check my reply above to see if you agree with it. If you agree with my thinking, then same TRP can be ensured by implementation and no additional thing is needed. If you have further comment, please provide it below. |
| Ericsson | As shown above, relevant configurations are listed for configuring TRS (QCL-Info) and PRS (PathlossReferenceRS-Config, SRS-SpatialRelationInfo). We can agree with moderator view, if the intention is that existing gNB configurations are borrowed to fulfil the needs of PDC. We support proposals like these: - For PDC, RRC configuration ensures that DL RS, UL RS, and reference time are associated with a same TRP via SSB-Index. - For PDC, RRC configuration ensures that reference info is associated with the same TRP that transmits the SS/PBCH block, where the SS/PBCH block is QCL-ed with TRS. - For PDC, RRC configuration ensures that SRS uses the TRS and/or the SS/PBCH as reference RS, where the SS/PBCH block is QCL-ed with TRS. [Feature lead]: Please check my replies to the above question. |

Third round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Proposed conclusion: For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP.

- Note: No specification impact expected

| Company | View |
| --- | --- |
| Feature lead | Please do go back check the discussions in section 2.2.4.1 before providing your views here. |
| Nokia/NSB | Support the conclusion. We agree with the FL assessment. |
| ZTE | We are fine with this conclusion. |
| Intel | Agree, otherwise there is not much sense in the resulting RTT measurement |
| OPPO | Agree. |
| Ericsson | Agree in principle. Following FL explanation, we understand the intention is, gNB will configure properly to achieve this effect. For example, gNB configures the same ‘SSB-Index’ for both (a) QCL reference RS for TRS/PRS, and (b) spatialReation reference RS for SRS. With the above, we agree that this can be achieved via reusing RRC signaling mechanism. Thus, we suggest to change the note as follows. If RAN2 thinks something need to be clarified in 38.331, then that’s still allowed. - Note: No RAN1 specification impact is expected [Feature lead]: Ok, I think it is fine to update the note as it is. |
| Samsung | Agree |
| HW/HiSi | Agree |

TRS/PRS and SRS configuration

Ericsson (R1-2111191) - 3.2 TRS configuration TRS is CSI-RS for tracking. Currently TRS can be periodic or aperiodic, where the configuration of aperiodic TRS depends on that of periodic TRS. Aperiodic TRS and periodic TRS resource have the same bandwidth (with same RB location) and the aperiodic TRS being configured with qcl-Type set to 'typeA' and 'typeD', where applicable, with the periodic CSI-RS resources. For time synchronization purpose, it is beneficial to also support semi-persistent TRS. This allows the gNB to trigger periodic TRS transmission when the TSN UE needs to perform propagation delay compensation, and disable it with DCI when the UE has finished updating the clock time. In terms of bandwidth (RB location) and qcl-Type, the semi-persistent TRS can share the same as periodic and aperiodic TRS. Introduce semi-persistent TRS for propagation delay compensation. - 3.3 PRS configuration Currently, for positioning purpose, the configuration parameters of PRS are sent from LMF. When used for time synchronization purpose, PRS configuration should be introduced in RRC signalling, so that the parameters are sent from the gNB to the UE. Only configuration for serving cell PRS is needed, and those of neighbor cells are not needed. Furthermore, the critical parameters are those for sequence generation and time-frequency resources are needed. These parameters include (see TS 38.211): - For pseudo random sequence generation: dl-PRS-SequenceID; - For mapping to physical resources: dl-PRSResourceSymbolOffset, dl-PRS-NumSymbols, dl-PRS-CombSizeN, dl-PRSCombSizeN-AndReOffset, dl-PRS-ResourceBandwidth, dl-PRS-StartPRB, dl-PRS-PointA; - For mapping to slots in a downlink PRS resource set: dl-PRS-Periodicity-and- - ResourceSetSlotOffset, dl-PRSResourceSlotOffset, dl-PRSResourceRepetitionFactor, dl-PRS-ResourceTimeGap; - For defining the quasi co-location information of the DL PRS resource with other reference signals: dl-PRS-QCL-Info; Regarding PRS muting, this is for coordinating a UE’s PRS reception from different cells, including non-serving cells. Thus, muting related parameters (e.g., dl-PRS-MutingOption1, dl-PRS-MutingOption2) are not needed for propagation delay compensation with the serving cell. For QCL (Quasi Co-Location) information of PRS, the reference signal can be: - SS/PBCH block, where the SSB index(es) is associated with the TRP of referenceTimeInfo; - TRS In summary, an IE should be introduced to provide basic RRC parameters for configuring PRS within the PCell. Introduce RRC parameters for configuring PRS within the PCell. The IE for PRS configuration include parameters for sequence generation, mapping to physical resources, mapping to slots in a downlink PRS resource set, and quasi co-location information. - 3.4 SRS configuration For positioning purposes, SRS configuration is according to higher layer parameter SRS-PosResource. For time synchronization purpose, a separate SRS configuration is needed to provide SRS configuration, for example, SRS-SyncResource. If the higher layer parameter spatialRelationInfoPos is configured, the corresponding reference signal (RS) information can be those used for time synchronization: - SS/PBCH block, where the SSB index(es) is associated with the TRP of referenceTimeInfo; - TRS - PRS Introduce RRC IE for configuration SRS for time synchronization purpose.

Feature lead: Views from companies are needed before making any proposal here.

First round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 2.2.5.1-1: do you agree to introduce semi-persistent TRS for RTT-based PDC? If the answer is yes, you can also provide the detailed configuration for semi-persistent TRS.

| Company | View |
| --- | --- |
| Nokia, NSB | No We do not see the benefits. It should be sufficient to reply on periodic TRS and leave it for gNB to decide and possible align the periodicity to the delivery of referenceTimeInfo. |
| ZTE | We don’t see the need to introduce semi-presistent TRS only for PDC. |
| OPPO | Not really. The need for such new configuration plus new triggering is not strong. In our understanding, DL-RS in RTT-based PDC should be periodic and common to multiple UEs in practice, so as to be used together with ReferenceTimeInfo in SIB9 for PDC purpose. Of course, this does not prevent it from being used with ReferenceTimeInfo sent in UE-dedicated RRC. |
| Samsung | We don’t see the need to introduce SP-TRS for PDC only. |
| vivo | Periodic TRS is enough. We don’t see the need to introduce semi-persistent TRS for PDC only. Is the purpose for overhead reduction? |
| Intel | No |
| Ericsson | Yes Semi-persistent TRS can complement periodic TRS to provide PDC support for TSN use cases that don’t have periodic clock refresh. |
| HW/HiSi | No. |

Feature lead: Except Ericsson, all other companies don’t want to introduce semi-persistent TRS. Considering that the current periodic TRS and aperiodic TRS could work, and we only have this meeting for Rel-17, it is recommended not to introduce semi-persistent TRS to do the further optimization.

Question 2.2.5.1-2: do you agree to introduce new RRC IE for SRS configuration for RTT-based PDC?

| Company | View |
| --- | --- |
| Feature lead | Based on the agreements we achieved in previous meetings below, my assumption is that new RRC IE will be introduced. However, if there is misunderstanding, we can clarify here again. Agreement Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured - At least one SRS configuration for Rx – Tx time difference estimation at gNB side Agreement For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported. |
| Nokia, NSB | No, we were just thinking of including the configuration as part of SRS-config (which is UL BWP specific), as had been also done for R16 position enhancements. [Feature lead]: Sorry I think I didn’t make it clearer. New RRC IE means introducing some new IE under the SRS-config. Maybe the name IE is not appropriate to use now, that’s why there is confusion to companies. |
| ZTE | We are open to this issue. |
| OPPO | No, if RAN1 does not introduce new IE element to SRS configuration, which leaves the current SRS configuration reusable. |
| Samsung | Similar view as Nokia. |
| Vivo | Similar view with OPPO. |
| Intel | No need |
| Ericsson | Yes. New SRS configuration IE is needed to pair with the corresponding DL RS for timing measurement. The fields in the IE can be very similar to existing SRS configuration @Nokia: if the intention is to have something similar to R16 positioning enhancements, we are fine with introducing something similar to this below SRS-PosResource-r16::= SEQUENCE { srs-PosResourceId-r16 SRS-PosResourceId-r16, … } Above is in 38.331 and for R16 positioning. [Feature lead]: Actually the example given above by Ericsson is what in my mind before. Maybe we can set the question more specific in the next round discussion. |

Question 2.2.5.1-3: do you agree to introduce RRC parameters for configuring PRS within the PCell for RTT-based PDC? If your answer is yes, do you agree that the IE for PRS configuration include parameters for sequence generation, mapping to physical resources, mapping to slots in a downlink PRS resource set, and quasi co-location information?

| Company | View |
| --- | --- |
| Nokia, NSB | Yes, as there is no PRS configuration option through RRC yet. The list of parameters suggested moderator seems to be appropriate. |
| ZTE | Yes, we agree to introduce the related RRC parameters for PRS if RTT-based PDC is supported. We are fine with these parameters since they are what PRS needs |
| OPPO | The question should be conditioned on “RTT-based PDC is supported”. No. We can be ok to define an instance of NR-DL-PRS-Info-r16 inside Rel-17 RRC for PDC purpose, but do not think it is necessary to define a new IE such as NR-DL-PRS-Info-r17 from scratch for PDC purpose. |
| Qualcomm | Yes. |
| Intel | RTT-based PDC should work w/o LPP, thus separate RRC parameters are required. |
| Ericsson | Yes |
| HW/HiSi | Yes to both questions. |

Second round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Feature lead: Since the proposals here are mainly related to RRC parameters, let’s move the discussion to the RRC email thread, where I will make some draft RRC parameters for further discussion.

Revised Proposal 2.2.5.2-1: If RTT-based PDC is supported, introduce the following in SRS-config to configure the SRS for RTT-based PDC:

- SRS-PDCResourceSet-r17

- srs-PDCResource List

- SRS-PDCResource-r17

| Company | View |
| --- | --- |
| Feature lead | 1. It is assumed that only SRS set is needed for PDC; 2. It is assumed that multiple SRS resources can be configured in the SRS resource set for PDC. |
| Nokia/NSB | Support Please note, that based on the RRC moderator guidance, we can give lists but don’t need to defined ‘AddMod/AddRelease’ (we can leave this up to RAN2). So maybe the 2nd & 3rd bullet could be collapsed to the list only. [Feature lead]: Update based on your comments, I will think more if any more update is needed. |
| ZTE | Only the first parameters seems sufficient. |
| Ericsson | Support We also agree with Nokia comment about ‘AddMod/ToRelease’ |
| OPPO | We do not see the need to define these new IEs in SRS-config given the proposal does not show any potential content difference from the “non-PDC” versions. Instead, we think the following configuration framework should work: PDC_configuration ::= { dl-RS-Resource-Set CHOICE { dl-PRS DL-PRS-PDC-Info; dl-TRS NZP-CSI-RS-ResourceSetId; } OPTIONAL srs-ResourceSet SRS-ResourceSetId OPTIONAL … } where SRS-ResourceSetId is defined in SRS-Config for a general SRS resource set. [Feature lead]: This is for the case of PRS, we need something that is independent of PRS also. |
| H3C | In general, we are fine with this proposal and we propose to modify RRC IE name on srs-PDCResource List as below srs-PDCResource List=> srs-PDCResourceId List |

Proposal 2.2.5.2-2: If RTT-based PDC is supported, introduce the following to configure PRS for RTT-based PDC:

- New IE DL-PRS-PDC-Info

- The following included in DL-PRS-PDC-Info

  - NR-DL-PRS-PDC-ResourceSet-r17

  - For sequence generation: dl-PRS-SequenceID

  - For mapping to physical resources: dl-PRSResourceSymbolOffset, dl-PRS-NumSymbols, dl-PRS-CombSizeN, dl-PRSCombSizeN-AndReOffset,  dl-PRS-ResourceBandwidth, dl-PRS-StartPRB, dl-PRS-PointA

  - For mapping to slots in a PRS resource set: dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRSResourceSlotOffset, dl-PRSResourceRepetitionFactor, dl-PRS-ResourceTimeGap

  - For quasi co-location information: dl-PRS-QCL-Info

| Company | View |
| --- | --- |
| Nokia/NSB | Support in principle May need to cross-check the details still in the RRC parameter discussions. |
| ZTE | We are fine with this proposal. |
| Ericsson | Support |
| H3C | We are fine with FL proposal. |

How to make sure the gNB/UE measure the same RS pair

ZTE (R1-2110917) In RAN1#106b-e, it was agreed that one CSI-RS for tracking (TRS)/PRS configuration for Rx-Tx time difference estimation at UE side and one SRS configuration for Rx-Tx time difference estimation at gNB side are configured for PDC in Rel-17 if RTT-based PDC is supported. However, if the UE and the network measure the different RS pair, it may lead to double gNB Rx-Tx time difference errors and UE Rx-Tx time difference errors in the evaluation. More details can be found in the section 5.2 below copied from our contribution [4]. Therefore, since Alt 1 for evaluation has been adopted with only one gNB Rx-Tx time difference error and UE Rx-Tx time difference error considered, measuring the same RS pair should be ensured. For example, the information that which RS pair is measured should be indicated from the network to the UE. Proposal 2: The network should indicate to the UE that which RS is measured such that the network and the UE can measure the same RS, if RTT-based PDC is supported. For example, the frame number for the TRS/PRS and SRS can be indicated.

Huawei (R1-2111926) In RAN1#106bis-e, it was discussed about how to ensure the same pair of TRS/PRS and SRS for RTT measurement but unfortunately no consensus yet. Based on the Rx-Tx time difference definition below, TUE-RX is the UE received timing of downlink subframe #i if TRS is in DL subframe #i, and TUE-TX is the UE transmitted timing of uplink subframe #j that is closest in time to the subframe #i. So if there is also SRS transmission in subframe #(j+1), then the UE still uses the uplink timing of subframe #j rather than subframe #(j+1) to obtain the Rx-Tx time difference. The pair should be DL subframe #i and UL subframe #j. For the gNB RTT measurement it can be same as UE RTT measurement to ensure the same pair of TRS/PRS and SRS. For the A-SRS transmission in a subframe, we think the same mechanism can be used, i.e. if this subframe is closest in time to the subframe #i, then it should be used for RTT measurement. ### 5.1.30 UE Rx – Tx time difference Definition; The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.. Applicable for; RRC_CONNECTED.. Observation 2: UE RTT measurement can be based on the pair of DL subframe #i and UL subframe #j that is closest in time to the subframe #j if PRS/TRS is transmitted in subframe #i and SRS is transmitted in subframe #j.

OPPO (R1-2111344) Observation 5: With no solution to handle inconsistent Rx-Tx time difference measurements between gNB and UE, the evaluation of RTT-based PDC should consider an additional error component as small as 89.5ns and as large as 260ns for 15kHz SCS and 130ns for 30kHz SCS. Observation 7: The RTT-based PDC evaluation shows that, for some given channel conditions, - If inconsistent RTT measurements between UE and gNB do not happen, RTT-based PDC can meet 275ns error budget for BW no smaller than 10MH and for both 15kHz and 30kHz SCS. - If inconsistent RTT measurements between UE and gNB can happen at least due to UE autonomous adjustment, RTT-based PDC can meet 275ns error budget for BW no smaller than 20MHz for both 15kHz and 30kHz SCS, conditioned on that the DL SFN timing detection is based on full-bandwidth DL-RS (e.g.,CSI-RS).

Intel (R1-2111492) From the last meetings, the following details of RTT-based scheme were identified: - Whether / how to handle inconsistent RTT measurement in gNB and UE due to change of uplink TX timing - It seems a baseline to assume that the change in uplink TX timing is not expected when RTT procedure is being performed - Alternatively, gNB may request a UE to defer TX timing adjustments until RTT measurement procedure is completed

Feature lead: Based on the discussion in RAN1#106bis-e meeting, it seems companies think either there is no inconsistent RTT measurement issue, or if the inconsistent exist it should be avoided by some other way, which means there is no additional error component to be added in the RTT-based PDC caused by a change in the uplink TX timing. In RAN1#106bis-e meeting, the following proposal was discussed also to address the potential inconsistent RTT measurement issue, but there is no consensus achieved.

========

For one pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, UE/gNB may assume that the latest CSI-RS for tracking (TRS)/PRS and SRS transmission event is used for RTT measurement.

========

In this meeting, both ZTE and Huawei proposed some way to solve the issues. Views from companies are needed.

First round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 2.2.6.1-1: Which option do you prefer to ensure the same pair of RS is used for RTT measurement for RTT-based PDC?

- Option 1: RTT measurement is based on the pair of DL subframe #i and UL subframe #j that is closest in time to the subframe #i if PRS/TRS is transmitted in subframe #i and SRS is transmitted in subframe #j.

- Option 2: Indicate to the UE the subframe number for the TRS/PRS and SRS for RTT measurement.

- Option 3: None of the above

| Company | View |
| --- | --- |
| Nokia, NSB | Option 3. Reuse the existing Rx-Tx measurement definition. Based on our understanding, the existing Rx-Tx measurement definition could & should be reused and is way accurate for the purpose of time synchronization. The existing definition does not restrict the UE to measure only on a subframe #j where it is also transmitting SRS while receiving PRS/TRS in DL subframe #i. As already agreed, the gNB will configure the UE with a single configuration of PRS/CSI-RS and SRS to enable RTT based PDC. This should leave no uncertainty of which RS is used for the measurement. [Feature lead]: Please check the RAN4 spec below, the UE Rx-Tx time difference measurement accuracy is not applied if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance, which is inconsistent measurement raised By OPPO. OPPO also provided some more detailed analysis with examples in their replies below, please check. If we just want to reuse the existing Rx-Tx measurement definition without introducing any restriction as shown in option 1, one way we can take whatever defined in RAN4 for positioning to handle this inconsistent measurement issue, or alternatively we need to rely on gNB/UE implementation to avoid this kind of inconsistent measurement, e.g. the gNB should not update Timing Advance during the UE Rx-Tx measurement period for RTT-based PDC, and similar at the UE side, UE need to avoid the autonomous timing adjustment also. I will add an option for the next round discussion. |
| ZTE | We prefer Option 2. And it should be system frame number. Without a solution, the network and the UE may measure the DL RS transmitted at different time. It leads to extra error components for RTT-based PDC. For example, the network measures the DL RS at frame i1 while the UE measures the DL RS at frame i2. For option 1, it is not clear how to ensure the network and the UE measur the same DL RS and the same UL RS. [Feature lead]: Option 1 with the change from OPPO below would be very clear. For option 2 it is not clear whether only indicating the frame number is sufficient or not, since one frame may configure many sub-frames. Since not many companies support option 2, I would remove option 2 for the next round discussion. Hopefully it can be acceptable for you. |
| OPPO | We support the principle in Option 1. But ZTE raised a valid point. The simple amendment is as following to identify “which pair”. Option 1: RTT measurement is based on the pair of DL subframe #i and UL subframe #j that is closest in time to the subframe #i if PRS/TRS is transmitted in subframe #i and SRS is transmitted in subframe #j. If UE (or gNB) receives a RTT measurement report from gNB (or UE), the UE (or gNB) assume the RTT contained in the report is measured by gNB (or UE) based on the last pair of DL subframe #i and UL subframe #j before the transmission of RTT measurement report. [Feature lead]: I can update option 1 as you suggested to make it clearer. In addition, please check my replies to Nokia above also, to see if you can accept the existing definition with some potential clarifications as shown in proposal 2.2.6.2-1. We would like to highlight: - The current spec does not support UE to measure one single RTT based on TRS/PRS in SF #i and SRS in SF #j. The spec (38.215) only allows UE to measure based on SF#i and its closest UL subframe. The same restriction for gNB. - RAN1 agreed to have a single configuration of PRS/TRS (which could be periodic) and a single configuration of SRS (which could be periodic). But the measurements in gNB and UE could be asynchronous, and an additional change of UL Tx timing between these two measurements could introduce additional error. For example, in figure below, gNB may inform UE RTT_{BS}(t1) while UE uses RTT_{MS}(t2). We would like to hear companies’ views on how to remove effect of ∆ without Option 1 and 2. We do not prefer Option 2 because Option 2 seems require UE to remember multiple Rx-Tx measurements (UE only needs to remember the last one in Option 1) and a timestamp in RTT indication signaling from gNB to UE, which we think as additional complexity comparing to Option 1. |
| Samsung | We think the actual measurement on DL might not need to be specified, while the report will need a reference. [Feature lead]: Please check my replies to Nokia above to see more the thinking on the proposal here. |
| Intel | Agree with Nokia. Current definitions allows processing over multiple PRS/SRS occasions. [Feature lead]: Please check my replies to Nokia above to see more the thinking on the proposal here. |
| Ericsson | We also think existing Rx-Tx measurement definition should be reused, except minor change to RS. [Feature lead]: Please check my replies to Nokia above to see more the thinking on the proposal here. |
| HW/HiSi | Option 1, which means in our understanding to apply the existing Rx-Tx measurement definition. We do not see a need to indicate the sub-frame number. |

Second & Third round discussion

Based on the inputs and analysis above, the following questions/proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Feature lead: It seems there are no sufficient views on the proposals here in the second round email discussions. Companies are encouraged to check my replies to company comments, and provide your further view.

Proposal 2.2.6.2-1: To avoid inconsistent RTT measurement for RTT-based PDC,

- Option 1: RTT measurement is based on the pair of DL subframe #i and UL subframe #j that is closest in time to the subframe #i if PRS/TRS is transmitted in subframe #i and SRS is transmitted in subframe #j. If UE (or gNB) receives a RTT measurement report from gNB (or UE), the UE (or gNB) assume the RTT contained in the report is measured by gNB (or UE) based on the last pair of DL subframe #i and UL subframe #j before the transmission of RTT measurement report.

  - Support: Samsung, OPPO

- Option 2: For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC.

  - Note: The inconsistent RTT measurement will be avoided by taking the mechanism defined for positioning in RAN4 if applicable, and/or avoided by gNB/UE implementation.

    - Support: Nokia, NSB, Ericsson, Huawei, HiSilicon, Intel,

| Company | View |
| --- | --- |
| Feature lead | Please check the discussion and my replies in section 2.2.6.1 to understand more background of the proposal here. |
| Samsung | We prefer opt 1. |
| Nokia, NSB | Option 2 Thanks to the FL for the good comments. As we see it, the inconsistency issue when the UE changes UL transmission timing during an Rx-Tx measurement is also handled within the context of Positioning and we can reuse their solution. To our understanding, the existing Rx-Tx measurement definition (when extended to CSI-RS/TRS, is adequate. |
| ZTE | For Option 1, we agree that it can avoid the inconsistent measurement. But we don’t think the UE only remember the last one for option 1 commented by OPPO. From the UE perspective, it does not know when the network transmits the measurement report to it and vice versa. So the UE does not know which one is the last one. The UE should perform measurement and record for each configured PRB/TRS. We believe this is a big burden for UE and the gNB ( in the case of gNB pre-compensation). So the method to reduce the burden should be considered. That’s the reason why we propose option 2 in Question 2.2.6.1-1. When the network plans to measure a PRS/TRS and SRS, it indicates the frame number of the PRS/TRS and SRS to the UE. So the network and the UE can measure the same RS. In this case, the UE does not need to measure and record the RS other than the one indicated by the network. In our view, we assume the PRS/TRS or SRS periodicity is not less. So frame number is sufficient. Of course, if companies think finer indication is needed, frame number + slot number can also be used. For option 2 in this proposal, it has the same issue on measurement burden if this is avoided by gNB/UE implementation. [Feature lead]: Thank you very much for the detailed explanation on your thinking. For option 2 in the current proposal, it just follow the mechanism defined for positioning, thus if measurement burden is not a problem for positioning it should not be a concern here. |
| H3C | Although we slightly prefer to Option 1, we propose to add time stamp in measurement report to avoid inconsistent RTT measurement for RTT-based PDC. And this method is already used in positioning based on multi-RTT. [Feature lead]: What do you think of Option 2? With option 2, in my understanding the inconsistent RTT measurement issue will not be there. If we can address the issue without adding time stamp in the measurement report, that would be good I think. |
| Ericsson | Option 2 We also think that mechanisms used in positioning can be reused. This saves much work for RAN1 and RAN4. |
| OPPO | Option 1. For Option 2, FL mentions the following possible “solutions”: - “one way we can take whatever defined in RAN4 for positioning to handle this inconsistent measurement issue” [OPPO]: RAN4 did not define anything to handle this particular issue. [Feature lead]: This is an open issue in RAN4 and they are trying to address it in my understanding. For example, for the case of autonomous timing adjustment it seems the current understanding is that the defined Rx-Tx measurement accuracy is still applied if the DL RS and SRS belongs to the same cell. So if in the end they will have clear conclusion on this, it can be applied to PDC also. - “or alternatively we need to rely on gNB/UE implementation to avoid this kind of inconsistent measurement, e.g. the gNB should not update Timing Advance during the UE Rx-Tx measurement period for RTT-based PDC” [OPPO]: TA is an on-demand procedure fundamental to UE operation. RAN2 even defines TA expiration timer to ensure the TA command being sent and UL-Tx timing being adjusted. Further, RAN4 spec says clearly that the TA command immediately restart UE Rx-Tx measurement period. So there seems no gap between two measurement periods, which means there is no time gap for gNB to update TA if gNB does not do so during measurement period. [Feature lead]: In my understanding, TA command will not be updated so often, e.g. the typical duration for TA expiration timer is about hundreds of milliseconds. It should be able to put DL RS and UL RS for Rx-Tx measurement within a window without change of the TA command. - “and similar at the UE side, UE need to avoid the autonomous timing adjustment.” [OPPO]: Autonomous adjustment is the only procedure for UE to make the UL-Tx timing fall into the window of [-Te, +Te] around the desired UL-Tx time instance. [Feature lead]: According to the latest discussion in RAN4, I think the common understanding is that the Rx-Tx measurement accuracy requirements is still applied for the case of autonomous timing adjustment, if the DL RS and SRS belongs to the same cell. Therefore, we don’t need to consider this case. Back to ZTE’s comment of “From the UE perspective, it does not know when the network transmits the measurement report to it and vice versa. So the UE does not know which one is the last one”. In our understanding, - Every time UE obtains a new RTT measurement, this new RTT measurement becomes the “last one”. - Because gNB and UE measures the same pair of DL and UL transmissions, gNB has to send RTT measurement report to UE after measuring this “last RTT”, where this “last RTT” is supposed to be included in the RTT measurement report sent to UE. |
| HW/HiSi | Option 2 |
| H3C 2 | For Option 2, I want to clarify how to ensure Rx-Tx measurement based o the same pair of PRS and SRS by UE and gNB. Configuration on PRS and SRS transmission is one-shot or periodic? For PRS and SRS periodic transmission, how to determine the pair of PRS and SRS? In addition, for current positioning based on multi-RTT, time stamp is already supported The IE NR-TimeStamp defines the UE measurement associated time stamp (refer to TS 37.355). Why not reuse this IE for resolving the inconsistent RTT measurement issue? [Feature lead]: Then it means taking the existing mechanism is fine I guess? i.e. the direction of option 2. One thing I am not so sure is that if the existing timestamp already addressed the issue, then what is the reason RAN4 is also discussing this inconsistent issue for Rx-Tx measurement for positioning? It would be appreciated if you can explain more. [Feature lead 2]: After further checking, I think only relying on timestamp cannot solve the issue, since the timestamp just reports the time for the measurement, which may help identify whether there is any RTT inconsistent measurement for the measurement report or not, but cannot ensure there is no RTT inconsistent measurement. -- ASN1START NR-TimeStamp-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, -- Need ON nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, -- Need ON nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, -- Need ON nr-SFN-r16 INTEGER (0..1023), nr-Slot-r16 CHOICE { scs15-r16 INTEGER (0..9), scs30-r16 INTEGER (0..19), scs60-r16 INTEGER (0..39), scs120-r16 INTEGER (0..79) }, ... } -- ASN1STOP |
| Nokia/NSB 2 (Third round) | The existing Rx-Tx measurement definition does not need to relate a DL RS and an UL RS pair, and it is not needed. The UE will report its SFN boundary offset from its intented transmission time till its SFN boundary. The SFN boundary will be estimated by the one or multiple (if available) PRS receptions in the subframe (gNB need to ensure at least one PRS is available in the subframe). The UE does not need a specific SRS configuration related to it, as it will reports its intended transmission time. The effect of the SRS is then present in the gNB Rx-Tx measurement. The benefit of not doing so, is that multiple PRS/SRS can be used to enhance the existing SFN boundary and also allows for averaging to combat small scale fading on the detector side. If we limit to a single occurrence we will also potentially loose out on the accuracy enhancements achievable if multiple instances is used and is allowed by the existing definition. [Feature lead]: Agree. I think we can try to ask RAN4 to define some measurement period for PDC also, just similar as positioning, and also as OPPO commented below it can leave sufficient time for UEs to average. Originally I think even this Rx-Tx measurement period thing is the solution for positioning included by the current note, but I think ok to make it clearer if people prefer that way. Please check the revised proposal in section 3. So in the end, we see the need for an SRS and PRS/TRS configuration. A good gNB implementation will ensure one of these pairs in available in the subframe where an Rx-Tx measurement is conducted and we can reuse the existing definition (but still expanded to TRS) |
| ZTE2 (Third round) | We agree that every time UE obtains a new RTT measurement, this new RTT measurement becomes the “last one”. So it means the UE has to measure each DL RS. What we concern is the UE burden to measure each DL RS. [Feature lead]: I don’t think UE needs to measure each DL RS, at most need to store the RS. Then only when necessary do the measurement. For example, once the reference information is received, then UE would do the measurement. Regarding the comments from FL, we would like to point out, in positioning, the UE does not need to measure each DL RS. So the UE measurement efforts are not an issue. However, here things are different. This issue should be resolved first before adopting Option 1 and Option 2 in this proposal. [Feature lead]: In my understanding, there is no need to measure each DL RS, therefore the issue should not be there. |
| Intel | We prefer Option 2. At least in our reading Option 1 has an issue when PRS is configured with multiple resources in a burst/occasion for coherent processing – it does not reflect that PRS/TRS/SRS span multiple consecutive subframes. |
| OPPO (3rd round) | In our view, Option 2 can work only if both gNB and UE are given enough time to run their internal filtering to average out the RTT change on either side (according to RAN4 spec, the RTT change due to autonomous UL-TX adjustment does NOT restart UE Rx-Tx measurement period so it may happen within measurement period), so even if gNB and UE have no coordinated RTT measurements, the inconsistency would be eventually averaged out. But this effect needs time. That is to say, Option 2 may or may not work for the PD estimation performed in the beginning duration, e.g., the first 200ms, of a Rx-Tx measurement period. So with Option 2, we see one potential need for gNB to NOT send RTT measurement report to a UE until sufficient time runs out after the restart of current Rx-Tx measurement period for the same UE. This is for UE-side PDC. Similar need upon UE is applicable to gNB-side pre-compensation. In addition, we do not think any RAN4 Rx-Tx accuracy requirements like the ones in 38.133 sections 10.1.25.2 and 13.2.2 suggest any “mechanism” as solution. So we can live with Option 2 if it has following new note: - Note: The inconsistent RTT measurement will be avoided by taking the mechanism defined for positioning in RAN4 if applicable, and/or avoided by gNB/UE implementation, e.g. gNB(for UE-side compensation) /UE(for gNB-side precompensation) does not send RTT measurement report until sufficient time runs after restart of UE Rx-Tx measurement period. - The above example may require RAN4 to define UE Rx-Tx measurement period for PDC purpose and to make the measurement period restart condition(s) knowable to gNB. Note that the above gNB/UE implementation example is just a necessary condition to weaken inconsistency issue, but not a sufficient condition, because gNB and UE do not know how the internal filtering is implemented on the other side, so it is hard for them to judge “sufficient time”. [Feature lead]: Thank you very much for being flexible with option 2 and good comments here. Originally I was thinking defining Rx-Tx measurement period belongs to “the mechanism defined for positioning” also, but I think no harm to make it clearer. I will update option 2 as you suggested above with some modification, please check the one in section 3. Basically since RAN4 know well on the Rx-Tx measurement period due to the work for positioning, no need to describe too much to avoid potential confusion to RAN4, therefore I kind of simplifying the suggestions you made above. Please check if it appropriate. For Option-1, we agree the current reading does limit one RTT measurement sample per RTT measurement report. But this can be simply fixed by signaling design rather than the principle (same pair of DL/UL) of Option 1, i.e., changing “the last one” to “the last N”, i.e., - [Modified Option 1]: One RTT measurement is based on the pair of DL subframe #i and UL subframe #j that is closest in time to the subframe #i if PRS/TRS is transmitted in subframe #i and SRS is transmitted in subframe #j. If UE (or gNB) receives a RTT measurement report from gNB (or UE), the UE (or gNB) assume the N RTT measurements contained in the report is are measured by gNB (or UE) based on the last N pairs of DL subframe #i and UL subframe #j before the transmission of RTT measurement report. Option 1 does not require RAN4 to define UE Rx-Tx measurement period for PDC purpose, since it does not need to identify the restarting moment for the period and can handle any change of UL-Tx timing, e.g., due to TA command and a modified NTA,offset that currently triggers the restart of Rx-Tx measurement period. As final, our preference is still Option 1 or modified Option 1. [Feature lead]: I can modify option 1 as you suggested. However, considering more companies prefer the direction of option 2, hopefully you can live with it. I will modify option 2 to reflect your comments also. Thank you very much for being flexible. |
| Ericsson (3rd round) | Option 2. In fact, it’s hard to tell why Option 1 avoids inconsistency better than Option 2. See existing definitions below where it already specifies that subframe #j is closest to subframe #i. (The existing definition contains more info beyond this). With below definition, we think the inconsistency is taken care of. The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP.. The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE.. Thus, we do not see any reason to add “restart of UE Rx-Tx measurement period”, or other RAN4 related change, to Option 2. [Feature lead]: In positioning, they define the Rx-Tx measurement period also, so I think at least we can consider to spell out this for PDC to address OPPO concern. I agree with you that we don’t need to say something like re-start though, RAN4 would know well what this period is used, RAN1 should avoid using description that may make RAN4 confusing. Maybe the point of Option 1 is, RxTxTimeDiff in RTT measurement report is based on last pair before the measurement report. We agree that this is a valid concern, e.g., receiver can assume that the RxTxTimeDiff received is reasonably fresh for PDC purpose. On the other hand, as companies mentioned, the reporting entity may generate RxTxTimeDiff based on aggregating multiple measurements. That is, how to generate the value is up to implementation. The sender of the message know that RxTxTimeDiff needs be reasonably fresh for PDC calculation, and can take this into account. In summary, this issue is valid, but up to implementation. |
| Samsung | Thanks to the good discussion. I wonder, maybe we shall not define what to measure (it could be up to UE implementation), but only to define what is the reference of Rx-Tx time difference. If this is true, and if there is a way to clearly define the reference DL time, option 2 is OK. [Feature lead]: In positioning, they define the Rx-Tx measurement period also, so I think at least we can consider to spell out this for PDC to address OPPO concern. Please check my replies to OPPO and also the updated proposal in section 3. |

How to signal the Rx-Tx time difference measurement report

ZTE (R1-2110917) In positioning, the Rx-Tx time difference measurement is reported via RRC signaling. From RAN1 persepctive, there is no issue on the reporting latency since the receiver can keep its measurement report record for a long time. In addition, the PDC is performed only when needed. Therefore, RRC signaling for Rx-Tx time difference measurement reporting is adequate if RTT-based PDC is supported. Proposal 4: RRC signaling for Rx-Tx time difference measurement reporting should be supported if RTT-based PDC is supported.

Intel (R1-2111492) Proposal 1 - For RTT-based UE side compensation, the gNB Rx-Tx time difference measurement is indicated to UE(s) using L1 group-common DCI signaling, - FFS details

Feature lead: This issue was discussed in RAN1#106b-e meeting, but no any conclusion achieved. Some companies think that it should belong to RAN2 scope, while some prefer L1 signaling instead of RRC signaling.

First round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals/questions before providing your views here.

Question 2.2.6.1-1: Which option do you prefer for Rx-Tx time difference reporting signaling?

- Option 1: Detailed signaling design is up to RAN2.

- Option 2: The Rx-Tx time difference is reported via RRC signaling.

- Option 3: The Rx-Tx time difference is reported via L1 group-common DCI signaling.

- Option 4: None of the above

| Company | View |
| --- | --- |
| Nokia, NSB | Option 1 - first preference (and if not left for RAN2, then Option 2) There is no need to go for a L1 signalling option, as PD related signalling is not latency critical. |
| ZTE | We support Option 2 and we don’t see the need to introduce L1 group-common DCI signaling since PDC should not be frequent. |
| OPPO | Option 2. Option 3 is not compatible to existing error formula for RTT-based PDC, due to its group-common property. With a L1 group-common indication, it is hard for gNB to tell each UE about the exact/true gNB-side RTT associated with that UE; instead, the gNB can only tell a “desired” RTT for the group of UE and simply wishes the individual UL-Rx timing from each UE can be aligned with that “desired” UL-Rx timing -- the gNB can only reply on TA for such UL-Rx alignment, then an error of half TA granularity should be unavoidable and therefore conflict with agreed total-error formula. |
| Samsung | Option 1. This is a feature led by RAN 2 and RAN 2 just waits for RAN 1’s view on which solution to go. |
| vivo | Option 1. We think signaling design belongs to RAN2 scope. |
| Qualcomm | Option 1. |
| Intel | First preference Option 3, second preference is Option 1 |
| Ericsson | Option 2 It’s routine procedure that RAN1 can agree to use RRC and then send the RRC parameter list to RAN2. Further details on capturing RRC parameters agreed by RAN1 is up to RAN2. |
| HW/HiSi | Option 1 as first preference, otherwise Option 2. |

Feature lead: It seems option 2 achieves the most support. However, also several companies share the view that this should be up to RAN2. Therefore, it is recommended not to further discuss in RAN1, and we can just leave it to RAN2.

Second round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion: The detailed signaling design for Rx-Tx time difference measurement reporting for RTT-based PDC is up to RAN2, if RTT-based PDC is supported.

| Company | View |
| --- | --- |
| Samsung | Support |
| Nokia/NSB | Support |
| ZTE | We are fine with this proposal. |
| Ericsson | We can accept On the other hand, we’d like to make sure that above does not prevent RAN1 from making agreements about RRC parameters and then notify RAN2. Thus, recommend adding the note below: “Note: the conclusion above does not prevent RAN1 from making agreements about RRC parameters and then notify RAN2.” [Feature lead]: Sure this will not have impact on the RRC parameter discussions. It is obvious, thus we don’t need to add such a note. |
| OPPO | Not support. It is the tradition in the past for RAN1 to firstly decide whether to use MAC-CE or RRC. [Feature lead]: Indeed RAN2 would be the experts for RAN2 signaling design. If there is different views in RAN1, we can just leave it to RAN2. Hopefully it can be acceptable for you. |

Third round email discussion

Based on the inputs and analysis above, the following observations and/or conclusions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposed conclusion: The detailed signaling design (e.g. MAC CE or RRC) for Rx-Tx time difference measurement reporting for RTT-based PDC is up to RAN2.

| Company | View |
| --- | --- |
| Feature lead | @OPPO Please check my replies to your comments above. Hopefully this can be acceptable for you since it is fine for all companies. |
| H3C | We support this proposal. |
| Nokia/NSB | Support the proposal. We do prefer RRC for Rx-Tx measurement report, but if we cannot agree in RAN1, better to leave this for RAN2 to decide. |
| ZTE | We share the same view with Nokia that RRC should be supported. We are fine to let RAN2 make a decision. |
| Intel | Agree with the conclusion |
| OPPO | We also support RRC. If the major voting competition is between Option 1 (leave it to RAN2) and Option 2 (use RRC), maybe the offline discussion could try a proposal for RRC, given there is no other major signaling method gaining as many supports as RRC. Anyhow, if the majority wants to do this in a way opposite to the tradition (i.e., Option 1), we would not object. [Feature lead]: I do agree with you that if we can make further progress that would be great. In the first round several companies share the view we should leave it to RAN2 though, let’s check. |
| Ericsson | Regarding MAC CE vs RRC: we support RRC. From companies’ input, nobody objects to RRC. Thus we suggest to make this simple agreement to support RRC. In fact so far there is only measurement report in 38.331, and no measurement report in 38.321. If some company objects to RRC, it should be clearly explained why this measurement report is so different from all other measurement reports. |

Alternative Proposal 2.2.7.3-1: For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling.

| Supporting companies |  |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Based on the replies to the above conclusion, it seems most companies prefer RRC signalling. Therefore, as OPPO mentioned, we could try see if companies can accept RRC signalling also. Only if really cannot achieve the consensus, then we can consider to go to the conclusion. |

UE/gNB Rx – Tx time difference definition for RTT-based PDC

The current definition for UE Rx-Tx time difference and gNB Rx-Tx time difference defined in TS 38.215 for positioning is shown below.

5.1.30 UE Rx – Tx time difference Definition; The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.. Applicable for; RRC_CONNECTED. 5.2.3 gNB Rx – Tx time difference Definition; The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector..

In RAN1#107-e meeting, Ericsson propose to reuse the current definition with updates to the DL RS and UL RS description for RTT-based PDC.

Ericsson (R1-2111191) For the RTT-based method, the relevant measurement quantities are: - UE Rx – Tx time difference - gNB Rx – Tx time difference Depending on the entity that performs the propagation delay compensation, either (a) or (b) is reported. For example, if UE is the entity that performs the compensation, then gNB measures Rx – Tx time difference and sends the measurement to the UE. Both (a) and (b) have been carefully defined in 38.215 for the purpose of positioning, see below. For the time synchronization purpose, the definitions can be reused as is, except that the reference signals should be updated to include those for time synchronization also. Specifically, the yellow highlight sentence for UE Rx – Tx time difference need to be updated to include TRS, and the yellow highlight sentence for gNB Rx – Tx time difference need to be updated to include SRS for propagation delay compensation. In terms of the reference point for measurements, the existing definition should be used, e.g., Rx antenna connector, Tx antenna connector. The reference point cannot be baseband. For example, in the latest RAN4 discussion of reference point for Te, it was agreed to include ‘antenna’ in the Te definition (see R4-2115371). Existing definitions of UE Rx – Tx time difference and gNB Rx – Tx time difference are reused with updates to the DL RS and UL RS description. TS 38.215 V16.4.0: Definition; The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS resources can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE.. Applicable for; RRC_CONNECTED. Definition; The gNB Rx – Tx time difference is defined as TgNB-RX – TgNB-TX Where: TgNB-RX is the Transmission and Reception Point (TRP) [18] received timing of uplink subframe #i containing SRS associated with UE, defined by the first detected path in time. TgNB-TX is the TRP transmit timing of downlink subframe #j that is closest in time to the subframe #i received from the UE. Multiple SRS resources for positioning can be used to determine the start of one subframe containing SRS. The reference point for TgNB-RX shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. The reference point for TgNB-TX shall be: - for type 1-C base station TS 38.104 [9]: the Tx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Tx antenna (i.e. the centre location of the radiating region of the Tx antenna), - for type 1-H base station TS 38.104 [9]: the Tx Transceiver Array Boundary connector..

Feature lead: In RAN1#106bis-e meeting, this issue was discussed but with no consensus. Some companies commented that how to update the spec can be done later based on the agreed mechanism. Some other companies felt pre-mature to discuss now since it depends on the discussion on other aspects. Therefore, it is recommended not to discuss this issue for now. Once we get other issues done, we can see if any good time to discuss it.

Implicit propagation delay compensation

OPPO (R1- 2111344) proposes an implicit PDC method as below

OPPO (R1- 2111344) For implicit PDC, the principle of propagation delay compensation is given in Figure 2. Figure 2 Implicit PDC In Figure 2, the gNB clock (clkBS(t)) and UE clock (clkUE(t)) at the same moment of t, after UE reception of the UE-specific RRC in figure (corresponding to k=1) or a follow-up SIB9 ReferenceTimeInfo (corresponding to k=2,3…), satisfy clkBSt-mUL,RX+mDL,TX,k2=clkUEt-nUL,TX+nDL,RX,k2, assuming the DL PD equals to UL PD which is also the assumption in one-way propagation delay estimation in explicit PDC. This means the UE can estimate clkBSt as clkBS,implicitt=clkUEt+mUL,RX+mDL,TX,k2-nUL,TX+nDL,RX,k2 (3) Note that eclk≜clkBSt-clkUEt=mUL,RX+mDL,TX,k2-nUL,TX+nDL,RX,k2, i.e., the clock difference between gNB and UE, is exactly what we defined in our earlier contributions[2] [3]. The corresponding clock sync error for implicit PDC is given by etotal,implicit=12eDL,TX+eDL,RX+eUL,TX+eUL,RX+2eQ (4) where eQ is the quantization error in carrying mDL,TX,k and mUL,RX. - eDL,TX and eDL,RX are the SFN timing measurement errors for mDL,TX,k and nDL,RX,k, respectively, i.e., the same as for explicit PDC. - eUL,TX and eUL,RX are timing measurement errors for nUL,TX and mUL,RX, respectively, associated with the SRS transmission. - eQ is the quantization error in RRC signaling carrying mUL,RX and mDL,TX,k. The comparison between etotal,explicit in (2) and etotal,implicit in (4) shows: - The accuracy of explicit PDC relates to errors of six timing measurements, including two measurements giving mDL,TX,k and nDL,RX,k, and four timing measurements in deriving PD; while the accuracy of implicit PDC relates to errors of four timing measurements, including mDL,TX,k, nDL,RX,k, nUL,TX and mUL,RX. - The errors associated with mDL,TX,k and nDL,RX,k have coefficients of 1 for explicit PDC and coefficient of ½ for implicit PDC. We further have etotal,explicit-etotal,implicit=12eDL,TX+eDL,RXSFN+ePD-12eUL,TX+eUL,RXSRS>0 where the inequality of “>0” comes from the fact that the one-way propagation delay estimation on PD would use the timing measurements whose errors correspond to 12eUL,TX and 12eUL,RX (Remember PD takes the same time reference as the local clock in both Figure 1 and Figure 2, and the nUL,TX and mUL,RX are associated with UL-SRS in Figure 2 --- the same type of UL-RS that could be used in PD estimation in Figure 1). In general, ePD can be lower-bounded by ePD≥12eDL,TX+eDL,RXPRS/TRS+12eUL,TX+eUL,RXSRS, we can have etotal,explicit-etotal,implicit≥12eDL,TX+eDL,RXSFN+12eDL,TX+eDL,RXPRS/TRS>0 (5) where eDL,TX+eDL,RXSFN is the total DL Tx/Rx timing error associated with the DL transmission (e.g., SIB9) that helps UE to determine the SFN timing, and eDL,TX+eDL,RXPRS/TRS is the total DL Tx/Rx timing error associated with the DL transmission used in one-way delay estimation. Observation 2: The synchronization error of implicit PDC on Uu-interface can be evaluated as etotal,implicit=12eDL,TX+eDL,RXSFN+12eUL,TX+eUL,RXSRS+eQ,ReferenceTimeInfo This error is smaller than that of explicit PDC, if the same assumptions are made between the two methods for the following timing error components - UL Tx/Rx timing errors associated with SRS transmission; and - DL Tx/Rx timing errors associated with DL transmission containing ReferenceTimeInfo IE; and - time granularity in ReferenceTimeInfo IE. - 2.4 Evaluation of implicit PDC The evaluation for implicit PDC based on equation (4) is given in Table 4, where the most generous upper-bound of eDL,RX+eUL,TX≤Te is applied to BW≥5MHz for both SCS=15kHz and SCS=30kHz. For BW=5MHz and SCS=15kHz, additional assumption of SFN boundary timing detection based on DL-RS also allows more accurate timing evaluation, as noted as “Evaluation method 2” and “Evaluation method 3” in Table 4. Table 4 Evaluation of implicit PDC SCS; BW; Errors with SFN timing (ns); Errors with UL SRS transmission (ns); Quantization error in signaling; Total sync error (ns) (against 280ns budget). eDL,TX/2; eDL,RX/2; eUL,TX/2; eUL,RX/2; eQ. 15kHz; 5MHz (25RB, Evaluation method 1); 32.5; ≤Te2=12∙64Tc2=195.3; 50; ≤ 2.2; ≤280. 5MHz (25RB, Evaluation method 2, SIB9 along w/ DL-RS); 32.5; 120.5/12SCS∙BW+δ≤55.6+δ2; 120.5/12SCS∙BW≤55.6; 5; ≤148.7+δ2. 5MHz (25RB, Evaluation method 3, SIB9 along w/ DL-RS); 32.5; eUE,RxTxDiff2= 59.3+δ2; egNB,RxTxDiff2= 58.3; 5; ≤155.1+δ2. ≥10MHz (52RB); 32.5; ≤Te2=12∙64Tc2=195.3; ≤12max0.5/12SCS∙BW,egNB,RxTxDiff≤max26.7,38.2=38.2; 5; 271. 30kHz; Any BW≥5MHz; 32.5; ≤Te2=8∙64Tc2=130.2; 50; 5; ≤217.7.

Feature lead: This implicit PDC was discussed a lot in RAN1#106b-e meeting but still no consensus yet. The situation is that almost all companies don’t want to continue the discussion and want to have the conclusion that “There is no consensus to support/specify implicit PDC in Rel-17” based on the discussion in RAN1#106b-e. This is the last meeting of Rel-17, from moderator perspective, I don’t have any good idea on how to move forward, and especially the implicit PDC itself is actually RAN2 mechanism and RAN1 is not able to judge whether it is feasible or not.

First round discussion

Based on the inputs and analysis above, the following questions are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Question 4.4-1: Do you think we need to continue the discussion of implicit PDC? Please provide your reasons also.

| Company | View |
| --- | --- |
| Nokia, NSB | No, Given that the top priority for now should be to get some PDC method, we should focus on those with largest support. Implicit PDC is not one of them and can be dropped. |
| ZTE | No, we should focus on TA-based PDC and RTT-based PDC. |
| OPPO | Before people discusses whether they are willing to discuss implicit PDC, I would like to ask whether people can show technical proof to against following observations: - implicit PDC can have almost half error as RTT/TA-based PDC. As people can derive and as we showed in R1- 2111344, the error formula for the three PDC solutions are : - For implicit PDC: the following error tests against 280ns budget etotal,implicit=12eDL,TX+eDL,RXSFN+12eUL,TX+eUL,RXSRS+eQ,ReferenceTimeInfo - For RTT-based PDC: the following error tests against 275ns budget etotal,RTT-based=eDL,TX+eDL,RXSFN+eUL,TX+eUL,RXSRS+eDL,TX+eDL,RXDLRS+einconsistRTT2 - For TA-based PDC: the following error tests against 275ns budget etotal,TA-based=eDL,TX+eDL,RXSFN+eUL,TX+eUL,RXSRS+eDL,TX+eDL,RXDLRS+eTA2 - There are some cases, such as narrow BW, both RTT-based PDC and TA-based PDC can fail, but implicit PDC can meet Uu budget. - We answered all raised questions/concerns in the past meetings, and so far there is no comment from other companies indicating pending technical concern. If people cannot show unsolvable technical concern, implicit PDC should be maintained as a candidate for RAN1 to consider. |
| Qualcomm | No. |
| Ericsson | No |

Question 4.4-2: Do you have any further comment/question/views on implicit PDC based on the previous discussions and the latest contribution from OPPO?

| Company | View |
| --- | --- |

Question 4.4-3: Do you have any good idea on how to handle implicit PD?

| Company | View |
| --- | --- |
| OPPO | We believe the fair approach is to get all PDC solutions on the table to compare. But so far what we see is that the implicit PDC is not even given a chance for comparison. For example, when FL mentions the observation 2.2.1.1-3 that RTT-based PDC needs smaller BW than TA-based PDC, could implicit PDC be in the same comparison? If no, what is the reason to refuse a comparison with a solution that has the advantage in the comparison? |

Feature lead: Based on the current inputs, companies don’t want to continue the discussion of implicit PDC, as companies commented that we should focus on those with more support since there is no sufficient time left. From feature lead perspective, really appreciate the effort and analysis from OPPO to submit implicit PDC as one of the solutions to be considered, however since till now only 1 company has interest in it, it is expected more discussions won’t help. Maybe it can be considered again in future release if there is chance.

Way forward on PDC in RAN1 for Rel-17

Companies show views about how to move forward on PDC in RAN1 for Rel-17

Nokia (R1-2111141) As seen from the analyses, either RTT or TA-based PDC can satisfy the synchronicity budget for the control-to-control use case under certain conditions. So, a decision on which method should be specified for covering the strictest use case should take into account other factors, such as, overhead of the method, power limitation, complexity, specification effort, etc. In our view, enhanced TA procedure has no advantage compared to RTT based PDC considering that a dedicated reference signal and also a dedicated indicator is needed for the enhanced TA method. Additionally, as can be seen from the analyses, RTT can achieve a higher accuracy with same or lower reference signal bandwidth than enhanced TA. This can be translated to a lower ‘sounding’ overhead for RTT method, and a better multiplexing capability when providing service to multiple UEs within the coverage area. Lower power limitation constraint is also a plus for RTT method, given that lower bandwidth reference signals can be used, so it is more likely that cell edge UEs have sufficient power budget when transmitting the UL reference signal. At the same time, an enhanced TA based PDC will require more specification effort as there are not yet available accuracy requirements for a reduced Te, and a separate TA signalling need to be introduced only for the sake of PDC in a way that it does not affect the normal transmit timing procedure and requirements. While for RTT there is Rx-Tx specification from positioning that just need to be extended for PDC, i.e., including the support for the reference signals for PDC and Rx-Tx report. Observation 2.3.1: Between enhanced TA and RTT based PDC, the latter has better accuracy, lowest overhead, better coverage and is less complex to specify by reusing definitions from positioning specifications. For covering the less strict time synchronization use cases, legacy TA based PDC is obviously the simplest method. It does not require any additional measurement procedure or any complex capability in UE side, as every UE should already be able to obtain TA and, from that, determining propagation delay is straightforward. It also has no additional overhead, as it does not require any additional reference signal and report exchange. Moreover, it only requires RAN2 specification effort to enable/disable legacy TA based PDC. Observation 2.3.2: Legacy TA based PDC reuses existing capability of UEs, has no overhead, has the lowest specification effort, and is sufficient to satisfy the synchronicity budget of less strict use cases as concluded in RAN1. Not supporting Legacy TA based PDC should be carefully justified. Proposal 2.3.1: RAN1 to support Legacy TA-based PDC to cover the less strict use cases with the zero overhead and support RTT-based PDC method for covering the strictest time synchronization use cases.

LGE (R1- 2112055) Observation #1: Based on reply LS from RAN4, discuss to support either RTT-based PDC or TA-based PDC for minimal specification effort in RAN1#107-e.

Qualcomm (R1-2112212) Proposal 1: TA based propagation delay compensation is not considered for enhancement for propagation delay compensation. Proposal 2: RTT based propagation delay compensation (option 2) is good candidate for propagation delay compensation.

Feature lead: As expected, we need to make decision on what enhanced PDC method to support in this meeting. For TA-based PDC, there is no consensus in RAN4 on the reduced Te value unfortunately. I don’t expect that RAN1 can conclude on the reduced Te too. Without the reduced Te, I expect that it would be very difficult to agree the support of TA-based PDC. For implicit PDC, based on the previous discussions, it seems no way to agree on it either. From moderator perspective, it seems RTT-based PDC is the one that we may be able to go. Hopefully companies can be flexible and constructive, considering that this is the last meeting in Rel-17.

Some initial comparison between TA-based PDC and RTT-based PDC are summarized as below also.

- Support Legacy TA-based PDC to cover the less strict use cases

  - Nokia, NSB

  - Reasons

    - Worst performance

      - Can only meet the synchronization budget of less strict use cases

    - Least specification effort

      - does not require any additional measurement procedure

      - only requires RAN2 specification effort to enable/disable legacy TA based PDC

    - Least additional signaling/RS overhead

      - No additional overhead

    - Least UE complexity

      - does not require any complex capability in UE side, as every UE should already be able to obtain TA

- Support RTT-based PDC

  - Nokia, NSB, Ericsson

  - Reasons

    - Best performance

      - Higher accuracy with the same or lower reference signal bandwidth compared to TA-based PDC

      - Better multiplexing capability when providing service to multiple UEs

      - Lower power limitation constraint due to lower bandwidth reference signals

    - Lower Signaling/RS overhead compared to enhanced TA-based PDC

      - Less RS overhead required to achieve similar performance

    - Comparable specification effort compared to TA-based PDC

      - Require additional measurement procedure for Rx-Tx time difference

      - Define signalling for reporting Rx-Tx time difference

      - Define a dedicated reference signal for RTT measurement

    - More complexity compared to legacy TA-based PDC

- Support enhanced TA-based PDC

  - ZTE,

  - Reasons

    - Medium Performance

      - Lower accuracy with the same or lower reference signal bandwidth compared to TA-based PDC

      - Higher accuracy than the legacy TA-based PDC

    - Higher Signaling/RS overhead compared to RTT-based PDC

      - Higher RS overhead expected

    - Comparable specification effort compared to RTT-based PDC

      - Define accuracy requirements for a reduced Te in RAN4

      - Define a separate TA signalling for PDC

      - Define a dedicated reference signal for enhanced Te

    - More complexity compared to legacy TA-based PDC

First round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Proposal 2.4.1-1: For Rel-17 PDC,

- Support legacy TA-based PDC for at least the less strict time sync use cases, e.g. smart grid scenario

- Support RTT-based PDC method for at least the strictest time synchronization use cases, e.g. control-to-control scenario

| Company | View |
| --- | --- |
| Feature lead | Based on the current situation, it seems the proposal above is the potential compromised way we can do. Note that for legacy TA-based PDC, there might be no RAN1 impact, but there should be some RAN2 impact, e.g. enable/disable legacy TA based PDC at the UE side |
| Nokia, NSB | Support It is clear to us that RTT is to be preferred over enhanced TA based method to deliver accuracies required by the most challenging use cases. But we also need to remember that the RTT method still provies a large overhead compared to legacy TA, when the accuracy enhancement is not needed. In fact this would be the case for the majority of use cases in 22.104. So from our point of view, this is a very reasonable agreement, and despite supporting legacy TA only required RAN2 specification, it is important that RAN1 lets RAN2 know that it is to be supported along side with RTT based PDC. |
| ZTE | We don’t support this proposal especially the second bullet. First, it should be noted the legacy TA-based PDC can be used for the less strict use cases even without this proposal since TA-based PDC has already supported in Rel-16 and it can also be used in Rel-17. [Feature lead]: As I expressed above, RAN2 enhancements needed also for the legacy TA-based PDC, e.g. enable/disable legacy TA based PDC at the UE side. If enhancements needs to be done in RAN2, then better to include it here to give a whole picture. Of course, if people has concern, we can consider to only conclude the enhanced method from RAN1 perspective, and leave it to RAN2 to decide whether to do RAN2 enhancements for legacy TA-based PDC. Based on the observation above, both TA-based PDC and RTT-based PDC can satisfy the requirement. Therefore, a unified solution is better from the standard effort and the network/UE implementation perspective. For RTT-based PDC, a new measurement type is needed for both UE and gNB. In addition, a new RS type should be supported by the UE if PRS is used. This requires more UE capability. For TA-based PDC, all the related RS can have multiple functions. For example, the TRS should be configured anyway for better time and frequency synchronization. So it may not increase the resource overhead. Therefore, we think TA-based PDC should be considered first to keep a unified solution. [Feature lead]: Please check my summary on the pros and cons for different candidate solutions above also. As to RS, for TA-based PDC, you need some PDC specific configuration also if you want to ensure the performance. Therefore, for the strict use case if we can only pick one then RTT-based PDC seems better. |
| OPPO | Do not support. If the inconsistent RTT measurement in RTT-based PDC is not solved, our analysis shows RTT-based PDC has no better capability than TA-based PDC in meeting Uu error budget, even if there is no enhancement to TA granularity. To be more specific, [Feature lead]: If RTT-based PDC is picked, for sure the inconsistent RTT measurement will be solved. Both TA-based and RTT-based PDC can have a unified total error formulated as: etotal,TA/RTT=eDL,TX+eDL,RXSFN+eUL,TX+eUL,RXSRS+eDL,TX+eDL,RXDLRS+X2 where X is - half TA granularity (8*64Tc=260ns for 15kHz and 4*64Tc=130ns for 30kHz) for TA-based RTT, and - at least size of UE autonomous TA adjustment (5.5*64Tc=179ns) for RTT-based PDC. Note that error bound in terms of Te is still a upper bound, but no longer a tight bound due to use of DL-RS (RAN1 #106bis-e agreed the error formula in terms of DL-RS BW for TA-based PDC). - For 15kHz, eDL,TX+eDL,RXSFN+X/2=165+X/2 equals to 295ns for TA-based PDC and 254.5ns for RTT-based PDC, which leaves almost no room for one-way PD estimation under 275ns budget target. So two methods equally fail for almost all BWs. - For 30kHz, eDL,TX+eDL,RXSFN+X/2=165+X/2 equals to 230ns for TA-based PDC, which gives a room of 45ns for one-way PD estimation, while RTT-based PDC has the same PD error room (about 20ns) as for 15kHz SCS. In addition, according to 38.213, “If a UE has multiple active UL BWPs, as described in clause 12, in a same TAG, including UL BWPs in two UL carriers of a serving cell, the timing advance command value is relative to the largest SCS of the multiple active UL BWPs. ” So when PDC is performed on a PCell BWP with SCS=15kHz, if the UE has another UL BWP with SCS=30kHz or even 60kHz, TA-based PDC on the 15kHz PCell can still outperform RTT-based PDC which suffers the inconsistent RTT measurement. In summary, our preference for the PDC solution is: #1: Implicit PDC (due to smallest error and wide range of supported BWs) #2: RTT-based PDC with gNB/UE using the same pair of DL/UL transmissions for RTT measurements. #3: TA-based PDC w/o enhancement to Te/TA but with the assistance from SRS/CSIRS (but not PRS), which is supported by existing spec. It may not be able to support all BW for meeting Uu error budget, but RTT-based PDC other than in #2 can fail as well or even perform worse. |
| Samsung | We share similar view with ZTE. With larger BW, TA based solution can meet the requirements, and this is unified solution for all cases. There is no need to support multiple solutions for different use cases. |
| vivo | We share the similar view with ZTE and Samsung. TA-based method has been widely applied in NR and previous system and has less impact on gNB implementation and UE capability requirement. [Feature lead]: To ensure the performance, new procedures needed for TA-based PDC also, including defining accuracy requirements for a reduced Te in RAN 4; 2)define a separate TA signalling for PDC to match the enhanced TA command indication granularity; 3)define a dedicated reference signal for enhanced Te, e.g. TRS configuration and/or SRS configuration According to observations as discussed above, TA-based method can meet error accuracy requirement. For the larger bandwidth of RS than RTT-based method, we don’t think overhead is a serious issue for URLLC use case. We suggest a unified solution for all use cases. |
| Qualcomm | Do not support. We prefer RTT-based only solution. It is not efficient to support two solutions. [Feature lead]: Please note that TA-based PDC in the proposal is the legacy TA-based PDC, not enhanced TA-based PDC. |
| Intel | Support the proposal. We see the TA-based procedure as being fulfilling the requirements in a smaller space of conditions/parameters, while RTT-based procedure provides much better potential for the achievable error budget. |
| Ericsson | Do not support. Same view as QC that RTT-based only is simple and satisfy all requirements. We disagree with statements TA based solution can meet the requirements. The optimistic error values of enhanced TA come with the most extreme assumptions (max UL BW, max DL BW, reduced Te, 1/16 TA indication granularity). These requires introducing the whole new set of DL RS config, UL RS config, measurement accuracy requirements, signalling MAC CE, UE capability etc. This is even more difficult than RTT-based, since RTT-based method can reuse much existing material from positioning. |
| HW/HiSi | We would prefer to support enhanced TA-based PDC, but given the current situation and the feedback from RAN4 that there is no consensus on the Te, we can accept the FL proposal as a compromise |

Feature lead recommendation: The views are still controversial. To me I think proposal 2.4.1-1 is a reasonable compromised direction to go. I don’t know if any further email discussion can help if no company is willing to change their position. Maybe we can try to go step by step in the next round.

Second round discussion

Based on the inputs and analysis above, the following proposals are made for further discussion. Please all companies check the discussion/analysis above to understand the reason to make these proposals before providing your views here.

Table Comparison between TA-based PDC and RTT-based PDC

| Legacy TA-based PDC | Worst performance: Can only meet the synchronization budget of less strict use cases Least specification effort: Only requires RAN2 specifications to enable/disable legacy TA based PDC at the UE side (Other methods requires this also) Least additional signaling/RS overhead: No additional overhead needed Least UE complexity: does not require any complex capability at UE side, as every UE should already be able to obtain TA |
| --- | --- |
| Enhanced TA-based PDC | Medium performance: Lower accuracy with the same or lower reference signal bandwidth compared to RTT-based PDC, and higher accuracy than the legacy TA-based PDC Comparable or a little bit less specification effort compared to RTT-based PDC: 1)define accuracy requirements for a reduced Te in RAN 4; 2)define a separate TA signalling for PDC; 3)define a dedicated reference signal for enhanced Te; Higher RS overhead compared to RTT-based PDC due to higher bandwidth required under the similar performance More complexity compared to legacy TA-based PDC |
| RTT-based PDC | Best performance: 1) Higher accuracy with the same or lower reference signal bandwidth compared to enhanced TA-based PDC; 2) better multiplexing capability when providing service to multiple UEs; 3) Lower power limitation constraint due to lower bandwidth reference signals Comparable specification effort compared to RTT-based PDC: 1) require additional measurement procedure for Rx-Tx time difference; 2) define signalling for reporting Rx-Tx time difference; 3) define a dedicated reference signal for RTT measurement; Lower RS overhead: Less RS overhead required to achieve similar performance More complexity compared to legacy TA-based PDC |

Revised Proposal 2.4.2-1: For Rel-17 PDC,

- Option 1: Support enhanced TA-based PDC method (e.g. with reduced Te and enhanced TA command indication granularity) at least for the strictest time synchronization use cases, e.g. control-to-control scenario

  - Support: ZTE, Vivo, Samsung, Huawei/HiSilicon (first preference), MediaTek, H3C

- Option 2: Support RTT-based PDC method at least for the strictest time synchronization use cases, e.g. control-to-control scenario

  - Support: Qualcomm, Ericsson, Nokia/NSB, Intel, Huawei/HiSilicon (can accept)

If you want to change your position or if you have different angle to compare option 1 and option 2, please provide your comments here, otherwise no need to repeat your arguments in the first round email discussion.

| Company | View |
| --- | --- |
| Samsung | We suggest to specify both, as optional features, to let UE to choose one over the others, because either option has its own pro/cons. For example, for a UE who may already support PRS might prefer to support RTT based PDC. For a UE who may not like to implement PRS for DL timing estimation, enhanced TA-based methods with larger DL/UL BW can still work. |
| Nokia/NSB | Still same opinion (Option 2). In contrast to Samsung, we don’t think both should be supported. This will just lead to market fragmentation unnecessarily. We just need to down-select here! |
| ZTE | We still prefer Option 1. And we can also accept the suggestion from Samsung to support both schemes and it is up to UE to select one of them. |
| H3C | Considering limited time on Rel-17, we support Option 1 with minimum spec impact. |
| Ericsson | Option 2 We also do not support specifying two methods for control-to-control. A method that can support all Rel-17 use cases and allow future enhancement should be adopted. |
| OPPO | It is not clear to us what “enhanced TA-based PDC” mean. It can be either of the following: - 1st: TA-based PDC with reduced Te and/or reduced TA granularity; - 2nd: TA-based PDC with legacy Te and legacy TA granularity but relying on DL/UL RS for both SFN timing detection and TA control/measurement. We support the 2nd one above. We even think the 2nd one above is already allowed by existing spec. We do not see a chance that the inconsistent RTT measurement issue in 2.2.6 can be solved completely. If so, there is no much difference of capability to meet Uu error budget between RTT-based PDC and the 2nd TA-based PDC above. So we do not support RTT-based PDC which has tons of spec impacts. [Feature lead]: enhanced TA-based PDC here means the first one above, i.e. with reduced Te and enhanced TA command indication granularity. I revised the proposal above to further clarify. For the inconsistent RTT measurement issue, you can see that both options under the discussion will avoid inconsistent RTT measurement, the controversial point so far is which option to take. |
| Qualcomm | Option 2 |

Proposal 2.4.2-2: For Rel-17 PDC, support legacy TA-based PDC with potential RAN2 enhancements (e.g. enable/disable UE-side legacy TA-based PDC) for the less strict time sync use cases, e.g. smart grid scenario.

  - Support: Samsung, Nokia/NSB, MediaTek, ZTE, H3C, Huawei/HiSilicon, Vivo

  - Not Support: Ericsson, OPPO, Qualcomm

| Company | View |
| --- | --- |
| Feature lead | Since either enhanced TA-based PDC or RTT-based PDC will bring additional complexity compared to legacy TA-based PDC, for the scenarios with less strict time sync requirement, it is reasonable to just use legacy TA-based PDC with the potential enhancements in RAN2. If we cannot make decision in RAN1, we can leave it to RAN2 since the potential enhancements in R17 for legacy TA-based PDC is in RAN2, and there is no RAN1 impact. |
| Samsung | Support. |
| Nokia/NSB | Strongly support. |
| MediaTek | Support |
| ZTE | We support this proposal. |
| H3C | We support FL proposal. |
| Ericsson | Do not support. It is simpler jut to adopt one PDC method which can support both control-to-control and smart grid scenarios. [Feature lead]: The support of legacy TA-based PDC for less strict time sync use case won’t bring much additional complexity on top of the support of one enhanced method for the strictest time synchronization use cases, as described above only requires RAN2 specifications to enable/disable legacy TA based PDC at the UE side, which anyway will be supported for either enhanced TA-based PDC or RTT-based PDC. Meanwhile, with it there is no need to support RTT-based PDC or TA-based PDC if the use case is only smart grid, which can reduce the complexity of the UE. |
| OPPO | We do not think RAN1 needs agreement for a legacy solution. The proposal says something already supported by current spec. [Feature lead]: As explained above, the enhancements is mainly at the RAN2 side, here just give a whole picture on the PDC methods used in Rel-17. |
| Hw/HiSi | Support |
| vivo | Support. |
| Qualcomm | Do not support. It does not make sense to support two schemes for two scenarios since it make the standard more complicate. [Feature lead]: Please check my replies to Ericsson above, the support of legacy TA-based PDC won’t result in more complicate. |

Feature lead: If we cannot achieve consensus on proposal 2.4.2-2 above, alternatively we can make the following proposal to inform RAN2.

Alternative Proposal 2.4.2-2: It is up to RAN2 on whether to support legacy TA-based PDC with potential RAN2 enhancements (e.g. enable/disable UE-side legacy TA-based PDC) for the less strict time sync use cases, e.g. smart grid scenario.

LS to RAN2/RAN4 on propagation delay compensation

A LS on propagation delay compensation is needed from the following aspects:

- According to the agreement below, once RTT-based PDC is agreed, we need to send LS to RAN4 to ask them define the Rx-Tx time difference measurement accuracy.

Agreement

Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

- gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

- Based on the reply LS from RAN4 as shown in R1-2112594 and also copied below, from the part highlight in yellow RAN4 will continue work on enhanced TA-based PDC. Since now enhanced TA-based PDC will be precluded, it is better for us to let them know the status.

Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most. Answer: - There is no consensus in RAN4 whether it is feasible to support a smaller value than the current Te assuming the existing conditions in TS 38.133 for Te requirement. - RAN4 considers that it is feasible to support a smaller Te value than the current Te for the use of propagation delay compensation under the assumption of using TRS (or other RS used for Te estimation) instead of SSB. - A smaller Te can be achieved when TRS (or other RS) bandwidth is larger than SSB bandwidth. - A Smaller Te can be achieved for UE is operating in RRC_CONNECTED mode. - There is no consensus in RAN4 about how much Te value can be reduced at most or the related conditions, which RAN4 will discuss further. - Support of Smaller Te can be defined as an optional Rel-17 capability and should not apply to all UEs. - From RAN4 view it is still being studied whether a smaller Te can be achieved for the first transmission in the DRX cycle and would like to ask RAN1 the following: - If the presence of TRS (or other RS) can be guaranteed during the DRX OFF? Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2), similar to the granularity for Rel-16 IAB based on the Timing Delta MAC CE) and related condition. Answer: - It is feasible to introduce enhanced TA command indication granularity. - It is RAN4 understanding that: - Enhanced granularity will apply for UE supporting PDC - This agreement does not impact UL timing accuracy requirement - TA command indication granularity can be reduced to 64Tc for both 15 kHz SCS and 30 kHz SCS. - RAN4 will further discuss whether there are any specific conditions to use improved TA command indication granularity

- RAN2 is also waiting for the outcome from RAN1 on the PDC methods to be supported, which will have impact on their future work. In addition, we do have several conclusions that leave some work to either RAN2 or RAN4, e.g. the reporting range of Rx-Tx time difference for RTT-based PDC.

Therefore, the following proposal is made for further check:

Proposal 2.4.3-1: Send an LS to RAN2 and RAN4 with the content including:

- The agreements made in RAN1#107-e for propagation delay compensation.

- Ask RAN4 to define the following for RTT-based propagation delay compensation:

  - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

  - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

- Inform RAN4 that enhanced TA-based with reduced Te and enhanced TA command granularity is precluded in RAN1.

| Company | View |
| --- | --- |
| Nokia/NSB | We support the proposal. Despite we send some agreements on RTT in the previous meeting to RAN2, we could consider sending all relevant PDC agreements (from previous meetings as well) to ensure that RAN2 has the complete picture about the RTT and legacy TA based methods. [Feature lead]: The reason why RAN1#107-e mentioned in the first bullet is because last meeting we already sent LS to RAN2 and CC RAN4 on the agreements achieved in the previous meetings. |
| ZTE | We are fine with this proposal. But we should resolve all the issues in section 2.2.6 successfully first. |
| OPPO | Depending on outcome of section 2.2.6, RAN4 may need to do something more, e.g., the UE Rx-Tx measurement period procedure/requirement for PDC. [Feature lead]: Yes |
| Samsung | We think LS can include all the agreement and potential request from RAN 1 perspective to other WGs. [Feature lead]: Please check my replies to Nokia |
| HW/HiSi | Support |

4th round email discussion for remaining issues on PDC

Please all companies check the discussion/analysis in section 2.2.4.2, 2.2.6.2, 2.2.7.3 and 2.4.3 to understand the reasons to make these proposals/questions before providing your views here.

Detection of Reference Time as in section 2.2.4.2

Proposed conclusion: For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP.

- Note: No RAN1 specification impact is expected

| Company | View |
| --- | --- |
| Feature lead | The conclusion should be stable based on the previous inputs under section 2.2.4.2. If you are fine with it, no need to provide inputs again, unless you have concern on it. |
| H3C | Agree |
| LG | Agree |
| Nokia, NSB | Agree / support |
| Vivo | Agree |
| ZTE | Agree |
| Intel | Agree |
| HW/HiSi | Agree |
| OPPO | Partially but not fully agree. The new note seems to suggest RAN1 is not sure whether any RAN2 spec change is needed. But in that case, how could RAN2 know the answer given any technique-driven (not signaling-design driven) RRC change should come from RAN1. To compromise Ericsson’s intention to allow RAN2 to do signaling-design perspective spec change (like reuse existing RRC signaling for new RRC IE), we would like to propose following note: - Note: No Rel-17 specification impact is expected by RAN1 to further clarify/enhance QCL/TRP relations among above signals. This note also provides a guidance to RRC discussion in RAN1. The latest FL-proposed note seems to encourage the similar discussion to happen again in RAN1’s RRC discussion. [Feature lead]: Firstly, for sure no intention to guidance to RRC discussion in RAN1, it is the common understanding in RAN1, so don’t worry too much on this. Secondly, as Ericsson explained below, usually it is not that good for RAN1 to make decision on whether to do any spec change for other working group, so let’s add RAN1 back. Thirdly, to avoid further debate on the wording of the note which is actually not critical, especially most of companies already shared the views that they agreed my original version, let’s try to minimize the change. I will update the note as below: - Note: No RAN1 specification impact is expected for this conclusion With the addition of “for this conclusion”, it is clear that it is for QCL/TRP relations among the signals. |
| Ericsson | Agree with FL version above. Also prefer FL version over OPPO version. We don’t think more editing is necessary, since the proposed is a conclusion which already implies that no more technical work is expected. We have following issues with OPPO note even when considering OPPO intention. - Change back to RAN1 spec. RAN1 is not in a position to decide what RAN2 should do with their spec. OPPO also indicates that RAN2 can do signalling design type of spec change. - Remove ‘clarify’. We think it’s a bit too confident to predict the future and already conclude that RAN1 will never clarify. RAN1 is still clarifying Rel-15 specifications. In our understanding, if something is not clear for some reason, clarification should be always allowed. This is our understanding of normal RAN1 practice, nothing specific to the PDC issue. Note: No RAN1 specification impact is expected to further enhance QCL/TRP relations among above signals. [Feature lead]: Please see my replies to OPPO above. [Feature lead]: Yes I |

How to make sure the gNB/UE measure the same RS pair (section 2.2.6.2)

Revised Proposal 2.2.6.2-1: To avoid inconsistent RTT measurement for RTT-based PDC,

- Option 2: For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC.

  - Similar as Rx-Tx time difference measurement for positioning, define UE Rx-Tx measurement period for PDC. Details are up to RAN4.

  - Note: The inconsistent RTT measurement if any is avoided by gNB/UE implementation.

| Supporting companies | Nokia/NSB, |
| --- | --- |
| Objecting companies | OPPO (We do not think any implementation in Option 2 can truly “avoid any inconsistent measurement”. This debate does not seem likely to reach a consensus in short of time and we would rather drop the whole discussion at this point. Please see our detail comments below.) |

| Company | View |
| --- | --- |
| Feature lead | Please all check my replies to your comments in section 2.2.6.2. |
| H3C | We are fine with this proposal. |
| LG | Support the proposal. |
| Nokia/NSB | We support the revised proposal. IF the discussion continues (which we think is not needed), we would suggest that we distinguish between two issues that are present in the previous discussion; - Ensuring that the both (and the latest) PRS/TRS and SRS is used for determining subframe transmission and reception timing for Rx-Tx measurement. - Ensuring that a possible issue occurring if the UE changes UL transmission timing after having determined its Tx timing in the Rx-Tx measurement, but before having received the gNB Rx-Tx measurement report. As to the first point, from our perspective this is ensured with the full procedure of both gNB and UE Rx-Tx measurements and hence no need to restrict the UE (or gNB) to change its Tx timing reference. That means that the effect of SRS transmission resolution and detection is captured in the gNB Rx-Tx and TRS/PRS in the UE. The Rx and Tx timing reference for measurements is the same (nearest subframe timing) and the UE use this transmission timing again for its SRS transmission. In short, we think the existing procedure captures both the use of the TRS/PRS and SRS (and also the latest). Further, lets recall that the existing method works sufficiently well for Positioning, and will therefore also be way sufficient for the purpose of PDC. As to the second point, this issue being addressed by RAN4 and they have already agreed on a solution when the timing changes due to a TA command as summarized with a way forward In R4-2115301 which states: - UE shall restart UE Rx-Tx time difference measurement after UE TX timing change due to TA command during the measurement period So it could make sense that we add a UE measurement period which the UE can use to access if it needs to restart the measurement due to a change on Tx timing, as mentioned by the FL. As far as we understand RAN4 continues to discuss how to handle if the UE changes Tx timing due to autonomous adjustments. |
| Vivo | We are fine with this proposal. |
| ZTE | The issue of UE burden we mentioned exists in both Option 1 and Option 2 in our view. We agree with FL that UE does not need to perform measure but just store the RS. In theory, this can reduce the UE efforts. But we are not sure how much it can reduce. Anyway, if majority of companies especially UE vendors don’t think this is an issue, we are also fine. The purpose of the this proposal is to avoid inconsistent RTT measurement. Then looking at the two options in section 2.2.6.2, Option 2 states that this is handled by gNB/UE implementation. Option 1 provides the detailed UE behavior to get the the consistent RTT measurement. From this view, Option 1 is the details solution of Option 2. So we are fine with this direction. For Option 2, we don’t think the main bullet is needed because it has stated the inconsistent RTT measurement is avoided by gNB/UE implementation. If the purpose of the main bullet is to reflect a single pair of TRS/PRS and SRS configured for RTT-based PDC. We still think it is not needed since this can be reflected by RRC signaling, i.e., only one TRS/PRS and one SRS can be configured by RRC signaling. |
| Intel | Support |
| HW/HiSi | Support |
| OPPO | The note has a tone too strong and is problematic as well. - “Issue A if any is avoided by implementation” means that issue is super-cleanly removed by implementation. But we do not see any implementation can reach that pointfor the problem concerned here. If there is any, please company supporting this note give us an implementation example, but please do not raise the example not compatible to spec, such as UE not doing any UE autonomous UL-Tx timing change. - Option 2 can hardly “avoid” inconsistent RTT measurement at least caused by UE autonomous UL-Tx timing change given gNB does not know when that happens on UE side and RTT measurements on gNB/UE sides are not coordinated. Option 2 can only “lower impacts” of inconsistent RTT measurement by some implementations. - In our understanding, if using the similar RxTx measurement procedure as in positioning, this inconsistency issue shows its maximum impact at the early phase of one UE RxTx measurement period and its impact gets faded as time goes within that measurement period. Then, this strong “if any is avoided” statement may force RAN4 to setup Rx-Tx measurement procedure for PDC differently from the existing one for positioning, which is not desirable by UE implementation. Given the current situation, we do not see it likely to reach an agreeable thing to handle inconsistent RTT measurement at any circumstance under Option 2. So we would rather suggest to stop the discussion and make the following change to proposal, basically making it irrelevant to RTT inconsistency. Revised Proposal 2.2.6.2-1: To avoid inconsistent RTT measurement for RTT-based PDC, - Option 2: For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC. - Similar as Rx-Tx time difference measurement for positioning, define UE Rx-Tx measurement period for PDC. Details are up to RAN4. - Note: The inconsistent RTT measurement if any is avoided by gNB/UE implementation. @Nokia: We agree the inconsistent RTT measurement caused by UL-Tx time change due to TA command can be solved by Rx-Tx measurement period restart, as you mentioned. But the discussion has been concerning for a while the UE autonomous UL-Tx time change (not necessarily due to TA command). [Feature lead]: We can go to the simplified version as you suggested if you do prefer this way. I understand that since you suggested this simplified version, you won’t use it as the reason to revert working assumption we made in the previous GTW. By the way, it is true that RAN4 is discussing this UE autonomous UL-Tx time change issue, and they have two options on the table. But the controversial point in RAN4 is position specific, for PDC since it is clear the DL RS and UL RS belong to the same serving cell, it seems common understanding in RAN4 that the measurement accuracy can be applied. Anyway, let’s not further discuss it in RAN1 here. |
| Ericsson | Agree with intention of the Proposal. Suggest to adopt Revised Proposal 2.2.6.2-1 by OPPO. We also think that FL version is loaded with too many things that cannot be sorted out correctly by the end of RAN1#107e. The main idea is as shown by OPPO, and it’s adequate to agree to it in this meeting. [Feature lead]: Please see my replies to OPPO above. |

How to signal the Rx-Tx time difference measurement report (section 2.2.7.3)

Proposal 2.2.7.3-1: For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling.

| Supporting companies | Nokia/NSB, Ericsson |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Based on the replies to the above conclusion, it seems almost all companies prefer RRC signalling. Therefore, let’s try see if we can make more progress here. If not, then alternatively we will go to the conclusion below. |
| H3C | We are fine with this proposal. |
| LG | We think it should be up to RAN2. However, if all companies want to go for RRC signaling, we can accept the proposal. |
| Nokia/NSB | Support the proposal. Agree with E/// in the third round discussion. |
| vivo | We are fine with this proposal. |
| ZTE | We support this proposal. |
| HW/HiSi | Would have slightly preferred to leave it TO RAN2, but we are also find with the proposal |
| OPPO | Support. |
| Ericsson | Support |

(Alternative) Proposed conclusion: The detailed signaling design (e.g. MAC CE or RRC) for Rx-Tx time difference measurement reporting for RTT-based PDC is up to RAN2.

LS to RAN2/RAN4 on propagation delay compensation (section 2.4.3)

Proposal 2.4.3-1: Send an LS to RAN2 and RAN4 with the content including:

- The agreements made in RAN1#107-e for propagation delay compensation.

- Ask RAN4 to define the following for RTT-based propagation delay compensation:

  - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

  - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

  - UE Rx-Tx measurement period for PDC.

- Inform RAN4 that enhanced TA-based with reduced Te and enhanced TA command granularity is precluded in RAN1.

| Supporting companies | Nokia/NSB, |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Please all check more background and my replies to some of your comments in section 2.4.3 |
| H3C | We are fine with FL proposal on LS to RAN4 |
| LG | We are fine with the proposal. |
| Nokia/NSB | Support the proposal. |
| Vivo | We are fine with this proposal. |
| ZTE | We are fine with this proposal. |
| Intel | In general, we are fine. |
| HW/HiSi | We are fine with the proposal |
| OPPO | Given we drop the discussion on RTT inconsistency, there is no motivation from RAN1 agreements to make RAN4 to define Rx-Tx measurement period. So the corresponding bullet can be removed to leave RAN4 with full freedom. - UE Rx-Tx measurement period for PDC. We are fine with other parts of proposal. [Feature lead]: Will update accordingly |
| Ericsson | - Agree with OPPO to remove bullet for UE Rx-Tx measurement period. - Add a bullet about PRS - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on PRS (including reuse existing spec if appropriate) - Add the same note to gNB measurement based on SRS. - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS (including reuse existing spec if appropriate) [Feature lead]: Fine to update accordingly. |

5th round email discussion for remaining issues on PDC

Please all companies check the discussion/analysis in section 2.2.4.2, 2.2.6.2, 2.2.7.3 and 2.4.3 to understand the reasons to make these proposals/questions before providing your views here.

How to make sure the gNB/UE measure the same RS pair (section 2.2.6.2)

Revised Proposal 2.2.6.2-1: For RTT-based PDC, existing definitions of UE Rx – Tx time difference (i.e. section 5.1.30 in TS 38.215) and gNB Rx – Tx time difference (i.e. section 5.2.3 in TS 38.215) are reused, with updates to reflect the single pair of TRS/PRS and SRS configured for RTT-based PDC.

  - Similar as Rx-Tx time difference measurement for positioning, define UE Rx-Tx measurement period for PDC. Details are up to RAN4.

  - Note: The inconsistent RTT measurement if any is avoided by gNB/UE implementation.

| Supporting companies |  |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Please check the replies to your comments in section 3. |

LS to RAN2/RAN4 on propagation delay compensation (section 2.4.3)

Revised Proposal 2.4.3-1: Send an LS to RAN2 and RAN4 with the content including:

- The agreements made in RAN1#107-e for propagation delay compensation.

- Ask RAN4 to define the following for RTT-based propagation delay compensation:

  - UE Rx-Tx time difference measurement accuracy based on CSI-RS for tracking

  - UE Rx-Tx time difference measurement accuracy based on PRS (including reuse existing spec if appropriate)

  - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff based on SRS (including reuse existing spec if appropriate)

- Inform RAN4 that enhanced TA-based PDC with reduced Te and enhanced TA command granularity is precluded in RAN1.

| Supporting companies |  |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Please all check more background and my replies to some of your comments in section 3 |

Draft LS on propagation delay compensation to RAN2/4

I also made R1-21xxxxx Draft LS on propagation delay compensation_v00 corresponding to revised Proposal 2.4.3-1 above for check. Since we don’t have much time left before the meeting is closed, we cannot wait after the proposal is approved. If needed, it will be updated according to the outcome of proposal 2.4.3-1 above.

| Company | View |
| --- | --- |

Detection of Reference Time as in section 2.2.4.2

Proposed conclusion: For RTT-based PDC, it is assumed that the transmission of DL TRS/PRS, UL SRS and reference time information are associated with a same TRP.

- Note: No RAN1 specification impact is expected for this conclusion

| Company | View |
| --- | --- |
| Feature lead | Please check the replies to your comments in section 3. |

How to signal the Rx-Tx time difference measurement report (section 2.2.7.3)

Proposal 2.2.7.3-1: For RTT-based propagation delay compensation, the Rx-Tx time difference is reported via RRC signaling.

| Supporting companies | Nokia, NSB, Ericsson, H3C, LG, vivo, ZTE, Huawei, HiSilicon, OPPO |
| --- | --- |
| Objecting companies |  |

| Company | View |
| --- | --- |
| Feature lead | Please check the replies to your comments in section 3. |

- References

- RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR , Nokia, Nokia Shanghai Bell

- R1-2100024 Reply LS on propagation delay compensation enhancements

- R1-2110917	Discussion on propagation delay compensation enhancements	ZTE

- R1-2111008	Remaining issues on propagation delay compensation enhancements	vivo

- R1-2111141	Discussion on enhancements for PDC and CSI	Nokia, Nokia Shanghai Bell

- R1-2111191	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

- R1-2111251	Discussion on propagation delay compensation enhancements	CATT

- R1-2111344	Enhancement for support of time synchronization	OPPO

- R1-2111492	Remaining open issues for propagation delay compensation	Intel Corporation

- R1-2111733	Discussion for propagation delay compensation enhancements	Samsung

- R1-2111926	Enhancements for support of time synchronization	Huawei, HiSilicon

- R1-2112055	Discussion on propagation delay compensation enhancements	LG Electronics

- R1-2112212	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

- 3GPP RAN1#105-e, R1-2104171, Reply LS on UE transmit timing error

- Appendix Agreements in the past meetings

RAN1#102-e

Agreements:

- Take the following use cases as the representative use cases for further study on propagation delay compensation enhancements in Rel-17.

| User-specific clock synchronicity accuracy level | Number of devices in one Communication group for clock synchronisation | 5GS synchronicity budget requirement (note) | Service area | Scenario |
| --- | --- | --- | --- | --- |
| 2 | Up to 300 UEs | ≤900 ns | ≤ 1000 m x 100 m | - Control-to-control communication for industrial controller |
| 4 | Up to 100 UEs | <1 µs | < 20 km2 | - Smart Grid: synchronicity between PMUs |

Agreements:

- ± 8∙64∙Tc/2μ8*64*Tc/2 as the TA indicating error is assumed in the evaluation.

Agreements:

For 5GS synchronicity budget requirement,

- One Uu interface is assumed for smart grid.

- Two Uu interfaces are assumed for control-to-control.

Agreements:

For BS transmit timing error, further study the following three options:

- Option 1: 65 ns

- Option 2:±130ns for the indoor scenario and ±200ns for the smart grid scenario

- Option 3:82.5 ns

Agreements:

The value defined in Table 7.1.2-1 for initial transmit timing error (Te) in TS 38.133 should be considered for evaluation of the time synchronization.

Agreements:

Asymmetry between downlink and uplink channel for control-to-control scenario is not considered.

Agreements:

100 ns is assumed for BS detecting error.

Agreements:

Timing advance adjustment accuracy defined in Table 7.3.2.2-1 in TS 38.133 is assumed for evaluation of the time synchronization.

Agreements:

Both 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization.

Agreements:

Send an LS to RAN2 with the content including

- Inform RAN2 the two representative use cases concluded in RAN1 for further study;

- Ask RAN2 for input about Uu interface error budget for each of the two use cases;

Agreements:

The following options for propagation delay compensation are further studied in RAN1

- Option 1: TA-based propagation delay

  - Option 1a: Propagation delay estimation based on legacy Timing advance (potentially with enhanced TA indication granularity).

  - Option 1b: Propagation delay estimation based on timing advanced enhanced for time synchronization (as 1a but with updated RAN4 requirements to TA adjustment error and Te)

  - Option 1c: Propagation delay estimation based on a new dedicated signaling with finer delay compensation granularity (Separated signaling from TA so that TA procedure is not affected)

- Option 2: RTT based delay compensation:

  - Propagation delay estimation based on an RAN managed Rx-Tx procedure intended for time synchronization (FFS to expand or separate procedure/signaling to positioning).

Draft LS R1-2007445 is approved, with final LS in R1-2007446.

RAN1#103-e

Agreements:

- Take 65 ns as the assumption of transmit timing error for evaluation of the overall time synchronization error for control-to-control.

- Asymmetry between downlink and uplink channel for smart grid scenario is not considered.

- TA adjustment accuracy is not considered for the evaluation of time synchronization error.

- errorBS,DL,TXerrorBS, DL, TX is included in the equation for calculating the overall time synchronization error.

Agreements:

TA adjustment accuracy is not considered for the evaluation of time synchronization error.

Agreements:

For evaluation of the overall time synchronization error for smart grid, companies can take one of the following two options as the assumption for BS transmit timing error:

- Option 1: 200 ns

- Option 2: 65 ns

RAN1#104-e

Agreements: Take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RXerrorUE, DL, RX) at the UE for evaluation of the overall time synchronization error for TA based propagation delay compensation, if downlink frame timing detection error needs to be considered separately.

- Send a LS to RAN4 to ask for clarification on whether downlink frame timing detection error is included in Te or not

  - In the LS, to include more details about option 1 (included) & option 2 (not included); also including the necessary background

- FFS whether to apply the same value to RTT-based propagation delay compensation, and the corresponding condition (if any) if the same value will be applied

Decision: As per email posted on feb 5th, the draft LS is endorsed. Final LS is approved in R1-2102245.

RAN1#104b-e

Agreements: If downlink frame timing detection error needs to be considered separately from propagation delay estimation error, take ±100 ns as the assumption for downlink frame timing detection error (errorUE,DL,RX) at the UE for evaluation of the overall time synchronization error for RTT based propagation delay compensation

Agreements: Take the following equation for evaluation of the DL propagation delay estimation error for TA based propagation delay compensation:

- Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245.

- FFS whether errorBS,DL,TX in the above equation should be included or not.

Agreements:

- Observation 1: Propagation delay compensation based on existing Rel-15/Rel-16 TA procedure and associated granularity, with no enhancements in RAN1, is sufficient for meeting the Uu interface synchronicity error budget in LS R2-2010837 for the smart grid scenario.

- Observation 2: RAN1 needs to further study and specify the feasible enhancement (if any with RAN1 spec impact) for propagation delay compensation for control-to-control scenario, in order to meet the synchronicity budget of Uu interface in LS R2-2010837.

Working assumption:

Agreement:

Take the following as the evaluation assumptions for both RTT-based PDC and TA-based PDC.

- The UE may acquire an up-to-date PD estimation after waking up from DRX. This implies that gNB may signal an update timing advance value or complete a Rx-Tx measurement procedure.

- errorUE,DL,RXerrorUE, DL, RX is based on other signals (e.g. CSI-RS) instead of SSB.

- errorBS, UL,RXerrorUE, DL, RX iss based on other uplink signals instead of contention based PRACH, e.g. SRS.

- Further study and specify new procedure/signaling (if necessary) to ensure that the PD estimation can be acquired after DRX for the adopted PDC method.

Agreement:

Existing DL reference signal(s) are used for Rx – Tx time difference estimation at UE side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- FFS whether PRS can be used for UE Rx – Tx time difference estimation or not

- FFS which DL reference signal(s) to be used if/when PRS is not used

Conclusion:

- Leave it to RAN2 to decide whether to support UE based compensation and/or gNB based compensation for any propagation delay compensation method RAN1 may adopt for Rel-17, if applicable.

RAN1#106-e

Agreement

SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

Agreement

Send LS to RAN4 to ask for feedback on the following questions:

- Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most.

- Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition.

- Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4

- Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also.

- Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information.

- Note 4: Whether RAN1 will introduce specification enhancements is still undetermined.

Agreement

If RTT-based propagation delay compensation is supported,

- CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE.

- PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE.

Agreement

Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking

- gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS

R1-2108513	Feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

Agreement

Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported.

- At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured

- At least one SRS configuration for Rx – Tx time difference estimation at gNB side

Agreement

If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least:

- gNB Rx-Tx time difference at a given granularity

- FFS whether to include SRS-Resource-ID

Agreement

Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e.

- Alt. 1:

  - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively.

  - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference

- Alt. 2:

  - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference

  - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock

Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing

R1-2108618 Draft LS on TA-based propagation delay compensation 	Moderator (Huawei)

Decision: The draft LS is endorsed with the following note

- Note: It’s pending further discussion in RAN1 whether the WA is to be confirmed including which alternative is to be selected

Final LS is approved in R1-2108635.

RAN1#106bis-e

Agreement

For evaluation of the overall time synchronization error for RTT-based propagation delay compensation,

- Alt.1 for RTT-based PDC

Agreement

For evaluation of the overall time synchronization error for TA-based propagation delay compensation,

- Alt.1 for TA-based PDC

Agreement

For evaluation of the overall time synchronization error for RTT-based propagation delay compensation with Alt.1, it is assumed that

- The UE Rx-Tx time difference measurement accuracy based on PRS defined in Table 10.1.25.2-2 in TS 38.133 v17.3.0 is taken as the reference for the UE Rx-Tx time difference measurement accuracy

- The gNB Rx-Tx time difference accuracy based on SRS for positioning defined in Table 13.2.2.2-1 in TS 38.133 v17.3.0 is taken as the reference for the gNB Rx-Tx time difference accuracy based on SRS for PDC

Agreement

For RTT-based PDC, only a single pair of CSI-RS for tracking (TRS)/PRS and SRS configuration, i.e. one CSI-RS for tracking (TRS)/PRS configuration for Rx – Tx time difference estimation at UE side and one SRS configuration for Rx – Tx time difference estimation at gNB side, is configured for PDC in Rel-17, if RTT-based PDC is supported.

Agreement

If RTT-based propagation delay compensation is supported and performed at the gNB side, the Rx-Tx measurement report provided from the UE to the gNB should include at least:

- UE Rx-Tx time difference at a given granularity

Conclusion

When evaluating enhanced TA-based PDC, there is no need to replace Te by TA adjustment error.

Agreement

Send an LS to RAN2 and CC RAN4 with the content including:

- The latest available status on PDC methods in RAN1, e.g. key agreements achieved for TA-based PDC and RTT-based PDC.

R1-2110594	Draft LS on propagation delay compensation	Huawei

Decision: The draft LS is endorsed. Final version is approved in R1-2110647.

Agreement

For evaluation and comparison of enhanced TA-based PDC and RTT-based PDC, the timing detection error = 0.5/(RS BW) = 0.5/(N_PRB*12*SCS) can be used to achieve errorUE,DL,RX and errorBS,UL,RX, if needed in the evaluation equation separately, where N_PRB is the number of PRBs of the RS bandwidth used in the detection by UE and gNB, respectively.

- Note: Detection error achieved by evaluations is not precluded if available.

Agreement

If enhanced TA-based PDC with reduced Te based on TRS is supported in Rel-17, one CSI-RS for tracking (TRS) configuration is configured for enhanced TA-based PDC.

- FFS whether/how to configure UL signal for enhanced TA-based PDC

Agreement

If enhanced TA-based PDC with enhanced TA command indication granularity is supported in Rel-17,

- The enhanced TA command indication granularity introduced for enhanced PDC is applied for PDC purpose, which doesn’t have impact on normal TA procedure, i.e. normal TA procedure will still follow the existing TA command indication granularity.

Agreement

If RTT-based propagation delay compensation is supported, the Rx-Tx time difference is reported with granularity 2k*Tc, where k is an integer satisfying 0<=k<=5.

- FFS the value of k

- FFS the reporting range of Rx-Tx time difference measurement for PDC