3GPP TSG RAN WG1 #107-e	R1-2112801

e-Meeting, 11th – 19th November, 2021

Title:	[Draft] LS on use of NCD-SSB in initial DL BWP used for paging for RedCap UE

Response to:

Release:	Rel-17

Work Item:	Support of reduced capability NR devices (NR_redcap)

Source:	Ericsson [To be RAN1]

To:	RAN2, RAN4

Cc:

Contact person:	Johan Bergman, johan (dot) bergman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN1#107-e has made the following agreements and working assumptions related to operation with a separate initial DL BWP for RedCap UEs (where the differences between the agreements for FR1 and FR2 are indicated in blue colour).

Agreement: - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

RAN1 respectfully requests RAN2 and RAN4 to provide feedback on whether the above highlighted working assumptions are acceptable from RAN2 and RAN4 perspectives, respectively.

# 2	Actions

To RAN2 and RAN4:

ACTION: 	RAN1 respectfully requests RAN2 and RAN4 to provide feedback on whether the above highlighted working assumptions are acceptable from RAN2 and RAN4 perspectives, respectively.

# 3	Dates of next TSG-RAN WG1 meetings

TSG-RAN WG1 Meeting #107bis-e	17th – 25th January 2022		Online

TSG-RAN WG1 Meeting #108-e	21st February – 3rd March 2022		Online