3GPP TSG RAN WG1 Meeting #107-e  																		 			                                                                                                               R1-2112849

e-Meeting, November 11th – 19th , 2021

Agenda Item:	8.4

Source:	Moderator (Thales)

Title:	Summary of [107-e-R17-RRC-NR-NTN] Email discussion on Rel-17 RRC parameters for NR to support NTN

Document for:	Discussion

# Introduction

There were preliminary discussions on RRC parameters for NR to support NTN [5]. In current RAN1 meeting, RRC parameters pertinent for NR NTN will be further discussed.

This document is the Summary of [107-e-R17-RRC-NR-NTN] Email discussion on Rel-17 RRC parameters for NR to support NTN

- Email discussion to start on November 15

# Timing relationship enhancements

## Related RRC parameters

Based on the agreements to date (up to RAN1#106-bis-e) and the companies proposals submitted to RAN1#107-e, an updated list of RRC parameters pertinent to NR NTN  and related to the timing relationship enhancements is given within the following table:

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CellSpecific_Koffset | New | CellSpecific_Koffset | The CellSpecific_K_offset is a scheduling offset used for the timing relationships that need to be modified for NTN [see TS 38.2xy]. The unit of K_offset is number of slots for a given subcarrier spacing of 15 kHz | [0 – 542 ms] | TBD | cell | Cell-specific | 38.331 | • For the reference subcarrier spacing value for the unit of K_offset in FR1, a value of 15 kHz is used. • FFS: FR2 | New-stable |
| K_mac | New | K_mac | K_mac is a scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH [see TS 38.2xy]. When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. The unit of K_mac is number of slots for a given subcarrier spacing. | [1 – 271 ms] | 0 | cell | Cell-specific | 38.331 | The clarifications on the usage of the parameters by “It is needed … “ would be captured in [see TS 38.2xy] If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+〖3N〗_slot^(subframe,µ)+K_mac, where µ is the SCS configuration for the PUCCH. For the reference subcarrier spacing value for the unit of K_mac in FR1, a value of 15 kHz is used. • FFS: FR2 | New-stable |
| dl-DataToUL-ACK-r17 | extension of R16 | dl-DataToUL-ACK-r17 | For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) | (0..31) |  | Per BWP | UE specific |  |  | stable |

## Company views

- Companies are encouraged to provide comments on RRC parameters list (section 2.1):

## Updated list of RRC parameters based on company views (First round of email discussions):

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CellSpecific_Koffset | The CellSpecific_K_offset is a scheduling offset used for the timing relationships that need to be modified for NTN [see TS 38.2xy]. The unit of K_offset is number of slots for a given subcarrier spacing of 15 kHz | 0 ...1023 ms |  | cell | Cell-specific | 38.331 | For the reference subcarrier spacing value for the unit of K_offset in FR1, a value of 15 kHz is used. • FFS: FR2 |
| K_mac | K_mac is a scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH [see TS 38.2xy]. When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. For the reference subcarrier spacing value for the unit of K_mac in FR1, a value of 15 kHz is used. The unit of K_mac is number of slots for a given subcarrier spacing. | 1 ...512 ms |  | cell | Cell-specific | 38.331 | The clarifications on the usage of the parameters by “It is needed … “ would be captured in [see TS 38.2xy] If a UE is provided with a K_mac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+〖3N〗_slot^(subframe,µ)+K_mac, where µ is the SCS configuration for the PUCCH. For the reference subcarrier spacing value for the unit of K_mac in FR1, a value of 15 kHz is used. • FFS: FR2 |
| dl-DataToUL-ACK-r17 | For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31) | (0..31) |  | Per BWP | UE specific |  |  |

# Enhancements on UL time and frequency synchronization

## Related RRC parameters

Based on the agreements to date (up to RAN1#106-bis-e) and the companies proposals submitted to RAN1#107-e, an updated list of RRC parameters pertinent to NR NTN and related the enhancements on UL time and frequency synchronization is given within the following table:

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TACommon | New | TACommon | TACommon is a network-controlled common TA, and may include any timing offset considered necessary by the network. The unit of TACommon is μs and it is given in step size of 4.07×10-3 μs TACommon with value of 0 is supported. | [0 ...66485757 (i.e: 0… 270.73 ms)] | 0 | cell | Cell-specific | 38.331 | In NTN, the Network may optionally indicate one or more of the following parameters: • Common TA , Common TA drift rate and Common TA drift rate variation. • FFS: Details of combination of Common TA parameters | New-stable |
| TACommonDrift | New | TACommonDrift | Indicate drift rate of the common TA The unit of TACommonDrift is μss and it given in step size of 0.2×10-3μss | [- 261935… + 261935 (i.e: --53.33 μss… +-53.33 μss) ] | 0 | cell | Cell-specific | 38.331 | In NTN, the Network may optionally indicate one or more of the following parameters: • Common TA , Common TA drift rate and Common TA drift rate variation. • FFS: Details of combination of Common TA parameters | New-stable |
| TACommonDriftVariation | New | TACommonDriftVariation | Indicate drift rate variation of the common TA The unit of TACommonDriftVariation is TBD | [0…29470 (0…0.60 μss2)] | 0 | cell | Cell-specific | 38.331 | In NTN, the Network may optionally indicate one or more of the following parameters: • Common TA , Common TA drift rate and Common TA drift rate variation. • FFS: Details of combination of Common TA parameters | New-stable |
| ServingSatelliteEphemerisStateVectorX | New | ServingSatelliteEphemerisStateVectorX | Indicate the x-coordinate of serving Satellite position state vector in ECEF (m) | [+/- 42 200 000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisStateVectorY | New | ServingSatelliteEphemerisStateVectorY | Indicate the y-coordinate of serving Satellite position state vector in ECEF (m) | [+/- 42 200 000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisStateVectorZ | New | ServingSatelliteEphemerisStateVectorZ | Indicate the z-coordinate of serving Satellite position state vector in ECEF (m) | [+/- 42 200 000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisStateVectorVx | New | ServingSatelliteEphemerisStateVectorVx | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF (m/s) | [+/- 8000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisStateVectorVy | New | ServingSatelliteEphemerisStateVectorVy | Indicate the y-coordinate of serving Satellite velocity state vector in ECEF (m/s) | [+/- 8000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisStateVectorVz | New | ServingSatelliteEphemerisStateVectorVz | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF (m/s) | [+/- 8000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisSemiMajorAxis | New | ServingSatelliteEphemerisSemiMajorAxis | Indicate the following ephemeris orbital parameter for the serving satellite: - Semi-major axis α (m) | [6500 000… 43000 000] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisEccentricityE | New | ServingSatelliteEphemerisEccentricityE | Indicate the following ephemeris orbital parameter for the serving satellite: - Eccentricity e | [0…0.015] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisArgumentOfPeriapsis | New | ServingSatelliteEphemerisArgumentOfPeriapsis | Indicate the following ephemeris orbital parameter for the serving satellite: - Argument of periapsis ω (rad) | [0…2π] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatellite EphemerisLongitudeOfAscendingNode | New | ServingSatellite EphemerisLongitudeOfAscendingNode | Indicate the following ephemeris orbital parameter for the serving satellite: - Longitude of ascending node Ω (rad) | [0…2π] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisInclinationI | New | ServingSatelliteEphemerisInclinationI | Indicate the following ephemeris orbital parameter for the serving satellite: - Inclination i (rad) | [- π/2 , + π/2] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ServingSatelliteEphemerisMeanAnomalyM | New | ServingSatelliteEphemerisMeanAnomalyM | Indicate the following ephemeris orbital parameter for the serving satellite: - Mean anomaly M (rad) at epoch time to | [0… 2π] |  | cell | Cell-specific | 38.331 |  | New-stable |
| ntnUlSyncValidityDuration | New | ntnUlSyncValidityDuration | A validity duration configured by the network for satellite ephemeris data which indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. The unit of ntnUlSyncValidityDuration is Second | { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, Infinity} |  | cell | Cell-specific | 38.331 |  | New-stable |

## Company views

- Companies are encouraged to provide comments on RRC parameters list (section 3.1):

| Companies | Comments |
| --- | --- |
| Ericsson | ntnUlSyncValidityDuration should reflect that the validity duration is used for both serving satellite ephemeris and Common TA. Right now it only mentions satellite ephemeris. the epoch time should be listed as an RRC parameter? I.e., the SFN+subframe number that are indicated. Or is that just reuse of existing parameters? |
| LG | - In row 2, reference SCS for FR2 of K_offset is FFS, but there is no description on it. - In the K_mac description in row 3, reference subcarrier spacing of 15KHz is missing. I can find it in Comment column, but prefer to have in description column as K_offset does. - For the value range of K_offset and K_mac, we are ok with delete msec. |
| ZTE | Regarding the Epoc time, the corresponding RRC parameter is definitely needed. And since we already agreed the defintion of reference point for epoch time, it is preferred to capture it in the description field of this parameter. Then, further spec impacts for others regarding this issue are not needed. Otherwise, it will be unclear for both UE and gNB Agreement The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. |
| OPPO | I guess you refer to the following agreement right? The RRC parameters are related to the explicit epoch time indication. Thus, we need one parameter for SFN number and the other one parameter for subframe number. BR, Hao Agreement • When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information. • Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted. • When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number. |

## Updated list of RRC parameters based on company views (First round of email discussions)

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TACommon | TACommon is a network-controlled common TA, and may include any timing offset considered necessary by the network. TACommon with value of 0 is supported. The granularity of TACommon is 4.07 × 10^(-3) μs Values are given in unit of corresponding granularity | 0 ...66485757 (i.e: 0… 270.73 ms) |  | cell | Cell-specific | 38.331 |  |
| TACommonDrift | Indicate drift rate of the common TA The granularity of TACommonDrift is 0.2 × 10^(-3) μs⁄s Values are given in unit of corresponding granularity | - 261935… + 261935 (i.e: --53.33 μs⁄s … +-53.33 μs⁄s ) |  | cell | Cell-specific | 38.331 |  |
| TACommonDriftVariation | Indicate drift rate variation of the common TA The granularity of TACommonDriftVariation is 0.2×10^(-4) μs⁄s^2 Value are given in unit of corresponding granularity | 0…29470 (i.e. 0…0.60 μs⁄s^2 ) |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorX | Indicate the x-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorY | Indicate the y-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorZ | Indicate the z-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorVx | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorVy | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 8000…+ 8000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisStateVectorVz | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisSemiMajorAxis | Indicate the following ephemeris orbital parameter for the serving satellite: - Semi-major axis α The unit of Semi-major axis α is m | 6500 000… 43000 000 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisEccentricityE | Indicate the following ephemeris orbital parameter for the serving satellite: - Eccentricity e | 0…0.015 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisArgumentOfPeriapsis | Indicate the following ephemeris orbital parameter for the serving satellite: - Argument of periapsis ω The unit of Argument of periapsis ω is Radian | 0…2π |  | cell | Cell-specific | 38.331 |  |
| ServingSatellite EphemerisLongitudeOfAscendingNode | Indicate the following ephemeris orbital parameter for the serving satellite: - Longitude of ascending node Ω The unit of Longitude of ascending nod is Radian | 0…2π |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisInclinationI | Indicate the following ephemeris orbital parameter for the serving satellite: - Inclination i The unit of Inclination is Radian | - π/2 ...+ π/2 |  | cell | Cell-specific | 38.331 |  |
| ServingSatelliteEphemerisMeanAnomalyM | Indicate the following ephemeris orbital parameter for the serving satellite: - Mean anomaly M [rad] at epoch time to The unit of Mean anomaly M is Radian | 0… 2π |  | cell | Cell-specific | 38.331 |  |
| ntnUlSyncValidityDuration | A validity duration configured by the network for uplink synchronization assistance information (i.e. Serving satellite ephemeris and Common TA parameters) which indicates the maximum time during which the UE can apply assistance information without having acquired new assistance information. The unit of ntnUlSyncValidityDuration is second | { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} |  | Cell | Cell-specific | 38.331 | • FFS: Additional values for GEO |
| EpochTime | Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. | 0 to 1023 to indicate SFN and 0 to 9 to indicate the sub-frame number. |  | Cell | Cell-specific | 38.331 |  |

# Enhancements on HARQ

## Related RRC parameters

Based on the agreements to date (up to RAN1#106-bis-e) and the companies proposals submitted to RAN1#107-e, an updated list of RRC parameters pertinent to NR NTN and related to the enhancements on HARQ is given within the following table:

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harq-ProcessNumberSizeDCI-1-2-r17 | extension of R16 | harq-ProcessNumberSizeDCI-1-2-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 , clause 7.3.1). | 0..5 |  | BWP | UE-specific | 38.331 |  | Stable |
| HARQ-ProcessNumberSizeDCI-0-2-r17 | extension of R16 | HARQ-ProcessNumberSizeDCI-0-2-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 , clause 7.3.1). | 0..5 |  | BWP | UE-specific | 38.331 | R17: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2 | Stable |
| HARQ-ProcessNumberSizeDCI-1-1-r17 | extension of R16 | HARQ-ProcessNumberSizeDCI-1-1-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 1_1 | 0..5 |  | BWP | UE-specific | 38.331 |  | Stable |
| HARQ-ProcessNumberSizeDCI-0-1-r17 | extension of R16 | HARQ-ProcessNumberSizeDCI-0-1-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 0_1 | 0..5 |  | BWP | UE-specific | 38.331 |  | Stable |
| nrofHARQ-ProcessesForPDSCH-r17 | extension of R16 | nrofHARQ-ProcessesForPDSCH-r17 | The number of HARQ processes to be used on the PDSCH of a serving cell. Value n2 corresponds to 2 HARQ processes, value n4 to 4 HARQ processes, and so on. If the field is absent, the UE uses 8 HARQ processes (see TS 38.214 , clause 5.1). | {n2, n4, n6, n10, n12, n16, n32} |  | BWP | UE-specific | 38.331 | The maximal supported HARQ process number is up to 32. | Stable |
| nrofHARQ-Processes-r17 | extension of R16 | nrofHARQ-Processes-r17 | The number of HARQ processes configured for UL configured grant. It applies for both Type 1 and Type 2. See TS 38.321 , clause 5.4.1 | (1..32) |  | BWP | UE-specific | 38.331 |  | Stable |
| nrofHARQ-Processes-r17 | extension of R16 | nrofHARQ-Processes-r17 | Number of configured HARQ processes for SPS DL. see TS 38.321, clause 5.8.1 | (1..32) |  | BWP | UE-specific | 38.331 |  | Stable |

## Company views

- Companies are encouraged to provide comments on RRC parameters list (section 4.1):

| Companies | Comments |
| --- | --- |
| Nokia | - Row 25 contains the configuration of the number of HARQ for DL, but there is no corresponding line for UL configuration (at least we have agreement that DCI format 0-1 is also supported for extension of HARQ processes). Is the line for UL HARQ missing? - Rows 26+27 are indicated to have the same parameter name in specifications, while the descriptor seems to indicate that these are for UL configured grant and DL SPS respectively. I would suggest that they are given unique names. - Row 29 seems to have lost its “WI code” and “subfeature group” - perhaps we can find it again 😊? Or is this because this feature is rather a RAN2 feature – in which case we should rather not have RRC parameters for this. After all, what is important for RAN1 is the knowledge of whether a specific HARQ process is feed-back disabled or not. |
| Ericsson | Regarding the configuration of the number of HARQ processes for DL and UL: In the UE features discussion for NR NTN it has been agreed to have separate UE capabilities for 32 HARQ processes in DL and UL. This is different from legacy where 16 processes are mandatory in both UL and DL. The RAN2 guideline R1-2001513 (Guidelines for UE capability definitions) states the following: 5 Avoid defining functionality that has no RRC configuration but is dependent on capability bits. The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting the reported capabilities. Therefore an RRC parameter should be added for configuration of the number of HARQ processes also for UL, to be able to configure 32 processes in one direction and 16 in the other direction. |
| Nokia | However, given that current operation for UL is that we have 16 processes, it may be appropriate if we only have the options of n16 and n32 values One minor nitpicking detail – the description uses n2 and n4. I would suggest to use the following: The number of HARQ processes to be used on the PUSCH of a serving cell. Value n16 corresponds to 16 HARQ processes, value n32 to 32 HARQ processes. If the field is absent, the UE uses 16 UL HARQ processes (see TS 38.214 , clause X.Y). |
| Apple | Regarding the parameters "nrofHARQ-ProcessesForPDSCH-r17” and "nrofHARQ-ProcessesForPUSCH-r17”, it is more flexible to introduce some possible values between 16 and 32. I could see several values below 16 for nrofHARQ-ProcessesForPDSCH-r17. As per the additional values to add, we are open, but may be 20, 24, 28 could be considered. |
| Moderator | The value range {n2, n4, n6, n10, n12, n16, n32}, was discussed and agreed in previous meeting. With max RTT up to 25.77 ms in LEO 600 km scenario, 28 HARQ processes are sufficient to avoid HARQ stalling With max RTT up to 541.46 ms in GEO 35786 km scenario, 32 HARQ processes only marginally avoid HARQ stalling. In case of 32 HARQ processes are used, the maximum peak rate due to HARQ stalling is reduced by about 94%, whereas with 16 HARQ processes it is reduced by 97%. I think Chunxuan you ‘re referring to other scenario lower than LEO 600 km? I think for time being we cannot change the values already agreed since 106-bis-e. maybe if relevant, other intermediate values could be discussed during R17 maintenance phase. |

## Updated list of RRC parameters based on company views (First round of email discussions)

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| harq-ProcessNumberSizeDCI-1-2-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 1_2 (see TS 38.212 , clause 7.3.1). | 0..5 |  | BWP | UE-specific | 38.331 |  |
| HARQ-ProcessNumberSizeDCI-0-2-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 0_2 (see TS 38.212 , clause 7.3.1). | 0..5 |  | BWP | UE-specific | 38.331 | R17: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2 |
| HARQ-ProcessNumberSizeDCI-1-1-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 1_1 | 5 |  | BWP | UE-specific | 38.331 |  |
| HARQ-ProcessNumberSizeDCI-0-1-r17 | Configure the number of bits for the field "HARQ process number" in DCI format 0_1 | 5 |  | BWP | UE-specific | 38.331 |  |
| nrofHARQ-ProcessesForPDSCH-r17 | The number of HARQ processes to be used on the PDSCH of a serving cell. Value n2 corresponds to 2 HARQ processes, value n4 to 4 HARQ processes, and so on. If the field is absent, the UE uses 8 HARQ processes (see TS 38.214 , clause 5.1). | {n2, n4, n6, n10, n12, n16, n32} |  | BWP | UE-specific | 38.331 | The maximal supported HARQ process number is up to 32. |
| nrofHARQ-ProcessesForPUSCH-r17 | The number of HARQ processes to be used on the PUSCH of a serving cell. Value n16 corresponds to 16 HARQ processes, value n32 to 32 HARQ processes. If the field is absent, the UE uses 16 UL HARQ processes (see TS 38.214 , clause X.Y). | { n16, n32} |  | BWP | UE-specific | 38.331 | The maximal supported HARQ process number is up to 32. |
| nrofHARQ-Processes-r17 | The number of HARQ processes configured for UL configured grant. It applies for both Type 1 and Type 2. See TS 38.321 , clause 5.4.1 | 1..32 |  | BWP | UE-specific | 38.331 |  |
| nrofHARQ-Processes-r17 | Number of configured HARQ processes for SPS DL. see TS 38.321, clause 5.8.1 | 1..32 |  | BWP | UE-specific | 38.331 |  |
| HARQ-feedbackEnablingforSPSactive-r17 | If enabled, UE reports ACK/NACK for the first SPS PDSCH after activation, regardless of if HARQ feedback is enabled or disabled corresponding to the first SPS PDSCH after activation Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation | BOOLEAN |  | BWP | UE-specific | 38.331 |  |
| HARQ-feedbackEnabling-disablingperHARQprocess-r17 | To enble/disable the HARQ-feedback per HARQ process | Bitmap with corresponding length determined by max(nrofHARQ-ProcessesForPDSCH-r17, nrofHARQ-Processes-r17) |  | BWP | UE-specific | 38.331 |  |

# Others

## Related RRC parameters

Based on the agreements to date (up to RAN1#106-bis-e) and the companies proposals submitted to RAN1#107-e, an updated list of RRC parameters related to the polarization indication in NR NTN is given within the following table:

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ntnPolarizationDL | New | ntnPolarizationDL | If present, this parameter indicates polarization information for Downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization | {RHCP, LHCP, Linear } |  | cell | Cell-specific | 38.331 | The indicated polarization information refers to the polarization type used at satellite side for DL transmission. | Stable |
| ntnPolarizationUL | New | ntnPolarizationUL | If present, this parameter indicates Polarization information for Uplink service link. If not and ntnPolarizationDL is present, UE assumes a same polarization for UL and DL | {RHCP, LHCP, Linear } |  | cell | Cell-specific | 38.331 | The indicated polarization information refers to the polarization type used at satellite side for UL reception. | Stable |

## Company views

- Companies are encouraged to provide comments within the following table:

| Companies | Comments |
| --- | --- |

## Updated list of RRC parameters based on company views (First round of email discussions)

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ntnPolarizationDL | If present, this parameter indicates polarization information for Downlink transmission on service link: including Right hand, Left hand circular polarizations (RHCP, LHCP) and Linear polarization | {RHCP, LHCP, Linear } |  | cell | Cell-specific | 38.331 | The indicated polarization information refers to the polarization type used at satellite side for DL transmission. |
| ntnPolarizationUL | If present, this parameter indicates Polarization information for Uplink service link. If not and ntnPolarizationDL is present, UE assumes a same polarization for UL and DL | {RHCP, LHCP, Linear } |  | cell | Cell-specific | 38.331 | The indicated polarization information refers to the polarization type used at satellite side for UL reception. |

# Reference

|  | - 3GPP TS 38.212 NR; Multiplexing and channel coding - 3GPP TS 38.331: NR; Radio Resource Control (RRC) protocol specification - 3GPP TS 38.214: "NR; Physical layer procedures for data" - R1-2108675 RRC parameter changes required for NR-NTN, up to RAN1 #106-e, Moderator (Thales) - R1-2110624 Summary of [106bis-e-R17-RRC-NR-NTN] Email discussion on Rel-17 RRC parameters for NR to support NTN |
| --- | --- |