3GPP TSG RAN WG1 #108-e			R1-2200878

e-Meeting, February 21st – March 3rd, 2022

3GPP TSG-RAN WG2 Meeting #115-e						R2-2201776

Electronic meeting, January 17th – 25th, 2022

Title:	Response LS on the reporting of the Tx TEG association information

Response to:	LS on the reporting of the Tx TEG association information (R1-2112968; contact: CATT)

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG2

To:	RAN WG1

Cc:	RAN WG4, RAN WG3

Contact Person:

#### Name:                   Jianxiang Li

E-mail Address:   lijianxiang@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

RAN2 thanks RAN1 for the LS on the reporting of the Tx TEG association information.

RAN2 discussed the parameters on Mitigation of UE Rx/Tx timing delays and Mitigation of TRP Rx/Tx timing delays indicated in RAN1 LS (R1-2112976) and found the issues listed below which will need to be confirmed by RAN1:

- These two duplicated parameters can be deleted in the RAN1 parameter table:

- numOfUERxTEG-PerPRSResource: duplicated with MeasPRSwithDiffRxTEGs_Request_RSTD

- numOfTRPRxTEG-PerPRSResource_RTOA:  duplicated with MeasPosSRSwithDiffRxTEGs_Request,

- The value range of maxNumOfUE-RxTEG is supposed to be 32, rather than as 8 in the table.

But RAN2 does not conclude that if the srs-PosResourceSetId is required or not in UE TxTEG association report.

Below please find the update from RAN2’s perspective:

| Sub-feature group | Parameter name | Description | Value range | Specification | Comment |
| --- | --- | --- | --- | --- | --- |
| Mitigation of UE Rx/Tx timing delays | [srs-PosResourceSetId] | FFS: whether there is a need to include the positioning SRS resource set ID in ueTxTEG |  | FFS for RAN2/RAN3 |  |
| Mitigation of UE Rx/Tx timing delays | maxNumOfUE-RxTEG | The maximum number of UE-RxTEG | [32 ] | FFS for RAN2 | Agreement: Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors: The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT] [32] The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT] [8] The maximum number of UE-RxTx TEGs [256] |

2. Actions:

To RAN WG1

ACTION: 	RAN2 kindly asks RAN1 to confirm the issues identified by RAN2. And RAN2 respectfully asks RAN1 to respond if srs-PosResourceSetId is required in UE TxTEG association report.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting #117-e         	February 21 – March 3, 2022		        	Electronic Meeting

TSG-RAN WG2 Meeting #118-e             May 16 – 27, 2022 				Electronic Meeting