3GPP TSG-RAN WG1 Meeting #108-e 	R1-2200922

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.5.3

Source:	Huawei, HiSilicon

Title:	Maintenance of DL-AoD enhancements

Document for:	Discussion and decision

Introduction

In this paper, we discuss the further correction to the DL-AoD enhancements.

RE-level RSRPP

As discussion in our companion paper [5], we have the following TP for modifying the DL PRS-RSRPP definition.

- Proposal 1: Adopt the following TP for DL PRS-RSRPP.

### 5.1.35 DL PRS reference signal received path power (DL PRS-RSRPP) Definition; DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power (in [W]) of the linear average of the channel response compensated by the i-th path delay on the resource elements that carry the DL PRS signals, where DL PRS-RSRPP for 1st path delay is the power corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.. Applicable for; RRC_CONNECTED, RRC_INACTIVE.

PRS-RSRPP for DL-AoD

Receiver diversity

- On the receiver diversity, two options were adopted for UL SRS-RSRPP based on the following agreement [1].

Agreement Apply the following changes to the definition for UL SRS-RSRPP in the previous agreement: Definition UL SRS reference signal received path power (UL SRS-RSRPP) is defined as the power of the received UL SRS signal configured for the measurement at the i-th path delay of the channel response, where UL SRS-RSRPP for 1st path delay is the power corresponding to the first detected path. - Working assumption: For frequency range 1, the reference point for the UL SRS-RSRPP shall be the antenna connector of the gNB. For frequency range 2, UL SRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. - FFS: For frequency range 1 and 2, if receiver diversity is in use by the gNB, the reported UL SRS-RSRPP value shall not be lower than the corresponding UL SRS-RSRPP of any of the individual receiver branches - FFS: Note: First and additional paths RSRP, when provided in the same report, use the same RX branch(es) selected for the first arrival path and for the UL SRS-RSRP if the UL SRS-RSRP is reported - FFS: whether/how to capture the note in the specifications Note: The following two options are supported by gNB to LMF: - Option 1 (RX diversity for the first path UL SRS-RSRPP) - The same RX branch(es) as applied for the first path UL SRS-RSRPP measurements are used for the additional paths UL SRS-RSRPP measurements if those are provided together - For frequency range 1 and 2, if receiver diversity is in use by the gNB for UL SRS-RSRPP measurements, then reported UL SRS-RSRPP value for the first path shall not be lower than the corresponding UL SRS-RSRPP for the first path of any of the individual receiver branches - Option 2 (RX diversity for UL SRS-RSRP) - The same RX branch(es) as applied for UL SRS-RSRP measurements are used for UL SRS-RSRPP measurements (i.e., the first and additional paths UL SRS-RSRPP if those are provided)

- We do not think what is proposed for gNB can work for UE, especially DL-AoD, where RSRPP is used to convert to the angle measurement.

We noticed that RAN4 also made the following agreements with regards to Rx branch of DL PRS-RSRPP.

Agreements: - Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement - Update the RSRPP definition to reflect that it is the path power on RE level - PRS-RSRPP measurement requirements in Rel-17 are defined for first path (i=1). - Note: RAN4 understanding that PRS-RSRPP definition is generic and applicable to all paths (i≥ 1). - Sent following LS reply to RAN1 containing above agreements: - R4-2202775, LS reply on definition of DL PRS path RSRP, Nokia

- From RAN1 perspective, for DL-AoD, ideally for the multiple Tx beams (PRS resources) from a TRP, a fixed Rx beam and a fixed Rx branch should be used to eliminate the power contribution difference on the Rx side. In this sense, Option 2 (i.e. the same Rx branch as the RSRP) should be considered for DL-AoD. In addition, given that RSRP reporting is currently mandatory for DL-AoD provided by ASN.1 structure [1].

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE { dl-PRS-ID-r16 INTEGER (0..255), nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-CellGlobalID-r16 NCGI-r15 OPTIONAL, nr-ARFCN-r16 ARFCN-ValueNR-r15 OPTIONAL, nr-DL-PRS-ResourceID-r16 NR-DL-PRS-ResourceID-r16 OPTIONAL, nr-DL-PRS-ResourceSetID-r16 NR-DL-PRS-ResourceSetID-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-DL-PRS-RSRP-Result-r16 INTEGER (0..126), nr-DL-PRS-RxBeamIndex-r16 INTEGER (1..8) OPTIONAL, nr-DL-AoD-AdditionalMeasurements-r16 NR-DL-AoD-AdditionalMeasurements-r16 OPTIONAL, ... }

- Proposal 2: Option 2 (the same Rx branch as the DL PRS-RSRP) should be adopted for DL-AoD DL PRS-RSRPP reporting.

Absolute and differential values

According to RAN4 reply LS [2], either absolute RSRPP (reusing the same absolute RSRP mapping table) and differential RSRPP (reusing the same differential RSRP mapping table) is possible.

The PRS-RSRPP can be reported in ways below : - PRS-RSRPP may be reported by reusing absolute and differential PRS-RSRP measurement report mapping tables in TS38.133 clause 10.1.24.3.1 and 10.1.24.3.2 respectively. - When differential reporting is used, PRS-RSRPP is reported as the difference in dB with respect to a reference measurement. RAN4 understands that it is up to RAN1/2 to decide what reference measurement would be for the PRS-RSRPP differential reporting. RAN4 concludes that these ways are possible to support PRS-RSRPP in Rel-17.

For DL-AoD, the current coding scheme for RSRP is that

nr-DL-PRS-RSRP-Result is with the absolute value

nr-DL-PRS-RSRP-ResultDiff is with the single-sided differential value, where the reference is nr-DL-PRS-RSRP-Result.

RSRPP reporting should be adopted in the same way.

- Proposal 3: Support the following encoding scheme for DL PRS-RSRPP in DL-AoD.

The main RSRPP value that serves as the reference for all the remaining RSRPP values is reported with the absolute value using 7 bits according to Table 10.1.24.3.1-1 of TS 38.133, which is the maximum RSRPP for the first path among all the reported DL PRS resources.

The remaining RSRPP values are reported with the single-sided differential value using 5 bits according to Table 10.1.24.3.2-1 of TS 38.133.

PRS-RSRPP for DL-TDOA and Multi-RTT

RAN1 also agreed to extend the RSRPP reporting for DL-TDOA, Multi-RTT and for the first path and additional paths.

The following two issues for DL-TDOA and Multi-RTT need further consideration.

Receiver diversity

The receiver diversity may be further affected by the existing TEG framework as discussed in our companion paper [3].

If TOA measurements for the same PRS resource using different Rx TEGs are reported, it is possible that only a single RSRPP reporting is reported on a specific Rx branch/Rx TEG for the target PRS resource.

We understand that the motivation of reporting the TOA measurement for the same PRS with multiple Rx TEGs is for the purpose of estimating inter-TEG timing error by the LMF, where signal strength (first path RSRPP) should be considered when determining whether the TOA measurement is reliable or not, and thus reporting PRS-RSRPP associated with multiple Rx TEGs for the same target PRS is useful.

- Observation 1: For DL-TDOA and Multi-RTT, it is beneficial to report the PRS-RSRPPs associated with the TOA measurements in different Rx TEGs for the same target PRS.

Then the requirement of Rx branch selection in the PRS-RSRPP reporting should be relaxed, and UE may be configured to report the PRS-RSRPP for one or multiple individual Rx branches for the same target PRS.

- Proposal 4: For DL-TDOA and Multi-RTT, when receiver diversity is in use, support UE to report PRS-RSRPP for multiple Rx branches for the same target PRS, if the TOA measurement for the target PRS is associated with multiple Rx or RxTx TEGs.

- For additional paths RSRPP reporting for DL-TDOA and Multi-RTT, we think the same Rx branch and potentially the same Rx TEGs should be assumed as the first path.

- Proposal 5: For DL-TDOA and Multi-RTT, when receiver diversity is in use, the Rx branch for the RSRPP associated with the additional paths should be the same as that associated with the first path.

Absolute and differential values

For DL-TDOA/Multi-RTT, the current coding scheme for RSRP is that

nr-DL-PRS-RSRP-Result is with the absolute value

nr-DL-PRS-RSRP-ResultDiff is with the double-sided differential value, where the reference is nr-DL-PRS-RSRP-Result.

RSRPP reporting for the first path could be adopted in the same way.

The current coding scheme for additional TOA is that

Only differential values is reported with single reference timing for the TRP, regardless of with which DL PRS resource the additional is associated to avoid intra-TRP 2-stage differential mapping as shown in Figure 1.

In specification, the field description for additional path is captured as follows for DL-TDOA and Multi-RTT.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].. nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.. nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].. nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values..

Reference Path

Reference TRP

Path#1

Neighbouring TRP

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Path#1

Path#1

Path#2

Path#2

Resource#0

Resource#1

Resource#2

Resource#0

Resource#1

Resource#2

RSTD

Figure 1 Additional path timing reporting in Rel-16

- It is preferred that a single reference for PRS-RSRPP is adopted for a TRP similar to Rel-16.

- Proposal 6: Support the following encoding scheme for DL PRS-RSRPP in DL-TDOA and Multi-RTT.

The main RSRPP values that serves as the reference for all the remaining RSRPP values (including the additional resource and additional paths) is reported with the absolute value using 7 bits according to Table 10.1.24.3.1-1 of TS 38.133, which is the RSRPP for the first path for a main DL PRS resource.

The remaining RSRPP values are reported with the double-sided differential value using 6 bits according to Table 10.1.24.3.2-2 of TS 38.133, including the additional paths for the main DL PRS resource, and the first path and additional paths for the other DL PRS resources.

TOA reporting for DL-AoD

During RAN1#107-e discussion [4], we used the following figure to show the necessity of TOA reporting for DL-AoD even for the first path.

For the case of a strong multiple, the Tx beam will have impact on the power of each individual path. As shown in Figure 2, where there is a strong LoS path and a group of moderate NLoS clusters. The beam direction for resource #2 matches the LoS path, showing the purple peak, while the beam direction for resource #1 matches the NLoS clsuters, and the meantime suppresses the power of the LoS path, showing a moderate green peak and even smaller green precursor. For resource #1, the figure shows two alternatives on the first path reporting, and there is no way that UE indicates whether Alt.1 or Alt.2 is selected, and if UE selects Alt.1, the AoD positioning will be severely deteriorated.

A simple way to combat this is to allow the first path TOA reporting for different PRS resources, so that network may have the chance to check whether the path is really the same first path as main PRS resource. We understand the introducing TOA reporting for DL-AOD still maintains the difference between DL-AoD and other timing based positioning method, in that DL-TDOA and multi-RTT still use the TOA of the global first path for the TRP, which does not exist for DL-AoD.

Another alternative way is to enforce a reasonable UE implementation to judge the first path trimming in the DL-AoD reporting.

First path of resource #2

Alt.1 first path of resource #1

Alt.2 first path of resource #1

Figure 2 CIR under different beams

- Proposal 7: Support either one of the following

Option. 1 For each additional first path PRS-RSRPP measurement reporting for DL-AoD, UE also reports the relative TOA for the first path to the first path of the reference PRS.

Option. 2 UE is expected to ensure that the first path PRS-RSRPP measurement reporting for DL-AoD across the reported DL-PRS resources are from the same path.

gNB beam pattern

- In RAN1#107-e, we reached the following agreement for DL-AoD beam pattern for UE-based positioning.

Agreement From the RAN1 perspective, for the TRP beam/antenna information to be optionally provided by the LMF to the UE for UE-based DL-AoD: - The LMF provides the quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - Note: the peak power per angle is not provided - Note: up to RAN3 to decide how the TRP beam information is provided to the LMF for both UE-assisted and UE-based - Send an LS to RAN2/RAN3 to decide on the signaling details

- In UL AoA agenda, we agreed that linear array can be used, which in turn further motivates the same enhancements for DL-AoD beam pattern.

- Proposal 8: For linear array, the angle entry in the gNB beam pattern can be ZoA only in the local coordinate system.

Send an LS to RAN2 to allow them to consider the signaling optimization.

- Regarding quantization of power difference, we believe this existing differential RSRP mapping for DL-AoD measurement reporting can be reused, i.e. 5 bits to denote the difference of 0dB to -31dB, as further lower differential RSRP values may be more prone to noise contamination.

- Proposal 9: The power difference between the target PRS and the peak power PRS is quantized using the existing 5-bit single-side differential mapping table.

Angle search window

- In RAN1#107-e, we reached the following agreement for the DL-AoD angle search window.

Agreement For the purpose of both UE-B and UE-A DL-AoD, and with regards to the support of AOD measurements with an expected uncertainty window, the following is supported - Indication of expected angle value and uncertainty (of the expected azimuth and zenith angle value) range(s) is signaled by the LMF to the UE - The type of expected angle and uncertainty can be requested by the UE, between the following options - Option 1: Indication of expected DL-AoD/ZoD value and uncertainty (of the expected DL-AoD/ZoD value) range(s) is signaled by the LMF to the UE - Option 2: Indication of expected DL-AoA/ZoA value and uncertainty (of the expected DL-AoA/ZoA value) range(s) is signaled by the LMF to the UE

- We think this feature is useful for both PRS resource measurement prioritization and UE Rx beamforming (if UE can know its orientation information) without Rx beam training. In fact, this can also be used for assisting NLoS detection.

Figure 3 Illustration of using angle search window for NLoS detection

As shown in Figure 3, the LMF sends the angle search window based on LoS propagation between a TRP and the target UE uncertainty shape. However if there is an unknown obstacle in between that blocks the LoS path, the Rx power may be attenuated at the angle, and this in terms may also help LMF determine the LoS condition. Therefore, we think that this should be useful for DL-TDOA and Multi-RTT.

For the angle search window in the form of DL-AoA, since LMF may not need to be aware of the UE orientation, it should be expressed in global coordinate system only. UE requesting such information from the LMF is responsible for converting the angle to local coordinate system using its own orientation information.

In addition, the UE may indicate whether the PRS is received within the angle search window as the confirmation to the LMF.

- Proposal 10: The feature of angle search window also applies for DL-TDOA and Multi-RTT.

- Proposal 11: The expected DL-AoA/ZoA should only be expressed in the global coordinate system.

- Proposal 12: Support UE to send the confirmation in the measurement report to the LMF for a TRP whether the PRS are received within the angle search window.

Conclusion

In this contribution, we have the following observations and proposals with regards to maintenance of DL-AoD enhancements.

- Observation 1: For DL-TDOA and Multi-RTT, it is beneficial to report the PRS-RSRPPs associated with the TOA measurements in different Rx TEGs for the same target PRS.

- Proposal 1: Adopt the following TP for DL PRS-RSRPP.

### 5.1.35 DL PRS reference signal received path power (DL PRS-RSRPP) Definition; DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power (in [W]) of the linear average of the channel response compensated by the i-th path delay on the resource elements that carry the DL PRS signals, where DL PRS-RSRPP for 1st path delay is the power corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch.. Applicable for; RRC_CONNECTED, RRC_INACTIVE.

- Proposal 2: Option 2 (the same Rx branch as the DL PRS-RSRP) should be adopted for DL-AoD DL PRS-RSRPP reporting.

- Proposal 3: Support the following encoding scheme for DL PRS-RSRPP in DL-AoD.

The main RSRPP value that serves as the reference for all the remaining RSRPP values is reported with the absolute value using 7 bits according to Table 10.1.24.3.1-1 of TS 38.133, which is the maximum RSRPP for the first path among all the reported DL PRS resources.

The remaining RSRPP values are reported with the single-sided differential value using 5 bits according to Table 10.1.24.3.2-1 of TS 38.133.

- Proposal 4: For DL-TDOA and Multi-RTT, when receiver diversity is in use, support UE to report PRS-RSRPP for multiple Rx branches for the same target PRS, if the TOA measurement for the target PRS is associated with multiple Rx or RxTx TEGs.

- Proposal 5: For DL-TDOA and Multi-RTT, when receiver diversity is in use, the Rx branch for the RSRPP associated with the additional paths should be the same as that associated with the first path.

- Proposal 6: Support the following encoding scheme for DL PRS-RSRPP in DL-TDOA and Multi-RTT.

The main RSRPP values that serves as the reference for all the remaining RSRPP values (including the additional resource and additional paths) is reported with the absolute value using 7 bits according to Table 10.1.24.3.1-1 of TS 38.133, which is the RSRPP for the first path for a main DL PRS resource.

The remaining RSRPP values are reported with the double-sided differential value using 6 bits according to Table 10.1.24.3.2-2 of TS 38.133, including the additional paths for the main DL PRS resource, and the first path and additional paths for the other DL PRS resources.

- Proposal 7: Support either one of the following

Option. 1 For each additional first path PRS-RSRPP measurement reporting for DL-AoD, UE also reports the relative TOA for the first path to the first path of the reference PRS.

Option. 2 UE is expected to ensure that the first path PRS-RSRPP measurement reporting for DL-AoD across the reported DL-PRS resources are from the same path.

- Proposal 8: For linear array, the angle entry in the gNB beam pattern can be ZoA only in the local coordinate system.

Send an LS to RAN2 to allow them to consider the signaling optimization.

- Proposal 9: The power difference between the target PRS and the peak power PRS is quantized using the existing 5-bit single-side differential mapping table.

- Proposal 10: The feature of angle search window also applies for DL-TDOA and Multi-RTT.

- Proposal 11: The expected DL-AoA/ZoA should only be expressed in the global coordinate system.

- Proposal 12: Support UE to send the confirmation in the measurement report to the LMF for a TRP whether the PRS are received within the angle search window.

- Reference

- TS 37.355, LTE Positioning Protocol (LPP).

- R1-2200905, Reply LS on reporting and definition of DL PRS path RSRP, RAN4 (Nokia), RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar., 2022.

- R1-2200920, Maintenance of Rx/Tx timing error, Huawei, HiSilicon, RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar., 2022.

- R1-2112643, FL summary #3 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions, Moderator (Ericsson), RAN1#107-e, e-Meeting, 11th November – 19th November, 2021.

- R1-2202461, Discussion on DL PRS-RSRPP definition, Huawei, HiSilicon, RAN1#108-e, e-Meeting, 21 Feb. – 3 Mar., 2022.