3GPP TSG-RAN WG1 Meeting #108-e 	R1-2200923

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.5.4

Source:	Huawei, HiSilicon

Title:	Maintenance of PRS measurement outside MG

Document for:	Discussion and decision

# Introduction

In this paper, we will discuss the remaining issues of latency improvement, particularly related to PRS measurement outside the MG (inside the PRS processing window).

# Capability type 2

In RAN1#106-e [3], RAN1 agreed the following working assumption on PRS measurement outside MG.

Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.) - For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected: - Alt. 1: Applicable to serving cell PRS only - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell. - Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. s - Further study - Further details of which other DL signals/channels to be prioritized - How the UE determines DL PRS’s priority based on one or more of the following: - Opt. 1: Based on indication/configuration from serving gNB - Opt. 2: Other options (e.g., implicit, signalling from LMF, etc) - Whether UE can do the measurement for both inside MG (if MG is configured) and outside MG in a measurement period - How to do the PRS measurement when the conditions cannot be satisfied, e.g. when BWP switching happens - Prioritization conditions of processing PRS over other DL channels/signals or vice versa. - Send an LS to RAN2, RAN3 and RAN4 informing them of this working assumption and requesting feedback in case they have concerns.

For capability 1A and 1B, the prioritization scheme was later refined as below in RAN1#107-e [1].

Agreement For capability 1A as per working assumption made in RAN1#106-e, the DL signalings/channels in a per UE fashion (i.e. both across NR & LTE) inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. For capability 1B as per working assumption made in RAN1#106-e, only the DL signalings/channels from a certain band inside the PRS processing window are dropped if the DL PRS is determined to be higher priority. Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.).

For capability 2, there was no progress on the interrupted cells. The following options were on the table.

Option 1: Only the target cell that contains the PRS

Option 2: All the cells in the same band as the cell that contains the PRS

In addition to capability 2, there was also the proposal to extend capability 1B to cover the case where the same Rx beam is assumed across multiple bands (e.g. FR2 inter band CA).

In our view, we also need to consider the impact on PRS being lower priority if something we agreed for PRS being higher priority is adopted. For example, if all the cells within a band are impacted by the PRS reception if PRS is higher priority, it means that if PRS is lower priority PRS will be impacted if any of the cells within the band has active data.

For capability 2, the processing multiplexing between data and PRS is already on a per-symbol basis, which is quite similar to a regular CSI-RS processing (and with the synchronization threshold as discussed in RAN4), and thus it does not make much sense to limit it to per band for FR1.

In addition, we also acknowledge that for FR2, the Rx beam plays an important rule when determining the scheduling restriction, and we would propose to generalize this case for both capability 1B and 2, and it should be up to RAN4 to decide, e.g. whether explicit signaling is required for this, or this can be hardcoded in RAN4 specification.

In summary, we have the following proposal.

- Proposal 1:

For capability 2 as per working assumption made in RAN1#106-e

  - For FR1 bands, the DL signals/channels from the target CC that contains the PRS inside the PRS processing window are dropped if the DL PRS is determined to be higher priority.

  - For FR2 bands, the DL signals/channels from all the CCs on the band that contains the PRS inside PRS processing window are dropped if the DL PRS is determined to be higher priority.

For capability 1B and capability 2, whether or not UE may indicate for each target FR2 band, a set of bands on which reception of the DL signals/channels may be interrupted due to a common Rx beam is up to RAN4.

# Low priority for PRS

As discussed in the section 2, any assumption made in high priority PRS should have its counterparts for low priority PRS. If the capability only mentions the capability of handling data dropping due to higher priority PRS, the procedure text should at least reflect the corresponding behaviour of PRS dropping due to high priority data.

- Observation 1: Procedure text is at least required for determining PRS dropping rule for higher priority data.

For capability 1A, if there is any high priority data within the PRS processing window regardless of which band (LTE/NR) the data is in, as shown in Figure 1, UE should drop the entire PRS processing window.

For capability 1B, if there is any high priority data within the PRS processing window on the same band, as shown in Figure 2, UE should drop the entire PRS processing window.

For capability 2, if there is any high priority data within the PRS processing window on the same cell for FR1 or on the same band for FR2, UE should drop the PRS on the symbols overlapped with the high priority data.

Figure 1 Dropping of PRS measurement if there is overlap between PDCCH/PDSCH/CSI-RS and the PRS processing window for capability 1A

Figure 2 Dropping of PRS measurement if there is overlap between PDCCH/PDSCH/CSI-RS and the PRS processing window for capability 1B

In addition, for dynamically scheduled reception, e.g. PDSCH scheduled by DCI, UE is aware of the reception only after DCI decoding. If the DCI is too close in time with the PRS processing window, UE may not be able to perform the PDSCH reception because it has already scheduled PRS measurement on the corresponding symbols. There could still be scheduling restriction if DCI is later than T before the scheduled symbol if the scheduled symbols is overlapped with serving cell or non-serving cell PRS, as shown in Figure 2.

Figure 3 Example of PDSCH scheduled on the PRS symbols

For UE capability 1A and 1B, the DCI for scheduling PDSCH or AP-CSI-RS during any symbol in the PPW should be T before the start of the PPW. If no such DCI is received and no DL signals/channels is configured during PPW, UE would perform PRS measurement during the PPW. This means DL signals/channels that is scheduled by DCI after T before the start of PPW will not be processed by the UE even if it is higher priority than PRS.

For UE capability 2, the DCI for scheduling PDSCH or AP-CSI-RS in a measured PRS symbol in the PPW should be T before the symbol. If no such DCI is received and no DL signals/channels is configured on the symbol, UE would perform PRS measurement. This means DL signals/channels on the measured PRS symbol that is scheduled by DCI after T before the symbol will not be processed by the UE even if it is higher priority than PRS.

Based on above analysis, we summarize the scheduling restriction due to PRS measurement outside MG as in the following table.

Proposal 2: Agree with the following UE behaviour for both high priority PRS and low priority PRS.

|  | Case 1: PRS measurement is of higher priority | Case 2: PRS measurement is of lower priority |
| --- | --- | --- |
| 1A | UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG. | UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than a threshold before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than a threshold before the start of the PRS processing window on any serving cell including SCG; otherwise the UE is not expected to receive the DL PRS within the PRS processing window. |
| 1B | UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS. | UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than a threshold before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than a threshold before the start of the PRS processing window on serving cells in the same band as the DL PRS; otherwise the UE is not expected to receive the DL PRS within the PRS processing window. |
| 2 | UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells | if the DL PRS is lower priority than the DL signals and channels, UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbols on the impacted serving cells, if the corresponding DCI is later than a threshold before the symbol and there is no DL signals/channels configured on the symbol on the impacted serving cells; otherwise the UE is not expected to receive the DL PRS on the symbol within the PRS processing window |

# PRS processing capability

Regarding a new PRS processing capability for the purpose of low latency feature, there were two alternatives on the enhancements based on discussion in RAN1#107-e [1].

Alt.1

  - During the first part of the window with duration of [at least] L-(T-N) or (L-T) msec, up to N msec of PRS symbols are expected to be buffered, where L is the duration of the PRS processing window, and (N,T) is the reported capability for MG-less PRS processing.

  - The UE is expected to be capable of reporting measurements derived on the PRS measured in the first window after T msec from the end of first part of the PRS processing window

  - UE is not expected to be configured a PRS processing window with duration smaller than T (i.e., L>(T-N) or L>T

Alt.2

  - UE has to report its capability of PRS computation time (Tcompute)

  - A time span (Tspan) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window

  - The value of Tspan is not expected to be smaller than the PRS computation time (Tcompute).

In RAN1#107bis-e UE feature discussion, the two alternatives seemed to be converted to a single FG proposal [2].

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG – buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol, where a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |
| --- | --- | --- | --- |

We would like to note the following facts

Even if we stick with the previous description of (N, T), the claimed functionality can be achieved considering the existing RAN4 PRS measurement period requirement TRSTD,i=CSSFPRS,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i, if the following conditions are met

  - At most N ms earliest symbols are received within the PRS processing window, i.e. Lavailable_PRS,i≤N

  - Single-sample PRS is processed, i.e. Nsample=1

  - The number of PRS resources in each slot covering the N ms PRS symbols is within the UE supported capability as reported in component 3, i.e. NPRS,islot<N'

  - No Rx beam switching is used, i.e. NRxBeam,i=1

  - Single positioning frequency layer is present

This values is not suited for capability 2, because for capability 2, the window is used to indicate the duration in which data reception on specific symbols may be subject to dropping if collided with PRS.

- The feature lead also asked the following questions during the offline summary [4].

### Question 4.1.2-2 (input requested) - Do you think PRS processing window should consist of buffering period and processing period, and thus the length should depend on the UE PRS processing capability (N, T)? - This is intended to address the capability enhancements proposed by ZTE, CATT, Intel, Qualcomm. ### Question 4.1.2-3 (input requested) - Do you think UE may be expected to measure PRS outside MG if there is no PRS processing window or if the PRS is outside the PRS processing window? - This is intended to address the meaningfulness of PRS processing window raised by Samsung. ### Question 4.1.2-4 (input requested) - Do you think the MG-less PRS measurement can also be used for other scenarios that do not persue low latency feature? - This is a general question on compatability for the sake of better understanding of the FL.

For Question 4.1.2-3, we think there should be a clear understanding that for PRS not covered by the MG (activated/configured) or by the PRS processing window, UE is not required to measure them, while the some answers are trying to address whether MG can be requested by the UE if UE needs to measure the PRS that are outside the PRS processing window, which should not be the intention.

- Observation 2: UE is not required to measure the PRS that are not inside either the MG or the PRS processing window.

Then it should be understood that the PRS processing window is the duration in which that the network expects UE to measure the PRS, similar to SMTC. There may be other PRS outside the PRS processing window, but those PRS are not the ones that network expects UE to process. For example, if the PRS periodicity is 20ms for some low latency positioning deployment, but the PRS processing window can be configured with 80ms for some UEs to decimate PRS occasion simply because either UE is not capable of processing PRS so fast or network does not want the communication to drop for those target UEs.

- Observation 3: One major functionality of PRS processing window is to set a time boundary for PRS measurement, similar to SMTC.

For example, the periodicity of PRS processing window can be a multiple of PRS periodicity, so as to create a decimation of PRS measurement occasion in the time domain.

For Question 4.1.2-4, it seems like companies tend to agree that PRS processing window can also be used for other non-latency-critical scenarios. The reply implied that there was concern over explicit signaling on the low latency feature. However, if everyone can accept the PRS processing window can somehow be used for those scenarios that do not pursue low latency, we should at least keep the existing (N, T) values in the capability signaling to allow UE/network that may not support the strictest latency requirement to also benefit from the scheduling flexibility of PRS processing window.

- Observation 4: If PRS processing window can also be used for the scenarios that do not pursue the low latency feature, the existing (N, T) in the UE capability should be kept.

Based on the discussion above, with regards to the first question Question 4.1.2-2, we think that functionality of PRS processing window should primarily cover the duration of PRS that network expects UE to measure, and thus extending the PRS processing window to cover the post-buffer processing should not be mandatory, and the benefit to other scenarios that do not pursue the low latency features are no longer valid if we restrict the PRS processing window to also cover the post-buffer processing period.

- Proposal 3: For PRS processing window, at least the existing PRS duration calculation and signaling structure for PRS processing capability are reused. (FG 27-3-3)

The changes to the existing FG 27-3-3 in R1-2200764 should be reverted.

For the cases that cannot be covered by the existing PRS processing capability, a new optional operation can be introduced, in which the following modification to the existing one can be considered

During the PPW, only the first N ms duration of PRS calculated by the existing clause 5.1.6.5 of TS 38.214 is measured by the UE.

For processing type 1A and 1B, the PRS processing window duration should cover the T-N ms after the last symbol of the first N ms duration of PRS.

Smaller T values are added to the UE capability, e.g. 1ms, 2ms, 4ms.

- Other potential modification can be discussed by RAN4, e.g. changing the Tlast according to the above assumption, so that the discussion on when the measurement report is available can be avoided in RAN1, allowing for the tolerance on higher layer signaling processing.

- Proposal 4: For PRS processing window with advance low latency feature

Introduce an indication from network to enable the following operation that

  - UE may only measure the first N ms PRS within a PRS processing window

  - For processing type 1A and 1B, UE expects that the PRS processing window covers T-N ms after the last symbol of the first N ms PRS.

Introduce a new optional UE feature group with prerequisite FG being the original FG 27-3-3.

| 27. NR_pos_enh | 27-3-3a | DL PRS Processing Capability outside MG – Advanced buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE s, where a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: UE that supports this feature and supports type 1A or type 1B capability processing in FG 27-3-2, may be indicated by the network to only process the first N ms PRS within the PRS processing window that extends T-N ms after the last symbol of the first N ms PRS | 27-3-3 |
| --- | --- | --- | --- | --- |

# UE capability of supporting multiple types

We think in order to support the capability reporting of multiple processing types among 1A, 1B and 2, the impact with other FGs should also be considered, including

Supported number of priority options: UE may indicate support of both type 1A and 2, but may choose to support two priority states for type 1A and single priority states for type 2.

The PRS processing capability should also be different from one processing type to another, including the potential support of the advanced buffering capability.

- We do not consider it being a high priority for this release, and would suggest to focus on stabilizing the current FGs for the low latency feature. In addition, we have both MG-based and MG-less to achieve the low latency target, further complicating the MG-less UE capability seems unnecessary.

- Proposal 5: Do support the capability reporting enhancement to allow UE to report support of multiple processing types among 1A, 1B, and 2.

# Text proposal

- Based on the discussion in section 2, 3 and 4 of the paper, we propose to adopt the following TP.

- =================== START of TP ===================

The UE is expected to measure the DL PRS outside the measurement gap, subject to UE capability, if the DL PRS is inside the active DL BWP and has the same numerology as the active DL BWP and is within the DL PRS processing window indicated by higher layer parameter [PRSProcessingWindow].

For receiving the DL PRS outside the measurement gap and within the DL PRS processing window, the UE may be indicated by higher layer parameter [PRS-priority-indicator] subject to UE capability that

-	the DL PRS is higher priority than all the DL signal/channels except SSB, or

-	the DL PRS is lower priority than PDCCH and the PDSCH scheduled by DCI formats 1_1 or 1_2 with the priority indicator field in the corresponding DCI format set to 1, and is higher priority than other DL signals/channels except SSB, or

-	the DL PRS is lower priority than all the DL signals/channels except SSB.

When the UE is expected to measure the DL PRS outside the measurement gap and is indicated by the higher layer parameter [ProcessingType] for Type-1A processing

-	if the DL PRS is higher priority than the DL signals and channels, the UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG;

-	if the DL PRS is lower priority than the DL signals and channels, the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than [SchedulingThresholdBeforePPW] before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than [SchedulingThresholdBeforePPW] before the start of the PRS processing window on any serving cell including SCG; otherwise the UE is not expected to receive the DL PRS within the PRS processing window.

When the UE is expected to measure the DL PRS outside the measurement gap and is indicated by the higher layer parameter [ProcessingType] for Type-1B processing

-	if the DL PRS is higher priority than the DL signals and channels, the UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS;

-	if the DL PRS is lower priority than the DL signals and channels, the UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than [SchedulingThresholdBeforePPW] before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than [SchedulingThresholdBeforePPW] before the start of the PRS processing window on serving cells in the same band as the DL PRS; otherwise the UE is not expected to receive the DL PRS within the PRS processing window.

When the UE is expected to measure the DL PRS outside the measurement gap and is indicated by the higher layer parameter [ProcessingType] for Type-2 processing

-	if the DL PRS is higher priority than the DL signals and channels, the UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells;

-	if the DL PRS is lower priority than the DL signals and channels, UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbols on the impacted serving cells, if the corresponding DCI is later than [SchedulingThresholdBeforePPW] before the symbol and there is no DL signals/channels configured on the symbol on the impacted serving cells; otherwise the UE is not expected to receive the DL PRS on the symbol within the PRS processing window;

-	The impacted serving cells refer to the serving cell with the active DL BWP that covers the DL PRS bandwidth and has the same numerology as the DL PRS for FR1, and the serving cells in the same band as the DL PRS for FR2.

- =================== END of TP ===================

# Conclusion

In this contribution, we have the following observations and proposals with regards to PRS measurement outside MG.

- Observation 1: Procedure text is at least required for determining PRS dropping rule for higher priority data.

- Observation 2: UE is not required to measure the PRS that are not inside either the MG or the PRS processing window.

- Observation 3: One major functionality of PRS processing window is to set a time boundary for PRS measurement, similar to SMTC.

For example, the periodicity of PRS processing window can be a multiple of PRS periodicity, so as to create a decimation of PRS measurement occasion in the time domain.

- Observation 4: If PRS processing window can also be used for the scenarios that do not pursue the low latency feature, the existing (N, T) in the UE capability should be kept.

- Proposal 1:

For capability 2 as per working assumption made in RAN1#106-e

  - For FR1 bands, the DL signals/channels from the target CC that contains the PRS inside the PRS processing window are dropped if the DL PRS is determined to be higher priority.

  - For FR2 bands, the DL signals/channels from all the CCs on the band that contains the PRS inside PRS processing window are dropped if the DL PRS is determined to be higher priority.

For capability 1B and capability 2, whether or not UE may indicate for each target FR2 band, a set of bands on which reception of the DL signals/channels may be interrupted due to a common Rx beam is up to RAN4.

Proposal 2: Agree with the following UE behaviour for both high priority PRS and low priority PRS.

|  | Case 1: PRS measurement is of higher priority | Case 2: PRS measurement is of lower priority |
| --- | --- | --- |
| 1A | UE is not expected to receive the DL signals and channels within the PRS processing window on all serving cells including SCG. | UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on all serving cells including SCG, if the corresponding DCI is later than a threshold before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than a threshold before the start of the PRS processing window on any serving cell including SCG; otherwise the UE is not expected to receive the DL PRS within the PRS processing window. |
| 1B | UE is not expected to receive the DL signals/channels within a PRS processing window on the serving cells in the same band as the DL PRS. | UE is not expected to receive the scheduled DL signals/channels in the PRS processing window on the serving cells in the same band as the DL PRS, if the corresponding DCI is later than a threshold before the start of the PRS processing window and there is no DL signals/channels configured during the PRS processing window or scheduled during the PRS processing window with DCI earlier than a threshold before the start of the PRS processing window on serving cells in the same band as the DL PRS; otherwise the UE is not expected to receive the DL PRS within the PRS processing window. |
| 2 | UE is not expected to receive any DL signals/channels on a DL PRS symbol within the PRS processing window on the impacted serving cells | if the DL PRS is lower priority than the DL signals and channels, UE is not expected to receive the scheduled DL signals/channels on the DL PRS symbols on the impacted serving cells, if the corresponding DCI is later than a threshold before the symbol and there is no DL signals/channels configured on the symbol on the impacted serving cells; otherwise the UE is not expected to receive the DL PRS on the symbol within the PRS processing window |

- Proposal 3: For PRS processing window, at least the existing PRS duration calculation and signaling structure for PRS processing capability are reused. (FG 27-3-3)

The changes to the existing FG 27-3-3 in R1-2200764 should be reverted.

- Proposal 4: For PRS processing window with advance low latency feature

Introduce an indication from network to enable the following operation that

  - UE may only measure the first N ms PRS within a PRS processing window

  - For processing type 1A and 1B, UE expects that the PRS processing window covers T-N ms after the last symbol of the first N ms PRS.

Introduce a new optional UE feature group with prerequisite FG being the original FG 27-3-3.

| 27. NR_pos_enh | 27-3-3a | DL PRS Processing Capability outside MG – Advanced buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE s, where a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: UE that supports this feature and supports type 1A or type 1B capability processing in FG 27-3-2, may be indicated by the network to only process the first N ms PRS within the PRS processing window that extends T-N ms after the last symbol of the first N ms PRS | 27-3-3 |
| --- | --- | --- | --- | --- |

- Proposal 5: Do support the capability reporting enhancement to allow UE to report support of multiple processing types among 1A, 1B, and 2.

# Reference

- RAN1, Chair’s Notes RAN1#107-e v18, RAN1#107-e, e-Meeting, 11th November – 19th November, 2021.

- R1-2200767, Summary of UE features for NR positioning enhancements, RAN1#107bis-e, e-Meeting, 17th Jan  – 25th Jan, 2022.

- RAN1, Chair’s Notes RAN1#106-e v23, RAN1#106-e, e-Meeting, 16th  – 27th Aug., 2021.

- R1-2112459, Summary #3 of [107-e-NR-ePos-04] latency improvements, Moderator (Huawei), , RAN1#107-e, e-Meeting, 11th November – 19th November, 2021.