3GPP TSG-RAN WG1 Meeting #108-e 	R1-2200925

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.15.5

Source:	Huawei, HiSilicon

Title:	Remaining issues of Rel-17 positioning UE feature

Document for:	Discussion and decision

# Introduction

- In this paper, we provided our view on the Rel-17 UE feature list for positioning.

# Discussion

## UE Rx/Tx timing error

| 27. NR_pos_enh | 27-1-1 | UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | 1. Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the UE Rx timing errors for the measurements | per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {UE-assisted DL TDOA, Multi-RTT positioning, UE-assisted DL TDOA and Multi-RTT positioning} Component 2 candidate values: {1, 2, 3, 4, 6, 8} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported If the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the UE Rx timing errors for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-1-1

- No need to further refine component 1. RAN2 can figure out how to capture it.

| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG for SRS resource for positioning, which is supported and reported by UE for UL TDOA | 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA Note: If the UE does not include TxTEG-ID associated with a SRS resource for positioning, no assumption can be made on the UE Tx timing error for this SRS resource for positioning. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-1-2

- “mitigation of” may be removed.

| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported and no assumption can be made on the UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported If the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the UE Tx timing errors for this SRS resource for positioning Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-1-2a

- “mitigation of” may be removed.

| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4 and 13-8 | No |  | UE RxTx for Multi-RTT is not supported and no assumption can be made on the UE RxTx timing error for the measurement | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 36, 48, 64} Need for location server to know if the feature is supported If the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the UE RxTx timing errors for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-1-3

- Keep error

| 27. NR_pos_enh | 27-7 | Multiple measurement instances which can be included in a single measurement report | Support of mutiple measurement instances which can be included in a single measurement report |  | No |  | Multiple measurement instances in a single measurement report is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-7

- No need to keep component 2.

| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band | n/a | n/a FR2 only | n/a | Component 2 candidate values: {1,2,4,6} Need for location server to know if the feature is supported | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-9

- The candidate value can be {1,2,4,6} according to RAN4 LS.

## DL AoD

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 or 27-2-2 | No |  |  | Per UE | No | Yes | No | Component 2 candidate values: 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-2-1

- For type: we think it should be per UE with FR1/FR2 differentiation.

- The Note could be kept.

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. 2. Supported resource set relationship for the target PRS resource and the associated subset 3. Support associated subset measurement reporting with or without the target PRS |  | No |  | PRS subset association for DL-AoD is not supported by the UE. | Per UE | No | No | No | Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet} Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-20

- Support the current component 2

- For component 3, we think UE supporting this feature should support reporting with or without the target PRS, and network may choose whether the target PRS is requested. No need to have candidate values for component 3.

## Latency

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | 13-1 | Yes |  |  | per band | n/a | n/a | n/a | Component 1 candidate values: One of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: A UE that supports FG 27-3-2 also needs to support FG 27-3-2a | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-3-2

- No need to keep the yellow text. The details should be kept in the procedure text.

- Do not support reporting multiple types per band.

- FG 27-3-2a should be merged into the component.

|  |  |  |  |  |  |  |  |  |  |  |  |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-3-2a

- The Note should be implemented, and it should be merged into FG 23-3-2.

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-3-3

- The original proposal should be agreed as part of the basic FG.

- Any advanced buffering capability should be introduced based on this.

| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements |  | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-10a

- No need to keep the prerequisite FG, since they are reported to different nodes.

## MP/NLoS

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  |  | Per UE | No | No | No | Component 1 candidate values: {hard value, soft value[, both]} Component 2 candidate values: {trpSpecific, resourceSpecific, both} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-4-1

- We support the existing candidate values for component 1.

- We support both for the candidate values for component 2.

- We support the Note on the single value for both methods if supported.

- No need to keep the FFS

| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | No |  |  | Per UE | No | No | No | Component 1 candidate values: {softValue, hardValue, both} Component 2 candidate values: {resourceSpecific, trpSpecific, both} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-12

- We support all the existing candidate values, including “both”.

| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | 1. Support of additional detected path timing reporting for K>2 additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths if UE supports FG 27-13a. |  | No |  |  | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-13

- We support all the existing candidate values.

- The prerequisite FG, which also seems to be typo (13-13a  27-13a?) should be removed, since support of additional path reporting should have no relevance with support of first path RSRPP.

- We could add the condition for component 2 on the dependency with FG 27-13a

| 27. NR_pos_enh | 27-13a | First path reporting for UE-assisted DL-TDOA | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | Per UE | No | Yes | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-13a

- For type, we support per UE with FR1/FR2 differentiation.

| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of additional detected path timing reporting for K>2 additional paths for Multi-RTT 2. Support of RSRPP reporting for additional paths if UE supports FG 27-14a | 13-14a | No |  |  | Per UE | No | No | No | Component 1 candidate values: {4, 6, 8} Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-14

- We support all the existing candidate values.

- The prerequisite FG, which also seems to be typo (13-14a  27-14a?) should be removed, since support of additional path reporting should have no relevance with support of first path RSRPP.

- We could add the condition for component 2 on the dependency with FG 27-14a

| 27. NR_pos_enh | 27-14a | First path reporting for Multi-RTT | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | Per UE | No | Yes | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-13a

- For type, we support per UE with FR1/FR2 differentiation.

## RAN2-led objectives

| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in power efficiency mode | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot under it |  |  |  |  |  |  |  |  | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-6

- We propose to modify “RRC inactive state” to “power efficiency mode” to hide the RRC state from the LMF

| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of SRS Resources for positioning 3. Max number of SRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15b | Support of positioning SRS transmission in power efficiency mode for initial BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of SRS Resources for positioning 3. Max number of periodic SRS Resources for positioning Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component |  | No |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1,2,4,8,16,32,64} Need for location server to know if the feature is supported | Optional with capability signaling |

### FG 27-15

- [P/SP] can be removed, and the FG can be used in future if AP was introduced.

- The FG can be split into two (one for gNB and one for LMF), and the one reported to LMF could use “power efficiency mode” instead of “RRC_INACTIVE state”.

- A separate FG is required for SRS outside initial BWP.

| 27. NR_pos_enh | 27-15a | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15c | Support of positioning SRS transmission in power efficiency mode for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning | 27-15b | No |  |  |  |  |  |  | Component 1 candidate values: {1,2,4,8,16,32,64} Need for location server to know if the feature is supported | Optional with capability signaling |

### FG 27-15a

- The FG can be split into two (one for gNB and one for LMF), and the one reported to LMF could use “power efficiency mode” instead of “RRC_INACTIVE state”.

- A separate FG is required for SRS outside initial BWP.

| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-16a | OLPC for positioning SRS in power efficiency mode | Same as LPP OLPC-SRS-Pos-r16 |  | No |  |  |  |  |  |  | Need for location server to know if the feature is supported. | Optional with capability signaling |

### FG 27-16

- The FG is needed, but for LPP, “power efficiency mode” could be replace “RRC_INACTIVE state”.

| 27. NR_pos_enh | 27-17 | Support of positioning in RRC_INACTIVE state | Support of PRS processing in RRC_INACTIVE |  | Yes |  |  | Per UE | No | No | No | Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### FG 27-17

- No need to have any prerequisite FG.

- Need for gNB to know.

- Reporting type is per UE.

- No need for the location server to know.

- No need to keep the FFS

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in power efficiency mode for DL-TDOA | Support of PRS measurement in power efficiency mode for DL-TDOA |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for power efficiency mode | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in power efficiency modefor DL-AoD | Support of PRS measurement in power efficiency mode for DL-AoD |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for power efficiency mode. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in power efficiency mode for Multi-RTT | 1. Support of PRS measurement in power efficiency mode for Multi-RTT 2. Support of positioning SRS transmission in power efficiency mode |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for power efficiency mode | Optional with capability signaling. |

### FG 27-18a, 18b, 18c

- The FGs are needed, and “RRC_INACTIVE state” can be replaced by “power efficiency mode”.

- No need for the gNB to know.

- Reporting type is per UE.

- Need for the location server to know.

| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-19a | Spatial relation for positioning SRS in power efficiency mode | Same as LPP SpatialRelationsSRS-Pos-r16 |  | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |

### FG 27-19

- The FG is needed, but for LPP, “power efficiency mode” could be replace “RRC_INACTIVE state”.

## New FG proposals

| 27. NR_pos_enh | 27-15d | Support of positioning SRS transmission in RRC_INACTIVE state outside initial BWP | 1. SRS switching time (DL and UL) 2. Supported numerology for SRS 3. SRS bandwidth 4. SRS bandwidth with initial DL BWP | 27-15 | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {0us, 30us, 100us, 140us, 200us, 300us, 500us, 900us} for DL and UL, respectively Component 2 candidate values: {sameAsInitialUL-BWP, sameAsOrDifferentFromInitialUL-BWP} Component 3 candidate values: bitmap to indicate support of { 5MHz, 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz, 100MHz}. Component 4 candidate values: {srsBW-ContainsIntialDL-BWP, noRestriction} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15e | Support of positioning SRS transmission in power efficiency mode outside initial BWP | SRS bandwidth | 27-15b | No |  |  | Per band | n/a | n/a | n/a | Candidate values: bitmap to indicate support of { 5MHz, 10MHz, 15MHz, 20MHz, 25MHz, 30MHz, 40MHz, 45MHz, 50MHz, 60MHz, 70MHz, 80MHz, 90MHz, 100MHz}. Need for location server to know if the feature is supported. | Optional with capability signalling |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS 3. Scheduling threshold before PRS processing window when PRS is lower priority for Option 1 and Option 2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | 13-1 | Yes |  |  | per band | n/a | n/a | n/a | Component 1 candidate values: One of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Component 3 candidate values: {0.5ms, 1ms, 1.5ms, 2ms} Need for location server to know if the feature is supported Note: A UE that supports FG 27-3-2 also needs to support FG 27-3-2a | Optional with capability signaling. |

# Conclusion

In this contribution, we have provided our view on the existing FGs and new FGs for Rel-17 positioning. Detailed comments can be found in section 2.

# Reference

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderator (AT&T, NTT DOCOMO), RAN1#107bis-e, e-Meeting, 17th Jan  – 25th Jan, 2022.