3GPP TSG-RAN WG1 Meeting #108-e	R1-2200926

e-Meeting, February 21 – March 3, 2022

Agenda Item:	8.10.1

Source:	Huawei, HiSilicon

Title:	Remaining issues on resource multiplexing between backhaul and access for IAB

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues of resource multiplexing between backhaul and access for IAB duplexing enhancements.

Simultaneous operation

The following text was capture in CR of 38.213 [2].

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot.

The above text is to capture the following work assumption in RAN1#106bis-e.

Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

The above working assumption was discussed but not confirmed in RAN1#107-e because the IAB node and parent node may not be able to have the same understanding on the multiplexing mode of IAB node for a given resource. To be more specific, the parent node has no idea whether IAB-node uses simultaneous transmission and reception in a given slot or not; therefore, the parent node cannot know the resource configuration adopted by the IAB node, which may result in resource collisions. As a matter of fact, even though the above working assumption was not explicitly reverted, the following agreement was reached in RAN1#107-e [1].

Agreement: Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17

Accordingly, we propose to revert the working assumption in RAN1#106bis-e and agree on the following text proposal

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List when the IAB-node uses simultaneous transmission and reception in the slot.

Proposal 1: Revert the following working assumption in RAN1#106bis-e:

Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Proposal 2: Adopt the following TP in 38.213:

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List.

TDD configurations enhancement for IAB-MT

A prerequisite to enable simultaneous operation of IAB-node’s child and parent links is that the resource configurations of MT and DU should be in line with the TX/RX combinations on the resources where simultaneous operations occurs

- In case of full duplexing, i.e. simultaneous operation w.r.t. MT-TX/DU-RX and MT-RX/DU-TX, the IAB-MT and IAB-DU should have same link directions.

- In case of SDM/FDM, i.e. simultaneous operation w.r.t. MT-TX/DU-TX and MT-RX/DU-RX, the IAB-MT and IAB-DU should have opposite link directions.

In principle, the Rel-16 resource allocation schemes can be reused as a baseline to enable the simultaneous operation. However, to support the coexistence of TDM and no-TDM operations, some enhancements to the TDD configuration of IAB-MT are necessary.

Overriding rules of TDD-UL-DL-ConfigDedicated-IAB-MT

In Rel-16, the slot formats with the sequence order UL-Flexible-DL for IAB node MT were introduced, and the following agreement on the dedicated RRC signalling (referred as TDD-UL-DL-ConfigDedicated-IAB-MT) was achieved in RAN1#99:

Agreements: If the IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter TDD-UL-DL-ConfigDedicated-IAB-MT overrides all symbols (with a limitation that effectively only flexible symbols can be overwritten in Rel-16) per slot over the number of slots as provided by TDD-UL-DL-ConfigurationCommon.

The legacy dedicated TDD configuration can only override the flexible symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. According to the agreement, an enhanced overriding rule for IAB-MT is introduced, i.e., tdd-UL-DL-ConfigDedicated-IAB-MT can override all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

However, since Rel-16 IAB focuses on TDM operation, the enhanced overriding rule was not captured in specification, i.e., only flexible symbols can be overridden by tdd-UL-DL-ConfigDedicated-IAB-MT in Rel-16. To increase the opportunity of simultaneous operation, the limitation of the overriding rule in Rel-16 should be removed in Rel-17.

Proposal 3: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

Figure 1. tdd-UL-DL-ConfigurationDedicated-IAB-MT without Rel-16 limitation

Collision handling between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels

The UE behaviors related to tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated are captured in TS 38.213. In Rel-16, when tdd-UL-DL-ConfigDedicated-IAB-MT is provided, IAB-MT follows the UE behaviors by replacing tdd-UL-DL-ConfigurationDedicated with tdd-UL-DL-ConfigDedicated-IAB-MT. However, if tdd-UL-DL-ConfigurationDedicated-IAB-MT can override all symbols in Rel-17, the behavior of IAB-MT with tdd-UL-DL-ConfigurationDedicated-IAB-MT should be revisited.

For example, the following UE behaviors about TDD configurations and cell-specific signals/channels are specified in TS 38.213:

For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE. For a set of symbols of a slot corresponding to a valid PRACH occasion and Ngap symbols before the valid PRACH occasion, as described in Sublcause 8.1, the UE does not receive PDCCH, PDSCH, or CSI-RS in the slot if a reception would overlap with any symbol from the set of symbols. The UE does not expect the set of symbols of the slot to be indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

It can be concluded from the above that the TDD configurations (tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated) should be compatible with the cell-specific channels and signals.

However, retaining the above constraint for tdd-UL-DL-ConfigurationDedicated-IAB-MT can dramatically reduce the opportunities for simultaneous operations. Specifically, most of the downlink slots indicated by tdd-UL-DL-ConfigurationCommon contain SS/PBCH blocks in typical TDD UL/DL configurations. This is due to the fact that the slot configuration period is usually smaller than the period of SS/PBCH blocks, which means that these slots cannot be indicated as uplink by tdd-UL-DL-ConfigurationDedicated-IAB-MT, i.e. SDM Rx/Tx cannot be implemented in these slots. As an example in Figure 2, assuming the TDD configuration periodicity is 2.5 ms, it can be observed that all the first 4 slots within one TDD configuration period cannot be indicated as uplink according to current specification since the SS/PBCH blocks may be transmitted every 20 ms.

Figure 2. tdd-UL-DL-ConfigurationDedicated-IAB-MT colliding with SS/PBCH blocks

To solve this problem, the specification should allow the collision between cell-specific signals/channels and tdd-UL-DL-ConfigurationDedicated-IAB-MT. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT and it should be able to transmit/receive the cell-specific signals/channels. And in the TDD configuration periods or slots without collision, the IAB node can perform simultaneous operation according to the TDD configuration. As is shown in Figure 3, the tdd-UL-DL-ConfigurationDedicated-IAB-MT is ignored in the first TDD period because of the collision, but it can be valid in other TDD periods, which increase the resources for simultaneous operation significantly.

In conclusion, we have the following proposal:

Proposal 4: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes:

- SS/PBCH block

- CORESET for Type0-PDCCH CSS set

- PRACH

Based on proposal 3 and proposal 4, we propose the following TP

Proposal 5: Adopted the following TP in 38.213:

For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides - a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT - for each slot configuration from the set of slot configurations - a slot index for a slot provided by slotIndex - a set of symbols for a slot by symbols-IAB-MT where - if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink - if symbols-IAB-MT = allUplink, all symbols in the slot are uplink - if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible. - if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT does not apply - for a set of symbols of a slot that are indicated to an IAB-MT for reception of SS/PBCH blocks, or - for a set of symbols of a slot that are indicated to an IAB-MT for a CORESET for Type0-PDCCH CSS set, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT, or - for operation on a single carrier in unpaired spectrum, for a set of symbols of a slot corresponding to a PRACH occasion and Ngap symbols before the valid PRACH occasion,

Conclusions

In this contribution, we discussed the remaining issues on resource multiplexing for IAB. The following are proposed

Proposal 1: Revert the following working assumption in RAN1#106bis-e:

Working Assumption If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected: - Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

Proposal 2: Adopt the following TP in 38.213:

If an indication of hard, soft, or unavailable type is provided for an RB set in a symbol of a slot, the IAB-DU applies the configuration of hard, soft, or unavailable type provided by HSNA Slot Configuration List.

Proposal 3: If an IAB-MT is additionally provided TDD-UL-DL-ConfigDedicated-IAB-MT, the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon.

Proposal 4: To increase the resources for simultaneous operation, the specification should allow the collision between tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell-specific signals/channels. In the slots with the collision, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT. The list of cell-specific signals/channels includes:

- SS/PBCH block

- CORESET for Type0-PDCCH CSS set

- PRACH

Proposal 5: Adopted the following TP in 38.213:

For each serving cell, an IAB-MT can be provided an indication for a slot format over a number of slots by tdd-UL-DL-ConfigurationDedicated-IAB-MT. If the IAB-MT is provided tdd-UL-DL-ConfigurationDedicated-IAB-MT, the statements in clause 11.1 that include "tdd-UL-DL-ConfigurationDedicated" apply to the IAB-MT of an IAB node by replacing "tdd-UL-DL-ConfigurationDedicated" with "tdd-UL-DL-ConfigurationDedicated-IAB-MT" for the IAB-MT, except that the tdd-UL-DL-ConfigurationDedicated-IAB-MT provides - a set of slot configurations by slotSpecificConfigurationsToAddModList-IAB-MT - for each slot configuration from the set of slot configurations - a slot index for a slot provided by slotIndex - a set of symbols for a slot by symbols-IAB-MT where - if symbols-IAB-MT = allDownlink, all symbols in the slot are downlink - if symbols-IAB-MT = allUplink, all symbols in the slot are uplink - if symbols-IAB-MT = explicit, nrofDownlinkSymbols provides a number of downlink first symbols in the slot and nrofUplinkSymbols provides a number of uplink last symbols in the slot. If nrofDownlinkSymbols is not provided, there are no downlink first symbols in the slot and if nrofUplinkSymbols is not provided, there are no uplink last symbols in the slot. The remaining symbols in the slot are flexible. - if symbols-IAB-MT = explicit-IAB-MT, nrofUplinkSymbols provides a number of uplink first symbols in the slot and nrofDownlinkSymbols provides a number of downlink last symbols in the slot. If nrofUplinkSymbols is not provided, there are no uplink first symbols in the slot and if nrofDownlinkSymbols is not provided, there are no downlink last symbols in the slot. The remaining symbols in the slot are flexible. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides all symbols per slot over the number of slots as provided by tdd-UL-DL-ConfigurationCommon. - the parameter tdd-UL-DL-ConfigurationDedicated-IAB-MT does not apply - for a set of symbols of a slot that are indicated to an IAB-MT for reception of SS/PBCH blocks, or - for a set of symbols of a slot that are indicated to an IAB-MT for a CORESET for Type0-PDCCH CSS set, the IAB node should ignore tdd-UL-DL-ConfigurationDedicated-IAB-MT, or - for operation on a single carrier in unpaired spectrum, for a set of symbols of a slot corresponding to a PRACH occasion and Ngap symbols before the valid PRACH occasion,

- References

Chairman’s Notes RAN1#107-e.

R1-2112930, Introduction of enhanced IAB in NR.

Chairman’s Notes RAN1#106bis-e.