3GPP TSG-RAN WG1 Meeting #108-e		                                   					       												           R1-2200940

e-Meeting, February 21 – March 3 , 2022

Agenda Item:	8.16.4

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR NTN

Document for:	Discussion and Decision

# Introduction

In RAN1#107-e, a list of Rel-17 UE features for NR NTN were agreed as baseline for further discussions [1]. In this contribution, we address the remaining aspects.

# Discussion

FG 26-1

- Comment 1: For component 2,

  - Change “UE applies” to “UE calculates” since it focus on calculation of common TA (corresponding to UE-specific TA calculation in component 1).

  - Remove bracket for “[(UE considers common TA as 0 if the parameter is not provided)]”

- Comment 2: For component 4, remove FFS for “UE pre-compensates the calculated TA in its uplink transmissions”

- Comment 3: For component 5, remove bracket for “[by UE-gNB RTT]”

- Comment 4: Remove brackets for  [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported]

- Comment 5: This FG is a per UE capability.

| 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-3

- Comment 1: Merge FG 26-3 with FG 26-1

  - For component 1, remove FFS in the beginning; change “delaying” to “Delay”; add “if it is indicated” in the end

  - For component 2, remove FFS in the beginning; change “delaying” to “Delay”

  - Remove component 3

| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - FFS: delaying Delay the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - FFS: delaying Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated - FFS: separate FGs for cell specific Koffset and Kmac/UE-gNB RTT estimation/delay of RAR/MsgB response window | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-4

- Comment 1: Remove “[The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.]” since there is no need to list the exact content of UE reporting.

- Comment 3: This FG is a per UE capability.

| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - [The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No |  | [Per UE/Per band] | No | No |  |  | Optionalwith capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-5

Comment 1: FG 26-5 is discussed separately in our companion paper [2].

FG 26-6

- Comment 1: For component 1, remove “[for feedback-disabled HARQ processes]” to make it more generic.

- Comment 2: For component 2, remove FFS since HARQ disabling is a pre-requisit for HARQ-ACK codebook enhancement.

- Comment 3: This FGs is a per UE capability.

| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ-ACK codebook Enhancement | - Support of type-2 HARQ-ACK codebook enhancements [for feedback-disabled HARQ processes] - FFS: UE supports HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-6a

- Comment 1: FG 26-6a can be a separate FG from FG 26-1 to ease UE implementation.

- Comment 2: For component 2, remove FFS since HARQ disabling is a pre-requisit for HARQ-ACK codebook enhancement.

- Comment 3: Remove [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] in the last column since it is not a basic UE feature for NTN.

- Comment 4: This FGs is a per UE capability.

| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ-ACK codebook enhancement] | - Enhancement on Type-1 HARQ-ACK codebook in NTN - FFS: UE supports HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-8

- Comment 1: Remove component 2 since it has already been covered by the component 1 hence can be removed.

- Comment 2: This is not a basic feature for NTN and there is no need to merge it with FG26-1. The notes in the second last column and last columncan be removed.

- Comment 3: The feature is about reception of the polarization signaling hence the value for component 1 should be “support” or “not support” instead of the polarization applied at the UE.

- Comment 4: This FG is a per UE capability.

| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-9

- Comment 1: For component 2: remove brackets for [and PDCCH ordered PRACH]; add “if it is indicated.” at the end.

- Comment 2: This FG is a per UE capability.

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS Per UE | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

An update to NTN UE feature groups can be found in Appendix.

# Conclusion

In this contribution, we discuss the Rel-17 UE features for NR NTN. The following proposals are presented:

Proposal: Agree on the updated FGs as listed in the Appendix.

- References

- R1-2111809, “Summary of UE features for NR NTN”.

- R1-2202447, “On support of UE feature(s) for 32 HARQ processes in Rel-17 work items” , Huawei, HiSilicon

# Appendix

| Proposed Rel-17 UE features for NR_NTN_solutions 26. NR_NTN_solutions | 26-1 | Uplink Time pre-compensation | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Delay the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if it is indicated - Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated |  | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No |  | An NTN UE is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about the UE specific TA pre-compensation | Support UE reporting of information about the UE specific TA pre-compensation - [The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No |  | [Per UE/Per band] | No | No |  |  | Optionalwith capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6 | Type-2 HARQ-ACK codebook Enhancement | - Support of type-2 HARQ-ACK codebook enhancements [for feedback-disabled HARQ processes] - FFS: UE supports HARQ disabling |  | Yes | No |  | [Per UE/per band] | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-6a] | [Type-1 HARQ-ACK codebook enhancement] | - Enhancement on Type-1 HARQ-ACK codebook in NTN - FFS: UE supports HARQ disabling | [26-1, 26-2] | Yes | No |  | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | - Reception of UE-specific K_offset via MAC-CE - Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-3, 26-4 | Yes | No |  | FFS Per UE | No | No |  |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell and ATG cell, for terrestrial cell except for ATG cell this feature is not supported] |