3GPP TSG RAN WG1 Meeting #108-e                                    R1-2201021

e-Meeting, February 21st – March 3rd, 2022

Source:	New H3C

Title:	Remaining issues for HARQ-ACK feedback enhancements

Agenda Item:	8.3.1

Document for:	Discussion and Decision

- Introduction

In the RAN1 #107bis-e meeting, HARQ-ACK feedback enhancements in Rel.17 IIOT/URLLC were discussed. Some agreements were made as below [1].

Agreement

For dynamic PUCCH cell switching, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the dynamically indicated PUCCH cell.

Agreement

The following TP to 38.213 is endorsed for the editor’s CR.

9.2.5.4 UE procedure for deferring HARQ-ACK for SPS PDSCH If a UE is provided spsHARQdeferral and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs in a first slot, the UE determines a PUCCH resource for a PUCCH transmission with first HARQ-ACK information bits for SPS PDSCH receptions that the UE would report for a first time, and the PUCCH resource - is provided by SPS-PUCCH-AN-List as described in clause 9.2.1, or by n1PUCCH-AN if SPS-PUCCH-AN-List is not provided - overlaps with a symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated, or indicated for a SS/PBCH block by ssb-PositionsInBurst, or belonging to a CORESET associated with a Type0-PDCCH CSS set the UE - determines an earliest second slot and, after performing the procedures in clauses 9 and 9.2.5 to resolve overlapping among PUCCHs and PUSCHs, a PUSCH or a PUCCH in the earliest second slot to multiplex HARQ-ACK information bits that include second HARQ-ACK information bits from the first HARQ-ACK information bits - if the UE detects a DCI format in a PDCCH reception that triggers a PUCCH transmission with a Type-3 HARQ-ACK codebook in a slot as described in clause 9.1.4, the UE stops the procedure to determine the earliest second slot in the slot - if the UE is provided a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines the earliest second slot and a corresponding cell based on the periodic cell switching pattern as described in clause 9.A

Agreement

For one-shot HARQ-ACK re-transmission,

- the minimum value for the HARQ re-tx offset min_HARQ_retx_offset_value is -7.

- the maximum value for the HARQ re-tx offset max_HARQ_retx_offset_value is 24.

- Note: UE capability reporting on the UE supported value of the minimum value and maximum value range for HARQ_retx_offset in the scope of [min_HARQ_retx_offset_value, max_HARQ_retx_offset_value ] that can be indicated by the gNB for the UE can be further discussed in UE capabilities

Agreement

For one-shot triggering of HARQ-ACK re-transmission, the HARQ_retx_offset is indicated by the bits of the MCS field for transport block 1.

Agreement

Support the simultaneous configuration of one-shot triggering of HARQ re-transmission and SPS deferral

- One-shot HARQ-ACK re-transmission can trigger re-transmission SPS HARQ-ACK enabled with deferring from the initial SPS HARQ deferral slot.

- If the PUCCH slot indicated by the HARQ_retx_offset is the ‘target’ or earliest ‘second’ slot for SPS HARQ-ACK deferral, the HARQ-ACK CB including the deferred SPS HARQ-ACK bits will be retransmitted in the new retransmission PUCCH triggered by one-shot triggering DCI.

- For the SPS HARQ-ACK deferral procedure, the PUCCH slot with a one-shot triggered HARQ-ACK CB is regarded as a valid potential target PUCCH slot for SPS HARQ-ACK deferral with same PHY priority (at least for operation with Rel-16 PHY prioritization) as the PHY priority of the triggered one-shot HARQ-ACK re-transmission.

  - If the PUCCH slot with a one-shot triggered HARQ-ACK CB is determined by the UE as target or earliest second PUCCH slot for SPS HARQ-ACK deferral, the deferred SPS HARQ-ACK in a target slot is appended to the re-transmitted HARQ-ACK CB and initial, new HARQ-ACK (if any) following the operation of SPS HARQ-ACK deferral procedure.

Conclusion

There is no consensus on the support of HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission.

Conclusion

There is no consensus to support the following in Rel-17:

- For one-shot HARQ re-transmission on PUCCH, if certain HARQ process IDs of the requested HARQ CB to be retransmitted is replaced by new HARQ bits, the UE transmits the new content of HARQ process(es) being updated.

Agreement

For PUCCH repetition and one-shot HARQ-ACK re-transmission, if the gNB triggers the HARQ-ACK CB re-transmission from a PUCCH slot indicated by HARQ_retx_offset where a HARQ-ACK in a first PUCCH is dropped due to overlapping with another, second PUCCH, where the first PUCCH and second PUCCH have the same L1 priority, and at least one of the first PUCCH and the second PUCCH is subject to a repetition, the UE re-transmits the HARQ-ACK CB of the second PUCCH from the slot.

In this contribution, we further provide our considerations HARQ-ACK feedback enhancements.

- Discussion

  - Semi-static PUCCH cell switching and PUCCH repetition

Based on the discussion in RAN1#107bis-e meeting [2], for semi-static PUCCH cell switching and PUCCH repetition, the following proposal was proposed during the email discussion.

Mod Proposal 11: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported - A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is postponed as in Rel-16. Example figure for 4 repetitions:

PUCCH repetition transmission on different target PUCCH cells for the semi-static PUCCH pattern introduces more implementation complexity. From our perspective, purpose of PUCCH repetition is only for coverage and any repetition shouldn’t be dropped due to an invalid PUCCH slot. Therefore, the above proposal in RAN1#107bis-e meeting can be considered and this ensures the number of repetitions and the same latency as Rel-16.

Proposal 1: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

- A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is postponed as in Rel-16. Example figure for 4 repetitions:

  - SPS operation with dynamic PUCCH cell switching

Based on the discussion in RAN1#107bis-e meeting [2], for semi-static PUCCH cell switching and PUCCH repetition, the following proposal was proposed during the email discussion.

Alternative Proposal 5 For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement). The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot with UCI

The above proposal in RAN1#107bis-e meeting can be considered as compromise solution because this proposal addresses the hybrid CB construction issue and gNB scheduling is more flexible.

Proposal 2: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot with UCI.

- Conclusion

In this contribution, we discuss about the HARQ-ACK feedback enhancements with the following proposal.

Proposal 1: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported

- A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is postponed as in Rel-16. Example figure for 4 repetitions:

Proposal 2: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot with UCI.

- Reference

- Chairman’s Notes, RAN1#107bis-e, final, January 17th – 25th, 2022

- R1-2200776, Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT, Moderator（Nokia）