3GPP TSG RAN WG1 #108	R1-2201029

e-Meeting, February 21st – March 3rd, 2022

Agenda item:		7.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Absolute TPC timeline and support of absolute TPC with DCI format 2_2

Document for:		Discussion and Decision

# Introduction

During the Rel-17 Coverage Enhancements discussion in RAN1#107bis, the lack of clarity on the UE timeline for applying the group common TPC command received with DCI format 2_2, as well as the support for absolute TPC commands with DCI format 2_2 was identified [4].

# Discussion

## TPC command application timeline for Absolute PC mode

The Rel-17 specifications [2] defines the UE TPC application timeline for PUSCH transmission when tpc-Accumulation is not provided, i.e. for the accumulated TPC mode. Note that tpc-Accumulation polarity is such that if it is absent in the configuration then accumulation is used, if it is present in the configuration, then accumulation is disabled.

PUSCH-PowerControl ::= SEQUENCE {

tpc-Accumulation ENUMERATED { disabled }

tpc-Accumulation

If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled (see TS 38.213 [13], clause 7.1).

### 7.1.1 UE behaviour … - For the PUSCH power control adjustment state fb,f,c(i,l) for active UL BWP b of carrier f of serving cell c in PUSCH transmission occasion i - δPUSCH,b,f,c(i,l) is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in clause 11.3 … - fb,f,ci,l=fb,f,ci-i0,l+m=0CDi-1δPUSCH,b,f,c(m,l) is the PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion i if the UE is not provided tpc-Accumulation, where - The δPUSCH,b,f,c values are given in Table 7.1.1-1 - m=0CDi-1δPUSCH,b,f,c(m,l) is a sum of TPC command values in a set Di of TPC command values with cardinality CDi that the UE receives between KPUSCHi-i0-1 symbols before PUSCH transmission occasion i-i0 and KPUSCH(i) symbols before PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c for PUSCH power control adjustment state l, where i0>0 is the smallest integer for which KPUSCH(i-i0) symbols before PUSCH transmission occasion i-i0 is earlier than KPUSCH(i) symbols before PUSCH transmission occasion i - If a PUSCH transmission is scheduled by a DCI format, KPUSCH(i) is a number of symbols for active UL BWP b of carrier f of serving cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, KPUSCH(i) is a number of KPUSCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of serving cell c - If the UE has reached maximum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≥0, then fb,f,ci,l=fb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of serving cell c at PUSCH transmission occasion i-i0 and m=0CDi-1δPUSCH,b,f,c(m,l)≤0, then fb,f,ci,l=fb,f,ci-i0,l - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=0 - If j=1, l is provided by the value of powerControlLoopToUse - fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1

Observation 1: The timeline for applying the TPC command is provided for accumulated power control adjustment (if the UE is not provided tpc-Accumulation).

Observation 2: The timeline for applying the absolute power control adjustment (if the UE is provided tpc-Accumulation) is missing.

Proposal 1: For absolute power control mode, the same TPC command application timeline is used as for accumulated power control mode.

Proposal 2: Either take a RAN1 conclusion according to proposal 1, or agree to a TP as below:

- fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - The timeline for applying the new TPC command is the same as when the UE is not provided tpc-Accumulation

## Support for absolute TPC commands with DCI format 2_2

During the RAN1#107bis discussions a question on support for absolute TPC commands with DCI format 2_2 was raised.

The 38.213 [1] clearly defines the functionality for the DCI format 2_2 to provide TPC commands for both power control modes:

### 7.1.1 UE behaviour … Table 7.1.1-1: Mapping of TPC Command Field in DCI format 0_0, DCI format 0_1, or DCI format 2_2, with CRC scrambled by TPC-PUSCH-RNTI, or DCI format 2_3, to absolute and accumulated values or values TPC Command Field; Accumulated or [dB]; Absolute or [dB]. 0; -1; -4. 1; 0; -1. 2; 1; 1. 3; 3; 4. … ## 11.3 Group TPC commands for PUCCH/PUSCH … For PUSCH transmission on a serving cell, a UE can be provided - a TPC-PUSCH-RNTI for a DCI format 2_2 by tpc-PUSCH-RNTI - a field in DCI format 2_2 is a TPC command of 2 bits mapping to values as described in Clause 7.1.1 - an index for a location in DCI format 2_2 of a first bit for a TPC command field for an uplink carrier of the serving cell by tpc-Index - an index for a location in DCI format 2_2 of a first bit for a TPC command field for a supplementary uplink carrier of the serving cell by tpc-IndexSUL - an index of the serving cell by targetCell. If targetCell is not provided, the serving cell is the cell of the PDCCH reception for DCI format 2_2 - a mapping for the PUSCH power control adjustment state , by a corresponding {0, 1} value of a closed loop index field that is appended to the TPC command field for the uplink carrier or for the supplementary uplink carrier of the serving cell in DCI format 2_2 if the UE indicates a capability to support two PUSCH power control adjustment states, by twoDifferentTPC-Loop-PUSCH, and if the UE is configured for two PUSCH power control adjustment states by twoPUSCH-PC-AdjustmentStates

So the question boils down to UE capabilities, and there is one UE capability indication for absolute TPC command support, and another for supporting PUSCH power control commands on DCI format 2_2 (support for TPC-PUSCH-RNTI) [3]:

absoluteTPC-Command Indicates whether the UE supports absolute TPC command mode. … tpc-PUSCH-RNTI Indicates whether the UE supports group DCI message based on TPC-PUSCH-RNTI for TPC commands for PUSCH.

There are no other capabilities related to this functionality. Thus it should be clear that if the UE indicates support for both absolute TPC command and DCI format 2_2 power control for PUSCH, then it supports the combination of the two: absolute PUSCH power control on DCI format 2_2.

Observation 3: If the UE indicates support for absoluteTPC-Command and tpc-PUSCH-RNTI, then that UE supports absolute power control with DCI format 2_2 for PUSCH

Proposal 3: Capture the observation 3 as a RAN1 conclusion as below

- Conclusion: It is RAN1’s understanding that If the UE indicates support for absoluteTPC-Command and tpc-PUSCH-RNTI, then that UE supports absolute power control with DCI format 2_2 for PUSCH

# Conclusion

In this document, the following observations and proposals are made:

Observation 1: The timeline for applying the TPC command is provided for accumulated power control adjustment (if the UE is not provided tpc-Accumulation).

Observation 2: The timeline for applying the absolute power control adjustment (if the UE is provided tpc-Accumulation) is missing.

Proposal 1: For absolute power control mode, the same TPC command application timeline is used as for accumulated power control mode.

Proposal 2: Either take a RAN1 conclusion according to proposal 1, or agree to a TP as below:

- fb,f,ci,l=δPUSCH,b,f,ci,l is the PUSCH power control adjustment state for active UL BWP b of carrier f of serving cell c and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - δPUSCH,b,f,c absolute values are given in Table 7.1.1-1 - The timeline for applying the new TPC command is the same as when the UE is not provided tpc-Accumulation

Observation 3: If the UE indicates support for absoluteTPC-Command and tpc-PUSCH-RNTI, then that UE supports absolute power control with DCI format 2_2 for PUSCH

Proposal 3: Capture the observation 3 as a RAN1 conclusion as below

- Conclusion: It is RAN1’s understanding that If the UE indicates support for absoluteTPC-Command and tpc-PUSCH-RNTI, then that UE supports absolute power control with DCI format 2_2 for PUSCH

# References

- 3GPP TS 38.213	Physical layer procedures for control, V15.14.0

- 3GPP TS 38.213	Physical layer procedures for control, V17.0.0

- 3GPP TS 38.306	User Equipment (UE) radio access capabilities, 15.16.0

- R1-2200790	[107bis-e-NR-R17-CovEnh-03] Summary of email discussion on joint channel estimation for PUSCH, moderator (China Telecom)