3GPP TSG RAN WG1 #108-e			R1-2201037

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.2.5

Source:	InterDigital, Inc.

Title:		Remaining issues for PDSCH/PUSCH enhancements to supporting 52.6-71 GHz band in NR

Document for:	Discussion and Decision

# Introduction

In RAN1 #107bis-e, RAN1 continued the discussion on enhancements for PDSCH and PUSCH to support NR operation in 52.6 GHz to 71 GHz band. Based on the discussion, the agreements and the conclusions given in the Annex were reached on PDSCH/PUSCH enhancements alongside relevant TPs [1]. In this contribution, we discuss potential enhancements for remaining PDSCH/PUSCH related aspects in 52.6 – 71GHz and associated standard impacts.

# Discussions

## Out-of-Order Scheduling

In RAN1#106bis-e [3], it was agreed that for two multi-PDSCH (or two multi-PUSCH) scheduling DCIs, UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCI to lead to out-of-order scheduling. Also, it was agreed to further study whether to allow out-of-order scheduling for following two cases,

- One multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH).

- Two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans. Here the span of a DCI is defined from the beginning of the first scheduled SLIV until the end of the last scheduled SLIV by a DCI.

These two cases were extensively discussed in RAN1#107-e [2] and RAN1#107-bis-e [1] meetings and it was agreed to not to support the first case, i.e., to consider case 1 as an out-of-order scheduling in the last meeting. Also, it was noted that, this agreement may not need any specification update.

In Rel-15/Rel-16, the out-of-order scheduling was not supported as it can affect the ability to perform pipeline processing of PDSCHs at the UE making the PDSCH decoding process complicated. With multiple PDSCHs/PUSCHs scheduled by a single DCI, UE processing could be more complicated if out-of-order scheduling is allowed. Therefore, we propose to consider case 2 as out-of-order scheduling and thus not to support = such scheduling. Based on the discussion, we make the following proposal.

Proposal 1: UE does not expect to receive two multi-PDSCH (or multi-PUSCH) scheduling DCIs ending in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs having overlapping spans.

Also in the last meeting, the special case of one out of the two DCIs scheduling a single PDSCH (or PUSCH) was considered. This special case enables opportunistic scheduling of a high-priority single-PDSCH (or single PUSCH) since the multi-PDSCH/PUSCH scheduling is limited to having all PDSCHs/PUSCHs with the same priority index.

The practical benefits of such scheme are questionable given the short duration of slots in the high SCS considered for NR operation in 52.6 GHz to 71 GHz band.  We note here that in the event that this situation would occur, even in the worst case of 120kHz SCS, the increased delay by not allowing such scheduling for this case would not be significant. Given the added UE complexity associated to supporting such scheduling and the limited benefits, we make the following proposal.

Proposal 2: UE does not expect the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI end in the same symbol, and the single-PDSCH (or single-PUSCH) is scheduled during the span of multi-PDSCH (or multi-PUSCH) scheduling DCI.

Another design problem discussed in RAN1#107-bis-e [1] is related to whether valid SLIV or scheduled PDSCHs/PUSCHs should be used in determining if out-of-order scheduling is taking place.  In practice a UE does not receive or transmit when an invalid SLIV is detected.  From the UE complexity point-of-view there is no significant difference between having an invalid SLIV and not being scheduled.  As such in our opinion only the valid SLIV should be considered for determining if an out-of-order scheduling situation is taking place. Therefore, we propose that,

Proposal 3: Out-of-order scheduling determination should be based on valid SLIVs.

## DMRS Bundling Across Multiple PUSCHs

In RAN1#107-bis-e [1], whether to support DMRS bundling for multi-PUSCH scheduling by a single DCI in FR2-2 was discussed. Subsequently, it was agreed that the DMRS bundling across multiple PUSCHs scheduled by a single DCI is not supported for NR operation in FR2-2 in Rel-17. It was also agreed to further discuss the applicability of DMRS bundling FG developed in CovEnh WI for the same TB to FR2-2 with 120 kHz SCS. The above-mentioned agreements were made based on two main motivations. First, the CovEnh WI only considers contiguous scheduling of PUSCHs while multi-PUSCH scheduling in FR2-2 can have slot level gaps between scheduled PUSCHs. Second, in CovEnh WI, repetition of the same TB is considered while in multi-PUSCH scheduled by a single DCI in FR2-2, each slot corresponds to different TB. Having said that, it is worth to note that TB repetition across multiple contiguous slots can be supported in FR2-2 by using the existing features. In such a situation, DMRS bundling FG developed in CovEnh WI could be used to improve the channels estimation performance. Therefore, the use of this FG developed in CovEnh WI should not be prohibited in FR2-2. Based on the discussion, we make the following proposal.

Proposal 4: The use of DMRS bundling FG developed in CovEnh WI for the same TB should not be prohibited in FR2-2 with 120 kHz SCS.

# Summary

In this contribution, we discussed the issues on PDSCH/PUSCH enhancements of NR in 52.6 – 71 GHz. From the discussions, we made following observation and proposals:

Proposal 1: A UE does not expect to schedule with two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) scheduling DCIs have overlapping spans. Here, the span of a DCI is defined from the beginning of the first scheduled SLIV until the end of the last scheduled SLIV.

Proposal 2: A UE doesn’t expect the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI end in the same symbol, and the single-PDSCH (or single-PUSCH) is scheduled during the span of multi-PDSCH (or multi-PUSCH) scheduling DCI.

Proposal 3: Out-of-order scheduling determination should be based on valid SLIVs.

Proposal 4: The use of DMRS bundling FG developed in CovEnh WI for the same TB should not be prohibited in FR2-2 with 120 kHz SCS.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #107bis-e, January 2022,

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #107-e, November 2021,

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #106bis-e, October 2021.

# Annex

Agreement For NR operation with 480 kHz and/or 960 kHz SCS, select the following as the set of values for HARQ Feedback Timing Indicator field in successRAR. - {7, 8, 12, 16, 20, 24, 28, 32} for 480 kHz and {13, 16, 24, 32, 40, 48, 56, 64} for 960 kHz - Note: this is the same as PDSCH-to-HARQ_feedback timing indicator field in DCI format 1_0 for 480 kHz and 960 kHz SCS - The following example change to section 8.2A in TS 38.213 can be recommended to the editor to use at the editor’s discretion ----- Start of TP --- Unchanged parts omitted --- If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer - an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or - transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where - a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon - a slot for the PUCCH transmission is indicated by a HARQ Feedback Timing Indicator field of 3 bits in the successRAR having a value from {1, 2, 3, 4, 5, 6, 7, 8} for , from {7, 8, 12, 16, 20, 24, 28, 32} for , from {13, 16, 24, 32, 40, 48, 56, 64} for and, with reference to slots for PUCCH transmission having duration , the slot is determined as , where is a slot of the PDSCH reception, is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], is the SCS configuration of the active UL BWP, and is provided by Koffset in ServingCellConfigCommon; otherwise, if not provided, - the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than msec where is the PDSCH processing time for UE processing capability 1 [6, TS 38.214] --- Unchanged parts omitted --- ----- End of TP ----- Agreement For NR operation with 480 kHz and/or 960 kHz SCS, scale the value of N for 120 kHz SCS by 4 and 8 for 480 kHz and 960 kHz SCS respectively, where N symbols are for PDSCH corresponding to SI-RNTI in Clause 5.1 of TS38.214. - The following example change to 38.214 Section 5.1 can be recommended to the editor to use at the editor’s discretion ----- Start of TP --- Unchanged parts omitted --- In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration m, with N=13 for m=0, N=13 for m=1, N=20 for m=2, and N=24 for m=3, N=96 for m=5, and N=192 for m=6. --- Unchanged parts omitted --- ----- End of TP ----- Agreement For NR operation with 480 kHz and/or 960 kHz SCS, scale 14 symbols for SPS PDSCH cancelation in Clause 5.1 of TS38.214 by 4 and 8 for 480 kHz and 960 kHz SCS respectively. - The following example change to 38.214 Section 5.1 can be recommended to the editor to use at the editor’s discretion ----- Start of TP --- Unchanged parts omitted --- The UE is not expected to decode a PDSCH in a serving cell scheduled by a PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI and one or multiple PDSCH(s) required to be received according to this Clause in the same serving cell without a corresponding PDCCH transmission if the PDSCHs partially or fully overlap in time except if the PDCCH scheduling the PDSCH ends at least 14*2max{0,μ-3} symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, where m and the symbol duration is based on the smallest numerology between the scheduling PDCCH and the PDSCH, in which case the UE shall decode the PDSCH scheduled by the PDCCH. When the PDCCH candidates are associated with a search space set configured with searchSpaceLinking, for the purpose of determining the PDCCH with C-RNTI, CS-RNTI or MCS-C-RNTI scheduling the PDSCH ends at least 14*2max{0,μ-3} symbols before the earliest starting symbol of the PDSCH(s) without the corresponding PDCCH transmission, the PDCCH candidate that ends later in time among the two configured PDCCH candidates is used. --- Unchanged parts omitted --- ----- End of TP ----- Agreement For NR operation with 480 kHz and/or 960 kHz SCS, scale 42 symbols for SRS precoding information update in Clause 6.1.1.2 of TS38.214 by 4 and 8 for 480 kHz and 960 kHz SCS respectively. - The following example change to 38.214 Section 6.1.1.2 can be recommended to the editor to use at the editor’s discretion ----- Start of TP --- Unchanged parts omitted --- For non-codebook based transmission, the UE can calculate the precoder used for the transmission of SRS based on measurement of an associated NZP CSI-RS resource. A UE can be configured with only one NZP CSI-RS resource for the SRS resource set with higher layer parameter usage in SRS-ResourceSet set to ‘nonCodebook’ if configured. - If aperiodic SRS resource set is configured, the associated NZP-CSI-RS is indicated via SRS request field in DCI format 0_1 and 1_1, as well as DCI format 0_2 (if SRS request field is present) and DCI format 1_2 (if SRS request field is present), where AperiodicSRS-ResourceTrigger and AperiodicSRS-ResourceTriggerList (indicating the association between aperiodic SRS triggering state(s) and SRS resource sets), triggered SRS resource(s) srs-ResourceSetId, csi-RS (indicating the associated NZP-CSI-RS-ResourceId) are higher layer configured in SRS-ResourceSet. The SRS-ResourceSet(s) associated with the SRS request by DCI format 0_1 and 1_1 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModList and the SRS-ResourceSet(s) associated with the SRS request by DCI format 0_2 and 1_2 are defined by the entries of the higher layer parameter srs-ResourceSetToAddModListDCI-0-2. A UE is not expected to update the SRS precoding information if the gap from the last symbol of the reception of the aperiodic NZP-CSI-RS resource and the first symbol of the aperiodic SRS transmission is less than OFDM symbols, where the SCS configuration μ is the smallest SCS configuration between the NZP-CSI-RS resource and the SRS transmission. --- Unchanged parts omitted --- ----- End of TP ----- Conclusion DMRS bundling across multiple PUSCHs scheduled by a single DCI is not supported for NR operation in FR2-2 in Rel-17. - Note: applicability of DMRS bundling FG developed in CovEnh WI for same TB to FR2-2 with 120 kHz SCS can be further discussed. Agreement - In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability. Agreement - If the higher layer parameter maxNrofCodeWordsScheduledByDCI indicates that two codeword transmission is enabled and more than one PDSCH is scheduled by a multi-PDSCH scheduling DCI, - Either the first or the second transport block of all scheduled PDSCHs is disabled by the DCI if IMCS = 26 and if RV bits are set to ‘1’ for the corresponding transport block of all scheduled PDSCHs (i.e. irrespective of whether this is a valid PDSCH). Conclusion For multi-PDSCH or multi-PUSCH scheduling DCI, the following maximum value of a gap is not specified in Rel-17 and up to gNB scheduler. - The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH Conclusion HARQ process number configured for SPS PDSCH (or CG PUSCH) can be allocated to a PDSCH (or PUSCH) of multi-PDSCH (or multi-PUSCH) scheduling, as long as the timeline condition defined in Rel-15/16 is met. - Note: It is up to gNB implementation whether/how to avoid UL data retransmission due to HARQ process index collision and flushed HARQ transmit buffer. Agreement - Update the previous agreement made in RAN1#107-e, as follows: Agreement (RAN1#107-e) For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., enableTimeDomainHARQ-Bundling, to enable time domain bundling operation for type-1 HARQ-ACK codebook per serving cell. - If the RRC parameter enables time domain bundling operation, - To determine the set of candidate PDSCH reception occasions, - A row index is removed if at least one symbol of every PDSCH associated with the row index is configured as semi-static UL. (NOTE: This is similar to the case of slot aggregated PDSCH in Rel-16) - Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index. - Logical AND operation is applied across all valid PDSCHs associated with a determined candidate PDSCH reception occasion, at least for 1-TB case. - FFS: UE does not expect the last scheduled SLIV overlaps with a semi-static UL symbol when parameter enableTimeDomainHARQ-Bundling is configured Agreement For multi-PDSCH scheduling with a single DCI and for type-2 HARQ-ACK codebook generation, - Time domain bundling and spatial bundling can be independently configured. Conclusion - UE does not expect any of the scheduled PDSCHs (or PUSCHs) and the scheduling DCIs to lead to out-of-order scheduling, also for the case of one multi-PDSCH (or multi-PUSCH) scheduling DCI and one single-PDSCH (or single-PUSCH) scheduling DCI, where multi-PDSCH (or multi-PUSCH) scheduling DCI schedules more than one PDSCH (or PUSCH). - This may not have specification impact. - Note: It is separately discussed whether the scheduled PDSCHs (or PUSCHs or SLIV) is based on configured SLIV or valid SLIV. Conclusion UE does not expect any of the received PDSCHs (including SPS PDSCH) and the resource for the HARQ-ACK transmission to lead to out-of-order scheduling, for any scheduling DCIs (including multi-PDSCH scheduling DCI). Agreement For a DCI that can schedule multiple PDSCHs or multiple PUSCHs, - It is clarified that NDI/RV fields in the following previous agreements correspond to scheduled PDSCHs indicated by the TDRA information field. Agreement: (RAN1#104-bis) For a DCI that can schedule multiple PDSCHs, - NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH - RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH Agreement: (RAN1#106bis-e) For a DCI that can schedule multiple PDSCHs, and if RRC parameter configures that two codeword transmission is enabled, - NDI for the 2nd TB: This is signalled per PDSCH and applies to the 2nd TB of each PDSCH - RV for the 2nd TB: This is signalled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the 2nd TB of each PDSCH. - Above clarification also applies to the DCI scheduling multiple PUSCHs, i.e., NDI/RV fields in the DCI correspond to scheduled PUSCHs indicated by the TDRA information field. - The following example change to 38.214 Sections 5.1.3 and 6.1.4 can be recommended to the editor of 38.214 to use at the editor’s discretion ---------------------------Start of TP for TS 38.214 Clause 5.1.3 ----------------------------------------------- 5.1.3 Modulation order, target code rate, redundancy version and transport block size determination ================ Unchanged Text Omitted ======================= When the UE is scheduled with multiple PDSCHs by a DCI, as described in clause 5.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one-to-one mapped to the scheduled PDSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order, where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PDSCH indicated by the TDRA information field. ---------------------------------------------- End of TP -------------------------------------------------------------- ---------------------------Start of TP for TS 38.214 Clause 6.1.4 ----------------------------------------------- 6.1.4 Modulation order, redundancy version and transport block size determination ================ Unchanged Text Omitted ======================= When the UE is scheduled with multiple PUSCHs by a DCI, as described in clause 6.1.2.1, the bits of rv field and NDI field, respectively, in the DCI are one to one mapped to the scheduled PUSCH(s) indicated by the TDRA information field with the corresponding transport block(s) in the scheduled order where the LSB bits of the rv field and NDI field, respectively, correspond to the last scheduled PUSCH indicated by the TDRA information field. ---------------------------------------------- End of TP -------------------------------------------------------------- Conclusion It is clarified that the absence or presence of CBGTI field in the following previous agreement is determined based on scheduled PUSCHs indicated by the TDRA information field (i.e. irrespective of whether this is a valid PUSCH). Agreement: (RAN1#105-e) - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.. Agreement A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV. Agreement For type-1 HARQ-ACK codebook, if OACK+OSR+OCSI≤11, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH, as follows. - For a serving cell c configured with enableTimeDomainHARQ-Bundling, and for a DCI format indicating a TDRA row that includes more than one SLIV entry on the serving cell c, the UE considers a PDSCH (which carries one or two transport blocks enabled by the DCI format irrespective of whether the PDSCH is valid or not) only associated with the last SLIV as received, to determine Nm,creceived. The TP below for TS38.213v17.0.0 is endorsed. -------------------------------------------Start of TP#A1 for TS 38.213 Clause 9.1.2.1 ----------------------------------------------- If a UE is not provided ca-SlotOffset for any serving cell of PDSCH receptions and for the serving cell of corresponding PUCCH transmission with HARQ-ACK information while k<CK1 if modnU-K1,k+1,max2μUL-μDL,1=0 or subslotLengthForPUCCH is provided for the HARQ-ACK codebook Set nD=0 – index of a DL slot overlapping with an UL slot Set Nk to a number of DL slots overlapping with UL slot nU-K1,k if subslotLengthForPUCCH is provided for the HARQ-ACK codebook; otherwise, Nk=max2μDL-μUL,1 while nD<Nk …… if slot nU starts at a same time as or after a slot for an active DL BWP change on serving cell c or an active UL BWP change on the PCell and slot n0,k+nD is before the slot for the active DL BWP change on serving cell c or the active UL BWP change on the PCell, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and slot n0,k+nD overlaps with UL slot nU-K1,k-1, k>0, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, nD=nD+1; else while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of each PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while …… nD=nD+1; end if end while end if k=k+1; end while else …… end if -------------------------------------------End of TP#A1 for TS 38.213 Clause 9.1.2.1 ----------------------------------------------- The TP below for TS38.214v17.0.0 is endorsed. --------------------------------------------Start TP#C1 for TS 38.214 Clause 5.1.2.1.1 ----------------------------------------------- Table 5.1.2.1.1-1: Applicable PDSCH time domain resource allocation for DCI formats 1_0, 1_1, 4_0, 4_1 and 4_2 MCCH-RNTI; Type 0/0B common for broadcast; 1; No; -; No; -; Default A. 2; No; -; No; -; Default B. 3; No; -; No; -; Default C. 1,2,3; Yes; -; No; -; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon. 1,2,3; No/Yes; -; Yes; -; pdsch-TimeDomainAllocationList provided in pdsch-Config-MCCH. G-RNTI for broadcast; Type 0/0B common for broadcast; 1; No; -; No; -; Default A. 2; No; -; No; -; Default B. 3; No; -; No; -; Default C. 1,2,3; Yes; -; No; -; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon. 1,2,3; No/Yes; -; Yes; -; TimeDomainAllocationList provided in PDSCH-Config-MCCH. 1,2,3; No/Yes; -; Yes; -; pdsch-TimeDomainAllocationList provided in PDSCH-Config-MTCH. C-RNTI, MCS-C-RNTI, CS-RNTI; Any common search space associated with CORESET 0; 1, 2, 3; No; -; -; -; Default A. 1, 2, 3; Yes; -; -; -; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon. C-RNTI, MCS-C-RNTI, CS-RNTI; Any common search space not associated with CORESET 0 UE specific search space; 1,2,3; No; No; -; -; Default A. 1,2,3; Yes; No; -; -; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon. 1,2,3; No/Yes; Yes; -; -; pdsch-TimeDomainAllocationList provided in PDSCH-Config. 1,2,3; No/Yes; -; -; Yes; pdsch-TimeDomainAllocationListForMultiPDSCH-r17 provided in PDSCH-Config (Note 2). G-RNTI, G-CS-RNTI (for multicast); Type-X common search space for multiast; 1,2,3; No; -; No; -; Default A. 1,2,3; Yes; -; No; -; pdsch-TimeDomainAllocationList provided in PDSCH-ConfigCommon (Note 1). 1,2,3; No/Yes; -; Yes; -; pdsch-TimeDomainAllocationList provided in pdsch-Config-Multicast (Note 1). Note 1: For a UE that supports multicast, the same TDRA table applies to all G-RNTIs (configured for multicast) if configured on a given serving cell. Note 2: If pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided, it is applicable to DCI format 1_1 only.. --------------------------------------------End of TP#C1 for TS 38.214 Clause 5.1.2.1.1 ----------------------------------------------- The TP below for TS38.213v17.0.0 is endorsed. --------------------------------------------Start of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------ --------------------------------------------Start of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------ 9.1.2.1 Type-1 HARQ-ACK codebook in physical uplink control channel =============================== Unchanged Text Omitted =================================== while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if pdsch-TimeDomainAllocationListForMultiPDSCH-r17 is provided and HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while --------------------------------------------End of TP#F for TS 38.213 Clause 9.1.2.1 ------------------------------------------------ The TP below for TS38.214v17.0.0 is endorsed. ----------------------------------------------Start of TP#G for TS 38.214 Clause 6.1 -------------------------------------------------- 6.1 UE procedure for transmitting the physical uplink shared channel =============================== Unchanged Text Omitted =================================== For uplink, 16 HARQ processes per cell are supported by the UE, or subject to UE capability, a maximum of 32 HARQ processes per cell for the cases of = 5 or = 6. The number of processes the UE may assume will at most be used for the uplink is configured to the UE for each cell separately by higher layer parameter nrofHARQ-ProcessesForPUSCH, and when no configuration is provided the UE may assume a default number of 16 processes. ------------------------------------------------------------End of TP#G----------------------------------------------------------------