3GPP TSG RAN WG1 Meeting #108-e		R1-2201046

e-Meeting, February 21st – March 3rd, 2022

Title:	                     [Draft] Reply LS on PRU for enhanced positioning performance

Response to:	R2-2111488

Release:	Rel-17

Work Item:	         NR_pos_enh

Source:	vivo[RAN1]

To:	RAN2

Cc:	SA2

Contact person:

E-mail Address:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1	Overall description

RAN1 thanks RAN2 for the LS and would like to provide the response.

RAN2 kindly asks RAN1 whether the LMF determined "correction information" obtained from PRU measurements need to be provided to target UEs for UE-based mode of operation, and if so, kindly asks RAN1 to provide further details on the specific "correction information" which need to be provided to target UEs. RAN2 also kindly asks RAN1 to provide further details on the "PRU antenna orientation information" which should be provided to an LMF.

Answer :

RAN1 thinks providing "correction information" is beneficial for UE-based positioning. And at least RSTD correction information should be included for UE-based DL TDOA.

For  "PRU antenna orientation information", can be seen as the boresight direction information of PRU, and the contents are similar to (the boresight direction of a beam in AoD positioning(ie. DL-PRS-BeamInfoElement-r16)).

2	Actions

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into consideration in their future work.

3	Dates of next TSG RAN 1 meetings

TSG-RAN WG1 Meeting #109-e   16 – 27 May. 2022                                Electronic Meeting

TSG-RAN WG1 Meeting #110      22 – 26 August. 2022                            Toulouse, FR