3GPP TSG RAN WG1 #108-e			R1-2201051

e-Meeting, February 21st - March 3rd, 2022

Title:	Draft LS reply on UL BWP with PRACH resources only for RACH with Msg3 repetition

Response to:	R2-2201982 (R1-2200885)

Release:	Rel-17

Work Item:	NR_covEnh-Core

Source:	vivo [to be RAN1]

To:	RAN2

Contact Person:

#### Name:	Zhipeng Lin

E-mail Address:	zhipeng.lin@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN2 for the LS in R1-2200885 (R2-2201982) on whether the network can configure only CE RACH resources in a dedicated UL BWP.

RAN1 has discussed the following working assumption mentioned in the LS:

- “Working assumption: From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.”

From RAN1 perspective, it’s technically possible to configure a dedicated UL BWP with only CE RACH resources when CE UE is in RRC_CONNECTED state. However, following concerns are identified in RAN1 for such configuration:

- In a BWP without non-CE RACH resources, UE will always have to request Msg3 repetition without checking the measured RSRP, which may cause unnecessary Msg3 PUSCH resource waste when Msg3 repetition is not needed.

- It’s too complex and not pursued to have different procedures to determine the CE or non-CE for different BWPs at this maintenance stage.

- CE only BWP means the support of separate RO configuration for Msg3 repetition which is not agreed yet.

- The motivation of only supporting CE RACH resource in a BWP is not clear.

According to above, RAN1 does not see it feasible to configure a dedicated UL BWP with only CE RACH resources.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information in to account.

3. Date of Next RAN1 Meetings:

TSG RAN1 Meeting #109-e	16th - 27th May 2022		e-Meeting

TSG RAN1 Meeting #110		22nd - 26th Aug. 2022 		Toulouse, FR