3GPP TSG RAN WG1 #108e	                                                              R1-2201062

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Remaining issues on Rel-17 Tx switching enhancements

Agenda Item:	5.1

Document for:	Discussion and Decision

- Introduction

In the last several RAN1 meetings, the Rel-17 2Tx-2Tx switching between two uplink carriers and the 1Tx-2Tx and 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B are discussed and agreed. In this contribution, we analyze the relationship between the conclusions and the latest R17 specification and suggest to making some clarification.

- Discussion

For a UE configured with UL CA and with 1Tx-2Tx UL Tx switching, the mechanism of uplink switching specified in section 6.1.6.2, with 2Tx-2Tx UL Tx switching, the mechanism of uplink switching specified in section 6.1.6.2.1 of TS 38.214. We suggest the following changes to clarify the UL Tx switching mechanism with the same color remark:

- To include the three carriers Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, change “any to the two carriers” and “either of the two carriers” to “any of the uplink carriers”, change “is” and “was” to “included”.

- To clarify the Tx switching between two different bands also to include the three carriers Tx switching cases, add “on one band” and “on another band” in the description for UE transmission. To clarify the simultaneous transmissions on two bands requiring more than 2 ports are not allowed, add “on one band” and “on another band” in the last but one paragraph in each section.

- To exclude the Tx switching between the two carriers in Band B, add “or on another contiguous carrier on the same band” for the UE configured with uplinkTxSwitchingOption set to 'dualUL'.

- In section 6.1.6.2.1, to clarify the cases only when UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, the Tx chain is differentiated by RRC parameter, change “1 port transmission” to “only 1 port transmission” for the UE configured with uplinkTxSwitchingOption set to 'dualUL'.

- In section 6.1.6.2.1, the following Tx switching cases are not included for the UE configured with uplinkTxSwitchingOption set to 'dualUL', e.g.:

  - Rel-17 2Tx-2Tx switching between two uplink carriers

    - case 2{0P+1P}case1{1P+1P}

    - case 3{1P+0P}case 1{1P+1P}

- Rel-17 2Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B

    - case 2{0P+(1P+0P),0P+(0P+1P),0P+(1P+1P)}case 1{1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P)}

    - case 3{1P+(0P+0P)}case 1{1P+(1P+0P), 1P+(0P+1P), 1P+(1P+1P)}

So, the following description should be added for the UE configured with uplinkTxSwitchingOption set to 'dualUL': when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

Proposal 1: TP1 for TS 38.214 clause 6.1.6.2 is adopted.

-------------------------------------------- TP1 for 38.214 6.1.6.2-------------------------------------------------------------

<unchanged part omitted>

6.1.6.2	Uplink switching for carrier aggregation

For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with uplink carrier aggregation:

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-    When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-    For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-    For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port transmission on the same uplink carrier  or on another contiguous carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-	For the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the  uplink carriers.

-    The UE is not expected to be scheduled or configured with uplink transmissions that result in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band.

-	In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

<unchanged part omitted>

-------------------------------------------------END OF TP1-----------------------------------------------------------------

Proposal 2: TP2 for TS 38.214 clause 6.1.6.2.1 is adopted.

-------------------------------------------- TP2 for 38.214 6.1.6.1-------------------------------------------------------------

<unchanged part omitted>

6.1.6.2.1	2Tx Uplink switching for carrier aggregation

[For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch2TX for a band combination, and if it is for that band combination configured with uplink carrier aggregation:]

-	If the UE is configured with uplink switching with parameter uplinkTxSwitching, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s):

-	When the UE is to transmit a 2-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port or 2-port transmission on another uplink carrier on another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-	When the UE is to transmit a 1-port or 2-port transmission on one uplink carrier on one band if the preceding uplink transmission included a 2-port transmission on another uplink carrier on  another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-    For the UE configured with uplinkTxSwitchingOption set to 'switchedUL', when the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission included a 1-port transmission on another uplink carrier on  another band, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-    For the UE configured with uplinkTxSwitchingOption set to 'dualUL',

-	When the UE transmitted 1-port or 2-port transmission on one carrier on one band followed by no transmission on this carrier or on another contiguous carrier on the same band and only 1-port transmission on the other carrier on another band, if the UE was configured with [yyy] for subsequent switching consideration the UE shall consider this as if 1-port transmission was transmitted on both uplinks, otherwise the UE shall consider this as if 2-port transmission took place on the transmitting carrier.

-    When the UE is to transmit a 1-port transmission on one uplink carrier on one band and if the preceding uplink transmission was a 1-port transmission on another uplink carrier on another band and the UE is under the operation state in which 2-port transmission can be supported on the same uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the uplink carriers.

-	The UE is not expected to be scheduled or configured with uplink transmissions that results in simultaneous transmission on two antenna ports on one uplink carrier on one band, and any transmission on another uplink carrier on another band.

-	When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission included a 1-port on the same uplink carrier or on another contiguous carrier on the same band and the UE is under the operation state in which 2-port transmission cannot be supported in either uplink carrier, then the UE is not expected to transmit for the duration of  NTx1-Tx2 on any of the uplink carriers.

-  In all other cases the UE is expected to transmit normally all uplink transmissions without interruptions.

<unchanged part omitted>

-------------------------------------------------END OF TP2-----------------------------------------------------------------

- Conclusion

In this contribution, we discuss the remaining RAN1 issues to support the Rel-17 enhanced UL Tx switching and have the following proposals:

Proposal 1: TP1 for TS 38.214 clause 6.1.6.2 is adopted.

Proposal 2: TP2 for TS 38.214 clause 6.1.6.2.1 is adopted.

References

- 3GPP TS 38.214 V17.0.0 (2021-12), Physical layer procedures for data

- R1-2112955, Introduction of UL Tx Switching enhancements, 3GPP TSG-RAN WG1 Meeting #107-e