3GPP TSG RAN WG1 Meeting #108-e					R1-2201125

e-Meeting, February 21st – March 3rd, 2022

Source:	vivo

Title:	Discussion on UE features for UE power saving enhancements

Agenda Item:	8.16.7

Document for:	Discussion and Decision

- Introduction

This contribution addresses our proposals for Rel-17 power saving UE features. The discussion is based on the latest version of UE features summary in R1-2200678. [1].

R1-2200768	LS on UE capability for paging enhancement	RAN1, Ericsson

Agreement

- FG 29-3c is confirmed as “3 search space sets group switching” as follows

| 29. NR_UE_pow_sav_enh | [29-3c] | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching FFS whether to merge with 29-3b | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 29-3d is confirmed as “2 search space sets group switching with PDCCH skipping” as follows

| 29. NR_UE_pow_sav_enh | [29-3d] | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-23b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Discussion on Rel-17 UE Power saving features

29-1 PEI

The current version of PEI from [1] is as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Descriptions of the components

Paging early indication is not specification terminology in TS38.213. A new DCI format 2_7 and new PEI-RNTI is created for this. Hence it should be updated.

Proposal 1:

- Update the descriptions of 29-1 as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indicationSupport of detection of DCI format 2_7 with CRC scrambled with PEI-RNTI for early indication of paging 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

29-2 TRS

The current version of PEI from [1] is as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Descriptions of the components

RAN1 has agreed that the L1 indication for TRS availability can be either PEI (DCI 2_7) or paging DCI (DCI 1_0). However, receiving TRS availability by PEI (DCI 2_7) requires UE to support 29-1 first. Hence, it is  better to separate it as a independent feature and add 29-1 as its prerequisite .

Proposal 2:

- Update the descriptions of 29-2 as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2a | TRS resources for idle/inactive UEs | 3. Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7 | 29-1, 29-2 | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |

29-3 PDCCH monitoring adaptation within an active BWP

The following is agreed in RAN1#107bis-E,

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

- For 29-3a, the description of the component is stable enough so that the yellow color can be removed.

- For 29-3c,  the description of the component need some minor correction. It should be clearly stated that 29-3c is for without PDCCH skipping (similar to the description of 29-3b)

- For all the 29-3a/3b/3c/3d, it should be per UE feature, and no need for differential for TDD/FDD, FR1/FR2

Proposal 3:

- Update the descriptions of 29-3 as follows,

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching, without PDCCH skipping | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

- Conclusions

This contribution addresses our proposals for Rel-17 power saving UE features.,

Proposal 1:

- Update the descriptions of 29-1 as follows,

| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indicationSupport of detection of DCI format 2_7 with CRC scrambled with PEI-RNTI for early indication of paging 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2:

- Update the descriptions of 29-2 as follows,

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability via DCI 1_0 |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-2a | TRS resources for idle/inactive UEs | 3. Support receiving L1 indication for TRS availability via DCI 2_7 if the UE supports reception of DCI 2_7 | 29-1, 29-2 | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |

Proposal 3:

- Update the descriptions of 29-3 as follows,

| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skipping |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching, without PDCCH skipping | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

# Reference

- R1-2200678		Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.)

- RP-212611,		Moderator’s summary for discussion [93e-18-PowSav-WI]: Final Round, 	Moderator (CMCC)