3GPP TSG RAN WG1 #108-e		R1-2201126

e-Meeting, February 21st - March 3rd, 2022

Source:	vivo

Title:	Discussion on UE features for NR coverage enhancement

Agenda Item:	8.16.8

Document for:	Discussion and Decision

- Introduction

This contribution includes our proposals for Rel-17 coverage enhancement UE features. The discussion is based on the latest version of UE features summarized in R1-2200780 [1].

- Discussion

- Type-A PUSCH repetition Enh. related features

Agreement - Further discuss whether/how to separate/merge FGs 30-1 and 30-1a from following options - Option 1: Keep current structure, i.e. FG 30-1 for DG, 30-1a for type 1 and 2 CG - Option 3: Merge FGs 30-1 and 30-1a into an FG

In RAN1#107bis-e meeting, for increased maximum number of dynamic grant based Type A PUSCH repetitions, whether FGs are separated for CG PUSCH and DG PUSCH are discussed. Since in Rel-15, CG-PUSCH and DG-PUSCH are separate capabilities, FG 30-1 and 30-1a should also be separated capabilities.

Similarly, current structure of FG 30-2 and 30-2a should be kept, i.e. FG 30-2 for DG, 30-2a for type 1 and 2 CG.

Proposal 1: For increased maximum number of Type A PUSCH repetitions, current FG structure should be kept, i.e. FG 30-1 for DG, 30-1a for type 1 and 2 CG.

Proposal 2: Current structure of FG 30-2 and 30-2a should be kept, i.e. FG 30-2 for DG, 30-2a for type 1 and 2 CG.

- TBoMS related features

In RAN1#106-e meeting, it has been agreed that the number of slots for TBoMS is counted based on available slots for UL transmission, and the available slots are determined based on the mechanisms determined for Type A PUSCH repetition counted on available slots. Hence, feature 30-2, i.e., type-A PUSCH repetitions counted on available slots, should be considered as prerequisite feature for feature 30-3.

Proposal 3: For TBoMS, feature 30-2 should be included as prerequisite feature for feature 30-3.

Agreement - Further discuss whether/how to separate FG 30-3 from following options - Option 1: Keep current structure - Option 3: Split 30-3 into 2 separate FGs: 1st one for DG, 2nd one for CG

Besides, FG 30-2 is split to 2 separate FGs, i.e. one for CG and one for DG. Similarly, FG 30-3 should be split into 2 FGs for CG and DG respectively.

Proposal 4: For TBoMS, split FG 30-3 into 2 FGs for DG and CG respectively.

- DMRS bundling related features

For DMRS bundling related features, UEs may have different capabilities on maintaining phase continuity and power consistency on different bands, according to RAN4 LS R4-2202368 [2]. Hence, DMRS bundling related features should be per band.

Proposal 5: DMRS bundling related FGs should be per band.

- Conclusion

In this contribution, we discussed the coverage enhancement related UE features, and we have the following proposals.

Proposal 1: For increased maximum number of Type A PUSCH repetitions, current FG structure should be kept, i.e. FG 30-1 for DG, 30-1a for type 1 and 2 CG.

Proposal 2: Current structure of FG 30-2 and 30-2a should be kept, i.e. FG 30-2 for DG, 30-2a for type 1 and 2 CG.

Proposal 3: For TBoMS, feature 30-2 should be included as prerequisite feature for feature 30-3.

Proposal 4: For TBoMS, split FG 30-3 into 2 FGs for DG and CG respectively.

Proposal 5: DMRS bundling related FGs should be per band.

- References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e.

- R4-2202368, Reply LS on Maximum duration for DMRS bundling, RAN4#101bis.