3GPP TSG RAN WG1 Meeting #108-e	R1-2201173

e-Meeting, February 21st – March 3rd, 2022

Source: 	ZTE

Title: 	Discussion on RRC parameters of Rel-17 MBS

Agenda item:	8.12.4

Document for:	Discussion/Decision

Introduction

In this contribution, we provide our views on RRC parameters [1] for Rel-17 NR MBS.

Discussion

2.1	Default value of dl-DataToUL-ACK-MulticastDciFormat4_1

As agreement copied below, if the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. That is, the default value of dl-DataToUL-ACK-MulticastDciFormat1_0 is {1, 2, 3, 4, 5, 6, 7, 8}.

Agreement A list of up to 8 k1 values can be configured by higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 to be applied to multicast DCI format 1_0 for RRC_CONNECTED UEs. If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. - The size of ‘PDSCH-to-HARQ_feedback timing indicator’ field of multicast DCI format 1_0 is fixed at 3 bits.

Note: RAN1 has changed names of the first DCI format for multicast scheduling from DCI format 1_0 to DCI format 4_1.

Based on the above analysis, we propose the following proposal.

- Proposal 1: Adopt the following revisions for the RRC parameter related with Default value of dl-DataToUL-ACK-MulticastDciFormat4_1.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_MBS-Core | Group scheduling for multicast | dl-DataToUL-ACK-MulticastDciFormat4_1 | new | List of timing for given group-common PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK-MulticastDciFormat4_1 applies to DCI format 4_1 for multicast (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat4_1 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 4_1. | dl-DataToUL-ACK-MulticastDciFormat4_1 | {1, 2, 3, 4, 5, 6, 7, 8} | per UL BWP, included in pucch-Config-Multicast1 | UE-specific | Note: RAN1 has changed names of the first DCI format for multicast scheduling from DCI format 1_0 to DCI format 4_1. Note: If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. Agreement: A list of up to 8 k1 values can be configured by higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 to be applied to multicast DCI format 1_0 for RRC_CONNECTED UEs. If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. ? The size of ‘PDSCH-to-HARQ_feedback timing indicator’ field of multicast DCI format 1_0 is fixed at 3 bits. |

Conclusion

In this contribution, the discussion on RRC parameter of Rel-17 NR MBS is presented. And the following proposal is also provided.

- Proposal 1: Adopt the following revisions for the RRC parameter related with Default value of dl-DataToUL-ACK-MulticastDciFormat4_1.

| WI code | Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_MBS-Core | Group scheduling for multicast | dl-DataToUL-ACK-MulticastDciFormat4_1 | new | List of timing for given group-common PDSCH to the DL ACK (see TS 38.213 [13], clause 9.1.2). The field dl-DataToUL-ACK-MulticastDciFormat4_1 applies to DCI format 4_1 for multicast (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 9.2.3). If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat4_1 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 4_1. | dl-DataToUL-ACK-MulticastDciFormat4_1 | {1, 2, 3, 4, 5, 6, 7, 8} | per UL BWP, included in pucch-Config-Multicast1 | UE-specific | Note: RAN1 has changed names of the first DCI format for multicast scheduling from DCI format 1_0 to DCI format 4_1. Note: If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. Agreement: A list of up to 8 k1 values can be configured by higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 to be applied to multicast DCI format 1_0 for RRC_CONNECTED UEs. If the higher layer parameter dl-DataToUL-ACK-MulticastDciFormat1_0 is not provided, k1 list {1, 2, 3, 4, 5, 6, 7, 8} is applied to multicast DCI format 1_0. ? The size of ‘PDSCH-to-HARQ_feedback timing indicator’ field of multicast DCI format 1_0 is fixed at 3 bits. |

Reference

3GPP RAN1#107bis-e, R1-2200699, Consolidated higher layers parameter list for Rel-17 NR, Moderator (Ericsson).