3GPP TSG RAN WG1 Meeting #108-e	                                                                R1-2201202

# e-Meeting, February 21th – March 3rd, 2022

# Title:	Draft reply LS on MPE information signaling

# Response to:	R2-2111600

# Release:	Release 17

# Work Item:	NR_feMIMO-Core

# Source:	ZTE [RAN1]

# To:	RAN2

# Contact Person:

Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank RAN2 for LS R2-2111600 on MPE information signaling in Rel-17. In the LS, RAN2 had provided corresponding questions as follows.

RAN2 has discussed the RAN1 parameter list for the Rel-17 FeMIMO WI and noted that there are new signalling aspects (i.e. parameters mpe-Reporting-FR2-r17, numberOfN and mpe-ResourcePool for the sub-feature "MultiBeam") for the MPE RRC configuration and corresponding changes to MAC CE signallling (i.e. P-MPR reporting in MAC CE for multiple beams according to RRC configuration). RAN2 understands that these parameters apply to the inter-cell beam management (ICBM) framework, but would like to understand if these MPE reporting changes would also apply to the multi-TRP (mTRP) framework?

RAN1 has the following reply to the above questions:

- Regarding inter-cell beam management (ICBM), RAN1 confirms that these RRC parameters including mpe-Reporting-FR2-r17, numberOfN and mpe-ResourcePool apply to the ICBM framework as well.

- Regarding mTRP framework, RAN1 has not discussed whether these MPE reporting changes would also apply to mTRP framework. Once applying to mTRP framework, we may need to consider two parameter combinations of {numberOfN, mpe-ResourcePool} corresponding to respective TRPs, which is analogous to the enhanced group based beam reporting in Rel-17.

# 2. Actions:

# To RAN2 group

RAN1 respectfully requests RAN2 to take the above information into consideration.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e		                               		May 16th –27th, 2022