3GPP TSG RAN WG1 Meeting #108-e	R1-2201216

e-Meeting, February 21st – March 3rd, 2022

Agenda Item: 	8.4.2

Source:	MediaTek Inc.

Title:	Enhancements on UL Time and Frequency Synchronisation for NR-NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#107-e [1], they were agreements with further discussions on identified aspects of validity duration for satellite ephemeris data. In this contribution, we provide our views on validity duration for satellite ephemeris data.

- 2 Discussion

## 2.1 Acquire new or additional assistance information

The issue about UL synchronization timer and associated assistance information (i.e. serving satellite ephemeris data or Common TA parameters) has been discussed in several RAN1 meetings. The following agreements were made:

Agreement: (RAN1 106-e) - A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration. FFS: Whether the same validity duration can be applied for Common TA. Agreement: (RAN1 106b-e) The UE assumes that it has lost uplink synchronization if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration. - FFS: details on how to acquire new or additional assistance information

Based on above agreements, when UE does not acquire new or additional assistance information within the validity duration, the UE declares the uplink synchronization lost, then UE needs a mechanism to reacquire the uplink synchronization.

Observation 1: UE’s behaviour needs to be specified when UL synchronization is lost, due to expiry of the UL validity timer.

One method is that UE still in RRC_Connected and reads the NTN-specific SIBx to re-acquire ephemeris and common TA parameters before UL validity duration expires. RAN2 has agreed “NTN-specific SIB is introduced (SIBx), scheduled by SIB1” [2].

R2-2201757: LS on NTN-specific SIB RAN2 has agreed to introduce a new NTN-specific SIB (SIBx) which is scheduled by SIB1. And at least the following serving cell information will be broadcast by SIBx: 1) Ephemeris; 2) Common TA parameters; 3) Validity duration for UL sync information; 4) t-Service (the timing information on when the serving cell is going to stop serving the area); 5) Cell reference location; 6) Epoch time; 7) K_mac; 8) Cell-specific Koffset; 9) Indication for network enabled/disabled TA report. Note that, based on RAN2 agreements so far, 4) can only be broadcast by quasi-earth fixed cells not by earth moving cells, and 5) can be broadcast by quasi-earth fixed cells (FFS for earth moving cells). RAN2 also agreed that the validity duration for UL sync information applies to the whole SIBx and UE acquires the updated SIBx when the timer expires (FFS if this applies only to RRC_CONNECTED mode or to RRC_IDLE UEs as well). Since some parameters also have RAN1 impact, RAN2 kindly asks if RAN1 foresees any problem (e.g., due to latency requirement) with the above agreements.

Observation 2: Before expiry of UL validity timer, the connected UE can read the NTN-specific SIBx to re-acquire new assistance information.

RAN2 also agreed that UE acquires the updated SIBx when the timer expires (FFS if this applies only to RRC_CONNECTED mode or to RRC_IDLE UEs as well).

Proposal 1: RAN2 can discuss on how to acquire new or additional assistance information if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration.

In case UL synchronization is lost for UE caused by expiry of UL validity timer, the UE can be assumed to be still synchronized on the DL. The UE can monitor PDCCH monitoring during the SI window to re-acquire the NTN-specific SIB for UE in RRC_Connected. If not, UE can re-acquire the NTN-specific SIB.

Observation 3: There is no limitation in NR NTN for UE to re-acquire the NTN-specific SIB when UL synchronization is lost.

While we are of the view that some parameters like cell-specific Koffset, k_mac, Indication for network enabled/disabled TA report, Cell reference location, t-Service, Validity duration for UL sync information do not need to be updated frequently, it is beneficial from UE’s perspective to obtain all the NTN-related parameters on one NTN-specific SIB.

Observation 4: On NTN cell access when paged, a UE may need to read the NTN-specific SIB within a typical time in the order of a second. It may not be necessary to re-acquire SIB-1 or SIB-2 assuming these SIBs have not changed within the current system information notification period.

Observation 5: It may be more flexible to have parameters like cell-specific Koffset, k_mac, Indication for network enabled/disabled TA report, Cell reference location, t-Service, Validity duration for UL sync information in NTN-specific SIB in case it is beneficial for the network to update these within the system information notification period.

## 2.2 GEO values for UL validity timer

In RAN #107-e, there was the agreement made for UL validity timer.

Agreement NTN validity duration is configured per cell and indicated to the UE in X bits with: - Value range { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, Infinity} - Unit is second - FFS (to be resolved in current meeting): Additional values for GEO

Considering the characteristic of GEO, Satellites and the earth are in a relatively static state. The UE can predict the delay drift over the service link and the common delay over the feeder link with an accuracy consistent with transmit timing error requirements with a validity timer duration in the order of half an hour. This configuration value was determined via simulations using ephemeris data and common delay data. Note that since the max Doppler shift is 0.93 ppm and 24 ppm for GEO and LEO respectively, the validation timer duration can be expected to be in the order of 30 (=~24 ppm/0.93 ppm) longer for GEO than for LEO. A validity timer duration of 30 seconds for LEO could be a reasonable target, so a reasonable target for GEO can be in the order of 900 seconds. The longer validity timer duration of 1800 seconds may be achievable depending on the UE complexity.

Proposal 2: Add the GEO candidate values for UL validity timer: {300 400 500 600 700 800 900 1000 1100 1200 1300 1400 1500 1600 1700 1800}.

Validity timer duration is configured per cell and indicated to the UE in X=5 bits with:

- Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, 300 400 500 600 700 800 900 1000 1100 1200 1300 1400 1500 1600 1700 1800}

- Unit is second

## 2.3 Combination of open and closed loop TA control

During last meeting, RAN1 discussed the issues on double correction of closed loop and opened loop TA. Based on the reply LS [3] that RAN4 has reached some agreements to address the impact on NTN UE UL timing accuracy due to “double-correction” issue.

R4-2120417: Reply LS on combination of open and closed loop TA control in NTN RAN4 has reached an agreement that RAN4 defines a requirement to ensure the impact on NTN UE UL timing accuracy due to “double-correction” issue is properly addressed. There are the following two alternatives for further discussion. - Option 1: - RAN4 to replace gradual timing adjustment requirement with NTN UE initial timing accuracy requirement, i.e. NTN UE initial timing accuracy requirement applies to all UL transmissions. - Option 2: - RAN4 to define a requirement based on the framework of gradual timing adjustment accuracy requirement, e.g. the requirement regulates the maximum amount of UE specific TA change of shot adjustment due to UE position change, the minimum and maximum aggregate adjustment rates. - FFS on whether the requirement regulating “double-correction” issue would be a stand-alone requirement - FFS on whether and how to incorporate the current gradual timing adjustment defined in 7.1.2.1 of TS38.133 - FFS on whether and how to incorporate UE specific change due to satellite position change and feeder link delay change - FFS on the detailed requirement values and the definition of reference time in terms of UL timing error measurement

In RAN1#107-e, the FL made proposal below, as there was no consensus on options discussed in RAN1.

R1-2111126 Issue#6: Combination of open and closed loop TA control Moderator understanding: The solution to resolve Issue#6 is up to the UE implementation to meet the RAN4 gradual timing adjustment requirement (yet to be defined).

Since RAN4 is discussing this issue, we have preference that RAN4 continues to discuss this issue, as there was no consensus in RAN1 past meetings.

Proposal 3: RAN4 can further discuss and conclude on combination of open and closed loop TA control in NTN.

## 2.4 Pseudo CR

CR 38.211

There is modification of the equation TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETs. Hence, Figure 4.1.1-1 UL-DL timing relation in Pseudo CR TS 38.211 [5] should be updated with NTA,adjcommon+NTA,adjUE.

Proposal 4: Agree Pseudo CR to TS 38.211 Section 4.3.1 to update Figure 4.1.1-1: Uplink-downlink timing relation with NTA,adjcommon+NTA,adjUE.

# 3 Conclusion

In this contribution, the following observations and proposals are presented:

Observation 1: UE’s behaviour needs to be specified when UL synchronization is lost, due to expiry of the UL validity timer.

Observation 2: Before expiry of UL validity timer, the connected UE can read the NTN-specific SIBx to re-acquire new assistance information.

Proposal 1: RAN2 can discuss on how to acquire new or additional assistance information if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration.

Observation 3: There is no limitation in NR NTN for UE to re-acquire the NTN-specific SIB when UL synchronization is lost.

Observation 4: On NTN cell access when paged, a UE may need to read the NTN-specific SIB within a typical time in the order of a second. It may not be necessary to re-acquire SIB-1 or SIB-2 assuming these SIBs have not changed within the current system information notification period.

Observation 5: It may be more flexible to have parameters like cell-specific Koffset, k_mac, Indication for network enabled/disabled TA report, Cell reference location, t-Service, Validity duration for UL sync information in NTN-specific SIB in case it is beneficial for the network to update these within the system information notification period.

Proposal 2: Add the GEO candidate values for UL validity timer: {300 400 500 600 700 800 900 1000 1100 1200 1300 1400 1500 1600 1700 1800}.

Validity timer duration is configured per cell and indicated to the UE in X=5 bits with:

- Value range {5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240, 300 400 500 600 700 800 900 1000 1100 1200 1300 1400 1500 1600 1700 1800}

- Unit is second

Proposal 3: RAN4 can further discuss and conclude on combination of open and closed loop TA control in NTN.

Proposal 4: Agree Pseudo CR to TS 38.211 Section 4.3.1 to update Figure 4.1.1-1: Uplink-downlink timing relation with NTA,adjcommon+NTA,adjUE.

# 4 References

- R1-2112890, “3GPP TSG-RAN WG1 Agreements under 8.4 up to eMeeting RAN1#107-e.”

- R2-2201757, “LS on NTN-specific SIB.”

- R4-2120417, “Reply LS on combination of open and closed loop TA control in NTN.”

- R1-2111126, “FL Summary #3 on enhancements on UL time and frequency synchronization for NR NTN.”

- R1-2112921, “CR 38.211”