3GPP TSG RAN WG1 Meeting #108-e  																		 			                                                                                                               R1-220xxxx

e-Meeting, February 21st  – March 3rd , 2022

Agenda Item:	8.14

Source:	Moderator (MediaTek)

Title:	Summary of [108-e-R17-RRC-IoT-NTN] Email discussion on Rel-17 RRC parameters for IoT NTN

Document for:	Discussion

# Introduction

A list of RRC parameters for IoT NTN was endorsed in RAN1#107-e [1, 2]. Further discussions aiming to finalize the list was done in RAN1#108-e.

This document is the Summary of [108-e-R17-RRC-IoT-NTN] Email discussion on Rel-17 RRC parameters for IoT to support NTN

- 1st check point for first LS in [108-e-R17-RRC]: February 24

- Final check point for second LS in [108-e-R17-RRC] if necessary: March 3

# Enhancements on UL time and frequency synchronization

## Related RRC parameters

Based on the agreements to date (up to RAN1#107-e) and the companies proposals submitted to RAN1#108-e, an updated list of RRC parameters pertinent to NR NTN and related the enhancements on UL time and frequency synchronization is given within the following table:

Moderator Note: A typo was corrected by the moderator for the for the NTACommonDrift-NB-r17 and NTACommonDrift-r17 as shown in red highlighted text to align with the RAN1#107-e agreement

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
| TACommonDrift | - 261935… + 261935 (i.e: -53.33 μs/s… +53.33 μs/s) | 0.2×10-3μs/s | 19 bits |

| Rel-17 NB-IoT NTN |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| NTA common-NB-r17 | NTACommon is a network-controlled common TA, and may include any timing offset considered necessary by the network. NTACommon with value of 0 is supported. The granularity of TACommon is 32.55208 ×10^(-3) μs Values are given in unit of corresponding granularity | 0, 8316827 | 0 | Cell | Cell-specific | 36.331 | For NB-IoT Ts=32.55208 ns (TS 36.211) | Stable |
| NTA commonDrift-NB-r17 | Indicate drift rate of the common TA The granularity of TACommonDrift is 0.2×10^(-3) μs⁄s Value are given in unit of corresponding granularity | - 261935… + 261935 | 0 | Cell | Cell-specific | 36.331 | For NB-IoT field is 19 bits TBC | Stable |
| NTACommonDriftVariation-NB-r17 | Indicate drift rate variation of the common TA The granularity of TACommonDriftVariation is 0.2×10^(-4) μs⁄s^2 Value are given in unit of corresponding granularity | 0, ..., 29479 | 0 | Cell | Cell-specific | 36.331 | For NB-IoT field is 15 bits TBC | Stable |
| ServingSatelliteEphemerisStateVectorX-NB-r17 | Indicate the x-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisStateVectorY-NB-r17 | Indicate the y-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisStateVectorZ-NB-r17 | Indicate the z-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 |  | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisStateVectorXv-NB-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisStateVectorYv-NB-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisStateVectorZv-NB-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisSemiMajorAxis-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Semi-major axis α The unit of Semi-major axis α is m | 6500 000… 43000 000 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisEccentricityE-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Eccentricity e | 0…0.015 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisArgumentOfPeriapsis-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Argument of periapsis ω The unit of Argument of periapsis ω is Radian | 0…2π | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatellite EphemerisLongitudeOfAscendingNode-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Longitude of ascending node Ω The unit of Longitude of ascending nod is Radian | 0…2π | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisInclinationI-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Inclination i The unit of Inclination is Radian | - π/2 ...+ π/2 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ServingSatelliteEphemerisMeanAnomalyM-NB-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Mean anomaly M [rad] at epoch time to The unit of Mean anomaly M is Radian | 0… 2π | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| ntnServingSatULSyncValidityDuration-NB-r17 | A validity duration configured by the network for satellite ephemeris data and common TA parameters which indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. The unit of ntnUlSyncValidityDuration is second | { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} | 0 | Cell | Cell-specific | 36.331 | For NB-IoT • FFS: Additional values for GEO (e.g. up to 2 hours) The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time. A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB. Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information. | Stable |
| EpochTime-NB-r17 | Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. | 0 to 1023 to indicate SFN and 0 to 9 to indicate the sub-frame number. | 0 | Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| TransmissionDurationNPRACH-NB-r17 | Configuration of UL transmission segment is indicated on SIB | Format 0 and 1: {2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)} -Format 2: {1.6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ) } | 0 | TBD | TBD | 36.331 | For NB-IoT N | Stable |
| TransmissionDurationNPUSCH-NB-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED | [2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms ] | 0 | TBD | TBD | 36.331 | For NB-IoT | Stable |

| Rel-17 eMTC NTN |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| NTA common-r17 | NTACommon is a network-controlled common TA, and may include any timing offset considered necessary by the network. NTACommon with value of 0 is supported. he granularity of TACommon is 32.55208 ×10^(-3) μs Values are given in unit of corresponding granularity | 0, ..., 8316827 | 0 | Cell | Cell-specific | 36.331 | For NB-IoT Ts=32.55208 µs (TS 36.211), field is 23 bits | Stable |
| NTA commonDrift-r17 | Indicate drift rate of the common TA The granularity of TACommonDrift is 0.2×10^(-3) μs⁄s Value are given in unit of corresponding granularity | - 261935… + 261935 | 0 | Cell | Cell-specific | 36.331 | For eMTC field is 19 bits TBC | Stable |
| NTACommonDriftVariation-r17 | Indicate drift rate variation of the common TA The granularity of TACommonDriftVariation is 0.2×10^(-4) μs⁄s^2 Value are given in unit of corresponding granularity | 0, ..., 29479 | 0 | Cell | Cell-specific | 36.331 | For eMTC field is 15 bits TBC | Stable |
| ServingSatelliteEphemerisStateVectorX-r17 | Indicate the x-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range based on NR NTN progress | Stable |
| ServingSatelliteEphemerisStateVectorY-r17 | Indicate the y-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range for based on NR NTN progress | Stable |
| ServingSatelliteEphemerisStateVectorZ-r17 | Indicate the z-coordinate of serving Satellite position state vector in ECEF The unit is m | -42200000…+42200000 |  | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range f based on NR NTN progress | Stable |
| ServingSatelliteEphemerisStateVectorXv-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range based on NR NTN progress | Stable |
| ServingSatelliteEphemerisStateVectorYv-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range for based on NR NTN progress | Stable |
| ServingSatelliteEphemerisStateVectorZv-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, TBD Value range f based on NR NTN progress | Stable |
| ServingSatelliteEphemerisSemiMajorAxis-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Semi-major axis α The unit of Semi-major axis α is m | 6500 000… 43000 000 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ServingSatelliteEphemerisEccentricityE-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Eccentricity e | 0…0.015 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ServingSatelliteEphemerisArgumentOfPeriapsis-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Argument of periapsis ω The unit of Argument of periapsis ω is Radian | 0…2π | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ServingSatellite EphemerisLongitudeOfAscendingNode-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Longitude of ascending node Ω The unit of Longitude of ascending nod is Radian | 0…2π | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ServingSatelliteEphemerisInclinationI-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Inclination i The unit of Inclination is Radian | - π/2 ...+ π/2 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ServingSatelliteEphemerisMeanAnomalyM-r17 | Indicate the following ephemeris orbital parameter for the serving satellite: - Mean anomaly M [rad] at epoch time to The unit of Mean anomaly M is Radian | 0… 2π | 0 | Per Cell | Cell-specific | 36.331 | For eMTC, | Stable |
| ntnServingSatULSyncValidityDuration-r17 | A validity duration configured by the network for satellite ephemeris data and common TA parameters which indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris. The unit of ntnUlSyncValidityDuration is second | { 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 55, 60, 120, 180, 240} | 0 | Cell | Cell-specific | 36.331 | For eMTC • FFS: Additional values for GEO (e.g. up to 2 hours) The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time. A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB. Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information. | Stable |
| EpochTime-r17 | Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. | 0 to 1023 to indicate SFN and 0 to 9 to indicate the sub-frame number. | 0 | Cell | Cell-specific | 36.331 | For eMTC | Stable |
| TransmissionDurationPRACH-r17 | Configuration of UL transmission segment is indicated on SIB | (TCP+TSEQ+TGP), 2*(TCP+TSEQ+TGP), 4*(TCP+TSEQ+TGP), 8*(TCP+TSEQ+TGP), 16*(TCP+TSEQ+TGP), 32*(TCP+TSEQ+TGP), 64*(TCP+TSEQ+TGP), 128*(TCP+TSEQ+TGP) | 0 | TBD | TBD | 36.331 | For eMTC | Stable |
| TransmissionDurationPUSCH-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED | • Full-PRB allocation (unit: subframes): 2 4 8 16 32 64 128 256 • Sub-PRB allocation (unit: resource units): 1 2 4 8 16 32 64 128 | 0 | TBD | TBD | 36.331 | For eMTC, | Stable |
| TransmissionDurationPUCCH-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED | (unit: subframes): 2 4 8 16 32 64 128 | 0 | TBD | TBD | 36.331 | For eMTC, | Stable |

## FIRST ROUND Discussions

Qualcomm discussed that the RRC parameter list is missing the RRC parameter corresponding to the indication of the 2 LSBs of the ARFCN in the MIB for NB-IoT, in accordance with the relevant agreement in RAN1#107-e. This should be added to the RRC parameter list [3]

Huawei, HiSilicon made the following observation and proposal [4]:

Observation 1: The velocity range (+/- 8000 m/s) is not correct for the current agreement.

Proposal 1: The velocity range of +/- 8000 m/s should change to [-7864, 7863] according to the bit allocation and granularity.

Moderator proposal 1: Add an RRC parameter to the RRC parameter list, corresponding to the 2 LSBs of the ARFCN in the MIB for NB-IoT and eMTC, in accordance with the RAN1#107-e agreement.

Agreement

For DL synchronization enhancements:

- Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used, otherwise for bands where RAN4 can introduce a 200 kHz channel raster there is no signalling of the part-of ARFCN indication on MIB.

Agreement

For IoT NTN, indicate two LSBs of the ARFCN in the MIB.

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PartOfArfcnMIB -NB-r17 | • Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used | 00, 01, 10, 11 | 00 | Cell | Cell-specific | 36.331 | For NB-IoT | New |
| PartOfArfcnMIB -r17 | • Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used | 00, 01, 10, 11 | 00 | Cell | Cell-specific | 36.331 | For eMTC | New |

Moderator proposal 2: Revise RRC parameter for velocity range of   +/- 8000 m/s to [-7864, 7863] according to the bit allocation and granularity in RAN1#107-e agreement.

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ServingSatelliteEphemerisStateVectorXv-NB-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorYv-NB-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorZv-NB-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorXv-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |
| ServingSatelliteEphemerisStateVectorYv-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |
| ServingSatelliteEphemerisStateVectorZv-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |

Several companies contributed their views on additional values for GEO

- Nokia commented to re-use NR NTN solution for IoT NTN [5].

- Apple, CMCC, Xiaomi proposed that larger configuration values are used for GEO [6,7,8]

- MediaTek propose reasonable target is 900 seconds for GEO, with up to 1800 seconds if needed.[9]

- Thales proposed in NR NTN additional value of 900s (instead of infinity) for GEO with a 4-bit indication.[10].

- Ericsson proposed additional values {900 s, 1800 s, 3600 s, 7200 s} and remove values {25 s, 35 s, 45 s, 55 s}, with a 4-bit indication [11].

- Marvenir proposed up to 2 hours [12].

- ZTE proposed no additional values [13].

- Qualcomm proposed for GEO NTNs, a UE applies common TA only in accordance with the common TA term, TACommon, and is not required to process the drift/variation parameters TACommonDrift and TACommonDriftVariation [14].

Moderator recommendation on additional values (as in FL Summary for AI 8.14.1 – Section 5.2):

- First discuss for additional values of validity timer for GEO in NR NTN AI 8.4.2. For IoT NTN, adopt the NR NTN agreement without modification for additional values of validity timer for GEO.

- Companies are encouraged to provide comments on updated RRC parameters list (section 3.1):

| Ericsson | Issue#1: Typo in the “Comment” field for “NTA common-r17”: NB-IoT should be replaced by eMTC Issue#2: The description/value range for “TransmissionDurationPUSCH-r17” needs to be updated to reflect the following agreement in RAN1#107-e. This is because the current value range for sub-PRB allocation is valid for 6 tones only. The cases for sub-PRB allocation for 3 and 2-out-of-3 tones need to be captured too. Agreement UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB. - For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH - The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively. Issue#3: Proposal 1 mentions NB-IoT but omits eMTC although RRC parameters for both are included in the table. Moreover, the table description should be updated to clarify that it is the 2 LSB bits of the ARFCN that are signaled. Issue#4: Support Proposal 2. The RAN1 agreements from RAN1#107-e on value ranges for the state vector ephemeris parameters are not consistent with the agreements on field sizes and step sizes. To align, we propose to adjust the value ranges based on current agreements on field sizes and step sizes: - Position state vectors: 26 bits with step size 1.3 m à value range = {-33554432..33554431} * 1.3 m = {-43620761.6 m, -43620760.3 m,…, 43620759.0 m, 43620760.3 m} - Velocity state vectors: 18 bits with step size 0.06 m/s à value range = {-131072..131071} * 0.06 m/s = {-7864.32 m/s, -7864.26 m/s,…, 7864.20 m/s,7864.26 m/s} |
| --- | --- |
| SONY2 | [Issue#1] NTA common-NB-r17 In case the reference point for timing alignment is at the satellite, the nTA-Common parameter will not integrate the feeder link delay, but only any TA margin value, that should be negative to compensate for TA overestimation. So in that case, we think the nTA-Common parameter should allow negative values. Additionally, the value range should be separated with an ellipsis: 0 … 8316827 [Issue#2] NTA common-r17 Same comments as for NTA common-NB-r17: In case the reference point for timing alignment is at the satellite, the nTA-Common parameter will not integrate the feeder link delay, but only any TA margin value, that should be negative to compensate for TA overestimation. So in that case, we think the nTA-Common parameter should allow negative values. Additionally, the value range should be separated with an ellipsis: 0 … 8316827 [Issue#3] NTA common-r17: comment section Suggest the following corrections: For NB-IoT eMTC Ts=32.55208 µs ns (TS 36.211), field is 23 bits [we don’t even see the need for a comment on what the value of Ts is since the Ts value seems only marginally related to what is in the “description” column Note: the yellow highlight is issue#1 from Ericsson. We think the cyan highlight also needs to be addressed. [Issue#4] TBD values in ephemeris data For the following eMTC parameters, we can presumably use the same values as in the NB-IoT table ServingSatelliteEphemerisStateVectorX-r17 ServingSatelliteEphemerisStateVectorY-r17 ServingSatelliteEphemerisStateVectorZ-r17 ServingSatelliteEphemerisStateVectorXv-r17 ServingSatelliteEphemerisStateVectorYv-r17 ServingSatelliteEphemerisStateVectorZv-r17 While “moderator proposal 2” removes the “TBD Value range based on NR NTN progress” for the cyan parameters, we can also remove this TBD text for the yellow highlighted parameters. |

## FIRST ROUND Discussion conclusions

Typo in the “Comment” field for “NTA common-r17”: NB-IoT should be replaced by eMTC was corrected in RRC parameter spreadsheet

“TBD Value range  based on NR NTN progress” in ServingSatelliteEphemerisStateVectorXv-r17, ServingSatelliteEphemerisStateVectorYv-r17, ServingSatelliteEphemerisStateVectorZv-r17 were removed

Negative value in range for nTA-Common-NB-r17 and nTA-Common-r17 parameters  in  NB-IoT and eMTC will require discussons in RAN1. The issue of TA margin is not new.

Negative value in range for NTACommonDriftVariation-NB-r17 and NTACommonDriftVariation- r17 parameters  in  NB-IoT and eMTC is under discussion in 8.4.2 for GEO.

Since there was no downscoping of K candidate values for NB-IoT, the bracket in RRC parameters for TransmissionDurationNPUSCH-NB-r17 can be removed

Agreement:

- For NB-IoT/eMTC NTN, the network configures one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH in a k-bit field.

- For NB-IoT, maximum 3-bit field with a maximum number of K=8 candidate values 2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms

- FFS: Down scoping of K candidate values, size of k-bit field

Description/value range for “TransmissionDurationPUSCH-r17 is updated based on RAN1 agreement

Agreement

For eMTC PUCCH, a 3-bit field to indicate K=7 values for the uplink transmission segment duration:

·         2 4 8 16 32 64 128 subframes

Agreement

UL transmission segment duration with one value X per NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC may be indicated on SIB.

- For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH

- The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively.

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range |
| --- | --- | --- | --- | --- |
| TransmissionDurationPUSCH-NB-r17 | new | TransmissionDurationNPUSCH-NB-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED UL transmission segment duration with one value X per NPUSCH for NB-IoT may be indicated on SIB. • For NB-IoT/eMTC, X is one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH/PUCCH | [2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms ] |

| Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range |
| --- | --- | --- | --- | --- |
| TransmissionDurationPUSCH-r17 | new | TransmissionDurationPUSCH-r17 | Configuration of UL transmission segment is indicated on SIB and reconfigured by RRC signalling in RRC_CONNECTED For eMTC PUCCH, a 3-bit field to indicate K=7 values for the uplink transmission segment duration: · 2 4 8 16 32 64 128 subframes UL transmission segment duration with one value X per PUSCH/PUCCH for eMTC may be indicated on SIB. - The value X for eMTC PUSCH applies for full-PRB allocation and should be divided by 2, 4 and 8 for sub-PRB allocation of 6, 3 and 2-out-of-3 tones allocation, respectively. | • Full-PRB allocation (unit: subframes): 2 4 8 16 32 64 128 256 • Sub-PRB allocation (unit: resource units for 6 tones): 1 2 4 8 16 32 64 128 Sub-PRB allocation (unit: resource units for 3 tones): 1 2 4 8 16 32 64 Sub-PRB allocation (unit: resource units for 2-out-of-3 tones): 1 2 4 8 16 32 |

Add an RRC parameter to the RRC parameter list, corresponding to the 2 LSBs of the ARFCN in the MIB for NB-IoT and eMTC, in accordance with the RAN1#107-e agreement.

Agreement

For DL synchronization enhancements:

- Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used, otherwise for bands where RAN4 can introduce a 200 kHz channel raster there is no signalling of the part-of ARFCN indication on MIB.

Agreement

For IoT NTN, indicate two LSBs of the ARFCN in the MIB.

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PartOfArfcnMIB -NB-r17 | • Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used | 00, 01, 10, 11 | 00 | Cell | Cell-specific | 36.331 | For NB-IoT | New |
| PartOfArfcnMIB -r17 | • Signal Part-of ARFCN indication on MIB for bands where RAN4 cannot introduce a 200 kHz channel raster and the legacy 100 kHz raster is used | 00, 01, 10, 11 | 00 | Cell | Cell-specific | 36.331 | For eMTC | New |

Revise RRC parameter for velocity range of   +/- 8000 m/s to [-7864, 7863] according to the bit allocation and granularity in RAN1#107-e agreement.

- 8000…+ 8000

| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ServingSatelliteEphemerisStateVectorXv-NB-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorYv-NB-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 8000…+ 8000 - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorZv-NB-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | New-Stable |
| ServingSatelliteEphemerisStateVectorXv-r17 | Indicate the x-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | -- 8000…+ 8000 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |
| ServingSatelliteEphemerisStateVectorYv-r17 | Indicate the y-coordinate of serving Satellite velocity state vector in ECEFThe unit is m/s | - 8000…+ 8000 - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |
| ServingSatelliteEphemerisStateVectorZv-r17 | Indicate the z-coordinate of serving Satellite velocity state vector in ECEF The unit is m/s | - 8000…+ 8000 - 7864…+ 7863 | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | New-Stable |

# Timing relationship enhancements

## Related RRC parameters

Based on the agreements to date (up to RAN1#107-e) and the companies proposals submitted to RAN1#108-e, there is no update on RRC parameters related to the timing relationship enhancements for IoT NTN.

| Rel-17 NB-IoT NTN |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| CellSpecificKoffset-NB -r17 | The CellSpecific_K_offset is a scheduling offset used for the timing relationships that need to be modified for NTN [see TS 36.2xy]. The unit of K_offset is number of slots for a given subcarrier spacing of 15 kHz For IoT NTN, adopt the NR NTN agreement without modification for FR1: (a) the value range (i.e. 1 ms), (b) the quantity signalled (e.g. a differential UE specific K_offset) for the UE specific K_offset. For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE. | 0 ...1023 ms | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| K_mac-NB-r17 | K_mac is a scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH [see TS 36.2xy]. When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. The unit of K_mac is number of slots for a given subcarrier spacing. | 1 ...512 ms. | 0 | Per Cell | Cell-specific | 36.331 | For NB-IoT | Stable |
| TA_Report-NB-r17 | UE-specific TA reporting is supported in IoT-NTN Detail contents can be either a TA or UE location, as configured by network | TBD | 0 | Per UE | UE-specific | 36.331 | For NB-IoT NR NTN agreed the granularity of the reported TA is slot. • FFS how to round TA value to slot level granularity | Stable |

| Rel-17 eMTC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment | Status [Post 107-e] |
| CellSpeificKoffset-r17 | The CellSpecific_K_offset is a scheduling offset used for the timing relationships that need to be modified for NTN [see TS 36.2xy]. The unit of K_offset is number of slots for a given subcarrier spacing of 15 kHz For IoT NTN, adopt the NR NTN agreement without modification for FR1: (a) the value range (i.e. 1 ms), (b) the quantity signalled (e.g. a differential UE specific K_offset) for the UE specific K_offset. For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE. | 0 ...1023 ms | 0 | Per Cell, Per UE | Cell-specific, UE-specific | 36.331 | For eMTC | Stable |
| K_mac-r17 | K_mac is a scheduling offset provided by network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH [see TS 36.2xy]. When UE is not provided by network with a K_mac value, UE assumes K_mac = 0. The unit of K_mac is number of slots for a given subcarrier spacing. | 1 ...512 ms. | 0 | Per Cell | Cell-specific | 36.331 | For eMTC | Stable |
| TA_Report-r17 | UE-specific TA reporting is supported in IoT-NTN Detail contents can be either a TA or UE location, as configured by network | TBD | 0 | Per UE | UE-specific | 36.331 | For eMTC NR NTN agreed the granularity of the reported TA is slot. • FFS how to round TA value to slot level granularity | Stable |

## FIRST ROUND Discussions

There is no company proposal on timing relationship enhancements for RRC parameters in IoT NTN

- Companies are encouraged to provide comments on RRC parameters list for timing relationship enhancements if necessary:

| SONY2 | [Issue#1] typo in naming of CellSpecificKoffset-r17 Typo: CellSpecificKoffset-r17 [Issue#2] CellSpecificKoffset-r17 – per cell This parameter is listed as “per cell, per UE” and “cell-specific, UE-specific”. This is inconsistent with both NB-IoT and NR NTN. We understand that this parameter is just cell specific. Hence, the following correction is proposed. “Per” column: Per Cell, Per UE “UE-specific or Cell-specific” column Cell-specific, UE-specific |
| --- | --- |

## FIRST ROUND Discussion Conclusions

Typo corrected for CellSpecificKoffset-r17

Corrected CellSpecificKoffset-r17 as “per cell, UE-specific”

This parameter is listed as “per cell, per UE” and “cell-specific, UE-specific”. This is inconsistent with both NB-IoT and NR NTN. We understand that this parameter is just cell specific. Hence, the following correction is proposed.

# Reference

|  | - R1-2111377, Moderator (MediaTek), List of IoT over NTN Rel-17 RRC parameters, RAN1#107-e, November 2021 - R1-2112975, Moderator (Ericsson), Consolidated higher layers parameter list for Rel-17 LTE, RAN1#107-e, November 2021 - R1-2201652, Qualcomm, Enhancements to time and frequency synchronization, RAN1#107-e, February 2022 - R1-220941, Huawei, Maintenance on time and frequency synchronization enhancement for IoT in NTN, RAN1#107-e, February 2022 - R1-2201587, Nokia, Nokia Shanghai Bell, Remaining issues of time and frequency synchronization for NB-IoT/eMTC over NTN, RAN1#107-e, February 2022 - R1-2111904, Apple, Time and Frequency Synchronization in IoT NTN, RAN1#106bis-e, October 2021 - R1-2111633, CMCC, Enhancements on time and frequency synchronization for IoT NTN, RAN1#106bis-e, October 2021 - R1-2111557, Xiaomi, Discussion on time and frequency synchronization for IoT NTN, RAN1#106bis-e, October 2021 - R1-2201217, MediaTek, Enhancements to time and frequency synchronization for IoT NTN, RAN1#107-e, February 2022 - R1-2201011, Thales, Maintenance on UL timing and frequency synchronization in NTN, RAN1#107-e, February 2022R1-2111420, Ericsson, On time and frequency synchronization enhancements for IoT NTN, RAN1#106bis-e, October 2021 - R1-2202479, Marvenir, Enhancements to time and frequency synchronization, RAN1#107-e, February 2022R1-2201275, OPPO, Discussion on enhancements to time and frequency synchronization, RAN1#107-e, February 2022 - R1-2111662, ZTE, Discussion on synchronization for IoT-NTN, RAN1#106bis-e, October 2021 - R1-2201652, Qualcomm, Enhancements to time and frequency synchronization, RAN1#107-e, February 2022 |
| --- | --- |