3GPP TSG RAN WG1 Meeting #108-e	R1-2201221

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.16.14

Source:	MediaTek Inc.

Title:	UE features for IoT over NTN

Document for:	Discussion and Decision

- Introduction

In RAN#107-e, the RAN1 UE features are provided in [1] for Rel-17 IoT NTN. In this contribution, we provide our perspectives on the update RAN1 UE features.

- Discussion

FG 2-1

- Component 1-1 can be kept without yellow highlight considering agreemens in RAN1 107e [2].

Agreement The UE autonomously determines its GNSS validity duration X and reports information associated with this valid duration to the network via RRC signalling. - X = {10s, 20s, 30s, 40s, 50s, 60s, 5 min, 10 min, 15 min, 20 min, 25 min, 30 min, 60 min, 90 min, 120 min, infinity}

- Component 7 can be kept without yellow highlight considering taking NR NTN UE features [3] as baseline.

- Component 8 should delete the description of “[in DL] [and] [in UL]”.

- Component 14 and Component 15 should be merged to “Start of ra-ResponseWindow is delayed by an offset. The RA-window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset)”, in order to align with RAN2 agreements [4] on start of RAR window.

Agreement(RAN2 115e) Start of ra-ResponseWindow is delayed by an offset. Postpone discussion on the offset value until further agreements regarding RACH are made in RAN1. Agreement(RAN2 116e) The ra window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset)

- Component 16 can be kept without yellow highlight considering agreemens in RAN1 agreements.

Agreement For IoT NTN, the information of K_mac is carried in system information.

FG 2-1a

- The component can be change to “Support segment duration configuration by the network for NGSO”, considering only agreements for NGSO have been made.

Agreement UL Segmented transmission NPRACH/NPUSCH for NB-IoT is not supported in GEO based on UE feature.

# Conclusion

In this contribution, we discuss the Rel-17 UE features for NB-IoT and eMTC over NTN. The following proposals are presented:

Proposal 1: Agree on the updated UE features in the Appendix

FG 2-1 1

- Components 1-1, 7, 16: keep text without yellow highlight

- Component 8: delete “[in DL] [and] [in UL]”

- Merge components 14/15 as “Start of ra-ResponseWindow is delayed by an offset. The RA-window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset)”

FG  2-1a

- Update to add “for NGSO” in component “Support segment duration configuration by the network for NGSO”

# References

- R1-2111813, “Summary of UE features for IoT over NTN.”

- R1-2112891, “3GPP TSG-RAN WG1 Agreements for Release-17 IoT NTN (post RAN1#106-bis-e).”

- R1-2112902, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #107-e.”

- R2 115e Meeting Reports

- R2 116e Meeting Reports

# Appendix A

Proposed revision for Rel-17 UE features for LTE NB-IoT eMTC over NTN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS implementation measurements 1-1. UE report the validity duration of GNSS 2. [Indicate/receive] serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Derive epoch time of serving satellite ephemeris from implicit epoch time] 3. Receive ephemeris orbital parameter for the serving satellite 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris. 5. UE specific TA calculation [in [RRC_IDLE and RRC_CONNECTED state] based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) (if any) 7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network 11. Support segment duration configuration is by the network [and updating time and frequency pre-compensation between segments but not within segments during UL repetition] 12. UE receives cell specific K_offset 13. UE applies cell specific K_offset in timing relationship enhancements 14. Start of ra-ResponseWindow is delayed by an offset. The RA-window start offset is defined as sum (current offset, UE-eNB RTT) and current offset is defined in TS36.321 (FFS if applicable to NB-IoT 41ms offset). UE estimates of UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] 16. UE receives cell specific K_mac 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions |  | [No] Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Basic IoT over NTN support for NSGO scenarios Segment duration configuration | 1. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 2. Support segment duration configuration by the network for NGSO. |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |